3GPP TSG-RAN WG4 Meeting #78bis	R4-161597

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	7.10.3

Source: 	Nokia

Title: 	Analysis and simulation results on BS RF filtering for 2.6 GHz Supplemental DL band for Region 1

Document for:	Discussion

# 1.	Introduction

The revised WI proposal [1] for 2.6 GHz Supplemental DL band for Region 1 was approved in RAN#71. One of the objectives of this WI is to specify the band numbering and RF characteristics of the new LTE FDD 2.6GHz SDL band (2570-2620 MHz) for Region 1.

In this paper, we provide an analysis on the Base Station (BS) Radio Frequency (RF) Transmit (TX) and Receive (RX) filter requirements for the 2.6 GHz Supplemental DL band for Region 1, based on the coexistence parameters used to define the 3GPP requirements in the RAN4 specifications, and provide simulation results to show the feasibility of the RF filter implementations to meet such requirements.

# 2.	Discussion

The band plan of the 2.6 GHz Supplemental DL band for Region 1 is shown in Figure 1 below.

Figure 1: 2.6 GHz Supplemental DL band plan for Region 1

It can be seen from Figure 1 that the 2.6 GHz Supplemental DL band for Region 1 is adjacent to the Band 7 UL. Hence the Band 7 BS receiver should be protected from its own or different new-SDL band BS transmitter.

To protect a BS receiver from its own or different BS transmitter, the operators should ensure the following:

- Its own or different BS transmitter unwanted emissions received by the BS receiver do not cause unacceptable BS receiver desensitization.

- The total carrier power of its own or different BS transmitter attenuated by the BS receiver RF, IF and baseband filters do not result in BS receiver blocking.

Since there is no frequency separation between the new-SDL band and Band 7 UL, for this analysis we assume a 5 MHz guard band is allowed within the new-SDL band. We also assume the averaged RF TX filter passband insertion loss across the entire passband shall be <1.0 dB, and the average passband insertion loss across the first 5 MHz in the new-SDL band (2575-2580 MHz) shall be <1.2 dB. For the Band 7 RF RX filter, we assume the averaged passband insertion loss across the entire passband (2500-2570 MHz) shall be <2.0 dB.

## 2.1	Transmitter spurious emissions requirements for co-existence

The BS spurious emissions limit for co-existence with systems operating in other frequency bands is currently specified in 3GPP [2] as -49 dBm/MHz in the UL frequency range of the BS receiver for Wide Area BS. This requirement value is obtained assuming a 5 dB BS noise figure, a 67 dB BS to BS minimum coupling loss (MCL) and a 0.8 dB victim BS receiver desensitization [3].

For this analysis we assume the out-of-band (OOB) emission from the power amplifier (PA) is designed to meet the -15 dBm/MHz Category B operating band unwanted emissions specified in 3GPP [2], then the required rejection by the RF filter to meet the -49 dBm/MHz emissions limit will be (49 – 15 =) 34 dB.

First we assume the same BS transmitter is used to cover both the new-SDL band and Band 7 DL frequency ranges, i.e. the passband frequencies of the RF TX filter will be 2575-2690 MHz. The BS RF TX filter simulation results were provided in [4] and copied below in Figure 2 for ease of reference. It can be seen that the required minimum rejection of 34 dB over the Band 7 UL frequency range can be met (with likely drift of ~100 kHz due to manufacturing and environmental variations), with an acceptable transmit passband insertion loss of <1.0/1.2 dB (including an additional ~0.2 dB for connectors and internal transmission lines). Note that temperature-compensation and implementation margin were not included in the simulation, thus the simulation results should only be used as an approximation but not the expectation of actual products performance.

Figure 2: Simulated BS RF TX (2575-2690 MHz) Filter Characteristics (BS co-existence requirements)

Next we assume a separate BS transmitter is used to cover the new-SDL band frequency ranges, i.e. the passband frequencies of the RF TX filter will be 2575-2620 MHz. The BS RF TX filter simulation results are shown below in Figure 3. It can be seen that the required minimum rejection of 34 dB over the Band 7 UL frequency range can be met with a significant reduction of four resonators (with likely drift of ~100 kHz due to manufacturing and environmental variations), with an acceptable transmit passband insertion loss of <1.0/1.2 dB (including an additional ~0.2 dB for connectors and internal transmission lines). Note that temperature-compensation and implementation margin were not included in the simulation, thus the simulation results should only be used as an approximation but not the expectation of actual products performance.

Figure 3: Simulated BS RF TX (2575-2620 MHz) Filter Characteristics (BS co-existence requirements)

## 2.2	Transmitter spurious emissions requirements for co-location

The BS spurious emissions limit for protection of the BS receiver of own or different (co-located) BS is currently specified in 3GPP [2] as -96 dBm/100 kHz (-86 dBm/MHz) in the UL frequency range of the BS receiver for Wide Area BS. This requirement value is obtained assuming a 5 dB BS noise figure, a 30 dB BS to BS MCL and a 0.8 dB victim BS receiver desensitization [3].

For this analysis we assume the OOB emission from the PA is designed to meet the -15 dBm/MHz Category B operating band unwanted emissions specified in 3GPP [2], then the required rejection by the RF filter to meet the -86 dBm/MHz emissions limit will be (86 – 15 =) 71 dB. Note that for BS with common transmit and receive antenna port, there may not be any considerable coupling loss between the BS transmitter and receiver, hence the 30 MCL cannot be used and the required rejection by the RF filter shall be (71 + 30 =) 101 dB.

First we assume the same BS transmitter is used to cover both the new-SDL band and Band 7 DL frequency ranges, i.e. the passband frequencies of the RF TX filter will be 2575-2690 MHz. The BS RF TX filter simulation results were provided in [4] and copied below in Figure 4 for ease of reference. It can be seen that the required minimum rejection of 71 dB over the Band 7 UL frequency range cannot be met within the 5 MHz guard band while maintaining the acceptable RF filter passband insertion loss. Note that temperature-compensation and implementation margin were not included in the simulation, thus the simulation results should only be used as an approximation but not the expectation of actual products performance.

Figure 4: Simulated BS RF TX (2575-2690 MHz) Filter Characteristics (BS co-location requirements)

Next we assume a separate BS transmitter is used to cover the new-SDL band frequency ranges, i.e. the passband frequencies of the RF TX filter will be 2575-2620 MHz. The BS RF TX filter simulation results are shown below in Figure 5. It can be seen that the required minimum rejection of 71 dB over the Band 7 UL frequency range still cannot be met within the 5 MHz guard band while maintaining the acceptable RF filter passband insertion loss. Note that temperature-compensation and implementation margin were not included in the simulation, thus the simulation results should only be used as an approximation but not the expectation of actual products performance.

Figure 5: Simulated BS RF TX (2575-2620 MHz) Filter Characteristics (BS co-location requirements)

It can be seen from Figures 4 and 5 the RF filter can achieve ~50 dB over the upper frequency range of Band 7 UL in both cases. On the other hand, the required RF filter rejection is 71 dB for co-location based upon a BS to BS MCL of 30 dB. Therefore, a minimum of 20 dB additional rejection (via external filter which will increase the insertion loss) or antenna isolation between the BS transmitter and receiver (via site-engineering solution) is needed for the two BS to be co-located.

## 2.3	Receiver blocking

Now we look at Band 7 BS receiver blocking requirements in order to avoid receiver blocking by the carrier power of the new-SDL band BS transmitter. Taking into account the BS to BS MCL of 30 dB and the interfering BS TX carrier power of 46 dBm, the Band 7 BS receiver desensitization (i.e. receiver blocking) caused by the new-SDL band BS TX carrier power received at the Band 7 BS Equipment Antenna Connector (EAC) should also be 0.8 dB to match the Band 7 receiver desensitization of 0.8 dB caused by the new-SDL band BS transmitter spurious emission power falling into the Band 7 BS receiver.

If it is assumed that the blocking level of +16 dBm in the new-SDL band BS TX band (2575-2620 MHz) does not activate the Band 7 BS receiver protection circuit (e.g. an attenuator) or result in receiver overload/saturation, the Band 7 BS receiver blocking requirement of +16 dBm with 6 dB receiver desensitization for collocation with the new-SDL band BS transmitter can be converted into a receiver blocking requirement of +4 dBm with 0.8 dB receiver desensitization.  We assume that Band 7 BS receiver blocking without RF RX filter can comply with the 3GPP [2] in-band general blocking requirement of -43 dBm with 5 MHz guard band and 6 dB receiver desensitization which can be converted into a blocking requirement of -55 dBm with 0.8 dB receiver desensitization.  Consequently, if the new-SDL band BS TX carrier power received at the Band 7 BS EAC is +4 dBm and the acceptable Band 7 BS receiver sensitization is 0.8 dB, then the Band 7 BS RF RX filter should provide (4 + 55 =) 59 dB rejection in the new-SDL BS TX frequency range (2575-2620 MHz) to co-locate with the new-SDL BS transmitter. When the new-SDL BS TX carrier power received at the Band 7 BS EAC increases from +4 dBm to +16 dBm, the Band 7 BS RF RX filter should provide (59 + 16 - 4 =) 71 dB rejection in the new-SDL band BS TX frequency range.

The RX RF filter simulation results are shown below in Figure 6. It can be seen that the required minimum rejections (for 0.8 dB receiver desensitization) of 71 dB over the new-SDL TX frequency range can be achieved (with likely drift of ~100 kHz due to manufacturing and environmental variations), with an acceptable receive passband insertion loss of <2.0 dB (including an additional ~0.2 dB for connectors and internal transmission lines). Note that temperature-compensation and implementation margin were not included in the simulation, thus the simulation results should only be used as an approximation but not the expectation of actual products performance.

Figure 6: Simulated BS RF RX (2500 - 2570 MHz) Filter Characteristics (BS co-location requirements)

Therefore, the simulation results show that it could be feasible for Band 7 BS RF RX filter to provide the required rejections to avoid BS receiver blocking for co-location with the new-SDL band BS, with small degradation in other aspects of the filter performance (e.g. insertion loss and modulation accuracy).

# 3.	Conclusions

In this paper, we have provided an analysis on the BS RF filter requirements for the 2.6 GHz Supplemental DL band for Region 1 based on the coexistence parameters used to define the 3GPP requirements in the RAN4 specifications, and provided simulation results to show the feasibility of the RF filter implementations to meet such requirements.

The simulation results have shown that the 3GPP transmitter spurious emissions requirements for co-existence of -49 dBm/MHz as well as the receiver blocking requirements for co-location (+16 dBm blocking level with 0.8 dB receiver desensitization) can be met, with small degradation in other aspects of the RF filter performance (e.g. insertion loss and modulation accuracy).

On the other hand, the simulation results have shown that the 3GPP transmitter spurious emissions requirements for co-location of -86 dBm/MHz cannot be met for RF TX filter covering the new-SDL band (with or without the Band 7 DL) frequency range with 5 MHz guard band from the Band 7 UL frequency range while maintaining the acceptable RF filter passband insertion loss. The required RF filter rejection is 71 dB for co-location based upon a BS to BS MCL of 30 dB, while the RF filter can achieve ~50 dB over the upper frequency range of Band 7 UL. Therefore, a minimum of 20 dB additional rejection (via external filter) or antenna isolation between the BS transmitter and receiver (via site-engineering solution) is needed for the two BS to be co-located.

# References

[1]	RP-141037, “New Work Item proposal: LTE 2.6 GHz FDD SDL band (2570-2620 MHz) and LTE Carrier Aggregation (2DL/1UL) with Band 3 for Region 1”, Orange.

[2]	3GPP TS 36.104 v13.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[3]	3GPP TS 25.942 v13.0.0, “Radio Frequency (RF) system scenarios”.

[4]	R4-160257, “Analysis and simulation results on BS TX RF filtering for 2.6 GHz Supplemental DL band for Region 1”, Nokia Networks, Alcatel-Lucent.