3GPP TSG-RAN4 Meeting #79	R4-164462

# Nanjing, China, 23rd May 2016 - 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR |  | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for In-band emission for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  | Date: |  |  | 2016-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In order to guarantee co-existence among NB-IoT UEs which share a PRB, in-band emission requirements for NB-IoT is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in-band emission requirements for NB-IoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In-band emission requirements for NB-IoT are still missing and the co-existence may not be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2F.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.521-1 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

6.5.2F.2		Carrier leakage

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power of category NB1 UE shall not exceed the values specified in Table 6.5.2F.2-1.

Table 6.5.2F.2-1: Minimum requirements for relative carrier leakage power

| Parameters | Relative limit (dBc) |
| --- | --- |
| 0 dBm ≤ Output power | -25 |
| -30 dBm ≤ Output power ≤ 0 dBm | -20 |
| -40 dBm Output power < -30 dBm | -10 |

6.5.2F.3	In-band emissions

The in-band emission is defined as a function of the tone offset from the edge of the allocated UL transmission tone(s) within the transmission bandwidth configuration. The in-band emission is measured as the ratio of the UE output power in a non–allocated tone to the UE output power in an allocated tone. The basic in-band emissions measurement interval is defined over one slot in the time domain.

The category NB1 UE relative in-band emission shall not exceed the values specified in Table 6.5.2F.3-1.

Table 6.5.2F.3-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Image frequencies (NOTES 2, 3) |
| Carrier leakage | dBc | -25 | 0 dBm ≤ Output power | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated tone. For each such tone, the minimum requirement is calculated as the higher of Ptone - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. Ptone is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured average power per allocated tone, where the averaging is done across all allocated tones. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated tones. NOTE 4: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured total power in all allocated tones. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the tones containing the DC frequency if is odd, or in the two tones immediately adjacent to the DC frequency if is even, but excluding any allocated tone. NOTE 6: is the Transmission Bandwidth (tones). NOTE 7: is the Transmission Bandwidth Configuration (tones). NOTE 8: is the starting frequency offset between the allocated tone and the measured non-allocated tone. (e.g. or for the first adjacent tone outside of the allocated bandwidth. NOTE 9: is the transmitted power per 3.75 kHz or 15 kHz in allocated tones, measured in dBm. |  |  |  |  |

<Unchanged sections omitted>