3GPP TSG RAN WG4 Meeting #79	 		R4-164678

Nanjing, China, 23rd – 27th May 2016

Title:	Response LS on PUSCH transmission for eLAA

Release:		Rel-14

Work Item:		eLAA_LTE

Source:		TSG RAN WG4

To:		TSG RAN WG1

Cc:

Contact Person:

#### Name:	Hisashi Onozawa

E-mail Address:	hisashi.onozawa@nokia.com

Attachments:		-

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on PUSCH transmission for eLAA.

In RAN4 #79, RAN4 has investigated the eLAA PUSCH transmission scheme using realistic non-linear PA models. Simulations indicate that the ACLR is lower for randomly mapped (unequally spaced) RBs compared to equal spaced RBs for a given UE output power, whereas equally spaced RBs may result in higher spectral peaks in the out-of-band domain. Therefore, the necessary power back-off is smaller for equally spaced RBs with regard to the ACLR requirement. For compliance with spectrum masks, there appear to be no significant difference between the power back-off necessary for the two methods that would motivate the use of unequally spaced RBs. The same conclusion holds for EVM.

In summary, RAN4 does not have a concern on the RAN1 working assumptions.

2. Actions:

RAN4 respectfully asks RAN1 to take the above information into account for the work on PUSCH transmission for eLAA.

3. Date of Next TSG RAN WG1 Meetings:

TSG-RAN WG4 Meeting #80	22 – 26 August 2016	Gothenburg, Sweden

TSG-RAN WG4 Meeting #80bis	10 – 14 October 2016	Ljubljana, Slovenia