3GPP TSG-RAN WG4 Meeting #79	R4-164702

Nanjing, China, 23 – 27 May 2016

Agenda Item:	7.11.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei

Title: 	TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands

Document for:	Approval

# 1.	Introduction

The Multi-Band (MB) Base Station (BS) testing with three or more bands work item was approved at RAN#70 [1]. The necessary changes to the core requirements were discussed at RAN4#78 [2, 3] and RAN4#78bis [4-6] where a text proposal was approved [7]. In this contribution, we discuss how to implement the identified changes (if necessary) in the specifications, and provide a text proposal to include the proposals into the TR [8] of this work item.

# 2.	Discussion

Three clauses for changes were identified in [2-6], namely: Definitions, Symbols, and Operating band unwanted emissions. Below we further discuss the identified changes and provide proposals to implement them (if necessary) in the specifications.

2.1	Operating band unwanted emissions

It was discussed in [2, 6] that the existing specification text in the Operating band unwanted emissions clause (extracted below) may implicitly hint that it is a singular band. We agree that there may be ambiguity, and propose to clarify the phrase as follows:

For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in other supported operating band(s). In this case where there is no carrier transmitted in an operating band, the operating band unwanted emission limit, as defined in the tables of the present subclause for the largest frequency offset (fmax), of a band where there are no carriers transmitted shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted. And no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted.

# 3.	Conclusion

In this contribution, we have discussed how to implement the identified changes (if necessary) in the specifications. It is proposed to include the text proposal below into TR 37.871.

# 4.	Text proposal

<Start of change>

## 5	Necessary changes to the core requirements

5.2	Operating band unwanted emissions

It was discussed in [2, 6] that the existing specification text in the Operating band unwanted emissions clause (extracted below) may implicitly hint that it is a singular band. We agree that there may be ambiguity, and propose to clarify the phrase as follows:

For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in other supported operating band(s). In this case where there is no carrier transmitted in an operating band, the operating band unwanted emission limit, as defined in the tables of the present subclause for the largest frequency offset (fmax), of a band where there are no carriers transmitted shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted. And no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted.

<End of change>

# References

[1]	RP-152205, “New Work Item proposal: Multi-Band Base Station testing with three or more bands”, Alcatel-Lucent.

[2]	R4-160430, “Impact on core requirements”, Ericsson.

[3]	R4-160579, “Discussion on Multi-band definition”, Huawei.

[4]	R4-161599, “TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands”, Nokia, Alcatel-Lucent Shanghai Bell.

[5]	R4-162368, “Correction on multi-band definition”, Huawei.

[6]	R4-162513, “Impact on core requirements”, Ericsson.

[7]	R4-163133, “TP for TR 37.871: Necessary changes to the core requirements for Multi-Band Base Station testing with three or more bands”, Nokia, Alcatel-Lucent Shanghai Bell, Huawei.

[8]	R4-164192, “TR 37.871 V0.2.0”, Nokia.