3GPP TR 37.871 V0.3.0 (2016-05)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Multi-Band Base Station testing with three or more bands

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	6

4.1	Task description	6

4.1.1	Objective of Core part WI	6

4.1.2	Objective of Performance part WI	6

4.2	Band combinations	6

5	Necessary changes to the core requirements	6

6	Necessary changes to the test requirements	7

7	Project plan	7

Annex A: Change history	8

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report of the Multi-Band Base Station testing with three or more bands work item which was approved at TSG RAN #70 [2]. The objective of this work item is to provide specification support for Multi-Band Base Station testing with three or more bands.

The report provides motivation, requirements and a list of recommended changes to the specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-152205, " New Work Item proposal: Multi-Band Base Station testing with three or more bands".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	Background

## 4.1	Task description

### 4.1.1	Objective of Core part WI

Identify the necessary changes, if any, to the existing multi-band BS requirements for BS capable of operation in three or more bands. In addition, identify additional band combinations beside Band (8+20+28) for feasible implementation of multi-band BS capable of operation in three or more bands.

### 4.1.2	Objective of Performance part WI

To specify multi-band BS testing with three or more bands in the RAN4 specifications, using the existing multi-band BS testing with two bands as a base. The work will focus on the following steps:

1.	Conclude whether a generic testing approach can be applied to all the identified band combinations or separate testing approaches are necessary.

2.	In case a generic testing approach cannot be applied to all the identified band combinations with an initial focus on the completion of testing for Band (8+20+28).

2.	Specify multi-band BS testing for all the identified band combinations. In particular, to decide:

1.	The RF bandwidth location and carrier placement within the operating bands and frequency ranges supported by the BS.

2.	Conclude on the multi-band combinations to be tested out of the supported ones for BS that supports multiple multi-band combinations.

3.	Other aspects of multi-band BS testing that are necessary to complete the specification.

## 4.2	Band combinations

The following band combinations are considered feasible for implementation of multi-band BS capable of operation in three or more bands:

1.	Bands (8+20+28) in Europe.

2.	Bands (1+3+7).

3.	Bands (25+4+7).

Only these band combinations will be considered in this work item.

# 5	Necessary changes to the core requirements

5.1	Symbols

Figure 3.2-3 in TS37.104 illustrates a dual-band BS only. As the figure is only used for illustration of the Radio Bandwidth and the figure would become cumbersome as the number of supported bands continue to increase in the future with advance in technology, it is agreed to simply clarify the title of the figure as follows:

Figure 3.2-3: Illustration of Radio Bandwidth related symbols and definitions for Multi-band Multi-standard Radio (Dual-band Base Station)

5.2	Operating band unwanted emissions

It was discussed in [2, 6] that the existing specification text in the Operating band unwanted emissions clause (extracted below) may implicitly hint that it is a singular band. We agree that there may be ambiguity, and propose to clarify the phrase as follows:

For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in other supported operating band(s). In this case where there is no carrier transmitted in an operating band, the operating band unwanted emission limit, as defined in the tables of the present subclause for the largest frequency offset (fmax), of a band where there are no carriers transmitted shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted. And no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted.

# 6	Necessary changes to the test requirements

Considering the identified band combinations, the 3rd order IMD products caused in the BS by transmitting of Band 1, Band 3 and Band 7 downlink (DL) carriers have been studied and it has been shown that the 3rd order IMD products will not fall into the BS receive band of Band 1, 3 or 7.

On the other band, it has been shown that the 3rd order IMD products caused in the BS by transmitting of Band 2, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Since Band 25 is a superseding-band of Band 2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of Band 25, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Hence one may argue that a special testing approach is required for MB BS supporting Bands (25+4+7), e.g. the test configuration would need to ensure that the 3rd order IMD products of the BS transmitter(s) under test will fall into its receiver of Band 7. However, this issue is not specific to this band combination and would also happen for single band (SB) BS operating in certain frequency range, e.g. in Band 2 and Band 3, and currently there is no specific test approach defined in the RAN4 specifications for such SB BS. Therefore, one may also argue that no special testing approach is required for MB BS supporting Bands (25+4+7).

In view of the above discussion, it is agreed that a generic testing approach can be applied to the three identified band combinations, while separate testing approaches may still be considered if found necessary for testing of some requirements.

# 7	Project plan

To facilitate the discussion and simulation work, the work plan is proposed as follows:

- RAN4#78 (February 2016):

  - Approve skeleton TR

  - Approve project work plan

  - Identify band combinations

- RAN4#78bis (April 2016):

  - Identify the necessary changes, if any, to the existing multi-band BS requirements

  - Conclude whether a generic testing approach can be applied to all the identified band combinations

- RAN4#79 (May 2016):

  - Discuss and decide multi-band BS testing for all the identified band combinations

- RAN4#80 (August 2016):

  - Resolve any remaining open issues

  - Approve Core and Performance CRs

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2016-02 | RAN4#78 | R4-160473 |  |  | Skeleton TR | N/A | 0.0.1 |
| 2016-04 | RAN4#78bis | R4-161598 |  |  | Updated TR including the approved text proposals in RAN4#78: R4-160258, R4-161377 | 0.0.1 | 0.1.0 |
| 2016-05 | RAN4#79 | R4-164192 |  |  | Updated TR including the approved text proposals in RAN4#78bis: R4-163133 | 0.1.0 | 0.2.0 |
| 2016-05 | RAN4#79 | R4-164933 |  |  | Updated TR including the approved text proposals in RAN4#79: R4-164702, R4-164194 | 0.2.0 | 0.3.0 |