3GPP TSG-RAN WG4 Meeting #80	   	                                                                 R4-166114

Gothenburg, Sweden, 22 – 26 Aug, 2016

Title:                           MSD for CA_8A-41A 2UL.

Source: 	CMCC

Agenda Item:	8.6.2

Document for:	Approval

- Introduction

The revised WID of supporting the LTE Advanced 2 Band Carrier Aggregation (2DL/2UL) of Band 8 and Band 41 was approved in RAN#70.

In this contribution, MSD requirements are proposed for LTE Advanced 2 Band Carrier Aggregation (2DL/2UL) of Band 8 and Band 41.

2.	Discussion

CA_8A-41A 2UL has two intermodulation products IMD3 and IMD5 falling on B8. Table 2-1 and table 2-2 show the co-existence analysis for IMD3 and IMD5 of 2UL CA_8A-41A [1].

Table 2-1: IMD3

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 – 1830 |  | 4992 – 5380 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 666–930 |  | 4077–4500 |  |

Table 2-2: IMD5

| IMD5 | IMD 5(MHz) |
| --- | --- |
| ¦2*fx - 3*fy¦ | 5658 – 6310 |
| ¦2*fx + 3*fy¦ | 9318 – 9830 |
| ¦2*fy - 3*fx¦ | 2247– 2740 |
| ¦fy – 4*fx¦ | 830 –1164 |

In the last meeting, MSD definition for 2UL Inter-band CA combinations that have multiple IMD’s for one band was discussed and following WF was agreed [2]. According to the WF, IMD3 and IMD5 need to be analyzed, and each MSD ≥[2]dB will be specified.

- “Proposal1: For each UL Inter-band CA only the highest MSD is specified automatically. Each IMD that falls on top of own DL channel and has ≥[2]dB MSD will be specified as well. This applies from Rel14 onwards.

- Proposal 2: A column shall be added in Rel14 UL Inter-band CA MSD tables to tell the source of IMD.

- Proposal 3: An informative note to UL Inter-band CA MSD tables shall be added in Rel14 to indicate if there are other IMD’s than the specified one(s) for each band.

- Proposal 4: Case when two or more IMD’s fall on top of each other is FFS ”

We analyze the MSD using the following assumption:

Table 2-3 Front-end component linearity parameters for MSD calculation

| Component | IP3 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 65 | 53 |  |
| Diplexer | 83 | 53 |  |
| Duplexer | 71 | 53 |  |
| PA forward mixing | 30 | 27 |  |
| PA reverse mixing | 28 | 32 |  |
| LNA | -6 | -10 |  |

Table 2-4 Front-end component isolation parameters for MSD calculation

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Diplexer | 15 | Low/high band isolation |
| PA (out) to PA (out) | 60 | PCB isolation (PA reverse mixing) |
| LNA (in) to PA (out) | 60 | Cross-band (B41 filter + Diplexer) |
| LNA (in) to PA (out) | 70 | PCB isolation (B41 PA leakage into B8 LNA) |
| Duplexer | 50 | Tx band rejection at Rx band |

With the above assumptions, we can conclude IMD3 and IMD5 as follow:

Table 2-5: MSD for IMD3 and IMD5

| CA | band | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 880.7 | 1.4 | 6 | 925.7 | 16 |
|  | B41 |  |  | 2685 | 10 | 50 |  | N/A |
|  | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 13.1 |
|  | B41 |  |  | 2685 | 10 | 50 |  | N/A |
|  | B8 | IMD5 | 4*fB8-fB41 | 900 | 1.4 | 6 | 945 | 14.2 |
|  | B41 |  |  | 2655 | 10 | 50 |  | N/A |
|  | B8 | IMD5 | 4*fB8-fB41- | 900 | 5 | 25 | 945 | 13.5 |
|  | B41 |  |  | 2655 | 10 | 50 |  | N/A |

3.  Conclusion

In this contribution, the MSD for B8+B41 2UL CA was analyzed.

| CA | band | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 880.7 | 1.4 | 6 | 925.7 | 16 |
|  | B41 |  |  | 2685 | 10 | 50 |  | N/A |
|  | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 13.1 |
|  | B41 |  |  | 2685 | 10 | 50 |  | N/A |
|  | B8 | IMD5 | 4*fB8-fB41 | 900 | 1.4 | 6 | 945 | 14.2 |
|  | B41 |  |  | 2655 | 10 | 50 |  | N/A |
|  | B8 | IMD5 | 4*fB8-fB41- | 900 | 5 | 25 | 945 | 13.5 |
|  | B41 |  |  | 2655 | 10 | 50 |  | N/A |

- Reference

[1] R4-160553, “TP for TR 36.714-02-02 on operating bands, channel bandwidths per operating band and co-existence analysis for dual uplink of CA_8A-41A”, CMCC

[2] R4-164839, “MSD definition for 2UL Inter-band CA combinations that have multiple IMD’s for one band ”, Huawei, Hisilicon, NTT DOCOMO Inc,