3GPP TSG RAN4 Meeting #80	R4-166143

Gothenburg, Sweden, 22 – 26 August, 2016

Title: 	MSD for 8+41 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.6.2

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 8+41 UL CA.

# 2	Discussion

This CA combination has two intermodulation products falling on top of own B8 RX channel, IMD3 and IMD5. IMD3 occurs only in case when B41 allocation is at the very high end of B41 and B8 allocation is at the very low end of B8. IMD5 occurs more often. This combination is specific because it has also 1.4MHz and 3MHz BW’s defined for B8.

The following example test points would be suitable for IMD3 and IMD5 evaluation. In case of IMD3 the intermodulation hits own DL CC only partially when the BW is 5MHz. When B8 BW is 1.4MHz the intermodulation is centred around own DL CC. In case of IMD5 the frequencies can be chosen in a way that the intermodulation is centred around own DL CC.

According to recent agreement [1], the intermodulations with MSD≥[2]dB are specified.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 880.7 | 1.4 | 6 | 925.7 | 1.4 |
|  | B41 |  |  | 2685 | 10 | 25 |  |  |
|  | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 5 |
|  | B41 |  |  | 2685 | 10 | 25 |  |  |
|  | B8 | IMD5 | 4*fB8-fB42 | 900 | 1.4 | 25 | 945 | 1.4 |
|  | B41 |  |  | 2655 | 10 | 25 |  |  |
|  | B8 | IMD5 | 4*fB8-fB42 - | 900 | 5 | 25 | 945 | 5 |
|  | B41 |  |  | 2655 | 10 | 25 |  |  |

Table 1 Example test points

We use the following RF front-end architecture in our analysis. We note that harmonic trap is not used even there is 3rd harmonic relation between B8 UL and B41because the trap was not used in respective 1UL/2DL specification.

Figure 1 Common diplexer -architecture

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP3 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 68 | 53 |  |
| Diplexer | 86 | 53 |  |
| Duplexer | 74 | 53 |  |
| PA forward mixing | 31.5 | 30 |  |
| PA reverse mixing | 33 | 33 |  |
| LNA | -6 | -10 |  |

Table 2 General linearity parameters

B8 duplexer TX rejection at B41	35

B8 duplexer RX rejection at B41	40

B41 filter attenuation at B8 8X		36

B41 filter attenuation at B8 TX		40

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation PAout-RX				65

RFIC Pout B8/B41					0

B8 duplexer TX-RX isolation		50

Diplexer isolation						15

Based on these parameters we get the following MSD’s. Please note that the CF’s are the same as in [2]. B8 PA forward mixing is dominant factor for both IMD3 and IMD5. PA forward mixing results are based on measurements on commercial PA’s.

IMD3 1.4MHz dominant factors:

IMD5 5MHz dominant factors:

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 880.7 | 1.4 | 6 | 925.7 | 1.4 | 8.14 | 14.0 |
|  | B41 |  |  | 2685 | 10 | 50 |  |  | N/A |  |
|  | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 5 | 6 | 11.1 |
|  | B41 |  |  | 2685 | 10 | 50 |  |  | N/A |  |
|  | B8 | IMD5 | 4*fB8-fB42 | 900 | 1.4 | 25 | 945 | 1.4 | 7.8 | 5.1 |
|  | B41 |  |  | 2655 | 10 | 50 |  |  | N/A |  |
|  | B8 | IMD5 | 4*fB8-fB42 - | 900 | 5 | 25 | 945 | 5 | 3.34 | 4.7 |
|  | B41 |  |  | 2655 | 10 | 50 |  |  | N/A |  |

MSD for both IMD3 and IMD5 will be specified because MSD’s are larger than [2]dB.  At least based on this contribution, 1.4MHz BW has larger MSD than 5MHz BW. Thus, based on this contribution MSD for 1.4MHz BW for IMD3 and IMD5 should be specified.

# 3	Conclusion

Example test points for B8+B41 UL CA MSD’s were provided and different alternatives discussed.

| CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL | 3rd DL Fc (MHz) | DL BW (MHz) | CF | MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | RB # |  |  |  |  |
| B8+B41 | B8 | IMD3 | fB41 – 2*fB8 | 880.7 | 1.4 | 6 | 925.7 | 1.4 | 8.14 | 14.0 |
|  | B41 |  |  | 2685 | 10 | 50 |  |  | N/A |  |
|  | B8 | IMD3 | fB41 – 2*fB8 | 882.5 | 5 | 25 | 927.5 | 5 | 6 | 11.1 |
|  | B41 |  |  | 2685 | 10 | 50 |  |  | N/A |  |
|  | B8 | IMD5 | 4*fB8-fB42 | 900 | 1.4 | 25 | 945 | 1.4 | 7.8 | 5.1 |
|  | B41 |  |  | 2655 | 10 | 50 |  |  | N/A |  |
|  | B8 | IMD5 | 4*fB8-fB42 - | 900 | 5 | 25 | 945 | 5 | 3.34 | 4.7 |
|  | B41 |  |  | 2655 | 10 | 50 |  |  | N/A |  |

MSD for both IMD3 and IMD5 will be specified because MSD’s are larger than [2]dB.  At least based on this contribution, 1.4MHz BW has larger MSD than 5MHz BW. Thus, based on this contribution MSD for 1.4MHz BW for IMD3 and IMD5 should be specified.

# 4	References

[1] R4-164839, “WF: MSD definition for 2UL Inter-band CA combinations that have multiple IMD’s for one band”, Huawei, Hisilicon, NTT DOCOMO INC, RAN4#79

[2] R4-161889, “XDL 2UL IMDs, testpoints and CFs”, Qualcomm Inc., RAN4#78bis