3GPP TSG RAN4 Meeting #81	R4-1609974

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 3+5+7 2UL/3DL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 3+5+7 2UL/3DL CA.

# 2	Discussion

This CA combination has IMD2 and IMD3 falling on top of own B7 RX and IMD3 falling on top of own B5 RX. The test point for B7 IMD2 and B5 IMD3 were approved in [1] and the test point for B7 IMD3 was suggested in [2]:

| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 1780 | 10 | 50 | 2625 | 10 | TBD | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B5 |  |  | 845 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -*fB7 | 1715 | 10 | 50 | 880 | 5 | TBD | TBD |
|  | B7 |  |  | 2550 | 10 | 50 |  |  |  |  |

| B3+B5+B7 | B5 | IMD3 | 2*fB3 -fB5 | 835 | 880 | 5 | 25 | B7 | 2655.0 | 2655.0 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B3 |  |  | 1745 | 1840 | 10 | 50 |  |  |  |  |

We use both conventional common diplexer and conventional common triplexer based architectures in our analysis with the following component assumptions. Diplexer/Triplexer is not assumed in diversity. Please note that using triplexer allows implementing B3 with for instance B1+B3 quadplexer in the middle branch.

| Component | IP2 (dBm) | IP3 (dBm) |
| --- | --- | --- |
| Antenna switch | 112 | 68 |  |
| Diplexer/Triplexer | 115 | 86 |  |
| Duplexer/Quadplexer | 100 | 74 |  |
| LNA | 5 | -6 |  |

B7 MSD:

B3+B7 quadplexer cross-band TX/RX iso								  50

B5 duplexer TX attenuation at B7 RX						                    34

B3 TX attenuation at B7 RX												  35

B1+B3 quadplexer attenuation at B7 RX								  28

B3+B7 quadplexer B7 RX attenuation at B5 TX					        45

B3 PA forward mixing IMD2 at PA output with 60dB isolation	       -22

B3 PA forward mixing RFIC Pin2Pin IMD2 at PA output				 -26.5

B3 PA forward mixing IMD3 at PA output with 60dB isolation	       -50

B3 PA forward mixing RFIC Pin2Pin IMD3 at PA output				 -54.5

B7 FE loss (switch+trace+filter)											    5.5

B5 MSD:

B3+B7 quadplexer B3 TX attenuation at B5 RX						   32

B3+B7 quadplexer B7 TX attenuation at B5 RX					         30

B5 duplexer RX attenuation at B3 TX									   40

B5 duplexer RX attenuation at B3 TX									   35

B5 FE loss (switch+trace+filter)											    4.5

General

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s.

B7 IMD2 dominant factors, diplexer:

B7 IMD2 dominant factors, triplexer:

B5 IMD3 dominant factors, diplexer:

B5 IMD3 dominant factors, triplexer:

B7 IMD3 dominant factors, diplexer:

B7 IMD3 dominant factors, triplexer:

Finally the MSD numbers for diplexer based architecture:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 1780 | 10 | 50 | 2625 | 10 | 1 | 33.6 |
|  | B5 |  |  | 845 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB5 | 835 | 5 | 25 | 2665 | 5 | 1 | 17.8 |
|  | B5 |  |  | 1745 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -*fB7 | 1715 | 10 | 50 | 880 | 5 | 3 | 21.7 |
|  | B7 |  |  | 2550 | 10 | 50 |  |  |  |  |

..and for the triplexer based architecture:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 1780 | 10 | 50 | 2625 | 10 | 1 | 31.9 |
|  | B5 |  |  | 845 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB5 | 835 | 5 | 25 | 2665 | 5 | 1 | 17.4 |
|  | B5 |  |  | 1745 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -*fB7 | 1715 | 10 | 50 | 880 | 5 | 3 | 20.3 |
|  | B7 |  |  | 2550 | 10 | 50 |  |  |  |  |

We propose to use the following MSD numbers based on this analysis. In addition, we propose to define both IMD2 and IMD3 for B7 as has been agreed in WF some meetings ago.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 1780 | 10 | 50 | 2625 | 10 | 1 | 33.6 |
|  | B5 |  |  | 845 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB5 | 835 | 5 | 25 | 2665 | 5 | 1 | 17.8 |
|  | B5 |  |  | 1745 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -*fB7 | 1715 | 10 | 50 | 880 | 5 | 3 | 21.7 |
|  | B7 |  |  | 2550 | 10 | 50 |  |  |  |  |

# 3	Conclusion

MSD analysis for CA_5A-7A-46A was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 1780 | 10 | 50 | 2625 | 10 | 1 | 33.6 |
|  | B5 |  |  | 845 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB5 | 835 | 5 | 25 | 2665 | 5 | 1 | 17.8 |
|  | B5 |  |  | 1745 | 10 | 50 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -*fB7 | 1715 | 10 | 50 | 880 | 5 | 3 | 21.7 |
|  | B7 |  |  | 2550 | 10 | 50 |  |  |  |  |

# 4	References

[1] R4-168857, “MSD Test configurations for xDL/2UL CA band combinations with self interference issues”, LG, RAN4#80bis

[2] R4-168105, “2UL 3DL Possible testpoints”, Qualcomm Inc, RAN4#80bis