3GPP TSG-RAN WG4 #81	R4- 1610162

November 14th ‒ 18th, 2016

Reno, Nevada, USA

Agenda item:	11.3.1

Source: 	Qualcomm Incorporated

Title: 	Simulation results for NR coexistence study: dense urban deployment at 30GHz

Document for:	Discussion

# Introduction

In this contribution we present simulation results for the NR adjacent channel coexistence study. The paper addresses dense urban deployment at 30GHz. Both Up Link (UL) and Down Link (DL) cases are analysed. The simulation assumptions adopted in this study are the ones agreed in RAN4 #80bis [1][2][3].

# Discussion

The simulation results presented in this paper refer to the dense urban layout with a carrier frequency of 30GHz.

The dense urban layout is made of fixed cluster circles within a macro cell, in particular 3 clusters within macro cell and 1 one micro BS per cluster are considered. Micro BSs are randomly dropped on an edge of the cluster circle. It is also worth mentioning that all UEs communicate with micro BSs, i.e. macro cells are only used for determining position of micro BSs.

Regarding the macro cell layout, the classical hexagonal grid with 19 macro sites and 3 sectors is assumed. Inter site distance (ISD) for macro BSs is 200m. A pictorial representation of the dense urban layout is shown in Figure 1.

Figure 1. Dense urban deployment: macro and micro base stations placement [1].

As already observed in [4], the relative impact of the two agreed noise figures (namely 9dB and 11dB for 30GHz) on the final required ACIR results is negligible. Therefore in the following, for the sake of reducing simulation time and helping the presentation of the results, we will only consider NF=11dB.

The list of the remaining simulation assumptions for the dense urban scenario is reported in Table 1.

Table 1. List of simulation assumptions for dense urban scenario [1].

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 28.9 m |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi | Specified in TR38.900 |
| Shadowing correlation |  | Between cite: 0.5 | Table 6.3.2.1 in TR37.809 |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with ISD = 200m is assumed. |  |  |  |

## DL results

In this section we present DL results, i.e. the throughput is measured at the victim UEs and the aggressor nodes are micro BSs operating in adjacent channel. Figure 2 shows the throughput degradation as a function of ACIR. Mean and 5%-tile throughput are considered. The results indicated that an ACIR in the 8-12dB range would allow to keep the degradation due to adjacent channel interference (ACI) within 5%.

Figure 2. DL throughput degradation as a function of ACIR.

## UL results

In this section we present UL results, i.e. the throughput is measured at the victim micro BSs and the aggressor nodes are UEs operating in adjacent channel.

Figure 3. UL throughput degradation as a function of ACIR.

Figure 3 shows the UL throughput degradation as a function of ACIR. As it can be observed the impact on mean throughput is very negligible. The 5%-tile throughput is the more affected, however an ACIR around 12dB would meet the target degradation criteria.

# Conclusions

In this contribution we provided simulation results for NR coexistence study in dense urban scenario. A carrier frequency of 30GHz is considered, both DL and UL cases were analysed.

Based on the results obtained so far, for DL case an ACIR within the 8-12dB range would provide enough margin against adjacent channel interference. For UL case, an ACIR in the 5-12dB range ballpark would be enough to keep degradation due to ACI within 5%.

# Reference

- R4-168953, “Simulation assumptions for WP5D co-existence simulation”, NTT DOCOMO, INC.

- R4-168794, “Further Way forward on urban micro layout and BS Beamforming Model”, Huawei Technologies Co., Ltd.

- R4-168795, “WF on simulation parameters for Urban macro and TPC model”, NTT DOCOMO, INC., Qualcomm Incorporated.

- R4-1610161, “Simulation results for NR coexistence study: macro deployment at 30GHz”, Qualcomm Incorporated.