3GPP TSG-RAN WG4 Meeting #82        							        R4-1701315

Athens, Greece, 13 – 17 February 2017

Source: 	Huawei, Hisilicon

Title: 	Phase noise model for above 6 GHz

Agenda Item:	10.1.1.1

Document for:	Approval

Introduction

At carrier frequency above 6 GHz, phase noise becomes significant and could result in performance degradations if not considered properly.  In Spokane NR AH meeting, it was approved to study phase noise model in RAN4 [1].

In this contribution, we discuss the RF material properties and outline a phase noise model to be considered and utilized for the design and evaluation of the new radio in the bands above 6 GHz.

Discussion

Fabrication Methods and Materials

While there are many different fabrication methods, the most common fabrication materials are CMOS, GaAs, SiGe and GaN. With a review of the state of the art [8]-[25], a summary on the phase noise level achieved by different fabrication methods and materials is given in Figure 2.  From this it can be seen that:

Observation 1: For 30GHz band, the typical phase noise level measured at 1 MHz offset is from -114 to -93 dBc/Hz, while that for 70 GHz band is from -108 to -81 dBc/Hz.

While GaAs-based devices can provide a lower phase noise level, it is still expensive and power-consuming. The CMOS-based devices are available at lower cost and have less power consumption. Taking the cost and power constraint at the UE side into consideration, it appears reasonable to assume CMOS-based design for the UE side. On the other hand, for the BS side, GaAs-based design may be assumed.

Figure 2. A brief summary of the phase noise level achieved by different fabrication methods and materials

Proposed Model and Parameters

In this contribution, we utilize the PLL-based phase noise model to express the phase noise. To be specific, the PSD of the phase noise is characterized by:

STotalf=SReff+SPLLf,   when f≤loop BWSVCO_ v2f+SVCO_v3(f),    when f>loop BW

where

SRef / PLL / VCO_v2 / VCO_v3= PSD0∙1+ffzk1+fk (dB)

PSD0=FOM+20logfc-10logP1 mW (dB)

FOM is the figure of merit, fc is the carrier frequency and P is the consumed power. After reviewing the recent literature [27]-[35], based our expectation for the phase noise level achievable with reasonable cost and power consumption, we propose to consider the following parameters for the phase noise model at the UE (CMOS-based) and BS (GaAs-based) side, respectively (see Table 1). The PSD of the proposed phase noise models at both UE and BS side for 30 GHz are depicted in Figure 4.

Table 1. Parameters for proposed phase noise models

|  | Model 1, UE, Loop BW = 187kHz |  |  |  | Model 2, BS, Loop BW = 112kHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | REF clk | PLL | VCO V2 | VCO V3 | REF clk | PLL | VCO V2 | VCO V3 |
| FOM | -215 | -240 | -175 | -130 | -240 | -245 | -187 | -130 |
| fz | Inf | 1.00E+04 | 50.30E+06 | Inf | Inf | 1.00E+04 | 8.00E+06 | Inf |
| P (mW) | 10 | 20 | 20 |  | 10 | 20 | 50 |  |
| k | 2 | 1 | 2 | 3 | 2 | 1 | 2 | 3 |

Figure 4. PSD of proposed phase noise model at both UE and BS side (fc=30 GHz)

Conclusion

Based on the discussions in this paper, for the phase noise model above 6 GHz, we have the following proposal:

Proposal 1: PLL-based model is adopted the design and evaluation of the new radio for above 6GHz.

Proposal 2: The phase noise models described in Table 1 are adopted for the design and evaluation of the new radio for above 6GHz.

.

- References

R4-1700282, “Way forward on subcarrier spacing for NR”, Huawei, Ericsson, Intel, Nokia, ZTE

3GPP, R1-163113, NTT DOCOMO, “Initial views on numerology for new radio access technology”, Apr. 2016.

3GPP, R1-162386, Intel, “Numerology for new radio interface”, Apr. 2016.

3GPP, R1-162885, Nokia, Alcatel-Lucent Shanghai Bell, “On the phase noise model for 5G New Radio evaluation”, Apr. 2016.

3GPP, R1-163227, Ericsson, “Numerology for NR”, Apr. 2016.

K. Zeng et. al., “Considerations on phase noise model for 802.11ay”, IEEE Document 802.11-16-0390r0, Mar. 2016.

Rogers Corporation, “RO4000® series high frequency circuit materials”.

N. Pohl et. al., “A low-power 80 GHz FMCW radar transmitter with integrated 23 GHz downconverter VCO”, IEEE Bipolar/BiCMOS Circuits Tech. Meeting, Oct. 2011.

J. Lee et. al., “A dual-band VCO using inductor splitting for automotive radar system at W-band”, Int. Symp. Integrated Circuits, Dec. 2014.

C. Ameziane et. al., “79GHz push-push oscillators in 0.13μm SiGe BiCMOS technology”, IEEE Int. Conf. Elec., Circuits Systems, Aug. 2008.

C. Ameziane et. al., “An 80GHz range synchronized push-push oscillator for automotive radar application”, IEEE Radio Freq. Integrated Circuits Symp., May 2010.

Z. Liu et. al., “A 70GHz VCO with 8GHz tuning range in 0.13um CMOS technology”, European Microwave Integrated Circuit Conf., Oct. 2008.

D. D. Kim et. al., “A 70GHz manufacturable complementary LC-VCO with 6.14GHz tuning range in 65nm SOI CMOS”, IEEE Int. Solid-State Circuits Conf., Feb. 2007.

W. Fei et. al., “Design and analysis of wide frequency-tuning-range CMOS 60 GHz VCO by switching inductor loaded transformer”, IEEE Trans. Circuits Systems I, Oct. 2013.

A. Chakraborty et. al., “A low-phase-noise monolithically integrated 60 GHz push push VCO for 122 GHz applications in a SiGe bipolar technology”, IEEEE Bipolar/BiCMOS Circuits Tech. Meeting, Sep. 2013.

T. Y. Lu et. al., “Wide tunning range 60 GHz VCO and 40 GHz DCO using single variable inductor”, IEEE Trans. Circuits Systems I, Oct. 2012.

T. Nakamura et. al., “A push-push VCO with 13.9-GHz wide tuning range using loop-ground transmission line for full-band 60-GHz transceiver”, IEEE J. Solid-State, Apr. 2012.

Y. C. Chiang et. al., “60 GHz CMOS VCO using a fourth-order resonator”, IEEE Microwave Wireless Components Lett., Jul. 2015.

S. P. Sah et. al., “An 8th sub-harmonic injection locked V-band VCO for low power LO routing in mm-Wave beamformers”, IEEE Radio Freq. Integrated Circuits Symp., Jun. 2014.

X. Lan et. al., “A V-Band monolithic AlGaN/GaN VCO”, IEEE Microwave Wireless Components Lett., Jun. 2008

Y. H. Chang et. al., “A V-Band push-push VCO with wide tuning range using 0.18UM CMOS process”, IEEE Microwave Wireless Components Lett., Dec. 2014.

R. Shu et. al., “A 40 GHz CMOS VCO with resonated negative-conductance cell”, IEEE Int. Symp. Radio-Frequency Integration Tech., Nov. 2012.

J. C. Chien et. al., “A 40-GHz wide-tuning-range VCO in 0.18-μm CMOS”, Symp. VLSI Circuits, 2006.

H. D. Lee et. al., “A low-power 50-GHz LC-VCO in a 65-nm CMOS technology”, Asia-Pacific Microwave Conf., Dec. 2015.

T. Xi et. al., “Low-phase-noise 54GHz quadrature VCO and 76GHz/90GHz VCOs in 65nm CMOS process”, IEEE Radio Freq. Integrated Circuits Symp., Jun. 2014.

J. Breitbarth et. al., “Additive (residual) phase noise measurement of amplifiers, frequency dividers and frequency multipliers”, Microwave J., 2008.

W. Fei et. al., “A 96-GHz oscillator by high-Q differential transmission line loaded with complementary split-ring resonator in 65-nm CMOS”, IEEE Trans. Circuits Systems II, Mar. 2013.

F. M. De Paola et. al., “A 71–73 GHz voltage-controlled standing-wave oscillator in 90 nm CMOS technology”, European Solid-State Circuits Conf., Sep. 2008.

H. Li et. al., “Wide-band VCOs in SiGe production technology operating up to about 70GHz”, IEEE Microwave Wireless Comp. Lett., Oct. 2003.

R. Weber et. al., “A novel tuning concept for wideband VCOs based on a shunt-FET”, Microwave Integrated Circuits Conf. Sep. 2009.

T. Nakamura et. al., “A 59GHz push-push VCO with 13.9GHz tuning range using loop-ground transmission line for a full-band 60GHz transceiver”, Int. Solid-State Circuits Conf., Feb. 2009.

H. L. Kao et. al., “Low phase noise V-band push –push voltage controlled oscillator using 0.15 um GaAs pseudomorphic high electron-mobility transistor technology”, IET Microwaves, Antennas & Propagation, Apr. 2012.

H. L. Kao et. al., “A low phase noise Ka-band voltage controlled oscillator using 0.15 μm GaAs pHEMT technology”, Int. Symp. Design Diagnostics of Elec. Circuits & Systems, Apr. 2012.

D. D. Kim et. al, “A 70GHz manufacturable complementary LC-VCO with 6.14GHz tuning range in 65nm SOI CMOS”, IEEE Int. Solid-State Circuits Conf., Feb. 2007.

L. Li et. al., “Design and analysis of a 90 nm mm-wave oscillator using inductive-division LC tank”, IEEE J. Solid-State Circuits, Jul. 2009.