3GPP TSG-RAN WG4 Meeting #84 	R4-1707409

Berlin, Germany, 21 – 25 August, 2017

Agenda item:	10.1.1.1

Source: 	Intel Corporation

Title: 	Initial PDSCH simulation results for 8Rx with rank lower than or equal to 4

Document for:	Discussion

- Introduction

The Release 15 Study Item “Study on LTE DL 8Rx antenna ports” has been introduced and the Study Item Description is given in [1]. It is expected that the demodulation performance of 8Rx antenna configuration would be a lot different compared with a smaller number of receive antennas, such as, 4Rx antenna configuration. In this contribution, we provide our initial PDSCH simulation results for 8Rx with rank lower than or equal to 4.

- Simulation assumptions and results

We first consider an existing 4Rx testcase for TM9 Single-layer Spatial Multiplexing, which is an MU-MIMO testcase and specified in TS 36.101 [1] Section 8.10.1.1.5A, and is also summarized and listed below:

Table 8.10.1.1.5A-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with multiple CSI-RS configurations

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Beamforming model |  |  | Annex B.4.1 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 3 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0001000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 4) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | Yes (Note 3, 5) |
| PDSCH transmission mode |  |  | 9 |
| Number of MBSFN subframes |  | Subframes | NA |
| Note 1: . Note 2: The modulation symbols of the signal under test are mapped onto antenna port 7 or 8. Note 3: Modulation symbols of an interference signal is mapped onto the antenna port (7 or 8) not used for the input signal under test. Note 4: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 5: The two UEs’ scrambling identities are set to 0 for CDM-multiplexed DM RS with interfering simultaneous transmission test cases. |  |  |  |

Table 8.10.1.1.5A-2: Minimum performance for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 64QAM 1/2 | R.50 FDD | OP.1 FDD | EPA5 | 2x4 Low | 70 | 15.8 | ≥2 |
| Note 1: The reference channel applies to both the input signal under test and the interfering signal. |  |  |  |  |  |  |  |  |

We then try to re-use the above 4Rx testcase for 8Rx configuration as a measure of comparison between 4Rx and 8Rx antenna configurations. Note that, for 8Rx, besides the “ULA low correlation” matrix, we also consider the “ULA medium correlation A” and “ULA medium correlation” matrices. It is because that doubling the number of receive antenna would inevitably increase the MIMO channel correlation. Although the model of the accurate MIMO channel correlation for 8Rx is still for further study, Fig.1 below demonstrates the preliminary simulation results by using “ULA medium correlation A” and “ULA medium correlation” as the assumed MIMO channel correlation.

Fig.1 Throughput performance for TM9, single-layer spatial multiplexing, EPA5

Fig.2 Normalized throughput performance for TM9, single-layer spatial multiplexing, EPA5

Observation 1: For the low correlation case, 2×8 achieves about 1.5dB gain against 2×4 antenna configuration.

Observation 2: If higher correlation levels, such as “ULA medium correlation A” and “ULA medium correlation”, are considered for 8Rx, significant performance degradation is observed, that are about 5dB and 12dB loss compared with 4Rx low correlation level.

Proposal: The 8Rx antenna deployment can be rather challenging and different from 2Rx/4Rx scenarios due to the space/area limitation on mobile devices. One outstanding problem is the receive antenna correlation issue, which has already shown significant degradation brought to the throughput performance. Therefore, it is very important to have a comprehensive and practical study on the receive antenna correlation impact of 8Rx antenna configuration, compared with 4Rx scenarios.

- Conclusion

In this contribution, we provided our initial PDSCH simulation results for 8Rx.

Observation 1: For the low correlation case, 2×8 achieves about 1.5dB gain against 2×4 antenna configuration.

Observation 2: If higher correlation levels, such as “ULA medium correlation A” and “ULA medium correlation”, are considered for 8Rx, significant performance degradation is observed, that are about 5dB and 12dB loss compared with 4Rx low correlation level.

Proposal: The 8Rx antenna deployment can be rather challenging and different from 2Rx/4Rx scenarios due to the space/area limitation on mobile devices. One outstanding problem is the receive antenna correlation issue, which has already shown significant degradation brought to the throughput performance. Therefore, it is very important to have a comprehensive and practical study on the receive antenna correlation impact of 8Rx antenna configuration, compared with 4Rx scenarios.

- Reference

[1] TS 36.101 User Equipment (UE) radio transmission and reception v13.6.1 (2017-01).