3GPP TSG-WG RAN4 Meeting #84bis	R4-1711217

# Dubrovnik, Croatia, 8th – 12th Oct, 2017

Source: 	Nokia, Nokia Shanghai Bell, Verizon

Title: 					TP to TR 36.715-05-01: CA_5DL_13A-48C-48C_1UL_BCS0

Agenda Item: 			8.5.2

Document for:	Approval

# Introduction

This TP introduces 5DL/1UL CA configuration CA_13A-48C-48C in to the TR 36.715-05-01

**************************** Start of changes ********************************************

## 5.X	CA_13A-48C-48C_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_13A-48C-48C | 13 |  |  | Yes | Yes |  |  | 90 | 0 |
|  | 48 | See CA_48C-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 of TS36.101 |  |  |  |  |  |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 13 and Band 48 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 13 | 777 | 787 | 1554 | 1574 | 2331 | 2361 | 3108 | 3148 |
| 48 | 3550 | 3700 | 10300 | 11850 | 15450 | 17775 | - | - |

It can be seen from Table 5.X.2-1 that there is no harmonic issue for CA_13A-48C-48C.

### 5.X.3	∆TIB,c and ∆RIB,c values

For five simultaneous DLs and one UL of Band 13, and Band 48 for 1UL/5DL CA_13A-48C-48C, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively.

Table 5.X.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_13A-48C-48C | 13 | 0.3 |
|  | 48 | 0.3 |

Table 5.X.3-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_13A-48C-48C | 13 | 0 |
|  | 48 | 0 |

**************************** End of changes ********************************************