3GPP TSG-RAN WG4 RAN#85	R4-1713208

Reno, USA, November 27th-December 1st, 2017

Agenda item:	  9.7.10.1

Source: 	Qualcomm Incorporated

Title: 	PSS/SSS detection in NR: updated link level simulation results

Document for:	Discussion

# 1. 	Introduction

In this contribution, we present updated simulation results of PSS/SSS detection in NR, for both sub-6 and mmWave bands, according to the agreed simulation assumptions in [1].

# 2. 	Assumptions

PSS/SSS detection simulations have been done according to the assumption in [1] and with these particular cases:

- Synchronous deployment (relative delay of CP/2)

- 1 TX antenna with cross-polarization and 2 RX antennas with ideal receiver beamforming (Rx beam is pointing towards the Rx signal and no beam sweep is simulated)

- Prior knowledge of serving cell – cell ID = 337 (NID1, NID2 = {112,1});

- Single transmitted SS block per SS burst set (K=1)

- UE velocity of 3km/h for CDL channels

The selected parameters for subcarrier spacing, propagation conditions, Es/Noc levels and cell IDs are listed directly in Table 1 and Table 2 with the results, whereby the 90 percentiles of the detection time are reported. The SINR value is measured at the baseband (detector) and no RF impairments have been simulated.

# 3.	Results for 4 GHz

It can be observed that for both subcarrier spacing one burst suffice for most cases apart higher mobility cases where two attempts are necessary.

Table 1. PSS/SSS detection time for Cell identification at 4 GHz, in number of SS burst set periods

| Cell ID | SINR [dB] | AWGN |  | EPA5 |  | ETU30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz |
|  |  | 90% | 90% | 90% | 90% | 90% | 90% |
| 673 (colliding NID2) | -6 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | -7 | 1 | 1 | 2 | 1 | 2 | 2 |
|  | -8 | 1 | 1 | 3 | 3 | 2 | 2 |
| 674 | -6 | 1 | 1 | 1 | 1 | 2 | 2 |
|  | -7 | 1 | 1 | 1 | 1 | 2 | 2 |
|  | -8 | 1 | 1 | 1 | 1 | 2 | 2 |

| Cell ID | SINR [dB] | TDL-A |  | TDL-B |  | TDL-C |  | TDL-D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz |
|  |  | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% |
| 673 (colliding NID2) | -6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | -7 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | -8 | 2 | 2 | 2 | 3 | 2 | 2 | 3 | 3 |
| 674 | -6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | -7 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | -8 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 1 |

# 4.	Results for 30 GHz

It can be observed that for both subcarrier spacing one burst suffice for all AWGN cases. For the more challenging propagation conditions of the CDL and TDL models two burst are required in most of the cases instead.

Table 2. PSS/SSS detection time for Cell identification at 30 GHz, in number of SS burst set periods

| Cell ID | SINR [dB] | AWGN |  | CDL-A |  | CDL-B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz |
|  |  | 90% | 90% | 90% | 90% | 90% | 90% |
| 673 (colliding NID2) | -6 | 1 | 1 | 2 | 2 | 1 | 1 |
|  | -7 | 1 | 1 | 2 | 2 | 2 | 1 |
|  | -8 | 1 | 1 | 2 | 2 | 2 | 2 |
| 674 | -6 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | -7 | 1 | 1 | 2 | 2 | 1 | 1 |
|  | -8 | 1 | 1 | 2 | 2 | 2 | 2 |

| Cell ID | SINR [dB] | TDL-A |  | TDL-B |  | TDL-C |  | TDL-D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz |
|  |  | 90% | 90% | 90% | 90% | 90% | 90% | 90% | 90% |
| 673 (colliding NID2) | -6 | 1 | 2 | 2 | 2 | 2 | 1 | 1 | 2 |
|  | -7 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | -8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 674 | -6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | -7 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | -8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

# 5. 	Conclusions

In this paper, we provided updated results for PSS/SS detection in NR bands, for a number of parameter combinations according to the simulation assumptions agreed in [3].

Observation: For both bands, one burst is sufficient in AWGN channel conditions and two bursts are required in most of the cases for more challenging propagation conditions, apart non-colliding NID2 at higher SINR.

The results should be taken into account when RAN4 discusses the requirements.

# 6. 	References

- R4-1708698, “Updated link level simulation assumption of cell detection in NR”, Huawei, HiSilicon, NTT DOCOMO, INC.