3GPP TSG-RAN WG4 Meeting #86	R4-1802126

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia

Title: 					TP to TR 36.761: UE receiver characteristics for Extended-Band 12

Agenda Item: 			6.12.2

Document for:	Approval

# 1	Introduction

This contribution is a TP to introduce the UE receiver requirements for extended band 12 into the TR 36.761.

**************************** Start of the TP *****************************************

## 81	UE aspect issues

### 8.1.2	UE receiver characteristics

In this clause extended band 12 specific receiver requirements are addressed, all other recevier requirements are band agnostic.

#### 8.1.2.1		Reference sensitivity power level

Due to almost identical band arrangement with band 12 extended band reference sensitivity requirement is defined to be same as band 12.

Table 8.1.2.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| ext-12 |  |  | -97 | -94 |  |  | FDD |

Table 8.1.2.1-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD |
| 85 |  |  | 201 | 201 |  |  | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |

Table 8.1.2.1-3: Network signalling value for reference sensitivity

| E-UTRA Band | Network Signalling value |
| --- | --- |
| 12 | NS_06 |
| 85 | NS_06 |

Similarly for UE categories M1, M2, NB1 and NB2 the extended band 12 requirements for reference sensitivity are same as for band 12 for these categories.

#### 8.1.2.2	In-band blocking and Out-of-band blocking

Same in-band blocking and out-of-band blocking requirements apply for extended band 12 as for band 12.

**************************** End of the TP *****************************************