3GPPRAN4#86	R4-1803040

Athens, Greece, Feb 22th-Mar 2nd

Title: 	Further discussion on NR Handover Timeline

Source: 	Qualcomm Incorporated

Agenda item:	7.9.7.1 - Handover (Intra-NR handover) [NR_newRAT-Core]

Document for:	Discussion

# Introduction

In RAN4#AH-1801 we agreed on WF [1] for HO timelines in NR. It was agreed that the handover delay, Dhandover is defined as

Dhandover = TRRC_procedure_delay + Tinterrupt

Where Tinterrupt is defined as

Tinterrupt = Tsearch + TIU + Tmargin + TMIB ms

Where:

Tsearch: is the time for PSS/SSS detection, which is 0 for known cell. And [TBD] ms for unknown cell provided that the signal quality is sufficient for successful cell detection on the first attempt

TIU: is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to [TBD] ms.

Tmargin: it comprises at least Tprocessing_NR. Whether Tmargin also includes Tloops is FFS where:

  - Tprocessing_NR: is the UE processing time, which can be up to [TBD]ms

  - Tloops: is time for time refinement, which is up to [TBD] ms

TMIB: is the time required to decode MIB, if MIB decode is necessary, otherwise TMIB = 0.

In our previous paper [2], we had shown the procedure for HO. In this paper, we just provide our view on what the above-mentioned numbers should be.

# Discussion

## Interruption Time

The interruption time in NR handover will depend on the SSB periodicity in target cell. The UE needs SSB to run search, frequency/time correction loops, SFN acquisition etc. In FR1, the UE would need up to 1 SMTC to run search an unknown cell, provided that the cell is detectable in first instance. However, in FR2, the UE will need to cycle through its Rx beams, so the search time would depend on the number of beams that UE is using.

Proposal 1: The value of Tsearch during handover will be

- 0ms if the cell is known

- Up to 1 SMTC duration for a cell in FR1, provided the cell is detectable in first attempt

- Up to n SMTC durations for a cell in FR2, provided the cell is detectable in first attempt, where n is the number of UE Rx beams.

On further analysis, the UE should be able to achieve timing requirements for RACH during search without the need for additional loop refinement. Also, in case MIB decode is required it can be accomplished using the same SSB as that for search. To keep consistency between E-UTRA and NR spec we propose the rename Tmargin back to Tprocessing.

Proposal 2: The value of Tprocessing can be up to 20ms.

As before, TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

# Conclusions

In this contribution, we provide our inputs on interruption time during NR handover.

Proposal 1: The value of Tsearch during handover will be

- 0ms if the cell is known

- Up to 1 SMTC duration for a cell in FR1, provided the cell is detectable in first attempt

- Up to n SMTC durations for a cell in FR2, provided the cell is detectable in first attempt, where n is the number of UE Rx beams.

Proposal 2: The value of Tprocessing can be up to 20ms.

# References

[1] R4-1801308, Way forward on handover, RAN4#AH-1801, San Diego, US

[2] R4-1800911, HO timeline in NSA, RAN4#AH-1801, San Diego, US