3GPP TSG-RAN WG4 Meeting #86	R4-1803300

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	7.6.3

Source: 	Nokia, Nokia Shanghai Bell, Ericsson

Title: 	TP to TR 38.817-02: Simulation Assumptions for NR BS RF FRCs (Annex B)

Document for:	Approval

# 1.	Introduction

The way forward on NR BS RF FRCs was agreed in RAN4 AH-1801 [1], and the application of 3dB DMRS power boosting when no data in DMRS symbols as well as not to assume PTRS in the simulation have been agreed following RAN1 agreement.

This contribution provides a text proposal to record the agreed common simulation assumptions among the FRCs, including the 3dB DMRS power boosting and not to assume PTRS, into a new annex in TR 38.817-02 [2] for future reference. Individual FRC parameters are specified in TS 38.104 [3] and thus are not included.

# 2.	Text Proposal

<New annex>

Annex X (informative):
Simulation assumptions and results summary for RF Fixed Reference Channels

# Table X.1 Simulation assumptions for RF Fixed Reference Channels

| Parameter | Assumption (common) |
| --- | --- |
| DMRS pattern | Type 1: 1 symbol front loaded (symbol 2) + 1 additional (symbol 11), no FDM data in DMRS symbols |
| DMRS power boosting | 3 dB |
| Receiver type | MMSE |
| PTRS | None |
| Propagation | AWGN |
| Modulation | QPSK for REFSENS and ICS 16QAM for Dynamic Range |
| Expected coding rate | 1/3 for REFSENS and ICS 2/3 for Dynamic Range |
| Symbol type | CP-OFDM |
| HARQ | None |
| Number of receive antenna | 1 |
| Channel estimation | Practical |
| Criteria | SNR at 95%-tile throughput |
| MCS index | 4 for REFSENS and ICS 16 for Dynamic Range |

# Table X.2 Simulation parameters according to RAN1 agreements

| Parameter | Assumption (common) |
| --- | --- |
| Base graph | For coding rate ≤ 0.25, base graph 2 is selected For 0.25 ＜coding rate ≤ 0.67, base graph 2 is selected if transport block size ≤ 3824 （payload only, without CRC） For transport block size ≤ 292, base graph 2 is selected Otherwise, base graph 1 is selected |
| Transport block CRC | 24bits if transport block size > 3824 16bits if transport block size ≤ 3824 |
| Code block CRC | 24bits (if more than 1 code block) |
| Target coding rate | 308/1024 for REFSENS and ICS 658/1024 for Dynamic Range |

# Table X.3 Simulation results summary for RF Fixed Reference Channels

|  |  |  | Ericsson R4-1802310 R4-1802311 R4-1802312 R4-1802313 R4-1802314 | Huawei R4-1801675 R4-1801676 R4-1801677 R4-1801678 R4-1601679 | ZTE R4-1802412 R4-1802414 R4-1802415 R4-1802416 R4-1802417 | CATT R4-1801447 R4-1801448 R4-1801450 | Nokia R4-1802737 R4-1802738 R4-1802742 | Average | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | FR1 | G-FR1-A1-1 | -1.1 | -1.5 | -1.3 | -1.1 | -1.1 | -1.2 | 0.2 |
|  |  | G-FR1-A1-2 | -0.6 | -1 | -1 | -0.8 | -0.8 | -0.8 | 0.2 |
|  |  | G-FR1-A1-3 | -0.7 | -1 | -1 | -0.8 | -0.8 | -0.9 | 0.1 |
|  |  | G-FR1-A1-4 | -1.1 | -1.2 | -1.2 | -1 | -1.1 | -1.1 | 0.1 |
|  |  | G-FR1-A1-5 | -1.2 | -1.2 | -1.3 | -1.2 | -1.2 | -1.2 | 0 |
|  |  | G-FR1-A1-6 | -1 | -1.3 | -1.2 | -1.1 | -0.9 | -1.1 | 0.2 |
| ICS | FR1 | G-FR1-A1-7 | -0.8 | -1 | -1.1 | -1 | -0.8 | -0.9 | 0.1 |
|  |  | G-FR1-A1-8 | -0.4 | -0.4 | -0.8 | -1 | -0.6 | -0.6 | 0.3 |
|  |  | G-FR1-A1-6 | -0.4 | -0.4 | -0.8 | -1 | -0.6 | -0.6 | 0.3 |
| Dyn Range | FR1 | G-FR1-A2-1 | 8.9 | 10.1 | 9.1 | 9.1 | 9.2 | 9.3 | 0.5 |
|  |  | G-FR1-A2-2 | 8.9 | 9.7 | 9 | 9 | 9 | 9.1 | 0.3 |
|  |  | G-FR1-A2-3 | 8.9 | 9.7 | 9 | 9 | 9 | 9.1 | 0.3 |
|  |  | G-FR1-A2-4 | 8.8 | 10.1 | 9 | 9 | 9.1 | 9.2 | 0.5 |
|  |  | G-FR1-A2-5 | 8.9 | 10.6 | 9.1 | 9.2 | 9.2 | 9.4 | 0.7 |
|  |  | G-FR1-A2-6 | 9.1 | 9.6 | 9.2 | 9.3 | 9.3 | 9.3 | 0.2 |
| REFSENS | FR2 | G-FR2-A1-1 | -1.2 | -1.3 | -1.3 | -0.8 | -1.1 | -1.1 | 0.2 |
|  |  | G-FR2-A1-2 | -1.1 | -1.5 | -1.2 | -1 | -0.9 | -1.1 | 0.2 |
|  |  | G-FR2-A1-3 | -1.2 | -1.3 | -1.3 | -0.8 | -1.2 | -1.2 | 0.2 |
| ICS | FR2 | G-FR1-A1-4 | -1.1 | -1.6 | -1.3 | -1 | -1 | -1.2 | 0.3 |
|  |  | G-FR1-A1-5 | -0.8 | -1.3 | -1.1 | -0.8 | -0.8 | -1 | 0.2 |

<End of change>

# References

[1]	R4-1801031, “WF on NR BS RF FRCs”, Ericsson, Huawei, ZTE, CATT, Nokia, Nokia Shanghai Bell.

[2]	R4-1800270, “TR 38.817-02 v0.6.0: General aspects for BS RF for NR”, Ericsson.

[3]	3GPP TR 38.104 v15.0.0, “NR; Base Station (BS) radio transmission and reception”.