3GPPRAN4#87	R4-1807795

Busan, Korea, May 21st-25th

Title: 	Interruptions during BWP Switch

Source: 	Qualcomm Incorporated

Agenda item:	7.10.10.3 - Interruption and related requirements [NR_newRAT-Core]

Document for:	Discussion

# Introduction

During Ran4#86bis, a WF [1] was agreed to investigate interruptions on active carriers. In this contribution, we provide a proposal on interruptions that can be taken on all active CC’s, E-UTRA and NR, during a BWP switch on one of the NR carriers.

# Discussion

## Background

RAN4 has classified BWP switch in to the following four scenarios.

Scenario 1: The reconfiguration involves changing the center frequency of the BWP without changing its BW. The reconfiguration may or may not involve changing the SCS.

Scenario 2: The reconfiguration involves changing the BW of the BWP without changing its center frequency. The reconfiguration may or may not involve changing the SCS.

Scenario 3: The reconfiguration involves changing both the BW and the center frequency of the BWP. The reconfiguration may or may not involve changing the SCS.

Scenario 4: The reconfiguration involves changing only the SCS, where the center frequency and BW of the BWP remain unchanged.

Scenarios 1-3 involve RF changes on the UE, whereas scenario 4 is purely a baseband change. Since interruptions on other active carriers are only needed when UE changes its RF, there should no need for interruptions for scenario 4.

## Interruptions

From the UE RF perspective, a change in BWP is like cell activation/deactivation. We propose to re-use the interruptions requirements for cell activation/deactivation for BWP switch scenarios 1-3.  The current agreements for interruptions during cell activation/deactivation are as follows.

For E-UTRA victim, 1 sub-frame for sync 2 sub-frames for async.

For NR victim,

| SCS (kHz) | Sync (slots) | Async (slots) |
| --- | --- | --- |
| 15 | 1 | 2 |
| 30 | 1 | 2 |
| 60 | 3 |  |
| 120 | 5 |  |

Thus, we end up with the following two proposals

Proposal 1: Interruptions on active carriers needed only for BWP switch under scenarios 1-3. Interruptions on other active carriers are not needed for BWP switch that falls under scenario 4.

Proposal 2: Interruption for active carriers during a BWP switch are

For E-UTRA victim: 1 sub-frame for sync, 2 sub-frames for async. This is for both inter and intra-band

For NR victim

| SCS (kHz) | Sync (slots) | Async (slots) |
| --- | --- | --- |
| 15 | 1 | 2 |
| 30 | 1 | 2 |
| 60 | 3 |  |
| 120 | 5 |  |

# Conclusions

Proposal 1: Interruptions on active carriers needed only for BWP switch under scenarios 1-3. Interruptions on other active carriers are not needed for BWP switch that falls under scenario 4.

Proposal 2: Interruption for active carriers during a BWP switch are

For E-UTRA victim: 1 sub-frame for sync, 2 sub-frames for async. This is for both inter and intra-band

For NR victim

| SCS (kHz) | Sync (slots) | Async (slots) |
| --- | --- | --- |
| 15 | 1 | 2 |
| 30 | 1 | 2 |
| 60 | 3 |  |
| 120 | 5 |  |

# References

[1] R4-1805540, “Way forward on BWP Switching”, RAN4#86bis