3GPP TSG-RAN WG4 Meeting #88                 	R4-1810882

Gothenburg, SE, 20th – 24th Aug 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NR PUCCH

Agenda item:	7.13.2.3

Document for:	Discussion

Introduction

In this contribution we introduce simulation results using the parameters as agreed in [1].

Simulation assumptions

Simulation assumptions as agreed for NR PUCCH in [1] are shown below.

Table 1: Simulation assumptions for NR PUCCH

| PUCCH format | Format 0 | Format 1 | Format 2 | Format 3 |
| --- | --- | --- | --- | --- |
| Number of Tx | 1 | 1 | 1 | 1 |
| Number of Rx | 2 | 2 | 2 | 2 |
| SCS and BW | 15kHz: 10 30kHz: 40 60kHz: 100 120kHz: 100 | 15kHz: 10 30kHz: 40 60kHz: 100 120kHz: 100 | 15kHz: 10 30kHz: 40 60kHz: 100 120kHz: 100 | 15kHz: 10 30kHz: 40 60kHz: 100 120kHz: 100 |
| Number of OFDM-symbols | 1, 2 | 14 | 1, 2 | 14 |
| UCI bit number | 1 | 2 | 4, 22 | 16 |
| Number of PRB | 1 | 1 | 4, 9 | 1 |
| initialCyclicShift | 0 | 0 | - | - |
| startingSymbolIndex | 13 for 1 OFDM symbol, 12 for 2 OFDM symbol | 0 | 13 for 1 OFDM symbol, 12 for 2 OFDM symbol | 0 |
| The index of the orthogonal sequence i | - | 0 | - | - |
| Carrier Frequency [GHz] | FR1: 4 FR2:30 | FR1: 4 FR2:30 | FR1: 4 FR2:30 | FR1: 4 FR2:30 |
| DMRS pattern | - | - | - | with additional DMRS and without additional DMRS |
| Propagation Channel | FR1: TDL-A 30ns 10Hz FR2: AWGN | FR1: TDL-A 30ns 10Hz FR2: AWGN | FR1: TDL-A 30ns 10Hz FR2: AWGN | FR1: TDL-A 30ns 10Hz FR2: AWGN |
| Test metric | DTX to Ack probability <1% Missed Ack probability < 1% | DTX to Ack probability <1% Missed Ack probability < 1% NACK2ACK@0.1% | DTX to Ack probability <1%, Missed Ack probability < 1%, if number of bits <= 11 and BLER if number of bits > 11 | DTX to Ack probability <1%, Missed Ack probability < 1%, if number of bits <= 11 and BLER if number of bits > 11 |

Simulation results

Simulation results for NR PUCCH using the parameters from Table 1, and the DTX to ACK probability less than 1% are shown below in Table 2 for format 0, Table 3 for format 1, Table 4 for format 2, Table 5 and Table 6 for format 3.

Table 2: Initial simulation results for PUCCH format 0

| SCS [KHz] | CBW (MHz) | Number of OFDM-symbols | SNR [dB] with ACK missed detection probability 1 % |
| --- | --- | --- | --- |
| 15 | 10 | 1 | 6.8 |
| 15 | 10 | 2 | 3.3 |
| 30 | 40 | 1 | 7.9 |
| 30 | 40 | 2 | 2.5 |
| 60 | 100 | 1 | -0.5 |
| 60 | 100 | 2 | -2.9 |
| 120 | 100 | 1 | -0.5 |
| 120 | 100 | 2 | -2.9 |

Table 3: Initial simulation results for PUCCH format 1

| SCS [KHz] | CBW (MHz) | SNR [dB] with ACK missed detection probability 1 % | SNR [dB] with NACK to ACK probability 0.1 % |
| --- | --- | --- | --- |
| 15 | 10 | -6.2 | -5.1 |
| 30 | 40 | -6.5 | -5.5 |
| 60 | 100 | -11.5 | -9.6 |
| 120 | 100 | -11.5 | -9.6 |

Table 4: Initial simulation results for PUCCH format 2

| SCS [KHz] | CBW (MHz) | UCI bit number | SNR [dB] with ACK missed detection probability 1 % | SNR [dB] with BLER 1 % |
| --- | --- | --- | --- | --- |
| 15 | 10 | 4 | 1.3 | - |
| 15 | 10 | 22 | - | -4.0 |
| 30 | 40 | 4 | 2.8 | - |
| 30 | 40 | 22 | - | -4.5 |
| 60 | 100 | 4 | -5.9 | - |
| 60 | 100 | 22 | - | -8.9 |
| 120 | 100 | 4 | -5.9 | - |
| 120 | 100 | 22 | - | -8.8 |

Table 5: Initial simulation results for PUCCH format 3 and no additional DMRS

| SCS [KHz] | CBW (MHz) | SNR [dB] with BLER 1 % |
| --- | --- | --- |
| 15 | 10 | -1.5 |
| 30 | 40 | -2.0 |
| 60 | 100 | -7.0 |
| 120 | 100 | -7.0 |

Table 6: Initial simulation results for PUCCH format 3 with additional DMRS

| SCS [KHz] | CBW (MHz) | SNR [dB] with BLER 1 % |
| --- | --- | --- |
| 15 | 10 | -2.0 |
| 30 | 40 | -2.4 |
| 60 | 100 | -7.9 |
| 120 | 100 | -7.9 |

Conclusion

In this contribution we have presented simulation results for PUCCH with format 0, format 1, format 2 and format 3.

- References

- R4-1809556, Way forward on NR PUCCH demodulation performance in Rel-15, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, Samsung, ZTE