3GPP TSG-RAN WG4 Meeting RAN4#88                                               R4-180xxxxx

Gotherburg ,Sweden, 20 -24 Aug, 2018

Agenda Item:   7.13.1.5

Source:        Samsung

Title:          Test case design for fading CQI test cases

Document for:  Discussion

Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (FFS for wideband or sub-band) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for fading CQI test cases provided.

Discussion

Simulation assumption

| Parameter | Unit | Fading CQI test (FR1 FDD) | Fading CQI test (FR1 TDD) | Fading CQI test (FR2) |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | FFS | FFS |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | FFS |
| Correlation and antenna configuration |  | 2Rx 2*2 4Rx: 2*4 [ULA High] | 2Rx 2*2 4Rx: 2*4 [ULA High] | 2Rx 2*2 FFS |
| CSI-RS resource setting (periodic) |  |  |  |  |
| resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5/0 | 5/0 | 5/0 |
| NZP CSI-RS ports |  | 2 port {3000,3001} | 2 port {3000,3001} | 2 port {3000,3001} |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | [FD-CDM2/density 1] | [FD-CDM2/density 1] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | 000001 | 000001 | 000001 |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

Simulation results

Below test metrics were provided with initial simulation results:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to 0.02

Table 1: Initial results for FR1 FDD with 2Rx

| SNR | Percentage of Reported CQI Index not in { median CQI1 -1, median CQI1 +1} (%) | BLER | TP ratio |
| --- | --- | --- | --- |
| -4dB | 1.8 | 0.3826 | 1.193 |
| -2dB | 13.1 | 0.3446 | 1.149 |
| 0dB | 14.25 | 0.397 | 1.213 |
| 2dB | 30.25 | 0.4396 | 1.220 |
| 4dB | 34.2 | 0.544 | 1.061 |
| 6dB | 39.15 | 0.464 | 1.310 |
| 8dB | 30.3 | 0.3898 | 1.279 |
| 10dB | 35.35 | 0.3996 | 1.375 |
| 12dB | 31.35 | 0.3278 | 1.01 |
| 14dB | 39.8 | 0.3256 | 1.06 |
| 16dB | 52.15 | 0.2938 | 1.506 |
| 18dB | 38.8 | 0.3238 | 1.601 |
| 20dB | 31.95 | 0.3374 | 0.95 |
| 22dB | 9.25 | 0.3262 | 1.121 |
| 24dB | 11.4 | 0.2858 | 2.238 |

Table 1: Initial results for FR1 FDD with 4Rx

| SNR | Percentage of Reported CQI Index not in {median CQI1 -1, median CQI1 +1} (%) | BLER | TP ratio |
| --- | --- | --- | --- |
| -8dB | 0 | 0.5052 | 1.17025 |
| -6dB | 1.3 | 0.3808 | 1.204 |
| -4dB | 2.35 | 0.3844 | 1.284 |
| -2dB | 14.35 | 0.5928 | 0.673 |
| 0dB | 23.5 | 0.622 | 0.884 |
| 2dB | 32.15 | 0.5402 | 1.252 |
| 4dB | 25.65 | 0.4254 | 1.047 |
| 6dB | 28.2 |  |  |
| 8dB | 28.7 | 0.3354 | 1.04 |
| 10dB | 20.65 | 0.2326 | 1.022 |
| 12dB | 42.45 | 0.2424 | 1.347 |
| 14dB | 47.65 | 0.2084 | 1.408 |
| 16dB | 21.6 | 0.2062 | 1.761 |
| 18dB | 19.65 | 0.2212 | 1.134 |
| 20dB | 8.3 | 0.1878 | 1.100 |

Conclusion

In this contribution, we provided initial simulation results for fading CQI test.

Reference

[1] R4-1809358, “Way forward on NR UE CSI requirements”, Samsung