3GPP TSG-RAN WG4 WG4 #88bis 	R4-1813108

# Chengdu, China, 8 – 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104 on Outline for chapter 8 and 11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Within the performance part of the NR work item, work is ongoing to define and specify BS demodulation performance. The resulting performance requirements will be organized within chapter 8 and 11 of TS 38.104, plus Annexes for Propagation conditions, etc. Structure and specification text is now needed to complete the work on performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text for subclause 8.1 is proposed for the conducted requirements, based on the corresponding chapter 8.1 in TS 36.104. Headings are also proposed for PUSCH, PUCH and PRACH for clause 8 and 11, with separate subclauses for FR1 and FR2 in clause 11 for the radiated requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements would not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 8	Performance requirements

## 8.1	General

Conducted performance requirements for the BS are specified for the fixed reference channels defined in Annex A and the propagation conditions in Annex X. The requirements only apply to those FRCs that are supported by the base station.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a BS supporting carrier aggregation are defined in terms of single carrier requirements. For FDD operation the requirements in clause 8 shall be met with the transmitter(s) on.

NOTE:	In normal operating conditions the BS in FDD operation is configured to transmit and receive at the same time. The transmitter may be off for some of the tests as specified in 38.141-1 [5].

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S 	is the total signal energy in the subframe on a single antenna port.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe.

For enhanced performance requirements type A and type B, the SINR used in this clause is specified based on a single carrier and defined as:

Where:

is the total signal energy of one intra-cell UE in the subframe on a single antenna port.

is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna port. The respective energy of each inter-cell interferer relative to  is defined by its associated DIP value.

## 8.2	Applicability rule

## 8.3	Performance requirements for PUSCH

## 8.4	Performance requirements for PUCCH

## 8.5	Performance requirements for PRACH

# 11	Radiated performance requirements

## 11.1	General

## 11.2	Applicability rule

## 11.3	Performance requirements for PUSCH

### 11.3.1	Requirements in FR1

### 11.3.2	Requirements in FR2

## 11.4	Performance requirements for PUCCH

### 11.4.1	Requirements in FR1

### 11.4.2	Requirements in FR2

## 11.5	Performance requirements for PRACH

### 11.5.1	Requirements in FR1

### 11.5.2	Requirements in FR2