3GPP TSG-RAN WG4 Meeting #89	R4-1815244

# Spokane, WA, USA,  12 – 16 November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | <Rev#> | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TDD configuration for UE Tx test in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2018-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR TDD slot pattern (UL/DL configuration) used for UE UL tests may not be synchronized with LTE. Contiguous UL transmission time for SCS=60kHz is less than 1ms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a new TDD slot pattern used for UE Tx tests which is aligned with the LTE UL/DL configuration (UL/DL configuration 1 with special subframe configuration 5) assumed to be used for TDD-TDD LTE-NR EN-DC scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | According to the A.2.3, TDD slot pattern for REFSENS is applied for UE UL test. - If this pattern is used for TDD-TDD intra-band LTE-NR EN-DC scenario for example, it may cause the cross link interference and the UE may not satisfy the core requirement. - The core requirements of MOP, Transmit off power, and frequecny error shall be measured with the measurement period of 1ms. Since the current TDD slot patten for SCS=60kHz has only 0.25ms for UL period, RAN5 need to sepcify the measurment procesure with 4 discoutiguous UL slots. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## A.2.3	Reference measurement channels for TDD

TDD slot patterns as shown in Table A.2.3-1 will be used for UL RMCs defined below.

Table A.2.3-1 Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| UL-DL configuration | referenceSubcarrierSpacing | 15 kHz | 30 kHz | 60 kHz |
|  | Pattern 1 |  |  |  |
|  | dl-UL-TransmissionPeriodicity | 4 ms | 4 ms | 4 ms |
|  | nrofDownlinkSlots | 1 | 2 | 4 |
|  | nrofDownlinkSymbols | 3 | 6 | 12 |
|  | nrofUplinkSlot | 2 | 4 | 8 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
|  | Pattern 2 |  |  |  |
|  | dl-UL-TransmissionPeriodicity | 1 ms | 1 ms | 1 ms |
|  | nrofDownlinkSlots | 1 | 2 | 4 |
|  | nrofDownlinkSymbols | 0 | 0 | 0 |
|  | nrofUplinkSlot | 0 | 0 | 0 |
|  | nrofUplinkSymbols | 0 | 0 | 0 |

------------------------------------------------------------- End of change ---------------------------------------------------------