3GPP TSG RAN WG4 Meeting #90	R4-1901498

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	6.10.2.3

Source:	Huawei, HiSilicon

Title:	TP for synchronization procedures for NR-FR2-TM2 test model in TS 38.141-2

Document for:	Approval

# Introduction

This text proposal introduces a synchronization slot for the NR-FR2-TM2 test model in TS 38.141-2 [2]. An analogous text proposal for NR-FR1-TM2 test model in TS 38.141-1 is presented in [3].

# Discussion

The text proposal partitions the slots into measurement slots and synchronization slots as described in [1]. The slots not used for synchronization are available for measurements. The periodicity can be tailored for each numerology while still being aligned to TDD configuration.

# Text proposal

******************************* BEGIN ****************************

##### 4.9.2.2.2	NR FR2 test model 2 (NR-FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in section 4.9.2.2. Specific physical channel parameters for NR-FR2-TM2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| Slots available for measurements |  |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. Slots not used for synchronization are available for measurements |
| Slots used for synchronization |  |
| Slot number | 10×2μpk,k=0,⋯,p-1 p=[8] for µ=2; p=[8] for µ=3; |
| Number of RBs and location of RBs in synchronization slot | TBD |
| Ratio of PDSCH EPRE to DM-RS EPRE in synchronization slot | [-3 dB] |

******************************* END ****************************

# Reference

R4-1901496, “Discussion of synchronization procedures for TM2 test model”, Huawei, HiSilicon, RAN4#90, Feb. 25-Mar. 1, 2019.

TS 38.141-2

R4-1901497, “TP for synchronization procedures for NR-FR1-TM2 test model in TS 38.141-1”, Huawei, HiSilicon, RAN4#90, Feb. 25-Mar. 1, 2019.