3GPP TSG RAN WG4 Meeting #90bis	R4-1904353

Xian, China, April 8-12, 2019

Agenda Item:	6.11.5.6

Source: 	Ericsson

Title:	Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case in FR1

Document for:	Discussion

- Introduction

In the last RAN4 meeting the WF on the scope of RRM was approved [1]. The WF also contains list of Phase IV test cases as quoted below. All phase IV test cases are expected to be completed in Rel-15 by RAN4#91.

In this paper we briefly describe the methodology used for set of tests cases in test case scenario # 47.

# Inter-RAT NR Cell Search and Measurement Delay Tests

The purpose of the NR inter-RAT cell search and measurement delay testsis to verify the following requirements defined in TS 36.133:

- section 8.1.2.4.21 for E-UTRAN FDD-NR measurements and

- section 8.1.2.4.22 for E-UTRAN TDD-NR measurements.

The requirements are the same in both cases. Therefore, generic tests are developed. In each tests there are two cells: LTE PCell (cell 1) and neighbour NR cell (cell 2).

The test cases are defined with and without DRX cycles similar to the approach used in NR-NR inter-frequency test cases. In case of tests with DRX, DRX cycles of 40 ms and 640 ms are used since associated requirements are different. In FR1, test cases with and without SSB index are also developed.

In test cases in FR1, a measurement reporting Event B2 is used. The Event B2 is defined in TS 36.133 as follows:

- Event B2: PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2.

In all the test cases a margin of 8 dB is used for both thresholds i.e. related to LTE RSRP and SS-RSRP.

In test cases in FR2 (where cell 2 is FR2), the Event B2 is not feasible since it also requires comparison of LTE PCell. Therefore, Event B1 should be used in test cases in FR2.

In summary in this meeting the draft of the following 4 sets of NR inter-RAT cell search and measurement delay test cases in FR1 are provided:

- E-UTRAN inter-RAT NR cell search and measurement delay test case without SSB index and non-DRX in FR1

- E-UTRAN inter-RAT NR cell search and measurement delay test case without SSB index and in DRX in FR1

- E-UTRAN inter-RAT NR cell search and measurement delay test case with SSB index and non-DRX in FR1

- Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case with SSB index and in DRX in FR1

The corresponding test cases in FR2 shall be provided in RAN4#91.

- Summary

In this paper we have briefing summarized the set of NR inter-RAT cell search and measurement delay test cases in FR1, which used Event B2. CRs to introduce all 4 NR inter-RAT cell search and measurement delay tests in FR1 are provided in [2-5].

# References

- R4-1902032, Way forward on R15 NR RRM scope, Intel

- R4-1904354, Section A.8.4.2.1: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case without SSB index and non-DRX in FR1	Ericsson

- R4-1904355, Section A.8.4.2.2: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case without SSB index and in DRX in FR1	Ericsson

- R4-1904356, Section A.8.4.2.3: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case with SSB index and non-DRX in FR1	Ericsson

- R4-1904357, Section A.8.4.2.4: Phase IV-47: E-UTRAN inter-RAT NR cell search and measurement delay test case with SSB index and in DRX in FR1	Ericsson