3GPP TSG-RAN WG4 Meeting #93                 	 R4-1913590

Reno, USA, 18th – 22nd Nov, 2019

Agenda Item:	9.16.1.3

Source:	MediaTek Inc.

Title:	Discussion on measurement capability for CSI-RS RRM

Document for:		Discussion

# Introduction

A WI [1] was agreed to start discussing the requirement of CSI-RS based L3 measurement. One objective in [1] is about UE measurement capability, including number of frequency layer and number of cells etc. In this paper, we express our view on this issue.

# Discussion

Number of frequency layers to be monitored

Here, the frequency layers are inter-frequencies, if we follow the same definition for SSB in TS38.133. Therefore, the number of frequency layers to be monitored is highly related to the definition of intra and inter frequency. One important argument is that CSI-RS is not a standalone RS for the whole cell identification process. When CSI-RS was designed in RAN1, it was not targeting for any cell detection purposes. In other words, UE needs to first identify the cell based SSB to guarantee this cell is closed enough to UE. Then UE performs measurement on that CSI-RS of the nearby cell. UE should not just measure every CSI-RS configured by the network without checking if the cell is nearby or not. This is a waste of UE’s computation power and the usefulness of the measurement results are also questionable. SSB-based cell detection is still required for CSI-RS based L3 measurement. Configuring an MO with only CSI-RS but without SSB configuration would be weird, because UE will anyway perform SSB based cell detection and UE still needs SSB for handover, PSCell addition, SCell activation, CGI reading, … etc. Hence, we suggest not to define any requirements for this kind of MO.

Observation 1: SSB-based cell detection is still required for CSI-RS based L3 measurement.

Proposal 1: Requirement is only defined when

- the CSI-RS based measurement is configured with SSB based measurement within the same MO,

- UE can perform CSI-RS and SSB measurement with overlapped bandwidth in the same time duration

If the above proposal is agreeable to the group, then each MO which configures CSI-RS to be measured should also configure corresponding SSB. In this case, the number of layers to be monitored based on CSI-RS can be exactly the same as the number of layers to be monitored based on SSB. In other words, CSI-RS based L3 measurement does not add additional frequency layers to be monitored on top of the number specified for SSB based measurement.

Proposal 2: Given proposal 1 is agreed, CSI-RS based L3 measurement does not add additional frequency layers to be monitored on top of the number specified for SSB based measurement.

If above inter frequency definition is not agreeable to the group, then RAN4 will need to discuss the additional number of frequency layers to be monitored for MOs with only CSI-RS and for MOs with both CSI-RS and SSB. This means the configuration of CSI-RS measurement will consume additional frequency layers to be monitored by UE, even if it shares the same MO with SSB measurement. This is obviously not a preferred approach. For an example, in Figure 1 the total frequency layers to be monitored could be up to 9 if we have to treat SSB and CSI-RS separately. The consequence of allowing additional frequency layers only for CSI-RS is on the total measurement delay. As we know, inter-frequency measurement delay scales with the value of CSSF within gap, which is typically the total number of inter-frequency and inter-RAT layers. Therefore, the measurement delay of case b) in Figure 1 will be 1.5 longer than that of case a).

Observation 2: If proposal 1 is not agreed, CSI-RS based L3 measurement will add additional frequency layers to be monitored on top of the number specified for SSB based measurement, resulting the extended measurement delay of every frequency layer.

Figure 1. The total number of frequency layers to be monitored when both CSI-RS and SSB are configured in the same MO.

Another thing RAN4 needs to avoid is to allow some CSI-RS resources in the same MO to be intra-frequency and the other to be inter-frequency. This will also add the total number of layers to be monitored by UE, as shown in Figure 2.

Figure 2. Consequence of allow some CSI-RS resources in the same MO to be intra-frequency and the other to be inter-frequency

Observation 3: If RAN4 allows some CSI-RS resources in the same MO to be intra-frequency and the other to be inter-frequency, the total number of frequency layers to be monitored will be increased further.

Number of Cells to be monitored per layer

The 1st step of CSI-RS based cell identification is still relying on SSB-based cell search. Therefore, UE should never monitor those cells with CSI-RS, if the cells were not even detected through SSB before. This means that the cells to be monitored through L3 CSI-RS could only be a subset of the cell to be monitored through SSB. In other words, there is no need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB.

Proposal 3: There is no need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB.

Number of CSI-RS (beams) to be monitored per layer

According to TS38.133, UE can be configured with 96 CSI-RS configurations per MO. Frankly speaking, it is infeasible to ask UE to monitor 96 CSI-RS configurations in one frequency layer. Therefore, RAN4 needs to discuss the UE requirements, e.g., UE is only required to monitor X CSI-RS configurations out of 96 in a frequency layer. The value could be defined in similar way as SSB: different for intra frequency layer and inter frequency layer as well as FR2 dependent. Those values we agreed in SSB could be a good starting point for further discussion. Network may try to form several narrower CSI-RS beams within the spatial coverage of a SSB beam. In our view, network vendor can first provide some information on the possible deployments of CSI-RS beam, then RAN4 can decide the value based on network vendors’ input.

Proposal 4: Regarding the number of CSI-RS (beams) to be monitored per layer based on L3 CSI-RS, requirements defined for SSB is could be a starting point. Further input from network vendors are needed to decide the final values.

# Summary

In this paper, we discuss the measurement capability for L3 CSI-RS measurement. We have the following proposals.

Observation 1: SSB-based cell detection is still required for CSI-RS based L3 measurement.

Observation 2: If proposal 1 is not agreed, CSI-RS based L3 measurement will add additional frequency layers to be monitored on top of the number specified for SSB based measurement, resulting the extended measurement delay of every frequency layer.

Observation 3: If RAN4 allows some CSI-RS resources in the same MO to be intra-frequency and the other to be inter-frequency, the total number of frequency layers to be monitored will be increased further.

Proposal 1: Requirement is only defined when

- the CSI-RS based measurement is configured with SSB based measurement within the same MO,

- UE can perform CSI-RS and SSB measurement with overlapped bandwidth in the same time duration

Proposal 2: Given proposal 1 is agreed, CSI-RS based L3 measurement does not add additional frequency layers to be monitored on top of the number specified for SSB based measurement.

Proposal 3: There is no need to introduce additional number of cells to be monitored per layer based on L3 CSI-RS on top of the requirements already specified for SSB.

Proposal 4: Regarding the number of CSI-RS (beams) to be monitored per layer based on L3 CSI-RS, requirements defined for SSB is could be a starting point. Further input from network vendors are needed to decide the final values.

# Reference

[1] RP-191580, “New WID on RRM requirements for CSI-RS based L3 measurement”, CATT

[2] R4-1910821, “Definition of Intra and inter frequency for CSI-RS RRM”, MediaTek inc.