3GPP TSG-RAN WG4 #93	R4-1914634

Reno, USA, 14th – 18th November, 2019

Title: 					Discussion on Conditional handover requirements for NR

Source: 				Ericsson

Agenda item:	9.3.2.2

Document for:	Discussion

# Introduction

In RAN4#92bis, the way forward for conditional handover was agreed in [1]

- The starting point of the CHO procedure is from the end of the last TTI containing the RRC command carrying the CHO command. Handover delay is split into the following phases - Phase 0: CHO command RRC processing - Phase 1: Measurement - Phase 2: HO execution - From test case perspective, RAN4 shall follow RAN2 agreement that CHO execution does not trigger measurement report. - Handover delay is defined as: - DCHO = TRRC_1 + Tmeasure + TIU + TRRC_2 + Tprocessing + T∆ - Where: - TRRC_1: First segment of RRC processing time to process CHO command. - TRRC_2: Second segment of RRC processing time, starts after UE realizes the condition is met and identity of target cell is determined - Tmeasure: from the end of CHO RRC command is successfully decoded (no later than TRRC_1) until [UE realizes a handover condition for CHO is met]. - Tprocessing: is time for UE processing (same as existing requirement). - T∆: is time for fine time tracking and acquiring full timing information of the target cell (same as existing requirement). - Conditional handover requirements are needed for at least conditional handover and PSCell addition/change for standalone NR and standalone LTE. Depending on discussion in RAN2, requirements for interRAT PSCell addition in 36.133 or other requirements may be needed

# Discussion

Considering the way forward, there appear not to be too many open issues on the fundamental requirements for conditional handover, and the main aspect which now needs to be captured by RAN4 is how to capture the requirements in specifications. However, we first deal with some remaining open issues. We identify the following open issues

- How to capture Tmeasure in specifications.

Considering the formula for conditional handover delay DCHO = TRRC_1 + Tmeasure + TIU + TRRC_2 + Tprocessing + T∆, most of the factors are well known from legacy requirements. Tmeasure may be of indeterminant duration, depending on how long it takes for both a handover condition to be met, and the UE to realize that said condition has been met. From a requirements perspective, we think this is exactly the same as the situation with an event triggered measurement report, except that instead of sending a measurement report to gNB, the UE instead uses the outcome to trigger internal handover execution. This view is reinforced by the fact that RAN2 even discusses “events” when considering the conditions that trigger handover. Since the UE measurement model and requirements on measurement delays are already well known and specified for event triggered reporting by RAN4, it makes sense to use a similar way of specifying Tmeasure

Proposal 1 : Phase 1 of the conditional handover is specified by reusing event triggered reporting requirements

There are two alternatives for implementation of P1 in specifications.

Alt-1 : Modify and extend the scope of the existing event triggered reporting delay section to include conditional HO trigger evaluations. It should be noted that the existing requirement already excludes the additional time for UL measurement report insertion delay

Alt-2 : Make a new section with similar text as the event triggered reporting delay.

In draft CRs we have used alt-2 for clarity, although alt-1 also appears feasible and would avoid duplication of existing text.

- Split of Trrc_1 and Trrc_2

The RRC procedure delay for legacy handover in NR is 10ms. For the new procedure, RAN2 still needs to discuss RRC procedure delay. From RAN4 perspective the steps are split over phase 0 and phase 2. For simplicity we have assumed in CRs that Trrc_1=Trrc_2=10ms, although our view is that the actual RRC delays should be shorter (closer to  Trrc_1+Trrc_2=10ms). However, we do not see this as a particularly critical aspect as long as RAN2 does not significantly extend the procedure delay to much greater than 10ms..

Proposal 2 : RAN4 discusses and agrees split of Trrc_1 and Trrc_2

- Cases for conditional handover

In the RAN2 report, RAN2 agreed for NR-enhMOB

For conditional PScell change, A3/A5 execution condition should be supported while for conditional PScell addition, A4/B1 like execution condition should be supported.

Since B1 corresponds to Inter RAT neighbour becomes better than threshold, our understanding of this agreement is that interRAT addition of PSCell for EN-DC or NE-DC is supported, in addition to intraRAT PSCell addition and change.

Proposal 3 : RAN4 requirements support that interRAT addition of PSCell for EN-DC or NE-DC is supported, in addition to intraRAT PSCell addition and change and intraRAT handover

# Conclusions

Proposal 1 : Phase 1 of the conditional handover is specified by reusing event triggered reporting requirements

Proposal 2 : RAN4 discusses and agrees split of Trrc_1 and Trrc_2

Proposal 3 : RAN4 requirements support that interRAT addition of PSCell for EN-DC or NE-DC is supported, in addition to intraRAT PSCell addition and change and intraRAT handover

# References

- R4-1911041, “Way forward on NR mobility enhancement”, Intel