3GPP TSG-RAN WG4 Meeting #93	R4-1914828

# Reno, Nevada, USA, Nov 18th – 22nd, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0272 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introducing conditional handover requirements in TS38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conditional handover has been introduced in NR mobility enhancement WID, and the corresponding handover delay and interruption requirements shall be defined in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To define conditional handover requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conditional handover requirements will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.1.4	NR Conditional Handover

6.1.4.1	Introduction

The purpose of NR conditional handover is to change the NR PCell to another NR cell which meets the CHO execution condition configured for this NR cell.

6.1.4.2	NR FR1 - NR FR1 Conditional Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers from NR FR1 cell to NR FR1 cell.

6.1.4.2.1	Conditional handover delay

Procedure delays for the procedure that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message containing conditional handover execution conditions, the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC_1 + Tevaluation + Tinterrupt ms

Where:

TRRC_1 is the RRC process time to decode a RRC message containing the conditional handover execution conditions for the selected target cell.

Tevaluation is the time from the end of UE successfully decoding RRC message containing conditional handover execution conditions until the UE starts to execute a conditional handover.

Tinterrupt is the interruption time stated in clause 6.1.4.2.2.

During the time period Tevaluation, the UE shall perform the relevant intra-frequency or inter-frequency meansurements on the target cell, according to the measurements requirements described in Clause 9.2.5 for intra-frequency handover and Clause 9.3 for inter-frequency handover, and the UE shall evaluate the conditional handover execution contition defined in TS 38.331 [2].

6.1.4.2.2	Interruption time

The interruption time is the time between UE starts to execute a conditional handover and the UE starts transmission of the new PRACH.

When intra-frequency or inter-frequency conditional conditional handover is executed, the interruption time shall be less than Tinterrupt

Tinterrupt = TRRC_2 + Tprocessing + T∆ + Tmargin + TIU ms

Where:

TRRC_2 is the RRC process time to apply the stored RRC reconfiguration prepared for the selected target cell.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the conditional handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the conditional handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell..

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

6.1.4.3	NR FR2- NR FR1 Conditional Handover

The requirements in this clause are applicable to inter-frequency conditional handovers from NR FR2 cell to NR FR1 cell.

6.1.4.3.1	Conditional handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message containing conditional handover execution conditions, the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC_1 + Tevaluation + Tinterrupt ms

Where:

TRRC_1 is the RRC process time to decode a RRC message containing the conditional handover execution conditions for the selected target cell.

Tevaluation is the time from the end of UE successfully decoding RRC message containing conditional handover execution conditions until the UE starts to execute a conditional handover.

Tinterrupt is the interruption time stated in clause 6.1.4.3.2.

During the time period Tevaluation, the UE shall perform the relevant inter-frequency meansurements on the target cell, according to the measurements requirements described in Clause 9.3 for inter-frequency handover, and the UE shall evaluate the conditional handover execution contition defined in TS 38.331 [2]..

6.1.4.3.2	Interruption time

The interruption time is the time between UE starts to execute a conditional handover and the UE starts transmission of the new PRACH.

When inter-frequency conditional handover is executed, the interruption time shall be less than Tinterrupt

Tinterrupt = TRRC_2 + Tprocessing + T∆ + Tmargin + TIU ms

Where:

TRRC_2 is the RRC process time to apply the stored RRC reconfiguration prepared for the selected target cell.

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the conditional handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

6.1.4.4	NR FR2- NR FR2 Conditional Handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers from NR FR2 cell to NR FR2 cell.

6.1.4.4.1	Conditional handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message containing conditional handover execution conditions, the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC_1 + Tevaluation + Tinterrupt ms

Where:

TRRC_1 is the RRC process time to decode a RRC message containing the conditional handover execution conditions for the selected target cell.

Tevaluation is the time from the end of UE successfully decoding RRC message containing conditional handover execution conditions until the UE starts to execute a conditional handover.

Tinterrupt is the interruption time stated in clause 6.1.4.2.2.

During the time period Tevaluation, the UE shall perform the relevant intra-frequency or inter-frequency meansurements on the target cell, according to the measurements requirements described in Clause 9.2.5 for intra-frequency handover and Clause 9.3 for inter-frequency handover, and the UE shall evaluate the conditional handover execution contition defined in TS 38.331 [2].

6.1.4.4.2	Interruption time

The interruption time is the time between UE starts to execute a conditional handover and the UE starts transmission of the new PRACH.

When intra-frequency or inter-frequency conditional handover is executed, the interruption time shall be less than Tinterrupt

Tinterrupt = TRRC_2 + Tprocessing + T∆ + Tmargin + TIU ms

Where:

TRRC_2 is the RRC process time to apply the stored RRC reconfiguration prepared for the selected target cell.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the conditional handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the conditional handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

6.1.4.5	NR FR1- NR FR2 Conditional Handover

The requirements in this clause are applicable to inter-frequency conditional handovers from NR FR1 cell to NR FR2 cell.

6.1.4.5.1	Conditional handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message containing conditional handover execution conditions, the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC_1 + Tevaluation + Tinterrupt ms

Where:

TRRC_1 is the RRC process time to decode a RRC message containing the conditional handover execution conditions for the selected target cell.

Tevaluation is the time from the end of UE successfully decoding RRC message containing conditional handover execution conditions until the UE starts to execute a conditional handover.

Tinterrupt is the interruption time stated in clause 6.1.4.5.2.

During the time period Tevaluation, the UE shall perform the relevant inter-frequency meansurements on the target cell, according to the measurements requirements described in Clause 9.3 for inter-frequency handover, and the UE shall evaluate the conditional handover execution contition defined in TS 38.331 [2].

6.1.4.5.2	Interruption time

The interruption time is the time between UE starts to execute a conditional handover and the UE starts transmission of the new PRACH.

When inter-frequency conditional handover is executed, the interruption time shall be less than Tinterrupt

Tinterrupt = TRRC_2 + Tprocessing + T∆ + Tmargin + TIU ms

Where:

TRRC_2 is the RRC process time to apply the stored RRC reconfiguration prepared for the selected target cell.

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the conditional handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

<End of Change 1>