3GPP TSG-RAN WG4 Meeting #94-e	R4-2001250

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda Item:	6.7.2

Source: 	ZTE

Title: 	on the WRC-19 spurious emission for EESS band

Document for:	Approval

Introduction

The WRC-19 ended in December has accomplished a large number of conclusions and decisions which needs to be further studied within 3GPP. The spurious emission in EESS band was agreed and captured in the RESOLUTION 750[1]. For the EESS band, the European ECC also has its requirement listed in ECC Decision(18)06 [2]. These requirements are comparably high to the 3GPP current requirement in TS 38.104[3]. Some actions need to be taken within 3GPP to capture the decision in WRC-19.

Discussion

Different emission limit in WRC-19, 3GPP and ECC

During the WRC-19, the “Compatibility between the Earth exploration-satellite service (passive) and relevant active services” was discussed and the final conclusion is in [1]. Here we capture the requirement below in table 1:

Table 1 WRC-19 requirement for compatibility of EESS passive

| EESS (passive) band | Active service band | Active service | Limits of unwanted emission power from active service stations in a specified bandwidth within the EESS (passive) band1 |
| --- | --- | --- | --- |
| 23.6-24.0 GHz | 22.55-23.55 GHz | Inter-satellite | −36 dBW in any 200 MHz of the EESS (passive) band for non-geostationary (non-GSO) inter-satellite service (ISS) systems for which complete advance publication information is received by the Bureau before 1 January 2020, and −46 dBW in any 200 MHz of the EESS (passive) band for non-GSO ISS systems for which complete advance publication information is received by the Bureau on or after 1 January 2020 |
|  | 24.25-27.5 GHz | Mobile | −33 dBWa in any 200 MHz of the EESS (passive) band for IMT base stations5 −29 dBWb in any 200 MHz of the EESS (passive) band for IMT mobile stations5 |

From table 1 we know this is specifically for band n258 requirement. The upper mark of 5 in table 1 indicates as:

a 	A limit of −39 dB(W/200 MHz) will apply to IMT base stations brought into use after

1 September 2027. This limit will not apply to IMT base stations which have been brought into use prior to this date. For those IMT base stations, the limit of −33 dB(W/200 MHz) will continue to apply after this date.

b 	A limit of −35 dB(W/200 MHz) will apply to IMT mobile stations brought into use after

1 September 2027. This limit will not apply to IMT mobile stations which have been brought into use prior to this date. For those IMT mobile stations, the limit of −29 dB(W/200 MHz) will continue to apply after this date.

In other words, 6dB more stringent requirement will be applied after the year of 2027.

The requirement within table 1 is measured in 200MHz and it can be transferred to requirement measured in 1MHz as shown in table 2 below. Also the dBW has been transferred to dBm.

Talbe 2 WRC-19 requirement in different description

|  | BS before 2027 | UE before 2027 | BS after 2027 | UE after 2027 |
| --- | --- | --- | --- | --- |
| dBW/200MHz | -33 | -29 | -39 | -35 |
| dBm/200MHz | -3 | 1 | -9 | -5 |
| dBm/MHz | -26 | -22 | -32 | -28 |
| dBm/10MHz | -16 | -12 | -22 | -18 |

Observation 1: The WRC-19 Resolution 750 has agreed specific requirement for 23.6--24GHz for band n 258.

The requirement in 3GPP is captured in [3] for FR2 it should be as table 3 below:

Table 3 OTA OBUE requirement for FR2

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | Min(-5 dBm, Max(Prated,t,TRP – 35 dB, -12 dBm)) | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 43 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 33 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

Here the OBUE requirement is used instead of spurious emission is because the out-of-band domain delta fobue has been defined as 1.5GHz for FR2. Take 400MHz contiguous bandwidth which is currently used the most as an example, the requirement should be:

0 MHz to 40MHz: -5dBm/MHz

40MHz to 800MHz: -13dBm/MHz

800MHz to 1.5GHz: -5dBm/10MHz

Corresponding frequency range should be:

22.75GHz to 23.45GHz: -5dBm/10MHz

23.45GHz to 23.85GHz: -13dBm/MHz

23.85GHz to 24.25GHz: -5dBm/MHz

So the requirement within 23.6GHz to 24GHz should be:

23.6GHz to 23.85GHz: -13dBm/MHz

23.85GHz to 24GHz: -5dBm/MHz

If we compare this requirement in table 3 as -26dBm/MHz, the requirement in WRC-19 is 13dB more stringent than 3GPP.

Observation 2: The emission requirement within 23.6--24GHz agreed in WRC-19 is much more stringent than current 3GPP OBUE requirement.

The WRC-19 requirement applies globally while the ECC decision applied within Europe and it is even more stringent. From [2] the requirement is shown below:

Table 4 Requirement in ECC

-42dBW per 200MHz can be transferred as -35dBm/MHz which is 9dB more stringent than WRC-19 requirement. The figure 1 below shows the current situation of 3 different limits:

Figure 1 Three different emission limits with 23.6--24GHz

In figure 1, The 3GPP limit is in red, WRC-19 requirement is in green and ECC limit is in yellow.

Observation 3: ECC limit is 9dB more stringent than WRC-19 requirement.

The calculation to dBm/MHz of all the limits is to show the difference between 3 limits. In practical measurement, the larger measurement bandwidth usually means somehow less stringent requirement as the cumulative method will to some extent average the peak value. But the 13dB and 22dB difference shown in figure 1 need to be understood in 3GPP.

How to handle the difference

Firstly, we would like to talk about the most stringent requirement of ECC from [2]. There is on-going discussion within ECC during the January 2020 meeting that current ECC decision has more stringent requirement comparing to WRC-19. Weather to revise current decision has no agreement yet. Hence we can wait for ECC decision and take additional action after taht.

Observation 4: For ECC decision(18)06, we can wait for ECC decision.

Secondly, the WRC-19 requirement needs to be captured in 3GPP specification. We can see from table 1 that the WRC-19 requirement applies only for band n258. Current 3GPP specification captured the OBUE requirement with category A limit and category B limit. As the requirement is only for band n258, it is proposed to add this requirement in Cat A and Cat B limit table respectively and marked as “only for band n258”.

Corresponding CRs has been provided to capture the requirement.

Proposal: To capture the WRC-19 requirement in 3GPP specification.

Conclusion

In this contribution, we give analysis of WRC-19 decision and the observations and proposals are:

Observation 1: The WRC-19 Resolution 750 has agreed specific requirement for 23.6--24GHz for band n 258.

Observation 2: The emission requirement within 23.6--24GHz agreed in WRC-19 is much more stringent than current 3GPP OBUE requirement.

Observation 3: ECC limit is 9dB more stringent than WRC-19 requirement.

Observation 4: For ECC decision(18)06, we can wait for ECC decision.

Proposal: To capture the WRC-19 requirement in 3GPP specification.

- References

Resolution 750, WRC-19

ECC Decision(18)06

TS 38.104