3GPP TSG-WG4 Meeting #94-e	R4-2002148

# Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0277 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of unnecessary definition of offsetmax,IMD3 from Table 6.2.3.2-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility España SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-2-14 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The symbol offsetmax,IMD3 is not referenced outside of Table 6.2.3.2-1 and is not needed. The symbol can be replaced by its value and the row removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removes the row containing the definition of offsetmax,IMD3 and replaces the reference in the following row by its value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of A-MPR for NS_04 is more complicated than necessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections>>>

#### 6.2.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double counting MPR.

Allowed maximum power reduction is defined as A-MPR = max(MPR, A-MPR'),

Note that A-MPR' = 0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A2 relative to 23 dBm for power class 3 and PC2_A4 relative to 26 dBm for power class 2,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A1 relative to 23 dBm for power class 3 and PC2_A3 relative to 26 dBm for power class 2,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.2-1.

Table 6.2.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.2-2: A-MPR' values Access

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 |
|  | 16 QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 |
|  | 64 QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 |
|  | 256 QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 |
| NOTE: The A-MPR' values in this table apply for both A-MPR relative to 23 dBm for power class 3 and A-MPR relative to 26 dBm for power class 2 |  |  |  |  |  |

#### 6.2.3.3	A-MPR for NS_10

<<< End of changed sections>>>