3GPP TSG-RAN4 Meeting #94-e	R4-2000527

# Online, 24th Feb 2020 - 6th Mar 2020

Source: 	Nokia

Title: 					n26 A-MPR simulation results

Agenda Item: 			9.19.1

Document for:	Discussion

# 1	Introduction

In RAN4#92bis contribution [1] presented then scenarios for n26 for which NS-signalling with possible A-MPR needs to be introduced in order to protect other systems in adjacent bands. This proposal aligns with current E-UTRA specification. In this paper we present our simulation results for necessary A-MPR.

# 2	Discussion

First, we recapture the scenarios from [1].

Additional emission requirements are in Table 1.

Table 1:n26 additional emission requirements

| Range 1 | 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| --- | --- | --- | --- |
| Range 2 | 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| Range 3 | 851 ≤ f ≤ 859 | -53 | 6.25 kHz |

Four NS-values are required to accommodate different operator scenarios, these are as follows

NS-12:

Range 1 (806 ≤ f ≤ 813.5) additional emissions requirements:

- 5 MHz carrier in 814-819 MHz (SouthernLINC)

- 10 MHz carrier in 814-824 MHz (SouthernLINC)

NS-13:

Range 2 (806 ≤ f ≤ 816) additional emissions requirements:

- 5 MHz carrier in 817-822 MHz (Sprint)

- 5 MHz carrier in 819-824 MHz (Sprint)

NS-14:

Range 2 (806 ≤ f ≤ 816) additional emissions requirements:

- 5 MHz carrier between 824 and 849 MHz

- 10 MHz carrier between 824 and 849 MHz

- 15 MHz carrier between 824 and 849 MHz

- 20 MHz cattier between 824 and 849 MHz

NS-15:

Range 3 (851 ≤ f ≤ 859) additional emissions requirements:

- 5 MHz carrier between 814 and 824 MHz

- 5 MHz carrier between 824 and 849 MHz

- 10 MHz carrier in 814-824 MHz

- 10 MHz carrier in 825-835 MHz (Region 2 A block)

- 10 MHz carrier in 835-845 MHz (Region 2 B block)

- 10 MHz carrier in 839-849 MHz

- 15 MHz carrier in 824-839 MHz

- 15 MHz carrier in 834-849 MHz

- 20 MHz carrier in 824-844 MHz

- 20 MHz carrier in 829-849 MHz

In following pdf-file are presented pictorially associated with A-MPR triangles. These are NS-12, 13 and 14.

Based on our simulation we propose A-MPR as in Table 2. Due to high number of scenarios we do not propose modulation specific A-MPR instead a single A-MPR value per scenario is proposed except for cases 14, 16, 17 and 18 where 2 values are proposed. This approach is not as drastic as it may sound because when looking the A-MPR results the numbers are rather similar between the modulations.

Region A represents the cases when allocations on low frequency part of channel need A-MPR (left hand side)

Region B represents the cases when only narrow allocations on low frequency part of channel need A-MPR

Region C supplements Region B for those cases where also very wide allocations on low frequency part of channel need A-MPR

Regions D and E represent the cases when allocations on high and low frequency part of channel need A-MPR (right and left hand side, respectively) when using the RBEnd index instead of RBStart and LCRB

| Network Signalling | Cases | CBW (MHz) | Carrier Centre Freq., Fc (MHz) | Region A | AMPR (dB) | Region B |  | AMPR (dB) | Region C |  | AMPR (dB) | Regions D/E | AMPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | RBSstart |  | LCRB | RBStart |  | LCRB | RBStart |  | RBEnd |  |
| NS_12 | 1 | 5 | Fc ≥ 816.5 | ≤1.8MHz /(12*SCS) | 6.5 |  |  |  |  |  |  |  |  |
|  | 2 | 10 | Fc ≥ 819 | ≤3.6MHz /(12*SCS) | 6.5 |  |  |  |  |  |  |  |  |
| NS_13 | 3 | 5 | 819.5 ≤ Fc < 821.5 | ≤1.44MHz /(12*SCS) | 5 |  |  |  |  |  |  |  |  |
|  | 4 | 5 | Fc ≥ 821.5 |  |  | ≤540kHz /(12*SCS) | ≤540kHz /(12*SCS) | 4 | ≥3.6MHz /(12*SCS) | ≤360kHz /(12*SCS) | 2 |  |  |
| NS_14 | 6 | 10 | 830 ≤ Fc ≤ 844 |  |  | ≤540kHz /(12*SCS) | ≤360kHz /(12*SCS) | 4 |  |  |  |  |  |
|  | 7 | 15 | 837.5 ≤ Fc ≤ 841.5 |  |  | ≤1.08MHz /(12*SCS) | ≤2.16MHz /(12*SCS) | 4 |  |  |  |  |  |
|  | 8 | 20 | 834 ≤ Fc ≤ 839 |  |  | ≤2.16MHz /(12*SCS) | ≤3.42MHz /(12*SCS) | 4 | ≥12.96MHz /(12*SCS) | ≤2.7MHz /(12*SCS) | 2 |  |  |
| NS_15 | 10 | 5 | 840.5 ≤ Fc ≤ 846.5 |  |  |  |  |  |  |  |  | ≥2.52MHz /(12*SCS) | 10.5 |
|  | 13 | 10 | 830 < Fc ≤ 840 |  |  |  |  |  |  |  |  | ≥5.4MHz /(12*SCS) | 8 |
|  | 14 | 10 | 840 < Fc ≤ 844 |  |  |  |  |  |  |  |  | ≥4.14MHz /(12*SCS) | 11 |
|  |  |  |  |  |  |  |  |  |  |  |  | <4.14MHz /(12*SCS) | 2 |
|  | 15 | 15 | Fc ≤ 831.5 |  |  |  |  |  |  |  |  | ≥11.34MHz/(12*SCS) | 7.5 |
|  | 16 | 15 | 831.5 < Fc ≤ 841.5 |  |  |  |  |  |  |  |  | ≥6.48MHz /(12*SCS) | 10.5 |
|  |  |  |  |  |  |  |  |  |  |  |  | <6.48MHz /(12*SCS) | 2 |
|  | 17 | 20 | Fc ≤ 834 |  |  |  |  |  |  |  |  | ≥10.62MHz/(12*SCS) | 7.5 |
|  |  |  |  |  |  |  |  |  |  |  |  | <3.6MHz /(12*SCS) | 2 |
|  | 18 | 20 | 834 < Fc ≤ 839 |  |  |  |  |  |  |  |  | ≥8.46MHz /(12*SCS) | 10.5 |
|  |  |  |  |  |  |  |  |  |  |  |  | <8.46MHz /(12*SCS) | 2 |

# 3	Conclusion

In this contribution we have presented A-MPR simulation results for n26. All simulation results are in Annex.

# 4	References

[1] R4-1913078, n26 Emissions requirements, Sprint, Southern LINC, Dish Network, RAN4#92bis

# Appendix