3GPP TSG-RAN WG4 Meeting #94-e	R4-2001795

Electronic Meeting, 24th Feb. – 6th Mar., 2020

Agenda item:		8.6.3.2.1

Source:	MediaTek Inc.

Title:	Discussion on LTE CRS based and NR SSB based measurement in NR IDLE/INACTIVE mode

Document for:		Discussion

# 1	Introduction

In RAN4 #93 meeting, it is agreed that companies are encouraged to provide input and views on following early measurement reporting (EMR) issues [1]:

- Definition of overlapping carrier: - Option 1 (LTE definition): - An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements - A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements - Option 2: - An overlapping carrier is a carrier which the UE is actively measuring for mobility and EMR. - An non-overlapping carrier is a carrier which the UE is actively measuring for EMR only. - Tentative Agreement: Option 2. - Number of carriers to measure for EMR considering the use of overlapping and non-overlappnig carriers: - Option 1: Re-use the LTE euCA requirements for number of carriers to be reported for early measurements (Qualcomm) - Limit the number of overlapping and number of non-overlapping carriers (i.e. LTE EMR approach) - Option 2: Specify the total number of carriers (MediaTek) - Limit the number of overlapping + non-overlapping carriers (i.e. total number of carriers for EMR) - Option 3: No limit on carriers that can be configured for EMR (as long as it does not exceed UE measurement capability) (Nokia, Huawei) - Option 4: Limit the number of non-overlapping carriers only (New, option 3 + limit on number of non-overlapping carriers) FFS: UE behavior when the above limits are exceeded - Measurements for EMR and mobility: - Possible options: - S-measure threshold is assumed not applicable to carriers for EMR. - Overlapping EMR carriers are measured continuously. - Non-EMR carriers are measured according to s-Measure threshold. - Hence, beyond s-Measure threshold UE measures both EMR and non-EMR carriers. - To reduce overall measurement latency: - EMR carriers are measured when Serving cell RxLev is good - EMR carriers are not required to be measured or can be down selected when Serving cell RxLev is not good - Non-EMR carriers obey s-Measure threshold - RAN4 to discuss whether to apply threshold on serving cell condition for EMR: - Example: - EMR carriers will measured when Rxlev>threshold - EMR carriers are not required to be measured or can be down selected when Rxlev<threshold - Early measurement and cell change, discuss following aspects: - Possible Agreement: Follow RAN2 agreements which are (for information) - Dedicated: - UE performs measurements for EMR using the RRCRelease configured MOs independently from PCell changes while T331 is running. - SIB Configured: - UE performs EMR measurements according to serving cell SIB configuration while T331 is running. - FFS whether the above also applies when the carrier changes from overlapping to non-overlapping or from non-overlapping to overlapping after the cell change - Discuss if any additional rules are necessary from RAN4 side - At cell change, what is the UE behaviour related to reporting of EMR: - Option 1: leave it to UE implementation - Option 2: RAN4 defines specific rules - RAN4 to discuss introduction of the concept of unknown cells considering early measurement reporting - Option 1: Introduce known/unknown cells for EMR - Option 2: Do not introduce known/unknown cells for EMR - Impact on early measurements after T331 stops and before reporting EMR - FFS if RAN4 specifies applicable requirements for the early measurements performed and used for EMR after the expiry of the T331 timer (at least for overlapping carriers) - FFS if the UE shall be capable of reporting the results of the early measurements at least up to T0 after the expiry of T331 - On detected cell and cell detection: - The existing requirements under ca-IdleModeMeasurements as baseline, - FFS whether to defines requirements for the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode in 36.133 section 4.9.2.1 Possible options: - Option 1: Apply the same principles for NR inter-frequency cells. - Option 2: Do not apply the same principles for NR inter-frequency cells.

In this paper, we will discuss all the related issues for NR IDLE/INACTIVE CA and DC measurement.

# 2	Discussion on scenarios of the early measurement reporting

We would like to firstly discuss the scenarios for the early measurement reporting shown in Table 1. The first one is LTE CRS based measurement in LTE IDLE/INACTIVE mode and RAN4 had already discussed the corresponding requirements in the LTE euCA section. The remaining 3 scenarios are:

1. NR SSB based measurement in LTE IDLE/INACTIVE mode for DC purpose;

2. LTE CRS based measurement in NR IDLE/INACTIVE mode for DC purpose;

3. NR SSB based measurement in NR IDLE/INACTIVE mode for both DC and CA purposes.

Table 1: Measurements of inter-frequency CA candidate cells

| scenarios | EMR requirement | Purpose | Spec | Requirement Framework | Capability |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE CRS based measurement in LTE IDLE/INACTIVE | CA | 36.133 | Done | ca-IdleModeMeasurements-r15 in LTE RRC |
| 2 | NR SSB based measurement in LTE IDLE/INACTIVE | DC (EN-DC) | 36.133 | TBD | New proposed in LTE RRC (RAN2) |
| 3 | LTE CRS based measurement in NR IDLE/INACTIVE | DC (NE-DC) | 38.133 | TBD | New proposed in NR RRC (RAN2) |
| 4 | NR SSB based measurement in NR IDLE/INACTIVE | DC ,CA (NR-DC, NR-SA) | 38.133 | TBD | New proposed in NR RRC (RAN2) |

Considering that the capabilities for early measurement reporting are currently discussing in RAN2, we suggest that RAN4 should avoid the parallel discussion for capabilities and wait for the RAN2’s decision. RAN4 can firstly specify the corresponding requirements for scenarios 2, 3, and 4. Thus, in this paper, we will focus on the scenario 3 and 4.

# 3	Discussion on requirement framework of NR IDLE/INACTIVE DC and CA measurement

As listed in the WF [1], so far RAN4 has listed 5 related issues for the NR IDLE/INACTIVE mode DC and CA measurement. We will provide our own view on each of them.

- Definition of overlapping carrier:

From UE perspective, we don’t see the big impact of adopting the same overlapping definitions with LTE euCA. However, we agree that the option 2 might be a more proper one. One thing we would like to remind is that RAN4 should study whether there is any issue if adopting different overlapping definitions between LTE IDLE/INACTIVE mode DC/CA measurement and NR IDLE/INACTIVE mode DC/CA measurement.

Proposal 1: RAN4 to study whether there is any issue if adopting different overlapping definitions between LTE IDLE/INACTIVE mode DC and CA measurement and NR IDLE/INACTIVE mode DC and CA measurement.

- Number of carriers to measure for EMR:

As we mentioned in our previous paper [2], the restrictions on the number of reported overlapping or non-overlapping carriers might cause the issue that the carrier with better RSRP will not be reported to network. For example, assuming that UE is monitoring 8 carriers and the RSRP for these carriers are shown in Table 2.

Table 2: Example of EMR measurement with RSRP ranking

|  | Non-overlapping carriers | Overlapping carriers |
| --- | --- | --- |
| RSRP Ranking | F1: -82dBm | F4: -90dBm |
|  | F2: -86dBm | F5: -91dBm |
|  | F3: -89dBm | F6: -93dBm |
|  |  | F7: -95dBm |
|  |  | F8: -97dBm |

If RAN4 specifies the reported number of non-overlapping carrier, and the overall EMR number is 3:

- If the reported number of non-overlapping carrier is “1”, e.g., UE reports F1, F4, F5.

  - F2 and F3 with higher RSRPs than all overlapping carriers are not reported.

- If the reported number of non-overlapping carrier is “0”, e.g., UE reports F4, F5, F6.

  - F1, F2 and F3 with higher RSRPs than all overlapping carriers are not reported.

In our understanding, it’s unreasonable that UE cannot report the carrier with highest RSRP to network. So we suggest

Proposal 2: In NR IDLE/INACTIVE mode DC and CA measurement, RAN4 to only specify the total number of overlapping/non-overlapping carriers that UE should be able to report for early measurement reporting.

- Measurements for EMR and mobility:

In our understanding, UE might prefer to allocate more resources (time and power) for EMR when serving cell signal quality is good enough, i.e., Srxlev > Threshold,. On the other hand, when serving cell signal quality is no larger than a certain threshold, UE has less intention to perform frequently measurement for EMR. In this case, UE can spend more time and power on the re-selection measurement. So we suggest

Proposal 3: In NR IDLE/INACTIVE mode DC and CA measurement, UE monitors less carriers when one of the serving cell signal quality is no larger than a certain threshold and measures more carriers for DC and CA measurement otherwise.

- Early measurement and cell change:

- Dedicated and SIB configured EMR:

We see no issue to follow the RAN2 agreement shown as follows:

- The early measurement configuration can be different between that in RRCRelease and in SIB. If the UE receives the early measurement configuration from RRCRelease, this overrides the early measurement configuration provided in SIB (if any)

So we suggest that

Proposal 4: UE should measure the RRCRelease configured EMR carriers if they are configured; otherwise, UE should measure the SIB configured ERM carriers.

- UE behaviour related to reporting of EMR when PCell change:

From UE perspective, we prefer not to limit the UE behaviour related to EMR measurement while PCell changes and T331 is running. However, if companies agree that it is helpful to define some principles, we also provide our views:

- RRCRelease configured EMR: As mentioned in above item, RAN2 already agreed that “if UE receives the early measurement configuration from RRCRelease, this overrides the early measurement configuration provided in SIB.” Therefore, it is straightforward that UE should continue the monitoring of the RRCRelease configured EMR carrier while PCell changes and T331 is running.

- SIB configured EMR: There are 3 cases when UE is monitoring the SIB configured EMR carriers while PCell changes and T331 is running:

  - EMR carriers are listed in the old cell, but not listed in the new cell → 
UE should stop the monitoring

  - EMR carriers are listed in the old cell, and also listed in the new cell →
UE should continue the monitoring

  - EMR carriers are not listed in the old cell, but listed in the new cell → 
UE should start the monitoring

- Concept of unknown cells considering EMR:

The concept of known/unknown cell is already applied in the cell re-selection requirement and they are already specified in the re-selection requirement that:

- UE shall be able to evaluate a newly detectable cell within Tdetect

- UE shall be able to evaluate a cell has been detected within Tevaluate

From UE perspective, it would simplify the UE measurement scheduling in IDLE/INACTIV mode if requirements for EMR and re-selection follow the same principles. So we recommend that

Proposal 5: RAN4 to reuse the concept of known cell and unknown cell in Rel-15 re-selection requirement to specify the Rel-16 EMR measurement delay.

- Impact on early measurements after T331 stops:

Considering that RAN2 already agreed “it is up to UE implementation whether to continue monitoring the EMR after the expiry of T331 timer,” there is no need to specify the corresponding measurement delay for this case. However, we suggest to specify the measurement accuracy in case that UE would like to report the EMR measurement after T331 timer expires.

Proposal 6: Ran4 to specify the measurement accuracy for the overlapping and non-overlapping EMR carriers for the UE would like to report EMR after T331 timer expires.

- Apply detected cell NR inter-frequency requirement:

Before RAN4 determines whether to apply the same principles for NR detected inter-frequency cells when UE transitions from RRC CONNECTED mode to RRC IDLE mode and back to connected mode again, we prefer to discuss the necessary conditions first. For example, inter-frequency layer should also be configured as the EMR measurement object, otherwise UE might delete the cell ID and corresponding measurement results from the RRC when UE enters the idle mode.

Proposal 7: RAN4 to discuss the necessary conditions for UE to report measurement results based on the detected cell inter-frequency requirement when UE transitions from RRC connected mode to Idle mode and back to connected mode again.

The proposed overall CRS based and SSB Based EMR requirement framework in NR IDLE/INACTIVE mode can be found in Table 3. As we explained in Section 2, the measurement delay and accuracy of CRS based EMR measurement are specified for the NR UE to enter the NE-DC mode, and the measurement delay and accuracy of SSB based EMR measurement are specified for the NR UE to enter the NR-DC mode or the CA mode. RAN4 might need to discuss the evaluation methodology for measurement delay and accuracy in these 2 different cases. Besides, in LTE euCA [3], there is no corresponding requirement for non-overlapping carriers. RAN4 also has to determine whether to specify different measurement delay and accuracy requirement for overlapping and non-overlapping EMR carriers in NR. The proposed EMR requirement framework might be a good starting point, so we recommend that

Proposal 8: RAN4 to adopt the CRS based and SSB Based EMR requirement framework shown in following table as a baseline to further discuss the corresponding measurement delay and accuracy requirements

Table 3: CRS based and SSB Based EMR requirement framework in NR IDLE/INACTIVE mode

| Serving cell signal quality | Srxlev ≤ Threshold |  | Srxlev > Threshold |  |
| --- | --- | --- | --- | --- |
| Overlapping with carriers for re-selection | Overlapping | Non-overlapping | Overlapping | Non-overlapping |
| # of early measurement reporting carriers | At least [X] inter-freq. carriers |  | At least [X+Y] inter-freq. carriers, where Y>0 |  |
| CRS based Measurement delay | FFS | FFS | FFS | FFS |
| CRS based Measurement accuracy | FFS | FFS | FFS | FFS |
| SSB based Measurement delay | FFS | FFS | FFS | FFS |
| SSB based Measurement accuracy | FFS | FFS | FFS | FFS |

We also collect the aforementioned issues in Table 4, and the corresponding proposals from our side are also added.

Table 4: MTK’s views for the NR IDLE/INACTIVE CA and DC measurement

| Issue | Content | MTK’s views |
| --- | --- | --- |
| 1 | Definition of overlapping carrier | Both option 1 and option 2 are acceptable. Prefer option 2. |
| 2 | Number of carriers to measure for EMR | Option 2: Limit the number of overlapping + non-overlapping carriers (i.e. total number of carriers for EMR) |
| 3 | Measurements for EMR and mobility | Apply threshold on serving cell condition for EMR - EMR carriers will measured when Rxlev>threshold - EMR carriers are not required to be measured or can be down selected when Rxlev<threshold FFS: the value of the threshold. |
| 4 | Early measurement and cell change: - Dedicated and SIB configured EMR - UE behaviour related to reporting of EMR when PCell change - Concept of unknown cells considering EMR - Impact on early measurements after T331 stops | - Follow RAN2 agreement. UE measures the RRCRelease configured EMR carriers if they are configured; otherwise, UE measures the SIB configured ERM carriers. - When PCell change: - RRCRelease configured EMR: always continue the measurement of EMR - SIB configured EMR: - EMR carriers are listed in the old cell, but not listed in the new cell Stop - EMR carriers are listed in the old cell, and also listed in the new cell Continue - EMR carriers aren’t listed in the old cell, but listed in the new cell Start - Option 2: Not to introduce known/unknown cells for EMR (longer measurement delay for unknown cell) - RAN4 to specify the measurement accuracy (no measurement delay) for EMR after the expiry of the T331 timer |
| 5 | Apply detected cell NR inter-frequency requirement | FFS |

# 4	Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 to study whether there is any issue if adopting different overlapping definitions between LTE IDLE/INACTIVE mode DC and CA measurement and NR IDLE/INACTIVE mode DC and CA measurement.

Proposal 2: In NR IDLE/INACTIVE mode DC and CA measurement, RAN4 to only specify the total number of overlapping/non-overlapping carriers that UE should be able to report for early measurement reporting.

Proposal 3: In NR IDLE/INACTIVE mode DC and CA measurement, UE monitors less carriers when one of the serving cell signal quality is no larger than a certain threshold and measures more carriers for DC and CA measurement otherwise.

Proposal 4: UE should measure the RRCRelease configured EMR carriers if they are configured; otherwise, UE should measure the SIB configured ERM carriers.

Proposal 5: RAN4 to reuse the concept of known cell and unknown cell in Rel-15 re-selection requirement to specify the Rel-16 EMR measurement delay.

Proposal 6: Ran4 to specify the measurement accuracy for the overlapping and non-overlapping EMR carriers for the UE would like to report EMR after T331 timer expires.

Proposal 8: RAN4 to adopt the CRS based and SSB Based EMR requirement framework shown in following table as a baseline to further discuss the corresponding measurement delay and accuracy requirements

Table 3: CRS based and SSB Based EMR requirement framework in NR IDLE/INACTIVE mode

| Serving cell signal quality | Srxlev ≤ Threshold |  | Srxlev > Threshold |  |
| --- | --- | --- | --- | --- |
| Overlapping with carriers for re-selection | Overlapping | Non-overlapping | Overlapping | Non-overlapping |
| # of early measurement reporting carriers | At least [X] inter-freq. carriers |  | At least [X+Y] inter-freq. carriers, where Y>0 |  |
| CRS based Measurement delay | FFS | FFS | FFS | FFS |
| CRS based Measurement accuracy | FFS | FFS | FFS | FFS |
| SSB based Measurement delay | FFS | FFS | FFS | FFS |
| SSB based Measurement accuracy | FFS | FFS | FFS | FFS |

# Reference

- R4-1915842, Way Forward on Early measurement reporting, Nokia, Nov., 2019.

- R4-1914118, Discussion on IDLE/INACTIVE mode CA measurement, Nov., 2019.

- R4-1912845, Correction to S-measure threshold applicability in IDLE mode measurements, Oct., 2019.