3GPP TSG-WG RAN4 Meeting #95e	R4-2006290

# Electronic Meeting, 25 May - 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 306 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for suppporting Tx switching between two uplink carriers in UL CA and SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-05-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Time mask requirements need to be defined for Tx switching for inter-band CA and SUL band combinations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Time mask requirement is introduced for inter-band CA in section 6.3A.3.3. Time mask requirement is introduced for inter-band CA in 6.3C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is not allowed to support Tx switching between two uplink carriers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, New section 6.3A.3.3 and 6.3C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

< Unchanged part omitted >

TAE	Time Alignment Error

TAG	Timing Advance Group

Tx	Transmitter

UL MIMO	Uplink Multiple Antenna transmission

< End of first change >

< Start of second change >

6.3A.3	Transmit ON/OFF time mask for CA

6.3A.3.1	Void

6.3A.3.2	Void

6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

6.6A.3.3.1	General

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

6.3A.3.3.2	Time mask for UE supporting Tx chain switching between two uplink carriers

In addition to the requriements in 6.6A.3.3.1, the time mask specified in this sub-clause is applicable for UE supporting Tx chain switching between two uplink carriers configured on different NR bands. And it is only applicable when Tx chain is switched between NR UL carrier 1 with one transmit antenna connector and NR UL carrier 2 with two transmit antenna connectors.

The switching period X is located either on carrier 1 as described in Figure 6.3A.3.3.2-1a or on carrier 2 as described in Figure 6.3A.3.3.2-1b according to the indication by RRC signalling [7]. X is the value indicated by the UE capability uplinkTxSwitchingPeriod.

The requirements apply for the case of co-located and synchronized network deployment.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in sub-clause 4.2 of TS 38.213 [8].

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

< End of second change >

< Start of third change >

6.3C	Output power dynamics for SUL

6.3C.1	Void

6.3C.2	Void

6.3C.3	Transmit ON/OFF time mask for SUL

6.3C.3.1	Time mask for UE supporting Tx chain switching between two uplink carriers

The time mask specified in this sub-clause is applicable for UE supporting Tx chain switching between two uplink charriers from the SUL band and the NR band respectively. And it is only applicable when Tx chain is switched between NR UL carrier 1 with one transmit antenna connector and NR UL carrier 2 with two transmit antenna connectors.

The switching period X is located either on carrier 1 as described in Figure 6.3C.3.1-1a or on carrier 2 as described in Figure 6.3C.3.1-1b according to the indication by RRC signalling [7]. X is the value indicated by the UE capability uplinkTxSwitchingPeriod.

The requirements apply for the case of co-located and synchronized network deployment.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in sub-clause 4.2 of TS 38.213 [8].

Figure 6.3C.3.1-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.1-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

< End of third change >