3GPP TSG-RAN WG4 #95-e	R4-2007364

Electronic meeting, 25th May – 5th June 2020

Source: 	Ericsson

Title:  	URLLC BS ultra-low BLER test

Agenda Item:	6.9.1.1.2

Document for:	Discussion

# Introduction

During RAN4#94-e, a number of remaining parameters for the ultra-low BLER test were agreed. In this contribution, we present simulation results and proposals for the basestation requirement.

# Discussion

It has been agreed to specify the test with two sub-carrier spacings (i.e. 15kHz SCS and 30kHz SCS). Associated to each sub-carrier spacing is a bandwidth (10MHz for 15kHz SCS, 40MHz for 30kHz SCS). It has also been agreed to specify the test for PUSCH mapping type A and type B. The table below shows our current results for Ideal and Impairment SNR based on the assumptions agreed last meeting:

| Scenario | Ideal (dB) | Impairment (dB) |
| --- | --- | --- |
| 15kHz SCS, 10MHz, Mapping type A | -8.10 | -5.6 |
| 15kHz SCS, 10MHz, Mapping type B | -8.10 | -5.6 |
| 30kHz SCS, 40MHz, Mapping type A | -8.50 | -6.0 |
| 30kHz SCS, 40MHz, Mapping type B | -8.50 | -6.0 |

# Assumptions

| - Parameter | - Value |
| --- | --- |
| - Waveform | - CP-OFDM |
| - Antenna configuration | - 1x2 |
| - SCS | - 15kHz, 30kHz |
| - Bandwidth | - 10MHz (15kHz SCS), 40MHz (30kHz SCS) |
| - MCS | - MCS5 from table 3 |
| - PUSCH mapping type | - A, B |
| - Number of symbols | - 14 |
| - Start symbol | - 0 |
| - Number of resource blocks | - All available RB |
| - DM-RS | - Single symbol, type 1, 1+1 |
| - Retransmissions | - No retransmissions |
| - Propagation condition | - AWGN |