3GPP TSG RAN WG4 Meeting #95e	R4-200xxxx

Electronic meeting, 25th May – 5th June 2020

Title:	[DRAFT] LS on clarification for the definition of the UL duty cycle

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Apple Inc. [RAN WG4]

To:	RAN WG2

Cc:

Contact Person:

Name:	SAYENKO, Alexander

Tel. Number:

E-mail Address:	asayenko@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

During RAN4#91 meeting (May 2019), RAN WG4 sent LS to RAN WG2 asking to add a new UE capability to signal UE maximum UL duty cycle for FR2. The corresponding changes were implemented by RAN WG2 in R2-1908488 and R2-1908489. Referring to TS 38.306, current definition of the maximum UL duty cycle looks as follows:

maxUplinkDutyCycle-FR2

Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS38.101-2 [3].

RAN WG4 would like to clarify current definition of parameter by indicating explicitly that the UE maximum transmission power is assumed as a reference for the maximum UL duty cycle. The corresponding clarification might look as presented below:

maxUplinkDutyCycle-FR2

Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission at the UE maximum transmission power so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS38.101-2 [3].

RAN WG4 would also like to note that it is up to network implementation how this capability is applied to lower power levels other than the maximum transmission power.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN WG4 asks RAN WG2 to introduce changes into the definition of maxUplinkDutyCycle-FR2 parameter.

3. Date of Next RAN WG4 Meetings:

TSG WG4 Meeting #95		25th – 29th May 2020		Electronic meeting

TSG WG4 Meeting #96		24th – 28th August 2020	Toulouse, France