3GPP TSG RAN4 meeting #95e	R4-2008223

Electronic meeting, 25 May- 05 June, 2020	(revision of RP-200269)

- Source:	Huawei, HiSilicon

- Title:	WID revision: LTE Advanced inter-band CA Rel-16 for 2DL/2UL

- Document for:	Approval

Agenda Item:	5.5.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Information about Work Items can be found at http://www.3gpp.org/Work-Items

# Title: LTE Advanced inter-band CA Rel-16 for 2DL/2UL

Acronym: LTE_CA_ R16_2BDL_2BUL

Unique identifier: 800064

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

{Not applicable for Feature nor for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |
| 80064 | New WID on Rel16 LTE inter-band CA for 2 bands DL with 2 bands UL |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title.

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 80164 | Core part: New WID on Rel16 LTE inter-band CA for 2 bands DL with 2 bands UL | Child WID |
| 80264 | Perf. part: New WID on Rel16 LTE inter-band CA for 2 bands DL with 2 bands UL | Child WID |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

All new LTE inter band CA configurations for 2 different bands DL with 2 different bands UL will be defined under this WI. New configurations still emerge from exiting bands and whenever new band is specified, it will create a potential for several new LTE inter band CA configurations for 2 different bands DL with 2 different bands UL.

The LTE inter band CA configurations will be introduced in a release independent manner based on TS36.307, which will be updated depending on newly introduced CA configurations.

The preconditions to propose 2 different bands DL with 2 different bands UL in rel-16 are as follows.

- Constituent LTE band and LTE Intra band CA shall be completed and specified in advance.

Example 1: If the following CA configuration is proposed,

| Downlink CA configuration | Uplink CA configuration |
| --- | --- |
| CA_1C-2A | CA_1C-2A |

- LTE Band 2 requirements shall be completed and specified in advanced.

- LTE intra band CA of DL_1C_UL_1C requirements shall be completed and specified in advanced.

Example 2: If the following CA configuration is proposed,

| Downlink CA configuration | Uplink CA configuration |
| --- | --- |
| CA_1A-1C-2A | CA_1C-2A |

- LTE Band 2 requirements shall be completed and specified in advanced.

- LTE intra band CA of DL_CA_1A-1C_UL_CA_1C requirements shall be completed and specified in advanced.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

- Specify the band-combination specific RF requirements for all listed LTE inter band CA combinations for 2 different bands DL with 2 different bands UL including at least

  - Applicable frequencies

  - Applicable bandwidths and bandwidth sets

- Analyse combinations that have self-desensitization due to following reasons:

  - TX Harmonic and/or inter modulation overlap of receive band

  - TX signal overlap of receiver harmonic frequency

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons

- For the combination where self-desensitization exists, specify at least needed

  - ∆TIB and ∆RIB

  - Reference sensitivity excerptions

  - UL RB restrictions for REFSENS test

- Add conformance testing in RAN5 specifications (to follow at a later stage)

of all REL-16 CA combinations that fall into the category defined by the WI title. An overview table of these CA combinations is provided here:

CA configurations for 2 different bands DL with 2 different bands UL

Table 1-1: Individual combination names, proponents and supporting companies

| Combi nation | Num CC | CA configuration |  | REL-indep. from |  | contact name, company |  |  | contact email |  | other supporting companies (min. 3) |  |  | status (new, ongoing, completed, stopped) |  | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-11 | 2 | 3A-11A_UL_ 3A-11A_BCS0 |  | REL-11 |  | Xiao Shao, KDDI |  |  | ko-shou@kddi.com |  | Anritsu, Ericsson, Nokia, Samsung, Sumitomo |  |  | completed |  | 2B_2CC_DL_3A-11A_BCS0-completed |
| 26-48 | 2 | 26A-48A_UL_ 26A-48A_BCS0 |  | REL-14 |  | Shane Austin, Southern Linc |  |  | wsaustin@southernco.com |  | Ericsson, C Spire, Sprint |  |  | completed |  | 2B_2CC_DL_26A-48A_BCS0-completed |
| 26-48 | 3 | 26A-48A-48A_UL_ 26A-48A_BCS0 |  | REL-14 |  | Shane Austin, Southern Linc |  |  | wsaustin@southernco.com |  | Ericsson, C Spire, Sprint |  |  | completed |  | 2B_2CC_DL_26A-48A_UL 26A-48A_BCS0-completed 2B_3CC_DL_26A-48A-48A_UL_BCS0-completed |
| 26-48 | 3 | 26A-48C_UL_ 26A-48A_BCS0 |  | REL-14 |  | Shane Austin, Southern Linc |  |  | wsaustin@southernco.com |  | Ericsson, C Spire, Sprint |  |  | completed |  | 2B_2CC_DL_26A-48A_UL 26A-48A_BCS0--completed 2B_3CC_DL_26A-48C_BCS0-completed |
| 4-5 | 4 | 4A-4A-5B_UL_4A-5A_BCS0 |  | REL-11 |  | Zheng Zhao, Verizon |  |  | zheng.zhao@verizonwireless.com |  | Nokia, Qualcomm, Ericsson, Samsung |  |  | completed |  | 2B_4CC_DL_4A-4A-5B_UL_4A_BCS0 2B_4CC_DL_4A-4A-5B_UL_5A_BCS0 |
| 25-26 | 2 | 2BDL_25A-26A_2BUL_25A-26A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-26A_1UL_BCS0 completed |
| 25-26 | 3 | 2BDL_25A-25A-26A_2BUL_25A-26A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-25A-26A_1UL_BCS0 completed 25A-26A_2UL_25A-26A_BCS0 new |
| 25-41 | 2 | 2BDL_25A-41A_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-41A_1UL_BCS0 completed |
| 25-41 | 3 | 2BDL_25A-25A-41A_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-25A-41A_1UL_BCS0 completed |
| 25-41 | 3 | 2BDL_25A-41C_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-41C_1UL_BCS0 completed 25A-41A_2BUL_25A-41A_BCS0 new |
| 25-41 | 4 | 2BDL_25A-25A-41C_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-25A-41C_1UL_BCS0 completed 25A-41C_2BUL_25A-41A_BCS0 new |
| 25-41 | 4 | 2BDL_25A-41D_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-41D_1UL_BCS0 completed 25A-41C_2BUL_25A-41A_BCS0 new |
| 25-41 | 5 | 2BDL_25A-25A-41D_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 25A-25A-41D_1UL_BCS0 completed 2BDL_25A-41D_2BUL_25A-41A_BCS0 new 25A-25A-41C_2BUL_25A-41A_BCS0 new |
| 25-41 | 5 | 2BDL_25A-41E_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 41E_1UL_BCS0 completed 25A-41D_1UL_BCS0 completed 25A-41E_1UL_BCS0 new 25A-41D_2BUL_25A-41A_BCS0 new |
| 25-41 | 6 | 2BDL_25A-25A-41E_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 41E_1UL_BCS0 completed 25A-41D_1UL_BCS0 completed 25A-25A-41E_1UL_BCS0 new 25A-25A-41D_2BUL_25A-41A_BCS0 new 25A-41E_2BUL_25A-41A_BCS0 new |
| 25-41 | 6 | 2BDL_25A-41F_2BUL_25A-41A_BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 41F_1UL_BCS0 completed 25A-41E_1UL_BCS0 new 25A-41D_2BUL_25A-41A_BCS0 new |
| 25-41 | 7 | 25A-25A-41F_2BUL_25A-41A2BDL__BCS0 |  | REL-12 |  | George Cummings, Sprint |  |  | george.cummings@sprint.com |  | Nokia, Ericsson, Samsung |  |  | completed |  | 41F_1UL_BCS0 completed 25A-41F_1UL_BCS0 new 25A-25A-41E_1UL_BCS0 new 25A-25A-41E_2BUL_25A-41A_BCS0 new 25A-41F_2BUL_25A-41A_BCS0 new |
| 2-14 | 2 | 2BDL_2A-14A_2BUL_2A-14A_BCS0 |  | REL-12 |  | Marc Grant AT&T |  |  | marc.grant@att.com |  | Ericsson, Nokia, Qualcomm, Samsung |  |  | completed |  | 14A-66A_1UL_BSC0 (complete) |
| 14-30 | 2 | 2BDL_14A-30A_2BUL_14A-30A_BCS0 |  | REL-12 |  | Marc Grant AT&T |  |  | marc.grant@att.com |  | Ericsson, Nokia, Qualcomm, Samsung |  |  | completed |  | 14A-30A_1UL_BCS0 (complete) |
| 14-66 | 2 | 2BDL_14A-66A_2BUL_14A-66A_BCS0 |  | REL-12 |  | Marc Grant AT&T |  |  | marc.grant@att.com |  | Ericsson, Nokia, Qualcomm, Samsung |  |  | completed |  | 14A-66A_1UL_BCS0 (complete) |
| 2-14 | 3 | 3BDL_2A-2A-14A_2BUL_2A-2A_BCS0 |  | REL-12 |  | Marc Grant AT&T |  |  | marc.grant@att.com |  | Ericsson, Nokia, Qualcomm, Samsung |  |  | completed |  | 2A-2A_1UL_BCS0 (complete) 2A_14A_1UL_BCS0 (complete) |
| 14-66 | 3 | 3BDL_14A-66A-66A_2BUL_14A-66A_BCS0 |  | REL-12 |  | Marc Grant AT&T |  |  | marc.grant@att.com |  | Ericsson, Nokia, Qualcomm, Samsung |  |  | completed |  | 14A-66A_1UL_BCS0 (complete) 66A-66A_1UL_BCS0 (complete) |
| 14-66 | 4 | 4BDL_14A-66A-66A-66A_2BUL_14A-66A_BCS0 |  | REL-12 |  | Marc Grant AT&T |  |  | marc.grant@att.com |  | Ericsson, Nokia, Qualcomm, Samsung |  |  | completed |  | 14A-66A-66A_1UL_BCS0 (complete) 66A-66A-66A_1UL_BCS0 (complete) |
| 2-5 | 3 | 2BDL_2A-5B_2BUL_2A-5A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-5B_BCS0 Completed: 2BDL_2A-5A_2BUL_2A-5A_BCS0 |
| 13-66 | 2 | 2BDL_13A-66A_2BUL_13A-66A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: CA_2DL_13A-66A_BCS0 |
|  | 3 | 2BDL_13A-66A-66A_2BUL_13A-66A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_13A-66A-66A_BCS0 New: 2BDL_13A-66A_2BUL_13A-66A_BCS0 |
|  | 3 | 2BDL_13A-66B_2BUL_13A-66A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_13A-66B_BCS0 New: 2BDL_13A-66A_2BUL_13A-66A_BCS0 |
|  | 3 | 2BDL_13A-66C_2BUL_13A-66A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_13A-66C_BCS0 New: 2BDL_13A-66A_2BUL_13A-66A_BCS0 |
| 2-48 |  | 2BDL_2A-48A-48E_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | New: 2BDL_2A-48A-48E_BCS0 New: 2BDL_2A-48E_2BUL_2A-48A New: 2BDL_2A-48A-48D_2BUL_2A-48A |
|  |  | 2BDL_2A-48E_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48E_BCS0 New: 2BDL_2A-48D_2BUL_2A-48A |
|  |  | 2BDL_2A-48D_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48D_BCS0 |
|  |  | 2BDL_2A-48A-48D_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48A-48D_BCS0 New: 2BDL_2A-48A-48C_2BUL_2A-48A New: 2BDL_2A-48D_2BUL_2A-48A |
|  |  | 2BDL_2A-48A-48C_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48A-48C_BCS0 New: 2BDL_2A-48A-48A_2BUL_2A-48A New: 2BDL_2A-48C_2BUL_2A-48A |
|  |  | 2BDL_2A-48A-48A_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48A-48A_BCS0 New: 2BDL_2A-48A_2BUL_2A-48A |
|  |  | 2BDL_2A-48A_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48A_BCS0 |
|  |  | 2BDL_2A-48C_2BUL_2A-48A_BCS0 |  | REL-11 |  | Zheng Zhao Verizon |  |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung |  |  | completed |  | Completed: 2BDL_2A-48C_BCS0 New: 2BDL_2A-48A_2BUL_2A-48A |
| 4-28 | 2 | 4A-28A_BCS0 |  | Rel-11 |  | Israel Soto, AMX |  |  | israel.soto@americamovil.com |  | Nokia, Ericsson, Huawei, Samsung |  |  | completed |  | none |
| 1-7-7 | 3 | 1A-7A-7A_UL_1A-7A_BCS1 |  | 11 |  | Bo-Han Hsieh, CHTTL |  |  | pohanhsieh@cht.com.tw |  | Ericsson, Nokia, Mediatek, Google,SGS Wireless |  |  | completed |  | 2B_3CC_DL_1A-7A-7A_BCS1 (completed) 2B_2CC_DL_1A-7A_2B_2CC_UL_1A-7A_BCS0 (completed) |
| CA configuration |  |  | Uplink Configuration |  | BCS |  | REL-indep. from | contact name, company |  | contact email |  | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |  | supported next level fallback modes (in DL and UL) |  |
| CA_2A-2A-13A |  |  | CA_2A-13A |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | COMPLETED: DL_2A-2A _BCS0 COMPLETED: DL_2A-13A_UL_2A-13A_BCS0 |  |
| CA_2A-66B |  |  | CA_66B |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | COMPLETED: DL_2A-66A_BCS0 COMPELTED: DL_66B_UL_66B_BCS0 COMPLETED: DL_2A-66B_BCS0 |  |
| CA_5B-66B |  |  | CA_5B |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | NEW: DL_5B-66A_UL_5B NEW: DL_5A-66B_UL_5A-66A |  |
| CA_5B-66B |  |  | CA_66B |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | NEW: DL_5B-66A_UL_5A-66A NEW: DL_5A-66B_UL_66B |  |
| CA_2A-48C |  |  | CA_48C |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | COMPLETED: DL_2A-48C_BCS0 NEW: DL_48C_UL_48C |  |
| CA_2A-48A |  |  | CA_2A-48A |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | COMPLETED: DL_2A-48A_BCS0 |  |
| CA_2A-48C |  |  | CA_2A-48A |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | NEW: DL_2A-48A_UL_2A-48A COMPLETED: DL_2A-48C_BCS0 COMPLETED: DL_48C_BCS0 |  |
| CA_2A-48A-48A |  |  | CA_2A-48A |  | 0 |  | REL-11 | Zheng Zhao Verizon |  | zheng.zhao@Verizonwireless.com |  | Qualcomm, Ericsson, Nokia, Samsung | completed |  | NEW: DL_2A-48A_UL_2A-48A COMPLETED: DL_48A-48A_ BCS0 |  |

Bandwidth combination set for CA configuration

Table 2-1 Bandwidth combinations

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-11A | CA_3A-11A | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_4DL_46C-48C_2UL_48A-48A | CA_48A-48A | 46 | See CA_46C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3DL_46A-48C_2UL_48A-48A | CA_48A-48A | 46 | See CA_46A Bandwidth combination set 0 in 36.101 Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_26A-48A | CA_26A-48A | 26 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_26A-48A-48A | CA_26A-48A | 26 |  | Yes | Yes | Yes |  |  | 50 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_26A-48C | CA_26A-48A | 26 |  | Yes | Yes | Yes |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3DL_2A-2A-13A | CA_2A-13A | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
| CA_4DL_4A-4A-5B | CA_4A-5A | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4DL_4A-4A-5B | CA_5B | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-5A-66A-66B | CA_5A-66A | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-26A | CA_25A-26A | 25 |  | yes | yes | yes | yes | yes | 35 | 0 |
|  |  | 26 | yes | yes | yes | yes | yes |  |  |  |
| CA_25A-25A-26A | CA_25A-26A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 45 | 0 |
|  |  | 26 |  | yes | yes |  |  |  |  |  |
| CA_25A-41A | CA_25A-41A | 25 |  |  | yes | yes | yes | yes | 40 | 0 |
|  |  | 41 |  |  | yes | yes | yes | yes |  |  |
| CA_25A-25A-41A | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 41 |  |  | yes | yes | yes | yes |  |  |
| CA_25A-41C | CA_25A-41A | 25 |  |  | yes | yes | yes | yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-25A-41C | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-41D | CA_25A-41A | 25 |  |  | yes | yes | yes | yes | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-25A-41D | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-0 |  |  |  |  |  |  |  |
| CA_25A-41E | CA_25A-41A | 25 |  |  | yes | yes | yes | yes | 100 | 0 |
|  |  | 41 | See CA_41E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-25A-41E | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 120 | 0 |
|  |  | 41 | See CA_41E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-41F | CA_25A-41A | 25 |  |  | yes | yes | yes | yes | 120 | 0 |
|  |  | 41 | See CA_41F Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-25A-41F | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 140 | 0 |
|  |  | 41 | See CA_41F Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-14A | CA_2A-14A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |
| CA_14A-30A | CA_14A-30A | 14 |  |  | Yes | Yes |  |  | 15 | 0 |
|  |  | 30 |  |  | Yes |  |  |  |  |  |
| CA_14A-66A | CA_14A-66A | 14 |  |  | Yes | Yes |  |  | 25 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_13A-66A | CA_13A-66A | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_48A-66A | CA_48A-66A | 48 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-5B | CA_2A-5A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_5A-5A-66A | CA_5A-66A | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5B-66A-66A | CA_5A-66A | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_5A-66B | CA_5A-66A | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_5A-66C | CA_5A-66A | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_5B-66A | CA_5A-66A | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_13A-66A-66A | CA_13A-66A | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_13A-66B | CA_13A-66A | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_13A-66C | CA_13A-66A | 13 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_5B-66C | CA_5A-66A | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_2A-48A-48A | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_2A-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_5A-5A-66A-66A | CA_5A-66A | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_5A-5A-66B | CA_5A-66A | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_5A-5A-66C | CA_5A-66A | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_2A-48D | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_2A-48A-48C | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_2A-48E | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 of [1] |  |  |  |  |  |  |  |
| CA_2A-48A-48D | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48A-48E | CA_2A-48A | 2 |  |  | Yes | Yes | Yes | Yes | 120 | 0 |
|  |  | 48 | See the CA_48A-46E Bandwidth combination set 0 in Table 5.6A.1-3 of [1] |  |  |  |  |  |  |  |
| CA_4A-28A | CA_4A-28A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-7C | CA_4A-7A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-7A-7A | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

| CA_2A-2A-13A | CA_2A-13A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-5A | CA_2A-5A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 5 | Yes | Yes | Yes | Yes |  |  |  |  |
| CA_2A-66A-66A | CA_2A-66A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-66C | CA_2A-66A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-66A | CA_2A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_2A-66B | CA_2A-66A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-66A-66A | CA_2A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-2A-66B | CA_2A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-66C | CA_2A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-66C | CA_2A-66A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5B-66B | CA_5A-66A | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5B-66A | CA_5A-66A | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_5A-66B | CA_5A-66A | 5 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_48A-66A | CA_48A-66A | 48 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_48A-48A-66A | CA_48A-66A | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_48C-66A | CA_48A-66A | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1- |  |  |  |  |  | 60 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48A | CA_2A-48A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-48C | CA_2A-48A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1- |  |  |  |  |  |  |  |
| CA_2A-48A-48A | CA_2A-48A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_13A-48A | CA_13A-48A | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 48 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_13A-48A-48A | CA_13A-48A | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_13A-48C | CA_13A-48A | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1- |  |  |  |  |  |  |  |
| CA_13A-66A | CA_13A-66A | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz |  | 3 MHz | 5 MHz |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_5A-48A | CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_5A-48C | CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48D | CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A | CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| CA_13A-48C | CA_CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48D | CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48D-66A | CA_48A-66A | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | Yes | Yes |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_48D-66A-66A | CA_48A-66A | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48E-66A-66A | CA_48A-66A | 48 | See the CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66A-66A | CA_48A-66A | 48 | See the CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-66A-66A | CA_48A-66A | 48 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66A | CA_48A-66A | 48 | See the CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  | 60 | 0 |

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_48A_71A | CA_48A_71A | B48 |  |  |  | Yes | Yes | Yes | 25 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |
| CA_48B-71A | CA_48A_71A | B48 | See CA_48B Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  | 25 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |
| CA_48C_71A | CA_48A_71A CA_48C | B48 | See CA_48B Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  | 45 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-71A | CA_48A_71A | B48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 of 36.101 |  |  |  |  |  | 45 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |
| CA_48D-71A | CA_48A_71A CA_48C | B48 | See CA_48B Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  | 65 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-48A-71A | CA_48A_71A | B48 | See CA_48A-48A-48A Bandwidth combination set 0 in Table 5.6A.1-4 of 36.101 |  |  |  |  |  | 65 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-48A-48A-71A | CA_48A_71A | B48 | See CA_48A-48A-48A-48A Bandwidth combination set 0 in Table 5.6A.1-5 of 36.101 |  |  |  |  |  | 85 | 0 |
|  |  | B71 |  |  | Yes |  |  |  |  |  |

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-41C | CA_1A-41A CA_1A-41C | 1 |  |  |  | Yes | Yes |  | Yes |  | Yes |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-41C | CA_18A-41A CA_18A-41C | 18 |  |  |  | Yes |  | Yes |  | Yes |  |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-41A | CA_18A-41A | 18 |  |  |  | Yes |  | Yes |  | Yes |  |  | 35 | 0 |
|  |  | 41 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes to be added to release independence TS 36.307.

of all REL-16 CA combinations that fall into the category defined by the WI title. See overview table in 4.1 above.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 36.716-02-02 | LTE inter-band CA for 2 bands DL with 2 bands UL, Release 15 | TSG#87 | TSG#87 | rapporteur: LIU, Ye, Huawei, leo.liuye@huawei.com Core part |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.101 | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception | TSG#87 | Core part |
| 36.307 | LTE: Requirements on User Equipments (UEs) supporting a release-independent frequency band | TSG#87 | Perf. part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

LIU, Ye, Huawei, leo.liuye@huawei.com

## 7	Work item leadership

R4

## 8	Aspects that involve other WGs

None

NOTE:	For RAN WIDs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

NTT DOCOMO, INC.

Huawei

Hisilicon

Qualcomm

Nokia

Nokia Shanghai Bell

Ericsson

LGE