3GPP TR 37.716-21-11 V0.11.0 (2020-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

4.1	TR Maintenance	10

5	EN-DC of 2 bands LTE inter-band CA and 1 NR band: Specific Band Combination Part	10

5.1	Inter-band EN-DC within FR1	10

5.1.1	DC_3-18_n77	10

5.1.2	DC_3-18_n78	11

5.1.3	DC_3-18_n79	12

5.1.4	DC_3-41_n77	13

5.1.5	DC_3-41_n79	16

5.1.6	DC_1-5_n79	19

5.1.7	DC_3-5_n79	21

5.1.8	DC_5-41_n79	23

5.1.9	DC_1-7_n78	25

5.1.10	DC_3-8_n77	26

5.1.11	DC_3-8_n79	28

5.1.12	DC_28-41_n77	29

5.1.13	DC_28-41_n78	30

5.1.14	DC_28-41_n79	31

5.1.15	DC_1-8_n77	32

5.1.16	DC_1-8_n79	33

5.1.17	DC_3-3-7_n78, DC_3-3-7-7_n78	35

5.1.18	DC_18-42_n77	36

5.1.19	DC_18-42_n78	37

5.1.20	DC_18-42_n79	38

5.1.21	DC_8-20_n78	39

5.1.22	DC_20-38_n78	40

5.1.23	DC_66-(n)71	41

5.1.24	DC_2-(n)71	42

5.1.25	DC_7-8_n77	42

5.1.26	DC_7-8_n78, DC_7-7-8_n78	43

5.1.27	DC_2-7_n78	45

5.1.28	DC_7-66_n78	48

5.1.29	DC_3-8_n78	52

5.1.30	DC_3-28_n78	53

5.1.31	DC_5-7_n71	54

5.1.32	DC_2-7_n71	57

5.1.33	DC_2-66_n78	60

5.1.34	DC_1-3_n7	64

5.1.35	DC_3-28_n41	65

5.1.36	DC_1-11_n78	66

5.1.37	DC_8-11_n77	67

5.1.38	DC_8-11_n78	69

5.1.39	DC_2-30_n5 and DC_2-2-30_n5	70

5.1.40	DC_30-66_n5, DC_30-66-66_n5 and DC_30-66-66-66_n5	71

5.1.41	DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1	73

5.1.42	DC_1-3_n5	74

5.1.43	DC_1-7_n5	75

5.1.44	DC_1-28_n5	76

5.1.45	DC_3-7_n5	79

5.1.46	DC_3-28_n5	80

5.1.47	DC_7-28_n5	81

5.1.48	DC_1-11_n77	84

5.1.49	DC_3-20_n1	85

5.1.50	DC_1-20_n3	86

5.1.51	DC_2-66_n5, DC_2-2-66_n5, DC_2-66-66_n5 DC_2-2-66-66_n5, and DC_2-66-66-66_n5	88

5.1.52	DC_3-7_n77, DC_3-3-7_n77, DC_3-7-7_n77, DC_3-3-7-7_n77	89

5.1.53	DC_2-13_n66 and DC_2-2-13_n66	91

5.1.54	DC_2-7_n66, DC_2-7-7_n66	93

5.1.55	DC_2-66_n66	94

5.1.56	DC_7-66_n66, DC_7-7-66_n66	95

5.1.57	DC_3-8_n1, DC_3-3-8_n1	96

5.1.58	DC_7-8_n1, DC_7-7-8_n1	97

5.1.59	DC_1-8_n3	98

5.1.60	DC_8-42_n77	99

5.1.61	DC_3-40_n1	100

5.1.62	DC_7-40_n1	104

5.1.63	DC_2-66_n66	107

5.1.64	DC_5-66_n5 and DC_5-66-66_n5	108

5.1.65	DC_2-5_n66 and DC_2-5-5_n66	109

5.1.66	DC_13-66_n66 and DC_13A-66A-66A_n66A	110

5.1.67	DC_2-7_n66	111

5.1.68	DC_7-13_n66	114

5.1.69	DC_2-66_n41	117

5.1.70	DC_12-30_n2	118

5.1.71	DC_12-66_n2 12-66-66_n2	119

5.1.72	DC_30-66_n2, DC_30-66-66_n2	120

5.1.73	DC_1-8_n28	121

5.1.74	DC_3-3-8_n78	123

5.1.75	DC_2-46_n41	124

5.1.76	DC_2-46_n71	125

5.1.77	DC_46-66_n41	126

5.1.78	DC_46-66A_n71	127

5.1.79	DC_1-3_n38	128

5.1.80	DC_7-20_n3	129

5.1.81	DC_25-41_n41, DC_25-(n)41, DC_25-25-41_n41, DC_25-25-(n)41	130

5.1.82	DC_1-8_n3	131

5.1.83	DC_13-48_n2	132

5.1.84	DC_13-48_n66	133

5.1.85	DC_48-66_n5	134

5.1.86	DC_1-7_n3	135

5.1.87	DC_1-20_n3	136

5.1.88	DC_3-20_n38	137

5.1.89	DC_1-20_n38	138

5.1.90	DC_3-20_n38	139

5.1.91	DC_1-3_n77	139

5.1.92	 DC_2-4_n38	140

5.1.93	 DC_2-4_n41	143

5.1.94	 DC_1-3_n41	147

5.1.95	 DC_7-20_n1	151

5.1.96	DC_2-12_n2	154

5.1.97	DC_3-28_n7, DC_3-3-28_n7	155

5.1.98	DC_1-7_n7, DC_1-1-7_n7	157

5.1.99	DC_3-7_n7, DC_3-3-7_n7	158

5.1.100	DC_7-28_n7	159

5.1.101	DC_12-66_n66	160

5.1.102	DC_1-28_n7	161

5.1.103	DC_2-5_n66	162

5.1.104	DC_2-5_n2 and DC_2-5-5_n2	163

5.1.105	DC_2-5_n5 and DC_2-2-5_n5	164

5.1.105.4	REFSENS requirements	165

5.1.106	DC_2-13_n2	165

5.1.107	DC_2-13_n5 and DC_2-2-13_n5	166

5.1.108	DC_5-5-66_n66 and DC_5-5-66-66_n66	167

5.1.109	DC_5-13_n2	168

5.1.110	DC_5-66_n66 and DC_5-66-66_n66	169

5.1.111	DC_5-66_n78	170

5.1.112	DC_2-5_n71	172

5.1.113	DC_2-48_n71	175

5.1.114	DC_5-48_n71	178

5.1.115	DC_48_(n)12	181

5.1.116	DC_66_(n)12	182

5.1.117	DC_2-66_n38	183

5.1.118	DC_1-18_n3	184

5.1.119	DC_1-28_n3	185

5.1.120	DC_3-20_n7	186

5.1.121	 DC_1-3_n8	187

5.1.122	 DC_1-7_n8	191

5.1.123	 DC_1-20_n8	194

5.1.124	 DC_3-7_n8	198

5.1.125	 DC_3-20_n8	201

5.1.126	 DC_7-20_n8	205

5.1.127	 DC_1-41_n41	209

5.1.128	 DC_1-(n)41	211

5.1.129	DC_2-66_n25	213

5.1.130	DC_12-66_n25	214

5.1.131	DC_2-7_n38	216

5.1.132	DC_2-71_n38	217

5.1.133	DC_7-66_n38	218

5.1.134	DC_66-71_n38	219

5.1.135	DC_2-66_n38	220

5.1.136	DC_7-66_n71	221

5.1.137	DC_2-71_n66	222

5.1.138	DC_66-71_n66	223

5.1.139	DC_2-71_n78	224

5.1.140	DC_66-71_n78	225

5.1.141	DC_2-48_n12	226

5.1.142	DC_2-66_n12	227

5.1.143	DC_5-66_n71	228

5.1.144	DC_48-66_n12	229

5.1.145	DC_7-28_n3	230

5.1.146	DC_2-46_n66	231

5.1.147	DC_2-66_n48 and DC_2-66-66_n48	232

5.1.148	DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2	234

5.1.149	DC_13-46_n5	235

5.1.150	DC_13-66_n2 and DC_13-66-66_n2	236

5.1.151	DC_13-66_n48 and DC_13-66-66_n48	238

5.1.152	DC_46-66_n25	239

5.1.153	DC_1-41_n28	240

5.1.154	DC_11-18_n77	242

5.1.155	DC_11-18_n78	243

5.1.156	DC_18-41_n3	244

5.1.157	DC_2-48_n66	246

5.1.158	DC_1-11_n3	247

5.1.159	DC_3-8_n28	248

5.1.160	DC_8-11_n3	249

5.1.161	DC_1-20_n41	250

5.1.162	DC_3-20_n41	251

5.1.163	 DC_7-8_n3	252

5.1.164	 DC_1-(n)38	256

5.1.165	 DC_20-(n)38	258

5.1.166	 DC_20-(n)41	260

5.1.167	 DC_1-32_n78	262

5.1.168	 DC_3-32_n78	264

5.1.169	 DC_20-32_n78	267

5.1.170	DC_1-3_n40	269

5.1.171	DC_1-28_n40	270

5.1.172	DC_3-28_n40	271

5.1.173	DC_1-41_n3	272

5.1.174	DC_3-41_n28	273

5.1.175	DC_18-41_n77	274

5.1.176	DC_18-41_n78	276

5.1.177	 DC_1-3_n71	277

5.1.178	DC_1-7_n40	281

5.1.179	DC_3-7_n40	282

5.1.180	DC_7-28_n40	283

5.1.181	DC_2_(n)12	285

5.1.182	DC_5_(n)12	286

5.1.183	12_(n)5	287

5.1.184	48_(n)5	288

5.1.185	DC_48-66_n71	289

5.1.186	DC_2-29_n66 and DC_2-2-29_n66	290

5.1.187	DC_29-66_n2 and DC_29-66-66_n2	291

5.1.188	DC_1-42_n28	292

5.1.189	DC_3-42_n28	293

5.1.190	DC_8-42_n28	294

5.1.191	DC_2-14_n66	295

5.1.196	 DC_3-(n)41	300

5.1.197	 DC_3-41_n41	302

5.2	Inter-band EN-DC including FR2	304

5.2.1	DC_3-18_n257	305

5.2.2	DC_3-8_n257	305

5.2.3	DC_28-41_n257	307

5.2.4	DC_2-66_n257	307

5.2.5	DC_2-66_n261	309

5.2.6	DC_2-66_n260	310

5.2.7	DC_46-66_n261	312

5.2.8	DC_18-42_n257	314

5.2.9	DC_1-18_n257	315

5.2.10	DC_3-18_n257	316

5.2.11	DC_1-41_n257	317

5.2.12	DC_3-41_n257	318

5.2.13	DC_41-42_n257	319

5.2.14	DC_46-66_n260	321

5.2.15	DC_2-29_n260, DC_29-30_n260	322

5.2.16	DC_1-3_n257	323

5.2.17	DC_1-5_n257	324

5.2.18	DC_1-7_n257	325

5.2.19	DC_1-7-7_n257	326

5.2.20	DC_3-5_n257	327

5.2.21	DC_3-7_n257	328

5.2.22	DC_3-7-7_n257	329

5.2.23	DC_5-7_n257	330

5.2.24	DC_5-7-7_n257	331

5.2.25	DC_1-11_n257	331

5.2.26	DC_8-11_n257	332

5.2.27	DC_3-3-7_n257, DC_3-3-7-7_n257	333

5.2.28	DC_2-46_n261	334

5.2.29	DC_46-48_n261	335

5.2.30	DC_46-48_n260	337

5.2.31	DC_14-30_n260	338

5.2.32	DC_14-66_n260, DC_14-66-66_n260	339

5.2.33	DC_2-14_n260, DC_2-2-14_n260	340

5.2.34	DC_2-46_n258	342

5.2.35	DC_46-66_n258	343

5.2.36	DC_13-66_n260, DC_13-66-66_n260	344

5.2.37	DC_13-66_n261 and DC_13-66-66_n261	347

5.2.38	DC_2-66_n260 and DC_2-66-66_n260	351

5.2.39	DC_2-66_n261 and DC_2-66-66_n261	352

5.2.40	DC_2-13_n260	354

5.2.41	DC_2-13_n261	356

5.2.42	DC_2-5_n260	357

5.2.43	DC_2-5_n261	358

5.2.44	DC_5-66_n260 and DC_5-66-66_n260	359

5.2.45	DC_5-66_n261and DC_5-66-66_n261	360

5.2.46	DC_11-18_n257	362

5.2.47	DC_18-41_n257	363

Annex A: Change history	367

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of 3 different bands DL with 2 different bands UL (2 different LTE bands and 1 NR band) for the Rel-16 band combinations.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA and 1 NR band under Rel-16 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	EN-DC of 2 bands LTE inter-band CA and 1 NR band: Specific Band Combination Part

## 5.1	Inter-band EN-DC within FR1

## (three bands)

.2-1: Inter-band EN-DC configurations (three bands)

5.1.1	DC_3-18_n77

- 5.1.1.1	Operating bands for DC

Table 5.1.1.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-18_n77 | CA_3-18 | n77 | DC_3_n77 |

- 5.1.1.2	Configurations for DC

Table 5.1.1.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A | CA_3A-18A | n77 |

- 5.1.1.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n77 and Band-18 + Band n77 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n77 doesn’t fall into own Rx of band 18.

-	3rd order IMD generated by dual uplink of Band 18 + Band n77 may fall into own Rx of band 3.

Since Band 18 is only by one operator in Japan, frequency range of n77 can be constrained to 3.6GHz – 4.2GHz according to Japanese spectrum allocation. Then own Rx impact of 3rd band can be simplified as follows.

-	IMD generated by dual uplink of Band 3 + Band n77 doesn’t fall into own Rx of band 18.

-         IMD generated by dual uplink of Band 18 + Band n77 doesn’t fall into own Rx of band 3.

- 5.1.1.4	∆TIB and ∆RIB values

Table 5.1.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n77 | 0.5 |

- 5.1.1.5	REFSENS requirements

As stated in 5.1.1.3, there is no additional MSD requirement for this configuration according to Japanese spectrum allocation.

## 5.1.2	DC_3-18_n78

- 5.1.2.1	Operating bands for DC

Table 5.1.2.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-18_n78 | CA_3-18 | n78 | DC_3_n78 |

- 5.1.2.2	Configurations for DC

Table 5.1.2.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A | CA_3A-18A | n78 |

- 5.1.2.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n78 and Band-18 + Band n78 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 18.

-	3rd order IMD generated by dual uplink of Band 18 + Band n78 may fall into own Rx of band 3.

Since Band 18 is only by one operator in Japan, frequency range of n78 can be constrained to 3.6GHz – 3.8GHz according to Japanese spectrum allocation. Then own Rx impact of 3rd band can be simplified as follows.

-	IMD generated by dual uplink of Band 3 + Band n78 doesn’t fall into own Rx of band 18.

-         IMD generated by dual uplink of Band 18 + Band n78 doesn’t fall into own Rx of band 3.

- 5.1.2.4	∆TIB and ∆RIB values

Table 5.1.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n78 | 0.5 |

- 5.1.2.5	REFSENS requirements

As stated in 5.1.2.3, there is no additional MSD requirement for this configuration according to Japanese spectrum allocation.

## 5.1.3	DC_3-18_n79

- 5.1.3.1	Operating bands for DC

Table 5.1.3.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-18_n79 | CA_3-18 | n79 |  |

- 5.1.3.2	Configurations for DC

Table 5.1.3.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A | CA_3A-18A | n79 |

- 5.1.3.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n79 and Band-18 + Band n79 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings.

-	3rd and 4th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 19

-	IMD generated by dual uplink of Band 18 + Band n79 doesn’t fall into own Rx of band 3

Taking into account that Band 18 is currently operated only by a certain operator in Japan, the frequency range of Band 3 can be limited to UL/DL = 1710-1730/1805-1825 MHz. Then own Rx impact can be simplified as below.

-	IMD generated by dual uplink of Band 3 + Band n79 doesn’t fall into own Rx of band 18

-	IMD generated by dual uplink of Band 18 + Band n79 doesn’t fall into own Rx of band 3

- 5.1.3.4	∆TIB and ∆RIB values

Table 5.1.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18-n79 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n79 | 0 |

Table 5.1.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18-n79 | 3 | 0 |
|  | 18 | 0 |
|  | n79 | 0 |

- 5.1.3.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.4	DC_3-41_n77

- 5.1.4.1	Operating bands for DC

Table 5.1.4.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n77 | CA_3-41 | n77 | DC_3_n77 |

- 5.1.4.2	Configurations for DC

Table 5.1.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-41A_n77A | DC_3A_n77A DC_41A_n77A | CA_3A-41A | n77A |
| DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A | CA_3A-41C | n77A |

- 5.1.4.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n77 and Band-41 + Band n77 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 3 + Band n77 doesn’t fall into own Rx of band 41.

-	3rd order IMD generated by dual uplink of Band 41 + Band n77 may fall into own Rx of band 3.

- 5.1.4.4	∆TIB and ∆RIB values

Table 5.1.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-n77 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n77 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-n77 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- 5.1.4.5	REFSENS requirements

Table 5.1.4.5-1 lists the required MSD test configurations and Table 5.1.4.5-2 proposed the MSD levels from interested companies for the EN-DC 3A-41A_n77A band combination.

Table 5.1.4.5-1: MSD test configurations

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-41A_n77A | 3 | IMD5 | ¦3*fB3 -2*fn77¦ | 1720 | 5 | 25 | 1815 | 5 |
|  | n77 |  |  | 3900 | 10 | 52 | 3900 | 10 |
|  | 41 |  |  | 2640 | 5 | 25 | 2640 | 5 |
|  | 41 | IMD3 | ¦2*fB41 –fn77¦ | 2620 | 5 | 25 | 2620 | 5 |
|  | n77 |  |  | 3400 | 10 | 52 | 3400 | 10 |
|  | 3 |  |  | 1745 | 5 | 25 | 1840 | 5 |
|  | 41 | IMD4 | ¦2*fB41 –2*fn77¦ | 2520 | 5 | 25 | 2520 | 5 |
|  | n77 |  |  | 3445 | 10 | 52 | 3445 | 10 |
|  | 3 |  |  | 1755 | 5 | 25 | 1850 | 5 |

Table 5.1.4.5-2: MSD results from each company

| E-UTRA and NR Band |  |  |  |
| --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | MSD (LGE) | Source of IMD |
| DC_3A-41A_n77A | 3 | 5.3 | IMD5 ¦3*fB3-2*fn77¦ |
|  | n77 |  |  |
|  | 41 |  |  |
|  | 41 | 16.4 | IMD3 ¦2*fB41 - fn77¦ |
|  | n77 |  |  |
|  | 3 |  |  |
|  | 41 | 10.9 | IMD4 ¦2*fB41 - 2*fn77¦ |
|  | n77 |  |  |
|  | 3 |  |  |

Table 5.1.4.5-3 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 5.1.4.5-3: Proposed MSD requirements

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-41A_n77A | 3 | IMD5 | ¦3*fB3 -2*fn77¦ | 1720 | 5 | 25 | 1815 | 5 | 0.3 | N/A |
|  | n77 |  |  | 3900 | 10 | 52 | 3900 | 10 |  |  |
|  | 41 |  |  | 2640 | 5 | 25 | 2640 | 5 |  | 5.3 |
|  | 41 | IMD3 | ¦2*fB41 –fn77¦ | 2620 | 5 | 25 | 2620 | 5 | 1.4 | N/A |
|  | n77 |  |  | 3400 | 10 | 52 | 3400 | 10 |  |  |
|  | 3 |  |  | 1745 | 5 | 25 | 1840 | 5 |  | 16.4 |

## 5.1.5	DC_3-41_n79

- 5.1.5.1	Operating bands for DC

Table 5.1.5.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n79 | CA_3-41 | n79 | No |

- 5.1.5.2	Configurations for DC

Table 5.1.5.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-41A_n79A | DC_3A_n79A DC_41A_n79A | CA_3A-41A | n79A |
| DC_3A-41C_n79A | DC_3A_n79A DC_41A_n79A | CA_3A-41C | n79A |

- 5.1.5.3	Co-existence studies

Based on co-existence studies of Band 3 + Band n79 and Band 41 + Band n79 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings.

-	2nd and 5th order IMD generated by dual uplink of Band 3 + Band n79 may fall into own Rx of band 41.

-	2nd and 5th order IMD generated by dual uplink of Band 41 + Band n79 may fall into own Rx of band 3.

- 5.1.5.4	∆TIB and ∆RIB values

Table 5.1.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-n79 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-n79 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n79 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- 5.1.5.5	REFSENS requirements

Table 5.1.5.5-1 lists the required MSD test configurations and Table 5.1.5.5-2 proposed the MSD levels from interested companies for the EN-DC 3A-41A_n79A band combination.

Table 5.1.5.5-1: MSD test configurations

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-41A_n79A | 3 | IMD2 | ¦fB3 -fn79¦ | 1770 | 5 | 25 | 1865 | 5 |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 40 |
|  | 41 |  |  | 2670 | 5 | 25 | 2670 | 5 |
|  | 3 | IMD5 | ¦4*fB3 –fn79¦ | 1770 | 5 | 25 | 1865 | 5 |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 40 |
|  | 41 |  |  | 2640 | 5 | 25 | 2640 | 5 |
|  | 41 | IMD2 | ¦fB41 -fn79¦ | 2570 | 5 | 25 | 2570 | 5 |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 |
|  | 3 |  |  | 1755 | 5 | 25 | 1850 | 5 |
|  | 41 | IMD5 | ¦3*fB41 –2*fn79¦ | 2550 | 5 | 25 | 2550 | 5 |
|  | n79 |  |  | 4745 | 40 | 216 | 4745 | 40 |
|  | 3 |  |  | 1745 | 5 | 25 | 1840 | 5 |

Table 5.1.5.5-2: MSD results from each companies

| EUTRA and NR Band |  |  |  |
| --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | MSD (LGE) | Source of IMD |
| DC_3A-41A_n79A | 3 | 30.2 | IMD2 ¦fB3-fn79¦ |
|  | n79 |  |  |
|  | 41 |  |  |
|  | 3 | 5.0 | IMD5 ¦4*fB3-fn79¦ |
|  | n79 |  |  |
|  | 41 |  |  |
|  | 41 | 29.4 | IMD2 ¦fB41-fn79¦ |
|  | n79 |  |  |
|  | 3 |  |  |
|  | 41 | 4.8 | IMD5 ¦3*fB41 –2*fn79¦ |
|  | n79 |  |  |
|  | 3 |  |  |

Table 5.1.5.5-3 propose the required MSD levels for the dual connectivity configuration as average manner.

Table 5.1.5.5-3: Proposed MSD requirements

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-41A_n79A | 3 | IMD2 | ¦fB3 -fn79¦ | 1770 | 5 | 25 | 1865 | 5 | 2.2 | N/A |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 40 |  |  |
|  | 41 |  |  | 2670 | 5 | 25 | 2670 | 5 |  | 30.2 |
|  | 41 | IMD2 | ¦fB41 -fn79¦ | 2570 | 5 | 25 | 2570 | 5 | 2.3 | N/A |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 |  |  |
|  | 3 |  |  | 1755 | 5 | 25 | 1850 | 5 |  | 29.4 |

## 5.1.6	DC_1-5_n79

- 5.1.6.1	Operating bands for DC

Table 5.1.6.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-5_n79 | CA_1-5 | n79 | No |

- 5.1.6.2	Configurations for DC

Table 5.1.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A | CA_1A-5A | n79A |

- 5.1.6.3	∆TIB and ∆RIB values

For DC_1A-5A_n79A, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.6.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-5_n79 | 1 | 0.3 |
|  | 5 | 0.3 |
|  | n79 | 0 |

Table 5.1.6.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-5_n79 | 1 | 0 |
|  | 5 | 0 |
|  | n79 | 0 |

- 5.1.6.4	REFSENS requirements

For UE coexistence study of Band 5 + Band n79, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.6.4-1

Table 5.1.6.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 22000 | 25000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 3551 | 4176 | 5224 | 5849 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2702 | 3352 | 7951 | 9176 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6048 | 6698 | 9624 | 10849 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1853 | 2528 | 12351 | 14176 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6872 | 7547 | 14024 | 15849 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 7102 | 8352 | 10448 | 11698 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1004 | 1704 | 16751 | 19176 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 7696 | 8396 | 18424 | 20849 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6253 | 7528 | 11502 | 13352 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14848 | 16698 | 11272 | 12547 |

Based on co-existence study of Band 5 + Band n79 as presented in the table 5.1.6.4-1 and co-existence study of Band 1 + Band n79 captured in TR 37.863-01-01, own Rx impact of the 3rd band is shown as the followings.

- 3rd and 4th order IMD generated by dual uplink of Band 1 + Band n79 may also fall into own Rx of band 5

- 4th order IMD generated by dual uplink of Band 5 + Band n79 may also fall into own Rx of band 1

The MSD requirement for DC_1A-5A_n79A can reuse the requirements for DC_1A-18A_n79A and DC_1A-19A_n79A since the frequency range of Band5 are almost overlapped with Band 18 + Band 19 which means the architecture and components date will keep same between each other. Further, by adjusting the centre frequency point according to the specific combination of DC_1A-5A_n79A, the CF can also be reused with considering the same test channel bandwidth and same worst case that the victim channel bandwidth is just in the centre of total aggressive IMD bandwidth.

Table 5.1.6.4-2 lists the MSD required for DC_1A-5A_n79A. The same requirements of DC_1A-18A-n79 can be reused for this DC configuration.

Table 5.1.6.4-2: MSD for DC_1A-5A_n79A due to IMD issue

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 18.3 | FDD | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |  |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 8.9 | FDD | IMD4 |  |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | FDD | IMD4 |  |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | TDD | N/A |  |

## 5.1.7	DC_3-5_n79

- 5.1.7.1	Operating bands for DC

Table 5.1.7.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-5_n79 | CA_3-5 | n79 | No |

- 5.1.7.2	Configurations for DC

Table 5.1.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-5A_n79A | DC_3A_n79A DC_5A_n79A | CA_3A-5A | n79A |

- 5.1.7.3	∆TIB and ∆RIB values

For DC_3A-5A_n79A, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.7.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-5_n79 | 3 | 0.3 |
|  | 5 | 0.3 |
|  | n79 | 0 |

Table 5.1.7.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-5_n79 | 3 | 0 |
|  | 5 | 0 |
|  | n79 | 0 |

- 5.1.7.4	REFSENS requirements

For UE coexistence study of Band 5 + Band n79, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.7.4-1

Table 5.1.7.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 22000 | 25000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 3551 | 4176 | 5224 | 5849 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2702 | 3352 | 7951 | 9176 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6048 | 6698 | 9624 | 10849 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1853 | 2528 | 12351 | 14176 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6872 | 7547 | 14024 | 15849 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 7102 | 8352 | 10448 | 11698 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1004 | 1704 | 16751 | 19176 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 7696 | 8396 | 18424 | 20849 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6253 | 7528 | 11502 | 13352 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14848 | 16698 | 11272 | 12547 |

Based on co-existence study of Band 5 + Band n79 as presented in the table 5.1.7.4-1 and co-existence study of Band 3 + Band n79 captured in TR 37.863-01-01, own Rx impact of the 3rd band is shown as the followings.

- 3rd and 4th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band 5

- 4th order IMD generated by dual uplink of Band 5 + Band n79 may also fall into own Rx of band 3

The MSD requirement for DC_3A-5A_n79A can reuse the requirements for DC_3A-19A_n79A since the frequency range of Band5 are almost overlapped with Band 19 which means the architecture and components date will keep same between each other. Further, by adjusting the centre frequency point according to the specific combination of DC_3A-5A_n79A, the CF can also be reused with considering the same test channel bandwidth and same worst case that the victim channel bandwidth is just in the centre of total aggressive IMD bandwidth.

Table 5.1.7.4-2 lists the MSD required for DC_3A-5A_n79A. The same requirements of DC_3A-19A-n79 can be reused for this DC configuration.

Table 5.1.7.4-2: MSD for the DC configuration due to IMD issue

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 18.5 |  | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | TDD | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | FDD | IMD4 |
|  | 5 | 842.5 | 5 | 25 | 887.5 | N/A |  | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |

## 5.1.8	DC_5-41_n79

- 5.1.8.1	Operating bands for DC

Table 5.1.8.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-41_n79 | CA_5-41 | n79 | No |

- 5.1.8.2	Configurations for DC

Table 5.1.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A | CA_5A-41A | n79A |

- 5.1.8.3	∆TIB and ∆RIB values

For DC_5A-41A_n79A, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.8.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-41_n79 | 5 | 0.3 |
|  | 41 | 0.3 |
|  | n79 | 0 |

Table 5.1.8.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-41_n79 | 5 | 0 |
|  | 41 | 0 |
|  | n79 | 0 |

- 5.1.8.4	REFSENS requirements

For UE coexistence study of Band 5 + Band n79, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.8.4-1

Table 5.1.8.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 22000 | 25000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 3551 | 4176 | 5224 | 5849 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2702 | 3352 | 7951 | 9176 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6048 | 6698 | 9624 | 10849 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1853 | 2528 | 12351 | 14176 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6872 | 7547 | 14024 | 15849 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 7102 | 8352 | 10448 | 11698 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1004 | 1704 | 16751 | 19176 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 7696 | 8396 | 18424 | 20849 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6253 | 7528 | 11502 | 13352 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14848 | 16698 | 11272 | 12547 |

Based on co-existence study of Band 5 + Band n79 as presented in the table 6.x.3-1 and co-existence study of Band 41 + Band n79 captured in TR 37.863-01-01, own Rx impact of the 3rd band is shown as the followings.

- 3rd and 5th order IMD generated by dual uplink of Band 41 + Band n79 may also fall into own Rx of band 5

- 4th order IMD generated by dual uplink of Band 5 + Band n79 may also fall into own Rx of band 41

Table 5.1.8.4-2 lists the MSD required for DC_5A-41A_n79A. It is worth to note that the harmonic and harmonic mixing issues for LTE CA_5-41 are covered in clause 7.3.1A of 36.101. For the MSD case of the IMD4 falling into Band 41, there are two companies’ results proposed in R4-1814459 and R4-1815961. So the final MSD requirement is an average value based on these two results.

Table 5.1.8.4-2: MSD for the DC configuration due to IMD issue

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | FDD | IMD3 ¦2*fB41-fBn79¦ |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | TDD | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | TDD | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 0.9 | FDD | IMD5 ¦2*fBn79-3*fB41¦ |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | TDD | N/A |
|  | n79 | 4437.5 | 40 | 216 | 4437.5 | N/A | TDD | N/A |
| DC_5A-41A_n79A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | TDD | IMD4 ¦fBn79-3*fB5¦ |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |

## 5.1.9	DC_1-7_n78

- 5.1.9.1	Operating bands for DC

Table 5.1.9.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n78 | CA_1-7 | n78 | - |

- 5.1.9.2	Channel bandwidths per operating band for DC

Table 5.1.9.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-7C_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A | CA_1A-7C | n78A |

- 5.1.9.3	Co-existence studies

Co-existence was studied for DC_1A-7A_n78A in Rel-15 and the results are captured in 37.863-02-01. No further studies needed for DC_1A-7C_n78A.

- 5.1.9.4	∆TIB and ∆RIB values

For DC_1A-7C_n78A the TIB,c and RIB values are same as for DC_1A-7A_n78A given in the tables below.

Table 5.1.9..4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7C_n78A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7C_n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

- 5.1.9.5	REFSENS

Table 5.1.9.5-1 lists the MSD required for the dual connectivity configuration.

Table 5.1.9.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-7C_n78A | 1 | 1930 | 5 | 25 | 2120 | 8.3 | FDD | IMD4 ¦fB78-3*fB1¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | TDD | N/A |
| DC_1A-7C_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 ¦fB78-3*fB1¦ |
|  | n78 | 3305 | 10 | 52 | 3305 | N/A | TDD | N/A |
| DC_1A-7C_n78A | 1 | 1950 | 5 | 25 | 2140 | 8.7 | FDD | IMD4 ¦2*fB78-2*fB7¦ |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |

## 5.1.10	DC_3-8_n77

- 5.1.10.1	Operating bands for DC_3-8_n77

Table 5.1.10.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n77 | CA_3-8 | n77 | DC_3_n77 |

- 5.1.10.2	Configurations for DC_3-8_n77

Table 5.1.10.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n77A | DC_3A_n77A DC_8A_n77A | CA_3A-8A | n77A |

- 5.1.10.3	∆TIB and ∆RIB values

Based on the study results of lower order combinations, the following values are proposed:

Table 5.1.10.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n77 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.10.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n77 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |

- 5.1.10.4	REFSENS requirements

For this EN-DC, REFSENS exceptions due to IMD3 and IMD4 are specified as below:

Table 5.1.10.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A | No |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | FDD | IMD4 |  |
| DC_3A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A | No |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | FDD | IMD3 |  |

## 5.1.11	DC_3-8_n79

- 5.1.11.1	Operating bands for DC_3-8_n79

Table 5.1.11.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n79 | CA_3-8 | n79 | - |

- 5.1.11.2	Configurations for DC_3-8_n79

Table 5.1.11.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n79A | DC_3A_n79A DC_8A_n79A | CA_3A-8A | n79A |

- 5.1.11.3	∆TIB and ∆RIB values

Based on the study results of lower order combinations, the following values are proposed:

Table 5.1.11.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n79 | 3 | 0.3 |
|  | 8 | 0.3 |
|  | n79 | 0 |

Table 5.1.11.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n79 | 3 | 0 |
|  | 8 | 0 |
|  | n79 | 0 |

- 5.1.11.4	REFSENS requirements

For this EN-DC, REFSENS exceptions due to IMD3 and IMD4 are specified as below:

Table 5.1.11.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | FDD | N/A | No |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | TDD | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | FDD | IMD3 |  |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A | No |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | TDD | N/A |  |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | FDD | IMD4 |  |

## 5.1.12	DC_28-41_n77

- 5.1.12.1	Operating bands for DC_28-41_n77

Table 5.1.12.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28-41_n77 | CA_28-41 | n77 | No |

- 5.1.12.2	Configurations for DC_28-41_n77

Table 5.1.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_28A-41A_n77A | DC_28A_n77A DC_41A_n77A | CA_28A-41A | n77A |

- 5.1.12.3	∆TIB and ∆RIB values

Based on the study results of lower order combinations, the following values are proposed:

Table 5.1.12.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n77 | 28 | 0.5 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.12.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n77 | 28 | 0.2 |
|  | 41 | 0 |
|  | n77 | 0.5 |

- 5.1.12.4	REFSENS requirements

For this EN-DC, REFSENS exceptions due to two IMD2 scenarios are specified as below:

Table 5.1.12.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A | Yes |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |  |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |  |
| DC_28A-41A_n77A | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A | Yes |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |  |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |  |

## 5.1.13	DC_28-41_n78

- 5.1.13.1	Operating bands for DC_28-41_n78

Table 5.1.13.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28-41_n78 | CA_28-41 | n78 | No |

- 5.1.13.2	Configurations for DC_28-41_n78

Table 5.1.13.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_28A-41A_n78A | DC_28A_n78A DC_41A_n78A | CA_28A-41A | n78A |

- 5.1.13.3	∆TIB and ∆RIB values

Based on the study results of lower order combinations, the following values are proposed:

Table 5.1.13.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n78 | 28 | 0.5 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.13.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n78 | 28 | 0.2 |
|  | 41 | 0 |
|  | n78 | 0.5 |

- 5.1.13.4	REFSENS requirements

For this EN-DC, REFSENS exceptions due to two IMD2 scenarios are specified as below:

Table 5.1.13.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A | Yes |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |  |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |  |
| DC_28A-41A_n78A | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A | Yes |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |  |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |  |

## 5.1.14	DC_28-41_n79

- 5.1.14.1	Operating bands for DC_28-41_n79

Table 5.1.14.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28-41_n79 | CA_28-41 | n79 | No |

- 5.1.14.2	Configurations for DC_28-41_n79

Table 5.1.14.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_28A-41A_n79A | DC_28A_n79A DC_41A_n79A | CA_28A-41A | n79A |

- 5.1.14.3	∆TIB and ∆RIB values

Based on the study results of lower order combinations, the following values are proposed:

Table 5.1.14.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n79 | 28 | 0.3 |
|  | 41 | 0.3 |
|  | n79 | 0.8 |

Table 5.1.14.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n79 | 28 | 0 |
|  | 41 | 0 |
|  | n79 | 0.5 |

- 5.1.14.4	REFSENS requirements

For this EN-DC, REFSENS exceptions due to two IMD3 and IMD4 scenarios are specified as below:

Table 5.1.14.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A | No |
|  | n79 | 4739 | 40 | 216 | 4739 | N/A | TDD | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 8.6 | TDD | IMD4 |  |
| DC_28A-41A_n79A | 41 | 2650 | 5 | 25 | 2650 | N/A | TDD | N/A | No |
|  | n79 | 4502 | 40 | 216 | 4502 | N/A | TDD | N/A |  |
|  | 28 | 743 | 5 | 25 | 798 | 15.9 | FDD | IMD3 |  |

## 5.1.15	DC_1-8_n77

- 5.1.15.1	Operating bands for DC_1-8_n77

Table 5.1.15.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n77 | CA_1-8 | n77 | DC_1_n77 |

- 5.1.15.2	Configurations for DC_1-8_n77

Table 5.1.15.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n77A | DC_1A_n77A DC_8A_n77A | CA_1A-8A | n77A |

- 5.1.15.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n77A, (1) IMD5 of (3*B1 -2* n77) will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8A_n77A, (2) IMD3 of (2*B8 - n77) will fall into Rx band of Band 1.

- 5.1.15.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.15.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n77 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.15.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n77 | 1 | 0 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |

- 5.1.15.5	REFSENS requirements

As mentioned above, IMD5 of B1 and n77 to Band 8 Rx and IMD3 of B8 and n77 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.15.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A | No |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 3.3 | FDD | IMD5 |  |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A | No |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 14.4 | FDD | IMD3 |  |

## 5.1.16	DC_1-8_n79

- 5.1.16.1	Operating bands for DC_1-8_n79

Table 5.1.16.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n79 | CA_1-8 | n79 | no |

- 5.1.16.2	Configurations for DC_1-8_n79

Table 5.1.16.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n79A | DC_1A_n79A DC_8A_n79A | CA_1A-8A | n79A |

- 5.1.16.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n79A, IMD3 of (2*B1 - n79) and IMD4 of (3*B1 - n79) will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8A_n79A, IMD4 of (3*B8 - n79) will fall into Rx band of Band 1.

- 5.1.16.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.16.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n79 | 0 |

Table 5.1.16.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n79 | 1 | 0 |
|  | 8 | 0 |
|  | n79 | 0 |

- 5.1.16.5	REFSENS requirements

As mentioned above, IMD3 of B1 and n79 to Band 8 Rx and IMD4 of B8 and n79 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.16.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A | No |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | TDD | N/A |  |
|  | 8 | 900 | 5 | 25 | 945 | 15.8 | FDD | IMD3 |  |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A | No |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | TDD | N/A |  |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.2 | FDD | IMD4 |  |

## 5.1.17	DC_3-3-7_n78, DC_3-3-7-7_n78

- 5.1.17.1	Operating bands for DC

Table 5.1.17.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-3-7_n782 | CA_3-3-7 | n78 | DC_3_n78 |
| DC_3-3-7-7_n782 | CA_3-3-7-7 | n78 | DC_3_n78 |
| NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.1.17.2	Configuration for DC

Table 5.1.17.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-3A-7A_n78A | DC_3A_n78A, DC_7A_n78A | CA_3A-3A-7A | n78A |
| DC_3A-3A-7A-7A_n78A | DC_3A_n78A, DC_7A_n78A | CA_3A-3A-7A-7A | n78A |

- 5.1.17.3	∆TIB and ∆RIB values

For DC_3-3-7_n78, DC_3-3-7-7_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.17.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n78, DC_3-3-7-7_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.17.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3-7_n78, DC_3-3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

- 5.1.17.4	REFSENS requirements

When uplink DC configurations DC_7A_78A is paired with downlink DC configuration DC_3A-3A-7A_n78A or DC_3A-3A-7A-7A_n78A, the 3rd IMD and 4th IMD from 2 uplink operation would interfere the downlink of the Band 3.

However, these impacts are already covered in the fallback combination, DC_3A-7A_n78A, no further MSD analysis is needed.

## 5.1.18	DC_18-42_n77

- 5.1.18.1	Operating bands for DC

Table 5.1.18.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-42_n77 | CA_18-42 | n77 | No |

- 5.1.18.2	Configurations for DC

Table 5.1.18.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-42A_n77A | DC_18A_n77A | CA_18A-42A | n77 |
| DC_18A-42C_n77A | DC_18A_n77A | CA_18A-42C | n77 |

- 5.1.18.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

- LTE Band 42 and NR Band n77 are synchronized.

- Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported

- 5.1.18.4	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_19-42_n77 can be applied.

Table 5.1.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-42_n77 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18-42_n77 | 18 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

- 5.1.18.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.19	DC_18-42_n78

- 5.1.19.1	Operating bands for DC

Table 5.1.19.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-42_n78 | CA_18-42 | n78 | No |

- 5.1.19.2	Configurations for DC

Table 5.1.19.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-42A_n78A | DC_18A_n78A | CA_18A-42A | n78 |
| DC_18A-42C_n78A | DC_18A_n78A | CA_18A-42C | n78 |

- 5.1.19.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

- LTE Band 42 and NR Band n78 are synchronized.

- Simultaneous transmission of LTE Band 42 and NR Band n78 are not supported

- 5.1.19.4	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_19-42_n78 can be applied.

Table 5.1.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-42_n78 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 5.1.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18-42_n78 | 18 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

- 5.1.19.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.20	DC_18-42_n79

- 5.1.20.1	Operating bands for DC

Table 5.1.20.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-42_n79 | CA_18-42 | n79 | No |

- 5.1.20.2	Configurations for DC

Table 5.1.20.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-42A_n79A | DC_18A_n79A | CA_18A-42A | n79 |
| DC_18A-42C_n79A | DC_18A_n79A | CA_18A-42C | n79 |

- 5.1.20.3	Co-existence studies

Based on co-existence studies of Band 18 + Band n79 and Band 42 + Band n79 captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings..

- 2nd order IMD generated by dual uplink of Band 18 + Band n79 may also fall into own Rx of band 42. However, since Band 18 is only used in Japan and considering actual spectrum holdings, no IMD issue is expected.

- 2nd and 5th order IMD generated by dual uplink of Band 42 + Band n79 may also fall into own Rx of band 18.

As approved in R4-1814167, UL of Band 42 is not used in this configuration. So there is no own Rx impact of 3rd band for this configuration.

- 5.1.20.4	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_19-42_n79 can be applied.

Table 5.1.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-42_n79 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |

Table 5.1.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18-42_n79 | 18 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |

- 5.1.20.5	REFSENS requirements

There is no additional REFSENS requirement for this configuration.

## 5.1.21	DC_8-20_n78

- 5.1.21.1	Operating bands for DC_8-20_n78

Table 5.1.21.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-20_n782 | CA_8-20 | n78 | No |
| NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.1.21.2	Configurations for DC_8-20_n78

Table 5.1.21.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A | CA_8A-20A | n78 |

- 5.1.21.3	Co-existence Studies

Based on co-existence studies of DC_8_n78 and DC_20_n78, as captured in TR 37.863-01-01, own Rx impact on the 3rd band are the following:

- 4th order IM generated by dual uplink of Band 8 and n78 may fall into Band 20

- 4th order IM generated by dual uplink of Band 20 and n78 may fall into Band 8

- 5.1.21.4	∆TIB and ∆RIB values

Table 5.1.21.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-20_n78 | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.21.4-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8-20_n78 | 8 | 0.2 |
|  | n78 | 0.5 |

- 5.1.21.5	REFSENS requirements

Table 5.1.21.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 | 935 | N/A | FDD | N/A |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | TDD | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.1 | FDD | IMD4 |
|  | 8 | 895 | 5 | 25 | 940 | 12.1 | FDD | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | TDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | FDD | N/A |

## 5.1.22	DC_20-38_n78

- 5.1.22.1	Operating bands for DC_20-38_n78

Table 5.1.22.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-38_n78 | CA_20-38 | n78 | No |

- 5.1.22.2	Configurations for DC_20-38_n78

Table 5.1.22.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A | CA_20A-38A | n78A |

- 5.1.22.3	Co-existence Studies

Based on co-existence studies of DC_20_n78, as captured in TR 37.863-01-01, own Rx impact on the 3rd band are the following:

- 2nd order IM generated by dual uplink of Band 20 and n78 may fall into Band 38

- 3rd order harmonic generated by Band 20 may fall into Band 38

- 5.1.22.4	∆TIB and ∆RIB values

Table 5.1.22.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n78 | 20 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.22.4-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |

- 5.1.22.5	REFSENS requirements

Because B20 and B38 belong to LTE band. The reference sensitivity refers to TS 36.101 in Table 7.3.1A-0a (exceptions due to harmonic issue CA_20A-38A). There is no change.

## 5.1.23	DC_66-(n)71

- 5.1.23.1	Operating bands for EN-DC

Table 5.1.23.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66-(n)71 | CA_66-71 | n71 | No1 |
| NOTE 1: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. |  |  |  |

- 5.1.23.2	Configurations for EN-DC

Table 5.1.23.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A-(n)71AA | DC_66A_n71A | CA_66A-71A | n71A |

- 5.1.23.3	∆TIB and ∆RIB values

∆TIB values for DC_66_n71 are already specified in TS 38.101-3 and ∆RIB are zero.

- 5.1.23.4	REFSENS requirements

No REFSENS exceptions needed for DC_66A-(n)71AA. Impact of contiguous intraband EN-DC operation on band 71 on REFSENS is already specified in Table 7.3B.2.1-1 of TS 38.101-3.

## 5.1.24	DC_2-(n)71

- 5.1.24.1	Operating bands for EN-DC

Table 5.1.24.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-(n)71 | CA_2-71 | n71 | No1 |
| NOTE 1: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. |  |  |  |

- 5.1.24.2	Configurations for EN-DC

Table 5.1.24.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-(n)71AA | DC_2A_n71A | CA_2A-71A | n71A |

- 5.1.24.3	∆TIB and ∆RIB values

∆TIB values for DC_2_n71 are already specified in TS 38.101-3 and ∆RIB are zero.

- 5.1.24.4	REFSENS requirements

REFSENS exceptions needed for DC_2A-(n)71AA due to band 71 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3. Impact of contiguous intraband EN-DC operation on band 71 on REFSENS is already specified in Table 7.3B.2.1-1 of TS 38.101-3.

## 5.1.25	DC_7-8_n77

- 5.1.25.1	Operating bands for DC

Table 5.1.25.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-8_n77 | CA_7-8 | n77 | No |

- 5.1.25.2	Configuration for DC

Table 5.1.25.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-8A_n77A | DC_7A_n77A, DC_8A_n77A | CA_7A-8A | n77A |

- 5.1.25.3	Co-existence Studies

Based on co-existence studies of DC_7_n77 and DC_8_n77, as captured in TR 37.716-11-11 and TR 37.863-01-01, own Rx impact on the 3rd band are the following:

- 2nd and 5th order IM generated by dual uplink of Band 7 and n77 may fall into Band 8

- 2nd order IM generated by dual uplink of Band 8 and n77 may fall into Band 7

- 5.1.25.4	∆TIB and ∆RIB values

For DC_7-8_n77 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n77 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.25.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n77 | 7 | 0 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |

- 5.1.25.5	REFSENS requirements

Table below lists the MSD required for the dual connectivity configuration.

Table 5.1.25.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | N/A | FDD | IMD2 ¦fB77-fB7¦ |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | FDD |  |
|  | n77 | 3470 | 10 | 50 | 3470 | N/A | TDD |  |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | IMD5 ¦2*fB77-3fB7¦ |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | FDD |  |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD |  |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | 28 | FDD | IMD2 ¦fB77-fB8¦ |
|  | 8 | 895 | 5 | 25 | 940 | N/A | FDD |  |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD |  |

## 5.1.26	DC_7-8_n78, DC_7-7-8_n78

- 5.1.26.1	Operating bands for DC

Table 5.1.26.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-8_n78 | CA_7-8 | n78 | No |
| DC_7-7-8_n78 | CA_7-7-8 | n78 | No |

- 5.1.26.2	Configuration for DC

Table 5.1.26.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-8A_n78A | DC_7A_n78A, DC_8A_n78A | CA_7A-8A | n78A |
| DC_7A-7A-8A_n78A | DC_7A_n78A, DC_8A_n78A | CA_7A-8A | n78A |

- 5.1.26.3	Co-existence Studies

Based on co-existence studies of DC_7_n78 and DC_8_n78, as captured in TR 37.863-01-01, own Rx impact on the 3rd band are the following:

- 2nd and 5th order IM generated by dual uplink of Band 7 and n78 may fall into Band 8

- 2nd order IM generated by dual uplink of Band 8 and n78 may fall into Band 7

- 5.1.26.4	∆TIB and ∆RIB values

For DC_7-8_n78 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n78, DC_7-7-8_n78 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.26.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n78, DC_7-7-8_n78 | 7 | 0 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |

- 5.1.26.5	REFSENS requirements

Table below lists the MSD required for the dual connectivity configuration.

Table 5.1.26.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | FDD | IMD2 ¦fB78-fB7¦ |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | FDD |  |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | TDD |  |
| DC_7A-8A_n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | IMD5 ¦2*fB78-3fB7¦ |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | FDD |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD |  |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | 28 | FDD | IMD2 ¦fB78-fB8¦ |
|  | 8 | 895 | 5 | 25 | 940 | N/A | FDD |  |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | TDD |  |

## 5.1.27	DC_2-7_n78

- 5.1.27.1	Operating bands for DC

Table 5.1.27.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-7_n78 | CA_2-7 | n78 | DC_2_n78 |
| DC_2-7-7_n78 | CA_2-7 | n78 | DC_2_n78 |

- 5.1.27.2	Configurations for DC

Table 5.1.27.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-7A_n78A DC_2A-7C_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) | DC_2A_n78A DC_7A_n78A DC_7C_n78A | CA_2A-7A CA_2A-7C | n78A |
| DC_2A-7A-7A_n78A DC_2A-7A-7A_n78(2A) | DC_2A_n78A DC_7A_n78A | CA_2A-7A-7A | n78A |

- 5.1.27.3	∆TIB and ∆RIB values

For DC_2-7_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.27.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n78 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n78 | 0 |

Table 5.1.27.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n78 | 2 | 0 |
|  | 7 | 0 |
|  | n78 | 0 |

- 5.1.27.4	REFSENS requirements

For UE coexistence study of Band 7 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.27.4-1.

Table 5.1.27.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13900 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1460 | 2600 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 6200 | 6980 | 10630 | 12700 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 13300 | 14080 | 15700 | 17770 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 100 | 1110 | 4760 | 6400 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

For UE coexistence study of Band 2 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.27.4-2

Table 5.1.27.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13250 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2780 | 3900 | 10300 | 11420 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 3600 | 4340 | 11290 | 13350 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 10700 | 11440 | 15050 | 17110 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2050 | 870 | 6080 | 7700 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

Based on co-existence study as presented in the table 5.1.27.4-1 and 5.1.27.4-2, own Rx impact of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band 2

By adjusting the centre frequency of uplink of Band 7 + Band n78, the MSD requirement due to 4th order IMD for DC_2-7_n78 can be similar as the requirements of DC_3-7_n78 since the source of IMD is the same.

Table 5.1.27.4-3 lists the MSD required for DC_2-7_n78A.

Table 5.1.27.4-3: MSD for the DC configuration due to IMD issue (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 ¦2*fB78-2*fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3525 | 10 | 52 | 3475 | N/A | TDD | N/A |

## 5.1.28	DC_7-66_n78

- 5.1.28.1	Operating bands for DC

Table 5.1.28.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-66_n78 | CA_7-66 | n78 | No |
| DC_7-7-66_n78 | CA_7-7-66 | n78 | No |
| DC_7-66-66_n78 | CA_7-66-66 | n78 | No |
| DC_7-7-66-66_n78 | CA_7-7-66-66 | n78 | No |

- 5.1.28.2	Configurations for DC

Table 5.1.28.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) | DC_7A_n78A DC_7C_n78A DC_66A_n78A | CA_7A-66A CA_7C-66A | n78A CA_n78(2A) |
| DC_7A-7A-66A_n78A DC_7A-7A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A | CA_7A-7A-66A | n78A CA_n78(2A) |
| DC_7A-66A-66A_n78A DC_7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A | CA_7A-66A-66A | n78A CA_n78(2A) |
| DC_7A-7A-66A-66A_n78A DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A | CA_7A-7A-66A-66A | n78A CA_n78(2A) |
| DC_7C-66A-66A_n78A DC_7C-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A | CA_7C-66A-66A | n78A CA_n78(2A) |

- 5.1.28.3	∆TIB and ∆RIB values

For DC_7-66_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.28.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n78 DC_7-7-66_n78 DC_7-66-66_n78 DC_7-7-66-66_n78 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n78 | 0 |

Table 5.1.28.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n78 DC_7-7-66_n78 DC_7-66-66_n78 DC_7-7-66-66_n78 | 7 | 0 |
|  | 66 | 0 |
|  | n78 | 0 |

- 5.1.28.4	REFSENS requirements

For UE coexistence study of Band 7 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.28.4-1.

Table 5.1.28.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13900 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1460 | 2600 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 6200 | 6980 | 10630 | 12700 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 13300 | 14080 | 15700 | 17770 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 100 | 1110 | 4760 | 6400 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

For UE coexistence study of Band 66 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.28.4-2

Table 5.1.28.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2090 | 5010 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 380 | -260 | 4820 | 5890 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7360 | 8310 | 9380 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -1330 | -2040 | 8120 | 9690 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9140 | 11610 | 13110 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3040 | 4180 | 10020 | 11160 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | -3040 | -3820 | 11420 | 13490 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 10140 | 10920 | 14910 | 16980 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2470 | 1260 | 6340 | 7980 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13320 | 14960 | 11730 | 12940 |

Based on co-existence study as presented in the table 5.1.28.4-1 and 5.1.28.4-2, own Rx impact of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band 2

By adjusting the centre frequency of uplink of Band 7 + Band n78, the MSD requirement due to 4th order IMD for DC_7-66_n78 can be similar as the requirements of DC_1-7_n78 since the source of IMD is the same.

Table 5.1.28.4-3 lists the MSD required for DC_7-66_n78.

Table 5.1.28.4-3: MSD for the DC configuration due to IMD issue (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 8.7 | FDD | IMD4 ¦2*fB78-2*fB7¦ |
|  | n78 | 3625 | 10 | 52 | 3475 | N/A | TDD | N/A |

## 5.1.29	DC_3-8_n78

- 5.1.29.1	Operating bands for DC

Table 5.1.29.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C-8A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

- 5.1.29.2	Channel bandwidths per operating band for DC

Table 5.1.29.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C-8A-n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  | 150 |
|  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

- 5.1.29.3	Co-existence studies

Co-existence was studied for DC_3A-8A_n78A in Rel-15 and the results are captured in TR 37.863-02-01. No further studies needed for DC_3C-8A_n78A.

- 5.1.29.4	∆TIB and ∆RIB values

For DC_3C-8A-n78A, the TIB,c and RIB values are given in the tables below.

Table 5.1.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3C-8A-n78A | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.29.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3C-8A-n78A | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |

- 5.1.29.5	REFSENS

According the co-existing studies and REFSENS study in TR 37.863-02-01, although 2nd harmonic generated by dual uplink of Band 8 + Band n78 may also fall into own Rx of band 3, there is significant opportunity to avoid the harmonic interference from the transmitter into the receiver. Therefore, it is no need to specific additional MSD for Band 3 caused by 2nd order harmonic of Band 8 for DC_3C-8A_n78A.

## 5.1.30	DC_3-28_n78

- 5.1.30.1	Operating bands for DC

Table 5.1.30.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-28_n782 | CA_3-28 | n78 | DC_3_n78 |
| DC_3-3-28_n782 | CA_3-3-28 | n78 | DC_3_n78 |
| NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.1.30.2	Configuration for DC

Table 5.1.30.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-3A-28A_n78A DC_3C-28A_n78A | DC_3A_n78A DC_28A_n78A | CA_3A-3A-28A CA_3C-28A | n78A |

- 5.1.30.3	∆TIB and ∆RIB values

For DC_3-28_n78 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.30.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.30.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n78 | 3 | 0.2 |
|  | n78 | 0.5 |

- 5.1.30.4	REFSENS requirements

The impacts are covered in the fallback combination DC_3A-28A_n78A and will also apply for DC_3C-28A_n78A and need to be included in Table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 5.1.30.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-3A-28A_n78A DC_3C-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | FDD | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | TDD | N/A |

## 5.1.31	DC_5-7_n71

- 5.1.31.1	Operating bands for DC

Table 5.1.31.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-7_n71 | CA_5-7 | n71 | No |

- 5.1.31.2	Configurations for DC

Table 5.1.31.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-7A_n71A | DC_5A_n71A DC_7A_n71A | CA_5A-7A | n71A |

- 5.1.31.3	∆TIB and ∆RIB values

For DC_2-7_n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.31.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7_n71 | 5 | 0.5 |
|  | 7 | 0.3 |
|  | n71 | 0.6 |

Table 5.1.31.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7_n71 | 5 | 0 |
|  | 7 | 0 |
|  | n71 | 0.2 |

- 5.1.31.4	REFSENS requirements

For UE coexistence study of Band 7 + Band n71, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.31.4-1.

Table 5.1.31.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1802 | 1907 | 3163 | 3268 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1244 | 1104 | 4302 | 4477 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3826 | 3966 | 5663 | 5838 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 581 | 406 | 6802 | 7047 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4489 | 4664 | 8163 | 8373 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3604 | 3814 | 6326 | 6536 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 82 | 292 | 9302 | 9617 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5152 | 5362 | 10663 | 10978 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3151 | 2906 | 6104 | 6384 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 8826 | 9106 | 6989 | 7234 |

For UE coexistence study of Band 5 + Band n71, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.31.4-2

Table 5.1.31.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 4120 | 4245 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 126 | 186 | 1487 | 1547 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 477 | 572 | 950 | 1035 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 2150 | 2245 | 2311 | 2396 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1140 | 1270 | 1774 | 1884 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 2813 | 2943 | 3135 | 3210 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 252 | 372 | 2974 | 3094 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1803 | 1968 | 2598 | 2733 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 3476 | 3641 | 3959 | 4094 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | -291 | -446 | 1076 | 1221 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 3798 | 3943 | 3637 | 3792 |

Based on co-existence study as presented in the table 5.1.31.4-1 and 5.1.31.4-2, own Rx impact of the 3rd band is shown as the followings.

- 5th order IMD generated by dual uplink of Band 5 + Band n71 may fall into own Rx of band 7

Table 5.1.31.4-3 lists the MSD required for DC_5-7_n71A.

Table 5.1.31.4-3: MSD for the DC configuration due to IMD issue (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 | 880 | N/A | FDD |  |
|  | 7 | 2540 | 5 | 25 | 2660 | 6.5 | FDD | IMD5 ¦4*fB5-fBn71¦ |
|  | n71 | 680 | 5 | 52 | 634 | N/A | TDD | N/A |

## 5.1.32	DC_2-7_n71

- 5.1.32.1	Operating bands for DC

Table 5.1.32.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-7_n71 | CA_2-7 | n71 | No |

- 5.1.32.2	Configurations for DC

Table 5.1.32.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A | CA_2A-7A | n71A |

- 5.1.32.3	∆TIB and ∆RIB values

For DC_2-7_n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.32.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n71 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n71 | 0.6 |

Table 5.1.32.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n71 | 2 | 0 |
|  | 7 | 0 |
|  | n71 | 0.2 |

- 5.1.32.4	REFSENS requirements

For UE coexistence study of Band 7 + Band n71, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.32.4-1.

Table 5.1.32.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1802 | 1907 | 3163 | 3268 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1244 | 1104 | 4302 | 4477 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3826 | 3966 | 5663 | 5838 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 581 | 406 | 6802 | 7047 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4489 | 4664 | 8163 | 8373 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3604 | 3814 | 6326 | 6536 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 82 | 292 | 9302 | 9617 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5152 | 5362 | 10663 | 10978 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3151 | 2906 | 6104 | 6384 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 8826 | 9106 | 6989 | 7234 |

For UE coexistence study of Band 2 + Band n71, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.32.4-2

Table 5.1.32.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1152 | 1247 | 2513 | 2608 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 584 | 454 | 3002 | 3157 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3176 | 3306 | 4363 | 4518 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 79 | 244 | 4852 | 5067 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 3839 | 4004 | 6213 | 6393 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2304 | 2494 | 5026 | 5216 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 742 | 942 | 6702 | 6977 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 4502 | 4702 | 8063 | 8338 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 1831 | 1606 | 4154 | 4404 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 6876 | 7126 | 5689 | 5914 |

Based on co-existence study as presented in the table 5.1.32.4-1 and 5.1.32.4-2, there is no MSD issue on the own Rx impact of the 3rd band.

## 5.1.33	DC_2-66_n78

- 5.1.33.1	Operating bands for DC

Table 5.1.33.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n78 | CA_2-66 | n78 | No |
| DC_2-66-66_n78 | CA_2-66-66 | n78 | No |

- 5.1.33.2	Configurations for DC

Table 5.1.33.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A | CA_2A-66A | n78A |
| DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A | CA_2A-66A-66A | n78A |

- 5.1.33.3	∆TIB and ∆RIB values

For DC_2-66_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.33.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n78 DC_2-66-66_n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.33.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n78 DC_2-66-66_n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |

- 5.1.33.4	REFSENS requirements

For UE coexistence study of Band 2 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.33.4-1.

Table 5.1.33.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 3800 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 7600 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 11400 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 15200 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 19000 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1950 | 1390 | 5150 | 5710 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 4690 | 5750 | 100 | 520 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8450 | 9510 | 7000 | 7620 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 7990 | 9550 | 1750 | 2430 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 11750 | 13310 | 8850 | 9030 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3900 | 2780 | 10300 | 11420 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 11290 | 13350 | 3600 | 4340 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6080 | 7700 | 2050 | 870 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 12150 | 13330 | 13600 | 15220 |

For UE coexistence study of Band 66 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.33.4-2

Table 5.1.33.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2090 | 5010 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 380 | 260 | 4820 | 5890 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7360 | 8310 | 9380 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1330 | 2040 | 8120 | 9690 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9140 | 11610 | 13110 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3040 | 4180 | 10020 | 11160 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 3040 | 3820 | 11420 | 13490 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 10140 | 10920 | 14910 | 16980 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 2470 | 1260 | 6340 | 7980 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 13320 | 14960 | 11730 | 12940 |

Based on co-existence study as presented in the table 5.1.33.4-1 and 5.1.33.4-2, own Rx impact of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 2 + Band n78 may fall into own Rx of band 66

- 2nd order IMD generated by dual uplink of Band 66 + Band n78 may fall into own Rx of band 2

- 4th order IMD generated by dual uplink of Band 66 + Band n78 may fall into own Rx of band 2

- 5th order IMD generated by dual uplink of Band 66 + Band n78 may fall into own Rx of band 2

Table5.1.33.4-3 propose the required MSD levels for the DC configuration.

Table 5.1.33.4-3: Proposed MSD requirements

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | [10.3] | FDD | IMD4 ¦3*fB2-fBn78¦ |
|  | n78 | 3480 | 10 | 52 | 3480 | N/A | TDD | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | [32.1] | FDD | IMD2 ¦fBn78-fB66¦ |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | N/A | TDD | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | [9.1] | FDD | IMD4 ¦3*fB66-fBn78¦ |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n78 | 3350 | 10 | 52 | 3350 | N/A | TDD | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | [2.1] | FDD | IMD5 ¦2*fBn78-3*fB66¦ |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 52 | 3620 | N/A | TDD | N/A |

## 5.1.34	DC_1-3_n7

- 5.1.34.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3_n7 | CA_1-3 | n7 | No |

- 5.1.34.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n7A DC_1A-3C_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A | CA_1A-3A CA_1A-3C | n7 |

- 5.1.34.3		∆TIB and ∆RIB values

For DC_1-3_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-7, and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n7 | 0.6 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n7 | 1 | 0 |
|  | 3 | 0 |
|  | n7 | 0 |

## 5.1.35	DC_3-28_n41

- 5.1.35.1	Operating bands for DC

Table 5.1.35.1-1: band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3A-28A_n41A | CA_3-28 | n41 |  |

- 5.1.35.2	Configuration for DC

Table 5.1.35.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-28A_n41A | DC_3A_n41 DC_28A_n41 | CA_3A-28A | n41A |

- 5.1.35.3	∆TIB and ∆RIB values

For DC_3A-28A_n41A, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.35.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n41 | 3 | 0.5 |
|  | 28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.1.35.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n41 | 3 | 0 |
|  | 28 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

- 5.1.35.4	REFSENS requirements

Based on co-existence studies of Band 3 + Band 28 and Band 41 captured in TR 36.101, own Rx impact of the 3rd band is IMD2 into own Rx of band 41.

Table 5.1.35.6-1: 3DL/2UL inter band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| DC_3A-28A_n41A | DC_3A_n41A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 5 | N/A | TDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | 26.0 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

## 5.1.36	DC_1-11_n78

- 5.1.36.1	Operating bands for DC_1-11_n78

Table 5.1.36.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n78 | CA_1-11 | n78 |  |

- 5.1.36.2	Configurations for DC_1-11_n78

Table 5.1.36.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n78A | DC_1A_n78A DC_11A_n78A | CA_1A-11A | n78A |

- 5.1.36.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n78A, (1) IMD2 of (B1 - n78) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n78A, (2) IMD2 of (B11 - n78) will fall into Rx band of Band 1.

- 5.1.36.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.36.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n78 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | n78 | 0.8 |

Table 5.1.36.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n78 | 1 | 0 |
|  | 11 | 0 |
|  | n78 | 0.5 |

- 5.1.36.5	REFSENS requirements

As mentioned above, IMD2 of B1 and n78 to Band 11 Rx and IMD2 of B11 and n78 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.36.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n78A | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A |  |
|  | n78 | 3441 | 10 | 50 | 3441 | N/A | TDD | N/A |  |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | FDD | IMD2 |  |
| DC_1A-11A_n78A | 11 | 1438 | 5 | 25 | 1486 | N/A | FDD | N/A |  |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | FDD | IMD2 |  |

## 5.1.37	DC_8-11_n77

- 5.1.37.1	Operating bands for DC_8-11_n77

Table 5.1.37.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-11_n77 | CA_8-11 | n77 |  |

- 5.1.37.2	Configurations for DC_8-11_n77

Table 5.1.37.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-11A_n77A | DC_8A_n77A DC_11A_n77A | CA_8A-11A | n77A |

- 5.1.37.3	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n77A, (1) IMD3 of (2*B8 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD4 of (3*B11 - n77) will fall into Rx band of Band 8.

- 5.1.37.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.37.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n77 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |

Table 5.1.37.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n77 | 8 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |

- 5.1.37.5	REFSENS requirements

As mentioned above, IMD3 of B8 and n77 to Band 11 Rx and IMD3 of B11 and n77 to Band 8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.37.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |  |
|  | n77 | 3311 | 10 | 50 | 3311 | N/A | TDD | N/A |  |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | FDD | IMD3 |  |
| DC_8A-11A_n77A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | FDD | N/A |  |
|  | n77 | 3791 | 10 | 50 | 3791 | N/A | TDD | N/A |  |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | FDD | IMD3 |  |

## 5.1.38	DC_8-11_n78

- 5.1.38.1	Operating bands for DC_8-11_n78

Table 5.1.38.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-11_n78 | CA_8-11 | n78 |  |

- 5.1.38.2	Configurations for DC_8-11_n78

Table 5.1.38.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-11A_n78A | DC_8A_n78A DC_11A_n78A | CA_8A-11A | n78A |

- 5.1.38.3	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n78A, (1) IMD3 of (2*B11 - n78) will fall into Rx band of Band 8.

- 5.1.38.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.38.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n78 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n78 | 0.8 |

Table 5.1.38.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n78 | 8 | 0.2 |
|  | 11 | 0 |
|  | n78 | 0.2 |

- 5.1.38.5	REFSENS requirements

As mentioned above, IMD3 of B11 and n78 to Band 8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.38.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |  |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | TDD | N/A |  |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | FDD | IMD3 |  |
| DC_8A-11A_n78A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | FDD | N/A |  |
|  | n78 | 3791 | 10 | 50 | 3791 | N/A | TDD | N/A |  |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | FDD | IMD3 |  |

## 5.1.39	DC_2-30_n5 and DC_2-2-30_n5

- 5.1.39.1	Operating bands for DC

Table 5.1.39.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-30_n5 | CA_2-30 | n5 | No |

- 5.1.39.2	Configuration for DC

Table 5.1.39.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A | CA_2A-30A | n5A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A | CA_2A-2A-30A | n5A |

- 5.1.39.3	Coexistence studies

Based on co-existence studies of DC_2A_n5A and DC_30A_n5A in TR 37.863-01-01, there is no harmonic or IMD impact to any of three own Rx bands.

- 5.1.39.4	∆TIB and ∆RIB values

For DC_2-30_n5 and DC_2-2-30_n5, the TIB,c and RIB,c values are derived from CA_2-5-30 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n5 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n5 | 0 |

- 5.1.39.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.40	DC_30-66_n5, DC_30-66-66_n5 and DC_30-66-66-66_n5

- 5.1.40.1	Operating bands for DC

Table 5.1.40.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_30-66_n5 | CA_30-66 | n5 | No |

- 5.1.40.2	Configuration for DC

Table 6.1.X.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A | CA_30A-66A | n5A |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A | CA_30A-66A-66A | n5A |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A | CA_30A-66A-66A-66A | n5A |

- 5.1.40.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the 2nd and 5th IMD of 66+n5 may fall into the band n5 downlink and Pcell REFSENS excpetions can be represented by the REFSENS expcetion already specified for DC_66A-n5A.

Based on co-existence studies of DC_30A_n5A in TR 37.863-01-01, 5th IMD product of 30+n5 may fall into band 66 downlink.

- 5.1.40.4	∆TIB and ∆RIB values

For DC_30-66_n5, DC_30-66-66_n5 and DC_30-66-66-66_n5, the TIB,c and RIB,c values are derived from CA_5-30-66 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 | 30 | 0 |
|  | 66 | 0.4 |
|  | n5 | 0.5 |

- 5.1.40.5	Refsens requirements

Pcell REFSENS exceptions for band n5 due to 2 uplink of band 66 and band n5 can be represented by the REFSENS expcetion already specified for DC_66A-n5A in TS 38.101-3.

Scell REFSENS exceptions for band 66 due to 2 uplink of band 30 and band n5, MSD is specified in Table 5.1.40.5-1

Table 5.1.40.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_30A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | [2.5 dB] | FDD | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |

## 5.1.41	DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1

- 5.1.41.1	Operating bands for DC

Table 5.1.41.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n1 | CA_3-7 | n1 | DC_3_n1 |
| DC_3-3-7_n1 | CA_3-3-7 | n1 | DC_3_n1 |
| DC_3-7-7_n1 | CA_3-7-7 | n1 | DC_3_n1 |
| DC_3-3-7-7_n1 | CA_3-3-7-7 | n1 | DC_3_n1 |

- 5.1.41.2	Configuration for DC

Table 5.1.41.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A_n1A | DC_3A_n1A, DC_7A_n1A | CA_3A-7A | n1A |
| DC_3A-7C_n1A | DC_3A_n1A, DC_7A_n1A, DC_7C_n1A | CA_3A-7C | n1A |
| DC_3C-7A_n1A | DC_3A_n1A, DC_3C_n1A, DC_7A_n1A | CA_3C-7A | n1A |
| DC_3C-7C_n1A | DC_3A_n1A, DC_3C_n1A, DC_7A_n1A, DC_7C_n1A | CA_3C-7C | n1A |
| DC_3A-3A-7A_n1A | DC_3A_n1A, DC_7A_n1A | CA_3A-3A-7A | n1A |
| DC_3A-7A-7A_n1A | DC_3A_n1A, DC_7A_n1A | CA_3A-7A-7A | n1A |
| DC_3A-3A-7A-7A_n1A | DC_3A_n1A, DC_7A_n1A | CA_3A-3A-7A-7A | n1A |

- 5.1.41.3	Co-existence Studies

Based on co-existence studies of DC_3_n1 and DC_7_n1, as captured in TR 37.716-11-11, no own Rx impact on the 3rd band is observed.

- 5.1.41.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 5.1.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1 | 3 | 0.3 |
|  | 7 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.41.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1 | 3 | 0 |
|  | 7 | 0 |
|  | n1 | 0 |

- 5.1.41.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.42	DC_1-3_n5

- 5.1.42.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n5 | CA_1-3 | n5 | DC_3_n5 |

- 5.1.42.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A | CA_1A-3A CA_1A-3C | n5 |

- 5.1.42.3	Co-existence Studies

Co-existence analysis from DC_3_n5 shows that there is no impact from DC_3_n5 UL to Band 1 DL.

Co-existence analysis from DC_1_n5 shows that there is no impact from DC_1_n5 UL to Band 3 DL.

- 5.1.42.4		∆TIB and ∆RIB values

For DC_1-3_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-5, and are given in the tables below.

Table 5.1.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n5 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n5 | 1 | 0 |
|  | 3 | 0 |
|  | n5 | 0 |

- 5.1.42.5	REFSENS requirements

MSD does not need to be defined.

## 5.1.43	DC_1-7_n5

- 5.1.43.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n5 | CA_1-7 | n5 | DC_7_n5 |

- 5.1.43.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A | CA_1A-7A CA_1A-7C | n5 |

- 5.1.43.3	Co-existence Studies

Co-existence analysis from DC_7_n5 shows that there is no impact from DC_7_n5 UL to Band 1 DL.

Co-existence analysis from DC_1_n5 shows that there is no impact from DC_1_n5 UL to Band 7 DL.

- 5.1.43.4		∆TIB and ∆RIB values

For DC_1-7_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_1-5-7, and are given in the tables below.

Table 5.1.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n5 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n5 | 0.3 |

Table 5.1.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n5 | 1 | 0 |
|  | 7 | 0 |
|  | n5 | 0 |

- 5.1.43.5	REFSENS requirements

MSD does not need to be defined.

## 5.1.44	DC_1-28_n5

- 5.1.44.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-28_n53 | CA_1-28 | n5 | No |
| NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

- 5.1.44.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-28A_n5A4 | DC_1A_n5A DC_28A_n5A | CA_1A-28A | n5 |
| NOTE 4: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

- 5.1.44.3	Co-existence Studies

In these analyses the frequency range in band n28 is restricted to 703-733 MHz for the UL and 758-788 MHz for the DL

Co-existence for DC_28A_n5A UL into band 1 DL is investigated in Table 5.1.44.3-1.

Table 5.1.44.3-1: Harmonic and IMD analysis DC_28A_n5A UL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 | 824 | 849 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1466 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2199 | 2472 | 2547 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2932 | 3296 | 3396 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3665 | 4120 | 4245 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4218 | 4398 | 4944 | 5094 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4921 | 5131 | 5768 | 5943 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 91 | 1527 | 1582 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 642 | 915 | 995 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2315 | 2351 | 2431 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 182 | 3054 | 3164 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1375 | 1739 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3048 | 3175 | 3280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2563 | 2108 | 1963 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4129 | 3636 | 3781 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 1006 | 551 | 411 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4013 | 3757 | 3897 |

Co-existence analysis in Table 5.1.44.3-1 shows that there is no impact from DC_28_n5 UL to Band 1 DL.

Co-existence for DC_1A_n5A UL into band 28 DL is investigated in Table 5.1.44.3-2.

Table 5.1.44.3-2: Harmonic and IMD analysis DC_1A_n5A UL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9600 | 9900 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 11520 | 11880 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 13440 | 13860 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1156 | 1071 | 2744 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 332 | 222 | 2991 | 3136 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3568 | 3678 | 4664 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2312 | 2142 | 5488 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 492 | 627 | 4911 | 5116 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4392 | 4527 | 6584 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7096 | 6831 | 1476 | 1316 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8504 | 8769 | 5216 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4292 | 4062 | 1293 | 1488 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7408 | 7638 | 6312 | 6507 |

Co-existence analysis in Table 5.1.44.3-2 shows that there is no impact from DC_1_n5 UL to Band 28 DL.

- 5.1.44.4		∆TIB and ∆RIB values

For DC_1-28_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_1-18-28, and are given in the tables below.

Table 5.1.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n5 | 1 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |

Table 5.1.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n5 | 1 | 0 |
|  | 28 | 0 |
|  | n5 | 0 |

- 5.1.44.5	REFSENS requirements

As shown in the co-existence analysis in 5.1.44.3, MSD is not needed to be defined.

## 5.1.45	DC_3-7_n5

- 5.1.45.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n5 | CA_3-7 | n5 | DC_3_n5 DC_7_n5 |

- 5.1.45.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A | CA_3A-7A CA_3C-7A CA_3A-7C CA_3C-7C | n5 |

- 5.1.45.3	Co-existence Studies

Co-existence analysis from DC_3_n5 shows that there is IMD2 and IMD3 impact from DC_3_n5 UL to Band 7 DL.

Co-existence analysis from DC_7_n5 shows that there is no impact from DC_7_n5 UL to Band 3 DL.

- 5.1.45.4		∆TIB and ∆RIB values

For DC_3-7_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_3-5-7, and are given in the tables below.

Table 5.1.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n5 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n5 | 3 | 0 |
|  | 7 | 0 |
|  | n5 | 0 |

- 5.1.45.5	REFSENS requirements

MSD need to be defined in TS 38.101-3, see below.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | FDD | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 |  | IMD21 |
|  | n5 | 845 | 5 | 25 | 890 | N/A |  | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 5.1.46	DC_3-28_n5

- 5.1.46.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-28_n5 | CA_3-28 | n5 | DC_3_n5 |

- 5.1.46.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_28A_n5A | CA_3A-28A CA_3C-28A | n5 |

- 5.1.46.3	Co-existence Studies

Co-existence analysis from DC_28_n5 shows that there is IMD4 impact from DC_28_n5 UL to Band 3 DL.

Co-existence analysis from DC_3_n5 shows that there is IMD4 impact from DC_3_n5 UL to Band 28 DL.

- 5.1.46.4		∆TIB and ∆RIB values

For DC_3-28_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_3-5-28, and are given in the tables below.

Table 5.1.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n5 | 3 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |

Table 5.1.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n5 | 3 | 0 |
|  | 28 | 0.1 |
|  | n5 | 0.1 |

- 5.1.46.5	REFSENS requirements

Co-existence analysis from shows that there is IMD4 impact from DC_28_n5 UL to Band 3 DL, as well as IMD4 impact from DC_3_n5 UL to Band 28 DL. Therefore, MSD values are needed to be introduced in 38.101-3.

MSD values are reused from DC_20_n28-n78 which has IMD4 in n28 DL and n78 DL.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | 1735 | 5 | 25 | 1830 | 8.7 | FDD | IMD4 |
|  | 28 | 705 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | FDD | IMD4 |
|  | n5 | 845 | 5 | 25 | 874 | N/A | FDD | N/A |

## 5.1.47	DC_7-28_n5

- 5.1.47.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-28_n53 | CA_7-28 | n5 | DC_7_n5 |
| NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

- 5.1.47.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n5A4 DC_7C-28A_n5A4 | DC_7A_n5A DC_7C_n5A DC_28A_n5A | CA_7A-28A CA_7C-28A | n5 |
| NOTE 4: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

- 5.1.47.3	Co-existence Studies

In these analyses the frequency range in band n28 is restricted to 703-733 MHz for the UL and 758-788 MHz for the DL

Co-existence for DC_28A_n5A UL into band 7 DL is investigated in Table 5.1.47.3-1.

Table 5.1.47.3-1: Harmonic and IMD analysis DC_28A_n5A UL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 733 | 824 | 849 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1466 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2199 | 2472 | 2547 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2932 | 3296 | 3396 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3665 | 4120 | 4245 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4218 | 4398 | 4944 | 5094 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4921 | 5131 | 5768 | 5943 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 91 | 1527 | 1582 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 642 | 915 | 995 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2315 | 2351 | 2431 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 182 | 3054 | 3164 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1375 | 1739 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3048 | 3175 | 3280 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2563 | 2108 | 1963 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4129 | 3636 | 3781 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 1006 | 551 | 411 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4013 | 3757 | 3897 |

Co-existence analysis in Table 5.1.47.3-1 shows that there is IMD5 impact from DC_28_n5 UL to Band 7 DL.

Co-existence for DC_7A_n5A UL into band 28 DL is investigated in Table 5.1.47.3-2.

Table 5.1.47.3-2: Harmonic and IMD analysis DC_7A_n5A UL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12500 | 12850 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 4944 | 5094 | 15000 | 15420 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 5768 | 5943 | 17500 | 17990 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1746 | 1651 | 3324 | 3419 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 922 | 802 | 4151 | 4316 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3492 | 3302 | 6648 | 6838 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9456 | 9151 | 896 | 726 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6062 | 5802 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

Co-existence analysis in Table 5.1.47.3-2 shows that there is IMD5 impact from DC_7_n5 UL to Band 28 DL.

- 5.1.47.4		∆TIB and ∆RIB values

For DC_7-28_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_5-7-28, and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n5 | 7 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n5 | 7 | 0 |
|  | 28 | 0 |
|  | n5 | 0 |

- 5.1.47.5	REFSENS requirements

Co-existence analysis in 5.1.47.3 shows that there is IMD5 impact from DC_7_n5 UL to Band 28 DL, as well as IMD5 impact from DC_28_n5 UL to Band 7 DL. Therefore, MSD values are needed in 38.101-3.

For band 28 it is used the IMD5 MSD value for band 28 in DC_18A-28A_n77A.

For band 7 it is used the IMD5 MSD value for band 7 in DC_7A-20A_n28A.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | FDD | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | FDD | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | FDD | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | FDD | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | FDD | N/A |

## 5.1.48	DC_1-11_n77

- 5.1.48.1	Operating bands for DC_1-11_n77

Table 5.1.48.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n77 | CA_1-11 | n77 | DC_1_n77 |

- 5.1.48.2	Configurations for DC_1-11_n77

Table 5.1.48.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n77A | DC_1A_n77A DC_11A_n77A | CA_1A-11A | n77A |

- 5.1.48.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n77A, (1) IMD2 of (B1 - n77) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n77A, (2) IMD2 of (B11 - n77) will fall into Rx band of Band 1.

- 5.1.48.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.48.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |

Table 5.1.48.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n77 | 1 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |

- 5.1.48.5	REFSENS requirements

As mentioned above, IMD2 of B1 and n77 to Band 11 Rx and IMD2 of B11 and n77 to Band 1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.48.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A |  |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | TDD | N/A |  |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | FDD | IMD2 |  |
| DC_1A-11A_n77A | 11 | 1438 | 5 | 25 | 1486 | N/A | FDD | N/A |  |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | FDD | IMD2 |  |

## 5.1.49	DC_3-20_n1

- 5.1.49.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20_n1 | CA_3-20 | n1 | DC_3_n1 |

- 5.1.49.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n1A | DC_3A_n1A DC_20A_n1A | CA_3A-20A | n1 |
| DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A | CA_3C-20A | n1 |

- 5.1.49.3		∆TIB and ∆RIB values

For DC_3-20_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-20, and are given in the tables below.

Table 5.1.49.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.3 |

Table 5.1.49.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n1 | 3 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |

- 5.1.49.4	REFSENS requirements

Co-existence analysis from DC_3_n1 shows that there is no impact from DC_3_n1 UL to Band 20 DL.

Co-existence analysis from DC_20_n1 shows that there is no impact from DC_20_n1 UL to Band 3 DL.

Therefore, MSD does not need to be defined.

## 5.1.50	DC_1-20_n3

- 5.1.50.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-20_n3 | CA_1-20 | n3 | 1_n3 |

- 5.1.50.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-20A_n3A | DC_1A_n3A DC_20A_n3A | CA_1A-20A | n3 |

- 5.1.50.3		∆TIB and ∆RIB values

For DC_1-20_n3, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-20, and are given in the tables below.

Table 5.1.50.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.50.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n3 | 1 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |

- 5.1.50.4	REFSENS requirements

Co-existence analysis from DC_1_n3 shows that there is no impact from DC_1_n3 UL to Band 20 DL.

Co-existence analysis from DC_20_n3 shows that there is no impact from DC_20_n3 UL to Band 1 DL.

Therefore, MSD does not need to be defined.

## 5.1.51	DC_2-66_n5, DC_2-2-66_n5, DC_2-66-66_n5 DC_2-2-66-66_n5, and DC_2-66-66-66_n5

- 5.1.51.1	Operating bands for DC

Table 5.1.51.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n5 | CA_2-66 | n5 | No |

- 5.1.51.2	Configuration for DC

Table 5.1.51.2: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A | n5A |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-2A-66A | n5A |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A-66A | n5A |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-2A-66A-66A | n5A |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A-66A-66A | n5A |

- 5.1.51.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the 2nd and 5th IMD of 66+n5 may fall into the band n5 downlink and Pcell REFSENS excpetions can be represented by the REFSENS expcetion already specified for DC_66A-n5A.

Co-existence studies for DC_2A_n5A is also available in TR 37.863-01-01. For this 4th IMD product of 2+n5 may fall into band 66 downlink.

- 5.1.51.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are derived from CA_2-5-66 in TS 36.101.

Table 5.1.51.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A, DC_2-66-66-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.51.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A, DC_2-66-66-66_n5 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0 |

- 5.1.51.5	Refsens requirements

Pcell REFSENS exceptions for band n5 due to 2 uplink of band 66 and band n5 can be represented by the REFSENS expcetion already specified for DC_66A-n5A in TS 38.101-3.

Scell REFSENS exceptions for band 66 due to 2 uplink of band 2 and band n5, MSD is specified in Table 5.1.51.5-1

Table 5.1.51.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | FDD | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |

The MSD value is based on predefined value for LTE CA_2A-2A-5A-66A-66A / CA_2A-5A_66A.

## 5.1.52	DC_3-7_n77, DC_3-3-7_n77, DC_3-7-7_n77, DC_3-3-7-7_n77

- 5.1.52.1	Operating bands for DC

Table 5.1.52.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n77 | CA_3-7 | n77 | DC_3_n77 |
| DC_3-3-7_n77 | CA_3-3-7 | n77 | DC_3_n77 |
| DC_3-7-7_n77 | CA_3-7-7 | n77 | DC_3_n77 |
| DC_3-3-7-7_n77 | CA_3-3-7-7 | n77 | DC_3_n77 |

- 5.1.52.2	Configuration for DC

Table 5.1.52.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A_n77A | DC_3A_n77A, DC_7A_n77A | CA_3A-7A | n77A |
| DC_3A-3A-7A_n77A | DC_3A_n77A, DC_7A_n77A | CA_3A-3A-7A | n77A |
| DC_3A-7A-7A_n77A | DC_3A_n77A, DC_7A_n77A | CA_3A-7A-7A | n77A |
| DC_3A-3A-7A-7A_n77A | DC_3A_n77A, DC_7A_n77A | CA_3A-3A-7A-7A | n77A |

- 5.1.52.3	Co-existence Studies

Based on co-existence studies of DC_7_n77 and DC_3_n77, as captured in TR 37.716-11-11 and TR 37.863-01-01, own Rx impact on the 3rd band are the following:

-	5th order IM generated by dual uplink of Band 3 and n77 may fall into Band 7

-	3rd and 4th order IM generated by dual uplink of Band 7 and n77 may fall into Band 3

- 5.1.52.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 5.1.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.52.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

- 5.1.52.5	REFSENS requirements

Table below lists the MSD required for the dual connectivity configuration.

Table 5.1.52.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 ¦fB77-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD |  |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD |  |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 ¦2*fB77-2*fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD |  |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | TDD |  |
| DC_3A-7A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | FDD | IMD5 ¦fB77-4*fB3¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | FDD |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD |  |
| DC_3A-7A_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | FDD | IMD5 ¦2*fB77-3*fB3¦ |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | FDD |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD |  |

## 5.1.53	DC_2-13_n66 and DC_2-2-13_n66

- 5.1.53.1		Operating bands for DC

Table 5.1.53.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-13_n66 | CA_2-13 | n66 | No |
| DC_2-2-13_n66 | CA_2-2-13 | n66 | No |

- 5.1.53.2	Configuration for DC

Table 5.1.53.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A | CA_2A-13A | n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A | CA_2A-2A-13A | n66A |

- 5.1.53.3	∆TIB and ∆RIB values

For DC_2-13_n66 and DC_2-2-13_n66, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_2A-13A-66A

Table 5.1.53.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.53.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.3 |
|  | 13 | 0 |
|  | n66 | 0.3 |

- 5.1.53.4	REFSENS requirements

From Band 2 +Band n66 analysis, harmonics up to 7th order and IMD up to 5th order does not fall into own 3rd receiving band. Thus Table 5.1.53.4-1 lists Band 13 +Band n66 2UL DC up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.53.4-1: Band 13 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4662 | 4722 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5439 | 5509 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1003 | 923 | 2487 | 2567 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 797 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3786 | 3556 | 1059 | 1229 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6684 | 6914 | 5751 | 5921 |

Based on Table 5.1.53.4-1;

-	4th order IMD may also fall into own Rx frequencies of band 2. MSD would be required as below table. The number re-use from LTE CA_2A-4A-13A_2UL_4A-13A

Table 5.1.53.4-2: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | 1860 | 5 | 25 | 1940 | 6.2 | FDD | IMD4 ¦2*fn66-2*fB13¦ |
|  | 13 | 780 | 10 | 50 | 749 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | TDD | N/A |

## 5.1.54	DC_2-7_n66, DC_2-7-7_n66

- 5.1.54.1	Operating bands for DC

Table 5.1.54.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-7_n66 | CA_2-7 | n66 |  |
| DC_2-7-7_n66 | CA_2-7-7 | n66 |  |

- 5.1.54.2	Configurations for DC

Table 5.1.54.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-7A_n66A DC_2A-7C_n66A DC_2A-7A-7A_n66A | DC_2A_n66A1 DC_7A_n66A | CA_2A-7A CA_2A-7C CA_2A-7A-7A | n66A |
| NOTE1: Single UL is allowed |  |  |  |

- 5.1.54.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.54.4	∆TIB and ∆RIB values

DC_2-7_n66 and DC_2-7-7_n66 could reuse the TIB,c and RIB,c values of CA_2A-7A-66A  as given in the below tables.

Table 5.1.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n66 DC_2-7-7_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.54.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n66 DC_2-7-7_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

- 5.1.54.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.55	DC_2-66_n66

- 5.1.55.1	Operating bands for DC

Table 5.1.55.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n66 | CA_2-66 | n66 |  |

- 5.1.55.2	Configurations for DC

Table 5.1.55.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n66A | DC_2A_n66A1 DC_66A_n66A2 | CA_2A-66A | n66A |
| NOTE1: Single UL is allowed NOTE2: Only single switched UL is supported |  |  |  |

- 5.1.55.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.55.4	∆TIB and ∆RIB values

For DC_2-66_n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.55.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

- 5.1.55.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.56	DC_7-66_n66, DC_7-7-66_n66

- 5.1.56.1	Operating bands for DC

Table 5.1.56.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-66_n66 | CA_7-66 | n66 |  |
| DC_7-7-66_n66 | CA_7-7-66 | n66 |  |

- 5.1.56.2	Configurations for DC

Table 5.1.56.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-66A_n66A DC_7C-66A_n66A DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A1 | CA_7A-66A CA_7C-66A CA_7A-7A-66A | n66A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.56.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.56.4	∆TIB and ∆RIB values

For DC_7-66_n66 and DC_7-7-66_n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n66 DC_7-7-66_n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.56.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n66 DC_7-7-66_n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

- 5.1.56.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.57	DC_3-8_n1, DC_3-3-8_n1

- 5.1.57.1	Operating bands for DC

Table 5.1.57.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n1 | CA_3-8 | n1 | DC_3_n1 |
| DC_3-3-8_n1 | CA_3-3-8 | n1 | DC_3_n1 |

- 5.1.57.2	Configuration for DC

Table 5.1.57.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n1A | DC_3A_n1A, DC_8A_n1A | CA_3A-8A | n1A |
| DC_3C-8A_n1A | DC_3A_n1A, DC_8A_n1A | CA_3C-8A | n1A |
| DC_3A-3A-8A_n1A | DC_3A_n1A, DC_8A_n1A | CA_3A-3A-8A | n1A |

- 5.1.57.3	Co-existence Studies

Based on co-existence studies of DC_3_n1 and DC_8_n1, as captured in TR 37.716-11-11, no own Rx impact on the 3rd band is observed.

- 5.1.57.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 5.1.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n1, DC_3-3-8_n1 | 3 | 0.3 |
|  | 8 | 0.3 |
|  | n1 | 0.3 |

Table 5.1.57.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n1, DC_3-3-8_n1 | 3 | 0 |
|  | 8 | 0 |
|  | n1 | 0 |

- 5.1.57.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.58	DC_7-8_n1, DC_7-7-8_n1

- 5.1.58.1	Operating bands for DC

Table 5.1.58.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-8_n1 | CA_7-8 | n1 | No |
| DC_7-7-8_n1 | CA_7-7-8 | n1 | No |

- 5.1.58.2	Configuration for DC

Table 5.1.58.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-8A_n1A | DC_7A_n1A, DC_8A_n1A | CA_7A-8A | n1A |
| DC_7A-7A-8A_n1A | DC_7A_n1A, DC_8A_n1A | CA_7A-7A-8A | n1A |

- 5.1.58.3	Co-existence Studies

Based on co-existence studies of DC_7_n1 and DC_8_n1, as captured in TR 37.716-11-11, the following own Rx impact on the 3rd band is observed.

- 5th order IMD generated by dual uplink of Band 7 + Band n1 may fall into own Rx of band 8. However, based on the studies for DL CA_1A-7A-8A paired with CA_1A-7A in TR 36.714-00-02, the IMD5 is fairly small and the overlapped region is quite small portion, no MSD requirement is needed.

- 5.1.58.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 5.1.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n1, DC_7-7-8_n1 | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.58.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n1, DC_7-7-8_n1 | 7 | 0 |
|  | 8 | 0.2 |
|  | n1 | 0 |

- 5.1.58.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.59	DC_1-8_n3

- 5.1.59.1	Operating bands for DC_1-8_n3

Table 5.1.59.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n3 | CA_1-8 | n3 | DC_1_n3 |

- 5.1.59.2	Configurations for DC_1-8_n3

Table 5.1.59.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A | CA_1A-8A | n3A |

- 5.1.59.3	Co-existence studies

When Uplink EN-DC configuration is DC_1_n3, no IMDs will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8_n3, no IMDs will fall into Rx band of Band 1.

Thus, it is not needed to evaluate.

- 5.1.59.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.59.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.59.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n3 | 1 | 0 |
|  | 8 | 0 |
|  | n3 | 0 |

- 5.1.59.5	REFSENS requirements

As mentioned in 5.1.59.3, REFSENS exceptions are not expected.

## 5.1.60	DC_8-42_n77

- 5.1.60.1	Operating bands for DC_8-42_n77

Table 5.1.60.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-42_n77 | CA_8-42 | n77 | No |

- 5.1.60.2	Configurations for DC_8-42_n77

Table 5.1.60.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-42A_n77A | DC_8A_n77A | CA_8A-42A | n77A |
| DC_8A-42C_n77A | DC_8A_n77A | CA_8A-42C | n77A |

- 5.1.60.3	Co-existence studies

The co-existence studies for the lower order 2DL/2UL DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration based on the following conditions.

- LTE Band 42 and NR Band n77 are synchronized.

- Simultaneous transmission of LTE Band 42 and NR Band n77 are not supported.

- 5.1.60.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.60.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.60.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

- 5.1.60.5	REFSENS requirements

As mentioned in 5.1.60.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 5.1.61	DC_3-40_n1

- 5.1.61.1	Operating bands for DC

Table 5.1.61.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-40_n1 | CA_3-40 | n1 | No |

- 5.1.61.2	Configurations for DC

Table 5.1.61.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-40A_n1A | DC_3A_n1A DC_40A_n1A | CA_3A-40A | n1A |

- 5.1.61.3	∆TIB and ∆RIB values

For DC_3-40_n1, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.61.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-40_n1 | 3 | 0.5 |
|  | 40 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.61.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-40_n1 | 3 | 0 |
|  | 40 | 0 |
|  | n1 | 0 |

- 5.1.61.4	REFSENS requirements

For UE coexistence study of Band 3 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.61.4-1.

Table 5.1.61.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

For UE coexistence study of Band 40 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.61.4-2

Table 5.1.61.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 11500 | 12000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

Based on co-existence study as presented in the table 5.1.61.4-1, IMD5 of band3 and band n1 will fall into band40 Rx.

Based on co-existence study as presented in the table 5.1.61.4-2, there is no MSD issue on the own Rx impact of the 3rd band when band40 and band n1 are uplink.

According to Table 7.3.1A-0g from TS 36.101, the MSD of CA_1A-3A-40A caused by IMD5 can be reused. The MSD value is shown below.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | TDD | IMD5 |

## 5.1.62	DC_7-40_n1

- 5.1.62.1	Operating bands for DC

Table 5.1.62.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-40_n1 | CA_7-40 | n1 | No |

- 5.1.62.2	Configurations for DC

Table 5.1.62.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-40A_n1A | DC_7A_n1A DC_40A_n1A | CA_7A-40A | n1A |

- 5.1.62.3	∆TIB and ∆RIB values

For DC_7-40_n1, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.62.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-40_n1 | 7 | 0.8 |
|  | 40 | 0.9 |
|  | n1 | 0.6 |

Table 5.1.62.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-40_n1 | 7 | 0.3 |
|  | 40 | 0.8 |
|  | n1 | 0 |

- 5.1.62.4	REFSENS requirements

For UE coexistence study of Band 40 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.62.4-1.

Table 5.1.62.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 11500 | 12000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

For UE coexistence study of Band 7 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.62.4-2

Table 5.1.62.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3440 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1300 | 1040 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on co-existence study as presented in the table 5.1.62.4-1, IMD3 of band40 and band n1 will fall into band7 Rx.

Based on co-existence study as presented in the table 5.1.62.4-2, there is no MSD issue on the own Rx impact of the 3rd band when band7 and band n1 are uplink.

According to the analysis, the MSD value is shown below, caused by IMD3 of band40 and n1.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-40A_n1A | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 32.1 | FDD | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |

## 5.1.63	DC_2-66_n66

- 5.1.63.1	Operating bands for DC

Table 5.1.63.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n66 | CA_2-66 | n66 | No |

- 5.1.63.2	Configuration for DC

Table 5.1.63.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n66A | DC_2A_n66A | CA_2A-66A | n66A |

- 5.1.63.3	Coexistence studies

Based on co-existence studies of DC_2A_n66A in TR 37.863-01-01, the third order IMD may fall into band 2 and/or n66; the fifth order IMD may fall into band 66/n66.

- 5.1.63.4	∆TIB and ∆RIB values

For DC_2-66_n66, the TIB,c and RIB,c values are derived from CA_2-66 in TS 36.101.

Table 5.1.63.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.63.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

- 5.1.63.5	REFSENS requirements

MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_2A_n66A) in TS 38.101-3, where band 2 and n66 MSD are specified. To avoid the specification complexity, band 66 MSD is not specified as the same MSD is applied if band 66 carrier frequency is replaced by band n66 carrier frequency.

## 5.1.64	DC_5-66_n5 and DC_5-66-66_n5

- 5.1.64.1	Operating bands for DC

Table 5.1.64.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n5 | CA_5-66 | n5 | No |
| DC_5-66-66_n5 | CA_5-66-66 | n5 | No |

- 5.1.64.2	Configuration for DC

Table 5.1.64.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-66A_n5A | DC_66A_n5A | CA_5A-66A | n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A | CA_5A-66A-66A | n5A |

- 5.1.64.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the second order IMD may fall into band 5 and/or n5.

- 5.1.64.4	∆TIB and ∆RIB values

For DC_5-66_n5 and DC_5-66-66_n5, the TIB,c and RIB,c values are derived from CA_5-66 in TS 36.101.

Table 5.1.64.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n5 DC_5-66-66_n5 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.64.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n5 DC_5-66-66_n5 | 5 | 0 |
|  | 66 | 0 |
|  | n5 | 0 |

- 5.1.64.5	REFSENS requirements

MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_66A_n5A) in TS 38.101-3, where band n5 MSD is specified. To avoid the specification complexity, band 5 MSD is not specified as the same MSD is applied if band n5 carrier frequency is replaced by band 5 carrier frequency.

## 5.1.65	DC_2-5_n66 and DC_2-5-5_n66

- 5.1.65.1	Operating bands for DC

Table 5.1.65.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n66 | CA_2-5 | n66 | No |
| DC_2-5-5_n66 | CA_2-5-5 | n66 | No |

- 5.1.65.2	Configuration for DC

Table 5.1.65.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2-5_n66 | DC_2A_n66A DC_5A_n66A | CA_2A-5A | n66A |
| DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5B | n66A |
| DC_2A-5A-5A_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5A-5A | n66A |

- 5.1.65.3	Coexistence studies

Based on co-existence studies of DC_2A_n66A in TR 37.863-01-01, the third order IMD may fall into band 2 and/or n66; the fifth order IMD may fall into band n66.

Based on co-existence studies of DC_5A_n66A in TR 37.863-01-01, the second order IMD may fall into band 5.

- 5.1.65.4	∆TIB and ∆RIB values

For DC_2-5_n66 and DC_2-5-5_n66, the TIB,c and RIB,c values are derived from CA_2-5-66 in TS 36.101.

Table 5.1.65.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.65.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | n66 | 0.3 |

- 5.1.65.5	REFSENS requirements

The MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_2A_n66A and DC_5A_n66A) in TS 38.101-3. There is no specific MSD for 3DL2UL DC.

## 5.1.66	DC_13-66_n66 and DC_13A-66A-66A_n66A

- 5.1.66.1	Operating bands for DC

Table 5.1.66.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-66_n66 | CA_13-66 | n66 | No |
| DC_13-66-66_n66 | CA_13-66-66 | n66 | No |

- 5.1.66.2	Configuration for DC

Table 5.1.66.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-66A_n66A | DC_13A_n66A | CA_13A-66A | n66A |

- 5.1.66.3	Coexistence studies

Based on co-existence studies of DC_13A_n66A in TR 37.716-11-11 (or R4-1904907), there is no harmonic or IMD issues for this band combination.

- 5.1.66.4	∆TIB and ∆RIB values

For DC_13-66_n66 and DC_13-66-66_n66, the TIB,c and RIB,c values are derived from CA_13-66 in TS 36.101.

Table 5.1.66.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n66 DC_13-66-66_n66 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.66.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n66 DC_13-66-66_n66 | 13 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

- 5.1.66.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.67	DC_2-7_n66

- 5.1.67.1	Configurations for DC

Table 5.1.67.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-7A_n66A DC_2A-7A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |

- 5.1.67.2	∆TIB and ∆RIB values

For DC_2-7_n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.67.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.67.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

- 5.1.67.3	REFSENS requirements

For UE coexistence study of Band 7 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.67.4-1.

Table 5.1.67.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 720 | 860 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -850 | -1060 | 3220 | 3430 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5920 | 6130 | 6710 | 6920 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -2560 | -2840 | 5720 | 6000 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7630 | 7910 | 9210 | 9420 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4270 | 4620 | 8220 | 8570 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 10 | -340 | 3940 | 4290 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10920 | 11270 | 10130 | 10480 |

For UE coexistence study of Band 2 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.67.4-2

Table 5.1.67.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 70 | 200 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -1510 | -1710 | 1920 | 2110 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5270 | 5470 | 5410 | 5600 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -3220 | -3490 | 3770 | 4020 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6980 | 7250 | 7260 | 7440 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4930 | 5270 | 5620 | 5930 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | -1310 | -1640 | 1990 | 2310 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 8970 | 9290 | 8830 | 9160 |

Based on co-existence study as presented in the table 5.1.67.4-1 and 5.1.67.4-2, there is no MSD issue on the own Rx impact of the 3rd band.

## 5.1.68	DC_7-13_n66

- 5.1.68.1	Configurations for DC

Table 5.1.68.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-13A_n66A DC_7A-7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |

- 5.1.68.2	∆TIB and ∆RIB values

For DC_7-13_n66, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.68.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-13_n66 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.68.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-13_n66 | 7 | 0.5 |
|  | 13 | 0 |
|  | n66 | 0.5 |

- 5.1.68.3	REFSENS requirements

For UE coexistence study of Band 7 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.68.3-1.

Table 5.1.68.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 720 | 860 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -850 | -1060 | 3220 | 3430 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5920 | 6130 | 6710 | 6920 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -2560 | -2840 | 5720 | 6000 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7630 | 7910 | 9210 | 9420 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4270 | 4620 | 8220 | 8570 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 10 | -340 | 3940 | 4290 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10920 | 11270 | 10130 | 10480 |

For UE coexistence study of Band 13 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.68.3-2

Table 5.1.68.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 8550 | 8900 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 923 | 1003 | 2487 | 2567 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 2783 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4041 | 4141 | 5907 | 6117 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1846 | 2006 | 4974 | 5134 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1328 | 1438 | 6053 | 6343 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 4818 | 4928 | 7617 | 7907 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 1229 | 1059 | 3556 | 3786 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 6684 | 6914 | 5751 | 5921 |

Based on co-existence study as presented in the table 5.1.68.3-1 and 5.1.68.3-2, own Rx impact of the 3rd band is shown as the followings.

- 2nd order IMD generated by dual uplink of Band 7 + Band n66 may fall into own Rx of band 13

- 3rd order IMD generated by dual uplink of Band 13+ Band n66 may fall into own Rx of band 7

MSD Values for DC_7-13_n66 are specified in Table 5.1.68.3-3.

Table 5.1.68.3-3: MSD due to IMD issue

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_7-13_n66 | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 31 | FDD | IMD2 ¦fB7-fB66¦ |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
| DC_7-13_n66 | 7 | 2540 | 5 | 25 | 2660 | 18 | FDD | IMD3 ¦2*fBn66-fB13¦ |
|  | 13 | 780 | 5 | 25 | 749 | N/A | FDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | TDD | N/A |

## 5.1.69	DC_2-66_n41

- 5.1.69.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66_n41 | CA_2-66 | n41 | No |

- 5.1.69.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n41A | DC_2A_n41A DC_66A_n41A | CA_2A-66A | n41 |

- 5.1.69.3		∆TIB and ∆RIB values

For DC_2-66_n41, the TIB,c and RIB,c values are reused from the LTE combination CA_2-7-66, and are given in the tables below.

Table 5.1.69.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.69.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- 5.1.69.4	REFSENS requirements

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band 66 DL.

Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL.

The IMD4 MSD value for the impact on band 2 DL are same as for band n25 DL in DC_66_n25-n41.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 ¦2*fBn41-2*fB66¦ |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | TDD | N/A |

## 5.1.70	DC_12-30_n2

- 5.1.70.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-30_n2 | CA_12-30 | n2 | No |

- 5.1.70.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A | CA_12A-30A | n2 |

- 5.1.70.3		∆TIB and ∆RIB values

For DC_12-30_n2, the TIB,c and RIB,c values are reused from the LTE combination CA_2-12-30, and are given in the tables below.

Table 5.1.70.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-30_n2 | 12 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.70.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-30_n2 | 12 | 0 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |

- 5.1.70.4	REFSENS requirements

Co-existence analysis from DC_30_n2 shows that there is no impact from DC_30_n2 UL to Band 12 DL.

Co-existence analysis from DC_12_n2 shows that there is IMD4 impact from DC_12_n2 UL to Band 30 DL. The MSD values for the impact on band 30 DL are reused from CA_2-12_30.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | FDD | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 12.0 | FDD | IMD4 |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |

## 5.1.71	DC_12-66_n2
12-66-66_n2

- 5.1.71.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n2 | CA_12-66 | n2 | DC_66_n2 |

- 5.1.71.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A | CA_12A-66A | n2 |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A | CA_12A-66A-66A | n2 |

- 5.1.71.3		∆TIB and ∆RIB values

For DC_12-66_n2, the TIB,c and RIB,c values are reused from the LTE combination CA_2-12-66, and are given in the tables below.

Table 5.1.71.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n2 | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.71.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n2 | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

- 5.1.71.4	REFSENS requirements

Co-existence analysis from DC_12_n2 shows that there is no impact from DC_12_n2 UL to Band 66 DL.

Co-existence analysis from DC_66_n2 shows that there is no impact from DC_66_n2 UL to Band 12 DL.

## 5.1.72	DC_30-66_n2, DC_30-66-66_n2

- 5.1.72.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 30-66_n2 | CA_30-66 | n2 | DC_66_n2 |

- 5.1.72.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A | CA_30A-66A | n2 |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A | CA_30A-66A-66A | n2 |

- 5.1.72.3		∆TIB and ∆RIB values

For DC_30-66_n2, the TIB,c and RIB,c values are reused from the LTE combination CA_2-30-66, and are given in the tables below.

Table 5.1.72.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_30-66_n2 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.72.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_30-66_n2 | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

- 5.1.72.4	REFSENS requirements

Co-existence analysis from DC_30_n2 shows that there is no impact from DC_30_n2 UL to Band 66 DL.

Co-existence analysis from DC_66_n2 shows that there is no impact from DC_66_n2 UL to Band 30 DL.

## 5.1.73	DC_1-8_n28

- 5.1.73.1	Operating bands for DC_1-8_n28

Table 5.1.73.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n28 | CA_1-8 | n28 | No |

- 5.1.73.2	Configurations for DC_1-8_n28

Table 5.1.73.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A | CA_1A-8A | n28A |

- 5.1.73.3	Co-existence studies

When Uplink EN-DC configuration is DC_1_n28, 5th IMD will fall into Rx band of Band 8. When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1.

However, when consider the 5MHz UL channel BW for DC_8_n28, the IMD5 does not fall into the B1 own Rx band.

- 5.1.73.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.73.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.73.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

- 5.1.73.5	REFSENS requirements

As mentioned in 5.1.73.3, IMD5 of B1 and n28 to Band 8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.73.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |  |
|  | n28 | 730 | 5 | 25 | 785 | N/A | FDD | N/A |  |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | FDD | IMD5 |  |
| DC_1A-8A_n28A | 8 | 882.5 | 5 | 25 | 927.5 | N/A | FDD | N/A |  |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |  |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | 0 | FDD | IMD5 |  |

## 5.1.74	DC_3-3-8_n78

- 5.1.74.1	Operating bands for DC

Table 5.1.74.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-3-8_n78 | CA_3-3-8 | n78 | No |

- 5.1.74.2	Configuration for DC

Table 5.1.74.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-3A-8A_n78A | DC_3A_n78A, DC_8A_n78A | CA_3A-8A | n78A |

- 5.1.74.3	Co-existence Studies

Co-existence was studied for DC_3A-8A_n78A in Rel-15 and the results are captured in TR 37.863-02-01. No further studies needed for DC_3A-3A-8A_n78A.

- 5.1.74.4	∆TIB and ∆RIB values

For DC_3-3-8_n78 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-8_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.74.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |

- 5.1.74.5	REFSENS requirements

According to the REFSENS study in TR 37.863-02-01 for DC_3A-8A_n78A, no additional MSD requirement is need.

## 5.1.75	DC_2-46_n41

- 5.1.75.1	Operating bands for DC

Table 5.1.75.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-46_n41 | CA_2-46 | n41 | No |

- 5.1.75.2	Configuration for DC

Table 5.1.75.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |

- 5.1.75.3	Coexistence studies

Co-existence studies of DC_2A_n41A are captured in TR 37.863-01-01.

- 5.1.75.4	∆TIB and ∆RIB values

For DC_2-46_n41, the TIB,c and RIB,c values are derived from DC_2-41 in TR 38.101-3.

Table 5.1.75.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n41 | 2 | 0.5 |
|  | 46 | 0 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.75.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n41 | 2 | 0 |
|  | 46 | 0 |
|  | n41 | 01 |
|  |  | 02 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- 5.1.75.5	REFSENS requirements

No MSD is needed.

## 5.1.76	DC_2-46_n71

- 5.1.76.1	Operating bands for DC

Table 5.1.76.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-46_n71 | CA_2-46 | n71 | No |

- 5.1.76.2	Configuration for DC

Table 5.1.76.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |

- 5.1.76.3	Coexistence studies

Co-existence studies of DC_2A_n71A are captured in TR 37.863-01-01.

- 5.1.76.4	∆TIB and ∆RIB values

For DC_2-46_n71, the TIB,c and RIB,c values are derived from CA_2-46, CA_2-71 and CA_46-71 in TS 36.101.

Table 5.1.76.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n71 | 2 | 0 |
|  | 46 | 0 |
|  | n71 | 0 |

Table 5.1.76.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n71 | 2 | 0 |
|  | 46 | 0 |
|  | n71 | 0 |

- 5.1.76.5	REFSENS requirements

MSD due to harmonics that affects both Band 2 and Band n71 have been specified for DC_2A_n71A in TR 38.101-3. REFSENS have been specified for CA_46A/C/D-71A in 36.101.

## 5.1.77	DC_46-66_n41

- 5.1.77.1	Operating bands for DC

Table 5.1.77.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n41 | CA_46-66 | n41 | No |

- 5.1.77.2	Configuration for DC

Table 5.1.77.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |

- 5.1.77.3	∆TIB and ∆RIB values

For DC_46-66_n41, the TIB,c and RIB,c values are derived from DC_66-n41 in TR 38.101-3 and CA_46-66 from 36.101.

Table 5.1.77.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n41 | 46 | 0 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.77.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n41 | 46 | 0 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- 5.1.77.4	REFSENS requirements

MSD due to close proximity is needed and is already specified in TS 38.101-3 for DC_66_n41.

## 5.1.78	DC_46-66A_n71

- 5.1.78.1	Operating bands for DC

Table 5.1.78.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n71 | CA_46-66 | n71 | No |

- 5.1.78.2	Configuration for DC

Table 5.1.78.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |

- 5.1.78.3	Coexistence studies

Co-existence studies of DC_66A_n71A are captured in TR 37.863-01-01.

- 5.1.78.4	∆TIB and ∆RIB values

For DC_46-66A_n71, the TIB,c and RIB,c values are derived from DC_66A_n71A in TR 38.101-3 / TR 37.863-01-01.

Table 5.1.78.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66A_n71 | 46 | 0 |
|  | 66 | 0.3 |
|  | n71 | 0.3 |

Table 5.1.78.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_46-66A_n71 | 46 | 0 |
|  | 66 | 0 |
|  | n71 | 0 |

- 5.1.78.5	REFSENS requirements

MSD have been specified for DC_66A_n71A in TR 38.101-3. REFSENS have been specified for CA_46A/C/D-71A in 36.101.

## 5.1.79	DC_1-3_n38

- 5.1.79.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3_n38 | CA_1-3 | n38 | No |

- 5.1.79.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A | CA_1A-3A | n38 |

- 5.1.79.3		∆TIB and ∆RIB values

For DC_1-3_n38, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-38, and are given in the tables below.

Table 5.1.79.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n38 | 0.5 |

Table 5.1.79.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n38 | 1 | 0 |
|  | 3 | 0 |
|  | n38 | 0 |

- 5.1.79.4	REFSENS requirements

Co-existence analysis from DC_1_n38 shows that there is no impact from DC_1_n38 UL to Band 3 DL.

Co-existence analysis from DC_3_n38 shows that there is no impact from DC_3_n38 UL to Band 1 DL.

## 5.1.80	DC_7-20_n3

- 5.1.80.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-20_n3 | CA_7-20 | n3 | No |

- 5.1.80.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-20A_n3A | DC_7A_n3A DC_20A_n3A | CA_7A-20A | n3 |

- 5.1.80.3		∆TIB and ∆RIB values

For DC_7-20_n3, the TIB,c and RIB,c values are reused from the LTE combination CA_3-7-20, and are given in the tables below.

Table 5.1.80.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n3 | 7 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |

Table 5.1.80.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20_n3 | 7 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |

- 5.1.80.4	REFSENS requirements

Co-existence analysis from DC_7_n3 shows that there are IMD2 impact from DC_7_n3 UL to Band 20 DL. The MSD values for the impact on band 20 DL are reused from CA_3A-7A-20A.

Co-existence analysis from DC_20_n3 shows that there are IMD2 and IMD3 impact from DC_20_n3 UL to Band 7 DL. The MSD values for the impact on band 7 DL are reused from CA_3A-7A-20A.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | FDD | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | FDD | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | FDD | IMD21 |
|  | 20 | 855 | 5 | 25 | 896 | N/A | FDD | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | FDD | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 5.1.81	DC_25-41_n41, DC_25-(n)41, DC_25-25-41_n41, DC_25-25-(n)41

- 5.1.81.1	Void

- 5.1.81.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.1.81.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |

- 5.1.81.3	∆TIB and ∆RIB values

For DC_25_41_n41 and DC_25_(n)41, the TIB,c and RIB,c values are reused from the LTE combination CA_25-41 and CA_25-24-41, and are given in the tables below.

Table 5.1.81.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 | 25 | 0.5 |
|  | 41 | 0.41 |
|  |  | 0.92 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 5.1.81.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 | 41 | 01 |
|  |  | 0.52 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

- 5.1.81.4	REFSENS requirements

Coexistence analysis of DC_25A_n41A shows there is no 2 UL IMD impacting the band 25 downlink.

Coexistence analysis of DC_41A_n41A and DC_(n)41AA shows there is no 2 UL IMD impacting the band 25 downlink.

## 5.1.82	DC_1-8_n3

- 5.1.82.1	Operating bands for DC_1-8_n3

Table 5.1.82.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-8_n3 | CA_1-8 | n3 | DC_1_n3 |

- 5.1.82.2	Configurations for DC_1-8_n3

Table 5.1.82.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A | CA_1A-8A | n3A |

- 5.1.82.3	Co-existence studies

- There is no additional harmonic and intermodulation impact for the additional band receiver.

- 5.1.82.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.82.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.82.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8_n3 | 1 | 0 |
|  | 8 | 0 |
|  | n3 | 0 |

- 5.1.82.5	REFSENS requirements

As mentioned in 5.1.82.3, REFSENS exceptions are not expected.

## 5.1.83	DC_13-48_n2

- 5.1.83.1	Operating bands for DC

Table 5.1.83.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-48_n2 | CA_13-48 | n2 | No |

- 5.1.83.2	Configurations for DC

Table 5.1.83.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A | CA_13A-48A CA_13A-48B CA_13A-48D CA_13A-48E | n2A |

- 5.1.83.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.83.4	∆TIB and ∆RIB values

DC_13-48_n2 could reuse the TIB,c and RIB,c values of CA_2A-13A-48A  as given in the below tables.

Table 5.1.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n2 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |

Table 5.1.83.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n2 | 13 | 0 |
|  | 48 | 0.5 |
|  | n2 | 0.2 |

- 5.1.83.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.84	DC_13-48_n66

- 5.1.84.1	Operating bands for DC

Table 5.1.84.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-48_n66 | CA_13-48 | n66 | No |

- 5.1.84.2	Configurations for DC

Table 5.1.84.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A | CA_13A-48A CA_13A-48B CA_13A-48D CA_13A-48E | n66A |

- 5.1.84.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.84.4	∆TIB and ∆RIB values

DC_13-48_n66 could reuse the TIB,c and RIB,c values of CA_13A-48A-66A  as given in the below tables.

Table 5.1.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n66 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |

Table 5.1.84.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n66 | 13 | 0 |
|  | 48 | 0.5 |
|  | n66 | 0.2 |

- 5.1.84.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.85	DC_48-66_n5

- 5.1.85.1	Operating bands for DC

Table 5.1.85.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_48-66_n5 | CA_48-66 | n5 | DC_66-n5 |

- 5.1.85.2	Configurations for DC

Table 5.1.85.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A | CA_48A-n66A CA_48B-n66A CA_48D-n66A CA_48E-n66A | n5A |

- 5.1.85.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.85.4	∆TIB and ∆RIB values

DC_48-66_n5 could reuse the TIB,c and RIB,c values of CA_5A-48A-66A  as given in the below tables.

Table 5.1.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n5 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n5 | 0.3 |

Table 5.1.85.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n5 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n5 | 0 |

- 5.1.85.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.86	DC_1-7_n3

- 5.1.86.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-7_n3 | CA_1-7 | n3 | DC_1_n3 |

- 5.1.86.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n3A | DC_1A_n3A DC_7A_n3A | CA_1A-7A | n3 |

- 5.1.86.3		∆TIB and ∆RIB values

For DC_1-7_n3, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-7, and are given in the tables below.

Table 5.1.86.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.86.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n3 | 1 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |

- 5.1.86.4	REFSENS requirements

Co-existence analysis from DC_1_n3 shows that there is no impact from DC_1_n3 UL to Band 7 DL.

Co-existence analysis from DC_7_n3 shows that there is no impact from DC_7_n3 UL to Band 1 DL.

## 5.1.87	DC_1-20_n3

- 5.1.87.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-20_n3 | CA_1-20 | n3 | DC_1_n3 |

- 5.1.87.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-20A_n3A | DC_1A_n3A DC_20A_n3A | CA_1A-20A | n3 |
| DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A | CA_1C-20A | n3 |

- 5.1.87.3		∆TIB and ∆RIB values

For DC_1-20_n3, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-20, and are given in the tables below.

Table 5.1.87.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.87.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n3 | 1 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |

- 5.1.87.4	REFSENS requirements

Co-existence analysis from DC_1_n3 shows that there is no impact from DC_1_n3 UL to Band 20 DL.

Co-existence analysis from DC_20_n3 shows that there is no impact from DC_20_n3 UL to Band 1 DL.

## 5.1.88	DC_3-20_n38

- 5.1.88.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20_n38 | CA_3-20 | n38 | No |

- 5.1.88.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A | CA_3A-20A | n38 |

- 5.1.88.3		∆TIB and ∆RIB values

For DC_3-20_n38, the TIB,c and RIB,c values are reused from the LTE combination CA_3-7-20, and are given in the tables below.

Table 5.1.88.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n38 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |

Table 5.1.88.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n38 | 3 | 0 |
|  | 20 | 0 |
|  | n38 | 0 |

- 5.1.88.4	REFSENS requirements

Co-existence analysis from DC_3_n38 shows that there are IMD2 and IMD3 impact from DC_3_n38 UL to Band 20 DL. The MSD values for the impact on band 20 DL are reused from DC_3A-28A_n41A, which also have IMD2 and IMD3 to the DL of the low band (but only have IMD2 specified).

Co-existence analysis from DC_20_n38 shows that there no impact from DC_20_n38 UL to Band 3 DL.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | FDD | N/A |
|  | 20 | 852 | 10 | 20 | 811 | 26.0 | FDD | IMD21 |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | FDD | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 5.1.89	DC_1-20_n38

- 5.1.89.1	Operating bands for DC

Table 5.1.89.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-20_n38 | CA_1-20 | n38 | NO |

- 5.1.89.2	Configurations for DC

Table 5.1.89.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A | CA_1A-20A | n38 |

- 5.1.89.3	Co-existence studies

The co-existence studies of band 20 + band n38 have been captured in TR 37.716-11-11, where there are no additional harmonic and IMD impact for the additional band receiver.

There is no additional harmonic and IMD impact for band 20 Rx with DC_1A_n38A uplink.

- 5.1.89.4	∆TIB and ∆RIB values

Table 5.1.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |

Table 5.1.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n38 | 1 | 0 |
|  | 20 | 0 |
|  | n38 | 0 |

- 5.1.89.5	REFSENS requirements

According the co-existing studies, there is no need to specify additional MSD requirement for this UL DC configuration.

## 5.1.90	DC_3-20_n38

- 5.1.90.1	Operating bands for DC

Table 5.1.90.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-20_n38 | CA_3-20 | n38 | NO |

- 5.1.90.2	Configurations for DC

Table 5.1.90.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-20A_n38A | DC_20A_n38A | CA_3A-20A | n38 |

- 5.1.90.3	Co-existence studies

The co-existence studies of band 20 + band n38 have been captured in TR 37.716-11-11, where there are no additional harmonic and IMD impact for the additional band receiver.

## 5.1.91	DC_1-3_n77

- 5.1.91.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n77 | CA_1-3 | n77 | DC_1_n77 DC_3_n77 |

- 5.1.91.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n77(2A) | DC_1A_n77A DC_3A_n77A | CA_1A-3A | CA_n77(2A) |

- 5.1.91.3	Co-existence Studies

Refer to TR 37.863-01-01 section 6.35.3 for DC_1A-3A_n77.

- 5.1.91.4	∆TIB and ∆RIB values

Refer to TR 37.863-01-01 section 6.35.4 for DC_1A-3A_n77.

- 5.1.91.5	REFSENS requirements

Refer to TR 37.863-01-01 section 6.35.5 for DC_1A-3A_n77.

## 5.1.92		DC_2-4_n38

- 5.1.92.1	 Operating bands for DC

Table 5.1.92.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-4_n38 | CA_2-4 | n38 | no |

- 5.1.92.2 	Configuration for DC

Table 5.1.92.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A | CA_2-4 | n38 |

- 5.1.92.3	 Co-existence studies

For UE coexistence study of Band 2 + Band n38, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.92.3-1.

Table 5.1.92.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 660 | 770 | 4420 | 4530 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1080 | 1250 | 3230 | 3390 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6270 | 6440 | 6990 | 7150 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2930 | 3160 | 5800 | 6010 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8120 | 8350 | 9560 | 9770 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1540 | 1320 | 8840 | 9060 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8630 | 8370 | 5070 | 4780 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4160 | 3890 | 590 | 310 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12130 | 12390 | 9970 | 10260 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11410 | 11680 | 10690 | 10970 |

For UE coexistence study of Band 4 + Band n38, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.92.3-1.

Table 5.1.92.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7020 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8775 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 815 | 910 | 4280 | 4375 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 940 | 3385 | 3530 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5990 | 6130 | 6850 | 6995 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2510 | 2695 | 5955 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7700 | 7885 | 9420 | 9615 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1820 | 1630 | 8560 | 8750 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8770 | 8525 | 4450 | 4220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4440 | 4200 | 125 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11990 | 12235 | 9410 | 9640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11130 | 11370 | 10270 | 10505 |

Based on the tables, there is no co-existence issue.

- 5.1.92.4	 ∆TIB and ∆RIB values

Table 5.1.92.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-4_n38 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n38 | 0.5 |

Table 5.1.92.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-4_n38 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | n38 | 0.5 |

- 5.1.92.5	REFSENS requirements

There is no additional MSD requirement for this configuration.

## 5.1.93		DC_2-4_n41

- 5.1.93.1	 Operating bands for DC

Table 5.1.93.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-4_n41 | CA_2-4 | n41 | no |

- 5.1.93.2 	Configuration for DC

Table 5.1.93.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A | CA_2-4 | n41 |

- 5.1.93.3	 Co-existence studies

For UE coexistence study of Band 2 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.93.3-1.

Table 5.1.93.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 586 | 840 | 4346 | 4600 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1010 | 1324 | 3082 | 3530 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6196 | 6510 | 6842 | 7290 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2860 | 3234 | 5578 | 6220 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8046 | 8420 | 9338 | 9980 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1680 | 1172 | 8692 | 9200 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8910 | 8074 | 5144 | 4710 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4370 | 3668 | 738 | 170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11834 | 12670 | 9896 | 10330 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11188 | 11890 | 10542 | 11110 |

For UE coexistence study of Band 4 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.93.3-1.

Table 5.1.93.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3510 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5265 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7020 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8775 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 741 | 980 | 4206 | 4445 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1014 | 3237 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6200 | 6702 | 7135 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2769 | 5733 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 7955 | 9198 | 9825 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1482 | 8412 | 8890 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8229 | 4524 | 4150 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3978 | 273 | 250 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12515 | 9336 | 9710 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11580 | 10122 | 10645 |

Based on the tables, 4th order IMD generated by dual uplink of Band 4 + Band n41 may also fall into own Rx of band 2.

- 5.1.93.4	 ∆TIB and ∆RIB values

Table 5.1.93.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-4_n41 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n41 | 0.5 |

Table 5.1.93.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-4_n41 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | n41 | 0.5 |

- 5.1.93.5	REFSENS requirements

The MSD requirements are similar to DC_2A-66A_n41A which has specified in 38.101-3, so we can reuse it. Table 5.1.93.5-1 lists the required MSD test configurations for the EN-DC 2A-4A_n41A band combination.

Table 5.1.93.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-4A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 ¦2*fBn41-2*fB4¦ |
|  | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |

## 5.1.94		DC_1-3_n41

- 5.1.94.1	 Operating bands for DC

Table 5.1.94.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n41 | CA_1-3 | n41 | no |

- 5.1.94.2 	Configuration for DC

Table 5.1.94.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n41A DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A | CA_1-3 | n41 |

- 5.1.94.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.94.3-1.

Table 5.1.94.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 | 1464 | 3012 | 3460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6650 | 6912 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 | 3444 | 5508 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 | 8630 | 9408 | 10050 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1540 | 1032 | 8832 | 9340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 | 8004 | 5424 | 4990 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 | 3528 | 948 | 380 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 | 12740 | 10176 | 10610 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 | 12030 | 10752 | 11320 |

For UE coexistence study of Band 3 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.94.3-1.

Table 5.1.94.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1422 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

Based on the tables, there is no co-existence issue.

- 5.1.94.4	 ∆TIB and ∆RIB values

Table 5.1.94.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 5.1.94.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n41 | 1 | 0 |
|  | 3 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

- 5.1.94.5	REFSENS requirements

There is no additional MSD requirements for DC_1-3_n41.

## 5.1.95		DC_7-20_n1

- 5.1.95.1	 Operating bands for DC

Table 5.1.95.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-20_n1 | CA_7-20 | n1 | no |

- 5.1.95.2 	Configuration for DC

Table 5.1.95.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-20A_n1A | DC_7A_n1A DC_20A_n1A | CA_7-20 | n1 |
| DC_7C-20A_n1A | DC_7A_n1A DC_20A_n1A DC_7C_n1A | CA_7-20 | n1 |

- 5.1.95.3	 Co-existence studies

For UE coexistence study of Band 7 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.95.3-1.

Table 5.1.95.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 650 | 520 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3020 | 3220 | 1270 | 1460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6920 | 7120 | 6340 | 6530 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5520 | 5790 | 3190 | 3440 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9420 | 9690 | 8260 | 8510 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1040 | 1300 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5420 | 5110 | 8360 | 8020 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 940 | 620 | 3870 | 3540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10180 | 10490 | 11920 | 12260 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10760 | 11080 | 11340 | 11670 |

For UE coexistence study of Band 20 + Band n1, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.95.3-2.

Table 5.1.95.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1058 | 1148 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 316 | 196 | 2978 | 3128 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3584 | 3704 | 4672 | 4822 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 516 | 666 | 4898 | 5108 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4416 | 4566 | 6592 | 6802 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2296 | 2116 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7088 | 6818 | 1528 | 1348 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4276 | 4036 | 1254 | 1464 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 | 8782 | 5248 | 5428 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 | 7664 | 6336 | 6546 |

Based on the tables, 5th order IMD generated by dual uplink of Band 7 + Band n1 may also fall into own Rx of band 20.

- 5.1.95.4	 ∆TIB and ∆RIB values

Table 5.1.95.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n1 | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |

Table 5.1.95.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20_n1 | 7 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |

- 5.1.95.5	REFSENS requirements

The MSD requirements are similar to CA_1A-7A-28A with UL band 1 + band 7 which has specified in 36.101, so it can be reused. Table 5.1.95.5-1 lists the required MSD test configurations for the EN-DC 7A-20A_n1A band combination.

Table 5.1.95.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 ¦2*fB7-3*fBn1¦ |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |

## 5.1.96	DC_2-12_n2

- 5.1.96.1	Operating bands for DC

Table 5.1.96.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-12_n2 | CA_2-12 | n2 | No |

- 5.1.96.2	Configuration for DC

Table 5.1.96.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-12A_n2A | DC_12A_n2A | CA_2A-12A | n2A |

- 5.1.96.3	Coexistence studies

Based on co-existence studies of DC_12A_n2A in TR 37.716-01-01, there is no harmonic or IMD impact to any of own Rx bands.

- 5.1.96.4	∆TIB and ∆RIB values

For DC_2-12_n2, the TIB,c and RIB,c values are derived from CA_2-2-12 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n2 | 2 | 0.3 |
|  | 12 | 0.3 |
|  | n2 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n2 | 2 | 0 |
|  | 12 | 0 |
|  | n2 | 0 |

- 5.1.96.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.97	DC_3-28_n7, DC_3-3-28_n7

- 5.1.97.1	Operating bands for EN-DC

Table 5.1.97.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-28_n7 | CA_3-28 | n7 | No |

- 5.1.97.2	Configuration for DC

Table 5.1.97.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_3C_n7B DC_28A_n7B | CA_3A-28A CA_3C-28A | n7 |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B | CA_3A-28A CA_3C-28A | n7 |

- 5.1.97.3		∆TIB and ∆RIB values

For DC_3-28_n7, the TIB,c and RIB,c values are reused from LTE combination CA_3-7-28 and EN-DC combination DC_3-7_n28 and are given in the tables below.

Table 5.1.97.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n7 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n7 | 0.5 |

Table 5.1.97.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n7 | 3 | 0 |
|  | 28 | 0 |
|  | n7 | 0 |

- 5.1.97.4	REFSENS requirements

Co-existence analysis from DC_3_n7 shows that there is IMD2 impact from DC_3_n7 UL to Band 28 DL. MSD values are reused from CA_3A-7A-28A with CA_3A-7A as uplink.

Co-existence analysis from DC_28_n7 shows that there is IMD2 impact from DC_28_n7 UL to Band 3 DL. MSD values are reused from DC_3A-7A_n28A and CA_3A-7A-28A with CA_7A-28A as uplink.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |

## 5.1.98	DC_1-7_n7, DC_1-1-7_n7

- 5.1.98.1	Operating bands for EN-DC

Table 5.1.98.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-7_n7 | CA_1-7 | n7 |  |

- 5.1.98.2	Configuration for DC

Table 5.1.98.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A1 | CA_1A-7A | n7 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A1 | CA_1A-1A-7A | n7 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.98.3		Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.98.4	∆TIB and ∆RIB values

For DC_1-7_n7 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n7 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |

Table 5.1.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n7 | 1 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

- 5.1.98.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.99	DC_3-7_n7, DC_3-3-7_n7

- 5.1.99.1	Operating bands for EN-DC

Table 5.1.99.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-7_n7 | CA_3-7 | n7 |  |

- 5.1.99.2	Configuration for DC

Table 5.1.99.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A1 | CA_3A-7A CA_3C-7A | n7 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A1 | CA_3A-3A-7A | n7 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.99.3		Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.99.4	∆TIB and ∆RIB values

For DC_3-7_n7 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n7 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n7 | 3 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

- 5.1.99.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.100	DC_7-28_n7

- 5.1.100.1	Operating bands for EN-DC

Table 5.1.100.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-28_n7 | CA_7-28 | n7 |  |

- 5.1.100.2	Configuration for DC

Table 5.1.100.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n7A | DC_7A_n7A1 DC_28A_n7A | CA_7A-28A | n7 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.100.3		Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.100.4	∆TIB and ∆RIB values

For DC_7-28_n7 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n7 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n7 | 0.3 |

Table 5.1.100.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n7 | 7 | 0 |
|  | 28 | 0 |
|  | n7 | 0 |

- 5.1.100.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.101	DC_12-66_n66

- 5.1.101.1	Operating bands for EN-DC

Table 5.1.101.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 12-66_n66 | CA_12-66 | n66 |  |

- 5.1.101.2	Configuration for DC

Table 5.1.101.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A1 | CA_12A-66A | n66 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.101.3		Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

- 5.1.101.4	∆TIB and ∆RIB values

For DC_12-66_n66 the TIB,c and RIB,c values are given in the tables below.

Table 5.1.101.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n66 | 12 | 0.8 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.101.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n66 | 12 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

- 5.1.101.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.102	DC_1-28_n7

- 5.1.102.1	Operating bands for EN-DC

Table 5.1.102.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-28_n7 | CA_1-28 | n7 | No |

- 5.1.102.2	Configuration for DC

Table 5.1.102.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B | CA_1A-28A | n7 CA_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B | CA_1A-1A-28A | n7 CA_n7B |

- 5.1.102.3		∆TIB and ∆RIB values

For DC_1-28_n7, the TIB,c and RIB,c values are reused from LTE combination CA_1-7-28 and DC_1-7_n28, and are given in the tables below.

Table 5.1.102.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n7 | 1 | 0.5 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |

Table 5.1.102.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n7 | 1 | 0 |
|  | 28 | 0.2 |
|  | n7 | 0 |

- 5.1.102.4	REFSENS requirements

Co-existence analysis from DC_28_n7 shows that there is no impact from DC_28_n7 UL to Band 1 DL.

Co-existence analysis from DC_1_n7 shows that there is IMD5 impact from DC_1_n7 UL to Band 28 DL. MSD values are reused from CA_1A-7A-28A with CA_1A-7A in uplink.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |

## 5.1.103	DC_2-5_n66

- 5.1.103.1	Operating bands for DC

Table 5.1.103.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n66 | CA_2-5 | n66 | No |

- 5.1.103.2	Configuration for DC

Table 5.1.103.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2-5_n66 | DC_2A_n66A DC_5A_n66A | CA_2A-5A | n66A |

- 5.1.103.3	Coexistence studies

Based on co-existence studies of DC_2A_n66A in TR 37.863-01-01, the third order IMD may fall into band 2 and/or n66; the fifth order IMD may fall into band n66.

Based on co-existence studies of DC_5A_n66A in TR 37.863-01-01, the second order IMD may fall into band 5.

- 5.1.103.4	∆TIB and ∆RIB values

For DC_2-5_n66, the TIB,c and RIB,c values are derived from CA_2-5-66 in TS 36.101.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.103.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | n66 | 0.3 |

- 5.1.103.5	REFSENS requirements

The MSD due to IMD products are already specifed for 2DL2UL DC combos (DC_2A_n66A and DC_5A_n66A) in TS 38.101-3. There is no specific MSD for 3DL2UL DC.

## 5.1.104	DC_2-5_n2 and DC_2-5-5_n2

- 5.1.104.1	Operating bands for DC

Table 5.1.104.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n2 | CA_2-5 | n2 | No |
| DC_2-5-5_n2 | CA_2-5-5 | n2 | No |

- 5.1.104.2	Configuration for DC

Table 5.1.104.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A_n2A | DC_5A_n2A | CA_2A-5A | n2A |
| DC_2A-5B_n2A | DC_5A_n2A | CA_2A-5B | n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A | CA_2A-5A-5A | n2A |

- 5.1.104.3	Coexistence studies

Based on co-existence studies of DC_5A_n2A in TR 37.716-11-11, there is no harmonic or IMD issues for this band combination,.

- 5.1.104.4	∆TIB and ∆RIB values

For DC_2-5_n2, the ∆TIB,c and ∆RIB,c values reused the values of DC_5A_n2A in TS 38.101-3.

Table 5.1.104.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2 DC_2-5-5_n2 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n2 | 0.3 |

Table 5.1.104.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2 DC_2-5-5_n2 | 2 | 0 |
|  | 5 | 0 |
|  | n2 | 0 |

- 5.1.104.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.105	DC_2-5_n5 and DC_2-2-5_n5

- 5.1.105.1		Operating bands for DC

Table 5.1.105.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n5 | CA_2-5 | n5 | No |
| DC_2-2-5_n5 | CA_2-2-5 | n5 | No |

- 5.1.105.2	Configuration for DC

Table 5.1.105.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A_n5A | DC_2A_n5A | CA_2A-5A | n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A | CA_2A-2A-5A | n5A |

- 5.1.105.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no harmonic or IMD issues for this band combination.

- 5.1.105.4	∆TIB and ∆RIB values

For DC_2-5_n5 and DC_2-2-5_n5, the ∆TIB,c and ∆RIB,c values are given in the tables below. Numbers come from LTE CA_2A-5A.

Table 5.1.105.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n5 DC_2-2-5_n5 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.105.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n5 DC_2-2-5_n5 | 2 | 0 |
|  | 5 | 0 |
|  | n5 | 0 |

### 5.1.105.4	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.106	DC_2-13_n2

- 5.1.106.1	Operating bands for DC

Table 5.1.106.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-13_n2 | CA_2-13 | n2 | No |

- 5.1.106.2	Configuration for DC

Table 5.1.106.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-13A_n2A | DC_13A_n2A | CA_2A-13A | n2A |

- 5.1.106.3	Coexistence studies

Based on co-existence studies of DC_13A_n2A, there is no harmonic or IMD issues for this band combination,.

- 5.1.106.4	∆TIB and ∆RIB values

For DC_2-13_n2, the ∆TIB,c and ∆RIB,c values reused the values of CA_2A-13A in TS 36.101.

Table 5.1.106.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n2 | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n2 | 0.3 |

Table 5.1.106.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n2 | 2 | 0 |
|  | 13 | 0 |
|  | n2 | 0 |

- 5.1.106.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.107	DC_2-13_n5 and DC_2-2-13_n5

- 5.1.107.1		Operating bands for DC

Table 5.1.107.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-13_n5 | CA_2-13 | n5 | No |
| DC_2-2-13_n5 | CA_2-2-13 | n5 | No |

- 5.1.107.2	Configuration for DC

Table 5.1.107.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-13A_n5A | DC_2A_n5A | CA_2A-13A | n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A | CA_2A-2A-13A | n5A |

- 5.1.107.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no harmonic or IMD issues for this band combination.

- 5.1.107.4	∆TIB and ∆RIB values

For DC_2-13_n5 and DC_2-2-13_n5, the ∆TIB,c and ∆RIB,c values are given in the tables below. Numbers come from LTE CA_2A-5A-13A

Table 5.1.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n5 DC_2-2-13_n5 | 2 | 0.3 |
|  | 13 | 0.5 |
|  | n5 | 0.5 |

Table 5.1.107.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n5 DC_2-2-13_n5 | 2 | 0 |
|  | 13 | 0 |
|  | n5 | 0 |

- 5.1.107.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.108	DC_5-5-66_n66 and DC_5-5-66-66_n66

- 5.1.108.1	Operating bands for DC

Table 5.1.108.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-5-66_n66 | CA_5-5-66 | n66 | No |
| DC_5-5-66-66_n66 | CA_5-5-66-66 | n66 | No |

- 5.1.108.2	Configuration for DC

Table 5.1.108.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-5A-66A_n66A | DC_5A_n66A | CA_5A-5A-66A | n66A |
| DC_5A-5A-66A-66A_n66A | DC_5A_n66A | CA_5A-5A-66A-66A | n66A |

- 5.1.108.3	Coexistence studies

Based on co-existence studies of DC_5A_n66A in TR 37.863-01-01, there is no harmonic or IMD issues for this band combination.

- 5.1.108.4	∆TIB and ∆RIB values

For DC_5-5-66_n66 and DC_5-5-66-66_n66, the ∆TIB,c and ∆RIB,c values reused the values for DC_5_n66 in TS 38.101-3.

Table 5.1.108.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-5-66_n66 DC_5-5-66-66_n66 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.108.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-5-66_n66 DC_5-5-66-66_n66 | 5 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

- 5.1.108.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.109	DC_5-13_n2

- 5.1.109.1	Operating bands for DC

Table 5.1.109.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-13_n2 | CA_5-13 | n2 | No |

- 5.1.109.2	Configuration for DC

Table 5.1.109.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A | CA_5A-13A | n2A |

- 5.1.109.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no harmonic or IMD issues for this band combination.

- 5.1.109.4	∆TIB and ∆RIB values

For DC_5-13_n2, the TIB,c and RIB,c values reused the values of CA_2A-5A-13A in TS 36.101.

Table 5.1.109.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-13_n2 | 5 | 0.5 |
|  | 13 | 0.5 |
|  | n2 | 0.3 |

Table 5.1.109.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-13_n2 | 5 | 0 |
|  | 13 | 0 |
|  | n2 | 0 |

- 5.1.109.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.110	DC_5-66_n66 and DC_5-66-66_n66

- 5.1.110.1	Operating bands for DC

Table 5.1.110.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n66 | CA_5-66 | n66 | No |
| DC_5-66-66_n66 | CA_5-66-66 | n66 | No |

- 5.1.110.2	Configuration for DC

Table 5.1.110.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5B-66A_n66A | DC_5A_n66A | CA_5B-66A | n66A |
| DC_5A-66A-66A_n66A | DC_5A_n66A | CA_5A-66A-66A | n66A |
| DC_5B-66A-66A_n66A | DC_5A_n66A | CA_5B-66A-66A | n66A |

- 5.1.110.3	Coexistence studies

Based on co-existence studies of DC_5A_n66A in TR 37.863-01-01, there is no harmonic or IMD issues for this band combination.

- 5.1.110.4	∆TIB and ∆RIB values

For DC_5-66_n66 and DC_5-66-66_n66, the TIB,c and RIB,c values reused the values for DC_5_n66 in TS 38.101-3.

Table 5.1.110.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n66 DC_5-66-66_n66 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.110.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n66 DC_5-66-66_n66 | 5 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

- 5.1.110.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.111	DC_5-66_n78

- 5.1.111.1	Operating bands for DC

Table 5.1.111.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n78 | CA_5-66 | n78 | NO |

- 5.1.111.2	Configurations for DC

Table 5.1.111.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A | CA_5A-66A | n78 |

- 5.1.111.3	Co-existence studies

The co-existence studies of band 66 + band n78 are listed in table 5.1.111.3-1.

Table 5.1.111.3-1 Band 66 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1780 | 3300 | 3800 |
| 2nd order harmonics frequency range (MHz) | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd order harmonics frequency range (MHz) | 3420 | 3560 | 6600 | 7600 |
| 2nd order harmonics frequency range (MHz) | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd order harmonics frequency range (MHz) | 5130 | 5340 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 2090 | 5010 | 5580 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 260 | 4820 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7360 | 8310 | 9380 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3040 | 10020 | 11160 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2040 | 8120 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9140 | 11610 | 13180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11420 | 3820 | 3040 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16980 | 10140 | 10920 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6340 | 1260 | 2470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14960 | 11730 | 12940 |

According to table 5.1.111.3-1,  no additional harmonic and IMD product of band 66 + band n78 will fall into band 5 Rx(869MHz~894MHz).

The co-existence studies of band 5 + band n78 have been captured in TR 37.863-01-01 in Rel-15, where:

-	the IMD3 of the frequency range from 1602MHz~2152MHz will fall into band 66 Rx (2110MHz~2200MHz).

- 5.1.111.4	∆TIB and ∆RIB values

For DC_5-66_n78, the ∆TIB,c and ∆RIB,c values are given in the tables below.

Table 5.1.111.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n78 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.111.4-2: ΔRIB,c

| Inter-band EN-DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n78 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n78 | 0.5 |

- 5.1.111.5	REFSENS requirements

According the co-existing studies, it shall need to specify additional MSD requirement caused by IMD3 for this UL DC configuration. The requirement is proposed in the table 5.1.x.5-1.

Table 5.1.111.5-1 MSD requirements for DC_5A-66A_n78A

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 13.2 | FDD | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |

## 5.1.112	DC_2-5_n71

- 5.1.112.1	Operating bands for DC

Table 5.1.112.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n71 | CA_2-5 | n71 |  |

- 5.1.112.2	Configuration for DC

Table 5.1.112.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A | CA_2A-5A | n71A |

- 5.1.112.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes except the impact on own Rx on the 3rd band due to IMD. From Table 5.1.112.3-1 we can see that UL DC_2A_n71A has IMD5 interfering band 5 DL.

Table 5.1.112.3-1: UL DC_2A_n71A impact to band 5 DL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 1989 | 2094 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1247 | 1152 | 2513 | 2608 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3002 | 3157 | 584 | 454 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4363 | 4518 | 3176 | 3306 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4852 | 5067 | 79 | 244 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6213 | 6428 | 3839 | 4004 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2304 | 2494 | 5026 | 5216 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 942 | 742 | 6977 | 6702 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4502 | 4702 | 8063 | 8338 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1606 | 1831 | 4404 | 4154 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5689 | 5914 | 6876 | 7126 |

Table 5.1.112.3-2: UL DC_5A_n71A impact to  band 2 DL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 1989 | 2094 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 186 | 126 | 1487 | 1547 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 950 | 1035 | 477 | 572 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2311 | 2396 | 2150 | 2245 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 1774 | 1884 | 1140 | 1270 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3135 | 3245 | 2813 | 2943 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 252 | 372 | 2974 | 3094 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1968 | 1803 | 2733 | 2598 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3476 | 3641 | 3959 | 4094 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 446 | 291 | 1221 | 1076 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3637 | 3792 | 3798 | 3943 |

- 5.1.112.4	∆TIB and ∆RIB values

For DC_2-5_n71, the ∆TIB,c and ∆RIB,c values are derived from DC_5_n71 and DC_5_n2.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n71 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | n71 | 0.5 |

Table 5.1.112.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n71 | 2 | 0 |
|  | 5 | 0 |
|  | n71 | 0 |

- 5.1.112.5	REFSENS requirements

MSD need to be specified for the case that UL DC_2A_n71A has IMD5 interfering band 5 DL. MSD value is reused from DC_5A-66A_n71.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n71 | 686.5 | 5 | 25 | 640.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |

## 5.1.113	DC_2-48_n71

- 5.1.113.1	Operating bands for DC

Table 5.1.113.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-48_n71 | CA_2-48 | n71 |  |

- 5.1.113.2	Configuration for DC

Table 5.1.113.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A | CA_2A-48A | n71A |

- 5.1.113.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes except the impact on own Rx on the 3rd band due to IMD. From Table 5.1.113.3-1 and Table 5.1.113.3-2 we can see that there are no IMD interfering the 3rd DL.

Table 5.1.113.3-1: UL DC_2A_n71A impact to band 48 DL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 1989 | 2094 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1247 | 1152 | 2513 | 2608 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3002 | 3157 | 584 | 454 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4363 | 4518 | 3176 | 3306 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4852 | 5067 | 79 | 244 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6213 | 6428 | 3839 | 4004 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2304 | 2494 | 5026 | 5216 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 942 | 742 | 6977 | 6702 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4502 | 4702 | 8063 | 8338 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1606 | 1831 | 4404 | 4154 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5689 | 5914 | 6876 | 7126 |

Table 5.1.113.3-2: UL DC_48A_n71A impact to band 2 DL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 1989 | 2094 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3037 | 2852 | 4213 | 4398 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6402 | 6737 | 2374 | 2154 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7763 | 8098 | 4876 | 5096 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 9952 | 10437 | 1711 | 1456 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11313 | 11798 | 5539 | 5794 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5704 | 6074 | 8426 | 8796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 758 | 1048 | 14137 | 13502 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6202 | 6492 | 14863 | 15498 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5006 | 5411 | 9774 | 9254 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9089 | 9494 | 11976 | 12496 |

- 5.1.113.4	∆TIB and ∆RIB values

For DC_2-48_n71, the ∆TIB,c and ∆RIB,c values are derived from DC_2_n48 and DC_2_n71.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n71 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n71 | 0.3 |

Table 5.1.113.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n71 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n71 | 0 |

- 5.1.113.5	REFSENS requirements

No need for REFSENS requirement as shown in coexistence studies paragraph.

## 5.1.114	DC_5-48_n71

- 5.1.114.1	Operating bands for DC

Table 5.1.114.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-48_n71 | CA_5-48 | n71 |  |

- 5.1.114.2	Configuration for DC

Table 5.1.114.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A | CA_5A-48A | n71A |

- 5.1.114.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes except the impact on own Rx on the 3rd band due to IMD. From Table 5.1.114.3-1 and Table 5.1.114.3-2 we can see that DC_5A_n71A uplink has IMD5 to band 48 and from Table 5.1.114.3-2 we can see that DC_48A_n71A uplink has IMD5 to band 5.

Table 5.1.114.3-1: UL DC_5A_n71A impact to band 48 DL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 1989 | 2094 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 186 | 126 | 1487 | 1547 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 950 | 1035 | 477 | 572 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2311 | 2396 | 2150 | 2245 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 1774 | 1884 | 1140 | 1270 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3135 | 3245 | 2813 | 2943 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 252 | 372 | 2974 | 3094 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1968 | 1803 | 2733 | 2598 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3476 | 3641 | 3959 | 4094 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 446 | 291 | 1221 | 1076 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3637 | 3792 | 3798 | 3943 |

Table 5.1.114.3-2: UL DC_48A_n71A impact to band 5 DL

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3550 | 3700 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 7100 | 7400 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 10650 | 11100 | 1989 | 2094 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3037 | 2852 | 4213 | 4398 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6402 | 6737 | 2374 | 2154 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7763 | 8098 | 4876 | 5096 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 9952 | 10437 | 1711 | 1456 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11313 | 11798 | 5539 | 5794 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5704 | 6074 | 8426 | 8796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 758 | 1048 | 14137 | 13502 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6202 | 6492 | 14863 | 15498 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5006 | 5411 | 9774 | 9254 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9089 | 9494 | 11976 | 12496 |

- 5.1.114.4	∆TIB and ∆RIB values

For DC_5-48_n71, the ΔTIB,c and ΔRIB,c values are derived from DC_5_n48 and DC_5_n71.

Table 5.1.1.114.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48_n71 | 5 | 0.5 |
|  | 48 | 0.3 |
|  | n71 | 0.5 |

Table 5.1.114.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-48_n71 | 5 | 0 |
|  | 48 | 0 |
|  | n71 | 0 |

- 5.1.114.5	REFSENS requirements

MSD need to be specified for the case that UL DC_5A_n71A has IMD5 interfering band 48 DL and UL DC_48A_n71A has IMD5 interfering band 5 DL.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-48A_n71A | 5 | TBD | 5 | 25 | TBD | N/A | N/A |
|  | n71 | TBD | 5 | 25 | TBD | N/A | N/A |
|  | 48 | TBD | 5 | 25 | TBD | TBD | IMD5 |
|  | 48 | TBD | 5 | 25 | TBD | N/A | N/A |
|  | n71 | TBD | 5 | 25 | TBD | N/A | N/A |
|  | 5 | TBD | 5 | 25 | TBD | TBD | IMD5 |

## 5.1.115	DC_48_(n)12

- 5.1.115.1	Operating bands for DC

Table 5.1.115.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_48_(n)12 | CA_48-12 | n12 |  |

- 5.1.115.2	Configuration for DC

Table 5.1.115.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_48A_(n)12AA | DC_48A_n12A DC_(n)12AA1 | CA_48A-12A | n12A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.115.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes.

- 5.1.115.4	∆TIB and ∆RIB values

For DC_48_(n)12 the ΔTIB,c and ΔRIB,c values are derived from high-low band combination agreement.

Table 5.1.115.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_(n)12 | 12 | 0.3 |
|  | n12 | 0.3 |
|  | 48 | 0.3 |

Table 5.1.115.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_48_(n)12 | 12 | 0 |
|  | n12 | 0 |
|  | 48 | 0 |

- 5.1.115.5	REFSENS requirements

No additional REFSENS requirement is needed.

## 5.1.116	DC_66_(n)12

- 5.1.116.1	Operating bands for DC

Table 5.1.116.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66_(n)12 | CA_66-12 | n12 |  |

- 5.1.116.2	Configuration for DC

Table 5.1.116.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A_(n)12AA | DC_66A_n12A DC_(n)12AA1 | CA_66A-12A | n12A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.116.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes.

- 5.1.116.4	∆TIB and ∆RIB values

For DC_66_(n)12 the ΔTIB,c and ΔRIB,c values are derived from DC_12_n66 combination.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_(n)12 | 12 | 0.8 |
|  | n12 | 0.8 |
|  | 66 | 0.5 |

Table 5.1.116.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_(n)12 | 12 | 0 |
|  | n12 | 0 |
|  | 66 | 0 |

- 5.1.116.5	REFSENS requirements

No additional REFSENS requirement is needed.

## 5.1.117	DC_2-66_n38

- 5.1.117.1	Operating bands for DC

Table 5.1.117.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n38 | CA_2-66 | n38 | no |
| DC_2-66-66_n38 | CA_2-66-66 | n38 | no |

- 5.1.117.2	Configuration for DC

Table 5.1.117.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A | CA_2A-66A | n38A |
| DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A | CA_2A-66A-66A | n38A |

- 5.1.117.3	Co-existence Studies

Based on co-existence studies of DC_2_n38 and DC_6_n38, as captured in TR 37.716-11-11, no own Rx impact on the 3rd band is observed.

- 5.1.117.4	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 5.1.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n38 DC_2-66-66_n38 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n38 | 0.9 |

Table 5.1.117.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n38 DC_2-66-66_n38 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |

- 5.1.117.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.1.118	DC_1-18_n3

- 5.1.118.1	Operating bands for DC

Table 5.1.118.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-18_n3 | CA_1-18 | n3 | DC_1_n3 |

- 5.1.118.2	Configurations for DC

Table 5.1.118.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A | CA_1A-18A | n3 |

- 5.1.118.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n3 and Band-18 + Band n3 captured in TR 37.716-11-11, own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 1 + Band n3 doesn’t fall into own Rx of band 18.

-	IMD generated by dual uplink of Band 18 + Band n3 doesn’t fall into own Rx of band 1.

- 5.1.118.4	∆TIB and ∆RIB values

Table 5.1.118.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18-n3 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.118.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18-n3 | 1 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |

- 5.1.118.5	REFSENS requirements

As stated in 5.1.118.3, there is no additional MSD requirement for this configuration.

## 5.1.119	DC_1-28_n3

- 5.1.119.1	Operating bands for DC

Table 5.1.119.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-28_n3 | CA_1-28 | n3 | DC_1_n3 |

- 5.1.119.2	Configurations for DC

Table 5.1.119.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A | CA_1A-28A | n3 |

- 5.1.119.3	Co-existence studies

Based on co-existence studies of Band 1 + Band n3 and Band-28 + Band n3 captured in TR 37.716-11-11, own Rx impact of the 3rd band is the followings.

-	IMD generated by dual uplink of Band 1 + Band n3 doesn’t fall into own Rx of band 28.

-	4th IMD generated by dual uplink of Band 28 + Band n3 falls into own Rx of band 1.

- 5.1.119.4	∆TIB and ∆RIB values

Table 5.1.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28-n3 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |

Table 5.1.119.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28-n3 | 1 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |

- 5.1.119.5	REFSENS requirements

The MSD value for the impact on band 1 DL by dual uplink of Band 28 + Band n3 stated in 5.1.119.3 is shown in the following table.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |

## 5.1.120	DC_3-20_n7

- 5.1.120.1	Operating bands for EN-DC

Table 5.1.120.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20_n7 | CA_3-20 | n7 | no |

- 5.1.120.2	Configuration for DC

Table 5.1.120.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A | CA_3A-20A CA_3C-20A | n7 |

- 5.1.120.3		∆TIB and ∆RIB values

For DC_3-20_n7, the TIB,c and RIB,c values are reused from the LTE combination CA_3-7-20 given in the tables below.

Table 5.1.120.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n7 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.5 |

Table 5.1.120.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n7 | 3 | 0 |
|  | 20 | 0 |
|  | n7 | 0 |

- 5.1.120.4	REFSENS requirements

There are IMD2 impacts from DC_3_n7 UL to Band 20 DL. The MSD value for band 20 DL is reused from LTE CA_3-7-20 in TS 36.101.

Table 5.1.120.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | FDD | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | FDD | IMD2 |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | FDD | N/A |

## 5.1.121		DC_1-3_n8

- 5.1.121.1	 Operating bands for DC

Table 5.1.121.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n8 | CA_1-3 | n8 | no |

- 5.1.121.2 	Configuration for DC

Table 5.1.121.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A | CA_1-3 | n8 |

- 5.1.121.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.121.3-1.

Table 5.1.121.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 | 795 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2690 | 25 | 120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 | 4485 | 3470 | 3615 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 | 4475 | 855 | 1035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 | 6270 | 4350 | 4530 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1590 | 1810 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 1735 | 6260 | 5925 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 | 930 | 3595 | 3300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 | 5445 | 7720 | 8055 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 | 6315 | 6890 | 7185 |

For UE coexistence study of Band 1 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.121.3-1.

Table 5.1.121.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on the tables, there is no co-existence issue.

- 5.1.121.4	 ∆TIB and ∆RIB values

Table 5.1.121.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n8 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n8 | 0.3 |

Table 5.1.121.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n8 | 1 | 0 |
|  | 3 | 0 |
|  | n8 | 0 |

- 5.1.121.5	REFSENS requirements

There is no additional MSD requirement for this configuration.

## 5.1.122		DC_1-7_n8

- 5.1.122.1	 Operating bands for DC

Table 5.1.122.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n8 | CA_1-7 | n8 | no |

- 5.1.122.2 	Configuration for DC

Table 5.1.122.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A | CA_1-7 | n8 |

- 5.1.122.3	 Co-existence studies

For UE coexistence study of Band 7 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.122.3-1.

Table 5.1.122.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1690 | 1585 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4085 | 4260 | 810 | 670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5880 | 6055 | 4260 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6585 | 6830 | 70 | 245 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8380 | 8625 | 5140 | 5315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3380 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 950 | 9400 | 9085 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2255 | 2500 | 5950 | 5670 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6020 | 6230 | 10880 | 11195 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7640 | 7885 | 9260 | 9540 |

For UE coexistence study of Band 1 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.122.3-1.

Table 5.1.122.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

Based on the tables, there is no co-existence issue.

- 5.1.122.4	 ∆TIB and ∆RIB values

Table 5.1.122.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n8 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |

Table 5.1.122.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n8 | 1 | 0 |
|  | 7 | 0 |
|  | n8 | 0.2 |

- 5.1.122.5	REFSENS requirements

There is no additional MSD requirement for this configuration.

## 5.1.123		DC_1-20_n8

- 5.1.123.1	 Operating bands for DC

Table 5.1.123.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-20_n8 | CA_1-20 | n8 | no |

- 5.1.123.2 	Configuration for DC

Table 5.1.123.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A | CA_1-20 | n8 |

- 5.1.123.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.123.3-1.

Table 5.1.123.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18 | 83 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 749 | 844 | 898 | 998 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2544 | 2639 | 2592 | 2692 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 | 1706 | 1778 | 1913 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3501 | 3472 | 3607 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 166 | 36 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2828 | 2658 | 2568 | 2413 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1081 | 916 | 826 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 | 4522 | 4208 | 4363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 | 4469 | 4256 | 4416 |

For UE coexistence study of Band 1 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.123.3-1.

Table 5.1.123.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

IMD4 of Tx band 1 + band n8 may fall into Rx of band 20.

- 5.1.123.4	 ∆TIB and ∆RIB values

Table 5.1.123.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n8 | 1 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |

Table 5.1.123.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n8 | 1 | 0 |
|  | 20 | 0 |
|  | n8 | 0 |

- 5.1.123.5	REFSENS requirements

Table 5.1.123.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1-20_n8 | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |

## 5.1.124		DC_3-7_n8

- 5.1.124.1	 Operating bands for DC

Table 5.1.124.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n8 | CA_3-7 | n8 | no |

- 5.1.124.2 	Configuration for DC

Table 5.1.124.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A | CA_1-7 | n8 |

- 5.1.124.3	 Co-existence studies

For UE coexistence study of Band 7 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.124.3-1.

Table 5.1.124.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1690 | 1585 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4085 | 4260 | 810 | 670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5880 | 6055 | 4260 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6585 | 6830 | 70 | 245 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8380 | 8625 | 5140 | 5315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3380 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 950 | 9400 | 9085 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2255 | 2500 | 5950 | 5670 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6020 | 6230 | 10880 | 11195 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7640 | 7885 | 9260 | 9540 |

For UE coexistence study of Band 3 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.124.3-1.

Table 5.1.124.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 | 795 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2690 | 25 | 120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 | 4485 | 3470 | 3615 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 | 4475 | 855 | 1035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 | 6270 | 4350 | 4530 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1590 | 1810 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 1735 | 6260 | 5925 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 | 930 | 3595 | 3300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 | 5445 | 7720 | 8055 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 | 6315 | 6890 | 7185 |

Based on the tables, IMD2 and IMD3 for Tx band 3 + band n8 may fall into Rx of band 7.

- 5.1.124.4	 ∆TIB and ∆RIB values

Table 5.1.124.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n8 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n8 | 0.6 |

Table 5.1.124.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n8 | 3 | 0 |
|  | 7 | 0 |
|  | n8 | 0.2 |

- 5.1.124.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD2 and IMD3 for DC_3-7_n8 with UL DC_3_n8 is specified as below referring to the MSD for CA_3A-7A-8A from 36.101.

Table 5.1.124.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3-7_n8 | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD3x |
| NOTE x: This MSD requirement apply with both IMD2 and IMD3 products should be generated. |  |  |  |  |  |  |  |

## 5.1.125		DC_3-20_n8

- 5.1.125.1	 Operating bands for DC

Table 5.1.125.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-20_n8 | CA_3-20 | n8 | no |

- 5.1.125.2 	Configuration for DC

Table 5.1.125.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A | CA_1-20 | n8 |

- 5.1.125.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.125.3-1.

Table 5.1.125.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18 | 83 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 749 | 844 | 898 | 998 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2544 | 2639 | 2592 | 2692 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 | 1706 | 1778 | 1913 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3501 | 3472 | 3607 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 166 | 36 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2828 | 2658 | 2568 | 2413 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1081 | 916 | 826 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 | 4522 | 4208 | 4363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 | 4469 | 4256 | 4416 |

For UE coexistence study of Band 3 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.125.3-1.

Table 5.1.125.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 | 795 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2690 | 25 | 120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 | 4485 | 3470 | 3615 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 | 4475 | 855 | 1035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 | 6270 | 4350 | 4530 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1590 | 1810 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 1735 | 6260 | 5925 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 | 930 | 3595 | 3300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 | 5445 | 7720 | 8055 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 | 6315 | 6890 | 7185 |

Based on the tables：

IMD2 and IMD5 of Tx band 3 + band n8 may fall into Rx of band 20.

IMD4 of Tx band 20 + band n8 may fall into Rx of band 3.

- 5.1.125.4	 ∆TIB and ∆RIB values

Table 5.1.125.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n8 | 3 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |

Table 5.1.125.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n8 | 3 | 0 |
|  | 20 | 0 |
|  | n8 | 0 |

- 5.1.125.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD2 and IMD3 for DC_3-20_n8 with UL DC_3_n8 is specified as below

Table 5.1.125.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3-20_n8 | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD2 ¦fn8-fB3¦ |
| DC_3-20_n8 | 3 | 1765 | 5 | 25 | 1860 | 14.5 | IMD4 ¦3*fn8-fB20¦ |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |

## 5.1.126		DC_7-20_n8

- 5.1.126.1	 Operating bands for DC

Table 5.1.126.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-20_n8 | CA_7-20 | n8 | no |

- 5.1.126.2 	Configuration for DC

Table 5.1.126.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A | CA_7-20 | n8 |

- 5.1.126.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.126.3-1.

Table 5.1.126.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18 | 83 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 749 | 844 | 898 | 998 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2544 | 2639 | 2592 | 2692 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 | 1706 | 1778 | 1913 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3501 | 3472 | 3607 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 166 | 36 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2828 | 2658 | 2568 | 2413 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1081 | 916 | 826 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 | 4522 | 4208 | 4363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 | 4469 | 4256 | 4416 |

For UE coexistence study of Band 7 + Band n8, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.126.3-1.

Table 5.1.126.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1690 | 1585 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4085 | 4260 | 810 | 670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5880 | 6055 | 4260 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6585 | 6830 | 70 | 245 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8380 | 8625 | 5140 | 5315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3380 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 950 | 9400 | 9085 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2255 | 2500 | 5950 | 5670 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6020 | 6230 | 10880 | 11195 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7640 | 7885 | 9260 | 9540 |

Based on the tables：

IMD3 of Tx band 7 + band n8 may fall into Rx of band 20.

IMD3 and IMD5 of Tx band 20 + band n8 may fall into Rx of band 3.

- 5.1.126.4	 ∆TIB and ∆RIB values

Table 5.1.126.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n8 | 7 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |

Table 5.1.126.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20_n8 | 7 | 0 |
|  | 20 | 0 |
|  | n8 | 0 |

- 5.1.126.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD issues for DC_7-20_n8 with UL DC_7_n8 DC_20_n8 and is specified as below.

Table 5.1.126.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7-20_n8 | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 ¦2*fn8-fB7¦ |
| DC_7-20_n8 | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 ¦2*fn8+fB20¦ |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_7-20_n8 | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 ¦1*fn8+2*fB20¦ |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |

## 5.1.127		DC_1-41_n41

- 5.1.127.1	 Operating bands for DC

Table 5.1.127.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-41_n41 | CA_1-41 | n41 | no |

- 5.1.127.2 	Configuration for DC

Table 5.1.127.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A | CA_1-41 | n41 |

- 5.1.127.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.127.3-1.

Table 5.1.127.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 | 1464 | 3012 | 3460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6650 | 6912 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 | 3444 | 5508 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 | 8630 | 9408 | 10050 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1540 | 1032 | 8832 | 9340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 | 8004 | 5424 | 4990 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 | 3528 | 948 | 380 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 | 12740 | 10176 | 10610 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 | 12030 | 10752 | 11320 |

Based on the table, there is no harmonic and IMD issue.

- 5.1.127.4	 ∆TIB and ∆RIB values

Table 5.1.127.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |

Table 5.1.127.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n41 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |

- 5.1.127.5	REFSENS requirements

There is no the MSD requirements.

## 5.1.128		DC_1-(n)41

- 5.1.128.1	 Operating bands for DC

Table 5.1.128.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-(n)41 | CA_1-41 | n41 | no |

- 5.1.128.2 	Configuration for DC

Table 5.1.128.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A | CA_1-41 | n41 |

- 5.1.128.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.128.3-1.

Table 5.1.128.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 | 1464 | 3012 | 3460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6650 | 6912 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 | 3444 | 5508 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 | 8630 | 9408 | 10050 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1540 | 1032 | 8832 | 9340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 | 8004 | 5424 | 4990 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 | 3528 | 948 | 380 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 | 12740 | 10176 | 10610 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 | 12030 | 10752 | 11320 |

Based on the table, there is no harmonic and IMD issue.

- 5.1.128.4	 ∆TIB and ∆RIB values

Table 5.1.128.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-(n)41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |

Table 5.1.128.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-(n)41 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |

- 5.1.128.5	REFSENS requirements

There is no the MSD requirements.

## 5.1.129	DC_2-66_n25

- 5.1.129.1	Operating bands for DC

Table 5.1.129.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n25 | CA_2-66 | n25 | No |

- 5.1.129.2	Configuration for DC

Table 5.1.129.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A_n25A | DC_66A_n25A |

- 5.1.129.3	Coexistence studies

Co-existence studies of DC_66A_n25A and DC_66A_n2A are captured in TR 37.716-11-11.

- 5.1.129.4	∆TIB and ∆RIB values

For DC_2-66_n25, the TIB,c and RIB,c values are derived from CA_2-66 in TS 36.101 and DC_66A_n25A in TR 37.716-11-11.

Table 5.1.129.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n25 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.1.129.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n25 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |

- 5.1.129.5	REFSENS requirements

MSD is derived from DC_66A_n25A and DC_66A_n2A captured in TR 37.716-11-11.

Table 5.1.129.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_2A-66A_n25A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |

## 5.1.130	DC_12-66_n25

- 5.1.130.1	Operating bands for DC

Table 5.1.130.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_12-66_n25 | CA_12-66 | n25 | No |

- 5.1.130.2	Configuration for DC

Table 5.1.130.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |

- 5.1.130.3	Coexistence studies

Co-existence analysis for DC_66_n25 in TR 37.716-11-11 shows that there is no impact to Band 12. However, 3rd and 5th IMD might fall in own Rx frequencies.

Co-existence analysis for DC_12_n25 in TR 37.716-11-11 shows that 3rd harmonics of Band 12 might fall in Band 66. This is already addressed in Table 7.3B.2.3.1-2 in TS 38.101-3 based on analysis of DC_12A_n66A in 37.863-01-01

- 5.1.130.4	∆TIB and ∆RIB values

For DC_12-66_n25, the TIB,c and RIB,c values are derived from CA_12-66 in TS 36.101 and DC_66A_n25A in TR 37.716-11-11.

Table 5.1.130.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n25 | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.1.130.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n25 | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |

- 5.1.130.5	REFSENS requirements

MSD is derived from DC_66A_n25A and DC_12A_n25A captured in TR 37.716-11-11.

Table 5.1.130.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | FDD | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | FDD | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |

## 5.1.131	DC_2-7_n38

- 5.1.131.1	Operating bands for EN-DC

Table 5.1.131.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7_n381 | CA_2-7 | n38 |  |
| NOTE 1: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |

- 5.1.131.2	Configuration for DC

Table 5.1.131.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A_n38A | 2A1 | CA_2A-7A | n38 |
| DC_2A-2A-7A_n38A | 2A1 | CA_2A-2A-7A | n38 |
| NOTE 1: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |

- 5.1.131.3		∆TIB and ∆RIB values

For DC_2-7_n38, the TIB,c and RIB,c values are reused from LTE combination CA_1-7-38, and are given in the tables below.

Table 5.1.131.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n38 DC_2-2-7_n38 | 2 | 0.5 |

Table 5.1.131.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n38 DC_2-2-7_n38 | 2 | 0 |
|  | 7 | 0 |
|  | n38 | 0.2 |

- 5.1.131.4	REFSENS requirements

There is no impact to consider.

## 5.1.132	DC_2-71_n38

- 5.1.132.1	Operating bands for EN-DC

Table 5.1.132.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-71_n38 | CA_2-71 | n38 | No |

- 5.1.132.2	Configuration for DC

Table 5.1.132.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A | CA_2A-71A | n38 |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A | CA_2A-2A-71A | n38 |

- 5.1.132.3		∆TIB and ∆RIB values

For DC_2-71_n38, the ΔTIB,c and ΔRIB,c values are reused from LTE combination CA_2-7-12 and CA_2-7-28, and are given in the tables below.

Table 5.1.132.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n38 DC_2-2-71_n38 | 2 | 0.5 |
|  | 71 | 0.3 |
|  | n38 | 0.5 |

Table 5.1.132.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-71_n38 DC_2-2-71_n38 | 2 | 0 |
|  | 71 | 0 |
|  | n38 | 0 |

- 5.1.132.4	REFSENS requirements

Co-existence analysis from DC_2_n38 shows that there is no impact from DC_2_n38 UL to Band 71 DL.

Co-existence analysis from DC_71_n38 shows that there is IMD2 impact from DC_71_n38 UL to Band 2 DL.

MSD value for Band 2 DL affected by DC_71_n38 UL, are derived from CA_7-28 affecting band 3 DL in CA_3-7-28 and need to be defined in TS 38.101-3 like in table below:

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |

## 5.1.133	DC_7-66_n38

- 5.1.133.1	Operating bands for EN-DC

Table 5.1.133.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66_n381 | CA_7-66 | n38 |  |
| NOTE 1: UL carrier shall be supported in Band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |

- 5.1.133.2	Configuration for DC

Table 5.1.133.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A_n38A | 66A1 | CA_7A-66A | n38 |
| NOTE 1: UL carrier shall be supported in Band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |

- 5.1.133.3		∆TIB and ∆RIB values

For DC_7-66_n38, the ΔTIB,c and ΔRIB,c values are reused from LTE combination CA_1-7-38, and are given in the tables below.

Table 5.1.133.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n38 | 66 | 0.5 |

Table 5.1.133.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n38 | 7 | 0 |
|  | 66 | 0 |
|  | n38 | 0.2 |

- 5.1.133.4	REFSENS requirements

There is no impact to consider.

## 5.1.134	DC_66-71_n38

- 5.1.134.1	Operating bands for EN-DC

Table 5.1.134.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 66-71_n38 | CA_66-71 | n38 | No |

- 5.1.134.2	Configuration for DC

Table 5.1.134.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A | CA_66A-71A | n38 |

- 5.1.134.3		∆TIB and ∆RIB values

For DC_66-71_n38, the ΔTIB,c and ΔRIB,c values are reused from LTE combination CA_4-7-12, and are given in the tables below.

Table 5.1.134.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-71_n38 | 66 | 0.5 |
|  | 71 | 0.5 |
|  | n38 | 0.8 |

Table 5.1.134.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66-71_n38 | 66 | 0.5 |
|  | 71 | 0.5 |
|  | n38 | 0.5 |

- 5.1.134.4	REFSENS requirements

Co-existence analysis from DC_66_n38 shows that there is no impact from DC_66_n38 UL to Band 71 DL.

Co-existence analysis from DC_71_n38 shows that there is no impact from DC_71_n38 UL to Band 66 DL.

## 5.1.135	DC_2-66_n38

- 5.1.135.1	Operating bands for EN-DC

Table 5.1.135.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66_n38 | CA_2-66 | n38 | No |

- 5.1.135.2	Configuration for DC

Table 5.1.135.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A | CA_2A-66A | n38 |
| DC_2A-2A-66A_n38A | DC_2A_n38A DC_66A_n38A | CA_2A-2A-66A | n38 |

- 5.1.135.3		∆TIB and ∆RIB values

For DC_2-66_n38, the TIB,c and RIB,c values are reused from LTE combination CA_2-4-7, and are given in the tables below.

Table 5.1.135.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n38 DC_2-2-66_n38 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |

Table 5.1.135.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n38 DC_2-2-66_n38 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |

- 5.1.135.4	REFSENS requirements

Co-existence analysis from DC_66_n38 shows that there is no impact from DC_66_n38 UL to Band 2 DL.

Co-existence analysis from DC_2_n38 shows that there is no impact from DC_2_n38 UL to Band 66 DL.

## 5.1.136	DC_7-66_n71

- 5.1.136.1	Operating bands for EN-DC

Table 5.1.136.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66_n71 | CA_7-66 | n71 | No |

- 5.1.136.2	Configuration for DC

Table 5.1.136.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A | CA_7A-66A | n71 |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A | CA_7A-66A-66A | n71 |

- 5.1.136.3		∆TIB and ∆RIB values

For DC_7-66_n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.136.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n71 DC_7-66-66_n71 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.5 |

Table 5.1.136.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n71 DC_7-66-66_n71 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.1 |

- 5.1.136.4	REFSENS requirements

Co-existence analysis from DC_66_n71 shows that there is no impact from DC_66_n71 UL to Band 7 DL.

Co-existence analysis from DC_7_n71 shows that there is no impact from DC_7_n71 UL to Band 66 DL.

## 5.1.137	DC_2-71_n66

- 5.1.137.1	Operating bands for EN-DC

Table 5.1.137.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-71_n66 | CA_2-71 | n66 | No |

- 5.1.137.2	Configuration for DC

Table 5.1.137.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A | CA_2A-71A | n66 |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A | CA_2A-2A-71A | n66 |

- 5.1.137.3		∆TIB and ∆RIB values

For DC_2-71_n66, the TIB,c and RIB,c values are reused from DC_2-66_n71, and are given in the tables below.

Table 5.1.137.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n66 DC_2-2-71_n66 | 2 | 0,5 |
|  | 71 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.137.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-71_n66 DC_2-2-71_n66 | 2 | 0.3 |
|  | 71 | 0 |
|  | n66 | 0.3 |

- 5.1.137.4	REFSENS requirements

Co-existence analysis from DC_71_n66 shows that there is no impact from DC_71_n66 UL to Band 2 DL.

Co-existence analysis from DC_2_n66 shows that there is no impact from DC_2_n66 UL to Band 71 DL.

## 5.1.138	DC_66-71_n66

- 5.1.138.1	Operating bands for EN-DC

Table 5.1.138.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 66-71_n66 | CA_66-71 | n66 |  |

- 5.1.138.2	Configuration for DC

Table 5.1.138.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A1 | CA_66A-71A | n66 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.138.3		∆TIB and ∆RIB values

For DC_66-71_n66, the TIB,c and RIB,c values are reused from LTE combination CA_66-71, and are given in the tables below.

Table 5.1.138.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-71_n66 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.138.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66-71_n66 | 66 | 0 |
|  | 71 | 0 |
|  | n66 | 0 |

- 5.1.138.4	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.139	DC_2-71_n78

- 5.1.139.1	Operating bands for EN-DC

Table 5.1.139.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-71_n78 | CA_2-71 | n78 | No |

- 5.1.139.2	Configuration for DC

Table 5.1.139.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A | CA_2A-71A | n78 |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A | CA_2A-2A-71A | n78 |

- 5.1.139.3		∆TIB and ∆RIB values

For DC_2-71_n78, the TIB,c and RIB,c values are reused from DC_3-8_n78, and are given in the tables below.

Table 5.1.139.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n78 DC_2-2-71_n78 | 2 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.139.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-71_n78 DC_2-2-71_n78 | 2 | 0.2 |
|  | 71 | 0.2 |
|  | n78 | 0.5 |

- 5.1.139.4	REFSENS requirements

Co-existence analysis from DC_2_n78 shows that there is no impact from DC_2_n78 UL to Band 71 DL.

Co-existence analysis from DC_71_n78 shows that there is IMD3 impact from DC_71_n78 UL to Band 2 DL.

MSD value for band 2 DL affected by DC_71_n78 UL are derived from 8_n78 UL affecting band 3 DL in DC_3-8_n78 and need to be defined in TS 38.101-3 like in table below:

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |

## 5.1.140	DC_66-71_n78

- 5.1.140.1	Operating bands for EN-DC

Table 5.1.140.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 66-71_n78 | CA_66-71 | n78 | No |

- 5.1.140.2	Configuration for DC

Table 5.1.140.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A-71A_n78A | DC_71A_n78A DC_66A_n78A | CA_66A-71A | n78 |

- 5.1.140.3		∆TIB and ∆RIB values

For DC_66-71_n78, the TIB,c and RIB,c values are reused from LTE combination DC_3-8_n78, and are given in the tables below.

Table 5.1.140.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-71_n78 | 66 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.140.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66-71_n78 | 66 | 0.2 |
|  | 71 | 0.2 |
|  | n78 | 0.5 |

- 5.1.140.4	REFSENS requirements

Co-existence analysis from DC_66_n78 shows that there is no impact from DC_66_n78 UL to Band 71 DL.

Co-existence analysis from DC_71_n78 shows that there is IMD3 impact from DC_71_n78 UL to Band 66 DL. MSD values need to be defined in TS 38.101-3 like in table below:

MSD value for band 66 DL affected by DC_71_n78 UL are derived from 8_n78 UL affecting band 3 DL in DC_3-8_n78 and need to be defined in TS 38.101-3 like in table below:

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-71A_n78A | 66 | 1754 | 5 | 25 | 2154 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | N/A |

## 5.1.141	DC_2-48_n12

- 5.1.141.1	Operating bands for EN-DC

Table 5.1.141.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-48_n12 | CA_2-48 | n12 | No |

- 5.1.141.2	Configuration for DC

Table 5.1.141.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A | CA_2A-48A | n12 |

- 5.1.141.3		∆TIB and ∆RIB values

For DC_2-48_n12, the TIB,c and RIB,c values are reused from LTE combination CA_2-13-48, and are given in the tables below.

Table 5.1.141.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n12 | 2 | 0.6 |
|  | 48 | 0.3 |
|  | n12 | 0.8 |

Table 5.1.141.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n12 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n12 | 0 |

- 5.1.141.4	REFSENS requirements

Co-existence analysis from DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band 48 DL.

Co-existence analysis from DC_48_n12 shows that there is no impact from DC_48_n12 UL to Band 2 DL.

## 5.1.142	DC_2-66_n12

- 5.1.142.1	Operating bands for EN-DC

Table 5.1.142.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66_n12 | CA_2-66 | n12 | No |

- 5.1.142.2	Configuration for DC

Table 5.1.142.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A | CA_2A-66A | n12 |

- 5.1.142.3		∆TIB and ∆RIB values

For DC_2-66_n12, the TIB,c and RIB,c values are reused from LTE combination CA_2-12-66, and are given in the tables below.

Table 5.1.142.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n12 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n12 | 0.8 |

Table 5.1.142.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n12 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n12 | 0.5 |

- 5.1.142.4	REFSENS requirements

Co-existence analysis from DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band 66 DL.

Co-existence analysis from DC_66_n12 shows that there is no impact from DC_66_n12 UL to Band 2 DL.

## 5.1.143	DC_5-66_n71

- 5.1.143.1	Operating bands for EN-DC

Table 5.1.143.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-66_n71 | CA_5-66 | n71 | No |

- 5.1.143.2	Configuration for DC

Table 5.1.143.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A | CA_5A-66A | n71 |

- 5.1.143.3		∆TIB and ∆RIB values

For DC_5-66_n71, the TIB,c and RIB,c values for the 5_n71 part are reused from CA_4-5-13, and also checked with DC_5_n71 and DC_71_n5, and the values for 66 are checked from DC_5-7_n71 and DC_66_n71, and they are given in the tables below.

Table 5.1.143.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n71 | 5 | 0.5 |
|  | 66 | 0.3 |
|  | n71 | 0.5 |

Table 5.1.143.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n71 | 5 | 0 |
|  | 66 | 0 |
|  | n71 | 0 |

- 5.1.143.4	REFSENS requirements

Co-existence analysis from DC_5_n71 shows that there is IMD3 impact from DC_5_n71 UL to Band 66 DL. MSD value is 16.1 dB for 20_n3 UL affecting band n78 DL in DC_20_n3-n78. But test bench simulations have shown values closer to 10 dB. Therefor an MSD level of 13 dB is proposed for IMD3 in band 66 in table below

Co-existence analysis from DC_66_n71 shows that there is IMD5 impact from DC_66_n71 UL to Band 5 DL. MSD value is 3.3 dB when 1_n28 UL affecting band 8 DL in DC_1-8_n28. But since n71 TX frequency is lower than B28 that PA may have higher gain at lower frequency both wanted Tx and IMD spur level. Therefor an MSD level of 4.2 dB is proposed for IMD5 in band 5 in table below.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1761 | 5 | 25 | 2161 | 13 | IMD3 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |

## 5.1.144	DC_48-66_n12

- 5.1.144.1	Operating bands for EN-DC

Table 5.1.144.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 48-66_n12 | CA_48-66 | n12 | No |

- 5.1.144.2	Configuration for DC

Table 5.1.144.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A | CA_48A-66A | n12 |

- 5.1.144.3		∆TIB and ∆RIB values

For DC_48-66_n12, the TIB,c and RIB,c values are reused from DC_48-66_n5, CA_13-48-66 and CA_5-48-66, and are given in the tables below.

Table 5.1.144.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n12 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n12 | 0.3 |

Table 5.1.144.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n12 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n12 | 0 |

- 5.1.144.4	REFSENS requirements

Co-existence analysis from DC_48_n12 shows that there is IMD3 impact from DC_48_n12 UL to Band 66 DL.

Co-existence analysis from DC_66_n12 shows that there is IMD5 impact from DC_66_n12 UL to Band 48 DL.

MSD value for band 66 DL affected by DC_48_n12 UL are derived from 13-48 UL affecting band 66 DL in CA_13-48-66 and need to be defined in TS 38.101-3 like in table below:

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |

Table 5.1.144.3-1: ΔTIB,c

Table 5.1.144.3-2: ΔRIB

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

5.1.145	DC_7-28_n3

- 5.1.145.1	Operating bands for EN-DC

Table 5.1.145.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-28_n3 | CA_7-28 | n3 |  |

- 5.1.145.2	Configuration for DC

Table 5.1.145.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n3A | DC_7A_n3A DC_28A_n3A | CA_7A-28A | n3 |
| DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A | CA_7C-28A | n3 |

- 5.1.145.3		∆TIB and ∆RIB values

For DC_7-28_n3, the TIB,c and RIB,c values are same as for DC_3-28_n7 and are given in the tables below.

Table 5.1.145.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n3 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |

Table 5.1.145.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n3 | 7 | 0 |
|  | 28 | 0 |
|  | n3 | 0 |

- 5.1.145.4	REFSENS requirements

Co-existence analysis shows that there is IMD2 impact from DC_7_n3 UL to Band 28 DL MSD value are same as for DC_3-28_n7.

Co-existence analysis shows that there is IMD3 impact from DC_28_n3 UL to Band 7 DL MSD value are same as for DC_7-13_n66.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |

## 5.1.146	DC_2-46_n66

- 5.1.146.1	Operating bands for EN-DC

Table 5.1.146.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46_n66 | CA_2-46 | n66 | No |

- 5.1.146.2	Configuration for DC

Table 5.1.146.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A_n66A | DC_2A_n66A | CA_2A-46A | n66 |
| DC_2A-46C_n66A | DC_2A_n66A | CA_2A-46C | n66 |
| DC_2A-46D_n66A | DC_2A_n66A | CA_2A-46D | n66 |

- 5.1.146.3		∆TIB and ∆RIB values

For DC_2-46_n66, the TIB,c and RIB,c values are reused from LTE combination CA_2-46-66, and are given in the tables below.

Table 5.1.146.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n66 | 2 | 0.5 |
|  | 46 | 0 |
|  | n66 | 0.5 |

Table 5.1.146.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n66 | 2 | 0 |
|  | 46 | 0 |
|  | n66 | 0 |

- 5.1.146.4	REFSENS requirements

Co-existence analysis from DC_2_n66 shows that there is IMD3 and IMD5 impact from DC_2_n66 UL to Band 46 DL. MSD for DC_2-46_n66 is proposed to be defined like in table below, where Note x is derived from Note 8 in TS36.101 Table 7.3.1A-0eA.

Table 5.1.146.4-1: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| DC_2A_n66A | 46 |  |  |  | 0 |  | 0 |  | 0 | 0 |  | 0 |
| NOTE x: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. |

## 5.1.147	DC_2-66_n48 and DC_2-66-66_n48

- 5.1.147.1	Operating bands for DC

Table 5.1.147.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n48 | CA_2-66 | n48 | No |
| DC_2-66-66_n48 | CA_2-66-66 | n48 | No |

- 5.1.147.2	Configurations for DC

Table 5.1.147.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A | CA_2A-66A | n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A | CA_2A-66A | CA_n48B |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A | CA_2A-66A-66A | n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A | CA_2A-66A-66A | CA_n48B |

- 5.1.147.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, and the impact to own Rx of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 2 + Band n48 may fall into own Rx of band 66, the MSD value could reuse the LTE MSD value for CA_2A-48A-66A.

- 5th order IMD generated by dual uplink of Band 66 + Band n48 may fall into own Rx of band 2, the MSD value could reuse the LTE MSD value for CA_2A-48A-66A.

- 5.1.147.4	∆TIB and ∆RIB values

For DC_2-66_n48 and DC_2-66-66_n48, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.147.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n48 DC_2-66-66_n48 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |

Table 5.1.147.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n48 DC_2-66-66_n48 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n48 | 0.5 |

- 5.1.147.5	REFSENS requirements

Table 5.1.147.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.147.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | FDD | IMD4 ¦3*fB2-fBn48¦ |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | TDD | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1880 | 5 | 25 | 1960 | 28.3 | FDD | IMD5 ¦2*fBn48-3*fB66¦ |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | TDD | N/A |

## 5.1.148	DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2

- 5.1.148.1	Operating bands for DC

Table 5.1.148.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n2 | CA_5-66 | n2 | No |
| DC_5-5-66_n2 | CA_5-5-66 | n2 | No |
| DC_5-66-66_n2 | CA_5-66-66 | n2 | No |
| DC_5-5-66-66_n2 | CA_5-5-66-66 | n2 | No |

- 5.1.148.2	Configuration for DC

Table 5.1.148.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-66A_n2A | DC_5A_n2A | CA_5A-66A | n2A |
| DC_5B-66A_n2A | DC_5A_n2A | CA_5B-66A | n2A |
| DC_5A-5A-66A_n2A | DC_5A_n2A | CA_5A-5A-66A | n2A |
| DC_5A-66A-66A_n2A | DC_5A_n2A | CA_5A-66A-66A | n2A |
| DC_5B-66A-66A_n2A | DC_5A_n2A | CA_5B-66A-66A | n2A |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A | CA_5A-5A-66A-66A | n2A |

- 5.1.148.3	Coexistence studies

Based on co-existence studies of DC_5A_n2A in TR 37.863-01-01, and the impact to own Rx of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band5 + Band n2 may fall into own Rx of band 66, the MSD value could reuse the LTE MSD value for CA_2A-5A-66A.

- 5.1.148.4	∆TIB and ∆RIB values

For DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2, the TIB,c and RIB,c values reused the values for DC_5_n2 in TS 38.101-3.

Table 5.1.148.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.148.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 | 5 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

- 5.1.148.5	REFSENS requirements

Table 5.1.148.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.148.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5A-66A_n2A DC_5BA-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | 66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 ¦2*fB5-2*fBn2¦ |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |

## 5.1.149	DC_13-46_n5

- 5.1.149.1		Operating bands for DC

Table 5.1.149.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-46_n5 | CA_13-46 | n5 | No |

- 5.1.149.2	Configuration for DC

Table 5.1.149.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-46A_n5A | DC_13A_n5A | CA_13A-46A | n5A |

- 5.1.149.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no harmonic or IMD issues for this band combination.

- 5.1.149.4	∆TIB and ∆RIB values

For DC_13-46_n5, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_5A-13A-46A

Table 5.1.149.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-46_n5 | 13 | 0.5 |
|  | n5 | 0.5 |

Table 5.1.149.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-46_n5 | 13 | 0 |
|  | n5 | 0 |

- 5.1.149.5	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.150	DC_13-66_n2 and DC_13-66-66_n2

- 5.1.150.1	Operating bands for DC

Table 5.1.150.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-66_n2 | CA_13-66 | n2 | No |
| DC_13-66-66_n2 | CA_13-66-66 | n2 | No |

- 5.1.150.2	Configurations for DC

Table 5.1.150.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-66A_n2A | DC_13A_n2A DC_66A_n2A | CA_13A-66A | n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A | CA_13A-66A-66A | n2A |

- 5.1.150.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, and the impact to own Rx of the 3rd band is shown as the followings.

- 4th order IMD generated by dual uplink of Band 13+ Band n2 may fall into own Rx of band 66, the MSD value could reuse the LTE MSD value for CA_2A-13A-66A.

- 5.1.150.4	∆TIB and ∆RIB values

For DC_13-66_n2 and DC_13-66-66_n2, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.150.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n2 DC_13-66-66_n2 | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.150.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n2 DC_13-66-66_n2 | 13 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

- 5.1.150.5	REFSENS requirements

Table 5.1.150.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.150.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | 66 | 1736 | 5 | 25 | 2156 | 7..2 | FDD | IMD3 ¦2*fB13-2*fBn2¦ |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | FDD | N/A |

## 5.1.151	DC_13-66_n48 and DC_13-66-66_n48

- 5.1.151.1	Operating bands for DC

Table 5.1.151.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-66_n48 | CA_13-66 | n48 | No |
| DC_13-66-66_n48 | CA_13-66-66 | n48 | No |

- 5.1.151.2	Configurations for DC

Table 5.1.151.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-66A_n48A | DC_13A_n48A DC_66A_n48A | CA_13A-66A | n48A |
| DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A | CA_13A-66A | CA_n48B |
| DC_13A-66A-66A_n48A | DC_13A_n48A DC_66A_n48A | CA_13A-66A-66A | n48A |
| DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A | CA_13A-66A-66A | CA_n48B |

- 5.1.151.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, and the impact to own Rx of the 3rd band is shown as the followings.

- 3rd order IMD generated by dual uplink of Band 13+ Band n48 may fall into own Rx of band 66, the MSD value could reuse the LTE MSD value for CA_13A-48A-66A.

- 5.1.151.4	∆TIB and ∆RIB values

For DC_13-66_n48 and DC_13-66-66_n48, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.151.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n48 DC_13-66-66_n48 | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |

Table 5.1.151.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n48 DC_13-66-66_n48 | 13 | 0 |
|  | 66 | 0.2 |
|  | n48 | 0.5 |

- 5.1.151.5	REFSENS requirements

Table 5.1.151.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.151.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | 66 | 1731 | 5 | 25 | 2131 | 17.1 | FDD | IMD3 ¦2*fB13-fBn48¦ |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | TDD | N/A |

## 5.1.152	DC_46-66_n25

- 5.1.152.1	Operating bands for DC

Table 5.1.152.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n25 | CA_46-66 | n25 | No |

- 5.1.152.2	Configuration for DC

Table 5.1.152.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |

- 5.1.152.3	Coexistence studies

Co-existence analysis for DC_66_n25 in TR 37.716-11-11 shows that 3rd order harmonics and 3rd and 5th IMD might fall in Band 46.

- 5.1.152.4	∆TIB and ∆RIB values

For DC_46-66_n25, the TIB,c and RIB,c values are derived from CA_46-66 in TS 36.101 and DC_66A_n25A in TR 37.716-11-11.

Table 5.1.152.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n25 | 46 | 0 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.1.152.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n25 | 46 | 0 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |

- 5.1.152.5	REFSENS requirements

MSD is derived from DC_66A_n25A captured in TR 37.716-11-11. The values have also been aligned with the values proposed in R4-2002012 which are derived from Co-existence analysis for DC_2_n66 which shows that there is IMD3 and IMD5 impact from DC_2_n66 UL to Band 46 DL.

Table 5.1.152.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_46A-66A_n25A3 DC_46C-66A_n25A3 DC_46D-66A_n25A3 | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 5.1.153	DC_1-41_n28

- 5.1.153.1		Operating bands for DC

Table 5.1.153.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-41_n28 | CA_1-41 | n28 | No |

- 5.1.153.2	Configuration for DC

Table 5.1.153.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-41A_n28A | DC_1A_n28A DC_41A_n28A | CA_1A-41A | n28A |
| DC_1A-41C_n28A | DC_1A_n28A DC_41A_n28A DC_41C_n28A | CA_1A-41C | n28A |

- 5.1.153.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there are IMD impacting on the three band in this band combination, as below list:

-	2nd  order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band 41.

The MSD due to 2nd order IMD generated by dual uplink of band1+band n28 impacting to Rx of band 41 could  reuse the value of DC_1A-7A_n28A.

- 5.1.153.4	∆TIB and ∆RIB values

For DC_1-41_n28, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode

Table 5.1.153.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |

Table 5.1.153.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n28 | 1 | 0 |
|  | 41 | 0 |
|  | n28 | 0.2 |

- 5.1.153.5	REFSENS requirements

Table 5.1.153.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.153.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | TDD | IMD2 ¦fB1+fBn28¦ |

## 5.1.154	DC_11-18_n77

- 5.1.154.1	Operating bands for DC

Table 5.1.154.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_11-18_n77 | CA_11-18 | n77 | No |

- 5.1.154.2	Configurations for DC

Table 5.1.154.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A | CA_11A-18A | n77A |

- 5.1.154.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, based on the Operator’s spectrum, there only has:

-	3rd rder IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of band 18, and it may reuse the MSD of DC_19A-21A_n77A.

.

- 5.1.154.4	∆TIB and ∆RIB values

For DC_11-18_n77, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.154.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11-18_n77 | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.154.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11-18_n77 | 11 | 0 |
|  | 18 | 0 |
|  | n77 | 0.5 |

- 5.1.154.5	REFSENS requirements

Table 5.1.154.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.154.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 | 1491 | N/A | FDD | N/A |
|  | n77 | 3706 | 10 | 50 | 3706 | N/A | TDD | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | FDD | IMD3 ¦fBn77-2*fB11¦ |

## 5.1.155	DC_11-18_n78

- 5.1.155.1	Operating bands for DC

Table 5.1.155.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_11-18_n78 | CA_11-18 | n78 | No |

- 5.1.155.2	Configurations for DC

Table 5.1.155.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A | CA_11A-18A | n78A |

- 5.1.155.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, based on the Operator’s spectrum, there only has:

-	3rd order IMD generated by dual uplink of Band 11 + Band n78 may also fall into own Rx of band 18, and it may reuse the MSD of DC_19A-21A_n78A.

- 5.1.155.4	∆TIB and ∆RIB values

For DC_11-18_n78, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.155.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11-18_n78 | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.155.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11-18_n78 | 11 | 0 |
|  | 18 | 0 |
|  | n78 | 0.5 |

- 5.1.155.5	REFSENS requirements

Table 5.1.155.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.155.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 | 1491 | N/A | FDD | N/A |
|  | n78 | 3706 | 10 | 50 | 3706 | N/A | TDD | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | FDD | IMD3 ¦fBn77-2*fB11¦ |

## 5.1.156	DC_18-41_n3

- 5.1.156.1		Operating bands for DC

Table 5.1.156.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n3 | CA_18-41 | n3 | No |

- 5.1.156.2	Configuration for DC

Table 5.1.156.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n3A | DC_18A_n3A DC_41A_n3A | CA_18A-41A | n3A |
| DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A | CA_18A-41C | n3A |

- 5.1.156.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, the IMD issues for this band combination are as below list:

-  2nd and 3rd order IMDs generated by dual uplink of band 18 and band n3 maybe fall into the Rx of band 41. However, B18 only used in Japan. So specific band combinations can considered, then only IMD3 will be impact to the own Band 41.

-  2nd and 3rd order IMDs generated by dual uplink of band 41 and band n3 maybe fall into the Rx of band 18.

- 5.1.156.4	∆TIB and ∆RIB values

For DC_18-41_n3, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.156.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3 | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.1.156.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3 | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

- 5.1.156.5	REFSENS requirements

Table 5.1.156.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.156.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

|  | E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-41A_n3A1 DC_18A-41C_n3A1 |  | 18 | 820 | 5 | 25 | 865 | 5 | N/A | FDD | N/A |
|  |  | n3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2630 | 5 | 25 | 2630 | 5 | 16.0 | TDD | IMD3 ¦2*fB18-fBn3¦ |
|  |  | 18 | 820 | 5 | 25 | 865 | 5 | 28.9 | FDD | IMD21 ¦fB41-fBn3¦ |
|  |  | n3 | 1765 | 5 | 25 | 1860 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2630 | 5 | 25 | 2630 | 5 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD3 falling down band18 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

## 5.1.157	DC_2-48_n66

- 5.1.157.1	Operating bands for DC

Table 5.1.157.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-48_n66 | CA_2-66 | n48 | No |

- 5.1.157.2	Configurations for DC

Table 5.1.157.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-48A_n66A | DC_2A_n66A DC_48A_n66A | CA_2A-48A | n66A |

- 5.1.157.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, and the impact to own Rx of the 3rd band is shown as the followings.

- 2nd order IMD generated by dual uplink of Band 2 + Band n66 may fall into own Rx of band 48, it can reuse the MSD of DC_2A_n66A-n78A.

- 2nd order IMD generated by dual uplink of Band 48+ Band n66 may fall into own Rx of band 2, it can reuse the MSD of DC_2A-66A_n48A.

- 5.1.157.4	∆TIB and ∆RIB values

For DC_2-48_n66, the TIB,c and RIB,c values reused the values for DC_2A_n48A-n66A, as given in the tables below.

Table 5.1.157.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |

Table 5.1.157.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | n66 | 0.3 |

- 5.1.157.5	REFSENS requirements

Table 5.1.157.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.157.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-48A_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | FDD | IMD2 ¦fB2+Bn66¦ |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | TDD | N/A |
| DC_2A-48A_n66A | 2 | 1880 | 5 | 25 | 1960 | 28.3 | FDD | IMD2 ¦fBn48-fB66¦ |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | FDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | TDD | N/A |

## 5.1.158	DC_1-11_n3

- 5.1.158.1	Operating bands for DC_1-11_n3

Table 5.1.158.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n3 | CA_1-11 | n3 | DC_1_n3 |

- 5.1.158.2	Configurations for DC_1-11_n3

Table 5.1.158.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A | CA_1A-11A | n3A |

- 5.1.158.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n3A, (1) IMD3 of (B1 - n3) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n3A, (2) IMD3 of (B11 - n3) will fall into Rx band of Band 1.

5.1.158.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.158.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5.1.158.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

- 5.1.158.5	REFSENS requirements

Band 11 is the specific band used in only Japan. Considering the spectrum allocation in Japan, only IMD3 of B1 and n3 to Band11 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A | Yes |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | TDD | N/A | Yes |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | FDD | IMD3 |  |

## 5.1.159	DC_3-8_n28

- 5.1.159.1	Operating bands for DC_3-8_n28

Table 5.1.159.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n28 | CA_3-8 | n28 | - |

- 5.1.159.2	Configurations for DC_3-8_n28

Table 5.1.159.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A | CA_3A-8A | n28A |

- 5.1.159.3	Co-existence studies

There is no additional harmonic and intermodulation impact for the additional band receiver.

5.1.159.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.159.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |

Table 5.1.159.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n28 | 3 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.1 |

- 5.1.159.5	REFSENS requirements

As mentioned in 5.1.159.3, REFSENS exceptions are not expected.

## 5.1.160	DC_8-11_n3

- 5.1.160.1	Operating bands for DC_8-11_n3

Table 5.1.160.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-11_n3 | CA_8-11 | n3 | - |

- 5.1.160.2	Configurations for DC_8-11_n3

Table 5.1.160.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A | CA_8A-11A | n3A |

- 5.1.160.3	Co-existence studies

There is no additional harmonic and intermodulation impact for the additional band receiver.

5.1.160.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.160.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n3 | 8 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5.1.160.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n3 | 8 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

- 5.1.160.5	REFSENS requirements

As mentioned in 5.1.160.3, REFSENS exceptions are not expected.

## 5.1.161	DC_1-20_n41

- 5.1.161.1	Operating bands for DC

Table 5.1.161.1-1: band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1A-20A_n41A | CA_1-20 | n41 | DC_20A_n41A |

- 5.1.161.2	Configuration for DC

Table 5.1.161.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-20A_n41A | DC_1A_n41 DC_20A_n41 | CA_1A-20A | n41A |

- 5.1.161.3	∆TIB and ∆RIB values

For DC_1A-20A_n41A, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.161.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n41 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.51 |
|  |  | 1.22 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.161.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n41 | 1 | 0 |
|  | 20 | 0 |
|  | n41 | 0 |

- 5.1.161.4	REFSENS requirements

Based on co-existence studies of Band 1 + Band n41 and Band 20 + Band n41 captured in 38.101-3, 3rd and 5th intermodulation products of band 20 and n41 fall into receiver of band 20. Other than that there is no additional exceptions.

## 5.1.162	DC_3-20_n41

- 5.1.162.1	Operating bands for DC

Table 5.1.162.1-1: band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-20_n41 | CA_3-20 | n41 | DC_20_n41 |

- 5.1.162.2	Configuration for DC

Table 5.1.162.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A | CA_3A-20A | n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A | CA_3C-20A | n41A |

- 5.1.162.3	∆TIB and ∆RIB values

For DC_3-20_n41, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.162.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n41 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.51 |
|  |  | 1.22 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.1.162.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n41 | 3 | 0 |
|  | 20 | 0 |
|  | n41 | 0 |

- 5.1.162.4	REFSENS requirements

Based on co-existence studies of Band 3 + Band n41 and Band 20 + Band n41 captured in 38.101-3, 3rd and 5th intermodulation products of band 20 and n41 fall into receiver of band 20 while 4th intermodulation products of band 3 and n41 fall into receiver of band 3.

Additionally, when simultaneous transmission happens between band 20 and band n41, 2nd intermodulation products fall into the receiver of band 3. When simultaneous transmission happens between band 3 and band n41, 2nd and 3rd intermodulation products also fall into the receiver of band 20. Corresponding requirements are defined in the below table.

Table 5.1.162.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1744 | 5 | 25 | 1839 | 26.0 | FDD | IMD 2 |
|  | n41 | 2680 | 10 | 52 | 2680 | N/A | TDD | N/A |
|  | 20 | 841 | 10 | 50 | 800 | N/A | FDD | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1779 | 5 | 25 | 1874 | N/A | FDD | N/A |
|  | n41 | 2590 | 10 | 52 | 2590 | N/A | TDD | N/A |
|  | 20 | 852 | 10 | 50 | 811 | 26.0 | FDD | IMD 2 |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n41 | 2660 | 10 | 52 | 2660 | N/A | TDD | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.5 | FDD | IMD 3 |

## 5.1.163		DC_7-8_n3

- 5.1.163.1	 Operating bands for DC

Table 5.1.163.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-8_n3 | CA_7-8 | n3 | no |

- 5.1.163.2 	Configuration for DC

Table 5.1.163.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A | CA_7-8 | n3 |

- 5.1.163.3	 Co-existence studies

For UE coexistence study of Band 7 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.163.3-1.

Table 5.1.163.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 860 | 715 | 4210 | 4355 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3215 | 3430 | 850 | 1070 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6710 | 6925 | 5920 | 6140 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5715 | 6000 | 2560 | 2855 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9210 | 9495 | 7630 | 7925 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1430 | 1720 | 8420 | 8710 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4640 | 4270 | 8570 | 8215 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 355 | 10 | 4290 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340 | 9710 | 11710 | 12065 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130 | 10495 | 10920 | 11280 |

For UE coexistence study of Band 8 + Band n3, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.163.3-1.

Table 5.1.163.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 905 | 795 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2505 | 2690 | 25 | 120 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4300 | 4485 | 3470 | 3615 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4215 | 4475 | 855 | 1035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6010 | 6270 | 4350 | 4530 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1590 | 1810 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1950 | 1735 | 6260 | 5925 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 675 | 930 | 3595 | 3300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5230 | 5445 | 7720 | 8055 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6060 | 6315 | 6890 | 7185 |

IMD3 may fall into Rx of band 8 with UL DC_7_n3.

IMD2 and IMD3 may fall into Rx of band 7 with UL DC_8_n3.

- 5.1.163.4	 ∆TIB and ∆RIB values

Table 5.1.163.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n3 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n3 | 0.5 |

Table 5.1.163.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n3 | 7 | 0 |
|  | 8 | 0.2 |
|  | n3 | 0 |

- 5.1.163.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD3 for DC_7-8_n3 with UL DC_7_n3 is specified as below referring to the MSD for CA_3A-7A-8A from 36.101.

The reference sensitivity exception (MSD) due to IMD2 and IMD3 for DC_7-8_n3 with UL DC_8_n3 is specified as below referring to the MSD for CA_3A-7A-8A from 36.101.

Table 5.1.163.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD3y |
| NOTE y: This MSD requirement apply with both IMD2 and IMD3 products should be generated. |  |  |  |  |  |  |  |

## 5.1.164		DC_1-(n)38

- 5.1.164.1	 Operating bands for DC

Table 5.1.164.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-(n)38 | CA_1-38 | n38 | no |

- 5.1.164.2 	Configuration for DC

Table 5.1.164.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-(n)38AA | DC_1A_n38A | CA_1-38 | n38 |

- 5.1.164.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n38, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.164.3-1.

Table 5.1.164.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 590 | 700 | 4490 | 4600 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1220 | 1390 | 3160 | 3320 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6410 | 6580 | 7060 | 7220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3140 | 3370 | 5730 | 5940 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8330 | 8560 | 9630 | 9840 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1400 | 1180 | 8980 | 9200 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8560 | 8300 | 5350 | 5060 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4020 | 3750 | 800 | 520 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12200 | 12460 | 10250 | 10540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11550 | 11820 | 10900 | 11180 |

Based on the table, there is no harmonic and IMD issue.

- 5.1.164.4	 ∆TIB and ∆RIB values

Table 5.1.164.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-(n)38 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n38 | 0.5 |

Table 5.1.164.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-(n)38 | 1 | 0 |
|  | 38 | 0 |
|  | n38 | 0 |

- 5.1.164.5	REFSENS requirements

There is no the MSD requirements.

## 5.1.165		DC_20-(n)38

- 5.1.165.1	 Operating bands for DC

Table 5.1.165.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-(n)38 | CA_20-38 | n38 | no |

- 5.1.165.2 	Configuration for DC

Table 5.1.165.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-(n)38AA | DC_20A_n38A | CA_20-38 | n38 |

- 5.1.165.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n38, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.165.3-1.

Table 5.1.165.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 12850 | 13100 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1708 | 1788 | 3402 | 3482 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 956 | 846 | 4278 | 4408 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4234 | 4344 | 5972 | 6102 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 124 | 16 | 6848 | 7028 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5066 | 5206 | 8542 | 8722 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3576 | 3416 | 6804 | 6964 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9648 | 9418 | 878 | 708 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6196 | 5986 | 2554 | 2744 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11112 | 11342 | 5898 | 6068 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9374 | 9584 | 7636 | 7826 |

Based on the table, there is no harmonic and IMD issue.

- 5.1.165.4	 ∆TIB and ∆RIB values

Table 5.1.165.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-(n)38 | 20 | 0.3 |
|  | 38 | 0.3 |
|  | n38 | 0.3 |

Table 5.1.165.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-(n)38 | 20 | 0 |
|  | 38 | 0 |
|  | n38 | 0 |

- 5.1.165.5	REFSENS requirements

There is no the MSD requirements.

## 5.1.166		DC_20-(n)41

- 5.1.166.1	 Operating bands for DC

Table 5.1.166.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-(n)41 | CA_20-41 | n41 | no |

- 5.1.166.2 	Configuration for DC

Table 5.1.166.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A | CA_20-41 | n41 |

- 5.1.166.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.166.3-1.

Table 5.1.166.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 | 1858 | 3328 | 3552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 | 772 | 4130 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 | 4414 | 5824 | 6242 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 | 90 | 6626 | 7238 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 | 5276 | 8320 | 8932 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3716 | 3268 | 6656 | 7104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 | 9122 | 952 | 638 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 | 5764 | 2406 | 2884 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 | 11622 | 5824 | 6138 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 | 9794 | 7488 | 7966 |

Based on the table, there is no harmonic and IMD issue.

- 5.1.166.4	 ∆TIB and ∆RIB values

Table 5.1.166.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-(n)41 | 20 | 0.3 |
|  | 41 | 0.3 |
|  | n41 | 0.3 |

Table 5.1.166.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-(n)41 | 20 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |

- 5.1.166.5	REFSENS requirements

There is no the MSD requirements.

## 5.1.167		DC_1-32_n78

- 5.1.167.1	 Operating bands for DC

Table 5.1.167.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-32_n78 | CA_1-32 | n78 | no |

- 5.1.167.2 	Configuration for DC

Table 5.1.167.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | DC_1A_n78A | CA_1-32 | n78 |

- 5.1.167.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.167.3-1.

Table 5.1.167.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 1880 | 5220 | 5780 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

Based on the table, IMD2 and IMD5 may fall into Rx of band 32 with UL DC_1A_n78A.

- 5.1.167.4	 ∆TIB and ∆RIB values

Table 5.1.167.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-32_n78 | 1 | 0.5 |
|  | 32 | N/A |
|  | n78 | 0.8 |

Table 5.1.167.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-32_n78 | 1 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

- 5.1.167.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD issues for DC_1-32_n78 with UL DC_1_n78 is specified as below.

Table 5.1.167.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 31.8 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 0 | IMD5 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |

## 5.1.168		DC_3-32_n78

- 5.1.168.1	 Operating bands for DC

Table 5.1.168.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-32_n78 | CA_3-32 | n78 | no |

- 5.1.168.2 	Configuration for DC

Table 5.1.168.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | DC_3A_n78A | CA_3-32 | n78 |

- 5.1.168.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.168.3-1.

Table 5.1.168.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 270 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3030 | 10020 | 11170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11415 | 3840 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6330 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

Based on the table, IMD4 and IMD5 may fall into Rx of band 32 with UL DC_3A_n78A.

- 5.1.168.4	 ∆TIB and ∆RIB values

Table 5.1.168.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-32_n78 | 3 | 0.6 |
|  | 32 | N/A |
|  | n78 | 0.8 |

Table 5.1.168.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-32_n78 | 3 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

- 5.1.168.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD issues for DC_3-32_n78 with UL DC_3_n78 is specified as below.

Table 5.1.168.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 4.9 | IMD4 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1475 | 0 | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |

## 5.1.169		DC_20-32_n78

- 5.1.169.1	 Operating bands for DC

Table 5.1.169.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_20-32_n78 | CA_20-32 | n78 | no |

- 5.1.169.2 	Configuration for DC

Table 5.1.169.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-32A_n78A DC_20A-32A_n78(2A) | DC_20A_n78A | CA_20-32 | n78 |

- 5.1.169.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.169.3-1.

Table 5.1.169.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 2968 | 4132 | 4662 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 | 1576 | 5738 | 6768 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5524 | 7432 | 8462 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 | 714 | 9038 | 10568 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6386 | 10732 | 12262 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5936 | 4876 | 8264 | 9324 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 | 12338 | 148 | 472 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 | 8176 | 4014 | 5104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 16062 | 6628 | 7248 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 13124 | 9096 | 10186 |

Based on the table, There is no harmonic and IMD issue.

- 5.1.169.4	 ∆TIB and ∆RIB values

Table 5.1.169.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-32_n78 | 20 | 0.5 |
|  | 32 | N/A |
|  | n78 | 0.8 |

Table 5.1.169.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-32_n78 | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

- 5.1.169.5	REFSENS requirements

There is no MSD requirements.

## 5.1.170	DC_1-3_n40

- 5.1.170.1	Operating bands for EN-DC

Table 5.1.170.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3_n40 | CA_1-3 | n40 | No |

- 5.1.170.2	Configuration for DC

Table 5.1.170.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A | CA_1A-3A | n40 |

- 5.1.170.3		∆TIB and ∆RIB values

For DC_1-3_n40, the TIB,c and RIB,c values are reused from LTE combination CA_1-3-40, and are given in the tables below.

Table 5.1.170.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n40 | 0.5 |

Table 5.1.170.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n40 | 1 | 0 |
|  | 3 | 0 |
|  | n40 | 0 |

- 5.1.170.4	REFSENS requirements

There is no impact to consider.

## 5.1.171	DC_1-28_n40

- 5.1.171.1	Operating bands for EN-DC

Table 5.1.171.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-28_n40 | CA_1-28 | n40 | No |

- 5.1.171.2	Configuration for DC

Table 5.1.171.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A | CA_1A-28A | n40 |

- 5.1.171.3		∆TIB and ∆RIB values

For DC_1-28_n40, the TIB,c and RIB,c values are reused from LTE combination CA_1-28-40, and are given in the tables below.

Table 5.1.171.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n40 | 1 | 0.6 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |

Table 5.1.171.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n40 | 1 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0 |

- 5.1.171.4	REFSENS requirements

There is IMD4 impact from DC_1_n40 UL affecting band 28 DL. MSD value is derived from DC_1-5_n79 and DC_1-18_n79.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 8.9 | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |

## 5.1.172	DC_3-28_n40

- 5.1.172.1	Operating bands for EN-DC

Table 5.1.172.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-28_n40 | CA_3-28 | n40 | No |

- 5.1.172.2	Configuration for DC

Table 5.1.172.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A | CA_3A-28A | n40 |

- 5.1.172.3		∆TIB and ∆RIB values

For DC_3-28_n40, the TIB,c and RIB,c values are reused from LTE combination CA_3-28-40, and are given in the tables below.

Table 5.1.172.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n40 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |

Table 5.1.172.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n40 | 3 | 0 |
|  | 28 | 0 |
|  | n40 | 0 |

- 5.1.172.4	REFSENS requirements

There is no impact to consider.

## 5.1.173	DC_1-41_n3

- 5.1.173.1		Operating bands for DC

Table 5.1.173.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-41_n3 | CA_1-41 | n3 | No |

- 5.1.173.2	Configuration for DC

Table 5.1.173.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-41A_n3A | DC_41A_n3A | CA_1A-41A | n3A |
| DC_1A-41C_n3A | DC_41A_n3A DC_41C_n3A | CA_1A-41C | n3A |

- 5.1.173.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. There is IMD5 of band 1 UL and band n3 UL falling to band 41 DL, and the MSD could reuse the value for CA_1A-3A-41A.

- 5.1.173.4	∆TIB and ∆RIB values

For DC_1-41_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-41A

Table 5.1.173.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 5.1.173.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n3 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

- 5.1.173.5	REFSENS requirements

Table 5.1.173.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.173.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | TDD | IMD5 ¦3*fB1-2*fn3¦ |

## 5.1.174	DC_3-41_n28

- 5.1.174.1		Operating bands for DC

Table 5.1.174.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n28 | CA_3-41 | n28 | No |

- 5.1.174.2	Configuration for DC

Table 5.1.174.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-41A_n28A | DC_3A_n28A DC_41A_n28A | CA_3A-41A | n28A |
| DC_3A-41C_n28A | DC_3A_n28A DC_41A_n28A DC_41C_n28A | CA_3A-41C | n28A |

- 5.1.174.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there are IMD impacting on the three band in this band combination, as below list:

-	2nd  order IMD generated by dual uplink of Band 41 + Band n28 may also fall into own Rx of band 3, it could reuse the MSD value of DC_3A-7A_n28A.

-    2nd and 3rd  order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band 41.

There is no IMD impact to B41 considering the KDDI’s spectrum.

- 5.1.174.4	∆TIB and ∆RIB values

For DC_3-41_n28, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_3A-28A-41A

Table 5.1.174.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.3 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.174.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n28 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

- 5.1.174.5	REFSENS requirements

Table 5.1.174.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.174.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| Source of IMD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration |  | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) |  |
| DC_3A-41A_n28A DC_3A-41C_n28A |  | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  |  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  |  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 ¦fB41-fn28¦ |
|  |  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  |  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  |  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |
|  |  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  |  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  |  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 ¦2*fB3-fn28¦ |

## 5.1.175	DC_18-41_n77

- 5.1.175.1		Operating bands for DC

Table 5.1.175.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n77 | CA_18-41 | n77 | No |

- 5.1.175.2	Configuration for DC

Table 5.1.175.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n77A | DC_18A_n77A DC_41A_n77A | CA_18A-41A | n77A |
| DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A | CA_18A-41C | n77A |

- 5.1.175.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, the IMD issues for this band combination are as below list

-  2nd and 3rd order IMDs generated by dual uplink of band 18 and band n77 maybe fall into the Rx of band 41.

-  2nd and 3rd order IMDs generated by dual uplink of band 41 and band n77 maybe fall into the Rx of band 18.

There is only IMD5 impact to Band 18 considering KDDI’s spectrum, since B18 only was used in Japan.

- 5.1.175.4	∆TIB and ∆RIB values

For DC_18-41_n77, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.175.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n77 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.175.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n77 | 18 | 0 |
|  | 41 | 0 |
|  | n77 | 0.5 |

- 5.1.175.5	REFSENS requirements

Table 5.1.175.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.175.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | FDD | IMD5 ¦2*fn77-3*fB41¦ |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | FDD | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |

## 5.1.176	DC_18-41_n78

- 5.1.176.1		Operating bands for DC

Table 5.1.176.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n78 | CA_18-41 | n78 | No |

- 5.1.176.2	Configuration for DC

Table 5.1.176.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n78A | DC_18A_n78A DC_41A_n78A | CA_18A-41A | n78A |
| DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A | CA_18A-41C | n78A |

- 5.1.176.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, the IMD issues for this band combination are as below list:

-  2nd order IMDs generated by dual uplink of band 18 and band n78 maybe fall into the Rx of band 41.

-  2nd and 5th order IMDs generated by dual uplink of band 41 and band n78 maybe fall into the Rx of band 18.

There is only IMD5 impact to Band 18 considering KDDI’s spectrum, since B18 only was used in Japan.

- 5.1.176.4	∆TIB and ∆RIB values

For DC_18-41_n78, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.176.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n78 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.176.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n78 | 18 | 0 |
|  | 41 | 0 |
|  | n78 | 0.5 |

- 5.1.176.5	REFSENS requirements

Table 5.1.176.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.176.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| c |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | FDD | IMD5 ¦2*fn78-3*fB41¦ |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | FDD | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |

## 5.1.177		DC_1-3_n71

- 5.1.177.1	 Operating bands for DC

Table 5.1.177.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n71 | CA_1-3 | n71 | no |

- 5.1.177.2 	Configuration for DC

Table 5.1.177.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A | CA_1-3 | n71 |

- 5.1.177.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n71, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.177.3-1.

Table 5.1.177.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 1989 | 2094 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 2652 | 2792 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 3315 | 3490 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1222 | 2583 | 2678 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 | 3297 | 654 | 524 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4658 | 3246 | 3376 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 | 5277 | 9 | 174 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6638 | 3909 | 4074 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2444 | 2634 | 5166 | 5356 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 | 672 | 7257 | 6982 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 | 1971 | 4614 | 4364 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4772 | 8343 | 8618 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6054 | 7086 | 7336 |

For UE coexistence study of Band 3 + Band n71, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.177.3-1.

Table 5.1.177.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 663 | 698 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1326 | 1396 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 1989 | 2094 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 2652 | 2792 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 3315 | 3490 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1012 | 2373 | 2483 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 | 2907 | 459 | 314 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4268 | 3036 | 3181 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 | 4692 | 204 | 384 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6053 | 3699 | 3879 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2024 | 2244 | 4746 | 4966 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 | 867 | 6477 | 6142 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 | 1581 | 4029 | 3734 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4577 | 7503 | 7838 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5664 | 6456 | 6751 |

IMD5 may fall into Rx of band 3 with UL DC_1_n71.

IMD4 may fall into Rx of band 1 with UL DC_3_n71.

- 5.1.177.4	 ∆TIB and ∆RIB values

Table 5.1.177.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n71 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n71 | 0.3 |

Table 5.1.177.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n71 | 1 | 0 |
|  | 3 | 0 |
|  | n71 | 0 |

- 5.1.177.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD4 for DC_1-3_n71 with UL DC_3_n71 is specified as below referring to the MSD for DC_66A_n71A from 38.101-3.

The reference sensitivity exception (MSD) due to IMD5 for DC_1-3_n71 with UL DC_1_n71 is shown as below, based on the discussion.

Table 5.1.177.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | 1960 | 5 | 25 | 2150 | 5 | IMD4 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |

## 5.1.178	DC_1-7_n40

- 5.1.178.1	Operating bands for DC

Table 5.1.178.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n40 | CA_1-7 | n40 | No |

- 5.1.178.2	Configuration for DC

Table 5.1.178.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |

- 5.1.178.3	Coexistence studies

Co-existence studies for DC_1A_n40A are captured in 38.101-3 and for DC_7A_n40A in TR 37.716-11-11. IMD3 will fall in Rx band of 1 and 7.

- 5.1.178.4	∆TIB and ∆RIB values

For DC_1-7_n40, the TIB,c and RIB,c values are reused from CA_1-7-40 in TS 36.101.

Table 5.1.178.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |

Table 5.1.178.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n40 | 1 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |

- 5.1.178.5	REFSENS requirements

MSD is derived from similar combinations already captured for CA in 36.101 and EN-DC in 38.101-3. In this case CA_1A-7A-40A, DC_7_n40 and DC_7A-40A_n1A.

Table 5.1.178.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |

## 5.1.179	DC_3-7_n40

- 5.1.179.1	Operating bands for DC

Table 5.1.179.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n40 | CA_3-7 | n40 | No |

- 5.1.179.2	Configuration for DC

Table 5.1.179.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |

- 5.1.179.3	Coexistence studies

Co-existence studies for DC_3A_n40A are captured in 38.101-3 and for DC_7A_n40A in TR 37.716-11-. IMD5 might fall in Rx band of band 3.

- 5.1.179.4	∆TIB and ∆RIB values

For DC_3-7_n40, the TIB,c and RIB,c values are reused from CA_3-7-40 in TS 36.101.

Table 5.1.179.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |

Table 5.1.179.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |

- 5.1.179.5	REFSENS requirements

MSD is derived from similar combinations already captured for CA in 36.101 and EN-DC in 38.101-3. In this case CA_3A-7A-40A, DC_3A_n40A and DC_7A_n40A.

Table 5.1.179.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-7A_n40A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |

## 5.1.180	DC_7-28_n40

- 5.1.180.1	Operating bands for DC

Table 5.1.180.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_7-28_n40 | CA_7-28 | n40 | No |

- 5.1.180.2	Configuration for DC

Table 5.1.180.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |

- 5.1.180.3	Coexistence studies

Co-existence studies are captured for DC_7A_n40A and DC_28A_n40A in TR 37.716-11-11. IMD5 might fall in Rx band of band 7.

- 5.1.180.4	∆TIB and ∆RIB values

For DC_7-28_n40, the TIB,c and RIB,c values are reused from CA_7-28-40 in TS 36.101.

Table 5.1.180.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n40 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.6 |

Table 5.1.180.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n40 | 7 | 0 |
|  | 28 | 0 |
|  | n40 | 0.5 |

- 5.1.180.5	REFSENS requirements

MSD is derived from similar combinations already captured for CA in 36.101 and EN-DC in 38.101-3. In this case CA_7A-28A-40A, DC_28A_n40A and DC_7A_n40A.

Table 5.1.180.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-28A_n40A | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |

## 5.1.181	DC_2_(n)12

- 5.1.181.1	Operating bands for DC

Table 5.1.181.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2_(n)12 | CA_2-12 | n12 |  |

- 5.1.181.2	Configuration for DC

Table 5.1.181.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A_(n)12AA | DC_2A_n12A DC_(n)12AA1 | CA_2A-12A | n12A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.181.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the third band.

- 5.1.181.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are derived from DC_2_n12.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_(n)12 | 2 | 0.3 |
|  | 12 | 0.3 |
|  | n12 | 0.3 |

Table 5.1.181.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_(n)12 | 2 | 0 |
|  | 12 | 0 |
|  | n12 | 0 |

- 5.1.181.5	REFSENS requirements

No additional REFSENS requirement is needed.

## 5.1.182	DC_5_(n)12

- 5.1.182.1	Operating bands for DC

Table 5.1.182.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5_(n)12 | CA_5-12 | n12 |  |

- 5.1.182.2	Configuration for DC

Table 5.1.182.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A_(n)12AA | DC_5A_n12A DC_(n)12AA1 | CA_5A-12A | n12A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.182.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the third band.

- 5.1.182.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are derived from DC_5_n12.

Table 5..1.62.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_(n)12 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | n12 | 0.4 |

Table 5.1.182.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5_(n)12 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n12 | 0.3 |

- 5.1.182.5	REFSENS requirements

No additional REFSENS requirement is needed.

## 5.1.183	12_(n)5

- 5.1.183.1	Operating bands for DC

Table 5.1.183.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| 12_(n)5 | CA_5-12 | n5 |  |

- 5.1.183.2	Configuration for DC

Table 5.1.183.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_12A_(n)5AA | DC_12A_n5A DC_(n)5AA1 | CA_5A-12A | n5A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.183.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the third band.

- 5.1.183.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are derived from DC_12_n5

Table 5.1.183.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| 12_(n)5 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | n5 | 0.8 |

Table 5.1.183.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| 12_(n)5 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n5 | 0.5 |

- 5.1.183.5	REFSENS requirements

No additional REFSENS requirement is needed.

5.1.184	48_(n)5

- 5.1.184.1	Operating bands for DC

Table 5.1.184.1-1: Band combinations EN-DC (two bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| 48_(n)5 | CA_5-48 | n5 |  |

- 5.1.184.2	Configuration for DC

Table 5.1.184.2-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_48A_(n)5AA | DC_48A_n5A DC_(n)5AA1 | CA_5A-48A | n5A |
| NOTE1: Only single switched UL is supported |  |  |  |

- 5.1.184.3	Coexistence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the third band.

- 5.1.184.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are derived from DC_48_n5

Table 5.1.184.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| 48_(n)5 | 5 | 0.3 |
|  | 48 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.184.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| 48_(n)5 | 5 | 0 |
|  | 48 | 0 |
|  | n5 | 0 |

- 5.1.184.5	REFSENS requirements

No additional REFSENS requirement is needed.

5.1.185	DC_48-66_n71

- 5.1.185.1	Operating bands for EN-DC

Table 5.1.185.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 48-66_n71 | CA_48-66 | n71 | No |

- 5.1.185.2	Configuration for DC

Table 5.1.185.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A | CA_48A-66A | n71 |

- 5.1.185.3		∆TIB and ∆RIB values

For DC_48-66_n71, the TIB,c and RIB,c values for the 48_n71 part are reused from CA_13-48-66 and CA_5-48-66 and are given in the tables below.

Table 5.1.185.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n71 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n71 | 0.3 |

Table 5.1.185.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n71 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n71 | 0 |

- 5.1.185.4	REFSENS requirements

Co-existence analysis from DC_48_n71 shows that there is IMD3 impact from DC_48_n71 UL to Band 66 DL. MSD value for band 66 DL affected by DC_48_n71 UL are derived from 28_n77 UL affecting band 1 DL in DC_1-28_n77 and need to be defined in TS 38.101-3 like in table below.

Co-existence analysis from DC_66_n71 shows that there is IMD4 impact from DC_66_n71 UL to Band 48 DL. MSD value for band 48 DL affected by DC_66_n71 UL are derived from 19-21 UL affecting band 42 DL in CA_19-21-42 and need to be defined in TS 38.101-3 like in table below.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1774 | 5 | 25 | 2174 | 15.8 | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | 48 | 3697.5 | 5 | 25 | 3697.5 | 13.0 | IMD4 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |

## 5.1.186	DC_2-29_n66 and DC_2-2-29_n66

- 5.1.186.1	Operating bands for EN-DC

Table 5.1.186.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-29_n66 | CA_2-29 | n66 |  |
| DC_2-2-29_n66 | CA_2-2-29 | n66 |  |

- 5.1.186.2	Configuration for DC

Table 5.1.186.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A_n66A | DC_2A_n66A | CA_2A-29A | n66 |
| DC_2A-2A-29A_n66A | DC_2A_n66A | CA_2A-2A-29A | n66 |

- 5.1.186.3	Co-existence studies

There is no impact from UL configuration DC_2_n66 into DL band of 29.

- 5.1.186.4		∆TIB and ∆RIB values

For DC_2-29_n66 and DC_2-2-29_n66, the TIB,c and RIB,c values are reused from EN-DC combination DC_2_n66, and are given in the tables below.

Table 5.1.186.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29_n66 DC_2-2-29_n66 | 2 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.186.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29_n66 DC_2-2-29_n66 | 2 | 0.3 |
|  | n66 | 0.3 |

- 5.1.186.5	REFSENS requirements

There is no additional REFSENS requirement for this band combination.

## 5.1.187	DC_29-66_n2 and DC_29-66-66_n2

- 5.1.187.1	Operating bands for EN-DC

Table 5.1.187.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_29-66_n2 | CA_29-66 | n2 |  |
| DC_29-66-66_n2 | CA_29-66-66 | n2 |  |

- 5.1.187.2	Configuration for DC

Table 5.1.187.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_29A-66A_n2A | DC_66A_n2A | CA_29A-66A | n2 |
| DC_29A-66A-66A_n2A | DC_66A_n2A | CA_29A-66A-66A | n2 |

- 5.1.187.3		Co-existence studies

There is no impact from UL configuration DC_66_n2 into DL band of 29.

- 5.1.187.5		∆TIB and ∆RIB values

For DC_29-66_n2 and DC_29-66-66_n2, the TIB,c and RIB,c values are reused from EN-DC combination DC_66_n2, and are given in the tables below.

Table 5.1.187.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-66_n2 DC_29-66-66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.187.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-66_n2 DC_29-66-66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |

- 5.1.187.5	REFSENS requirements

There is no additional REFSENS requirement for this band combination.

## 5.1.188	DC_1-42_n28

- 5.1.188.1	Operating bands for DC_1-42_n28

Table 5.1.188.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-42_n28 | CA_1-42 | n28 | No |

- 5.1.188.2	Configurations for DC_1-42_n28

Table 5.1.188.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-42A_n28A | DC_1A_n28A DC_42A_n28A | CA_1A-42A | n28A |
| DC_1A-42C_n28A | DC_1A_n28A DC_42A_n28A DC_42C_n28A | CA_1A-42C | n28A |

- 5.1.188.3	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n28A, (1) IMD3 of (B1 - n28) will fall into Rx band of Band 42. When Uplink EN-DC configuration is DC_42A_n28A, (2) IMD3 of (B42 - n28) will fall into Rx band of Band 1.

- 5.1.188.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.188.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 5.1.188.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n28 | 1 | 0 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

- 5.1.188.5	REFSENS requirements

As mentioned above, IMD3 of B1 and n28 to Band42 Rx and IMD3 of B42 and n28 to Band1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.188.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |  |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |  |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | TDD | IMD3 |  |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | TDD | N/A |  |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |  |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | FDD | IMD3 |  |

## 5.1.189	DC_3-42_n28

- 5.1.189.1	Operating bands for DC_3-42_n28

Table 5.1.189.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-42_n28 | CA_3-42 | n28 | No |

- 5.1.189.2	Configurations for DC_3-42_n28

Table 5.1.189.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-42A_n28A | DC_3A_n28A DC_42A_n28A | CA_3A-42A | n28A |
| DC_3A-42C_n28A | DC_3A_n28A DC_42A_n28A DC_42C_n28A | CA_3A-42C | n28A |

- 5.1.189.3	Co-existence studies

There is no additional harmonic and intermodulation impact for the additional band receiver.

- 5.1.189.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.189.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-42_n28 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 5.1.189.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-42_n28 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

- 5.1.189.5	REFSENS requirements

As mentioned in 5.1.189.3, REFSENS exceptions are not expected.

## 5.1.190	DC_8-42_n28

- 5.1.190.1	Operating bands for DC_8-42_n28

Table 5.1.190.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-42_n28 | CA_8-42 | n28 | No |

- 5.1.190.2	Configurations for DC_8-42_n28

Table 5.1.190.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-42A_n28A | DC_8A_n28A DC_42A_n28A | CA_8A-42A | n28A |
| DC_8A-42C_n28A | DC_8A_n28A DC_42A_n28A DC_42C_n28A | CA_8A-42C | n28A |

- 5.1.190.3	Co-existence studies

When Uplink EN-DC configuration is DC_8A_n28A, IMD4 will fall into Rx band of Band 42.

- 5.1.190.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.190.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n28 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 5.1.190.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-42_n28 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

- 5.1.190.5	REFSENS requirements

As mentioned in section 3, REFSENS exceptions due to IMD4 are expected. The following values are proposed.

Table 5.1.190.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |  |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |  |
|  | 42 | 3443 | 5 | 25 | 3443 | 8.7 | TDD | IMD4 |  |

## 5.1.191	DC_2-14_n66

- 5.1.191.1	Operating bands for EN-DC

Table 5.1.191.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-14_n66 | CA_2-14 | n66 |  |

- 5.1.191.2	Configuration for DC

Table 5.1.191.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A | CA_2A-14A | n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A | CA_2A-2A-14A | n66A |

- 5.1.191.3		∆TIB and ∆RIB values

For DC_2-14_n66, the TIB,c and RIB,c values are reused from LTE combination CA_2-14-66, and are given in the tables below.

Table 5.1.191.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.191.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.3 |
|  | 14 | 0 |
|  | n66 | 0.3 |

- 5.1.191.4	REFSENS requirements

There is IMD4 impact from DC_14_n66 UL affecting band 2 DL. MSD value is derived from CA_2-13-66 which have IMD4 impact to band 2 from UL_2_13.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |

- 5.1.192	DC_2-14_n2

- 5.1.192.1	Operating bands for EN-DC

Table 5.1.192.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-14_n2 | CA_2-14 | n2 |  |

- 5.1.192.2	Configuration for DC

Table 5.1.192.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A_n2A | DC_2A_n2A1 DC_14A_n2A | CA_2A-14A | n2 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.192.3		∆TIB and ∆RIB values

For DC_2-14_n2, the TIB,c and RIB,c values are reused from LTE combination CA_2-14, and are given in the tables below.

Table 5.1.192.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n2 | 2 | 0.3 |
|  | 14 | 0.3 |
|  | n2 | 0.3 |

Table 5.1.192.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14_n2 | 2 | 0 |
|  | 14 | 0 |
|  | n2 | 0 |

- 5.1.192.4	REFSENS requirements

There is no additional requirement for this band combination.

- 5.1.193	DC_14-66_n2

- 5.1.193.1	Operating bands for EN-DC

Table 5.1.193.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-66_n2 | CA_14-66 | n2 |  |

- 5.1.193.2	Configuration for DC

Table 5.1.193.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A | CA_14A-66A | n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A | CA_14A-66A-66A | n2A |

- 5.1.193.3		∆TIB and ∆RIB values

For DC_14-66_n2, the TIB,c and RIB,c values are reused from LTE combination CA_2-14-66, and are given in the tables below.

Table 5.1.193.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.193.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

- 5.1.193.4	REFSENS requirements

There is IMD4 impact from DC_14_n2 UL affecting band 66 DL. MSD value is derived from CA_2-4-13 which have IMD4 impact to band 4 from UL_2_13.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |

- 5.1.194	DC_46-66_n5

- 5.1.194.1	Operating bands for EN-DC

Table 5.1.194.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 46-66_n5 | CA_46-66 | n5 |  |

- 5.1.194.2	Configuration for DC

Table 5.1.194.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A | DC_66A_n5A | CA_46A-66A CA_46C-66A CA_46D-66A CA_46E-66A | n5A |

- 5.1.194.3		∆TIB and ∆RIB values

For DC_46-66_n5, the TIB,c and RIB,c values are reused from LTE combination CA_5-46-66 and are given in the tables below.

Table 5.1.194.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.194.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n5 | 66 | 0 |
|  | n5 | 0 |

- 5.1.194.4	REFSENS requirements

There is IMD4 impact from DC_66_n5 UL affecting band 46 DL. MSD value is derived from CA_1-5-40 which have IMD4 impact to band 40 from UL_1-5, and from DC_12_n7-n78 which have IMD4 impact to band n78 from UL_12_n7.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_46A-66A_n5A | 46 | 5163 | 10 | 50 | 5163 | 9.0 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |

- 5.1.195	DC_14-66_n66

- 5.1.195.1	Operating bands for EN-DC

Table 5.1.195.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-66_n66 | CA_14-66 | n66 |  |

- 5.1.195.2	Configuration for DC

Table 5.1.195.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A1 | CA_14A-66A | n66 |
| NOTE 1: Only single switched UL is supported |  |  |  |

- 5.1.195.3		∆TIB and ∆RIB values

For DC_14-66_n66, the TIB,c and RIB,c values are reused from LTE combination CA_14-66, and are given in the tables below.

Table 5.1.195.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n66 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.195.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n66 | 14 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

- 5.1.195.4	REFSENS requirements

There is no additional requirement for this band combination.

## 5.1.196		DC_3-(n)41

- 5.1.196.1	 Operating bands for DC

Table 5.1.196.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-(n)41 | CA_3-41 | n41 | no |

- 5.1.196.2 	Configuration for DC

Table 5.1.196.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A | CA_3-41 | n41 |

- 5.1.196.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.196.3-1.

Table 5.1.196.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1422 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

Based on the table, there is no IMD issue for Scell band 41.

IMD4 may fall into band 3 Rx when DC_3_n41 is configured as UL.

- 5.1.196.4	 ∆TIB and ∆RIB values

Table 5.1.196.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-(n)41 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 5.1.196.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-(n)41 | 3 | 0 |
|  | 41 | 03 |
|  |  | 0.54 |
|  | n41 | 03 |
|  |  | 0.54 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

- 5.1.196.5	REFSENS requirements

B3 Rx with UL DC_3A_n41A suffers from IMD4 for which MSD is already specified in 38.101-3. There is no need to further specify the MSD requirements.

## 5.1.197		DC_3-41_n41

- 5.1.197.1	 Operating bands for DC

Table 5.1.197.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n41 | CA_3-41 | n41 | no |

- 5.1.197.2 	Configuration for DC

Table 5.1.197.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A | CA_3-41 | n41 |

- 5.1.197.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.197.3-1.

Table 5.1.197.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1422 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

Based on the table, there is no IMD issue for Scell band 41.

IMD4 may fall into band 3 Rx when DC_3_n41 is configured as UL.

- 5.1.197.4	 ∆TIB and ∆RIB values

Table 5.1.197.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41 | 3 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 5.1.197.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n41 | 3 | 0 |
|  | 41 | 03 |
|  |  | 0.54 |
|  | n41 | 03 |
|  |  | 0.54 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

- 5.1.197.5	REFSENS requirements

B3 Rx with UL DC_3A_n41A suffers from IMD4 for which MSD is already specified in 38.101-3. There is no need to further specify the MSD requirements.

## 5.2	Inter-band EN-DC including FR2

## (three bands)

|  | E-UTRA Band |  |  |
| --- | --- | --- | --- |

.2-1: Inter-band EN-DC configurations (three bands)

## 5.2.1	DC_3-18_n257

- 5.2.1.1	Operating bands for DC

Table 5.2.1.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-18_n257 | CA_3-18 | n257 |  |

- 5.2.1.2	Configurations for DC

Table 5.2.1.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-18A_n257A | DC_3A_n257A DC_18A_n257A | CA_3A-18A | n257 |

- 5.2.1.3	∆TIB and ∆RIB values

Table 5.2.1.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18-n257 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

Table 5.2.1.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18-n257 | 3 | 0 |
|  | 18 | 0 |
|  | n257 | 0 |

## 5.2.2	DC_3-8_n257

- 5.2.2.1	Operating bands for DC_3-8_n257

Table 5.2.2.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-8_n2571 | CA_3-8 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.2.2	Configurations for DC_3-8_n257

Table 5.2.2.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n257A | DC_3A_n257A DC_8A_n257A | CA_3A-8A | n257A |
| DC_3A-8A_n257D DC_3A-8A_n257E DC_3A-8A_n257F | DC_3A_n257A DC_8A_n257A | CA_3A-8A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-8A_n257G DC_3A-8A_n257H DC_3A-8A_n257I DC_3A-8A_n257J DC_3A-8A_n257K DC_3A-8A_n257L DC_3A-8A_n257M | DC_3A_n257A DC_8A_n257A | CA_3A-8A | n257A CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |
| DC_3C-8A_n257A | DC_3A_n257A DC_8A_n257A | CA_3C-8A | n257A |
| DC_3C-8A_n257D DC_3C-8A_n257E DC_3C-8A_n257F | DC_3A_n257A DC_8A_n257A | CA_3C-8A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3C-8A_n257G DC_3C-8A_n257H DC_3C-8A_n257I DC_3C-8A_n257J DC_3C-8A_n257K DC_3C-8A_n257L DC_3C-8A_n257M | DC_3A_n257A DC_8A_n257A | CA_3C-8A | n257A CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.2.3	∆TIB and ∆RIB values

The delta Tib/Rib of this EN-DC are as follows:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n257 | 3 | 0.3 |
|  | 8 | 0.3 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8_n257 | 3 | 0 |
|  | 8 | 0 |

## 5.2.3	DC_28-41_n257

- 5.2.3.1	Operating bands for DC_28-41_n257

Table 5.2.3.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28-41_n2571 | CA_28-41 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.3.2	Configurations for DC_28-41_n257

Table 5.2.3.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_28A-41A_n257A | DC_28A_n257A DC_41A_n257A | CA_28A-41A | n257A |

- 5.2.3.3	∆TIB and ∆RIB values

The delta Tib/Rib of this EN-DC are as follows:

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n257 | 28 | 0.3 |
|  | 41 | 0.3 |

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28-41_n257 | 28 | 0 |
|  | 41 | 0 |

## 5.2.4	DC_2-66_n257

- 5.2.4.1	Operating bands for DC

Table 5.2.4.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n257 | CA_2-66 | n257 | No |

- 5.2.4.2	Configurations for DC

Table 5.2.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n257A DC_2A-66A_n257(2A) | DC_2A_n257A DC_66A_n257A | CA_2A-66A | n257A CA_n257(2A) |

Table 5.2.4.2-2: Supported bandwidths per DC band combination of 2 LTE bands and 1 NR band

| DC operating band / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_2A-66A_n257A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |  |
| DC_2A-66A_n257(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 | See CA_n257(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 440 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 840 |  |  |

- 5.2.4.3	Spurious emission band UE co-existence for DC

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 2 and n257 has has been already made in clause 6.86.3 of TS 37.863-01-01.

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 66 and n257 has has been already made in clause 6.62.3 of TS 37.863-01-01.

Therefore, we skip these analysis in this TR.

Table 6.X.3-3 lists the protected bands required for the dual connectivity configuration.

Table 5.2.4.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2-66_n257 | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

- 5.2.4.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

- 5.2.4.5	∆TIB and ∆RIB values

For DC_2-66-n257, the TIB,c and RIB values are given in the tables below, reusing LTE CA configuration CA_2-66 for band 2 and 66 while no relaxation is introduced to band n257.

Table 5.2.4.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n257 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n257 | 0 |

Table 5.2.4.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n257 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n257 | 0 |

## 5.2.5	DC_2-66_n261

- 5.2.5.1	Operating bands for DC

Table 5.2.5.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_ n261 | CA_2-66 | n261 | No |

- 5.2.5.2	Configuration for DC

Table 5.2.5.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n261A DC_2A-66A_n261(2A) | DC_2A_n261A DC_66A_n261A | CA_2A-66A | n261A CA_n261(2A) |

Table 5.2.5.2-2: Supported bandwidths per DC band combination of 2 LTE bands and 1 NR band

| DC operating band / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz |  |  | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_2A-66A_n261A | 2 | 15 | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  | 240 |  |  |
|  |  | 120 |  |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  | 440 |  |  |
| DC_2A-66A_n261(2A) | 2 | 15 | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  |  |  |  | 440 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 840 |  |  |

- 5.2.5.3	Spurious emission band UE co-existence for DC

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 2 and n261 has has been already made in TR 37.716-01-01.

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 66 and n261 has has been already made in clause 6.93.3 of TS 37.863-01-01.

Therefore, we skip these analysis in this TR.

Table 5.2.5.3-3 lists the protected bands required for the dual connectivity configuration.

Table 5.2.5.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-66A_n261A DC_2A-66A_n261(2A) | Bands 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 48, 66, 70, 71, n260 | FDL_low | - | FDL_high | -50 | 1 |  |

- 5.2.5.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

- 5.2.5.5	∆TIB and ∆RIB values

For DC_2-66-n261, the TIB,c and RIB values are given in the tables below, reusing LTE CA configuration CA_2-66 for band 2 and 66 while no relaxation is introduced to band n261.

Table 5.2.5.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n261 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n261 | 0 |

Table 6.Y.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n261 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n261 | 0 |

## 5.2.6	DC_2-66_n260

- 5.2.6.1	Operating bands and Configurations for DC

Table 5.2.6.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n260 | CA_2-66 | n260 | No |

Table 5.2.6.1-2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n260(2A) DC_2A-66A_n260(3A) DC_2A-66A_n260(4A) | DC_2A_n260A DC_66A_n260A | CA_2A-66A | n260(2A) n260(3A) n260(4A) |

- 5.2.6.2	Channel bandwidths per operating band for DC

Table 5.2.6.2-1 Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | [200 MHz] | 400 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-66A_n260(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 (440 for Band n260 with 60 kHz SCS) |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260(2A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260(3A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 1240 (640 for Band n260 with 60 kHz SCS) |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260(3A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n260(4A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 1640 (840 for Band n260 with 60 kHz SCS) |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  | n260 | 60 | See CA_n260(4A) Bandwidth Combination in Table 5.5A.2-1 of 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |

- 5.2.6.3	Co-existence studies

Co-existence was studied for the lower order combination DC_2A-66A_n260A in Rel-15. These results are captured in 37.863-02-01. Bases on these results it is shown that neither DC_2A_n260A in UL or DC_66A_n260A in the UL cause any co-existence issues. Up to 7th order harmonics have no impact to any own Rx frequencies in LTE or NR bands. Therefor it is concluded that no further studies are needed for these new combinations.

- 5.2.6.4	∆TIB and ∆RIB values

For DC_2A-66A_n260(2A), DC_2A-66A_n260(3A) and DC_2A-66A_n260(4A) the TIB,c and RIB values are same as for the lower order combination DC_2A-66A_n260A given in the tables below.

Table 5.2.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n260A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.2.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-66A_n260A | 2 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

- 5.2.6.5	REFSENS

There is no specific REFSENS requirements.

## 5.2.7	DC_46-66_n261

- 5.2.7.1	Operating bands for DC

Table 5.2.7.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n261 | CA_46-66 | n261 | No |

- 5.2.7.2	Configurations for DC

Table 5.2.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_46A-66A_n261A DC_46C-66A_n261A DC_46D-66A_n261A DC_46A-66A_n261(2A) DC_46C-66A_n261(2A) DC_46D-66A_n261(2A) | DC_66A_n261A | DC_46A-66A DC_46C-66A DC_46D-66A DC_46A-66A DC_46C-66A DC_46D-66A | n261A CA_n261(2A) |

Table 5.2.7.2-2: Supported bandwidths per DC band combination of 2 E-UTRA bands and 1 NR band

| DC operating band / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |  |
| DC_46A-66A_n261A | 46 | 15 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |  |  |
| DC_46C-66A_n261A | 46 | 15 | See CA_46C bandwidth combination set 0 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 260 |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 460 |  |  |  |
| DC_46D-66A_n261A | 46 | 15 | See CA_46D bandwidth combination set 0 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 280 |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 480 |  |  |  |
| DC_46A-66A_n261(2A) | 46 | 15 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 440 |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 840 |  |  |  |
| DC_46C-66A_n261(2A) | 46 | 15 | See CA_46C bandwidth combination set 0 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 460 |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 860 |  |  |  |
| DC_46D-66A_n261(2A) | 46 | 15 | See CA_46D bandwidth combination set 0in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 480 |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 880 |  |  |  |

- 5.2.7.3	Spurious emission band UE co-existence for DC

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 66 and n261 has been already made in clause 6.62.3 of TS 37.863-01-01.

Table 5.2.7.3-3 lists the protected bands required for the dual connectivity configuration.

Table 5.2.7.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_46-66_n261 | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

- 5.2.7.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the E-UTRA bands below 6GHz and FR2 NR band due to large frequency separation.

The third order harmonic of band 66 may fall into band 46 as already specified in E-UTRA CA_46A-66A. The exclusion zone specified in TS36.101 Table 7.3.1A-0eC is applicable to have no MSD also for this EN-DC. The MSD analysis available in clause 6.23 of TR36.714-02-01 is also applicable to this EN-DC.

- 5.2.7.5	∆TIB and ∆RIB values

For DC_46-66-n261, the TIB,c and RIB values are given in the tables below, reusing E-UTRA CA configuration CA_46-66 for band 46 and 66 while no relaxation is introduced to band n261.

Note that RIB for band 46 is defined relative to the REFSENS of E-UTRA CA_46A-66A in TS36.101 as the single carrier REFSENS is not defined for band 46.

Table 5.2.7.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n261 | 66 | 0 |
|  | n261 | 0 |

Table 5.2.7.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n261 | 46 | 0 |
|  | 66 | 0 |
|  | n261 | 0 |

## 5.2.8	DC_18-42_n257

- 5.2.8.1	Operating bands for DC

Table 5.2.8.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-42_n257 | CA_18-42 | n257 | No |

- 5.2.8.2	Configurations for DC

Table 5.2.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-42A_n257A | DC_18A_n257A DC_42A_n257A | CA_18A-42A | n257 |
| DC_18A-42A_n257F | DC_18A_n257A DC_42A_n257A | CA_18A-42A | n257 |
| DC_18A-42A_n257M | DC_18A_n257A DC_42A_n257A | CA_18A-42A | n257 |
| DC_18A-42C_n257A | DC_18A_n257A DC_42A_n257A | CA_18A-42C | n257 |
| DC_18A-42C_n257F | DC_18A_n257A DC_42A_n257A | CA_18A-42C | n257 |
| DC_18A-42C_n257M | DC_18A_n257A DC_42A_n257A | CA_18A-42C | n257 |

- 5.2.8.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration.

- 5.2.8.4	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_19-42_n257 can be applied.

Table 5.2.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-42_n257 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 5.2.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18-42_n257 | 18 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

- 5.2.8.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.2.9	DC_1-18_n257

- 5.2.9.1	Operating bands for DC

Table 5.2.9.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-18_n257 | CA_1-18 | n257 | No |

- 5.2.9.2	Configurations for DC

Table 5.2.9.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-18A_n257F | DC_18A_n257A DC_1A_n257A | CA_1A-18A | n257 |
| DC_1A-18A_n257M | DC_18A_n257A DC_1A_n257A | CA_1A-18A | n257 |

- 5.2.9.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration.

- 5.2.9.4	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_1-19_n257 can be applied.

Table 5.2.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n257 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

Table 5.2.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n257 | 1 | 0 |
|  | 18 | 0 |
|  | n257 | 0 |

- 5.2.9.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.2.10	DC_3-18_n257

- 5.2.10.1	Operating bands for DC

Table 5.2.10.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-18_n257 | CA_3-18 | n257 | No |

- 5.2.10.2	Configurations for DC

Table 5.2.10.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-18A_n257F | DC_18A_n257A DC_3A_n257A | CA_3A-18A | n257 |
| DC_3A-18A_n257M | DC_18A_n257A DC_3A_n257A | CA_3A-18A | n257 |

- 5.2.10.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration.

- 5.2.10.4	∆TIB and ∆RIB values

The same ∆TIB and ∆RIB values of DC_3-19_n257 can be applied.

Table 5.2.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n257 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

Table 5.2.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18_n257 | 3 | 0 |
|  | 18 | 0 |
|  | n257 | 0 |

- 5.2.10.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.2.11	DC_1-41_n257

- 5.2.11.1	Operating bands for DC

Table 5.2.11.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-41_n257 | CA_1-41 | n257 | No |

- 5.2.11.2	Configurations for DC

Table 5.2.11.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-41A_n257F | DC_1A_n257A DC_41A_n257A | CA_1A-41A | n257 |
| DC_1A-41A_n257M | DC_1A_n257A DC_41A_n257A | CA_1A-41A | n257 |
| DC_1A-41C_n257F | DC_1A_n257A DC_41A_n257A | CA_1A-41C | n257 |
| DC_1A-41C_n257M | DC_1A_n257A DC_41A_n257A | CA_1A-41C | n257 |

- 5.2.11.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration.

- 5.2.11.4	∆TIB and ∆RIB values

Table 5.2.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n257 | 0 |

Table 5.2.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n257 | 0 |

- 5.2.11.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.2.12	DC_3-41_n257

- 5.2.12.1	Operating bands for DC

Table 5.2.12.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-41_n257 | CA_3-41 | n257 | No |

- 5.2.12.2	Configurations for DC

Table 5.2.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-41A_n257F | DC_3A_n257A DC_41A_n257A | CA_3A-41A | n257 |
| DC_3A-41A_n257M | DC_3A_n257A DC_41A_n257A | CA_3A-41A | n257 |
| DC_3A-41C_n257F | DC_3A_n257A DC_41A_n257A | CA_3A-41C | n257 |
| DC_3A-41C_n257M | DC_3A_n257A DC_41A_n257A | CA_3A-41C | n257 |

- 5.2.12.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration.

- 5.2.12.4	∆TIB and ∆RIB values

Table 5.2.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n257 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 5.2.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n257 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- 5.2.12.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.2.13	DC_41-42_n257

- 5.2.13.1	Operating bands for DC

Table 5.2.13.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_41-42_n257 | CA_41-42 | n257 | No |

- 5.2.13.2	Configurations for DC

Table 5.2.13.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_41A-42A_n257F | DC_41A_n257A DC_42A_n257A | CA_41A-42A | n257 |
| DC_41A-42A_n257M | DC_41A_n257A DC_42A_n257A | CA_41A-42A | n257 |
| DC_41A-42C_n257F | DC_41A_n257A DC_42A_n257A | CA_41A-42C | n257 |
| DC_41A-42C_n257M | DC_41A_n257A DC_42A_n257A | CA_41A-42C | n257 |
| DC_41C-42A_n257F | DC_41A_n257A DC_42A_n257A | CA_41C-42A | n257 |
| DC_41C-42A_n257M | DC_41A_n257A DC_42A_n257A | CA_41C-42A | n257 |
| DC_41C-42C_n257F | DC_41A_n257A DC_42A_n257A | CA_41C-42C | n257 |
| DC_41C-42C_n257M | DC_41A_n257A DC_42A_n257A | CA_41C-42C | n257 |

- 5.2.13.3	Co-existence studies

The co-existence studies for the lower order DC configurations are already captured in TR 37.863-01-01. For the additional 3rd receiver band, there is no MSD issue for this DC configuration.

- 5.2.13.4	∆TIB and ∆RIB values

Table 5.1.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41-42_n257 | 41 | 0.3 |
|  | 42 | 0.8 |
|  | n257 | 0 |

Table 5.1.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41-42_n257 | 41 | 0 |
|  | 42 | 0.5 |
|  | n257 | 0 |

- 5.2.13.5	REFSENS requirements

No REFSENS requirements are expected for this DC configuration.

## 5.2.14	DC_46-66_n260

- 5.2.14.1	Operating bands for EN-DC

Table 5.2.14.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n260 | CA_46-66 | n260 |  |

- 5.2.14.2	Configuration for DC

Table 5.2.14.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_46A-66A_n260A DC_46C-66A_n260A DC_46D-66A_n260A DC_46E-66A_n260A DC_46A-66A_n260G DC_46C-66A_n260G DC_46D-66A_n260G DC_46E-66A_n260G DC_46A-66A_n260H DC_46C-66A_n260H DC_46D-66A_n260H DC_46E-66A_n260H DC_46A-66A_n260I DC_46C-66A_n260I DC_46D-66A_n260I DC_46E-66A_n260I DC_46A-66A_n260J DC_46C-66A_n260J DC_46D-66A_n260J DC_46E-66A_n260J DC_46A-66A_n260K DC_46C-66A_n260K DC_46D-66A_n260K DC_46E-66A_n260K DC_46A-66A_n260L DC_46C-66A_n260L DC_46D-66A_n260L DC_46E-66A_n260L DC_46A-66A_n260M DC_46C-66A_n260M DC_46D-66A_n260M DC_46E-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M | CA_46A-66A CA_46C-66A CA_46D-66A CA_46E-66A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_46A-66A_n260(2A) DC_46C-66A_n260(2A) DC_46D-66A_n260(2A) | DC_66A_n260A | CA_46A-66A CA_46C-66A CA_46D-66A | CA_n260(2A) |

- 5.2.14.3		∆TIB and ∆RIB values

Table 5.2.14.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n260 | 66 | 0 |
|  | n260 | 0 |

Table 5.2.14.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n260 | 66 | 0 |

## 5.2.15	DC_2-29_n260, DC_29-30_n260

- 5.2.15.1	Operating bands for EN-DC

Table 5.2.15.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-29_n260 | CA_2-29 | n260 |  |
| 29-30_n260 | CA_29-30 | n260 |  |

- 5.2.15.2	Configuration for DC

Table 5.2.15.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A_n260A DC_2A-29A_n260G DC_2A-29A_n260H DC_2A-29A_n260I DC_2A-29A_n260J DC_2A-29A_n260K DC_2A-29A_n260L DC_2A-29A_n260M | DC_2A_n260A | CA_2A-29A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_29A-30A_n260A DC_29A-30A_n260G DC_29A-30A_n260H DC_29A-30A_n260I DC_29A-30A_n260J DC_29A-30A_n260K DC_29A-30A_n260L DC_29A-30A_n260M | DC_30A_n260A | CA_29A-30A | n260 CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |

- 5.2.15.3		∆TIB and ∆RIB values

Table 5.2.15.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29_n260 | 2 | 0.3 |
|  | n260 | 0 |
| DC_29-30_n260 | 30 | 0.3 |
|  | n260 | 0 |

Table 5.2.15.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29_n260 | 2 | 0 |
|  | n260 | 0 |
| DC_29-30_n260 | 30 | 0 |
|  | n260 | 0 |

## 5.2.16	DC_1-3_n257

- 5.2.16.1	Operating bands for DC

Table 5.2.16.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n257 | CA_1-3 | n257 | No |

- 5.2.16.2	Configuration for DC

Table 5.2.16.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n257D DC_1A-3A_n257E DC_1A-3A_n257F DC_1A-3A_n257G DC_1A-3A_n257H DC_1A-3A_n257I DC_1A-3A_n257J DC_1A-3A_n257K DC_1A-3A_n257L DC_1A-3A_n257M | DC_1A_n257A DC_3A_n257A | CA_1A-3A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.16.3	∆TIB and ∆RIB values

Table 5.2.16.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n257 | 0 |

Table 5.2.16.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n257 | 1 | 0 |
|  | 3 | 0 |
|  | n257 | 0 |

## 5.2.17	DC_1-5_n257

- 5.2.17.1	Operating bands for DC

Table 5.2.17.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-5_n257 | CA_1-5 | n257 | No |

- 5.2.17.2	Configuration for DC

Table 5.2.17.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-5A_n257D DC_1A-5A_n257E DC_1A-5A_n257F DC_1A-5A_n257G DC_1A-5A_n257H DC_1A-5A_n257I DC_1A-5A_n257J DC_1A-5A_n257K DC_1A-5A_n257L DC_1A-5A_n257M | DC_1A_n257A DC_5A_n257A | CA_1A-5A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.17.3	∆TIB and ∆RIB values

Table 5.2.17.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A_n257 | 1 | 0.3 |
|  | 5 | 0.3 |
|  | n257 | 0 |

Table 5.2.17.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A_n257 | 1 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

## 5.2.18	DC_1-7_n257

- 5.2.18.1	Operating bands for DC

Table 5.2.18.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7_n257 | CA_1-7 | n257 | No |

- 5.2.18.2	Configuration for DC

Table 5.2.18.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A_n257D DC_1A-7A_n257E DC_1A-7A_n257F DC_1A-7A_n257G DC_1A-7A_n257H DC_1A-7A_n257I DC_1A-7A_n257J DC_1A-7A_n257K DC_1A-7A_n257L DC_1A-7A_n257M | DC_1A_n257A DC_7A_n257A | CA_1A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.18.3	∆TIB and ∆RIB values

Table 5.2.18.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n257 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n257 | 0 |

Table 5.2.18.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n257 | 1 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.19	DC_1-7-7_n257

- 5.2.19.1	Operating bands for DC

Table 5.2.19.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-7-7_n257 | CA_1-7-7 | n257 | No |

- 5.2.19.2	Configuration for DC

Table 5.2.19.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A-7A_n257D DC_1A-7A-7A_n257E DC_1A-7A-7A_n257F DC_1A-7A-7A_n257G DC_1A-7A-7A_n257H DC_1A-7A-7A_n257I DC_1A-7A-7A_n257J DC_1A-7A-7A_n257K DC_1A-7A-7A_n257L DC_1A-7A-7A_n257M | DC_1A_n257A DC_7A_n257A | CA_1A-7A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.19.3	∆TIB and ∆RIB values

Table 5.2.19.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-7A_n257 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n257 | 0 |

Table 5.2.19.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-7A_n257 | 1 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.20	DC_3-5_n257

- 5.2.20.1	Operating bands for DC

Table 5.2.20.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-5_n257 | CA_3-5 | n257 | No |

- 5.2.20.2	Configuration for DC

Table 5.2.20.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-5A_n257D DC_3A-5A_n257E DC_3A-5A_n257F DC_3A-5A_n257G DC_3A-5A_n257H DC_3A-5A_n257I DC_3A-5A_n257J DC_3A-5A_n257K DC_3A-5A_n257L DC_3A-5A_n257M | DC_3A_n257A DC_5A_n257A | CA_3A-5A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.20.3	∆TIB and ∆RIB values

Table 5.2.20.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A_n257 | 3 | 0.3 |
|  | 5 | 0.3 |
|  | n257 | 0 |

Table 5.2.20.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A_n257 | 3 | 0 |
|  | 5 | 0 |
|  | n257 | 0 |

## 5.2.21	DC_3-7_n257

- 5.2.21.1	Operating bands for DC

Table 5.2.21.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n257 | CA_3-7 | n257 | No |

- 5.2.21.2	Configuration for DC

Table 5.2.21.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n257D DC_3A-7A_n257E DC_3A-7A_n257F DC_3A-7A_n257G DC_3A-7A_n257H DC_3A-7A_n257I DC_3A-7A_n257J DC_3A-7A_n257K DC_3A-7A_n257L DC_3A-7A_n257M | DC_3A_n257A DC_7A_n257A | CA_3A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.21.3	∆TIB and ∆RIB values

Table 5.2.21.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A_n257 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n257 | 0 |

Table 5.2.21.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A_n257 | 3 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.22	DC_3-7-7_n257

- 5.2.22.1	Operating bands for DC

Table 5.2.22.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7-7_n257 | CA_3-7-7 | n257 | No |

- 5.2.22.2	Configuration for DC

Table 5.2.22.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-7A_n257D DC_3A-7A-7A_n257E DC_3A-7A-7A_n257F DC_3A-7A-7A_n257G DC_3A-7A-7A_n257H DC_3A-7A-7A_n257I DC_3A-7A-7A_n257J DC_3A-7A-7A_n257K DC_3A-7A-7A_n257L DC_3A-7A-7A_n257M | DC_3A_n257A DC_7A_n257A | CA_3A-7A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.22.3	∆TIB and ∆RIB values

Table 5.2.22.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-7A_n257 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n257 | 0 |

Table 5.2.22.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-7A_n257 | 3 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.23	DC_5-7_n257

- 5.2.23.1	Operating bands for DC

Table 5.2.23.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-7_n257 | CA_5-7 | n257 | No |

- 5.2.23.2	Configuration for DC

Table 5.2.23.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-7A_n257D DC_5A-7A_n257E DC_5A-7A_n257F DC_5A-7A_n257G DC_5A-7A_n257H DC_5A-7A_n257I DC_5A-7A_n257J DC_5A-7A_n257K DC_5A-7A_n257L DC_5A-7A_n257M | DC_5A_n257A DC_7A_n257A | CA_5A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.23.3	∆TIB and ∆RIB values

Table 5.2.23.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A_n257 | 5 | 0.3 |
|  | 7 | 0.3 |
|  | n257 | 0 |

Table 5.2.23.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A_n257 | 5 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.24	DC_5-7-7_n257

- 5.2.24.1	Operating bands for DC

Table 5.2.24.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-7-7_n257 | CA_5-7-7 | n257 | No |

- 5.2.24.2	Configuration for DC

Table 5.2.24.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-7A-7A_n257D DC_5A-7A-7A_n257E DC_5A-7A-7A_n257F DC_5A-7A-7A_n257G DC_5A-7A-7A_n257H DC_5A-7A-7A_n257I DC_5A-7A-7A_n257J DC_5A-7A-7A_n257K DC_5A-7A-7A_n257L DC_5A-7A-7A_n257M | DC_5A_n257A DC_7A_n257A | CA_5A-7A-7A | CA_n257D CA_n257E CA_n257F CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |

- 5.2.24.3	∆TIB and ∆RIB values

Table 5.2.24.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A-7A_n257 | 5 | 0.3 |
|  | 7 | 0.3 |
|  | n257 | 0 |

Table 5.2.24.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A-7A_n257 | 5 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.25	DC_1-11_n257

- 5.2.25.1	Operating bands for DC_1-11_n257

Table 5.2.25.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n257 | CA_1-11 | n257 | No |

- 5.2.25.2	Configurations for DC_1-11_n257

Table 5.2.25.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n257A | DC_1A_n257A DC_11A_n257A | CA_1A-11A | n257A |
| DC_1A-11A_n257D | DC_1A_n257A DC_11A_n257A | CA_1A-11A | n257D |

- 5.2.25.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.2.25.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n257 | 1 | 0.3 |
|  | 11 | 0.3 |
|  | n257 | 0 |

Table 5.2.25.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n257 | 1 | 0 |
|  | 11 | 0 |
|  | n257 | 0 |

## 5.2.26	DC_8-11_n257

- 5.2.26.1	Operating bands for DC_8-11_n257

Table 5.2.26.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_8-11_n257 | CA_8-11 | n257 | No |

- 5.2.26.2	Configurations for DC_8-11_n257

Table 5.2.26.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_8A-11A_n257A | DC_8A_n257A DC_11A_n257A | CA_8A-11A | n257A |
| DC_8A-11A_n257D | DC_8A_n257A DC_11A_n257A | CA_8A-11A | n257D |

- 5.2.26.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.2.26.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n257 | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n257 | 0 |

Table 5.2.26.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-11_n257 | 8 | 0 |
|  | 11 | 0 |
|  | n257 | 0 |

## 5.2.27	DC_3-3-7_n257, DC_3-3-7-7_n257

- 5.2.27.1	Operating bands for DC

Table 5.2.27.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-3-7_n2571 | CA_3-3-7 | n257 | No |
| DC_3-3-7-7_n2571 | CA_3-3-7-7 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.27.2	Configuration for DC

Table 5.2.27.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A-3A-7A_n257A DC_3A-3A-7A_n257D DC_3A-3A-7A_n257E DC_3A-3A-7A_n257F DC_3A-3A-7A_n257G DC_3A-3A-7A_n257H DC_3A-3A-7A_n257I DC_3A-3A-7A_n257J DC_3A-3A-7A_n257K DC_3A-3A-7A_n257L DC_3A-3A-7A_n257M | DC_3A_n257A DC_7A_n257A | CA_3A-3A-7A | n257A |
| DC_3A-3A-7A-7A_n257A DC_3A-3A-7A-7A_n257D DC_3A-3A-7A-7A_n257E DC_3A-3A-7A-7A_n257F DC_3A-3A-7A-7A_n257G DC_3A-3A-7A-7A_n257H DC_3A-3A-7A-7A_n257I DC_3A-3A-7A-7A_n257J DC_3A-3A-7A-7A_n257K DC_3A-3A-7A-7A_n257L DC_3A-3A-7A-7A_n257M | DC_3A_n257A DC_7A_n257A | CA_3A-3A-7A-7A | n257A |

- 5.2.27.3	∆TIB and ∆RIB values

Table 5.2.27.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n257; DC_3-3-7-7_n257 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n257 | 0 |

Table 5.2.27.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n257; DC_3-3-7-7_n257 | 3 | 0 |
|  | 7 | 0 |
|  | n257 | 0 |

## 5.2.28	DC_2-46_n261

- 5.2.28.1	Operating bands for DC

Table 5.2.28.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-46_n261 | CA_2-46 | n261 | No |

- 5.2.28.2	Configurations for DC

Table 5.2.28.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A_n261A DC_2A-46C_n261A DC_2A-46D_n261A | DC_2A_n261A |

Table 5.2.28.2-2: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A_n261(2A) DC_2A-46C_n261(2A) DC_2A-46D_n261(2A) | DC_2A_n261A |

- 5.2.28.3	∆TIB and ∆RIB values

Table 5.2.28.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n261 | 2 | 0 |
|  | 46 | 0 |
|  | n261 | 0 |

Table 5.2.28.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n261 | 2 | 0 |
|  | 46 | 0 |
|  | n261 | 0 |

## 5.2.29	DC_46-48_n261

- 5.2.29.1	Operating bands for DC

Table 5.2.29.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-48_n261 | CA_46-48 | n261 | No |

- 5.2.29.2	Configurations for DC

Table 5.2.29.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_46A-48A_n261A | DC_48A_n261A | CA_46A-48A | n261A |
| DC_46C-48A_n261A | DC_48A_n261A | CA _46C-48A | n261A |
| DC_46D-48A_n261A | DC_48A_n261A | CA _46D-48A | n261A |
| DC_46A-48C_n261A | DC_48C_n261A | CA _46A-48C | n261A |
| DC_46A-48D_n261A | DC_48C_n261A | CA _46A-48D | n261A |
| DC_46C-48C_n261A | DC_48C_n261A | CA _46C-48C | n261A |
| DC_46C-48D_n261A | DC_48C_n261A | CA _46C-48D | n261A |
| DC_46D-48C_n261A | DC_48C_n261A | CA _46D-48C | n261A |
| DC_46D-48D_n261A | DC_48C_n261A | CA _46D-48D | n261A |
| DC_46A-48A_n261(2A) | DC_48A_n261A | CA _46A-48A | n261(2A) |
| DC_46C-48A_n261(2A) | DC_48A_n261A | CA _46C-48A | n261(2A) |
| DC_46D-48A_n261(2A) | DC_48A_n261A | CA _46D-48A | n261(2A) |
| DC_46A-48C_n261(2A) | DC_48C_n261A | CA _46A-48C | n261(2A) |
| DC_46A-48D_n261(2A) | DC_48C_n261A | CA _46A-48D | n261(2A) |
| DC_46C-48C_n261(2A) | DC_48C_n261A | CA _46C-48C | n261(2A) |
| DC_46C-48D_n261(2A) | DC_48C_n261A | CA _46C-48D | n261(2A) |
| DC_46D-48C_n261(2A) | DC_48C_n261A | CA _46D-48C | n261(2A) |
| DC_46D-48D_n261(2A) | DC_48C_n261A | CA _46D-48D | n261(2A) |

- 5.2.29.3	∆TIB and ∆RIB values

Table 5.2.29.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-48_n261 | 46 | 0 |
|  | 48 | 0.8 |
|  | n261 | 0 |

Table 5.2.29.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-48_n261 | 46 | 0 |
|  | 48 | 0.5 |
|  | n261 | 0 |

Delta values chosen based on similar values for CA_46A-48A and CA_46A-48C.

## 5.2.30	DC_46-48_n260

- 5.2.30.1	Operating bands for EN-DC

Table 5.2.30.1-1 Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-48_n260 | CA_46-48 | n260 | No |

- 5.2.30.2	Configuration for DC

Table 5.2.30.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_46A-48A_n260A DC_46C-48A_n260A DC_46D-48A_n260A DC_46A-48C_n260A DC_46A-48D_n260A DC_46C-48C_n260A DC_46C-48D_n260A DC_46D-48C_n260A DC_46D-48D_n260A | DC_48A_n260A DC_48C_n260A | CA_46A-48A CA_46C-48A CA_46D-48A CA_46A-48C CA_46A-48D CA_46C-48C CA_46C-48D CA_46D-48C CA_46D-48D | n260A |
| DC_46A-48A_n260(2A) DC_46C-48A_n260(2A) DC_46D-48A_n260(2A) DC_46A-48C_n260(2A) DC_46A-48D_n260(2A) DC_46C-48C_n260(2A) DC_46C-48D_n260(2A) DC_46D-48C_n260(2A) DC_46D-48D_n260(2A) | DC_48A_n260A DC_48C_n260A | CA_46A-48A CA_46C-48A CA_46D-48A CA_46A-48C CA_46A-48D CA_46C-48C CA_46C-48D CA_46D-48C CA_46D-48D | n260(2A) |
| DC_46A-48A_n260(3A) DC_46C-48A_n260(3A) DC_46D-48A_n260(3A) DC_46A-48C_n260(3A) DC_46A-48D_n260(3A) DC_46C-48C_n260(3A) DC_46C-48D_n260(3A) DC_46D-48C_n260(3A) DC_46D-48D_n260(3A) | DC_48A_n260A DC_48C_n260A | CA_46A-48A CA_46C-48A CA_46D-48A CA_46A-48C CA_46A-48D CA_46C-48C CA_46C-48D CA_46D-48C CA_46D-48D | n260(3A) |
| DC_46A-48A_n260(4A) DC_46C-48A_n260(4A) DC_46D-48A_n260(4A) DC_46A-48C_n260(4A) DC_46A-48D_n260(4A) DC_46C-48C_n260(4A) DC_46C-48D_n260(4A) DC_46D-48C_n260(4A) DC_46D-48D_n260(4A) | DC_48A_n260A DC_48C_n260A | CA_46A-48A CA_46C-48A CA_46D-48A CA_46A-48C CA_46A-48D CA_46C-48C CA_46C-48D CA_46D-48C CA_46D-48D | n260(4A) |

- 5.2.30.3	∆TIB and ∆RIB values

Table 5.2.30.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-48_n260 | 46 | 0 |
|  | 48 | 0.8 |
|  | n260 | 0 |

Table 5.2.30.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_46-48_n260 | 46 | 0 |
|  | 48 | 0.5 |
|  | n260 | 0 |

## 5.2.31	DC_14-30_n260

- 5.2.31.1	Operating bands for EN-DC

Table 5.2.31.1-1 Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_14-30_n260 | CA_14-30 | n260 | No |

- 5.2.31.2	Configuration for DC

Table 5.2.31.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A_n260A DC_14A-30A_n260G DC_14A-30A_n260H DC_14A-30A_n260I DC_14A-30A_n260J DC_14A-30A_n260K DC_14A-30A_n260L DC_14A-30A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M | CA_14A-30A CA_14A-30A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |

- 5.2.31.3	∆TIB and ∆RIB values

Table 5.2.31.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n260 | 14 | 0.3 |
|  | 30 | 0.3 |
|  | n260 | 0 |

Table 5.2.31.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_14-30_n260 | 14 | 0 |
|  | 30 | 0 |
|  | n260 | 0 |

## 5.2.32	DC_14-66_n260, DC_14-66-66_n260

- 5.2.32.1	Operating bands for EN-DC

Table 5.2.32.1-1 Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_14-66_n260 | CA_14-66 | n260 | No |
| DC_14-66-66_n260 | CA_14-66-66 | n260 | No |

- 5.2.32.2	Configuration for DC

Table 5.2.32.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I DC_14A-66A_n260J DC_14A-66A_n260K DC_14A-66A_n260L DC_14A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M | CA_14A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I DC_14A-66A-66A_n260J DC_14A-66A-66A_n260K DC_14A-66A-66A_n260L DC_14A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M | CA_14A-66A-66A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |

- 5.2.32.3	∆TIB and ∆RIB values

Table 5.2.32.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n260, DC_14-66-66_n260 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

Table 5.2.32.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n260, DC_14-66-66_n260 | 14 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

## 5.2.33	DC_2-14_n260, DC_2-2-14_n260

- 5.2.33.1	Operating bands for EN-DC

Table 5.2.33.1-1 Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-14_n260 | CA_2-14 | n260 | No |
| DC_2-2-14_n260 | CA_2-2-14 | n260 | No |

- 5.2.33.2	Configuration for DC

Table 5.2.33.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A_n260A DC_2A-14A_n260G DC_2A-14A_n260H DC_2A-14A_n260I DC_2A-14A_n260J DC_2A-14A_n260K DC_2A-14A_n260L DC_2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M | CA_2A-14A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-14A_n260A DC_2A-2A-14A_n260G DC_2A-2A-14A_n260H DC_2A-2A-14A_n260I DC_2A-2A-14A_n260J DC_2A-2A-14A_n260K DC_2A-2A-14A_n260L DC_2A-2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M | CA_2A-2A-14A | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |

- 5.2.33.3	∆TIB and ∆RIB values

Table 5.2.33.3-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n260, DC_2-2-14_n260 | 2 | 0.3 |
|  | 14 | 0.3 |
|  | n260 | 0 |

Table 5.2.33.3-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-14_n260, DC_2-2-14_n260 | 2 | 0 |
|  | 14 | 0 |
|  | n260 | 0 |

## 5.2.34	DC_2-46_n258

- 5.2.34.1	Operating bands for DC

Table 5.2.34.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-46_n258 | CA_2-46 | n258 | No |

- 5.2.34.2	Configurations for DC

Table 5.2.34.2-1: Inter-band EN-DC contiguous configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A_n258A DC_2A-46C_n258A DC_2A-46D_n258A | DC_2A_n258A |

Table 5.2.34.2-2: Inter-band EN-DC non-contiguous configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A_n258(2A) DC_2A-46A_n258(3A) DC_2A-46A_n258(4A) DC_2A-46A_n258(5A) DC_2A-46C_n258(2A) DC_2A-46C_n258(3A) DC_2A-46C_n258(4A) DC_2A-46C_n258(5A) DC_2A-46D_n258(2A) DC_2A-46D_n258(3A) DC_2A-46D_n258(4A) DC_2A-46D_n258(5A) | DC_2A_n258A |

- 5.2.34.3	∆TIB and ∆RIB values

Table 5.2.34.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n258 | 2 | 0 |
|  | 46 | 0 |
|  | n258 | 0 |

Table 5.2.34.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n258 | 2 | 0 |
|  | 46 | 0 |
|  | n258 | 0 |

## 5.2.35	DC_46-66_n258

- 5.2.35.1	Operating bands for DC

Table 5.2.35.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n258 | CA_46-66 | n258 | No |

- 5.2.35.2	Configurations for DC

Table 5.2.35.2-1: Inter-band EN-DC contiguous configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_46A-66A_n258A DC_46C-66A_n258A DC_46D-66A_n258A | DC_66A_n258A |

Table 5.2.35.2-2: Inter-band EN-DC non-contiguous configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_46A-66A_n258(2A) DC_46A-66A_n258(3A) DC_46A-66A_n258(4A) DC_46A-66A_n258(5A) DC_46C-66A_n258(2A) DC_46C-66A_n258(3A) DC_46C-66A_n258(4A) DC_46C-66A_n258(5A) DC_46D-66A_n258(2A) DC_46D-66A_n258(3A) DC_46D-66A_n258(4A) DC_46D-66A_n258(5A) | DC_66A_n258A |

- 5.2.35.3	∆TIB and ∆RIB values

Table 5.2.35.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n258 | 46 | 0 |
|  | 66 | 0 |
|  | n258 | 0 |

Table 5.2.35.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n258 | 46 | 0 |
|  | 66 | 0 |
|  | n258 | 0 |

## 5.2.36	DC_13-66_n260, DC_13-66-66_n260

- 5.2.36.1	Operating bands for DC

Table 5.2.36.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-66_n2601 | CA_13-66 | n260 | No |
| DC_13-66-66_n2601 | CA_13-66-66 | n260 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.36.2	Configurations for DC

Table 5.2.36.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A-66A_n260A DC_13A-66A_n260G DC_13A-66A_n260H DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260(2A) DC_13A-66A_n260(3A) DC_13A-66A_n260(4A) DC_13A-66A_n260(5A) DC_13A-66A_n260(6A) DC_13A-66A_n260(2G) DC_13A-66A_n260(2H) DC_13A-66A_n260(A-G) DC_13A-66A_n260(A-H) DC_13A-66A_n260(A-2G) DC_13A-66A_n260(2A-G) DC_13A-66A_n260(2A-2G) DC_13A-66A_n260(3A-G) DC_13A-66A_n260(G-H) DC_13A-66A-66A_n260A DC_13A-66A-66A_n260G DC_13A-66A-66A_n260H DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M DC_13A-66A-66A_n260(2A) DC_13A-66A-66A_n260(3A) DC_13A-66A-66A_n260(4A) DC_13A-66A-66A_n260(5A) DC_13A-66A-66A_n260(6A) DC_13A-66A-66A_n260(2G) DC_13A-66A-66A_n260(2H) DC_13A-66A-66A_n260(A-G) DC_13A-66A-66A_n260(A-H) DC_13A-66A-66A_n260(A-2G) DC_13A-66A-66A_n260(2A-G) DC_13A-66A-66A_n260(2A-2G) DC_13A-66A-66A_n260(3A-G) DC_13A-66A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260I DC_66A_n260I |

- 5.2.36.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_13-66_n260 have been given in TR37.863-02-01 as the below tables.

Table 5.2.36.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n260 DC_13-66-66_n260 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

Table 5.2.36.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n260 DC_13-66-66_n260 | 13 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

## 5.2.37	DC_13-66_n261 and DC_13-66-66_n261

- 5.2.37.1	Operating bands for DC

Table 5.2.37.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-66_n2611 | CA_13-66 | n261 | No |
| DC_13-66-66_n2611 | CA_13-66 | n261 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.37.2	Configurations for DC

Table 5.2.37.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A-66A_n261A DC_13A-66A_n261G DC_13A-66A_n261H DC_13A-66A_n261I DC_13A-66A_n261J DC_13A-66A_n261K DC_13A-66A_n261L DC_13A-66A_n261M | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261(2A) DC_13A-66A_n261(3A) DC_13A-66A_n261(4A) DC_13A-66A_n261(2G) DC_13A-66A_n261(2H) DC_13A-66A_n261(A-G) DC_13A-66A_n261(A-H) DC_13A-66A_n261(A-I) DC_13A-66A_n261(A-J) DC_13A-66A_n261(A-K) DC_13A-66A_n261(A-2G) DC_13A-66A_n261(A-G-H) DC_13A-66A_n261(A-G-I) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(2A-I) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(G-H) DC_13A-66A_n261(G-I) DC_13A-66A_n261(G-J)DC_13A-66A_n261(H-I) DC_13A-66A-66A_n261A DC_13A-66A-66A_n261G DC_13A-66A-66A_n261H DC_13A-66A-66A_n261I DC_13A-66A-66A_n261J DC_13A-66A-66A_n261K DC_13A-66A-66A_n261L DC_13A-66A-66A_n261M DC_13A-66A-66A_n261(2A) DC_13A-66A-66A_n261(3A) DC_13A-66A-66A_n261(4A) DC_13A-66A-66A_n261(2G) DC_13A-66A-66A_n261(2H) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(A-I) DC_13A-66A-66A_n261(A-J) DC_13A-66A-66A_n261(A-K) DC_13A-66A-66A_n261(A-2G) DC_13A-66A-66A_n261(A-G-H) DC_13A-66A-66A_n261(A-G-I) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(2A-I) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(G-I) DC_13A-66A-66A_n261(G-J) DC_13A-66A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261(A-G) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(2G) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(2G) | DC_13A_n261G DC_66A_n261G |
| DC_13A-66A_n261(A-H) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(G-H) DC_13A-66A_n261(2H) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(2H) | DC_13A_n261H DC_66A_n261H |
| DC_13A-66A_n261I DC_13A-66A_n261J DC_13A-66A_n261K DC_13A-66A_n261L DC_13A-66A_n261M DC_13A-66A-66A_n261I DC_13A-66A-66A_n261J DC_13A-66A-66A_n261K DC_13A-66A-66A_n261L DC_13A-66A-66A_n261M DC_13A-66A_n261(A-J) DC_13A-66A_n261(A-K) DC_13A-66A_n261(2A-I) DC_13A-66A_n261(G-I) DC_13A-66A_n261(G-J) DC_13A-66A_n261(H-I) DC_13A-66A-66A_n261(A-J) DC_13A-66A-66A_n261(A-K) DC_13A-66A-66A_n261(2A-I) DC_13A-66A-66A_n261(G-I) DC_13A-66A-66A_n261(G-J) DC_13A-66A-66A_n261(H-I) | DC_13A_n261I DC_66A_n261I |

- 5.2.37.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_13-66_n261 and DC_13-66-66_n261 are given as the below tables.

Table 5.2.37.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n261 DC_13-66-66_n261 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n261 | 0 |

Table 5.2.37.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n261 DC_13-66-66_n261 | 13 | 0 |
|  | 66 | 0 |
|  | n261 | 0 |

## 5.2.38	DC_2-66_n260 and DC_2-66-66_n260

- 5.2.38.1	Operating bands for DC

Table 5.2.38.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n2601 | CA_2-66 | n260 | No |
| DC_2-66-66_n2601 | CA_2-66-66 | n260 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.38.2	Configurations for DC

Table 5.2.38.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-66A_n260(5A) DC_2A-66A_n260(6A) DC_2A-66A_n260(2G) DC_2A-66A_n260(2H) DC_2A-66A_n260(A-G) DC_2A-66A_n260(A-H) DC_2A-66A_n260(A-2G) DC_2A-66A_n260(2A-G) DC_2A-66A_n260(2A-2G) DC_2A-66A_n260(3A-G) DC_2A-66A_n260(G-H) | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260I DC_66A_n260I |

- 5.2.38.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2-66_n260 have been given in TR37.863-02-01 as the below tables.

Table 5.2.38.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n260 DC_2-66-66_n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.2.38.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n260 DC_2-66-66_n260 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

## 5.2.39	DC_2-66_n261 and DC_2-66-66_n261

- 5.2.39.1	Operating bands for DC

Table 5.2.39.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n2611 | CA_2-66 | n261 | No |
| DC_2-66-66_n2611 | CA_2-66-66 | n261 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.39.2	Configurations for DC

Table 5.2.39.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-66A_n261G DC_2A-66A_n261H DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261(3A) DC_2A-66A_n261(4A) DC_2A-66A_n261(2G) DC_2A-66A_n261(2H) DC_2A-66A_n261(A-G) DC_2A-66A_n261(A-H) DC_2A-66A_n261(A-I) DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(A-2G) DC_2A-66A_n261(A-G-H) DC_2A-66A_n261(A-G-I) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(G-H) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) DC_2A-66A-66A_n261A DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(2H) DC_2A-66A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261(A-G) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(2G) DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) | DC_2A_n261G DC_66A_n261G |
| DC_2A-66A_n261(A-H) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(G-H) DC_2A-66A_n261(2H) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(2H) | DC_2A_n261H DC_66A_n261H |
| DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(H-I) | DC_2A_n261I DC_66A_n261I |

- 5.2.39.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2-66_n261 are given as the below tables.

Table 5.2.39.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n261 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n261 | 0 |

Table 5.2.39.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n261 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n261 | 0 |

## 5.2.40	DC_2-13_n260

- 5.2.40.1	Operating bands for DC

Table 5.2.40.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-13_n2601 | CA_2-13 | n260 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.40.2	Configurations for DC

Table 5.2.40.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-13A_n260G DC_2A-13A_n260H DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n260(2A) DC_2A-13A_n260(3A) DC_2A-13A_n260(4A) DC_2A-13A_n260(5A) DC_2A-13A_n260(6A) DC_2A-13A_n260(2G) DC_2A-13A_n260(2H) DC_2A-13A_n260(A-G) DC_2A-13A_n260(A-H) DC_2A-13A_n260(A-2G) DC_2A-13A_n260(2A-G) DC_2A-13A_n260(2A-2G) DC_2A-13A_n260(3A-G) DC_2A-13A_n260(G-H) | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260I DC_13A_n260I |

- 5.2.40.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2-13_n260 have been given in TR37.863-02-01 as the below tables.

Table 5.2.40.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n260 | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n260 | 0 |

Table 5.2.40.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n260 | 2 | 0 |
|  | 13 | 0 |
|  | n260 | 0 |

## 5.2.41	DC_2-13_n261

- 5.2.41.1	Operating bands for DC

Table 5.2.41.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-13_n2611 | CA_2-13 | n261 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.41.2	Configurations for DC

Table 5.2.41.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-13A_n261A DC_2A-13A_n261G DC_2A-13A_n261H DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M | DC_2A_n261A DC_13A_n261A |
| DC_2A-13A_n261(2A) DC_2A-13A_n261(3A) DC_2A-13A_n261(4A) DC_2A-13A_n261(2G) DC_2A-13A_n261(2H) DC_2A-13A_n261(A-G) DC_2A-13A_n261(A-H) DC_2A-13A_n261(A-I) DC_2A-13A_n261(A-J) DC_2A-13A_n261(A-K) DC_2A-13A_n261(A-2G) DC_2A-13A_n261(A-G-H) DC_2A-13A_n261(A-G-I) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(2A-I) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(G-H) DC_2A-13A_n261(G-I) DC_2A-13A_n261(G-J) DC_2A-13A_n261(H-I) | DC_2A_n261A DC_13A_n261A |
| DC_2A-13A_n261(A-G) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(2G) | DC_2A_n261G DC_13A_n261G |
| DC_2A-13A_n261(A-H) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(G-H) DC_2A-13A_n261(2H) | DC_2A_n261H DC_13A_n261H |
| DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M DC_2A-13A_n261(A-J) DC_2A-13A_n261(A-K) DC_2A-13A_n261(2A-I) DC_2A-13A_n261(H-I) DC_2A-13A_n261(G-I) DC_2A-13A_n261(G-J) | DC_2A_n261I DC_13A_n261I |

- 5.2.41.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2-13_n261 are given as the below tables.

Table 5.2.41.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n261 | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n261 | 0 |

Table 5.2.41.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n261 | 2 | 0 |
|  | 13 | 0 |
|  | n261 | 0 |

## 5.2.42	DC_2-5_n260

- 5.2.42.1	Operating bands for DC

Table 5.2.42.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n2601 | CA_2-5 | n260 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.42.2	Configurations for DC

Table 5.2.42.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M | DC_2A_n260I DC_5A_n260I |

- 5.2.42.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2-5_n260 are given as the below tables.

Table 5.2.42.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n260 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n260 | 0 |

Table 5.2.42.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n260 | 2 | 0 |
|  | 5 | 0 |
|  | n260 | 0 |

## 5.2.43	DC_2-5_n261

- 5.2.43.1	Operating bands for DC

Table 5.2.43.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5_n2611 | CA_2-5 | n261 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.43.2	Configurations for DC

Table 5.2.43.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A_n261A DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M | DC_2A_n261A DC_5A_n261A |
| DC_2A-5A_n261(A-G) DC_2A-5A_n261(A-H) DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(2A-H) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(G-H) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(2G) DC_2A-5A_n261(2H) DC_2A-5A_n261(H-I) | DC_2A_n261A DC_5A_n261A |
| DC_2A-5A_n261(A-G) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(2G) | DC_2A_n261G DC_5A_n261G |
| DC_2A-5A_n261(2A-H) DC_2A-5A_n261(G-H) DC_2A-5A_n261(2H) | DC_2A_n261H DC_5A_n261H |
| DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(H-I) | DC_2A_n261I DC_5A_n261I |

- 5.2.43.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2-5_n261 are given as the below tables.

Table 5.2.43.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n261 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n261 | 0 |

Table 5.2.43.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n261 | 2 | 0 |
|  | 5 | 0 |
|  | n261 | 0 |

## 5.2.44	DC_5-66_n260 and DC_5-66-66_n260

- 5.2.44.1	Operating bands for DC

Table 5.2.44.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n2601 | CA_5-66 | n260 | No |
| DC_5-66-66_n2601 | CA_5-66-66 | n260 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.44.2	Configurations for DC

Table 5.2.44.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260I DC_66A_n260I |

- 5.2.44.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_5-66_n260 and DC_5-66-66_n260 are given as the below tables.

Table 5.2.44.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n260 DC_5-66-66_n260 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

Table 5.2.44.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n260 DC_5-66-66_n260 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n260 | 0 |

## 5.2.45	DC_5-66_n261and DC_5-66-66_n261

- 5.2.45.1	Operating bands for DC

Table 5.2.45.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-66_n2611 | CA_5-66 | n261 | No |
| DC_5-66-66_n2611 | CA_5-66-66 | n261 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

- 5.2.45.2	Configurations for DC

Table 5.2.45.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-66A_n261A DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M | DC_5A_n261A DC_66A_n261A |
| DC_5A-66A_n261(2G) DC_5A-66A_n261(2H) DC_5A-66A_n261(A-G) DC_5A-66A_n261(A-H) DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(G-H) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) DC_5A-66A-66A_n261A DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(2H) DC_5A-66A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A |
| DC_5A-66A_n261(A-G) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(2G) DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) | DC_5A_n261G DC_66A_n261G |
| DC_5A-66A_n261(A-H) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(G-H) DC_5A-66A_n261(2H) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(2H) | DC_5A_n261H DC_66A_n261H |
| DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(H-I) | DC_5A_n261I DC_66A_n261I |

- 5.2.45.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_5-66_n261 and DC_5-66-66_n261 are given as the below tables.

Table 5.2.45.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n261 DC_5-66-66_n261 | 5 | 0.3 |
|  | 66 | 0.3 |
|  | n261 | 0 |

Table 5.2.45.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n261 DC_5-66-66_n261 | 5 | 0 |
|  | 66 | 0 |
|  | n261 | 0 |

## 5.2.46	DC_11-18_n257

- 5.2.46.1	Operating bands for DC

Table 5.2.46.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_11-18_n257 | CA_11-18 | n257 | No |

- 5.2.46.2	Configurations for DC

Table 5.2.46.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_11A-18A_n257A DC_11A-18A_n257G DC_11A-18A_n257H DC_11A-18A_n257I | DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | CA_11A-18A | n257 CA_n257G CA_n257H CA_n257I |

- 5.2.46.3	∆TIB and ∆RIB values

Table 5.2.46.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11-18_n257 | 11 | 0.3 |
|  | 18 | 0.3 |
|  | n257 | 0 |

Table 5.2.46.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_11-18_n257 | 11 | 0 |
|  | 18 | 0 |
|  | n257 | 0 |

## 5.2.47	DC_18-41_n257

- 5.2.47.1	Operating bands for DC

Table 5.2.47.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n257 | CA_18-41 | n257 | No |

- 5.2.47.2	Configurations for DC

Table 5.2.47.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n257A DC_18A-41A_n257G DC_18A-41A_n257H DC_18A-41A_n257I DC_18A-41C_n257A DC_18A-41C_n257G DC_18A-41C_n257H DC_18A-41C_n257I | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | CA_18A-41A CA_18A-41C | n257 CA_n257G CA_n257H CA_n257I |

- 5.2.47.3	∆TIB and ∆RIB values

Table 5.2.47.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n257 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n257 | 0 |

Table 5.2.47.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n257 | 18 | 0 |
|  | 41 | 0 |
|  | n257 | 0 |

- 5.2.48	DC_2-46_n260

- 5.2.48.1	Operating bands for EN-DC

Table 5.2.48.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46_n260 | CA_2-46 | n260 |  |

- 5.2.48.2	Configuration for DC

Table 5.2.48.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A_n260A DC_2A-46C_n260A DC_2A-46D_n260A DC_2A-46E_n260A DC_2A-46A_n260G DC_2A-46C_n260G DC_2A-46D_n260G DC_2A-46E_n260G DC_2A-46A_n260H DC_2A-46C_n260H DC_2A-46D_n260H DC_2A-46E_n260H DC_2A-46A_n260I DC_2A-46C_n260I DC_2A-46D_n260I DC_2A-46E_n260I DC_2A-46A_n260J DC_2A-46C_n260J DC_2A-46D_n260J DC_2A-46E_n260J DC_2A-46A_n260K DC_2A-46C_n260K DC_2A-46D_n260K DC_2A-46E_n260K DC_2A-46A_n260L DC_2A-46C_n260L DC_2A-46D_n260L DC_2A-46E_n260L DC_2A-46A_n260M DC_2A-46C_n260M DC_2A-46D_n260M DC_2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M | CA_2A-46A CA_2A-46C CA_2A-46D CA_2A-46E | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |
| DC_2A-2A-46A_n260A DC_2A-2A-46C_n260A DC_2A-2A-46D_n260A DC_2A-2A-46E_n260A DC_2A-2A-46A_n260G DC_2A-2A-46C_n260G DC_2A-2A-46D_n260G DC_2A-2A-46E_n260G DC_2A-2A-46A_n260H DC_2A-2A-46C_n260H DC_2A-2A-46D_n260H DC_2A-2A-46E_n260H DC_2A-2A-46A_n260I DC_2A-2A-46C_n260I DC_2A-2A-46D_n260I DC_2A-2A-46E_n260I DC_2A-2A-46A_n260J DC_2A-2A-46C_n260J DC_2A-2A-46D_n260J DC_2A-2A-46E_n260J DC_2A-2A-46A_n260K DC_2A-2A-46C_n260K DC_2A-2A-46D_n260K DC_2A-2A-46E_n260K DC_2A-2A-46A_n260L DC_2A-2A-46C_n260L DC_2A-2A-46D_n260L DC_2A-2A-46E_n260L DC_2A-2A-46A_n260M DC_2A-2A-46C_n260M DC_2A-2A-46D_n260M DC_2A-2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M | CA_2A-2A-46A CA_2A-2A-46C CA_2A-2A-46D CA_2A-2A-46E | n260A CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M |

- 5.2.48.3		∆TIB and ∆RIB values

For DC_2-46_n260 the TIB,c and RIB,c values are given in the tables below.

Table 5.2.48.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n260 DC_2-2-46_n260 | 2 | 0 |
|  | n260 | 0 |

Table 5.2.48.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n260 DC_2-2-46_n260 | 2 | 0 |

- 5.2.48.4	REFSENS requirements

There is no need for additional REFSENS requirements.

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |  |
| 2018-8 | 3GPP RAN4#88 | R4-1810566 |  |  |  | TR skeleton | 0.0.1 |  |
| 2018-10 | 3GPP RAN4#88bis | R4-1812692 |  |  |  | Implemented TP´s from RAN4 #88: R4-1810126 TP for TR 37.716-21-11: DC_3-18_n77 R4-1810127 TP for TR 37.716-21-11: DC_3-18_n78 R4-1810129 TP for TR 37.716-21-11: DC_3-18_n79 R4-1810130 TP for TR 37.716-21-11: DC_3-18_n257 R4-1810131 TP for TR 37.716-21-11: DC_3-41_n77 R4-1810132 TP for TR 37.716-21-11: DC_3-41_n79 R4-1810265 TP on MSD analysis for remaining LTE(2DL/1UL) + NR(1DL/1UL) DC UE R4-1810332 TP for TR 37.716-21-11: DC band combination of Band 1, 5 and n79 R4-1810333 TP for TR 37.716-21-11: DC band combination of Band 3, 5 and n79 R4-1810334 TP for TR 37.716-21-11: DC band combination of Band 5, 41 and n79 R4-1810399 TP to 37.716-21-11, DC_1A-7C_n78A R4-1810460 TP on TR 37.716-21-11 for EN-DC_3-8_n77 R4-1810462 TP on TR 37.716-21-11 for EN-DC_3-8_n79 R4-1810464 TP on TR 37.716-21-11 for EN-DC_3-8_n257 R4-1810466 TP on TR 37.716-21-11 for EN-DC_28-41_n77 R4-1810478 TP on TR 37.716-21-11 for EN-DC_28-41_n78 R4-1810480 TP on TR 37.716-21-11 for EN-DC_28-41_n79 R4-1810504 TP on TR 37.716-21-11 for EN-DC_28-41_n257 R4-1810804 TP for TR 37.716-21-11 Introduction of DC_2A-66A_n261A, DC_2A-66A_n261(2A), DC_2A-66A_n257A and DC_2A-66A_n257(2A) R4-1811434 TP to 37.716-21-11, DC_2A-66A_n260(2A) (3A) (4A) | 0.1.0 |  |
| 2018-11 | 3GPP RAN4#89 | R4-1816167 |  |  |  | Implemented TP´s from RAN4 #88bis: R4-1812243 TP on MSD analysis for LTE(2DL/1UL) + NR(1DL/1UL) DC UE R4-1812422 TP for TR 37.716-21-11 : EN-DC_1-8_n77 R4-1812423 TP for TR 37.716-21-11 : EN-DC_1-8_n79 R4-1812599 TP for TR 37.716-21-11 Introduction of DC_46-66_n261 R4-1812628 TP for TR 37.716-21-11: UE requirements for DC_3-3-7_n78, DC_3-3-7-7_n78 R4-1812725 TP for TR 37.716-21-11: DC_18-42_n77 R4-1812726 TP for TR 37.716-21-11: DC_18-42_n78 R4-1812727 TP for TR 37.716-21-11: DC_18-42_n79 R4-1812729 TP for TR 37.716-21-11: DC_18-42_n257 R4-1812730 TP for TR 37.716-21-11: DC_1-18_n257 R4-1812737 TP for TR 37.716-21-11: DC_3-18_n257 R4-1812738 TP for TR 37.716-21-11: DC_1-41_n257 R4-1812739 TP for TR 37.716-21-11: DC_3-41_n257 R4-1812741 TP for TR 37.716-21-11: DC_41-42_n257 R4-1812790 TP for correction of table 5.1.9.1 & 5.1.9.2 in TR 37.716-21-11 R4-1812795 TP for TR 37.716-21-11 for DC_46-66_n260 R4-1812804 TP for TR 37.716-21-11 for 2A-29A_n260A, 29A-30A_n260A R4-1813158 TP for TR 37.716-21-11: DC_8-20_n78 R4-1813159 TP for TR 37.716-21-11: DC_20-38_n78 R4-1813770 TP for TR 37.716-21-11 DC_LTE 2bands_n257 R4-1813775 TP to TR 37.716-21-11: Introduction of DC_66A-(n)71AA and DC_2A-(n)71AA R4-1813793 TP for TR 37.716-21-11: RefSens requirement for DC band combination of Band 5, 41 and n79 | 0.2.0 |  |
| 2019-02 | 3GPP RAN4#90 | R4-1900898 |  |  |  | Implemented TP´s from RAN4 #89: R4-1814909 TP for TR 37.716-21-11 UE requirements for DC_7-8_n77, DC_7-8_n78 R4-1814912 TP for TR 37.716-21-11: DC_18-42_n79 R4-1814914 TP for TR 37.716-21-11: DC_3-41_n79 R4-1814918 TP for TR 37.716-21-11: DC_3-18_n78 R4-1814919 TP for TR 37.716-21-11: DC_3-18_n77 R4-1815043 MSD analysis for DC band combination of Band 20, 38 and n78 R4-1815044 MSD analysis for DC band combination of Band 8, 20 and n78 R4-1815260 TP for TR 37.716-21-11: DC_2-7_n78 R4-1815346 TP for TR 37.716-21-11: DC_7-66_n78 R4-1815812 TP for 37.716-21-11 for 2A-29A_n260, 29A-30A_n260A R4-1816051 TP for TR 37.716-21-11: DC_3C-8A_n78A R4-1816163 TP for TR 37.716-21-11: DC_3-41_n77 R4-1816169 TP for 37.716-21-11 for 3C-28A_n78 R4-1816177 TP for TR 37.716-21-11: RefSens requirement for DC band combination of Band 5, 41 and n79 | 0.3.0 |  |
| 2019-04 | 3GPP RAN4#90bis | R4-1903644 |  |  |  | Implemented TP´s from RAN4 #90: R4-1900250 TP on MSD analysis for new LTE(2DL/1UL) + NR(1DL/1UL) DC UE : Softbank R4-1900725 TP for TR 37.716-21-11 UE MSD requirements for DC_7-8_n77, DC_7-8_n78 R4-1900901 TP for TR 37.716-21-11: DC_5A-7A_n71A R4-1900902 TP for TR 37.716-21-11: DC_2A-7A_n71A R4-1900903 TP for TR 37.716-21-11: DC_7A-66A-66A_n78A R4-1900904 TP for TR 37.716-21-11: DC_2-66_n78 R4-1901431 TP for TR 37.716-21-11 to include DC_1A-3A_n7A, DC_1A-3C_n7A R4-1902127 TP for 37.716-21-11 for 3A-3A-28A_n78 R4-1902206 TP for TR 37.716-21-11 for 3A+8A+n257(D-M) R4-1902209 TP to TR 37.716-21-11: DC_3A-28A_n41A | 0.4.0 |  |
| 2019-05 | 3GPP RAN4#91 | R4-1905557 |  |  |  | Implemented TP´s from RAN4 #90bis: R4-1902971 TP for TR 37.716-21-11: EN-DC_1-11_n257 R4-1902972 TP for TR 37.716-21-11: EN-DC_8-11_n257 R4-1903301 TP for TR 37.716-21-11: UE requirements for DC_3-3-7_n257, DC_3-3-7-7_n257 R4-1903652 TP for TR 37.716-21-11: DC_3C-8A_n257 (A, D-M) R4-1902996 TP for TR 37.716-21-11: EN-DC_1-11_n78 R4-1902997 TP for TR 37.716-21-11: EN-DC_8-11_n77 R4-1902998 TP for TR 37.716-21-11: EN-DC_8-11_n78 R4-1903004 TP for TR 37.716-21-11 Introduction of DC_2A-30A_n5A and DC_2A-2A-30A_n5A R4-1903078 TP for TR 37.716-21-11 DC_30A-66A-66A_n5A R4-1903300 TP for TR 37.716-21-11: UE requirements for DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1 R4-1903647 TP for TR 37.716-21-11: MSD for DC_2-66_n78 R4-1903650 TP for TR 37.716-21-11: DC_7A-7A-66A-66A_n78A, DC_7C-66A-66A_n78A R4-1904443 TP for TR 37.716-21-11 to include DC_1A-3A_n5A, 1A-3C_n5A R4-1904444 TP for TR 37.716-21-11 to include DC_1A-7A_n5A, 1A-7C_n5A R4-1904445 TP for TR 37.716-21-11 to include DC_1A-28A_n5A R4-1904446 TP for TR 37.716-21-11 to include DC_3A-7A_n5A, 3C-7A_n5A, 3A-7C_n5A, 3C-7C_n5A R4-1904447 TP for TR 37.716-21-11 to include DC_3A-28A_n5A, 3C-28A_n5A R4-1904448 TP for TR 37.716-21-11 to include DC_7A-28A_n5A, 7C-28A_n5A R4-1904905 TP for TR 37.716-21-11: EN-DC_1-11_n77 R4-1904909 TP for TR 37.716-21-11 to include DC_3A-20A_n1A R4-1904911 TP for TR 37.716-21-11 to include DC_1A-20A_n3A R4-1903012 TP on MSD analysis results for LTE(2DL/1UL) + NR(1DL/1UL) DC UE R4-1903077 TP for TR 37.716-21-11 DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A and DC_2-66-66-66_n5 | 0.5.0 |  |
| 2019-08 | 3GPP RAN4#92 | R4-1909226 |  |  |  | Implemented TP´s from RAN4 #91: R4-1906352 TP for TR 37.716-21-11: UE requirements for DC_3-7_n77 R4-1906425 TP for TR 37.716-21-11 MSD table for DC_30A-66A-66A_n5A R4-1906426 TP for TR 37.716-21-11 MSD value for DC_2A-66A_n5A R4-1906747 TP for TR 37.716-21-11 to include MSD for DC_3A-28A_n5A, 3C-28A_n5A R4-1907397 TP for TR 37.716-21-11 to include MSD for DC_1A-28A_n5A R4-1907398 TP for TR 37.716-21-11 to include MSD for DC_7A-28A_n5A, 7C-28A_n5A R4-1907401 TP for TR37.716-21-11 for DC_2A-13A_n66A | 0.6.0 |  |
| 2019-10 | 3GPP RAN4#92bis | R4-1910871 |  |  |  | Implemented TP´s from RAN4 #92: R4-1908100 TP for TR 37.716-21-11: DC_2-7_n66 and DC_2-7-7_n66 R4-1908101 TP for TR 37.716-21-11: DC_2-66_n66 R4-1908102 TP for TR 37.716-21-11: DC_7-66_n66 and DC_7-7-66_n66 R4-1908290 TP to TR 37.716-21-11: DC_2-46_n261 R4-1908298 TP for TR37.716-21-11: UE MSD requirements for DC_3-7_n77 R4-1908583 TP for TR 37.716-21-11: UE requirements for DC_3-8_n1, DC_7-8_n1 R4-1908601 TP for TR 37.716-21-11: EN-DC_1-8_n3 R4-1908602 TP for TR 37.716-21-11: EN-DC_8-42_n77 R4-1908938 TP for TR 37.716-21-11: DC_3-40_n1 R4-1908939 TP for TR 37.716-21-11: DC_7-40_n1 R4-1908978 TP for TR 37.716-21-11 Introduction of DC_2A-5A_n66A_BCS0 R4-1908979 TP for TR 37.716-21-11 Introduction of DC_2A-66A-n66A_BCS0 R4-1908980 TP for TR 37.716-21-11 Introduction of DC_5A-66A-n5A_BCS0 R4-1908981 TP for TR 37.716-21-11 Introduction of DC_5A-66A_n66A_BCS0 R4-1908982 TP for TR 37.716-21-11 Introduction of DC_13A-66A-n66A_BCS0 R4-1909194 TP for TR 37.716-21-11: DC_2-7_n66 R4-1909196 TP for TR 37.716-21-11: DC_7-13_n66 R4-1909258 TP for TR 37.716-21-11 DC_46-48_n261 R4-1909803 TP for TR 37.716-21-11 to include DC_2A-66A_n41A R4-1909808 TP for TR 37.716-21-11 to include DC_46A-48A_n260A R4-1909812 TP for TR 37.716-21-11 to include 12A-30A_n2A R4-1909813 TP for TR 37.716-21-11 to include 12A-66A_n2A and 12A-66A-66A_n2A R4-1909814 TP for TR 37.716-21-11 to include 30A-66A_n2A and 30A-66A-66A_n2A R4-1909818 TP for TR 37.716-21-11 to include 14A-30A_n260M R4-1909819 TP for TR 37.716-21-11 to include 14A-66A_n260M and 14A-66A-66A_n260M R4-1909820 TP for TR 37.716-21-11 to include 2A-14A_n260M and 2A-2A-14A_n260M | 0.7.0 |  |
| 2019-11 | 3GPP RAN4#93 | R4-1914595 |  |  |  | Implemented TP´s from RAN4 #92bis: R4-1911149 TP for TR 37.716-21-11: EN-DC_1-8_n28 R4-1911267 TP for TR 37.716-21-11: UE requirements for DC_3-3-8_n78 R4-1911269 TP for TR 37.716-21-11: Adding support of DC_3-3-8_n1, DC_7-7-8_n1, DC_7-7-8_n78 R4-1911272 TP for TR 37.716-21-11: adding support of DC_3A-3A-7A_n257D to DC_3A-3A-7A_n257M and DC_3A-3A-7A-7A_n257D to DC_3A-3A-7A-7A_n257M R4-1911602 TP for 37.716-21-11 to introduce DC_2A-46_n41 R4-1911603 TP for 37.716-21-11 to introduce DC_2A-46_n71 R4-1911604 TP for 37.716-21-11 to introduce DC_46-66A_n41 R4-1911605 TP for 37.716-21-11 to introduce DC_46-66A_n71 R4-1912248 TP for TR 37.716-21-11 to include DC_1-3_n38 R4-1912252 TP for TR 37.716-21-11 to include DC_7-20_n3 R4-1912269 TP for TR 37.716-21-11 to include DC_2-46_n258 R4-1912270 TP for TR 37.716-21-11 to include DC_46A-66A_n258A R4-1912428 TP for TR 37.716-21-11: Introduction of EN-DC combinations with Band 25, Band 41 and n41 R4-1912583 TP for TR 37.716-21-11: EN-DC_1-8_n3 R4-1912587 TP for TR 37.716-21-11: DC_13-66_n260 R4-1912588 TP for TR 37.716-21-11: DC_13-66_n261 R4-1912589 TP for TR 37.716-21-11: DC_2-66_n260 R4-1912590 TP for TR 37.716-21-11: DC_2-66_n261 R4-1912591 TP for TR 37.716-21-11: DC_13-48_n2 R4-1912592 TP for TR 37.716-21-11: DC_13-48_n66 R4-1912593 TP for TR 37.716-21-11: DC_48-66_n5 R4-1912601 TP for TR 37.716-21-11: DC_2-13_n260 R4-1912602 TP for TR 37.716-21-11: DC_2-13_n261 R4-1912604 TP for TR 37.716-21-11 to include DC_1-7_n3 R4-1912605 TP for TR 37.716-21-11 to include DC_1-20_n3 R4-1912607 TP for TR 37.716-21-11 to include DC_3-20_n38 | 0.8.0 |  |
| 2020-02 | 3GPP RAN4#94 | R4-2000840 |  |  |  | - Implemented TP´s from RAN4 #93: R4-1913609 TP for TR37.716-21-11_ DC_1A-20A_n38A R4-1913610 TP for TR37.716-21-11_ DC_3A-20A_n38A R4-1914018 TP for TR 37.716-21-11 DC_1A-3A_n77(2A) R4-1914310 TP for TR 37.716-21-11: DC_2A-4A_n38A R4-1914311 TP for TR 37.716-21-11: DC_2A-4A_n41A R4-1914312 TP for TR 37.716-21-11: DC_1A-3A_n41A R4-1914313 TP for TR 37.716-21-11: DC_7A-20A_n1A R4-1914314 updated TP for TR 37.716-21-11: adding configuration DC_3C-20A_n1A R4-1914315 updated TP for TR 37.716-21-11: adding configuration DC_3A-7C_n1A, DC_3C-7A_n1A and DC_3C-7C_n1A R4-1914328 TP for 37.716-21-11 to introduce DC_2-12_n2 R4-1914702 TP for TR 37 716-21-11 to include DC_3-28_n7 R4-1914706 TP for TR 37 716-21-11 to include DC_1-7_n7 R4-1914707 TP for TR 37 716-21-11 to include DC_3-7_n7 R4-1914708 TP for TR 37 716-21-11 to include DC_7-28_n7 R4-1914713 TP for TR 37 716-21-11 to include DC_12-66_n66 R4-1915640 TP for TR 37 716-21-11 to include DC_1-28_n7 R4-1915690 TP for TR 37.716-21-11: MSD for EN-DC_1-8_n28 - Some editorial changes - Move DC_2-5_n66 to sub-clause 5.1.103 | 0.9.0 |  |
| 2020-04 | 3GPP RAN4#94bis | R4-2004501 |  |  |  | Implemented TP´s from RAN4 #94-e：R4-2000237 TP for TR 37.716-21-11: DC_2-2-13_n66 R4-2000238 TP for TR 37.716-21-11: DC_2-5_n2 and DC_2-5-5_n2 R4-2000239 TP for TR 37.716-21-11: DC_2-5_n5 and DC_2-2-5_n5 R4-2000240 TP for TR 37.716-21-11: DC_2-5_n66 and DC_2-5-5_n66 R4-2000241 TP for TR 37.716-21-11: DC_2-13_n2 R4-2000242 TP for TR 37.716-21-11: DC_2-13_n5 and DC_2-2-13_n5 R4-2000244 TP for TR 37.716-21-11: DC_5-5-66_n66 and DC_5-5-66-66_n66 R4-2000245 TP for TR 37.716-21-11: DC_5-13_n2 R4-2000247 TP for TR 37.716-21-11: DC_5-66_n66 and DC_5-66-66_n66 R4-2000248 TP for TR 37.716-21-11: DC_5-66-66_n5 R4-2000252 TP for TR 37.716-21-11: DC_13A-66A-66A_n66A R4-2000253 TP for TR 37.716-21-11: DC_2-5_n260 R4-2000254 TP for TR 37.716-21-11: DC_2-5_n261 R4-2000255 TP for TR 37.716-21-11: DC_2-13_n260 R4-2000256 TP for TR 37.716-21-11: DC_2-13_n261 R4-2000257 TP for TR 37.716-21-11: DC_2-66_n260 and DC_2-66-66_n260 R4-2000258 TP for TR 37.716-21-11: DC_2-66_n261 and DC_2-66-66_n261 R4-2000259 TP for TR 37.716-21-11: DC_5-66_n260 and DC_5-66-66_n260 R4-2000260 TP for TR 37.716-21-11: DC_5-66_n261 and DC_5-66-66_n261 R4-2000261 TP for TR 37.716-21-11: DC_13-66_n260 and DC_13-66-66_n260 R4-2000262 TP for TR 37.716-21-11: DC_13-66_n261 and DC_13-66-66_n261 R4-2000479 TP for TR37.716-21-11: DC_5A-66A_n78 R4-2000532 TP to TR 37.716-21-11: DC_2A-5A_n71A R4-2000533 TP to TR 37.716-21-11: DC_2A-48A_n71A R4-2000534 TP to TR 37.716-21-11: DC_5A-48A_n71A R4-2000535 TP to TR 37.716-21-11: DC_48A_(n)12AA R4-2000536 TP to TR 37.716-21-11: DC_66A_(n)12AA R4-2000843 TP for TR 37.716-21-11: DC_2-66_n38 R4-2000844 TP for TR 37.716-21-11: DC_3C-8A_n1A R4-2000874 TP for TR 37.716-21-11: DC_1A-18A_n3A R4-2000876 TP for TR 37.716-21-11 DC_1A-28A_n3A R4-2001045 TP to TR 37.716-21-11: CA_3-20_n7 R4-2001049 TP for TR 37.716-21-11: DC_1A-3A_n8A R4-2001050 TP for TR 37.716-21-11: DC_1A-7A_n8A R4-2001051 TP for TR 37.716-21-11: DC_1A-20A_n8A R4-2001052 TP for TR 37.716-21-11: DC_3A-7A_n8A R4-2001053 TP for TR 37.716-21-11: DC_3A-20A_n8A R4-2001054 TP for TR 37.716-21-11: DC_7A-20A_n8A R4-2001055 updated TP for TR 37.716-21-11: add UL DC_1_n38 for DC_1-20_n38 R4-2001056 TP for TR 37.716-21-11: DC_1A-41A_n41A\DC_1A-41C_n41A R4-2001057 TP for TR 37.716-21-11: DC_1A-(n)41AA\DC_1A-(n)41CA R4-2001058 Updated TP for TR 37.716-21-11: DC_7C-20A_n1A R4-2001299 TP for 37.716-21-11 to introduce DC_2-66_n25 R4-2001300 TP for 37.716-21-11 to introduce DC_12-66_n25 R4-2001532 TP for TR 37.716-21-11 to include DC_2-7_n38 R4-2001533 TP for TR 37.716-21-11 to include DC_2-71_n38 R4-2001534 TP for TR 37.716-21-11 to include DC_7-66_n38 R4-2001535 TP for TR 37.716-21-11 to include DC_66-71_n38 R4-2001536 TP for TR 37.716-21-11 to include DC_2-66_n38 R4-2001537 TP for TR 37.716-21-11 to include DC_7-66_n71 R4-2001538 TP for TR 37.716-21-11 to include DC_2-71_n66 R4-2001539 TP for TR 37.716-21-11 to include DC_66-71_n66 R4-2001540 TP for TR 37.716-21-11 to include DC_2-71_n78 R4-2001541 TP for TR 37.716-21-11 to include DC_66-71_n78 R4-2001974 TP for TR 37 716-21-11 to include DC_2A-48A_n12A R4-2001975 TP for TR 37 716-21-11 to include DC_2A-66A_n12A R4-2001976 TP for TR 37 716-21-11 to include DC_5A-66A_n71A R4-2001979 TP for TR 37 716-21-11 to include DC_48A-66A_n12A R4-2001980 TP for TR 37 716-21-11 to include DC_48A-66A_n71A R4-2002007 TP for TR 37.716-21-11 to include DC_7A-28A_n3A, DC_7C-28A_n3A R4-2002012 TP for TR 37 716-21-11 to include DC_2-46_n66 R4-2002576 TP for TR 37.716-21-11: DC_2-66_n48 and DC_2-66-66_n48 R4-2002577 TP for TR 37.716-21-11: DC_5-66_n2, DC_5-5-66_n2, DC_5-66-66_n2 and DC_5-5-66-66_n2 R4-2002578 TP for TR 37.716-21-11: DC_13-46_n5 R4-2002579 TP for TR 37.716-21-11: DC_13-66_n2 and DC_13-66-66_n2 R4-2002580 TP for TR 37.716-21-11: DC_13-66_n48 and DC_13-66-66_n48 R4-2002610 TP for 37.716-21-11 to introduce DC_46-66_n25 | 0.10.0 |  |
| 2020-06 | 3GPP RAN4#95-e | R4-2007337 |  |  |  | - Implemented TP´s from RAN4 #94-e-Bis： R4-2003123 TP for TR 37.716-21-11: DC_1-41_n28 R4-2003129 TP for TR 37.716-21-11: DC_11A-18A_n77A R4-2003130 TP for TR 37.716-21-11: DC_11A-18A_n78A R4-2003131 TP for TR 37.716-21-11: DC_18-41_n3 R4-2003167 TP for TR 37.716-21-11: DC_2-48_n66 R4-2003451 TP for TR 37.716-21-11: EN-DC_1-11_n3 R4-2003454 TP for TR 37.716-21-11: EN-DC_3-8_n28 R4-2003457 TP for TR 37.716-21-11: EN-DC_8-11_n3 R4-2003488 TP for TR 37.716-21-11: support of DC_3-3-7_n77, DC_3-7-7_n77, DC_3-3-7-7_n77 R4-2003581 TP to TR 37.716-21-11 DC_1A-20A_n41A R4-2003582 TP to TR 37.716-21-11 DC_3A-20A_n41A R4-2003583 TP to TR 37.716-21-11 DC_1A-3C_n41A R4-2003927 Updated TP for TR 37.716-21-11: add configuration DC_1A-(n)41DA R4-2003929 TP for TR 37.716-21-11: DC_7A-8A_n3A R4-2003930 TP for TR 37.716-21-11: DC_1A-(n)38AA R4-2003931 TP for TR 37.716-21-11: DC_20A-(n)38AA R4-2003932 TP for TR 37.716-21-11: DC_20A-(n)41AA\DC_20A-(n)41CA\DC_20A-(n)41DA R4-2003934 TP for TR 37.716-21-11: DC_1A-32A_n78A\DC_1A-32A_n78(2A) R4-2003935 TP for TR 37.716-21-11: DC_3A-32A_n78A\DC_3A-32A_n78(2A) R4-2003936 TP for TR 37.716-21-11: DC_20A-32A_n78A\DC_20A-32A_n78(2A) R4-2004496 TP for TR 37.716-21-11: Additional band combinations for DC_2-66_n78 R4-2004497 TP for TR 37.716-21-11: Additional band combinations for DC_2-7_n78 R4-2004498 TP for TR 37.716-21-11: Additional band combinations for DC_7-66_n78 R4-2004610 TP for TR 37.716-21-11 to include DC_1A-3A_n40A R4-2004611 TP for TR 37.716-21-11 to include DC_1A-28A_n40A R4-2004612 TP for TR 37.716-21-11 to include DC_3A-28A_n40A R4-2004636 TP for TR 37.716-21-11: DC_25A-41D_n41A and DC_25A-25A-41D_n41A corrections R4-2005004 TP to TR 37.716-21-11: Addition of 2A_(n)12AA R4-2005005 TP to TR 37.716-21-11: Addition of 5A_(n)12AA R4-2005006 TP to TR 37.716-21-11: Addition of 12A_(n)5AA R4-2005007 TP to TR 37.716-21-11: Addition of 48A_(n)5AA R4-2005131 TP for TR 37.716-21-11: DC_1-41_n3 R4-2005132 TP for TR 37.716-21-11: DC_3-41_n28 R4-2005133 TP for TR 37.716-21-11: DC_18-41_n77 R4-2005134 TP for TR 37.716-21-11: DC_18-41_n78 R4-2005143 TP for TR 37.716-21-11: DC_1A-3A_n71A\DC_1A-3A_n71B R4-2005146 TP for 37.716-21-11 to introduce DC_1-7_n40 R4-2005147 TP for 37.716-21-11 to introduce DC_3-7_n40 R4-2005148 TP for 37.716-21-11 to introduce DC_7-28_n40 - Implemented TP´s from RAN4 #95-e： R4-2006498 TP for TR 37.716-21-11: DC_2-29_n66 and DC_2-2-29_n66 R4-2006499 TP for TR 37.716-21-11: DC_29-66_n2 and DC_29-66-66_n2 R4-2006679 TP for TR 37.716-21-11: EN-DC_1-42_n28 R4-2006681 TP for TR 37.716-21-11: EN-DC_3-42_n28 R4-2008340 TP for TR 37.716-21-11: EN-DC_8-42_n28 R4-2006689 TP update for TR 37.716-21-11: DC_1-11_n3 R4-2006934 TP to TR 37.716-21-11 DC_3C-20A_n41A R4-2007614 TP for TR 37.716-21-11 to include 2-14_n66 R4-2007615 TP for TR 37.716-21-11 to include 2-14_n2 R4-2007616 TP for TR 37.716-21-11 to include 14-66_n2 R4-2007617 TP for TR 37.716-21-11 to include 46-66_n5 R4-2007618 TP for TR 37.716-21-11 to include 14-66_n66 R4-2008345 TP for TR 37.716-21-11: DC_3A-(n)41AA\DC_3A-(n)41CA\DC_3A-(n)41DA R4-2008346 TP for TR 37.716-21-11: DC_3A-41A_n41A\DC_3A-41C_n41A\DC_3A-41D_n41A R4-2006396 TP for TR 37.716-21-11 DC_11-18_n257 R4-2006397 TP for TR 37.716-21-11 DC_18-41_n257 R4-2007619 TP for TR 37.716-21-11 to include 2-46_n260 R4-2007620 TP for TR 37.716-21-11 to include 46-66_n260 R4-2008378 TP for TR 37.716-21-11 to add MSD for DC_2A-5A_n71A - Move DC_48-66_n71 to clause 5.1.185 |  |  |