3GPP TR 37.716-21-21 V1.0.0 (2020-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

E-UTRA-NR Dual Connectivity (EN-DC) of x bands (x=1, 2, 3, 4)  LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	38

1	Scope	39

2	References	51

3	Definitions, symbols and abbreviations	52

3.1	Definitions	52

3.2	Symbols	52

3.3	Abbreviations	52

4	Background	52

4.1	TR Maintenance	52

5	EN-DC band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part	53

5.1	UE RF architectures	53

5.2	General treatment of ∆TIB and ∆RIB values	53

5.3	Summary of self-interference analysis	53

6	EN-DC band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	155

6.1	DC_3A_n1A-n77A	155

6.1.1	Operating bands for DC	155

6.1.2	Channel bandwidths per operating band for DC	156

6.1.3	Co-existence studies	156

6.1.4	∆TIB and ∆RIB values	158

6.1.5	MSD	158

6.2	DC_1A_n78A-n257	159

6.2.1	Operating bands for DC	159

6.2.2	Channel bandwidths per operating band for DC	159

6.2.3	Co-existence studies	160

6.2.4	∆TIB and ∆RIB values	161

6.2.5	MSD	161

6.3	DC_3_n78-n257	161

6.3.1	Operating bands for DC	161

6.3.2	Channel bandwidths per operating band for DC	161

6.3.3	Co-existence studies	164

6.3.4	∆TIB and ∆RIB values	164

6.3.5	MSD	164

6.4	DC_5A_n78A-n257	164

6.4.1	Operating bands for DC	164

6.4.2	Channel bandwidths per operating band for DC	164

6.4.3	Co-existence studies	166

6.4.4	∆TIB and ∆RIB values	166

6.4.5	MSD	166

6.5	DC_7A_n78A-n257	166

6.5.1	Operating bands for DC	166

6.5.2	Channel bandwidths per operating band for DC	167

6.5.3	Co-existence studies	168

6.5.4	∆TIB and ∆RIB values	168

6.5.5	MSD	168

6.6	DC_7A-7A_n78A-n257	168

6.6.1	Operating bands for DC	168

6.6.2	Channel bandwidths per operating band for DC	168

6.6.3	Co-existence studies	170

6.6.4	∆TIB and ∆RIB values	170

6.6.5	MSD	170

6.7	DC_1A_n3A-n78A	170

6.7.1	Operating bands for DC	170

6.7.2	Channel bandwidths per operating band for DC	171

6.7.3	Co-existence studies	171

6.7.4	∆TIB and ∆RIB values	171

6.7.5	MSD	171

6.8	DC_3_n1-n78	172

6.8.1	Operating bands for DC	172

6.8.2	Channel bandwidths per operating band for DC	173

6.8.3	Co-existence studies	173

6.8.4	∆TIB and ∆RIB values	175

6.8.5	MSD	175

6.9	DC_3A_n7A-n78A	176

6.9.1	Operating bands for DC	176

6.9.2	Channel bandwidths per operating band for DC	176

6.9.3	Co-existence studies	176

6.9.4	∆TIB and ∆RIB values	177

6.9.5	MSD	177

6.10	DC_3C_n7A-n78A	178

6.10.1	Operating bands for DC	178

6.10.2	Channel bandwidths per operating band for DC	178

6.10.3	Co-existence studies	178

6.10.4	∆TIB and ∆RIB values	178

6.10.5	MSD	178

6.11	DC_13A_n5A-n260A	179

6.11.1	Operating bands for DC	179

6.11.2	Channel bandwidths per operating band for DC	179

6.11.3	Co-existence studies	179

6.11.4	∆TIB and ∆RIB values	179

6.11.5	MSD	180

6.12	DC_66A_n5A-n260A	180

6.12.1	Operating bands for DC	180

6.12.2	Channel bandwidths per operating band for DC	180

6.12.3	Co-existence studies	180

6.12.4	∆TIB and ∆RIB values	180

6.12.5	MSD	181

6.13	DC_3A_n20A-n78A	181

6.13.1	Operating bands for DC	181

6.13.2	Channel bandwidths per operating band for DC	181

6.13.3	Co-existence studies	181

6.13.4	∆TIB and ∆RIB values	181

6.13.5	MSD	182

6.14	DC_28A_n8A-n78A	182

6.14.1	Operating bands for DC	182

6.14.2	Channel bandwidths per operating band for DC	182

6.14.3	Co-existence studies	183

6.14.4	∆TIB and ∆RIB values	183

6.14.5	MSD	183

6.15	DC_28A_n8A-n258A	184

6.15.1	Operating bands for DC	184

6.15.2	Channel bandwidths per operating band for DC	184

6.15.3	Co-existence studies	184

6.15.4	∆TIB and ∆RIB values	185

6.15.5	MSD	185

6.16	DC_3C_n28A-n78A	185

6.16.1	Operating bands for DC	185

6.16.2	Channel bandwidths per operating band for DC	186

6.16.3	Co-existence studies	186

6.16.4	∆TIB and ∆RIB values	186

6.16.5	MSD	187

6.17	DC_7C_n28A-n78A	187

6.17.1	Operating bands for DC	187

6.17.2	Channel bandwidths per operating band for DC	188

6.17.3	Co-existence studies	188

6.17.4	∆TIB and ∆RIB values	188

6.17.5	MSD	189

6.18	DC_66_n71-n260	189

6.18.1	Operating bands for DC	189

6.18.2	Channel bandwidths per operating band for DC	189

6.18.3	Co-existence studies	190

6.18.4	∆TIB and ∆RIB values	190

6.18.5	MSD	190

6.19	DC_66A_n71A-n261	191

6.19.1	Operating bands for DC	191

6.19.2	Channel bandwidths per operating band for DC	191

6.19.3	Co-existence studies	191

6.19.4	∆TIB and ∆RIB values	191

6.19.5	MSD	192

6.20	Void	192

6.21	DC_1A_n7A-n78A	192

6.21.1	Operating bands for DC	192

6.21.2	Channel bandwidths per operating band for DC	193

6.21.3	Co-existence studies	193

6.21.4	∆TIB and ∆RIB values	193

6.21.5	MSD	193

6.22	DC_8_n77-n257	194

6.22.1	Operating bands for DC	194

6.22.2	Channel bandwidths per operating band for DC	195

6.22.3	Co-existence studies	195

6.22.4	∆TIB and ∆RIB values	195

6.22.5	MSD	196

6.23	DC_1A_n40A-n78A	196

6.23.1	Operating bands for DC	196

6.23.2	Channel bandwidths per operating band for DC	197

6.23.3	Co-existence studies	197

6.23.4	∆TIB and ∆RIB values	197

6.24	DC_3_n1-n79	198

6.24.1	Operating bands for DC	198

6.24.2	Channel bandwidths per operating band for DC	198

6.24.3	Co-existence studies	199

6.24.4	∆TIB and ∆RIB values	202

6.25	DC_3_n40-n78	203

6.25.1	Operating bands for DC	203

6.25.2	Channel bandwidths per operating band for DC	203

6.25.3	Co-existence studies	204

6.25.4	∆TIB and ∆RIB values	204

6.26	DC_66_n25-n41	205

6.26.1	Operating bands for DC	205

6.26.2	Channel bandwidths per operating band for DC	205

6.26.3	Co-existence studies	205

6.26.4	∆TIB and ∆RIB values	208

6.27	DC_3_n1-n257, DC_3-3_n1-n257	209

6.27.1	Operating bands for DC	209

6.27.2	Channel bandwidths per operating band for DC	209

6.27.3	Co-existence studies	210

6.27.4	∆TIB and ∆RIB values	213

6.27.5	MSD	213

6.28	DC_1_n77-n258	213

6.28.1	Operating bands for DC	213

6.28.2	Channel bandwidths per operating band for DC	214

6.28.3	Co-existence studies	214

6.28.4	∆TIB and ∆RIB values	214

6.28.5	MSD	214

6.29	DC_1_n78-n258	215

6.29.1	Operating bands for DC	215

6.29.2	Channel bandwidths per operating band for DC	215

6.29.3	Co-existence studies	215

6.29.4	∆TIB and ∆RIB values	215

6.29.5	MSD	216

6.30	DC_1_n79-n258	216

6.30.1	Operating bands for DC	216

6.30.2	Channel bandwidths per operating band for DC	216

6.30.3	Co-existence studies	216

6.30.4	∆TIB and ∆RIB values	217

6.30.5	MSD	217

6.31	DC_3_n77-n258	217

6.31.1	Operating bands for DC	217

6.31.2	Channel bandwidths per operating band for DC	218

6.31.3	Co-existence studies	218

6.31.4	∆TIB and ∆RIB values	218

6.31.5	MSD	218

6.32	DC_3_n78-n258	219

6.32.1	Operating bands for DC	219

6.32.2	Channel bandwidths per operating band for DC	219

6.32.3	Co-existence studies	219

6.32.4	∆TIB and ∆RIB values	219

6.32.5	MSD	220

6.33	DC_3_n79-n258	220

6.33.1	Operating bands for DC	220

6.33.2	Channel bandwidths per operating band for DC	220

6.33.3	Co-existence studies	220

6.33.4	∆TIB and ∆RIB values	221

6.33.5	MSD	221

6.34	DC_3-3_n78_n257	221

6.34.1	Operating bands for DC	221

6.34.2	Channel bandwidths per operating band for DC	221

6.34.3	Co-existence studies	223

6.34.4	∆TIB and ∆RIB values	223

6.34.5	MSD	223

6.35	DC_20_n3-n78	223

6.35.1	Operating bands for DC	223

6.35.2	Channel bandwidths per operating band for DC	224

6.35.3	Co-existence studies	224

6.35.4	∆TIB and ∆RIB values	224

6.35.5	MSD	224

6.36	DC_7_n1-n78	225

6.36.1	Operating bands for DC	225

6.36.2	Channel bandwidths per operating band for DC	226

6.36.3	Co-existence studies	226

6.36.4	∆TIB and ∆RIB values	226

6.36.5	MSD	226

6.37	DC_20_n1-n78	227

6.37.1	Operating bands for DC	227

6.37.2	Channel bandwidths per operating band for DC	227

6.37.3	Co-existence studies	227

6.37.4	∆TIB and ∆RIB values	228

6.37.5	MSD	228

6.38	DC_7_n3-n78	228

6.38.1	Operating bands for DC	228

6.38.2	Channel bandwidths per operating band for DC	229

6.38.3	Co-existence studies	229

6.38.4	∆TIB and ∆RIB values	229

6.38.5	MSD	229

6.39	   DC_40A_n41A-n79A	230

6.39.1	 Operating bands for DC	230

6.39.2	 Channel bandwidths per operating band for DC	230

6.39.3	 Co-existence studies	231

6.39.4	 ∆TIB and ∆RIB values	231

6.39.5	 MSD	231

6.40	DC_3-3_n1-n78	232

6.40.1	Operating bands for DC	232

6.40.2	Channel bandwidths per operating band for DC	232

6.40.3	Co-existence studies	232

6.40.4	∆TIB and ∆RIB values	232

6.40.5	MSD	233

6.41	DC_7-7_n1-n78	233

6.41.1	Operating bands for DC	233

6.41.2	Channel bandwidths per operating band for DC	233

6.41.3	Co-existence studies	233

6.41.4	∆TIB and ∆RIB values	234

6.41.5	MSD	234

6.42	DC_1_n5-n78	234

6.42.1	Operating bands for DC	234

6.42.2	Channel bandwidths per operating band for DC	234

6.42.3	Co-existence studies	235

6.42.4	∆TIB and ∆RIB values	235

6.42.5	MSD	235

6.43	DC_3_n5-n78	236

6.43.1	Operating bands for DC	236

6.43.2	Channel bandwidths per operating band for DC	236

6.43.3	Co-existence studies	236

6.43.4	∆TIB and ∆RIB values	237

6.43.5	MSD	237

6.44	DC_7_n5-n78	237

6.44.1	Operating bands for DC	237

6.44.2	Channel bandwidths per operating band for DC	238

6.44.3	Co-existence studies	238

6.44.4	∆TIB and ∆RIB values	238

6.44.5	MSD	239

6.45	DC_28_n5-n78	239

6.45.1	Operating bands for DC	239

6.45.2	Channel bandwidths per operating band for DC	239

6.45.3	Co-existence studies	240

6.45.4	∆TIB and ∆RIB values	240

6.45.5	MSD	240

6.46	DC_2A_n41A-n261A	241

6.46.1	Operating bands for DC	241

6.46.2	Channel bandwidths per operating band for DC	241

6.46.3	Co-existence studies	241

6.46.4	∆TIB and ∆RIB values	241

6.46.5	MSD	242

6.47	DC_66A_n41A-n261A	242

6.47.1	Operating bands for DC	242

6.47.2	Channel bandwidths per operating band for DC	242

6.47.3	Co-existence studies	243

6.47.4	∆TIB and ∆RIB values	243

6.47.5	MSD	243

6.48	DC_2_n41-n260	244

6.48.1	Operating bands for DC	244

6.48.2	Channel bandwidths per operating band for DC	244

6.48.3	Co-existence studies	244

6.48.4	∆TIB and ∆RIB values	244

6.48.5	MSD	245

6.49	DC_66_n41-n260	245

6.49.2	Channel bandwidths per operating band for DC	245

6.49.3	Co-existence studies	246

6.49.4	∆TIB and ∆RIB values	246

6.49.5	MSD	247

6.50	DC_3_n77-n257	247

6.50.1	Operating bands for DC	247

6.50.2	Channel bandwidths per operating band for DC	248

6.50.3	Co-existence studies	249

6.50.4	∆TIB and ∆RIB values	249

6.50.5	MSD	249

6.51	DC_8_n3-n28	249

6.51.1	Operating bands for DC	249

6.51.2	Channel bandwidths per operating band for DC	250

6.51.3	Co-existence studies	250

6.51.4	∆TIB and ∆RIB values	250

6.52	DC_11_n77-n257	251

6.52.1	Operating bands for DC	251

6.52.2	Channel bandwidths per operating band for DC	252

6.52.3	Co-existence studies	253

6.52.4	∆TIB and ∆RIB values	253

6.52.5	MSD	253

6.53	 DC_3A_n40A-n41A	253

6.53.1	Operating bands for DC	253

6.53.2	Channel bandwidths per operating band for DC	254

6.53.3	Co-existence studies	254

6.53.4	∆TIB and ∆RIB values	254

6.53.5	MSD	255

6.54	DC_8A_n1A-n78A	255

6.54.1	Operating bands for DC	255

6.54.2	Channel bandwidths per operating band for DC	255

6.54.3	Co-existence studies	255

6.54.4	∆TIB and ∆RIB values	257

6.54.5	MSD	257

6.55	DC_66_n41-n71	258

6.55.1	Operating bands for DC	258

6.55.2	Channel bandwidths per operating band for DC	258

6.55.3	Co-existence studies	258

6.55.4	∆TIB and ∆RIB values	258

6.55.5	MSD	259

6.56	DC_8_n78-n257	259

6.56.1	Operating bands for DC	259

6.56.2	Channel bandwidths per operating band for DC	259

6.56.3	Co-existence studies	261

6.56.4	∆TIB and ∆RIB values	261

6.56.5	MSD	261

6.57	DC_2_n41-n71	261

6.57.1	Operating bands for DC	261

6.57.2	Channel bandwidths per operating band for DC	262

6.57.3	Co-existence studies	262

6.57.4	∆TIB and ∆RIB values	262

6.57.5	MSD	262

6.58	DC_1_n3-n28	263

6.58.1	Operating bands for DC	263

6.58.2	Channel bandwidths per operating band for DC	264

6.58.3	Co-existence studies	264

6.58.4	∆TIB and ∆RIB values	264

6.58.5	MSD	264

6.59 	DC_39_n41-n79	265

6.60	DC_12_n7-n78	267

6.60.1	Operating bands for DC	267

6.60.2	Channel bandwidths per operating band for DC	267

6.60.3	Co-existence studies	267

6.60.4	∆TIB and ∆RIB values	270

6.61	DC_2_n66-n78	271

6.62	DC_12_n66-n78	274

6.63	DC_7_n66-n78 and DC_7-7_n66-n78	275

6.63.1	Operating bands for DC	275

6.63.2	Channel bandwidths per operating band for DC	276

6.63.3	Co-existence studies	276

6.63.4	∆TIB and ∆RIB values	278

6.64	DC_20_n1-n28	279

6.64.1	Operating bands for DC	279

6.64.2	Channel bandwidths per operating band for DC	279

6.64.3	Co-existence studies	280

6.64.4	∆TIB and ∆RIB values	282

6.65	DC_20_n7-n28	282

6.66	DC_20_n7-n78	287

6.67	DC_3_n7-n28	287

6.68	DC_18_n3-n78	289

6.68.1	Operating bands for DC	289

6.68.2	Channel bandwidths per operating band for DC	290

6.68.3	Co-existence studies	290

6.68.4	∆TIB and ∆RIB values	290

6.69	DC_3_n40-n79	291

6.70	DC_3_n41-n79	292

6.71	DC_8_n40-n79	294

6.72	DC_8_n41-n79	296

6.73	DC_39_n40-n79	297

6.74	DC_7-7_n66-n78	299

6.75	DC_3A_n3A-n77A	300

6.75.1	Operating bands for DC	300

6.75.2	Channel bandwidths per operating band for DC	300

6.75.3	UE co-existence studies	300

6.75.4	ΔTIB and ΔRIB values	300

6.75.5	MSD	301

6.76	DC_3A_n3A-n78A	301

6.76.1	Operating bands for DC	301

6.76.2	Channel bandwidths per operating band for DC	302

6.76.3	UE co-existence studies	302

6.76.4	ΔTIB and ΔRIB values	302

6.76.5	MSD	303

6.77	DC_3_n1-n28	303

6.77.1	Operating bands for DC	303

6.77.2	Channel bandwidths per operating band for DC	303

6.77.3	Co-existence studies	304

6.77.4	∆TIB and ∆RIB values	305

6.77.5	MSD	306

6.78	DC_28_n3-n78	306

6.78.1	Operating bands for DC	306

6.78.2	Channel bandwidths per operating band for DC	307

6.78.3	Co-existence studies	307

6.78.4	∆TIB and ∆RIB values	307

6.79	DC_5_n7-n78	308

6.79.1	Operating bands for DC	308

6.79.2	Channel bandwidths per operating band for DC	308

6.79.3	Co-existence studies	308

6.79.4	∆TIB and ∆RIB values	309

6.79.5	MSD	309

6.80	DC_7_n7-n78	310

6.80.1	Operating bands for DC	310

6.80.2	Channel bandwidths per operating band for DC	310

6.80.3	Co-existence studies	310

6.80.4	∆TIB and ∆RIB values	310

6.80.5	MSD	311

6.81	DC_66_n7-n78	311

6.81.1	Operating bands for DC	311

6.81.2	Channel bandwidths per operating band for DC	311

6.81.3	Co-existence studies	312

6.81.4	∆TIB and ∆RIB values	313

6.81.5	MSD	314

6.82	DC_3_n1-n7	314

6.82.1	Operating bands for DC	314

6.82.2	Channel bandwidths per operating band for DC	315

6.82.3	Co-existence studies	315

6.82.4	∆TIB and ∆RIB values	315

6.82.5	MSD	316

6.83	DC_18_n78A-n257	316

6.83.1	Operating bands for DC	316

6.83.2	Channel bandwidths per operating band for DC	316

6.83.3	Co-existence studies	317

6.83.4	∆TIB and ∆RIB values	317

6.83.5	MSD	318

6.84	DC_1_n3A-n257	318

6.84.1	Operating bands for DC	318

6.84.2	Channel bandwidths per operating band for DC	318

6.84.3	Co-existence studies	319

6.84.4	∆TIB and ∆RIB values	319

6.84.5	MSD	320

6.85	DC_18_n3A-n257	320

6.85.1	Operating bands for DC	320

6.85.2	Channel bandwidths per operating band for DC	320

6.85.3	Co-existence studies	321

6.85.4	∆TIB and ∆RIB values	322

6.85.5	MSD	322

6.86	DC_28_n3A-n257	322

6.86.1	Operating bands for DC	322

6.86.2	Channel bandwidths per operating band for DC	322

6.86.3	Co-existence studies	323

6.86.4	∆TIB and ∆RIB values	324

6.86.5	MSD	324

6.87	DC_41_n78A-n257	324

6.87.1	Operating bands for DC	324

6.87.2	Channel bandwidths per operating band for DC	324

6.87.3	Co-existence studies	326

6.87.4	∆TIB and ∆RIB values	326

6.87.5	MSD	327

6.88	DC_42_n78A-n257	327

6.88.1	Operating bands for DC	327

6.88.2	Channel bandwidths per operating band for DC	327

6.88.3	Co-existence studies	329

6.88.4	∆TIB and ∆RIB values	329

6.88.5	MSD	330

6.89	DC_28_n78A-n257	330

6.89.1	Operating bands for DC	330

6.89.2	Channel bandwidths per operating band for DC	330

6.89.3	Co-existence studies	331

6.89.4	∆TIB and ∆RIB values	331

6.89.5	MSD	332

6.91	DC_2_n7-n78	335

6.91.1	Operating bands for DC	335

6.91.2	Channel bandwidths per operating band for DC	335

6.91.3	Co-existence studies	335

6.91.4	∆TIB and ∆RIB values	336

6.92	DC_1A_n8A-n78A	337

6.92.1	Operating bands for DC	337

6.92.2	Channel bandwidths per operating band for DC	337

6.92.3	Co-existence studies	337

6.92.4	∆TIB and ∆RIB values	337

6.92.5	MSD	338

6.93	DC_13_n48-n66	338

6.93.5	MSD	338

6.94	DC_66_n5-n48	339

6.94.5	MSD	339

6.95	DC_11_n3-n28	339

6.95.5	MSD	339

6.96	DC_1_n8-n40	340

6.96.1	Operating bands for DC	340

6.96.2	Channel bandwidths per operating band for DC	340

6.96.3	Co-existence studies	340

6.96.4	∆TIB and ∆RIB values	342

6.96.5	MSD	342

6.97	DC_1_n28-n40	343

6.97.1	Operating bands for DC	343

6.97.2	Configuration for DC	343

6.97.3	Co-existence studies	343

6.97.4	∆TIB and ∆RIB values	343

6.97.5	MSD	344

6.98	DC_3_n1-n40	344

6.98.1	Operating bands for DC	344

6.98.2	Configuration for DC	344

6.98.3	Co-existence studies	344

6.98.4	∆TIB and ∆RIB values	345

6.98.5	MSD	345

6.99	DC_3_n8-n78	345

6.99.1	Operating bands for DC	345

6.99.2	Configuration for DC	346

6.99.3	Co-existence studies	346

6.99.4	∆TIB and ∆RIB values	346

6.99.5	MSD	346

6.100	DC_7_n1-n40	347

6.100.1	Operating bands for DC	347

6.100.2	Configuration for DC	347

6.100.3	Co-existence studies	347

6.100.4	∆TIB and ∆RIB values	347

6.100.5	MSD	348

6.101	DC_7_n8-n78	348

6.101.1	Operating bands for DC	348

6.101.2	Configuration for DC	348

6.101.3	Co-existence studies	348

6.101.4	∆TIB and ∆RIB values	348

6.101.5	MSD	349

6.102	DC_13_n7-n78	349

6.102.5	MSD	349

6.103	DC_28_n7-n78	350

6.103.1	Operating bands for DC	350

6.103.2	Channel bandwidths per operating band for DC	350

6.103.3	Co-existence studies	350

6.103.4	∆TIB and ∆RIB values	351

6.103.5	MSD	351

6.104	DC_2_n41-n66	351

6.104.1	Operating bands for DC	351

6.104.2	Configuration for DC	352

6.104.3	Co-existence studies	352

6.104.4	∆TIB and ∆RIB values	352

6.104.5	MSD	352

6.105	DC_66_n25-n71	353

6.105.1	Operating bands for DC	353

6.105.2	Configuration for DC	353

6.105.3	Co-existence studies	353

6.105.4	∆TIB and ∆RIB values	353

6.105.5	MSD	354

6.106	DC_66_n12-n258	354

6.106.1	Operating bands for DC	354

6.106.2	Channel bandwidths per operating band for DC	354

6.106.3	Co-existence studies	354

6.106.4	∆TIB and ∆RIB values	354

6.106.5	MSD	355

6.107	DC_66_n12-n260	355

6.107.1	Operating bands for DC	355

6.107.2	Channel bandwidths per operating band for DC	355

6.107.3	Co-existence studies	355

6.107.4	∆TIB and ∆RIB values	355

6.107.5	MSD	356

6.108	DC_66_n12-n261	356

6.108.1	Operating bands for DC	356

6.108.2	Channel bandwidths per operating band for DC	356

6.108.3	Co-existence studies	357

6.108.4	∆TIB and ∆RIB values	357

6.108.5	MSD	357

6.109	DC_28_n77-n257	357

6.109.1	Operating bands for DC	357

6.109.2	Channel bandwidths per operating band for DC	358

6.109.3	Co-existence studies	358

6.109.4	∆TIB and ∆RIB values	358

6.109.5	MSD	359

6.110	DC_42_n77-n257	359

6.110.1	Operating bands for DC	359

6.110.2	Channel bandwidths per operating band for DC	359

6.110.3	Co-existence studies	362

6.110.4	∆TIB and ∆RIB values	362

6.110.5	MSD	362

6.111	DC_42_n79-n257	362

6.111.1	Operating bands for DC	362

6.111.2	Channel bandwidths per operating band for DC	362

6.111.3	Co-existence studies	365

6.111.4	∆TIB and ∆RIB values	365

6.111.5	MSD	365

6.112	DC_2_n12-n258	365

6.112.1	Operating bands for DC	365

6.112.2	Channel bandwidths per operating band for DC	365

6.112.3	Co-existence studies	366

6.112.4	∆TIB and ∆RIB values	366

6.112.5	MSD	366

6.113	DC_2_n12-n260	366

6.113.1	Operating bands for DC	366

6.113.2	Channel bandwidths per operating band for DC	367

6.113.3	Co-existence studies	367

6.113.4	∆TIB and ∆RIB values	367

6.113.5	MSD	367

6.114	DC_2_n12-n261	368

6.114.1	Operating bands for DC	368

6.114.2	Channel bandwidths per operating band for DC	368

6.114.3	Co-existence studies	368

6.114.4	∆TIB and ∆RIB values	368

6.114.5	MSD	369

6.115	DC_2_n71-n261	369

6.115.1	Operating bands for DC	369

6.115.2	Channel bandwidths per operating band for DC	369

6.115.3	Co-existence studies	369

6.115.4	∆TIB and ∆RIB values	370

6.115.5	MSD	370

6.122	DC_7_n8-n40	391

6.122.1	Operating bands for DC	391

6.122.2	Configuration for DC	391

6.122.3	Co-existence studies	391

6.122.4	∆TIB and ∆RIB values	391

6.122.5	MSD	392

6.123	DC_28_n40-n78	392

6.123.1	Operating bands for DC	392

6.123.2	Co-existence studies	392

6.123.3	∆TIB and ∆RIB values	392

6.123.4	MSD	393

6.124	DC_3_n28-n77	393

6.124.1	Operating bands for DC	393

6.124.2	Channel bandwidths per operating band for DC	394

6.124.3	Co-existence studies	394

6.124.4	∆TIB and ∆RIB values	394

6.124.5	MSD	395

6.125	DC_8_n28-n77	395

6.125.1	Operating bands for DC	395

6.125.2	Channel bandwidths per operating band for DC	396

6.125.3	Co-existence studies	396

6.125.4	∆TIB and ∆RIB values	396

6.125.5	MSD	397

6.126	DC_42_n28-n77	397

6.126.1	Operating bands for DC	397

6.126.2	Channel bandwidths per operating band for DC	398

6.126.3	Co-existence studies	399

6.126.4	∆TIB and ∆RIB values	399

6.126.5	MSD	399

6.127	DC_2_n38-n78	400

6.127.1	Operating bands for DC	400

6.127.2	Channel bandwidths per operating band for DC	400

6.127.3	Co-existence studies	400

6.127.4	∆TIB and ∆RIB values	403

6.127.5	MSD	403

6.128	DC_66_n38-n78	403

6.128.1	Operating bands for DC	403

6.128.2	Channel bandwidths per operating band for DC	404

6.128.3	Co-existence studies	404

6.128.4	∆TIB and ∆RIB values	406

6.128.5	MSD	407

6.129	DC_18_n3-n77	407

6.129.5	MSD	407

6.130	DC_28_n3-n77	408

6.130.5	MSD	408

6.131	DC_41_n3-n77	408

6.131.5	MSD	408

6.132	DC_41_n3-n78	409

6.132.5	MSD	409

6.133	DC_41_n28-n78	409

6.133.5	MSD	409

6.134	DC_1_n75-n78	410

6.134.5	MSD	412

6.135	DC_3_n75-n78	412

6.135.5	MSD	415

6.136	DC_1_n28-n77	415

6.136.1	Operating bands for DC	415

6.136.2	Channel bandwidths per operating band for DC	416

6.136.3	Co-existence studies	416

6.136.4	∆TIB and ∆RIB values	416

6.136.5	MSD	417

6.137	DC_7_n1-n257, DC_7-7_n1-n257	418

6.137.1	Operating bands for DC	418

6.137.2	Channel bandwidths per operating band for DC	418

6.137.3	Co-existence studies	418

6.137.4	∆TIB and ∆RIB values	418

6.137.5	MSD	419

6.138	DC_20_n78-n92	419

6.138.1	Operating bands for DC	419

6.138.2	Channel bandwidths per operating band for DC	419

6.138.3	Co-existence studies	419

6.138.4	∆TIB and ∆RIB values	420

6.138.5	MSD	420

6.141	 DC_1_n41-n78	422

6.142	 DC_3_n41-n78	427

6.143	 DC_20_n41-n78	431

6.144	 DC_20_n75-n78	435

6.145	DC_3_n8-n40	438

6.145.1	Operating bands for DC	438

6.145.2	Configuration for DC	438

6.145.3	Co-existence studies	438

6.145.4	∆TIB and ∆RIB values	438

6.145.5	MSD	439

6.146	DC_3_n28-n40	439

6.146.1	Operating bands for DC	439

6.146.2	Configuration for DC	439

6.146.3	Co-existence studies	439

6.146.4	∆TIB and ∆RIB values	439

6.146.5	MSD	440

6.147	DC_7_n28-n40	440

6.147.1	Operating bands for DC	440

6.147.2	Configuration for DC	440

6.147.3	Co-existence studies	440

6.147.4	∆TIB and ∆RIB values	441

6.147.5	MSD	441

6.150	 DC_(n)41-n78	445

6.151	DC_1_n28-n257	448

6.151.1	Operating bands for DC	448

6.151.2	Channel bandwidths per operating band for DC	448

6.151.3	Co-existence studies	448

6.151.4	∆TIB and ∆RIB values	448

6.151.5	MSD	449

6.152	DC_3_n28-n257	449

6.152.1	Operating bands for DC	449

6.152.2	Channel bandwidths per operating band for DC	450

6.152.3	Co-existence studies	450

6.152.4	∆TIB and ∆RIB values	450

6.152.5	MSD	451

6.153	DC_41_n3-n257	451

6.153.1	Operating bands for DC	451

6.153.2	Channel bandwidths per operating band for DC	452

6.153.3	Co-existence studies	453

6.153.4	∆TIB and ∆RIB values	453

6.153.5	MSD	453

6.154	DC_41_n28-n257	453

6.154.1	Operating bands for DC	453

6.154.2	Channel bandwidths per operating band for DC	454

6.154.3	Co-existence studies	456

6.154.4	∆TIB and ∆RIB values	456

6.154.5	MSD	456

6.155	DC_41_n77-n257	456

6.155.1	Operating bands for DC	456

6.155.2	Channel bandwidths per operating band for DC	457

6.155.3	Co-existence studies	458

6.155.4	∆TIB and ∆RIB values	458

6.155.5	MSD	458

7	EN-DC band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	459

7.1	DC_1A-3A_n78A-n257	459

7.1.1	Operating bands for DC	459

7.1.2	Channel bandwidths per operating band for DC	459

7.1.3	Co-existence studies	461

7.1.4	∆TIB and ∆RIB values	461

7.1.5	MSD	461

7.2	DC_1A-5A_n78A-n257	462

7.2.1	Operating bands for DC	462

7.2.2	Channel bandwidths per operating band for DC	462

7.2.3	Co-existence studies	464

7.2.4	∆TIB and ∆RIB values	464

7.2.5	MSD	464

7.3	DC_1A-7A_n78A-n257	465

7.3.1	Operating bands for DC	465

7.3.2	Channel bandwidths per operating band for DC	465

7.3.3	Co-existence studies	467

7.3.4	∆TIB and ∆RIB values	467

7.3.5	MSD	467

7.4	DC_1A-7A-7A_n78A-n257	468

7.4.1	Operating bands for DC	468

7.4.2	Channel bandwidths per operating band for DC	468

7.4.3	Co-existence studies	470

7.4.4	∆TIB and ∆RIB values	470

7.4.5	MSD	470

7.5	DC_3A-5A_n78A-n257	471

7.5.1	Operating bands for DC	471

7.5.2	Channel bandwidths per operating band for DC	471

7.5.3	Co-existence studies	473

7.5.4	∆TIB and ∆RIB values	473

7.5.5	MSD	473

7.6	DC_3A-7A_n78A-n257	474

7.6.1	Operating bands for DC	474

7.6.2	Channel bandwidths per operating band for DC	474

7.6.3	Co-existence studies	476

7.6.4	∆TIB and ∆RIB values	476

7.6.5	MSD	476

7.7	DC_3A-7A-7A_n78A-n257	477

7.7.1	Operating bands for DC	477

7.7.2	Channel bandwidths per operating band for DC	477

7.7.3	Co-existence studies	479

7.7.4	∆TIB and ∆RIB values	479

7.7.5	MSD	479

7.8	DC_5A-7A_n78A-n257	480

7.8.1	Operating bands for DC	480

7.8.2	Channel bandwidths per operating band for DC	480

7.8.3	Co-existence studies	482

7.8.4	∆TIB and ∆RIB values	482

7.8.5	MSD	482

7.9	DC_5A-7A-7A_n78A-n257	483

7.9.1	Operating bands for DC	483

7.9.2	Channel bandwidths per operating band for DC	483

7.9.3	Co-existence studies	485

7.9.4	∆TIB and ∆RIB values	485

7.9.5	MSD	485

7.10	DC_1A-3C_n28A-n78A	486

7.10.1	Operating bands for DC	486

7.10.2	Channel bandwidths per operating band for DC	486

7.10.3	Co-existence studies	486

7.10.4	∆TIB and ∆RIB values	486

7.10.5	MSD	487

7.11	DC_1A-7C_n28A-n78A	487

7.11.1	Operating bands for DC	487

7.11.2	Channel bandwidths per operating band for DC	488

7.11.3	Co-existence studies	488

7.11.4	∆TIB and ∆RIB values	488

7.11.5	MSD	488

7.12	DC_1-3_n7-n78	489

7.12.1	Operating bands for DC	489

7.12.2	Channel bandwidths per operating band for DC	490

7.12.3	Co-existence studies	490

7.12.4	∆TIB and ∆RIB values	490

7.12.5	MSD	491

7.13	DC_3-7_n28-n78	491

7.13.1	Operating bands for DC	491

7.13.2	Channel bandwidths per operating band for DC	492

7.13.3	Co-existence studies	492

7.13.4	∆TIB and ∆RIB values	492

7.13.5	MSD	493

7.14	DC_1-8_n77-n257	493

7.14.1	Operating bands for DC	493

7.14.2	Channel bandwidths per operating band for DC	494

7.14.3	Co-existence studies	496

7.14.4	∆TIB and ∆RIB values	496

7.14.5	MSD	496

7.15	DC_1-19_n77-n79	496

7.15.1	Operating bands for DC	496

7.15.2	Channel bandwidths per operating band for DC	497

7.15.3	Co-existence studies	497

7.15.4	∆TIB and ∆RIB values	497

7.15.5	MSD	497

7.16	DC_1-19_n78-n79	498

7.16.1	Operating bands for DC	498

7.16.2	Channel bandwidths per operating band for DC	498

7.16.3	Co-existence studies	498

7.16.4	∆TIB and ∆RIB values	498

7.16.5	MSD	499

7.17	DC_1-21_n77-n79	499

7.17.1	Operating bands for DC	499

7.17.2	Channel bandwidths per operating band for DC	499

7.17.3	Co-existence studies	500

7.17.4	∆TIB and ∆RIB values	500

7.17.5	MSD	500

7.18	DC_1-21_n78-n79	500

7.18.1	Operating bands for DC	500

7.18.2	Channel bandwidths per operating band for DC	501

7.18.3	Co-existence studies	501

7.18.4	∆TIB and ∆RIB values	501

7.18.5	MSD	502

7.19	DC_1-3_n77-n79	502

7.19.1	Operating bands for DC	502

7.19.2	Channel bandwidths per operating band for DC	502

7.19.3	Co-existence studies	502

7.19.4	∆TIB and ∆RIB values	502

7.19.5	MSD	503

7.20	DC_1-3_n78-n79	503

7.20.1	Operating bands for DC	503

7.20.2	Channel bandwidths per operating band for DC	503

7.20.3	Co-existence studies	504

7.20.4	∆TIB and ∆RIB values	504

7.20.5	MSD	504

7.21	DC_1-42_n77-n79	505

7.21.1	Operating bands for DC	505

7.21.2	Channel bandwidths per operating band for DC	505

7.21.3	Co-existence studies	505

7.21.4	∆TIB and ∆RIB values	505

7.21.5	MSD	506

7.22	DC_1-42_n78-n79	506

7.22.1	Operating bands for DC	506

7.22.2	Channel bandwidths per operating band for DC	506

7.22.3	Co-existence studies	507

7.22.4	∆TIB and ∆RIB values	507

7.22.5	MSD	507

7.23	DC_19-21_n77-n79	507

7.23.1	Operating bands for DC	507

7.23.2	Channel bandwidths per operating band for DC	508

7.23.3	Co-existence studies	508

7.23.4	∆TIB and ∆RIB values	508

7.23.5	MSD	508

7.24	DC_19-21_n78-n79	509

7.24.1	Operating bands for DC	509

7.24.2	Channel bandwidths per operating band for DC	509

7.24.3	Co-existence studies	509

7.24.4	∆TIB and ∆RIB values	509

7.24.5	MSD	510

7.25	DC_19-42_n77-n79	510

7.25.1	Operating bands for DC	510

7.25.2	Channel bandwidths per operating band for DC	510

7.25.3	Co-existence studies	511

7.25.4	∆TIB and ∆RIB values	511

7.25.5	MSD	511

7.26	DC_19-42_n78-n79	511

7.26.1	Operating bands for DC	511

7.26.2	Channel bandwidths per operating band for DC	512

7.26.3	Co-existence studies	512

7.26.4	∆TIB and ∆RIB values	512

7.26.5	MSD	512

7.27	DC_21-42_n77-n79	513

7.27.1	Operating bands for DC	513

7.27.2	Channel bandwidths per operating band for DC	513

7.27.3	Co-existence studies	513

7.27.4	∆TIB and ∆RIB values	513

7.27.5	MSD	514

7.28	DC_21-42_n78-n79	514

7.28.1	Operating bands for DC	514

7.28.2	Channel bandwidths per operating band for DC	514

7.28.3	Co-existence studies	515

7.28.4	∆TIB and ∆RIB values	515

7.28.5	MSD	515

7.29	DC_3-19_n77-n79	515

7.29.1	Operating bands for DC	515

7.29.2	Channel bandwidths per operating band for DC	516

7.29.3	Co-existence studies	516

7.29.4	∆TIB and ∆RIB values	516

7.29.5	MSD	516

7.30	DC_3-19_n78-n79	517

7.30.1	Operating bands for DC	517

7.30.2	Channel bandwidths per operating band for DC	517

7.30.3	Co-existence studies	517

7.30.4	∆TIB and ∆RIB values	517

7.30.5	MSD	518

7.31	DC_3-21_n77-n79	518

7.31.1	Operating bands for DC	518

7.31.2	Channel bandwidths per operating band for DC	518

7.31.3	Co-existence studies	519

7.31.4	∆TIB and ∆RIB values	519

7.31.5	MSD	519

7.32	DC_3-21_n78-n79	519

7.32.1	Operating bands for DC	519

7.32.2	Channel bandwidths per operating band for DC	520

7.32.3	Co-existence studies	520

7.32.4	∆TIB and ∆RIB values	520

7.32.5	MSD	520

7.33	DC_3-42_n77-n79	521

7.33.1	Operating bands for DC	521

7.33.2	Channel bandwidths per operating band for DC	521

7.33.3	Co-existence studies	521

7.33.4	∆TIB and ∆RIB values	521

7.33.5	MSD	522

7.34	DC_3-42_n78-n79	522

7.34.1	Operating bands for DC	522

7.34.2	Channel bandwidths per operating band for DC	522

7.34.3	Co-existence studies	523

7.34.4	∆TIB and ∆RIB values	523

7.34.5	MSD	523

7.35	DC_3-3-7_n78_n257, DC_3-3-7-7_n78_n257,	523

7.35.1	Operating bands for DC	523

7.35.2	Channel bandwidths per operating band for DC	524

7.35.3	Co-existence studies	527

7.35.4	∆TIB and ∆RIB values	527

7.35.5	MSD	527

7.36	DC_3-7_n1-n78	528

7.36.1	Operating bands for DC	528

7.36.2	Channel bandwidths per operating band for DC	529

7.36.3	Co-existence studies	529

7.36.4	∆TIB and ∆RIB values	529

7.36.5	MSD	530

7.37	DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78	530

7.37.1	Operating bands for DC	530

7.37.2	Channel bandwidths per operating band for DC	530

7.37.3	Co-existence studies	531

7.37.4	∆TIB and ∆RIB values	531

7.37.5	MSD	532

7.38	DC_1-3_n5-n78	532

7.38.1	Operating bands for DC	532

7.38.2	Channel bandwidths per operating band for DC	532

7.38.3	Co-existence studies	533

7.38.4	∆TIB and ∆RIB values	533

7.38.5	MSD	533

7.39	DC_1-7_n5-n78	533

7.39.1	Operating bands for DC	533

7.39.2	Channel bandwidths per operating band for DC	534

7.39.3	Co-existence studies	534

7.39.4	∆TIB and ∆RIB values	534

7.39.5	MSD	535

7.40	DC_1-28_n5-n78	535

7.40.1	Operating bands for DC	535

7.40.2	Channel bandwidths per operating band for DC	535

7.40.3	Co-existence studies	535

7.40.4	∆TIB and ∆RIB values	535

7.40.5	MSD	536

7.41	DC_3-7_n5-n78	536

7.41.1	Operating bands for DC	536

7.41.2	Channel bandwidths per operating band for DC	536

7.41.3	Co-existence studies	537

7.41.4	∆TIB and ∆RIB values	537

7.41.5	MSD	538

7.42	DC_3-28_n5-n78	538

7.42.1	Operating bands for DC	538

7.42.2	Channel bandwidths per operating band for DC	538

7.42.3	Co-existence studies	538

7.42.4	∆TIB and ∆RIB values	539

7.42.5	MSD	539

7.43	DC_7-28_n5-n78	539

7.43.1	Operating bands for DC	539

7.43.2	Channel bandwidths per operating band for DC	539

7.43.3	Co-existence studies	540

7.43.4	∆TIB and ∆RIB values	540

7.43.5	MSD	540

7.44	DC_1A-3A_n77A-n257	541

7.44.1	Operating bands for DC	541

7.44.2	Channel bandwidths per operating band for DC	541

7.44.3	Co-existence studies	542

7.44.4	∆TIB and ∆RIB values	543

7.44.5	MSD	543

7.45	DC_2-66_n41-n71	544

7.45.1	Operating bands for DC	544

7.45.2	Channel bandwidths per operating band for DC	544

7.45.3	Co-existence studies	544

7.45.4	∆TIB and ∆RIB values	544

7.45.5	MSD	545

7.46	DC_2-66_n41-n260	545

7.46.1	Operating bands for DC	545

7.46.2	Channel bandwidths per operating band for DC	546

7.46.3	Co-existence studies	546

7.46.4	∆TIB and ∆RIB values	546

7.46.5	MSD	547

7.47	DC_2A-66A_n41A-n261	547

7.47.1	Operating bands for DC	547

7.47.3	Co-existence studies	548

7.47.4	∆TIB and ∆RIB values	548

7.47.5	MSD	549

7.48	DC_3-8_n78-n257	549

7.48.1	Operating bands for DC	549

7.48.2	Channel bandwidths per operating band for DC	549

7.48.3	Co-existence studies	551

7.48.4	∆TIB and ∆RIB values	551

7.48.5	MSD	551

7.49	DC_1-8_n78-n257	551

7.49.1	Operating bands for DC	551

7.49.2	Channel bandwidths per operating band for DC	552

7.49.3	Co-existence studies	554

7.49.4	∆TIB and ∆RIB values	554

7.49.5	MSD	554

7.50	DC_2-7_n38-n78	555

7.50.1	Operating bands for DC	555

7.50.2	Channel bandwidths per operating band for DC	555

7.50.3	Co-existence studies	555

7.50.4	∆TIB and ∆RIB values	556

7.50.5	MSD	556

7.51	DC_7A-7A-66A_n38A-n78A	556

7.51.1	Operating bands for DC	556

7.51.2	Channel bandwidths per operating band for DC	557

7.51.3	Co-existence studies	557

7.51.4	∆TIB and ∆RIB values	557

7.51.5	MSD	557

7.52	 DC_1-3_n38-n78	558

7.52.1	 Operating bands for DC	558

7.52.2	Channel bandwidths per operating band for DC	558

7.52.3	Co-existence studies	558

7.52.4	∆TIB and ∆RIB values	558

7.52.5	MSD	559

7.53	DC_1-8_n3-n28	559

7.53.1	Operating bands for DC	559

7.53.2	Channel bandwidths per operating band for DC	560

7.53.3	Co-existence studies	560

7.53.4	∆TIB and ∆RIB values	560

7.53.5	MSD	560

7.54	DC_3-8_n1-n78, DC_3-3-8_n1-n78	561

7.54.1	Operating bands for DC	561

7.54.2	Channel bandwidths per operating band for DC	561

7.54.3	Co-existence studies	561

7.54.4	∆TIB and ∆RIB values	562

7.54.5	MSD	562

7.55	DC_7-8_n1-n78, DC_7-7-8_n1-n78	562

7.55.1	Operating bands for DC	562

7.55.2	Channel bandwidths per operating band for DC	563

7.55.3	Co-existence studies	563

7.55.4	∆TIB and ∆RIB values	563

7.55.5	MSD	564

7.56	DC_1A-18A_n3-n78	564

7.56.1	Operating bands for DC	564

7.56.2	Channel bandwidths per operating band for DC	565

7.56.3	Co-existence studies	565

7.56.4	∆TIB and ∆RIB values	565

7.57	DC_1A-28A_n3-n78	566

7.57.1	Operating bands for DC	566

7.57.2	Channel bandwidths per operating band for DC	566

7.57.3	Co-existence studies	566

7.57.4	∆TIB and ∆RIB values	566

7.58	DC_3-20_n1-n28	567

7.58.1	Operating bands for DC	567

7.58.3	Co-existence studies	568

7.58.4	∆TIB and ∆RIB values	568

7.58.5	MSD	568

7.59	DC_1-11_n77-n257	569

7.59.1	Operating bands for DC	569

7.59.2	Channel bandwidths per operating band for DC	570

7.59.3	Co-existence studies	572

7.59.4	∆TIB and ∆RIB values	572

7.59.5	MSD	572

7.60	DC_8-11_n77-n257	573

7.60.1	Operating bands for DC	573

7.60.2	Channel bandwidths per operating band for DC	574

7.60.3	Co-existence studies	576

7.60.4	∆TIB and ∆RIB values	576

7.60.5	MSD	576

7.61	DC_1A-18A_n3A-n257	576

7.61.1	Operating bands for DC	576

7.61.2	Channel bandwidths per operating band for DC	577

7.61.3	Co-existence studies	577

7.61.4	∆TIB and ∆RIB values	578

7.61.5	MSD	578

7.62	DC_1A-28A_n3A-n257	578

7.62.1	Operating bands for DC	578

7.62.2	Channel bandwidths per operating band for DC	579

7.62.3	Co-existence studies	579

7.62.4	∆TIB and ∆RIB values	579

7.62.5	MSD	580

7.63	DC_1A-41_n78A-n257	580

7.63.1	Operating bands for DC	580

7.63.2	Channel bandwidths per operating band for DC	580

7.63.3	Co-existence studies	582

7.63.4	∆TIB and ∆RIB values	582

7.63.5	MSD	582

7.64	DC_3A-41A_n78A-n257	582

7.64.1	Operating bands for DC	582

7.64.2	Channel bandwidths per operating band for DC	583

7.64.3	Co-existence studies	583

7.64.4	∆TIB and ∆RIB values	583

7.64.5	MSD	584

7.65	DC_3A-42A_n78A-n257	584

7.65.1	Operating bands for DC	584

7.65.2	Channel bandwidths per operating band for DC	584

7.65.3	Co-existence studies	585

7.65.4	∆TIB and ∆RIB values	585

7.65.5	MSD	586

7.66	DC_41A-42A_n78A-n257	586

7.66.1	Operating bands for DC	586

7.66.2	Channel bandwidths per operating band for DC	586

7.66.3	Co-existence studies	587

7.66.4	∆TIB and ∆RIB values	587

7.66.5	MSD	588

7.67	DC_1A-42A_n78A-n257	588

7.67.1	Operating bands for DC	588

7.67.2	Channel bandwidths per operating band for DC	588

7.67.3	Co-existence studies	589

7.67.4	∆TIB and ∆RIB values	589

7.67.5	MSD	590

7.68	DC_1A-18A_n78A-n257	590

7.68.1	Operating bands for DC	590

7.68.2	Channel bandwidths per operating band for DC	590

7.68.3	Co-existence studies	591

7.68.4	∆TIB and ∆RIB values	591

7.68.5	MSD	592

7.69	DC_3A-18A_n78A-n257	592

7.69.1	Operating bands for DC	592

7.69.2	Channel bandwidths per operating band for DC	592

7.69.3	Co-existence studies	593

7.69.4	∆TIB and ∆RIB values	593

7.69.5	MSD	594

7.70	DC_18A-42A_n78A-n257	594

7.70.1	Operating bands for DC	594

7.70.2	Channel bandwidths per operating band for DC	594

7.70.3	Co-existence studies	595

7.70.4	∆TIB and ∆RIB values	595

7.70.5	MSD	596

7.71	DC_3A-28A_n78A-n257	596

7.71.1	Operating bands for DC	596

7.71.2	Channel bandwidths per operating band for DC	596

7.71.3	Co-existence studies	597

7.71.4	∆TIB and ∆RIB values	597

7.71.5	MSD	598

7.72	DC_28A-41A_n78A-n257	598

7.72.1	Operating bands for DC	598

7.72.2	Channel bandwidths per operating band for DC	598

7.72.3	Co-existence studies	599

7.72.4	∆TIB and ∆RIB values	599

7.72.5	MSD	600

7.73	DC_28A-42A_n78A-n257	600

7.73.1	Operating bands for DC	600

7.73.2	Channel bandwidths per operating band for DC	600

7.73.3	Co-existence studies	601

7.73.4	∆TIB and ∆RIB values	601

7.73.5	MSD	602

7.74	DC_1A-28A_n78A-n257	602

7.74.1	Operating bands for DC	602

7.74.2	Channel bandwidths per operating band for DC	602

7.74.3	Co-existence studies	603

7.74.4	∆TIB and ∆RIB values	603

7.74.5	MSD	604

7.76	DC_2-66_n66-n78	606

7.76.1	Operating bands for DC	606

7.76.2	Channel bandwidths per operating band for DC	607

7.76.3	Co-existence studies	607

7.76.4	∆TIB and ∆RIB values	607

7.78	 DC_1-20_n3-n38	610

7.78.1	 Operating bands for DC	610

7.78.2	Channel bandwidths per operating band for DC	611

7.78.3	Co-existence studies	611

7.78.4	∆TIB and ∆RIB values	611

7.78.5	MSD	611

7.79	 DC_1-20_n3-n78	612

7.79.1	 Operating bands for DC	612

7.79.2	Channel bandwidths per operating band for DC	612

7.79.3	Co-existence studies	612

7.79.4	∆TIB and ∆RIB values	612

7.79.5	MSD	613

7.80	 DC_7-20_n3-n78	613

7.80.1	 Operating bands for DC	613

7.80.2	Channel bandwidths per operating band for DC	613

7.80.3	Co-existence studies	613

7.80.4	∆TIB and ∆RIB values	614

7.80.5	MSD	614

7.81	DC_3-28_n7-n78	614

7.81.1	Operating bands for DC	614

7.81.2	Channel bandwidths per operating band for DC	614

7.81.3	Co-existence studies	616

7.81.4	∆TIB and ∆RIB values	616

7.81.5	MSD	616

7.82	DC_1-28_n7-n78	616

7.82.1	Operating bands for DC	616

7.82.2	Channel bandwidths per operating band for DC	617

7.82.3	Co-existence studies	617

7.82.4	∆TIB and ∆RIB values	617

7.82.5	MSD	618

7.83	DC_7-28_n3-n78	618

7.83.1	Operating bands for DC	618

7.83.2	Channel bandwidths per operating band for DC	618

7.83.3	Co-existence studies	619

7.83.4	∆TIB and ∆RIB values	619

7.83.5	MSD	619

7.84	DC_2-46_n41-n66	619

7.84.1	Operating bands for DC	619

7.84.2	Channel bandwidths per operating band for DC	620

7.84.3	Co-existence studies	620

7.84.4	∆TIB and ∆RIB values	620

7.84.5	MSD	621

7.85	DC_2-66_n71-n261	621

7.85.1	Operating bands for DC	621

7.85.2	Channel bandwidths per operating band for DC	621

7.85.3	Co-existence studies	622

7.85.4	∆TIB and ∆RIB values	622

7.85.5	MSD	622

7.86	DC_1-19_n77-n257	622

7.86.1	Operating bands for DC	622

7.86.2	Channel bandwidths per operating band for DC	623

7.86.3	Co-existence studies	625

7.86.4	∆TIB and ∆RIB values	625

7.86.5	MSD	625

7.87	DC_1-19_n78-n257	625

7.87.1	Operating bands for DC	625

7.87.2	Channel bandwidths per operating band for DC	625

7.87.3	Co-existence studies	627

7.87.4	∆TIB and ∆RIB values	627

7.87.5	MSD	627

7.88	DC_1-19_n79-n257	627

7.88.1	Operating bands for DC	627

7.88.2	Channel bandwidths per operating band for DC	627

7.88.3	Co-existence studies	629

7.88.4	∆TIB and ∆RIB values	629

7.88.5	MSD	629

7.89	DC_1-21_n77-n257	629

7.89.1	Operating bands for DC	629

7.89.2	Channel bandwidths per operating band for DC	629

7.89.3	Co-existence studies	631

7.89.4	∆TIB and ∆RIB values	631

7.89.5	MSD	631

7.90	DC_1-21_n78-n257	631

7.90.1	Operating bands for DC	631

7.90.2	Channel bandwidths per operating band for DC	631

7.90.3	Co-existence studies	633

7.90.4	∆TIB and ∆RIB values	633

7.90.5	MSD	633

7.91	DC_1-21_n79-n257	633

7.91.1	Operating bands for DC	633

7.91.2	Channel bandwidths per operating band for DC	633

7.91.3	Co-existence studies	635

7.91.4	∆TIB and ∆RIB values	635

7.91.5	MSD	635

7.92	DC_1-3_n79-n257	635

7.92.1	Operating bands for DC	635

7.92.2	Channel bandwidths per operating band for DC	635

7.92.3	Co-existence studies	637

7.92.4	∆TIB and ∆RIB values	637

7.92.5	MSD	637

7.93	DC_1-42_n77-n257	637

7.93.1	Operating bands for DC	637

7.93.2	Channel bandwidths per operating band for DC	637

7.93.3	Co-existence studies	640

7.93.4	∆TIB and ∆RIB values	640

7.93.5	MSD	640

7.94	DC_1-42_n79-n257	640

7.94.1	Operating bands for DC	640

7.94.2	Channel bandwidths per operating band for DC	640

7.94.3	Co-existence studies	643

7.94.4	∆TIB and ∆RIB values	643

7.94.5	MSD	643

7.95	DC_19-21_n77-n257	643

7.95.1	Operating bands for DC	643

7.95.2	Channel bandwidths per operating band for DC	643

7.95.3	Co-existence studies	645

7.95.4	∆TIB and ∆RIB values	645

7.95.5	MSD	645

7.96	DC_19-21_n78-n257	645

7.96.1	Operating bands for DC	645

7.96.2	Channel bandwidths per operating band for DC	645

7.96.3	Co-existence studies	647

7.96.4	∆TIB and ∆RIB values	647

7.96.5	MSD	647

7.97	DC_19-21_n79-n257	647

7.97.1	Operating bands for DC	647

7.97.2	Channel bandwidths per operating band for DC	647

7.97.3	Co-existence studies	649

7.97.4	∆TIB and ∆RIB values	649

7.97.5	MSD	649

7.98	DC_19-42_n77-n257	649

7.98.1	Operating bands for DC	649

7.98.2	Channel bandwidths per operating band for DC	649

7.98.3	Co-existence studies	652

7.98.4	∆TIB and ∆RIB values	652

7.98.5	MSD	652

7.99	DC_19-42_n78-n257	652

7.99.1	Operating bands for DC	652

7.99.2	Channel bandwidths per operating band for DC	652

7.99.3	Co-existence studies	655

7.99.4	∆TIB and ∆RIB values	655

7.99.5	MSD	655

7.100	DC_19-42_n79-n257	655

7.100.1	Operating bands for DC	655

7.100.2	Channel bandwidths per operating band for DC	655

7.100.3	Co-existence studies	658

7.100.4	∆TIB and ∆RIB values	658

7.100.5	MSD	658

7.101	DC_21-42_n77-n257	658

7.101.1	Operating bands for DC	658

7.101.2	Channel bandwidths per operating band for DC	658

7.101.3	Co-existence studies	661

7.101.4	∆TIB and ∆RIB values	661

7.101.5	MSD	661

7.102	DC_21-42_n78-n257	661

7.102.1	Operating bands for DC	661

7.102.2	Channel bandwidths per operating band for DC	661

7.102.3	Co-existence studies	664

7.102.4	∆TIB and ∆RIB values	664

7.102.5	MSD	664

7.103	DC_21-42_n79-n257	664

7.103.1	Operating bands for DC	664

7.103.2	Channel bandwidths per operating band for DC	664

7.103.3	Co-existence studies	667

7.103.4	∆TIB and ∆RIB values	667

7.103.5	MSD	667

7.104	DC_3-19_n77-n257	667

7.104.1	Operating bands for DC	667

7.104.2	Channel bandwidths per operating band for DC	667

7.104.3	Co-existence studies	669

7.104.4	∆TIB and ∆RIB values	669

7.104.5	MSD	669

7.105	DC_3-19_n78-n257	669

7.105.1	Operating bands for DC	669

7.105.2	Channel bandwidths per operating band for DC	669

7.105.3	Co-existence studies	671

7.105.4	∆TIB and ∆RIB values	671

7.105.5	MSD	671

7.106	DC_3-19_n79-n257	671

7.106.1	Operating bands for DC	671

7.106.2	Channel bandwidths per operating band for DC	671

7.106.3	Co-existence studies	673

7.106.4	∆TIB and ∆RIB values	673

7.106.5	MSD	673

7.107	DC_3-21_n77-n257	673

7.107.1	Operating bands for DC	673

7.107.2	Channel bandwidths per operating band for DC	673

7.107.3	Co-existence studies	675

7.107.4	∆TIB and ∆RIB values	675

7.107.5	MSD	675

7.108	DC_3-21_n78-n257	675

7.108.1	Operating bands for DC	675

7.108.2	Channel bandwidths per operating band for DC	675

7.108.3	Co-existence studies	677

7.108.4	∆TIB and ∆RIB values	677

7.108.5	MSD	677

7.109	DC_3-21_n79-n257	677

7.109.1	Operating bands for DC	677

7.109.2	Channel bandwidths per operating band for DC	677

7.109.3	Co-existence studies	679

7.109.4	∆TIB and ∆RIB values	679

7.109.5	MSD	679

7.110	DC_3-42_n77-n257	679

7.110.1	Operating bands for DC	679

7.110.2	Channel bandwidths per operating band for DC	679

7.110.3	Co-existence studies	682

7.110.4	∆TIB and ∆RIB values	682

7.110.5	MSD	682

7.111	DC_3-42_n78-n257	682

7.111.1	Operating bands for DC	682

7.111.2	Channel bandwidths per operating band for DC	682

7.111.3	Co-existence studies	685

7.111.4	∆TIB and ∆RIB values	685

7.111.5	MSD	685

7.112	DC_3-42_n79-n257	685

7.112.1	Operating bands for DC	685

7.112.2	Channel bandwidths per operating band for DC	685

7.112.3	Co-existence studies	688

7.112.4	∆TIB and ∆RIB values	688

7.112.5	MSD	688

7.113	DC_2-46_n41-n71	688

7.113.1	Operating bands for DC	688

7.113.2	Co-existence studies	688

7.113.3	∆TIB and ∆RIB values	688

7.113.4	MSD	689

7.114	DC_46-66_n25-n41	689

7.114.1	Operating bands for DC	689

7.114.2	Co-existence studies	689

7.114.3	∆TIB and ∆RIB values	689

7.114.4	MSD	690

7.115	DC_46-66_n41-n71	690

7.115.1	Operating bands for DC	690

7.115.2	Co-existence studies	690

7.115.3	∆TIB and ∆RIB values	690

7.115.4	MSD	691

7.126	DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7-7_n1-n257	711

7.126.1	Operating bands for DC	711

7.126.2	Channel bandwidths per operating band for DC	711

7.126.3	Co-existence studies	711

7.126.4	∆TIB and ∆RIB values	711

7.126.5	MSD	712

7.127	DC_1-3_n40-n78	712

7.127.1	Operating bands for DC	712

7.127.2	Co-existence studies	712

7.127.3	∆TIB and ∆RIB values	712

7.127.4	MSD	713

7.128	DC_1-28_n40-n78	713

7.128.1	Operating bands for DC	713

7.128.2	Co-existence studies	713

7.128.3	∆TIB and ∆RIB values	713

7.128.4	MSD	714

7.129	DC_3-28_n40-n78	714

7.129.1	Operating bands for DC	714

7.129.2	Co-existence studies	714

7.129.3	∆TIB and ∆RIB values	714

7.129.4	MSD	715

7.130	DC_1-7_n7-n78	715

7.130.1	Operating bands for DC	715

7.130.2	Channel bandwidths per operating band for DC	715

7.130.3	Co-existence studies	716

7.130.4	∆TIB and ∆RIB values	716

7.130.5	MSD	716

7.131	DC_3-7_n7-n78	716

7.131.1	Operating bands for DC	716

7.131.2	Channel bandwidths per operating band for DC	717

7.131.3	Co-existence studies	717

7.131.4	∆TIB and ∆RIB values	717

7.131.5	MSD	718

7.132	DC_7-28_n7-n78	718

7.132.1	Operating bands for DC	718

7.132.2	Channel bandwidths per operating band for DC	718

7.132.3	Co-existence studies	719

7.132.4	∆TIB and ∆RIB values	719

7.132.5	MSD	719

7.134	DC_3-28_n77-n257	722

7.134.1	Operating bands for DC	722

7.134.2	Channel bandwidths per operating band for DC	723

7.134.3	Co-existence studies	726

7.134.4	∆TIB and ∆RIB values	726

7.134.5	MSD	726

7.136	DC_1-3_n28-n77	728

7.136.1	Operating bands for DC	728

7.136.2	Channel bandwidths per operating band for DC	729

7.136.3	Co-existence studies	729

7.136.4	∆TIB and ∆RIB values	729

7.136.5	MSD	730

7.137	DC_1-8_n28-n77	730

7.137.1	Operating bands for DC	730

7.137.2	Channel bandwidths per operating band for DC	731

7.137.3	Co-existence studies	731

7.137.4	∆TIB and ∆RIB values	731

7.137.5	MSD	732

7.138	DC_3-8_n28-n77	732

7.138.1	Operating bands for DC	732

7.138.2	Channel bandwidths per operating band for DC	733

7.138.3	Co-existence studies	733

7.138.4	∆TIB and ∆RIB values	733

7.138.5	MSD	734

7.139	 DC_3-20_n38-n78	734

7.139.1	 Operating bands for DC	734

7.139.2	Channel bandwidths per operating band for DC	734

7.139.3	Co-existence studies	734

7.139.4	∆TIB and ∆RIB values	735

7.139.5	MSD	735

7.140	DC_2-66_n38-n78	735

7.140.1	Operating bands for DC	735

7.140.2	Channel bandwidths per operating band for DC	735

7.140.3	Co-existence studies	736

7.140.4	∆TIB and ∆RIB values	736

7.140.5	MSD	736

7.141	 DC_1-3_n41-n78	736

7.142	 DC_1-20_n41-n78	737

7.143	 DC_3-20_n41-n78	739

7.144	 DC_1-7_n3-n78	740

7.145	 DC_3-20_n7-n28	741

7.146	DC_1-3_n28-n257	743

7.146.1	Operating bands for DC	743

7.146.2	Channel bandwidths per operating band for DC	744

7.146.3	Co-existence studies	744

7.146.4	∆TIB and ∆RIB values	744

7.146.5	MSD	745

7.147	DC_1-41_n28-n257	745

7.147.1	Operating bands for DC	745

7.147.2	Channel bandwidths per operating band for DC	746

7.147.3	Co-existence studies	748

7.147.4	∆TIB and ∆RIB values	748

7.147.5	MSD	749

7.148	DC_1-41_n77-n257	749

7.148.1	Operating bands for DC	749

7.148.2	Channel bandwidths per operating band for DC	750

7.148.3	Co-existence studies	752

7.148.4	∆TIB and ∆RIB values	752

7.148.5	MSD	753

7.149	DC_3-41_n28-n257	753

7.149.1	Operating bands for DC	753

7.149.2	Channel bandwidths per operating band for DC	754

7.149.3	Co-existence studies	756

7.149.4	∆TIB and ∆RIB values	756

7.149.5	MSD	757

7.150	DC_3-41_n77-n257	757

7.150.1	Operating bands for DC	757

7.150.2	Channel bandwidths per operating band for DC	758

7.150.3	Co-existence studies	760

7.150.4	∆TIB and ∆RIB values	760

7.150.5	MSD	761

7.151	DC_18-41_n3-n257	761

7.151.1	Operating bands for DC	761

7.151.2	Channel bandwidths per operating band for DC	762

7.151.3	Co-existence studies	764

7.151.4	∆TIB and ∆RIB values	764

7.151.5	MSD	765

7.152	 DC_41-42_n77-n257	765

7.152.1	Operating bands for DC	765

7.152.2	Channel bandwidths per operating band for DC	766

7.152.3	Co-existence studies	769

7.152.4	∆TIB and ∆RIB values	770

7.152.5	MSD	770

8	EN-DC band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	771

8.1	DC_1A-3A-5A_n78A-n257	771

8.1.1	Operating bands for DC	771

8.1.2	Channel bandwidths per operating band for DC	771

8.1.3	Co-existence studies	773

8.1.4	∆TIB and ∆RIB values	773

8.1.5	MSD	773

8.2	DC_1A-3A-7A_n78A-n257	774

8.2.1	Operating bands for DC	774

8.2.2	Channel bandwidths per operating band for DC	774

8.2.3	Co-existence studies	776

8.2.4	∆TIB and ∆RIB values	776

8.2.5	MSD	776

8.3	DC_1A-3A-7A-7A_n78A-n257	776

8.3.1	Operating bands for DC	776

8.3.2	Channel bandwidths per operating band for DC	777

8.3.3	Co-existence studies	779

8.3.4	∆TIB and ∆RIB values	779

8.3.5	MSD	779

8.4	DC_1A-5A-7A_n78A-n257	779

8.4.1	Operating bands for DC	779

8.4.2	Channel bandwidths per operating band for DC	780

8.4.3	Co-existence studies	782

8.4.4	∆TIB and ∆RIB values	782

8.4.5	MSD	782

8.5	DC_1A-5A-7A-7A_n78A-n257	782

8.5.1	Operating bands for DC	782

8.5.2	Channel bandwidths per operating band for DC	783

8.5.3	Co-existence studies	785

8.5.4	∆TIB and ∆RIB values	785

8.5.5	MSD	785

8.6	DC_3A-5A-7A_n78A-n257	785

8.6.1	Operating bands for DC	785

8.6.2	Channel bandwidths per operating band for DC	786

8.6.3	Co-existence studies	788

8.6.4	∆TIB and ∆RIB values	788

8.6.5	MSD	788

8.7	DC_3A-5A-7A-7A_n78A-n257	788

8.7.1	Operating bands for DC	788

8.7.2	Channel bandwidths per operating band for DC	789

8.7.3	Co-existence studies	791

8.7.4	∆TIB and ∆RIB values	791

8.7.5	MSD	791

8.8	DC_1-3-7_n28-n78	792

8.8.1	Operating bands for DC	792

8.8.2	Channel bandwidths per operating band for DC	792

8.8.3	Co-existence studies	792

8.8.4	∆TIB and ∆RIB values	793

8.8.5	MSD	793

8.9	DC_1-19-42_n77-n79	793

8.9.1	Operating bands for DC	793

8.9.2	Channel bandwidths per operating band for DC	793

8.9.3	Co-existence studies	794

8.9.4	∆TIB and ∆RIB values	794

8.9.5	MSD	794

8.10	DC_1-19-42_n78-n79	794

8.10.1	Operating bands for DC	794

8.10.2	Channel bandwidths per operating band for DC	795

8.10.3	Co-existence studies	795

8.10.4	∆TIB and ∆RIB values	795

8.10.5	MSD	796

8.11	DC_1-21-42_n77-n79	796

8.11.1	Operating bands for DC	796

8.11.2	Channel bandwidths per operating band for DC	796

8.11.3	Co-existence studies	796

8.11.4	∆TIB and ∆RIB values	796

8.11.5	MSD	797

8.12	DC_1-21-42_n78-n79	797

8.12.1	Operating bands for DC	797

8.12.2	Channel bandwidths per operating band for DC	797

8.12.3	Co-existence studies	798

8.12.4	∆TIB and ∆RIB values	798

8.12.5	MSD	798

8.13	DC_1-3-21_n77-n79	798

8.13.1	Operating bands for DC	798

8.13.2	Channel bandwidths per operating band for DC	799

8.13.3	Co-existence studies	799

8.13.4	∆TIB and ∆RIB values	799

8.13.5	MSD	799

8.14	DC_1-3-21_n78-n79	800

8.14.1	Operating bands for DC	800

8.14.2	Channel bandwidths per operating band for DC	800

8.14.3	Co-existence studies	800

8.14.4	∆TIB and ∆RIB values	800

8.14.5	MSD	801

8.15	DC_19-21-42_n77-n79	801

8.15.1	Operating bands for DC	801

8.15.2	Channel bandwidths per operating band for DC	801

8.15.3	Co-existence studies	802

8.15.4	∆TIB and ∆RIB values	802

8.15.5	MSD	802

8.16	DC_19-21-42_n78-n79	802

8.16.1	Operating bands for DC	802

8.16.2	Channel bandwidths per operating band for DC	803

8.16.3	Co-existence studies	803

8.16.4	∆TIB and ∆RIB values	803

8.16.5	MSD	803

8.17	DC_1-7-28_n5-n78	804

8.17.1	Operating bands for DC	804

8.17.2	Channel bandwidths per operating band for DC	804

8.17.3	Co-existence studies	804

8.17.4	∆TIB and ∆RIB values	804

8.17.5	MSD	805

8.18	DC_3-7-28_n5-n78	805

8.18.1	Operating bands for DC	805

8.18.2	Channel bandwidths per operating band for DC	805

8.18.3	Co-existence studies	806

8.18.4	∆TIB and ∆RIB values	806

8.18.5	MSD	807

8.19	DC_1-3-7_n5-n78	807

8.19.1	Operating bands for DC	807

8.19.2	Channel bandwidths per operating band for DC	807

8.19.3	Co-existence studies	808

8.19.4	∆TIB and ∆RIB values	808

8.19.5	MSD	809

8.20	DC_1-3-28_n5-n78	809

8.20.1	Operating bands for DC	809

8.20.2	Channel bandwidths per operating band for DC	809

8.20.3	Co-existence studies	809

8.20.4	∆TIB and ∆RIB values	810

8.20.5	MSD	810

8.21	DC_1-3-8-n78-n257	810

8.21.1	Operating bands for DC	810

8.21.2	Channel bandwidths per operating band for DC	810

8.21.3	Co-existence studies	813

8.21.4	∆TIB and ∆RIB values	813

8.21.5	MSD	814

8.22	 DC_1-7-20_n3-n78	814

8.22.1	 Operating bands for DC	814

8.22.2	Channel bandwidths per operating band for DC	814

8.22.3	Co-existence studies	814

8.22.4	∆TIB and ∆RIB values	815

8.22.5	MSD	815

8.23	DC_3-7-8_n1-n78, DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78	815

8.23.1	Operating bands for DC	815

8.23.2	Channel bandwidths per operating band for DC	816

8.23.3	Co-existence studies	816

8.23.4	∆TIB and ∆RIB values	817

8.23.5	MSD	817

8.24	DC_1-8-11_n77-n257	818

8.24.1	Operating bands for DC	818

8.24.2	Channel bandwidths per operating band for DC	819

8.24.3	Co-existence studies	821

8.24.4	∆TIB and ∆RIB values	821

8.24.5	MSD	821

8.25	DC_1A-3A-41A_n78A-n257	822

8.25.1	Operating bands for DC	822

8.25.2	Channel bandwidths per operating band for DC	822

8.25.3	Co-existence studies	823

8.25.4	∆TIB and ∆RIB values	823

8.25.5	MSD	824

8.26	DC_1A-3A-42A_n78A-n257	824

8.26.1	Operating bands for DC	824

8.26.2	Channel bandwidths per operating band for DC	824

8.26.3	Co-existence studies	825

8.26.4	∆TIB and ∆RIB values	825

8.26.5	MSD	826

8.27	DC_1A-41A-42A_n78A-n257	826

8.27.1	Operating bands for DC	826

8.27.2	Channel bandwidths per operating band for DC	826

8.27.3	Co-existence studies	827

8.27.4	∆TIB and ∆RIB values	827

8.27.5	MSD	828

8.28	DC_3A-41A-42A_n78A-n257	828

8.28.1	Operating bands for DC	828

8.28.2	Channel bandwidths per operating band for DC	828

8.28.3	Co-existence studies	829

8.28.4	∆TIB and ∆RIB values	829

8.28.5	MSD	830

8.29	DC_1A-3A-18A_n78A-n257	830

8.29.1	Operating bands for DC	830

8.29.2	Channel bandwidths per operating band for DC	830

8.29.3	Co-existence studies	831

8.29.4	∆TIB and ∆RIB values	831

8.29.5	MSD	832

8.30	DC_1A-18A-42A_n78A-n257	832

8.30.1	Operating bands for DC	832

8.30.2	Channel bandwidths per operating band for DC	832

8.30.3	Co-existence studies	833

8.30.4	∆TIB and ∆RIB values	833

8.30.5	MSD	834

8.31	DC_3A-18A-42A_n78A-n257	834

8.31.1	Operating bands for DC	834

8.31.2	Channel bandwidths per operating band for DC	834

8.31.3	Co-existence studies	835

8.31.4	∆TIB and ∆RIB values	835

8.31.5	MSD	836

8.32	DC_3A-28A-41A_n78A-n257	836

8.32.1	Operating bands for DC	836

8.32.2	Channel bandwidths per operating band for DC	836

8.32.3	Co-existence studies	837

8.32.4	∆TIB and ∆RIB values	837

8.32.5	MSD	838

8.33	DC_3A-28A-42A_n78A-n257	838

8.33.1	Operating bands for DC	838

8.33.2	Channel bandwidths per operating band for DC	838

8.33.3	Co-existence studies	839

8.33.4	∆TIB and ∆RIB values	839

8.33.5	MSD	840

8.34	DC_28A-41A-42A_n78A-n257	840

8.34.1	Operating bands for DC	840

8.34.2	Channel bandwidths per operating band for DC	840

8.34.3	Co-existence studies	841

8.34.4	∆TIB and ∆RIB values	841

8.34.5	MSD	842

8.35	DC_1A-3A-28A_n78A-n257	842

8.35.1	Operating bands for DC	842

8.35.2	Channel bandwidths per operating band for DC	842

8.35.3	Co-existence studies	843

8.35.4	∆TIB and ∆RIB values	843

8.35.5	MSD	844

8.36	DC_1A-28A-42A_n78A-n257	844

8.36.1	Operating bands for DC	844

8.36.2	Channel bandwidths per operating band for DC	844

8.36.3	Co-existence studies	845

8.36.4	∆TIB and ∆RIB values	845

8.36.5	MSD	846

8.38	 DC_1-20-38_n3-n78	848

8.38.1	 Operating bands for DC	848

8.38.2	Channel bandwidths per operating band for DC	849

8.38.3	Co-existence studies	849

8.38.4	∆TIB and ∆RIB values	849

8.38.5	MSD	849

8.39	DC_1-3-28_n7-n78	850

8.39.1	Operating bands for DC	850

8.39.2	Channel bandwidths per operating band for DC	850

8.39.3	Co-existence studies	851

8.39.4	∆TIB and ∆RIB values	851

8.39.5	MSD	851

8.40	DC_1-3-21_n78-n257	852

8.40.1	Operating bands for DC	852

8.40.2	Channel bandwidths per operating band for DC	852

8.40.3	Co-existence studies	853

8.40.4	∆TIB and ∆RIB values	853

8.40.5	MSD	854

8.41	DC_1-19-42_n78-n257	854

8.41.1	Operating bands for DC	854

8.41.2	Channel bandwidths per operating band for DC	854

8.41.3	Co-existence studies	856

8.41.4	∆TIB and ∆RIB values	856

8.41.5	MSD	856

8.42	DC_1-21-42_n78-n257	857

8.42.1	Operating bands for DC	857

8.42.2	Channel bandwidths per operating band for DC	857

8.42.3	Co-existence studies	859

8.42.4	∆TIB and ∆RIB values	859

8.42.5	MSD	859

8.43	DC_19-21-42_n78-n257	860

8.43.1	Operating bands for DC	860

8.43.2	Channel bandwidths per operating band for DC	860

8.43.3	Co-existence studies	861

8.43.4	∆TIB and ∆RIB values	861

8.43.5	MSD	862

8.44	DC_1-3-21_n79-n257	862

8.44.1	Operating bands for DC	862

8.44.2	Channel bandwidths per operating band for DC	862

8.44.3	Co-existence studies	863

8.44.4	∆TIB and ∆RIB values	863

8.44.5	MSD	864

8.45	DC_1-19-42_n79-n257	864

8.45.1	Operating bands for DC	864

8.45.2	Channel bandwidths per operating band for DC	864

8.45.3	Co-existence studies	865

8.45.4	∆TIB and ∆RIB values	866

8.45.5	MSD	866

8.46	DC_1-21-42_n79-n257	866

8.46.1	Operating bands for DC	866

8.46.2	Channel bandwidths per operating band for DC	867

8.46.3	Co-existence studies	868

8.46.4	∆TIB and ∆RIB values	868

8.46.5	MSD	869

8.47	DC_19-21-42_n79-n257	869

8.47.1	Operating bands for DC	869

8.47.2	Channel bandwidths per operating band for DC	869

8.47.3	Co-existence studies	870

8.47.4	∆TIB and ∆RIB values	870

8.47.5	MSD	871

8.48	DC_1-3-21_n77-n257	871

8.48.1	Operating bands for DC	871

8.48.2	Channel bandwidths per operating band for DC	871

8.48.3	Co-existence studies	872

8.48.4	∆TIB and ∆RIB values	872

8.48.5	MSD	873

8.49	DC_1-19-42_n77-n257	873

8.49.1	Operating bands for DC	873

8.49.2	Channel bandwidths per operating band for DC	873

8.49.3	Co-existence studies	875

8.49.4	∆TIB and ∆RIB values	875

8.49.5	MSD	875

8.50	DC_1-21-42_n77-n257	876

8.50.1	Operating bands for DC	876

8.50.2	Channel bandwidths per operating band for DC	876

8.50.3	Co-existence studies	877

8.50.4	∆TIB and ∆RIB values	877

8.50.5	MSD	878

8.51	DC_19-21-42_n77-n257	878

8.51.1	Operating bands for DC	878

8.51.2	Channel bandwidths per operating band for DC	878

8.51.3	Co-existence studies	880

8.51.4	∆TIB and ∆RIB values	880

8.51.5	MSD	880

8.52	DC_2-46-66_n41-n71	880

8.52.1	Operating bands for DC	880

8.52.2	Co-existence studies	881

8.52.3	∆TIB and ∆RIB values	881

8.52.4	MSD	881

8.53	DC_1-3-41_n28-n77	881

8.54	DC_1-3-41_n28-n78	883

8.55	DC_1-18-41_n3-n77	885

8.56	DC_1-18-41_n3-n78	886

8.57	DC_1-3-28_n40-n78	888

8.57.1	Operating bands for DC	888

8.57.2	Co-existence studies	888

8.57.3	∆TIB and ∆RIB values	888

8.57.4	MSD	888

8.58	DC_1-3-7_n7-n78	889

8.58.1	Operating bands for DC	889

8.58.2	Channel bandwidths per operating band for DC	889

8.58.3	Co-existence studies	889

8.58.4	∆TIB and ∆RIB values	889

8.58.5	MSD	890

8.59	DC_1-7-28_n7-n78	890

8.59.1	Operating bands for DC	890

8.59.2	Channel bandwidths per operating band for DC	890

8.59.3	Co-existence studies	891

8.59.4	∆TIB and ∆RIB values	891

8.59.5	MSD	891

8.60	DC_3-7-28_n7-n78	891

8.60.1	Operating bands for DC	891

8.60.2	Channel bandwidths per operating band for DC	891

8.60.3	Co-existence studies	892

8.60.4	∆TIB and ∆RIB values	892

8.60.5	MSD	892

8.61	DC_1-3-41_n28-n257	893

8.62	DC_1-18-41_n3-n257	894

8.63	 DC_1-3-20_n38-n78	896

8.63.1	 Operating bands for DC	896

8.63.2	Channel bandwidths per operating band for DC	896

8.63.3	Co-existence studies	897

8.63.4	∆TIB and ∆RIB values	897

8.63.5	MSD	897

8.64	 DC_1-3-20_n41-n78	897

8.65	DC_1-3-41_n77-n257	899

8.65.1	Operating bands for DC	899

8.65.2	Channel bandwidths per operating band for DC	900

8.65.3	Co-existence studies	902

8.65.4	∆TIB and ∆RIB values	903

8.65.5	MSD	903

8.66	DC_1-3-42_n77-n257	903

8.66.1	Operating bands for DC	903

8.66.2	Channel bandwidths per operating band for DC	904

8.66.3	Co-existence studies	905

8.66.4	∆TIB and ∆RIB values	905

8.66.5	MSD	906

8.67	DC_1-41-42_n77-n257	906

8.67.1	Operating bands for DC	906

8.67.2	Channel bandwidths per operating band for DC	907

8.67.3	Co-existence studies	911

8.67.4	∆TIB and ∆RIB values	912

8.67.5	MSD	912

8.68	DC_3-41-42_n77-n257	912

8.68.1	Operating bands for DC	912

8.68.2	Channel bandwidths per operating band for DC	913

8.68.3	Co-existence studies	917

8.68.4	∆TIB and ∆RIB values	918

8.68.5	MSD	918

9	EN-DC band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	919

9.1	DC_1A-3A-5A-7A_n78A-n257	919

9.1.1	Operating bands for DC	919

9.1.2	Channel bandwidths per operating band for DC	919

9.1.3	Co-existence studies	921

9.1.4	∆TIB and ∆RIB values	921

9.1.5	MSD	922

9.2	DC_1A-3A-5A-7A-7A_n78A-n257	922

9.2.1	Operating bands for DC	922

9.2.2	Channel bandwidths per operating band for DC	922

9.2.3	Co-existence studies	924

9.2.4	∆TIB and ∆RIB values	924

9.2.5	MSD	925

9.3	DC_1-3-7-28_n5-n78	925

9.3.1	Operating bands for DC	925

9.3.2	Channel bandwidths per operating band for DC	925

9.3.3	Co-existence studies	926

9.3.4	∆TIB and ∆RIB values	926

9.3.5	MSD	926

9.4	DC_1A-3A-41A-42A_n78A-n257	927

9.4.1	Operating bands for DC	927

9.4.2	Channel bandwidths per operating band for DC	927

9.4.3	Co-existence studies	928

9.4.4	∆TIB and ∆RIB values	928

9.4.5	MSD	929

9.5	DC_1A-3A-18A-42A_n78A-n257	929

9.5.1	Operating bands for DC	929

9.5.2	Channel bandwidths per operating band for DC	929

9.5.3	Co-existence studies	930

9.5.4	∆TIB and ∆RIB values	930

9.5.5	MSD	931

9.6	DC_3A-28A-41A-42A_n78A-n257	931

9.6.1	Operating bands for DC	931

9.6.2	Channel bandwidths per operating band for DC	931

9.6.3	Co-existence studies	932

9.6.4	∆TIB and ∆RIB values	932

9.6.5	MSD	933

9.7	DC_1A-3A-28A-42A_n78A-n257	933

9.7.1	Operating bands for DC	933

9.7.2	Channel bandwidths per operating band for DC	933

9.7.3	Co-existence studies	934

9.7.4	∆TIB and ∆RIB values	934

9.7.5	MSD	935

9.8	DC_1-3-7-28_n7-n78	935

9.8.1	Operating bands for DC	935

9.8.2	Channel bandwidths per operating band for DC	935

9.8.3	Co-existence studies	936

9.8.4	∆TIB and ∆RIB values	936

9.8.5	MSD	936

9.9	DC_1-3-41-42_n77-n257	937

9.9.1	Operating bands for DC	937

9.9.2	Channel bandwidths per operating band for DC	938

9.9.3	Co-existence studies	944

9.9.4	∆TIB and ∆RIB values	944

9.9.5	MSD	945

Annex A: Change history	946

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for E-UTRA-NR Dual connectivity (DC) band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL in Rel-16 as shown in Table 1-1.

Table 1-1: Release 16 EN-DC band combination of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL

| Combination |  |  |
| --- | --- | --- |
| LTE X bands (X=1,2,3,4) |  | NR band combinations (2 Bands) |
| LTE 1 Band | 1 | CA_n1-n78 |
|  | 1 | CA_n3-n28 |
|  | 1 | CA_n3-n78 |
|  | 1 | CA_n5-n78 |
|  | 1 | CA_n7-n78 |
|  | 1 | CA_n8-n40 |
|  | 1 | CA_n8-n78 |
|  | 1 | CA_n28-n40 |
|  | 1 | CA_n28-n77 |
|  | 1 | CA_n28-n257 |
|  | 1 | CA_n40-n78 |
|  | 1 | CA_n41-n78 |
|  | 1 | CA_n77-n78 |
|  | 1 | CA_n3-n257 |
|  | 1 | CA_n75-n78 |
|  | 1 | CA_ n77-n257 |
|  | 1 | CA_ n77-n258 |
|  | 1 | CA_ n78-n257 |
|  | 1 | CA_ n78-n258 |
|  | 1 | CA_ n79-n257 |
|  | 1 | CA_ n79-n258 |
|  | 2 | CA_n7-n66 |
|  | 2 | CA_n7-n78 |
|  | 2 | CA_n41-n66 |
|  | 2 | CA_n41-n71 |
|  | 2 | CA_n66-n71 |
|  | 2 | CA_n66-n78 |
|  | 2 | CA_n5-n258 |
|  | 2 | CA_n5-n260 |
|  | 2 | CA_n5-n261 |
|  | 2 | CA_n12-n258 |
|  | 2 | CA_n12-n260 |
|  | 2 | CA_n12-n261 |
|  | 2 | CA_n38-n66 |
|  | 2 | CA_n38-n78 |
|  | 2 | CA_n41-n260 |
|  | 2 | CA_n41-n261 |
|  | 2 | CA_n71-n258 |
|  | 2 | CA_n71-n260 |
|  | 2 | CA_n71-n261 |
|  | 3 | CA_n1-n7 |
|  | 3 | CA_n1-n28 |
|  | 3 | CA_n1-n40 |
|  | 3 | CA_n1-n77 |
|  | 3 | CA_n1-n78 |
|  | 3 | CA_n1-n79 |
|  | 3 | CA_n3-n77 |
|  | 3 | CA_n3-n78 |
|  | 3 | CA_n5-n78 |
|  | 3 | CA_n7-n28 |
|  | 3 | CA_n7-n78 |
|  | 3 | CA_n8-n40 |
|  | 3 | CA_n8-n78 |
|  | 3 | CA_n20-n78 |
|  | 3 | CA_n28-n40 |
|  | 3 | CA_n28-n77 |
|  | 3 | CA_n28-n78 |
|  | 3 | CA_n28-n257 |
|  | 3 | CA_n34-n258 |
|  | 3 | CA_n40-n41 |
|  | 3 | CA_n40-n258 |
|  | 3 | CA_n40-n78 |
|  | 3 | CA_n40-n79 |
|  | 3 | CA_n41-n78 |
|  | 3 | CA_n41-n79 |
|  | 3 | CA_n41-n258 |
|  | 3 | CA_n75-n78 |
|  | 3 | CA_n77-n78 |
|  | 3 | CA_n1-n257 |
|  | 3-3 | CA_n1-n257 |
|  | 3 | CA_ n77-n257 |
|  | 3 | CA_ n77-n258 |
|  | 3 | CA_ n78-n257 |
|  | 3-3 | CA_n1-n78 |
|  | 3-3 | CA_n7-n78 |
|  | 3-3 | CA_ n78-n257 |
|  | 3 | CA_ n78-n258 |
|  | 3 | CA_n79-n257 |
|  | 3 | CA_ n79-n258 |
|  | 5 | CA_n7-n78 |
|  | 5 | CA_ n78-n257 |
|  | 7 | CA_n1-n3 |
|  | 7 | CA_n1-n40 |
|  | 7 | CA_n1-n78 |
|  | 7 | CA_n3-n78 |
|  | 7 | CA_n5-n78 |
|  | 7 | CA_n7-n78 |
|  | 7 | CA_n8-n40 |
|  | 7 | CA_n8-n78 |
|  | 7 | CA_n28-n40 |
|  | 7 | CA_n28-n78 |
|  | 7 | CA_n66-n78 |
|  | 7 | CA_n1-n257 |
|  | 7-7 | CA_n1-n257 |
|  | 7 | CA_ n78-n257 |
|  | 7-7 | CA_n1-n78 |
|  | 7-7 | CA_n66-n78 |
|  | 7-7 | CA_n78-n257 |
|  | 8 | CA_n1-n78 |
|  | 8 | CA_n3-n28 |
|  | 8 | CA_n8-n78 |
|  | 8 | CA_n28-n77 |
|  | 8 | CA_n34-n258 |
|  | 8 | CA_n40-n41 |
|  | 8 | CA_n40-n79 |
|  | 8 | CA_n40-n258 |
|  | 8 | CA_n41-n79 |
|  | 8 | CA_n41-n258 |
|  | 8 | CA_ n77-n257 |
|  | 8 | CA_ n78-n257 |
|  | 8 | CA_n79-n258 |
|  | 11 | CA_n3-n28 |
|  | 11 | CA_ n77-n257 |
|  | 12 | CA_n7-n66 |
|  | 12 | CA_n7-n78 |
|  | 12 | CA_n66-n78 |
|  | 13 | CA_n5-n260 |
|  | 13 | CA_n5-n48 |
|  | 13 | CA_n7-n78 |
|  | 13 | CA_n48-n66 |
|  | 18 | CA_n3-n77 |
|  | 18 | CA_n3-n78 |
|  | 18 | CA_n3-n257 |
|  | 18 | CA_n78-n257 |
|  | 19 | CA_n77-n78 |
|  | 19 | CA_n77-n257 |
|  | 19 | CA_n78-n257 |
|  | 19 | CA_n79-n257 |
|  | 20 | CA_n1-n3 |
|  | 20 | CA_n1-n7 |
|  | 20 | CA_n1-n28 |
|  | 20 | CA_n1-n78 |
|  | 20 | CA_n3-n78 |
|  | 20 | CA_n7-n28 |
|  | 20 | CA_n7-n78 |
|  | 20 | CA_n20-n75 |
|  | 20 | CA_n20-n78 |
|  | 20 | CA_n41-n78 |
|  | 20 | CA_n75-n78 |
|  | 20 | CA_n78-n92 |
|  | 21 | CA_n77-n78 |
|  | 21 | CA_n77-n257 |
|  | 21 | CA_n78-n257 |
|  | 21 | CA_n79-n257 |
|  | 28 | CA_n3-n77 |
|  | 28 | CA_n3-n78 |
|  | 28 | CA_n5-n78 |
|  | 28 | CA_n7-n78 |
|  | 28 | CA_n8-n78 |
|  | 28 | CA_n28-n78 |
|  | 28 | CA_n3-n257 |
|  | 28 | CA_n8-n258 |
|  | 28 | CA_n40-n78 |
|  | 28 | CA_n77-n257 |
|  | 28 | CA_n78-n257 |
|  | 39 | CA_n40-n41 |
|  | 39 | CA_n40-n79 |
|  | 39 | CA_n40-n258 |
|  | 39 | CA_n41-n79 |
|  | 39 | CA_n41-n258 |
|  | 39 | CA_n79-n258 |
|  | 40 | CA_n41-n79 |
|  | 40 | CA_n41-n258 |
|  | 40 | CA_n79-n258 |
|  | 41 | CA_n3-n77 |
|  | 41 | CA_n3-n78 |
|  | 41 | CA_n3-n257 |
|  | 41 | CA_n28-n77 |
|  | 41 | CA_n28-n78 |
|  | 41 | CA_n28-n257 |
|  | 41 | CA_n77-n257 |
|  | 41 | CA_n78-n257 |
|  | 41 | CA_n79-n258 |
|  | 42 | CA_n28-n77 |
|  | 42 | CA_n77-n257 |
|  | 42 | CA_n78-n257 |
|  | 42 | CA_n79-n257 |
|  | 66 | CA_n5-n48 |
|  | 66 | CA_n7-n78 |
|  | 66 | CA_n25-n41 |
|  | 66 | CA_n25-n71 |
|  | 66 | CA_n38-n66 |
|  | 66 | CA_n38-n78 |
|  | 66 | CA_n41-n71 |
|  | 66 | CA_n66-n78 |
|  | 66 | CA_n5-n258 |
|  | 66 | CA_n5-n260 |
|  | 66 | CA_n5-n261 |
|  | 66 | CA_n12-n258 |
|  | 66 | CA_n12-n260 |
|  | 66 | CA_n12-n261 |
|  | 66 | CA_n41-n260 |
|  | 66 | CA_n41-n261 |
|  | 66 | CA_n71-n258 |
|  | 66 | CA_n71-n260 |
|  | 66 | CA_n71-n261 |
|  | 66-66 | CA_n7-n78 |
| LTE 2 Bands | CA_1A-3A CA_1A-3C | CA_n5-n78 |
|  | CA_1A-3A CA_1A-3C | CA_n7-n78 |
|  | CA_1A-3A | CA_n28-n77 |
|  | CA_1A-3A | CA_n28-n77(2A) |
|  | CA_1A-3A | CA_n28-n78 |
|  | CA_1A-3A | CA_n38-n78 |
|  | CA_1A-3A | CA_n40-n78 |
|  | CA_1A-3A | CA_n41-n78 |
|  | CA_1A-3A | CA_n77-n78 |
|  | CA_1A-3A | CA_n77-n79 |
|  | CA_1A-3A | CA_n78-n79 |
|  | CA_1A-3A | CA_n28-n257 |
|  | CA_1A-3A | CA_n77-n257 |
|  | CA_1A-3A | CA_n78-n257 |
|  | CA_1A-3A | CA_n79-n257 |
|  | CA_1A-5A | CA_n78-n257 |
|  | CA_1A-7A | CA_n3-n78 |
|  | CA_1A-7A CA_1A-7C | CA_n5-n78 |
|  | CA_1A-7A | CA_n7-n78 |
|  | CA_1A-7A CA_1A-7C | CA_n28-n78 |
|  | CA_1A-7A | CA_n78-n257 |
|  | CA_1A-7A-7A | CA_n78-n257 |
|  | CA_1A-8A | CA_n3-n28 |
|  | CA_1A-8A | CA_n28-n77 |
|  | CA_1A-8A | CA_n28-n77(2A) |
|  | CA_1A-8A | CA_n77-n257 |
|  | CA_1A-8A | CA_n78-n257 |
|  | CA_1A-11A | CA_n3-n28 |
|  | CA_1A-11A | CA_n77-n257 |
|  | CA_1A-18A | CA_n3-n77 |
|  | CA_1A-18A | CA_n3-n78 |
|  | CA_1A-18A | CA_n3-n257 |
|  | CA_1A-18A | CA_n78-n257 |
|  | CA_1A-19A | CA_n77-n79 |
|  | CA_1A-19A | CA_n78-n79 |
|  | CA_1A-19A | CA_n77-n257 |
|  | CA_1A-19A | CA_n78-n257 |
|  | CA_1A-19A | CA_n79-n257 |
|  | CA_1A-20A | CA_n3-n38 |
|  | CA_1A-20A | CA_n3-n78 |
|  | CA_1A-20A | CA_n41-n78 |
|  | CA_1A-21A | CA_n77-n78 |
|  | CA_1A-21A | CA_n77-n79 |
|  | CA_1A-21A | CA_n78-n79 |
|  | CA_1A-21A | CA_n77-n257 |
|  | CA_1A-21A | CA_n78-n257 |
|  | CA_1A-21A | CA_n79-n257 |
|  | CA_1A-28A | CA_n3-n77 |
|  | CA_1A-28A | CA_n3-n78 |
|  | CA_1A-28A | CA_n3-n257 |
|  | CA_1A-28A | CA_n5-n78 |
|  | CA_1A-28A | CA_n7-n78 |
|  | CA_1A-28A | CA_n28-n78 |
|  | CA_1A-28A | CA_n40-n78 |
|  | CA_1A-28A | CA_n78-n257 |
|  | CA_1A-41A CA_1A-41C | CA_n3-n77 |
|  | CA_1A-41A CA_1A-41C | CA_n3-n78 |
|  | CA_1A-41A CA_1A-41C | CA_n28-n77 |
|  | CA_1A-41A CA_1A-41C | CA_n28-n78 |
|  | CA_1A-41A CA_1A-41C | CA_n3-n257 |
|  | CA_1A-41A CA_1A-41C | CA_n28-n257 |
|  | CA_1A-41A CA_1A-41C | CA_n77-n257 |
|  | CA_1A-41A CA_1A-41C | CA_n78-n257 |
|  | CA_1A-42A CA_1A-42C | CA_n28-n77 |
|  | CA_1A-42A CA_1A-42C | CA_n28-n77(2A) |
|  | CA_1A-42A | CA_n77-n78 |
|  | CA_1A-42A | CA_n77-n79 |
|  | CA_1A-42A | CA_n78-n79 |
|  | CA_1A-42A CA_1A-42C | CA_n77-n257 |
|  | CA_1A-42A CA_1A-42C | CA_n78-n257 |
|  | CA_1A-42A CA_1A-42C | CA_n79-n257 |
|  | CA_2A-7A CA_2A-7A-7A CA_2A-7C | CA_n38-n66 |
|  | CA_2A-7A CA_2A-7A-7A CA_2A-7C | CA_n38-n78 |
|  | CA_2A-7A CA_2A-7C | CA_n66-n78 |
|  | CA_2A-7A-7A | CA_n66-n78 |
|  | CA_2A-46A CA_2A-46C CA_2A-46D | CA_n41A-n66A |
|  | CA_2A-46A CA_2A-46C CA_2A-46D | CA_n41A-n71A |
|  | CA_2A-46A CA_2A-46C CA_2A-46D | CA_n41(2A)-n71A |
|  | CA_2A-46A CA_2A-46C CA_2A-46D | CA_n66A-n71A |
|  | CA_2A-66A | CA_n38-n78 |
|  | CA_2A-66A | CA_n41-n71 |
|  | CA_2A-66A | CA_n66-n78 |
|  | CA_2A-66A | CA_n41-n260A CA_n41-n260(2A) CA_n41-n260(3A) CA_n41-n260(4A) |
|  | CA_2A-66A | CA_n41-n261 |
|  | CA_2A-66A | CA_n71-n261 |
|  | CA_3A-3A-7A | CA_n78-n257 |
|  | CA_3A-3A-7A-7A | CA_n78-n257 |
|  | CA_3A-3A-8A | CA_n1-n78 |
|  | CA_3A-5A | CA_n78-n257 |
|  | CA_3A-7A CA_3A-7C CA_3C-7C | CA_n28-n78 |
|  | CA_3A-7A CA_3A-7C CA_3C-7A CA_3C-7C CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n1-n78 |
|  | CA_3A-7A CA_3C-7A CA_3A-7C CA_3C-7C | CA_n5-n78 |
|  | CA_3A-7A CA_3A-3A-7A DC_3C-7A | CA_n7-n78 |
|  | CA_3A-7A CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n1-n257 |
|  | CA_3A-7A | CA_n78-n257 |
|  | CA_3A-7A-7A | CA_n78-n257 |
|  | CA_3A-8A | CA_n1-n78 |
|  | CA_3A-8A | CA_n28-n77 |
|  | CA_3A-8A | CA_n28-n77(2A) |
|  | CA_3A-8A | CA_n78-n257 |
|  | CA_3A-18A | CA_n78-n257 |
|  | CA_3A-19A | CA_n77-n78 |
|  | CA_3A-19A | CA_n77-n79 |
|  | CA_3A-19A | CA_n78-n79 |
|  | CA_3A-19A | CA_n77-n257 |
|  | CA_3A-19A | CA_n78-n257 |
|  | CA_3A-19A | CA_n79-n257 |
|  | CA_3A-20A CA_3C-20A | CA_n1-n7 |
|  | CA_3A-20A CA_3C-20A | CA_n1-n28 |
|  | CA_3C-20A | CA_n1-n78 |
|  | CA_3A-20A CA_3C-20A | CA_n7-n28 |
|  | CA_3A-20A CA_3C-20A | CA_n7-n78 |
|  | CA_3C-20A | CA_n28-n78 |
|  | CA_3A-20A | CA_n38-n78 |
|  | CA_3A-20A | CA_n41-n78 |
|  | CA_3A-28A | CA_n28-n78 |
|  | CA_3A-28A | CA_n40-n78 |
|  | CA_3A-21A | CA_n77-n78 |
|  | CA_3A-21A | CA_n77-n79 |
|  | CA_3A-21A | CA_n78-n79 |
|  | CA_3A-21A | CA_n77-n257 |
|  | CA_3A-21A | CA_n78-n257 |
|  | CA_3A-21A | CA_n79-n257 |
|  | CA_3A-28A CA_3C-28A | CA_n5-n78 |
|  | CA_3A-28A CA_3C-28A CA_3A-3A-28A | CA_n7A-n78A CA_n7B-n78A |
|  | CA_3A-28A | CA_n77-n257 |
|  | CA_3A-28A | CA_n78-n257 |
|  | CA_3A-41A CA_3A-41C | CA_n28-n77 |
|  | CA_3A-41A CA_3A-41C | CA_n28-n78 |
|  | CA_3A-41A CA_3A-41C | CA_n28-n257 |
|  | CA_3A-41A CA_3A-41C | CA_n77-n257 |
|  | CA_3A-41A CA_3A-41C | CA_n78-n257 |
|  | CA_3A-42A CA_3A-42C | CA_n28-n77 |
|  | CA_3A-42A CA_3A-42C | CA_n28-n77(2A) |
|  | CA_3A-42A | CA_n77-n78 |
|  | CA_3A-42A | CA_n77-n79 |
|  | CA_3A-42A | CA_n78-n79 |
|  | CA_3A-42A CA_3A-42C | CA_n77-n257 |
|  | CA_3A-42A CA_3A-42C | CA_n78-n257 |
|  | CA_3A-42A CA_3A-42C | CA_n79-n257 |
|  | CA_5A-7A | CA_n78-n257 |
|  | CA_5A-7A-7A | CA_n78-n257 |
|  | CA_7A-7A-8A | CA_n1-n78 |
|  | CA_7A-8A | CA_n1-n78 |
|  | CA_7A-20A | CA_n3-n78 |
|  | CA_7A-28A CA_7C-28A | CA_n3-n78 |
|  | CA_7A-28A CA_7C-28A | CA_n5-n78 |
|  | CA_7A-28A | CA_n7-n78 |
|  | CA_7A-28A | CA_n28-n78 |
|  | CA_7A-66A CA_7A-7A-66A CA_7C-66A | CA_n38-n78 |
|  | CA_7A-66A CA_7C-66A | CA_n66-n78 |
|  | CA_7A-7A-66A | CA_n66-n78 |
|  | CA_8A-11A | CA_n77-n257 |
|  | CA_8A-42A CA_8A-42C | CA_n28A-n77A |
|  | CA_8A-42A CA_8A-42C | CA_n28A-n77(2A) |
|  | CA_13A-66A | CA_n5A-n48A |
|  | CA_18A-41A CA_18A-41C | CA_n3-n77 |
|  | CA_18A-41A CA_18A-41C | CA_n3-n78 |
|  | CA_18A-41A CA_18A-41C | CA_n3-n257 |
|  | CA_18A-42A CA_18A-42C | CA_n78-n257 |
|  | CA_19A-21A | CA_n77-n78 |
|  | CA_19A-21A | CA_n77-n79 |
|  | CA_19A-21A | CA_n78-n79 |
|  | CA_19A-21A | CA_n77-n257 |
|  | CA_19A-21A | CA_n78-n257 |
|  | CA_19A-21A | CA_n79-n257 |
|  | CA_19A-42A | CA_n77-n78 |
|  | CA_19A-42A | CA_n77-n79 |
|  | CA_19A-42A | CA_n78-n79 |
|  | CA_19A-42A CA_19A-42C | CA_n77-n257 |
|  | CA_19A-42A CA_19A-42C | CA_n78-n257 |
|  | CA_19A-42A CA_19A-42C | CA_n79-n257 |
|  | CA_21A-42A | CA_n77-n78 |
|  | CA_21A-42A | CA_n77-n79 |
|  | CA_21A-42A | CA_n78-n79 |
|  | CA_21A-42A CA_21A-42C | CA_n77-n257 |
|  | CA_21A-42A CA_21A-42C | CA_n78-n257 |
|  | CA_21A-42A CA_21A-42C | CA_n79-n257 |
|  | CA_28A-41A CA_28A-41C | CA_n78-n257 |
|  | CA_28A-42A CA_28A-42C | CA_n78-n257 |
|  | CA_41A-42A CA_41A-42C CA_41C-42A CA_41C-42C | CA_n77-n257 |
|  | CA_41A-42A CA_41A-42C CA_41C-42A CA_41C-42C | CA_n78-n257 |
|  | CA_46A-66A CA_46C-66A CA_46D-66A | CA_n25A-n41A |
|  | CA_46A-66A CA_46C-66A CA_46D-66A | CA_n25A-n71A |
|  | CA_46A-66A CA_46C-66A CA_46D-66A | CA_n41A-n71A |
|  | CA_46A-66A CA_46C-66A CA_46D-66A | CA_n41(2A)-n71A |
| LTE 3 Bands | CA_1A-3A-5A | CA_n78-n257 |
|  | CA_1A-3A-7A CA_1A-3C-7A CA_1A-3A-7C CA_1A-3C-7C | CA_n5-n78 |
|  | CA_1A-3A-7A CA_1A-3A-7C | CA_n28-n78 |
|  | CA_1A-3A-7A | CA_n78-n257 |
|  | CA_1A-3A-7A-7A | CA_n78-n257 |
|  | CA_1A-3A-8A | CA_n28-n77 |
|  | CA_1A-3A-8A | CA_n28-n77(2A) |
|  | CA_1A-3A-8A CA_1A-3C-8A | CA_n78-n257 |
|  | CA_1A-3A-18A | CA_n78-n257 |
|  | CA_1A-3A-20A | CA_n38-n78 |
|  | CA_1A-3A-20A | CA_n41-n78 |
|  | CA_1A-3A-21A | CA_n77-n78 |
|  | CA_1A-3A-21A | CA_n77-n79 |
|  | CA_1A-3A-21A | CA_n78-n79 |
|  | CA_1A-3A-21A | CA_n77-n257 |
|  | CA_1A-3A-21A | CA_n78-n257 |
|  | CA_1A-3A-21A | CA_n79-n257 |
|  | CA_1A-3A-28A, CA_1A-3C-28A | CA_n5-n78 |
|  | CA_1A-3A-28A, CA_1A-3C-28A | CA_n7-n78 |
|  | CA_1A-3A-28A, CA_1A-3C-28A | CA_n28-n78 |
|  | CA_1A-3A-28A | CA_n40-n78 |
|  | CA_1A-3A-28A | CA_n78-n257 |
|  | CA_1A-3A-41A CA_1A-3A-41C | CA_n28-n77 |
|  | CA_1A-3A-41A CA_1A-3A-41C | CA_n28-n78 |
|  | CA_1A-3A-41A CA_1A-3A-41C | CA_n28-n257 |
|  | CA_1A-3A-41A CA_1A-3A-41C | CA_n77-n257 |
|  | CA_1A-3A-41A CA_1A-3A-41C | CA_n78-n257 |
|  | CA_1A-3A-42A CA_1A-3A-42C | CA_n77-n257 |
|  | CA_1A-3A-42A CA_1A-3A-42C | CA_n78-n257 |
|  | CA_1A-5A-7A | CA_n78-n257 |
|  | CA_1A-5A-7A-7A | CA_n78-n257 |
|  | CA_1A-7A-20A | CA_n3-n78 |
|  | CA_1A-7A-28A, CA_1A-7C-28A | CA_n5-n78 |
|  | CA_1A-7A-28A | CA_n7-n78 |
|  | CA_1A-7A-28A, CA_1A-7C-28A | CA_n28-n78 |
|  | CA_1A-8A-11A | CA_n3-n28 |
|  | CA_1A-8A-11A | CA_n77-n257 |
|  | CA_1A-8A-42A CA_1A-8A-42C | CA_n28-n77 |
|  | CA_1A-8A-42A CA_1A-8A-42C | CA_n28-n77(2A) |
|  | CA_1A-18A-41A CA_1A-18A-41C | CA_n3-n77 |
|  | CA_1A-18A-41A CA_1A-18A-41C | CA_n3-n78 |
|  | CA_1A-18A-41A CA_1A-18A-41C | CA_n3-n257 |
|  | CA_1A-18A-42A CA_1A-18A-42C | CA_n78-n257 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n77-n79 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n78-n79 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n77-n257 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n78-n257 |
|  | CA_1A-19A-42A CA_1A-19A-42C | CA_n79-n257 |
|  | CA_1A-20A-38A | CA_n3-n78 |
|  | CA_1A-21A-42A | CA_n77-n78 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n77-n79 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n78-n79 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n77-n257 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n78-n257 |
|  | CA_1A-21A-42A CA_1A-21A-42C | CA_n79-n257 |
|  | CA_1A-28A-42A CA_1A-28A-42C | CA_n78-n257 |
|  | CA_1A-41A-42A CA_1A-41A-42C CA_1A-41C-42A CA_1A-41C-42C | CA_n77-n257 |
|  | CA_1A-41A-42A CA_1A-41A-42C CA_1A-41C-42A CA_1A-41C-42C | CA_n78-n257 |
|  | CA_2A-7A-66A | CA_n66-n78 |
|  | CA_2A-7C-66A | CA_n66-n78 |
|  | CA_2A-7A-7A-66A | CA_n66-n78 |
|  | CA_2A-46A-66A CA_2A-46C-66A CA_2A-46D-66A | CA_n41-n71 |
|  | CA_3A-3A-7A-8A CA_3A-7A-7A-8A CA_3A-3A-7A-7A-8A | CA_n1-n78 |
|  | CA_3A-5A-7A | CA_n78-n257 |
|  | CA_3A-5A-7A-7A | CA_n78-n257 |
|  | CA_3A-7A-8A | CA_n1-n78 |
|  | CA_3A-7A-28A, CA_3A-7C-28A, CA_3C-7A-28A, CA_3C-7C-28A, | CA_n5-n78 |
|  | CA_3A-7A-28A, CA_3C-7A-28A, | CA_n7-n78 |
|  | CA_3A-7A-28A, CA_3A-7C-28A, CA_3C-7A-28A, CA_3C-7C-28A, | CA_n28-n78 |
|  | CA_3A-18A-42A CA_3A-18A-42C | CA_n78-n257 |
|  | CA_3A-28A-41A CA_3A-28A-41C | CA_n78-n257 |
|  | CA_3A-28A-42A CA_3A-28A-42C | CA_n78-n257 |
|  | CA_3A-41A-42A CA_3A-41A-42C CA_3A-41C-42A CA_3A-41C-42C | CA_n77-n257 |
|  | CA_3A-41A-42A CA_3A-41A-42C CA_3A-41C-42A CA_3A-41C-42C | CA_n78-n257 |
|  | CA_19A-21A-42A | CA_n77-n78 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n77-n79 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n78-n79 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n77-n257 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n78-n257 |
|  | CA_19A-21A-42A CA_19A-21A-42C | CA_n79-n257 |
|  | CA_28A-41A-42A CA_28A-41A-42C CA_28A-41C-42A CA_28A-41C-42C | CA_n78-n257 |
| LTE 4 Bands | CA_1A-3A-5A-7A | CA_n78-n257 |
|  | CA_1A-3A-5A-7A-7A | CA_n78-n257 |
|  | CA_1A-3A-7A-28A, CA_1A-3C-7A-28A, CA_1A-3A-7C-28A, CA_1A-3C-7C-28A | CA_n5-n78 |
|  | CA_1A-3A-7A-28A, CA_1A-3C-7A-28A, | CA_n7-n78 |
|  | CA_1A-3A-7A-28A, CA_1A-3C-7A-28A, CA_1A-3A-7C-28A, CA_1A-3C-7C-28A | CA_n28-n78 |
|  | CA_1A-3A-18A-42A CA_1A-3A-18A-42C | CA_n78-n257 |
|  | CA_1A-3A-28A-42A CA_1A-3A-28A-42C | CA_n78-n257 |
|  | CA_1A-3A-41A-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42A CA_1A-3A-41C-42C | CA_n77-n257 |
|  | CA_1A-3A-41A-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42A CA_1A-3A-41C-42C | CA_n78-n257 |
|  | CA_3A-28A-41A-42A CA_3A-28A-41A-42C CA_3A-28A-41C-42A CA_3A-28A-41C-42C | CA_n78-n257 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

4	Background

The present document is a technical report for EN-DC band combinations of LTE x bands (x=1,2,3,4) inter-band CA (x DL/1UL) + inter-band NR 2 bands (2DL/1UL) under Rel-16 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	EN-DC band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part

5.1	UE RF architectures

RAN4 consider same example RF architecture which are described in section 5.1 in TR37.863-01-01.  One different point is that LTE and NR modemchip can be used one chip solutions from rel-16.

To support multiple component carriers in LTE and NR, RAN4 consider specific RF components such as Diplexer, Triplexer, Quadplexer and Hexaplexer to derive self-interference problems according to the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL EN-DC band combinations.

5.2	General treatment of ∆TIB and ∆RIB values

For the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL EN-DC band combinations, RAN4 should consider to reuse agreed additional insertion losses for all EN-DC band combinations in rel-15 when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detail decription will be captured in some specific EN-DC band combinations.

5.3	Summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL EN-DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1 band & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration |  | Harmonic relation issues | intermodulation to own rx band |  | interference due to small frequency separation | MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1_n1-n78 | DC_1A_n78A |  |  | 4th IMD into n1 |  |  | - The 4th IMD already covered in 2DL/2UL DC_1A_n78A |  |
| DC_1_n3-n78 | DC_1A_n3A |  | 2nd harmonic from n3 into n78 | 2nd & 4th IMDs into n78 |  | Yes | - Harmonic problems already covered in DC_n3A-n78A. - 28.4 dB by 2nd IMD in rel-15 TR - 11.2 dB by 4th IMD in Rel-15 TR. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
|  | DC_1A_n78A |  | - | 2nd IMD into n3 |  | - | - 27.9 dB by 2nd IMD in rel-15 TR. |  |
| DC_1_n7-n78 | DC_1A_n7A DC_1A_n7B |  | - | 4th & 5th IMDs into n78 |  | - | - 10.1 dB by 4th IMD problem. - 3.8 dB by 5th IMD problem That were already covered in DC_7_n1-n78 in rel-15 TR |  |
|  | DC_1A_n78A, DC_1A_n78(2A) |  | - | 4th IMD into B7 |  | - | -9.1dB by 4th IMD problem that was already covered in DC_1-7_n78 in rel-15 TR |  |
| DC_1_n8-n78 | DC_1_n8 |  | 4th harmonic from n8 into n78 | 3rd IMD into n78 |  | - | 3rd IMD already covered in DC_1A-8A_n78A |  |
|  | DC_1_n78 |  | - | 5th IMD into n8 |  | - | 5th IMD already covered in DC_1A-8A_n78A |  |
| DC_1_n78-n257 | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I |  | 2nd harmonic from B1 into n78 | - |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
| DC_3_n1-n77 | DC_3A_n1A |  | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | Yes | - Harmonic problems will be covered in DC_3A_n77A. - 2nd & 4th IMDs problems was same as DC_3A_n1A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
|  | DC_3A_n77A |  | - | 2nd & 5th IMD into n1 |  | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. |  |
| DC_3_n1-n78 | DC_3A_n1A |  | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 |  | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
|  | DC_3A_n78A |  | - | 5th IMD into n1 |  | - | - 3.5 dB by 5th IMD in rel-15 TR |  |
| DC_3_n3-n77 | DC_3A_n3A1 |  | 2nd harmonic into n77 | - |  | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A |  |
|  | DC_3A_n77A |  | 2nd harmonic into n77 | - |  | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n77A In rel-15 |  |
| DC_3_n3-n78 | DC_3A_n3A1 |  | 2nd harmonic into n78 | 2nd , 3rd ,4th & 5th IMDs into n3 |  | - | - This harmonic & IMD problems are already covered in 2DL/2UL DC_3A-n78A |  |
|  | DC_3A_n78A |  | 2nd harmonic into n78 | - |  | - | - This harmonic impact already covered in 2DL/2UL DC_3A-n78A In rel-15. |  |
| DC_3_n7-n78 | DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B |  | 2nd harmonic from 3 into n78 | 3rd IMD into n78 |  | - | - Harmonic problem already solved in DC_3A_n78A - 16.1dB by 3rd IMD problem that already covered in DC_7A_n3A-n78A in rel-15 TR. |  |
|  | DC_3A_n78A, DC_3A_n78(2A) DC_3C_n78A |  | - | - |  | - | No issues |  |
| DC_3-3_n7-n78 | DC_3A_n7A DC_3A_n7B |  | 2nd harmonic from 3 into n78 | 3rd IMD into n78 |  | - | - Harmonic problem already solved in DC_3A_n78A - 16.1dB by 3rd IMD problem that already covered in DC_7A_n3A-n78A in rel-15 TR. |  |
|  | DC_3A_n78A |  | - | - |  | - | No issues |  |
| DC_3_n20-n78 | DC_3A_n20A |  | 2nd & 4th harmonics into n78 | 3rd & 5th IMDs into n78 |  | - | These harmonic issues are already covered in DC_3A_n78A and DC_20A_n78A - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |  |
|  | DC_3A_n78A |  | - | - |  | - | No issues |  |
| DC_3_n28-n78 | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 | 5th IMD into n78 |  | - | - Harmonic problem already discussed and will be solved in DC_3A_n78A or DC_28A_n78A - 4.5dB by 5th IMD problem in rel-15 TR |  |
|  | DC_3A_n78A |  | 2nd harmonic into n78 | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 |  | - | - 2nd harmonic problem already discussed and will be solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already discussed and will be solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |  |
| DC_3_n78-n257 DC_3-3_n78-n257 | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I |  | 2nd harmonic from B3 into n78 | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
| DC_5_n78-n257 | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_5A_n257A |  | 4th harmonic from B5 into n78 | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_7_n1-n3 | DC_7A_n1A |  |  |  |  | Yes | No issues |  |
|  | DC_7A_n3A |  |  |  |  | - | No issues |  |
| DC_7_n1-n78 | DC_7A_n1A, DC_7C_n1A |  |  | 4th & 5th IMDs into n78 |  | - | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. |  |
|  | DC_7A_n78A, DC_7C_n78A |  |  | 4th IMD into n1 |  | - | - 9.0 dB by 4th IMD in rel-15 TR. |  |
| DC_7_n3-n78 | DC_7A_n3A |  | 2nd harmonic from n3 into n78 | 3rd IMD into n78 |  | - | - Harmonic problem already solved in CA_n3A-n78A - 16.1 dB by 3rd IMD in rel-15 TR |  |
|  | DC_7A_n78A |  | - | 3rd & 4th IMDs into n3 |  | - | - 15.6 dB by 3rd IMD in rel-15 TR - 9.2 dB by 4th IMD in rel-15 TR. |  |
| DC_7_n28-n78 | DC_7A_n28A |  | 5th harmonic into n78 | 2nd,4th IMDs into n78 |  | - | - 5th harmonic problem will be solved in DC_28A_n78A - 29.7 dB by 2nd IMD in rel-15 TR - 10.3 dB by 4th IMD problem in rel-15 TR |  |
|  | DC_7A_n78A |  | - | 2nd & 5th IMDs into B28 4th IMD into n78 |  | - | - 28.8 dB by 2nd IMD in rel-15 TR - 4.0 dB by 5th IMD problems in rel-15 TR - 4th IMD problem is no issue for TDD band |  |
| DC_7-7_n78-n257 | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  | DC_7A_n257A |  | - | - |  | - | No issues |  |
| DC_7_n78-n257 | DC_7A_n78A |  | 7th & 8th harmonics from n78 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_7A_n257A |  | - | - |  | - | No issues |  |
| DC_8_n8-n78 | DC_8A_n78A |  | - | 4th IMD into n8 |  | - | The 4th IMD issue already covered in DC_8A-n78A |  |
| DC_8_n78-n257 | DC_8A_n78A |  | 7th & 8th harmonics from n78 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  | DC_8A_n257A |  | 4th harmonic from B8 into n78 | - |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_13_n5-n260 | DC_13A_n5A |  | - | - |  | - | No issue |  |
|  | DC_13A_n260A |  | - | - |  | - | No issue |  |
| DC_20_n1-n3 | DC_20A_n1A |  |  |  |  | - | No issues |  |
|  | DC_20A_n3A |  |  |  |  | - | No issues |  |
| DC_20_n1-n78 | DC_20A_n1A |  | 4th harmonic from B20 into n78 | 3rd IMD into n78 |  | - | - Harmonic problem already solved in DC_20A_n78A - 16.0 dB by 3rd IMD in rel-15 TR |  |
|  | DC_20A_n78A |  | - | 3rd IMD into n1 |  | - | - 15.3 dB by 3rd IMD in rel-15 TR |  |
| DC_20_n3-n78 | DC_20A_n3A |  | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 |  | - | - 16.1 dB by 3rd IMD in rel-15 TR - 4.5 dB by 5th IMD in rel-15 TR |  |
|  | DC_20A_n78A |  | - | 3rd IMD into n3 |  | - | - 15.7 dB by 3rd IMD in rel-15 TR |  |
| DC_20_n20-n78 | DC_20A_n78A |  | - | 4th IMD into n20 |  | - | This IMD impact already covered in 2DL/2UL DC_20A-n78A |  |
| DC_28_n8-n78 | DC_28A_n8A |  | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 |  | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - 9.1 dB by 4th IMD in rel-15 TR - 3.7 dB by 5th IMD in rel-15 TR |  |
|  | DC_28A_n78A |  | 5th harmonic into n78 | 5th IMD into B28 5th IMD into n8 |  | - | - 5th harmonic problem will be solved in DC_28A-n78A - 5th IMD problem already discussed and will be solved in DC_28A-n78A - 4.3 dB by 5th IMD in rel-15 TR |  |
| DC_28_n8-n258 | DC_28A_n8A |  | 33th harmonics into n258 27th harmonics into n258 | - |  | - | - No harmonic problems by 27th & 33th |  |
|  | DC_28A_n258A |  | - | - |  | - | - No issue |  |
| DC_28_n28-n78 | DC_28A_n78A |  | - | 5th IMD into n28 |  | - | This IMD impact already covered in 2DL/2UL DC_28A-n78A |  |
| DC_66_n5-n260 | DC_66A_n5A |  | 22th harmonics into n260 | - |  | - | Up to 7th harmonic order will be not any impact to n260. |  |
|  | DC_66A_n260A |  | - | - |  | - | No issues |  |
| DC_66_n71-n260 DC_66_n71-n260(2A) |  | DC_66A_n71A | 22th harmonics into n260 | - |  | - | Up to 7th harmonic order will be not any impact to n257. |  |
|  |  | DC_66A_n260A | - | - |  | - | No issues |  |
| DC_66_n71-n261 DC_66_n71-n261(2A) |  | DC_66A_n71A | 16th harmonics into n261 | - |  | - | Up to 7th harmonic order will be not any impact to n261. |  |
|  |  | DC_66A_n261A | - | - |  | - | No issues |  |
| DC_1_n77-n78 |  | DC_1A_n77A |  | 4th IMD & 5th IMD |  | - | These MSD issues are same in DC_1A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n77A |  |
|  |  | DC_1A_n78A | 2nd harmonic | 4th IMD & 5th IMD |  |  | 2nd harmonic issue already specified in TS38.101-1 for DC_1A_n77A These MSD issues are same in DC_1A_n78A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_1A_n78A |  |
| DC_3_n77-n78 |  | DC_3A_n77A | 2nd harmonic | 4th IMD & 5th IMD |  |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n78A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n77A |  |
|  |  | DC_3A_n78A | 2nd harmonic | 4th IMD & 5th IMD |  |  | 2nd harmonic issue already specified in TS38.101-1 for DC_3A_n77A These MSD issues are same in DC_3A_n77A. So the MSD level by 4th IMD can follow 2DL/2UL_DC_3A_n78A |  |
| DC_19_n77-n78 |  | DC_19A_n77A | 4th harmonic | 2nd IMD |  |  | 4th harmonic & 2nd IMD will be impact to n78 band. However, consider specific band for n77 in Japan, there was no IMD problem |  |
|  |  | DC_19A_n78A | 4th & 5th harmonic | 2nd IMD |  |  | 5th IMD already specified for DC_19A_n77A in TS38.101-3 Consider specific band for n78 in Japan, there was no IMD problem |  |
| DC_21_n77-n78 |  | DC_21A_n77A |  | 4th IMD & 5th IMD |  | - | These MSD issues are same in DC_21A_n77A. However, RAN4 consider the specific band for n77 in Japan, there was no IMD problem |  |
|  |  | DC_21A_n78A |  | 4th IMD |  |  | These MSD issue is same in DC_21A_n78A. However, RAN4 consider the specific band for n78 in Japan, there was no IMD problem |  |
| DC_1_n40-n78 |  | DC_1A_n40A |  | 4th IMD & 5th IMD |  | - | - 9.8dB by 4th IMD - 4.2dB by 5th IMD |  |
|  |  | DC_1A_n78A |  | 4th IMD |  | - | - 10.6dB by 4th IMD |  |
| DC_3_n1-n79 |  | DC_3A_n1A |  | 5th IMD |  | Yes | - 4.7dB by 5th IMD |  |
|  |  | DC_3A_n79A |  | 5th IMD |  | - | - 3.6dB by 5th IMD |  |
| DC_3_n40-n78 |  | DC_3A_n40A | 2nd harmonic | 5th IMD |  | - | Harmonic issue already covered in DC_3A_n78A - 4.8dB by 5th IMD |  |
|  |  | DC_3A_n78A |  | 5th IMD |  | - | - 4.4dB by 5th IMD |  |
| DC_66_n25-n41 |  | DC_66A_n25A | - | - |  | - | No issue |  |
|  |  | DC_66A_n41A |  | 4th IMD |  | - | - 11.0dB by 4th IMD |  |
| DC_1_n77-n257 DC_1_n77(2A)-n257 DC_1_n77-n257I |  | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I | 2nd harmonic from B1 into n77 | - |  | - | - Harmonic problem already covered in DC_1A_n77A |  |
| DC_1_n77-n258 |  | DC_1A_n77A | 6th, 7th & 8th Harmonics from n77 into n258 | - |  | - | - No harmonic problems by 6th, 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_1A_n258A | 2nd harmonic from B1 into n77 | - |  | - | - Harmonic problem already covered in DC_1A_n77A |  |
| DC_1_n78-n258 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n258 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_1A_n258A | - | - |  | - | No issue |  |
| DC_1_n79-n258 |  | DC_1A_n79A | 5th & 6th Harmonics from n79 into n258 | - |  | - | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |  |
|  |  | DC_1A_n258A | - | - |  | - | No issue |  |
| DC_3_n1-n257 DC_3-3_n1-n257 |  | DC_3A_n1A | - | - |  | Yes | No issue |  |
|  |  | DC_3A_n257A | - | - |  | - | No issue |  |
| DC_3_n77-n258 |  | DC_3A_n77A | 6th, 7th & 8th harmonics from n77 into n258 | - |  | - | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |  |
|  |  | DC_3A_n258A | 2nd harmonic from B3 into B77 | - |  | - | - Harmonic problem already covered in DC_3A_n77A |  |
| DC_3_n78-n258 |  | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - |  | - | - Harmonic problem already covered in DC_3A_n78A |  |
| DC_3_n79-n258 |  | DC_3A_n79A | 5th & 6th Harmonics from n79 into n258 | - |  | - | - Harmonic problem will be solved in DC_n79A-n258A (5th order) |  |
|  |  | DC_3A_n258A | - | - |  | - | No issue |  |
| DC_8_n77-n257 DC_8_n77(2A)-n257 |  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I | 4th harmonic from B8 into n77 | - |  | - | - Harmonic problem already covered in DC_8A_n77A |  |
| DC_1_n5-n78 |  | DC_1A_n5A | - | 3rd IMD |  | - | - 18.1dB by 3rd IMD |  |
|  |  | DC_1A_n78A | - | 5th IMD |  | - | Already specified in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_3_n5-n78 |  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 3rd & 5th IMDs |  | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.1dB by 3rd IMD |  |
|  |  | DC_3A_n78A, DC_3C_n78A | - | - |  | - | No issue |  |
| DC_7_n5-n78 |  | DC_7A_n5A, DC_7C_n5A | - | 2nd & 4th IMDs |  | - | - 29.7dB by 2nd IMD |  |
|  |  | DC_7A_n78A, DC_7C_n78A | - | 2nd & 5th IMDs |  | - | Already specified these IMDs problems for DC_5-7_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_28_n5-n78 |  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |  |
|  |  | DC_28A_n78A | - | 5th IMD |  | - | - 3.8dB by 5th IMD |  |
| DC_39_n41-n79 |  | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs |  | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - 29.8dB by 2nd IMD - 4.4dB by 5th IMD |  |
|  |  | DC_39A_n79A | - | 2nd & 5th IMDs |  | - | - 30.2dB by 2nd IMD - 4.3dB by 5th IMD |  |
| DC_40_n41-n79 |  | DC_40A_n41A | 2nd harmonic from n40 into n79 2nd harmonic from n41 into n79 | 2nd & 4th IMDs |  | - | - FFS by 2nd harmonic from B40 - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - 30.5dB by 2nd IMD - No impact to china band since over 4800MHz is considered in NR |  |
|  |  | DC_40A_n79A | - | 2nd , 4th & 5th IMDs |  | - | Band 40 and n41 should be operate the TDD synchronous operation No MSD into n41 |  |
| DC_2_n7-n66 |  | DC_2A_n7A | - | - |  | - | No issue |  |
|  |  | DC_2A_n66A | - | - |  | - | No issue |  |
| DC_12_n7-n66 |  | DC_12A_n7A | 3rd harmonic from B12 into n66 | - |  | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 |  |
|  |  | DC_12A_n66A | - | - |  | - | No issue |  |
| DC_5_n7-n78 |  | DC_5A_n7A | - | 2nd & 4th IMDs |  | - | - These MSD issue follow DC_7_n5-n78 |  |
|  |  | DC_5A_n78A | - | 2nd IMD |  | Yes | - 2nd IMD problems was covered in DC_5A-7A_n78A. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_7_n7-n78 |  | DC_7A_n7A2 | - | - |  | - | No issue |  |
|  |  | DC_7A_n78A | - | - |  | Yes | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_12_n7-n78 |  | DC_12A_n7A | 5th harmonic from B12 into n78 | 4th IMD |  |  | - 5th harmonic issue will be covered in DC_12A_n78A - 8.4dB by 4th IMD |  |
|  |  | DC_12A_n78A | - | 2nd IMD |  | Yes | - 28.7dB by 2nd IMD - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_66_n7-n78 |  | DC_66A_n7A | 2nd harmonic from B66 into n78 | 3rd IMD |  | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 -reuse MSD of DC_3A_n7-n78 for 3rd IMD problem |  |
|  |  | DC_66A_n78A | - | - |  | Yes | - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_2_n66-n78 |  | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 2nd & 4th IMDs |  | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 29.4dB by 2nd IMD - 8.9 dB by 4th IMD |  |
|  |  | DC_2A_n78A | - | 4th IMD |  | - | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_12_n66-n78 |  | DC_12A_n66A | 2nd harmonic from n66 inton78 5th harmonic from B12 into n78 | 5th IMD |  |  | - 2nd harmonic is same as DC_66A_n78A and 5th harmonic will be covered in DC_12A_n78A - 4.1dB by 5th IMD |  |
|  |  | DC_12A_n78A | 3rd harmonic from B12 into n66 | 3rd IMD |  |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 16.5dB by 3rd IMD |  |
| DC_3-3_n1_n78 |  | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs |  | - | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |  |
|  |  | DC_3A_n78A | - | 5th IMD |  | Yes | - 3.5 dB by 5th IMD in rel-15 TR - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |  |
| DC_7-7_n1_n78 |  | DC_7A_n1A |  | 4th & 5th IMDs |  | Yes | - 10.1 dB by 4th IMD in rel-15 TR. - 3.8 dB by 5th IMD in rel-15 TR. - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
|  |  | DC_7A_n78A |  | 4th IMD |  | - | - 9.0 dB by 4th IMD in rel-15 TR. |  |
| DC_2_n41-n260 |  | DC_2A_n41A | High order harmonic into n260 | - |  | - | - No harmonic problem |  |
| DC_2_n41-n261 |  | DC_2A_n41A | High order harmonic into n261 | - |  | - | - No harmonic problem |  |
| DC_66_n41_n260 |  | DC_66A_n41A | High order harmonic into n260 | - |  | - | - No harmonic problem |  |
| DC_66_n41_n261 |  | DC_66A_n41A | High order harmonic into n261 | - |  | - | - No harmonic problem |  |
| DC_2_n5-n258 |  | DC_2A_n5A | High order harmonic into n258 | - |  | - | - No harmonic problem |  |
|  |  | DC_2A_n258A | - | - |  | - | No issue |  |
| DC_2_n5-n260 |  | DC_2A_n5A | High order harmonic into n260 | - |  | - | - No harmonic problem |  |
|  |  | DC_2A_n260A | - | - |  | - | No issue |  |
| DC_2_n5-n261 |  | DC_2A_n5A | High order harmonic into n261 | - |  | - | - No harmonic problem |  |
|  |  | DC_2A_n261A | - | - |  | - | No issue |  |
| DC_2_n71-n258 |  | DC_2A_n71A | High order harmonic into n258 | - |  | - | - No harmonic problem |  |
|  |  | DC_2A_n258A | - | - |  | - | No issue |  |
| DC_2_n71-n260 |  | DC_2A_n71A | High order harmonic into n260 | - |  | - | - No harmonic problem |  |
|  |  | DC_2A_n260A | - | - |  | - | No issue |  |
| DC_2_n71-n261 |  | DC_2A_n71A | High order harmonic into n261 | - |  | - | - No harmonic problem |  |
|  |  | DC_2A_n261A | - | - |  | - | No issue |  |
| DC_66_n5-n258 |  | DC_66A_n5A | High order harmonic into n258 | - |  | - | - No harmonic problem |  |
|  |  | DC_66A_n258A | - | - |  | - | No issue |  |
| DC_66_n5-n261 |  | DC_66A_n5A | High order harmonic into n261 | - |  | - | - No harmonic problem |  |
|  |  | DC_66A_n261A | - | - |  | - | No issue |  |
| DC_66_n71-n258 |  | DC_66A_n71A | High order harmonic into n258 | - |  | - | - No harmonic problem |  |
|  |  | DC_66A_n258A | - | - |  | - | No issue |  |
| DC_3_n77-n257 DC_3_n77(2A)-n257 |  | DC_3A_n77A | 7th & 8th harmonics from n77 into n257 | - |  | - | - No harmonic problems by 6th ~ 8th order between FR1 and FR2 |  |
|  |  | DC_3A_n257A, DC_3A_n257D, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into B77 | - |  | - | - Harmonic problem already covered in DC_3A_n77A |  |
| DC_7_n66-n78 DC_7C_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD |  |  | - Harmonic problem already covered in DC_66A_n78A - 16.0dB by 3rd IMD |  |
|  |  | DC_7A_n78A | - | 4th IMD |  | Yes | - 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_66_n66-n78 |  | DC_66A_n66A2 | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs |  |  | - Harmonic problem already covered in DC_66A_n78A - For the DC_66A_n66A, only allowed single uplink transmission. So the 2nd & 4th IMDs do not impact to n78 |  |
|  |  | DC_66A_n78A | - | 5th IMD |  |  | - Follow MSD result for DC_66A_n78A |  |
| DC_20_n1-n7 |  | DC_20A_n1A | - | - |  | - | No issue |  |
|  |  | DC_20A_n7A | - | - |  | - | No issue |  |
| DC_3_n1-n7 |  | DC_3A_n1A, DC_3C_n1A | - | - |  | - | No issue |  |
|  |  | DC_3A_n7A, DC_3C_n7A | - | - |  | - | No issue |  |
| DC_20_n1-n28 |  | DC_20A_n1A | - | - |  | - | No issue |  |
|  |  | DC_20A_n28A | 3rd harmonic from n28 into n1 | 5th IMD |  | - | - Harmonic problem already covered in DC_1A_n28A - No need 5th IMD problem due to restrictedn28 freq. range in Europe |  |
| DC_3_n1-n28 |  | DC_3A_n1A, DC_3C_n1A | - | - |  | - | No issue |  |
|  |  | DC_3A_n28A, DC_3C_n28A | 3rd harmonic from n28 into n1 | 4th IMD |  | - | - Harmonic problem already covered in DC_1A_n28A - 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_20_n7-n28 |  | DC_20A_n7A | - | 3rd & 5th IMDs |  | - | - 15.7dB by 3rd IMD - 2.8dB by 5th IMD |  |
|  |  | DC_20A_n28A | - | 5th IMD |  | - | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_20_n7-n78 |  | DC_20A_n7A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs |  | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 28.8dB by 2nd IMD - 8.5dB by 4th IMD |  |
|  |  | DC_20A_n78A | - | 2nd IMD |  | Yes | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_3_n7-n28 |  | DC_3A_n7A, DC_3C_n7A |  | 2nd IMD |  | - | - 20.0dB by 2nd IMD |  |
|  |  | DC_3A_n28A, DC_3C_n28A | - | 3rd IMD |  | - | - 3rd IMD problem same as DC_3A-7A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
| DC_2_n41-n71 |  | DC_2A_n41A | - | 2nd & 5th IMDs |  | - | - 28.7dB by 2nd IMD - 3.8dB by 5th IMD |  |
|  |  | DC_2A_n71A | 4th harmonic from n71 into n41 | 2nd IMD |  | - | The harmonic problem will be treated CA_n41A_n71A - 29.2dB by 2nd IMD |  |
| DC_66_n41-n71 |  | DC_66A_n41A | - | - |  | - | No issue |  |
|  |  | DC_66A_n71A | 4th harmonic from n71 into n41 | - |  | - | The harmonic problem will be treated CA_n41A_n71A |  |
| DC_18_n3-n78 |  | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 | 3rd & 5th IMDs |  | - | These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 15.2dB by 3rd IMD - 4.0dB by 5th IMD |  |
|  |  | DC_18A_n78A | - | 3rd IMD |  |  | - Only one Japan operation use B18 and the n78 freq. range is allocated 3600 to 3800 MHz. So No IMD problem |  |
| DC_28_n3-n78 |  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 | 5th IMD |  | - | - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |  |
|  |  | DC_28A_n78A | 5th harmonic from B28 into n78 | 3rd IMD |  | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 3rd IMD already covered in DC_3A-28A_n78A |  |
| DC_8_n1-n78 |  | DC_8A_n1A | 4th harmonic from B8 into n78 | 3rd IMD |  | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 14.9dB by 3rd IMD |  |
|  |  | DC_8A_n78A | - | - |  | - | No issue |  |
| DC_3_n40-n41 |  | DC_3A_n40A | 3rd harmonic from B3 into n41 | - |  | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |  |
|  |  | DC_3A_n41A | - | - |  | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_3_n40-n79 |  | DC_3A_n40A | 2nd harmonic from n40 into n79 | 5th IMD |  | - | - The 2nd harmonic problem will be studied in CA_n40-n79. - 4.7dB by 5th IMD |  |
|  |  | DC_3A_n79A | - | 5th IMD |  | - | - 3.2dB by 5th IMD |  |
| DC_3_n41-n79 |  | DC_3A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs |  | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - 30.8dB by 2nd IMD - 5.2dB by 5th IMD - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
|  |  | DC_3A_n79A | - | 2nd & 5th IMDs |  | - | - Already MSD analysis are covered in DC_3A-41A_n79A |  |
| DC_8_n40-n41 |  | DC_8A_n40A | 3rd harmonic from B8 into n41 | - |  | - | - Harmonic problem will be studied in DC_8A_n41A. - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |  |
|  |  | DC_8A_n41A | - | 5th IMD |  | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |  |
| DC_8_n40-n79 |  | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD |  | - | - These harmonic problem will be treated lower order DC or CA band combos. - 10.7dB by 4th IMD |  |
|  |  | DC_8A_n79A | - | 4th IMD |  | - | - 9.2dB by 4th IMD |  |
| DC_8_n41-n79 |  | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD |  | - | - These harmonic problem will be treated lower order DC or CA band combos. - 16.3dB by 3rd IMD |  |
|  |  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD |  | - | - 15.5dB by 3rd IMD |  |
| DC_39_n40-n41 |  | DC_39A_n40A | - | 3rd IMD |  | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |  |
|  |  | DC_39A_n41A | - | - |  | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |  |
| DC_39_n40-n79 |  | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD |  | - | - The harmonic problem will be treated CA_n40-n79 band combos. - 5.8 dB by 4th IMD |  |
|  |  | DC_39A_n79A | - | - |  | - | No issue |  |
| DC_41_n78-n257 |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | - |  | - | No harmonic impact to n257 |  |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - |  | - | No issue |  |
| DC_42_n78-n257 |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |  |
| DC_18_n78-n257 |  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | - |  | - | No harmonic impact to n257 |  |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - |  | - | No issue |  |
| DC_28_n78-n257 |  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | - |  | - | No harmonic impact to n257 |  |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - |  | - | No issue |  |
| DC_1_n3-n257 |  | DC_1A_n3A | High order harmonic into n257 | - |  | -- | No harmonic impact to n257 |  |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - | - |  | Yes | Cross isolation issue already covered in Table 7.3.1A-0bA in TS36.101 |  |
| DC_18_n3-n257 |  | DC_18A_n3A | High order harmonics into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - |  | - | No issue |  |
| DC_28_n3-n257 |  | DC_28A_n3A | High order harmonics into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - |  | - | No issue |  |
| DC_1_n3-n28 |  | DC_1A_n3A | - | - |  | - | No issue |  |
|  |  | DC_1A_n28A | - | 5th IMD |  | Yes | - The 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation already treated in TS36.101 |  |
| DC_8_n3-n28 |  | DC_8A_n3A | - | 2nd & 5th IMDs |  | - | - 30.4dB by 2nd IMD - 2.6dB by 5th IMD |  |
|  |  | DC_8A_n28A | 2nd harmonic from B8 into n3 | - |  | - | The 2nd harmonic problem will be covered in DC_8A_n3A |  |
| DC_11_n77-n257 DC_11_n77(2A)-n257 |  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | - | - |  | - | No issue |  |
| DC_2_n12-n258 |  | DC_2A_n12A | High order harmonic into n258 | - |  | - | No harmonic problem to n258 |  |
|  |  | DC_2A_n258A | - | - |  | - | No issue |  |
| DC_2_n12-n260 |  | DC_2A_n12A | High order harmonic into n260 | - |  | - | No harmonic problem to n260 |  |
|  |  | DC_2A_n260A | - | - |  | - | No issue |  |
| DC_2_n12-n261 |  | DC_2A_n12A | High order harmonic into n261 | - |  | - | No harmonic problem to n261 |  |
|  |  | DC_2A_n261A | - | - |  | - | No issue |  |
| DC_66_n12-n258 |  | DC_66A_n12A | High order harmonic into n258 | - |  | - | No harmonic problem to n258 |  |
|  |  | DC_66A_n258A | - | - |  | - | No issue |  |
| DC_66_n12-n260 |  | DC_66A_n12A | High order harmonic into n260 | - |  | - | No harmonic problem to n260 |  |
|  |  | DC_66A_n260A | - | - |  | - | No issue |  |
| DC_66_n12-n261 |  | DC_66A_n12A | High order harmonic into n261 | - |  | - | No harmonic problem to n261 |  |
|  |  | DC_66A_n261A | - | - |  | - | No issue |  |
| DC_7-7_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD |  | - | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - 15.1dB by 3rd IMD |  |
|  |  | DC_7A_n78A | - | 4th IMD |  | Yes | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |  |
| DC_13_n5-n48 |  | DC_13A_n48A | - | - |  | - | - No issue |  |
| DC_13_n48-n66 |  | DC_13A_n48A | - | 3rd IMD |  | - | - 14.4 dB |  |
|  |  | DC_13A_n66A | 2nd harmonic from n66 into n48 | 5th IMD |  | - | - 2nd harmonic issue will be solved in CA_n48-n66 - 2.8 dB |  |
| DC_66_n5-n48 |  | DC_66A_n5A | 2nd harmonic from n66 into n48 | 5th IMD |  | - | - 2nd harmonic issue will be solved in CA_n48-n66 - 3.3 dB |  |
|  |  | DC_66A_n48A | - | - |  | - | No issue |  |
| DC_2_n41-n66 |  | DC_2A_n41A | - | - |  | - | No issue |  |
|  |  | DC_2A_n66A | - | - |  | - | No issue |  |
| DC_2_n66-n71 |  | DC_2A_n66A | - | - |  | - | No issue |  |
|  |  | DC_2A_n71A | - | - |  | - | No issue |  |
| DC_66_n25-n71 |  | DC_66A_n25A | - | - |  | - | No issue |  |
|  |  | DC_66A_n71A | 3rd harmonic from n71 into n25 | - |  | - | 3rd harmonic issue will be solved in CA_n25-n71 |  |
| DC_28_n7-n78 |  | DC_28A_n7A DC_28A_n7B | 5th harmonic from B28 into n78 | 2nd & 4th IMDs |  | - | - 5th harmonic issue will be solved in DC_28-n78 - These IMD issue were covered in DC_7_n28-n78 |  |
|  |  | DC_28A_n78A | - | 2nd IMD |  |  | - 2nd IMD issue was covered in DC_7-28_n78 |  |
| DC_11_n3-n28 |  | DC_11A_n3A | - | 5th IMD |  |  | - 2.5 dB |  |
|  |  | DC_11A_n28A | - | - |  | - | No issue |  |
| DC_1_n8-n78 |  | DC_1A_n8A | 4th harmonic from n8 into n78 | 3rd IMD |  | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - 14.9dB in DC_8A_n1A-n78A |  |
|  |  | DC_1A_n78A | - | 5th IMD |  | - | - 5th IMD already covered in DC_1A-8A_n78A |  |
| DC_1_n8-n40 |  | DC_1A_n8A | - | - |  | - | No issue |  |
|  |  | DC_1A_n40A | - | 4th & 5th IMDs |  |  | - 9.2 dB by 4th IMD |  |
| DC_20_n20-n75 |  | DC_20A_n20A1 | - | - |  | - | No issue |  |
| DC_1_n28-n40 |  | DC_1A_n28A | - | 4th IMD |  | - | - 10.1 dB |  |
|  |  | DC_1A_n40A | - | 4th IMD |  | - | - 8.6 dB |  |
| DC_3_n1-n40 |  | DC_3A_n1A | - | 5th IMD |  | - | - 8.0 dB |  |
|  |  | DC_3A_n40A | - | - |  | - | No issue |  |
| DC_3_n8-n40 |  | DC_3A_n8A | - | - |  | - | No issue |  |
|  |  | DC_3A_n40A | - | - |  | - | No issue |  |
| DC_3_n8-n78 |  | DC_3A_n8A | 4th harmonic from n8 into n78 | 3rd & 5th IMDs |  | - | - 4th harmonic issue was solved in DC_8A_n78A - 16.3 dB by 3rd IMD |  |
|  |  | DC_3A_n78A | - | - |  | - | No issue |  |
| DC_3_n28-n40 |  | DC_3A_n28A | - | - |  | - | No issue |  |
|  |  | DC_3A_n40A | - | - |  | - | No issue |  |
| DC_7_n1-n40 |  | DC_7A_n1A | - | - |  | - | No issue |  |
|  |  | DC_7A_n40A | - | 3rd & 5th IMDs |  | - | - 15.2 dB by 3rd IMD |  |
| DC_7_n8-n40 |  | DC_7A_n8A | - | 5th IMD |  | - | - 3.0 dB |  |
|  |  | DC_7A_n40A | - | - |  | - | No issue |  |
| DC_7_n8-n78 |  | DC_7A_n8A | 4th harmonic from n8 into n78 | 2nd & 4th IMDs |  |  | - 4th harmonic issue was solved in DC_8A_n78A - 28.5 dB by 2nd IMD |  |
|  |  | DC_7A_n78A | - | 2nd & 5th IMDs |  |  | - 29.7 dB by 2nd IMD |  |
| DC_7_n28-n40 |  | DC_7A_n28A | - | - |  | - | No issue |  |
|  |  | DC_7A_n40A | - | - |  | - | No issue |  |
| DC_2_n7-n78 DC_2_n7-n78(2A) DC_2_n7(2A)-n78 DC_2_n7(2A)-n78(2A) |  | DC_2A_n7A | 2nd harmonic from B2 into n78 | 5th IMD |  |  | - 4.2 dB |  |
|  |  | DC_2A_n78A | - | - |  | - | No issue |  |
| DC_13_n7-n78 |  | DC_13A_n7A | - | 2nd & 4th IMDs |  | - | - 29.0 dB by 2nd IMD |  |
|  |  | DC_13A_n78A | - | 2nd IMD |  | - | - 27.9 dB |  |
| DC_28_n77-n257 DC_28_n77(2A)-n257 |  | DC_28A_n77A | 7th & 8th harmonics from n77 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I | 5th harmonic from B28 into n77 | - |  | - | 5th harmonic problem was solved in DC_28A_n77A |  |
| DC_19_n77-n257 |  | DC_19A_n77A | 7th & 8th harmonics from n77 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 4th harmonic from B19 into n77 | - |  | - | 4th harmonic problem was solved in DC_19A_n77A |  |
| DC_21_n77-n257 |  | DC_21A_n77A | 7th & 8th harmonics from n77 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | - | - |  | - | No issue |  |
| DC_42_n77-n257 |  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - |  | - | No issue |  |
| DC_19_n78-n257 |  | DC_19A_n78A | 7th & 8th harmonics from n78 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 4th harmonic from B19 into n78 | - |  | - | 4th harmonic problem was solved in DC_19A_n78A |  |
| DC_21_n78-n257 |  | DC_21A_n78A | 7th & 8th harmonics from n78 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | - | - |  | - | No issue |  |
| DC_1_n79-n257 |  | DC_1A_n79A | 6th harmonic from n79 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I | - | - |  | - | No issue |  |
| DC_3_n79-n257 |  | DC_3A_n79A | 6th harmonic from n79 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I | - | - |  | - | No issue |  |
| DC_19_n79-n257 |  | DC_19A_n79A | 6th harmonic from n79 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 6th harmonic from B19 into n79 | - |  | - | No harmonic problem to n79 |  |
| DC_21_n79-n257 |  | DC_21A_n79A | 6th harmonic from n79 into n257 | - |  | - | No harmonic problem to n257 |  |
|  |  | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | - | - |  | - | No issue |  |
| DC_42_n79-n257 |  | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | - | - |  | - | No issue |  |
| DC_28_n40-n78 |  | DC_28A_n40A | 5th harmonic from B28 |  | 3rd & 4th IMDs | - |  | Harmonic issue was covered in DC_28A_n78A - No need IMD analysis since CA_n40-n78 only allow one uplink transmission |
|  |  | DC_28A_n78A | - |  | 3rd IMD | - |  | - No need IMD analysis since CA_n40-n78 only allow one uplink transmission |
| DC_1_n28-n77 DC_1_n28-n77(2A) |  | DC_1A_n28A | 2nd harmonic from B1 |  | 3rd & 4th IMDs | - |  | Harmonic issue was covered in DC_1A_n77A - Follow MSD on DC_1_n28-n78 with dual uplink_DC_1_n28 |
|  |  | DC_1A_n77A | - |  | 5th IMD | - |  | - Follow MSD on DC_1_n28-n78 with dual uplink DC_1_n78 |
| DC_3_n28-n77 DC_3_n28-n77(2A) |  | DC_3A_n28A | 2nd harmonic from B3 and 5th harmonic from n28 |  | 3rd , 4th & 5th IMDs | - |  | Harmonic issue were covered in DC_3A_n77A or DC_28A_n77A - 15.9 dB by 3rd IMD |
|  |  | DC_3A_n77A | - |  | 3rd IMD | - |  | - 15.3 dB |
| DC_8_n28-n77 DC_8_n28-n77(2A) |  | DC_8A_n28A | 4th harmonic from B8 and 5th harmonic from n28 |  | 4th & 5th IMDs | - |  | Harmonic issue were covered in DC_8A_n77A or DC_28A_n77A - 10.3 dB by 3rd IMD |
|  |  | DC_8A_n77A | - |  | 4th IMD | - |  | - 11.6 dB |
| DC_42_n28-n77 DC_42_n28-n77(2A) DC_42C_n28-n77(2A) |  | DC_42A_n28A DC_42C_n28A | 5th harmonic from n28 |  | 2nd IMD | - |  | Harmonic issue were covered in DC_28A_n77A - 29.2 dB |
| DC_2_n38-n78 |  | DC_2A_n38A | 2nd harmonic from B2 |  | 3rd IMD |  |  | Harmonic issue were covered in DC_2A_n78A - 14.8 dB |
|  |  | DC_2A_n78A | - |  | - | - |  | No issue |
| DC_2_n38-n66 |  | DC_2A_n38A | - |  | - | - |  | No issue |
|  |  | DC_2A_n66A | - |  | - | - |  | No issue |
| DC_66_n38-n78 |  | DC_66A_n38A | 2nd harmonic from B66 |  | 3rd IMD | - |  | Harmonic issue were covered in DC_66A_n78A - 15.0 dB |
|  |  | DC_66A_n78A | - |  | - | - |  | No issue |
| DC_66_n38-n66 |  | DC_66A_n38A | - |  | - | - |  | No issue |
|  |  | DC_66A_n66A2 | - |  | - | - |  | DC_66A_n66A, only allowed single uplink transmission. |
| DC_18_n3-n77 |  | DC_18A_n3A | 2nd harmonic from n3 and 4th &5th harmonic from B18 |  | 3rd & 5th IMDs | - |  | Harmonic issue were covered in DC_3A_n77A or DC_18A_n77A - 16.3 dB by 3rd IMD |
|  |  | DC_18A_n77A | - |  | 3rd IMD | - |  | - 15.7 dB |
| DC_28_n3-n77 |  | DC_28A_n3A | 2nd harmonic from n3 and 5th harmonic from B28 |  | 3rd & 4th IMDs | - |  | Harmonic issue were covered in DC_3A_n77A or DC_28A_n77A - 15.9 dB by 3rd IMD |
|  |  | DC_28A_n77A | - |  | 3rd & 4th IMDs | - |  | - 17.0 dB by 3rd IMD |
| DC_41_n3-n77 DC_41C_n3-n77 |  | DC_41A_n3A DC_41C_n3A | 2nd harmonic from n3 |  | 3rd & 5th IMDs | - |  | Harmonic issue were covered in DC_3A_n77A - 16.8 dB by 3rd IMD |
|  |  | DC_41A_n77A DC_41C_n77A | - |  | 3rd & 4th IMDs | - |  | - 16.4 dB by 3rd IMD |
| DC_41_n3-n78 DC_41C_n3-n78 |  | DC_41A_n3A DC_41C_n3A | 2nd harmonic from n3 |  | 3rd IMD | - |  | Harmonic issue were covered in DC_3A_n78A - 16.8 dB by 3rd IMD |
|  |  | DC_41A_n78A DC_41C_n78A | - |  | 3rd & 4th IMDs | - |  | - 16.4 dB by 3rd IMD |
| DC_41_n28-n77 DC_41C_n28-n77 |  | DC_41A_n28A DC_41C_n28A | 5th harmonic from n28 |  | 2nd , 3rd & 4th IMDs | - |  | Harmonic issue were covered in DC_28A_n77A - 28.2 dB by 2nd IMD |
|  |  | DC_41A_n77A DC_41C_n77A | - |  | 2nd & 3rd IMDs | - |  | - 30.8 dB by 2nd IMD |
| DC_41_n28-n78 DC_41C_n28-n78 |  | DC_41A_n28A DC_41C_n28A | 5th harmonic from n28 |  | 2nd & 4th IMDs | - |  | Harmonic issue were covered in DC_28A_n77A - 28.2 dB by 2nd IMD |
|  |  | DC_41A_n78A DC_41C_n78A | - |  | 2nd & 5th IMDs | - |  | - 30.8 dB by 2nd IMD |
| DC_1_n41-n78 |  | DC_1A_n41A | - |  | 3rd , 4th & 5th IMDs | - |  | - 19.6 dB - 10.1 dB - 3.8 dB |
|  |  | DC_1A_n78A | - |  | 4th IMD | - |  | - 11.5 dB |
| DC_3_n41-n78 |  | DC_3A_n41A | 2nd harmonic from B3 |  | 3rd IMD | - |  | Harmonic issue were covered in DC_3A_n78A - 16.4 dB |
|  |  | DC_3A_n78A | - |  | - | - |  | No issue |
| DC_20_n41-n78 |  | DC_20A_n41A | 4th harmonic from B20 |  | 2nd & 4th IMDs | - |  | Harmonic issue were covered in DC_20A_n78A - 28.8 dB |
|  |  | DC_20A_n78A | 3rd harmonic from B20 |  | 2nd IMD | - |  | Harmonic issue were covered in DC_20A_n41A - 29.8 dB |
| DC_1_n75-n78 DC_1_n75-n78(2A) |  | DC_1A_n78A | - |  | 2nd & 5th IMDs |  |  | - 29.0 dB |
| DC_3_n75-n78 DC_3_n75-n78(2A) |  | DC_3A_n78A | - |  | 2nd , 4th & 5th IMDs |  |  | - 10.0 dB by 2nd IMD |
| DC_20_n75-n78 DC_20_n75-n78(2A) |  | DC_20A_n78A | 2nd harmonic from B20 |  | - |  |  | Harmonic issue were covered in DC_20A_n75A |
| DC_20_n78-n92 DC_20_n78(2A)-n92 |  | DC_20A_n78A DC_20A_n92A_ULSUP-TDM-n78A | 2nd harmonic from B20 |  | - |  |  | Harmonic issue were covered in DC_20A_n92A |
| DC_3_n34-n258 |  | DC_3A_n34A | High order harmonic from B3 or n34 |  | - | - |  | No issue |
|  |  | DC_3A_n258A | - |  | - | - |  | No issue |
| DC_3_n40-n258 |  | DC_3A_n40A | High order harmonics from B3 or n40 |  | - | - |  | No issue |
|  |  | DC_3A_n258A | - |  | - | - |  | No issue |
| DC_3_n41-n258 |  | DC_3A_n41A | High order harmonics from B3 or n41 |  | - | - |  | No issue |
|  |  | DC_3A_n258A | - |  | - | - |  | No issue |
| DC_8_n34-n258 |  | DC_8A_n34A | High order harmonics from B8 or n34 |  | - | - |  | No issue |
|  |  | DC_8A_n258A | - |  | - | - |  | No issue |
| DC_8_n40-n258 |  | DC_8A_n40A | High order harmonics from B8 or n40 |  | - | - |  | No issue |
|  |  | DC_8A_n258A | - |  | - | - |  | No issue |
| DC_8_n41-n258 |  | DC_8A_n41A | High order harmonics from B8 or n41 |  | - | - |  | No issue |
|  |  | DC_8A_n258A | - |  | - | - |  | No issue |
| DC_8_n79-n258 |  | DC_8A_n79A | High order harmonics from B8 or n79 |  | - | - |  | No issue |
|  |  | DC_8A_n258A | - |  | - | - |  | No issue |
| DC_39_n40-n258 |  | DC_39A_n40A | High order harmonics from B39 or n40 |  | - | - |  | No issue |
|  |  | DC_39A_n258A | - |  | - | - |  | No issue |
| DC_39_n41-n258 |  | DC_39A_n41A | High order harmonics from B39 or n41 |  | - | - |  | No issue |
|  |  | DC_39A_n258A | - |  | - | - |  | No issue |
| DC_39_n79-n258 |  | DC_39A_n79A | High order harmonics from B39 or n79 |  | - | - |  | No issue |
|  |  | DC_39A_n258A | - |  | - | - |  | No issue |
| DC_40_n41-n258 |  | DC_40A_n41A | High order harmonics from B40 or n41 |  | - | - |  | No issue |
|  |  | DC_40A_n258A | - |  | - | - |  | No issue |
| DC_40_n79-n258 |  | DC_40A_n79A | High order harmonics from B40 or n79 |  | - | - |  | No issue |
|  |  | DC_40A_n258A | - |  | - | - |  | No issue |
| DC_41_n79-n258 |  | DC_41A_n79A | High order harmonics from B41 or n79 |  | - | - |  | No issue |
|  |  | DC_41A_n258A | - |  | - | - |  | No issue |
| DC_1_n28-n257 DC_1_n28-n257I |  | DC_1A_n28A | High order harmonics from B1 or n28 |  | - | - |  | No issue |
|  |  | DC_1A_n257A | - |  | - | - |  | No issue |
| DC_3_n28-n257 DC_3_n28-n257I |  | DC_3A_n28A | High order harmonics from B3 or n28 |  | - | - |  | No issue |
|  |  | DC_3A_n257A | - |  | - | - |  | No issue |
| DC_41_n3-n257 DC_41_n3-n257I DC_41C_n3-n257 DC_41C_n3-n257I |  | DC_41A_n3A DC_41C_n3A | High order harmonics from B41 or n3 |  | - | - |  | No issue |
|  |  | DC_41A_n257A DC_41A_n257I DC_41C_n257A DC_41C_n257I | - |  | - | - |  | No issue |
| DC_41_n28-n257 DC_41C_n28-n257 DC_41_n28-n257I DC_41C_n28-n257I |  | DC_41A_n28A DC_41C_n28A | High order harmonics from B41 or n28 |  | - | - |  | No issue |
|  |  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I | - |  | - | - |  | No issue |
| DC_41_n77-n257 DC_41C_n77-n257 DC_41_n77-n257I DC_41C_n77-n257I |  | DC_41A_n77A DC_41C_n77A | High order harmonics from B41 or n77 |  | - | - |  | No issue |
|  |  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I | - |  | - | - |  | No issue |
| DC_7_n1-n257 DC_7-7_n1-n257 |  | DC_7A_n1A | High order harmonics from B7 or n1 |  | - | - |  | No issue |
|  |  | DC_7A_n257A | - |  | - | - |  | No issue |
| Note 1: These band combinations should be completed the dual uplink EN-DC firstly Note 2: only single switched UL is supported |  |  |  |  |  |  |  |  |

Table 5.3-2 summarizes the EN-DC band combinations with self-interference problems for 4DL/2UL DC operation.

Table 5.3-2: Summary of Self-interference analysis for LTE 2 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration |  | Uplink DC Configuration |  | Harmonic relation issues |  | intermodulation to own rx band |  |  | interference due to small frequency separation | MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1-3_n7-n78 |  | DC_1A_n7A DC_1A_n7B |  | - |  | 4th & 5th IMDs into n78 |  |  | Yes | These IMD problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |  |
|  |  | DC_1A_n78A, DC_1A_n78(2A) |  | - |  | 2nd IMD into n3 4th IMD into n7 |  |  | Yes | These IMD issues were already covered in DC_1A-3A_n78A and DC_1A_n7A-n78A. |  |
|  |  | DC_3A_n7A DC_3A_n7B |  | 2nd harmonic from 3 into n78 |  | 3rd IMD into n78 |  |  | - | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem already covered in DC_3A_n7A-n78A. |  |
|  |  | DC_3A_n78A, DC_3A_n78(2A) |  |  |  | 5th IMD into B1 |  |  |  | -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |  |
| DC_1-3_n28-n78 |  | DC_1A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 3rd IMD into n78 5th IMD into B3 |  |  | Yes | - Harmonic problems already solved in DC_1A_n28A or DC_28A_n78A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A - 5th IMD problem was solved in 3DL_DC_1A-3A_n28A |  |
|  |  | DC_1A_n78A |  | - |  | 2nd IMD into B3 4th IMD into B1 4th IMD into n78 5th IMD into n28 |  |  | Yes | - 2nd IMD problem was solved in 3DL_DC_1A-3A_n78A - 4th IMD problem already covered in DC_1A_n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  |  | DC_3A_n28A |  | 2nd harmonic into n78 3rd harmonic into B1 5th harmonic into n78 |  | 4th IMD into B1 5th IMD into n78 |  |  |  | - Harmonic problems already solved in DC_3A_n78A or DC_1A_n28A or DC_28A_n78A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |  |
|  |  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 5th IMD into B1 |  |  |  | - 2nd harmonic problem already solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |  |
| DC_1-3_n78-n257 |  | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 2nd IMD into B3 |  |  | Yes | - Up to 7th harmonic order will be not any impact to n257 - This IMD problem will be covered in DC_1A-3A_n78A - Proximity also follow LTE_CA_1A-3A |  |
|  |  | DC_1A_n257A/D/G/H/I |  | 2nd harmonic from B1 into n78 |  | - |  |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  |  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 5th IMD into B1 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |  |
|  |  | DC_3A_n257A/D/G/H/I |  | 2nd harmonic from B3 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
| DC_1-5_n78-n257 |  | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B5 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-5A_n78A. |  |
|  |  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  |  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A. |  |
|  |  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_1-7_n28-n78 |  | DC_1A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 2nd IMD into B7 3rd IMD into n78 |  |  |  | - Harmonic problems already solved in DC_1A_n78A or DC_28A_n78A - 2nd IMD problem was solved in 3DL_DC_1A-7A_n28A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  |  | DC_1A_n78A |  | - |  | 4th IMD into B1 4th IMD into n7 5th IMD into n28 |  |  |  | - 4th IMD problem already solved in DC_1A_n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  |  | DC_7A_n28A |  | 3rd harmonic into B1 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  |  | - | - Harmonic problem was solved in DC_1A_n28A or DC_28A_n78A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |  |
|  |  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 4th IMD into n78 4th IMD into B1 |  |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A-28A_n78A - 4th IMD problem is no issue for TDD band - 4th IMD problem was solved in 3DL_DC_1A-7A_n78A |  |
| DC_1-7-7_n78-n257 |  | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B7 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problems will be covered in DC_1A-7A_n78A. |  |
|  |  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 4th IMD into B1 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A_7A-n78A |  |
|  |  | DC_7A_n257A |  | - |  | - |  |  | - | No issues |  |
| DC_1-7_n78-n257 |  | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B7 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problems will be covered in DC_1A-7A_n78A. |  |
|  |  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 4th IMD into B1 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A_7A-n78A |  |
|  |  | DC_7A_n257A |  | - |  | - |  |  | - | No issues |  |
| DC_1-8_n78-n257 |  | DC_1A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B8 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD already covered in DC_1A-8A_n78A |  |
|  |  | DC_1A_n257A |  | 2nd harmonic from B1 into n78 |  | - |  |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n78A |  |
|  |  | DC_8A_n78A |  | 7th & 8th Harmonics from n78 into n257 |  | - |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |  |
|  |  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_8A_n78A |  |
| DC_3-5_n78-n257 |  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  |  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  |  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B3 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A. |  |
|  |  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
| DC_3-7_n28-n78 |  | DC_3A_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 3rd IMD into B7 5th IMD into n78 |  |  | - | - Harmonic problems already solved in DC_3A_n78A or DC_28A_n78A - 3rd IMD problem was solved in 3DL_DC_3A-7A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |  |
|  |  | DC_3A_n78A |  | 2nd harmonic into n78 |  | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 |  |  |  | - 2nd harmonic problem already solved in DC_3A_n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A_n78A - 4th & 5th IMDs problems are no issue for TDD band |  |
|  |  | DC_7A_n28A |  | 5th harmonic into n78 |  | 2nd IMD into B3 2nd,4th IMDs into n78 |  |  | - | - Harmonic problem was solved in DC_28A_n78A - 2nd IMD problems was solved in 3DL_DC_3A-7A_n28A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |  |
|  |  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 3rd & 4th IMDs into B3 4th IMD into n78 |  |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A_n28A-n78A - 3rd & 4th IMDs problems were solved in 3DL_DC_3A-7A_n78A - 4th IMD problem is no issue for TDD band |  |
| DC_3-7-7_n78-n257 DC_3-3-7_n78-n257 DC_3-3-7-7_n78-n257 |  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  |  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A |  |
|  |  | DC_7A_n257A |  | - |  | - |  |  | - | No issues |  |
| DC_3-7_n78-n257 |  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  |  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A |  |
|  |  | DC_7A_n257A |  | - |  | - |  |  | - | No issues |  |
| DC_3-8_n78-n257 |  | DC_3A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - | - Up to 7th harmonic order will be not any impact to n257 |  |
|  |  | DC_3A_n257A |  | 2nd harmonic from B3 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_3A_n78A |  |
|  |  | DC_8A_n78A |  | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 |  |  | - | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |  |
|  |  | DC_8A_n257A |  | 4th harmonic from B8 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_8A_n78A |  |
| DC_5-7-7_n78-n257 |  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B7 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 2nd IMD problems will be covered in DC_5A-7A_n78A. |  |
|  |  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
|  |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 2nd & 5th IMDs into B5 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problems will be covered in DC_5A-7A_n78A |  |
|  |  | DC_7A_n257A |  | - |  | - |  |  | - | No issues |  |
| DC_5-7_n78-n257 |  | DC_5A_n78A |  | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B7 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 2nd IMD problems will be covered in DC_5A-7A_n78A. |  |
|  |  | DC_5A_n257A |  | 4th harmonic from B5 into n78 |  | - |  |  | - | - Harmonic problem will be solved in DC_5A_n78A |  |
|  |  | DC_7A_n78A |  | 7th harmonic from B7 into n257 |  | 2nd & 5th IMDs into B5 |  |  | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problems will be covered in DC_5A-7A_n78A |  |
|  |  | DC_7A_n257A |  | - |  | - |  |  | - | No issues |  |
| DC_1-3_n77-n78 |  | DC_1A_n77A |  | - |  | 2nd, 4th & 5th IMDs into B3 4th & 5th IMD into n78 |  |  | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  |  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 2nd IMD into B3 4th & 5th IMDs into n77 |  |  | Yes | Harmonic problem already covered in DC_1A_n77A 2nd IMD problem already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_1A_n78A 4th & 5th IMDs problem already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A |  |
|  |  | DC_3A_n77A |  | 2nd harmonic from B3 into n78 |  | 2nd & 5th IMD into B1 4th IMD & 5th IMD into n78 |  |  | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  |  | DC_3A_n78A |  | 2nd harmonic from B3 into n77 |  | 4th & 5th IMDs into n77 5th IMD into B1 |  |  | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
| DC_1-3_n77-n79 |  | DC_1A_n77A |  | - |  | 2nd, 4th & 5th IMDs into B3 3rd, 4th & 5th IMDs into n79 |  |  | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_1A_n77A. These MSD issues will be covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  |  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 5th IMD into n77 |  |  | Yes | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue is already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
|  |  | DC_3A_n77A |  | - |  | 2nd & 5th IMD into B1 3rd & 4th IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-3A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
|  |  | DC_3A_n79A |  | 2nd harmonic from B3 into n77 |  | 5th IMD into B1 5th IMD into n77 |  |  | - | 2nd harmonic issue already covered in DC_3A_n77A The MSD issue will be covered in 3DL_DC_1A-3A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
| DC_1-3_n78-n79 |  | DC_1A_n78A |  | - |  | 2nd IMD into B3 3rd & 5th IMDs into n79 |  |  | Yes | 2nd IMD problem already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_1A_n78A 3rd & 5th IMDs problem will be covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A |  |
|  |  | DC_1A_n79A |  | - |  | 5th IMD into n78 |  |  | Yes | The MSD issue is already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
|  |  | DC_3A_n78A |  | - |  | 5th IMD into B1 3rd IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  |  | DC_3A_n79A |  | 2nd harmonic from B3 into n78 |  | 5th IMD into B1 5th IMD into n78 |  |  | - | 2nd harmonic issue already covered in DC_3A_n78A The MSD issue will be covered in 3DL_DC_1A-3A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
| DC_1-19_n77-n79 |  | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMDs into n79 |  |  | - | 6th harmonic will not any impact to n79 The MSD issue will be covered in 3DL_DC_1A-19A_n77A with 2UL_DC_19A_n77A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n77A |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 4th IMD into B1 2nd & 3rd IMDs into n77 |  |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_1A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_1-19_n78-n79 |  | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMDs into n79 |  |  |  | 6th harmonic will not any impact to n79 The MSD issue will be covered in 3DL_DC_1A-19A_n78A with 2UL_DC_19A_n78A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n78A |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B1 2nd & 3rd IMDs into n78 |  |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_1A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_1-21_n77-n78 |  | DC_1A_n77A |  | - |  | 2nd & 5th IMDs into B21 4th & 5th IMD into n78 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  |  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 2nd & 5th IMDs into B21 4th & 5th IMD into n77 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |  |
| DC_1-21_n77-n79 |  | DC_1A_n77A |  | - |  | 2nd & 5th IMDs into B21 3rd, 4th & 5th IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  |  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 4th IMD into B21 5th IMD into n77 |  |  | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issues are already covered in 3DL_DC_1A-21A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-21_n78-n79 |  | DC_1A_n78A |  | - |  | 2nd & 5th IMDs into B21 3rd & 5th IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |  |
|  |  | DC_1A_n79A |  | - |  | 4th IMD into B21 5th IMD into n78 |  |  | - | The MSD issues are already covered in 3DL_DC_1A-21A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-42_n77-n78 |  | DC_1A_n77A |  | - |  | 4th & 5th IMD into B42 4th & 5th IMD into n78 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A. |  |
|  |  | DC_1A_n78A |  | 2nd harmonic from B1 into n77 |  | 4th IMD into B42 4th & 5th IMD into n77 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |  |
| DC_1-42_n77-n79 |  | DC_1A_n77A |  | - |  | 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n77A with 2UL_DC_1A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A. |  |
|  |  | DC_1A_n79A |  | 2nd harmonic from B1 into n77 |  | 5th IMD into B42 5th IMD into n77 |  |  | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-42A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |  |
| DC_1-42_n78-n79 |  | DC_1A_n78A |  | - |  | 4th IMD into B42 3rd & 5th IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_1A-42A_n78A with 2UL_DC_1A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |  |
|  |  | DC_1A_n79A |  | - |  | 5th IMD into B42 5th IMD into n78 |  |  | - | The MSD issue are already covered in 3DL_DC_1A-42A_n79A with 2UL_DC_1A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |  |
| DC_3-19_n77-n78 |  | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 3rd IMD into B3 2nd IMD into n78 |  |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_3A-19A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  |  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 3rd IMD into B3 2nd & 5th IMDs into n77 |  |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_3A-19A_n78A with 2UL_DC_19A_n78A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_3-19_n77-n79 |  | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B3 2nd, 3rd, 4th & 5th IMD into n79 |  |  | - | 6th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_3A-19A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 4th IMD into B3 2nd & 3rd IMDs into n77 |  |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_3A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_3-19_n78-n79 |  | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 3rd IMD into B3 2nd, 3rd, 4th & 5th IMD into n79 |  |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_3A-19A_n78A with 2UL_DC_19A_n78A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B3 2nd & 3rd IMDs into n78 |  |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_3A-19A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_3-21_n77-n78 |  | DC_3A_n77A |  | 2nd harmonic from B3 into n78 |  | 4th & 5th IMDs into B21 4th & 5th IMDs into n78 |  |  | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  |  | DC_3A_n78A |  | 2nd harmonic from B3 into n77 |  | 4th & 5th IMDs into B21 4th & 5th IMDs into n77 |  |  | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
|  |  | DC_21A_n77A |  | - |  | 2nd & 5th IMDs into B3 4th & 5th IMDs into n78 |  |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n77A |  |
|  |  | DC_21A_n78A |  | - |  | 2nd IMDs into B3 4th IMD into n77 |  |  | - | These MSD issue is already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_21A_n78A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
| DC_3-21_n77-n79 |  | DC_3A_n77A |  | - |  | 4th & 5th IMDs into B21 3rd & 4th IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n77A |  |
|  |  | DC_3A_n79A |  | 2nd harmonic from B3 into n77 |  | 3rd IMD into B21 5th IMD into n77 |  |  | - | 2nd harmonic issue already covered in DC_3A_n77A The MSD issue will be covered in 3DL_DC_3A-21A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
|  |  | DC_21A_n77A |  |  |  | 2nd & 5th IMDs into B3 2nd & 4th IMDs into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n77A |  |
|  |  | DC_21A_n79A |  | - |  | 3rd IMD into B3 2nd IMD into n77 |  |  |  | These MSD issues are already covered in 3DL_DC_3A-21A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n79A |  |
| DC_3-21_n78-n79 |  | DC_3A_n78A |  | - |  | 4th & 5th IMDs into B21 3rd IMD into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  |  | DC_3A_n79A |  | 2nd harmonic from B3 into n78 |  | 3rd IMD into B21 5th IMD into n78 |  |  | - | 2nd harmonic issue already covered in DC_3A_n78A The MSD issue will be covered in 3DL_DC_3A-21A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
|  |  | DC_21A_n78A |  | - |  | 2nd IMDs into B3 2nd & 4th IMDs into n79 |  |  | - | These MSD issue is already covered in 3DL_DC_3A-21A_n78A with 2UL_DC_21A_n78A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
|  |  | DC_21A_n79A |  | - |  | 3rd IMD into B3 2nd IMD into n78 |  |  | - | These MSD issues are already covered in 3DL_DC_3A-21A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n78A-n79A with 2UL_DC_21A_n79A |  |
| DC_3-42_n77-n78 |  | DC_3A_n77A |  | 2nd harmonic from B3 into B42 & n78 |  | 4th & 5th IMDs into B42 4th & 5th IMDs into n78 |  |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n78A These MSD issues are already covered in 3DL_DC_3A-42A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n77A |  |
|  |  | DC_3A_n78A |  | 2nd harmonic from B3 into B42 & n77 |  | 4th & 5th IMDs into B42 4th & 5th IMDs into n77 |  |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n77A These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n77A-n78A with 2UL_DC_3A_n78A |  |
| DC_3-42_n77-n79 |  | DC_3A_n77A |  | 2nd harmonic from B3 into B42 |  | 4th & 5th IMDs into B42 3rd & 4th IMD into n79 |  |  | - | 2nd harmonic issue already covered in DC_3A-42A These MSD issues are already covered in 3DL_DC_3A-42A_n77A with 2UL_DC_3A_n77A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n77A |  |
|  |  | DC_3A_n79A |  | 2nd harmonic from B3 into B42 & n77 |  | 5th IMD into B42 5th IMD into n77 |  |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n77A The MSD issue will be covered in 3DL_DC_3A-42A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n77A-n79A with 2UL_DC_3A_n79A |  |
| DC_3-42_n78-n79 |  | DC_3A_n78A |  | 2nd harmonic from B3 into B42 |  | 4th & 5th IMDs into B42 3rd IMD into n79 |  |  | - | 2nd harmonic issue already covered in DC_3A-42A These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n78A |  |
|  |  | DC_3A_n79A |  | 2nd harmonic from B3 into B42 & n78 |  | 5th IMD into B42 5th IMD into n78 |  |  | - | 2nd harmonic issue already covered in DC_3A-42A and DC_3A_n78A The MSD issue will be covered in 3DL_DC_3A-42A_n79A with 2UL_DC_3A_n79A and 3DL_3A_n78A-n79A with 2UL_DC_3A_n79A |  |
| DC_19-21_n77-n78 |  | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 4th IMD into B21 2nd IMD into n78 |  |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A-21A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  |  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 2nd & 5th IMDs into n77 |  |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_19-21_n77-n79 |  | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 4th IMD into B21 2nd, 3rd , 4th & 5th IMDs into n79 |  |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A-21A_n77A with 2UL_DC_19A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 5th IMD into B21 2nd & 3rd IMDs into n77 |  |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-21A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-21_n78-n79 |  | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd, 4th & 5th IMD into n79 |  |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 5th IMD into B21 2nd & 3rd IMDs into n78 |  |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-21A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-42_n77-n78 |  | DC_19A_n77A |  | 4th harmonic from B19 into n78 |  | 2nd IMD into n78 |  |  | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |  |
|  |  | DC_19A_n78A |  | 4th harmonic from B19 into n77 |  | 2nd & 5th IMDs into n77 |  |  | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |  |
| DC_19-42_n77-n79 |  | DC_19A_n77A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd , 4th & 5th IMDs into n79 |  |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n77 |  | 2nd IMD into B42 2nd & 3rd IMDs into n77 |  |  | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-42A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n77A-n79A with 2UL_DC_19A_n79A |  |
| DC_19-42_n78-n79 |  | DC_19A_n78A |  | 6th harmonic from B19 into n79 |  | 2nd, 3rd, 4th & 5th IMD into n79 |  |  | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |  |
|  |  | DC_19A_n79A |  | 4th harmonic from B19 into n78 |  | 2nd IMD into B42 2nd & 3rd IMDs into n78 |  |  | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-42A_n79A with 2UL_DC_19A_n79A and 3DL_19A_n78A-n79A with 2UL_DC_19A_n79A |  |
| DC_21-42_n77-n78 |  | DC_21A_n77A |  | - |  | 5th IMD into B42 4th & 5th IMDs into n78 |  |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n78A with 2UL_DC_21A_n77A |  |
|  |  | DC_21A_n78A |  | - |  | 4th IMD into n77 |  |  | - | These MSD issue is already covered in 3DL_21A_n77A-n78A with 2UL_DC_21A_n78A |  |
| DC_21-42_n77-n79 |  | DC_21A_n77A |  | - |  | 5th IMD into B42 2nd & 4th IMDs into n79 |  |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n77A with 2UL_DC_21A_n77A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n77A |  |
|  |  | DC_21A_n79A |  | - |  | 2nd IMD into B42 2nd IMD into n77 |  |  |  | These MSD issues are already covered in 3DL_DC_21A-42A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n77A-n79A with 2UL_DC_21A_n79A |  |
| DC_21-42_n78-n79 |  | DC_21A_n78A |  | - |  | 2nd & 4th IMDs into n79 |  |  | - | These MSD issue is already covered in 3DL_21A_n78A-n79A with 2UL_DC_21A_n78A |  |
|  |  | DC_21A_n79A |  | - |  | 2nd IMD into B42 2nd IMD into n78 |  |  | - | These MSD issues are already covered in 3DL_DC_21A-42A_n79A with 2UL_DC_21A_n79A and 3DL_21A_n78A-n79A with 2UL_DC_21A_n79A |  |
| DC_1-28_n28-n78 |  | DC_1A_n28A |  | 5th harmonic from n28 into n78 |  | 3rd IMD into n78 |  |  | - | - Harmonic problems already solved in CA_n28A-n78A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |  |
|  |  | DC_1A_n78A |  | - |  | 5th IMD into B28 & n28 |  |  | - | - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A or 3DL_DC_1A-28A_n78A |  |
|  |  | DC_28A_n28A1 |  | 3rd harmonic into B1 |  | 3rd & 5th IMDs into B1 5th IMD into n78 |  |  | - | 4th harmonic will be covered in DC_28A_n1A The MSD issue will be covered in 3DL_DC_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  |  | DC_28A_n78A |  | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 5th IMD into n28 |  |  | - | 3rd harmonic will be covered in CA_1A-28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_3-7_n1-n78 |  | DC_3A_n1A |  | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | Yes | 2nd harmonic will be covered in DC_3A_n78A The MSD issue will be covered in 3DL_3A_n1A-n78A with 2UL_DC_3A_n1A |  |
|  |  | DC_3A_n78A |  | - |  | 5th IMD into n1 |  |  | - | These MSD issues are already covered in 3DL_DC_3A_n1A-n78A with 2UL_DC_3A_n78A |  |
|  |  | DC_7A_n1A, DC_7C_n1A |  | - |  | 4th & 5th IMDs into n78 |  |  | Yes | These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A |  |
|  |  | DC_7A_n78A, DC_7C_n78A |  | - |  | 3rd & 4th IMDs into B3 4th IMD into n1 |  |  | - | These IMD problems were already covered in 3DL_DC_3A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n1A-n78A with 2UL_DC_7A_n78A |  |
| DC_3-28_n28-n78 |  | DC_3A_n28A DC_3C_n28A |  | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  |  | - | - Harmonic problems already solved in DC_3A_n78A or CA_n28A-n78A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |  |
|  |  | DC_3A_n78A DC_3C_n78A |  | - |  | - |  |  | - | No issue |  |
|  |  | DC_28A_n28A1 |  | - |  | 5th IMD into n78 |  |  | - | The MSD issue will be covered in 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  |  | DC_28A_n78A |  | - |  | 3rd IMD into B3 5th IMD into n28 |  |  | - | The MSD issue will be covered in 3DL_DC_3A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_7-28_n28-n78 |  | DC_7A_n28A DC_7C_n28A |  | 5th harmonic into n78 |  | 2nd,4th IMDs into n78 |  |  | - | - Harmonic problem was solved in DC_28A_n78A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |  |
|  |  | DC_7A_n78A DC_7C_n78A |  | - |  | 2nd & 5th IMDs into B28 & n28 |  |  | - | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A_n28A-n78A or 3DL_DC_7A-28A_n78A |  |
|  |  | DC_28A_n28A1 |  | - |  | 5th IMD into n78 |  |  | - | The MSD issue will be covered in 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |  |
|  |  | DC_28A_n78A |  | - |  | 2nd IMD into B7 5th IMD into n28 |  |  | - | The MSD issue will be covered in 3DL_DC_7A-28A_n78A with 2UL_DC_28A_n78A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n78A |  |
| DC_1-8_n77-n257 |  | DC_1A_n77A |  | 7th & 8th Harmonics from n77 into n257 |  | 5th IMD into B8 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD already covered in DC_1A-8A_n77A |  |
|  |  | DC_1A_n257A |  | 2nd harmonic from B1 into n77 |  | - |  |  | - | - Harmonic problem already discussed and will be solved in DC_1A_n77A |  |
|  |  | DC_8A_n77A |  | 7th & 8th Harmonics from n77 into n257 |  | 3rd IMD into B1 |  |  | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 The MSD issue will be covered in 3DL_DC_1A-8A_n77A with 2UL_DC_8A_n77A |  |
|  |  | DC_8A_n257A |  | 4th harmonic from B8 into n77 |  | - |  |  | - | - Harmonic problem will be solved in DC_8A_n77A |  |
| DC_1-3_n5-n78 |  | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  |  | Yes |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  |  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs into n78 |  |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3_n5A-n78A |
|  |  | DC_1A_n78A | - |  | 2nd IMD into B3 5th IMD into n5 |  |  | Yes |  |  | - The 2nd IMD issue will be covered in DC_1A-3A_n78A - The 5th IMD issue will be covered in DC_1A_n5A-n78A |
|  |  | DC_3A_n78A, DC_3C_n78A | - |  | 5th IMD into B1 |  |  | - |  |  | - The 5th IMD issue will be covered in DC_1A-3A_n78A |
| DC_1-7_n5-n78 |  | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  |  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  |  | DC_1A_n78A | - |  | 5th IMD into n5 4th IMD into B7 |  |  | Yes |  |  | The 5th IMD will be covered in DC_1A_n5A-n78A The 4th IMD will be covered in DC_1A-7A_n78A |
|  |  | DC_7A_n78A, DC_7C_n78A | - |  | 4th IMD into B1 2nd & 5th IMDs into n5 |  |  | Yes |  |  | The 4th IMD will be covered in DC_1A-7A_n78A The 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A |
| DC_1-28_n5-n78 |  | DC_1A_n5A | 4th harmonic from n5 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  |  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 5th IMD into B1 |  |  |  |  |  | - The 3rd harmonic already covered in CA_1A-28A. - The other harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD problem will be covered in DC_1A-28A_n5A |
|  |  | DC_1A_n78A | - |  | 5th IMD into n5 5th IMD into B28 |  |  | - |  |  | - The 5th IMD problem will be covered in DC_1A_n5A-n78A - The 5th IMD problem will be covered in DC_1A-28A_n78A |
|  |  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 5th IMD into n5 |  |  | - |  |  | 3rd harmonic will be covered in CA_1A-28A The 3rd IMD issue will be covered in DC_1A-28A_n78A The 5th IMD issue will be covered in DC_28A_n5A-n78A |
| DC_3-7_n5-n78 |  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B7 3rd & 5th IMDs into n78 |  |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A-7A_n5A - These IMD issue will be covered in DC_3A_n5A-n78A |
|  |  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  |  | DC_3A_n78A, DC_3C_n78A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_7A_n78A, DC_7C_n78A | - |  | 2nd & 5th IMDs into n5 3rd & 4th IMDs into B3 |  |  | Yes |  |  | The 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A |
| DC_3-28_n5-n78 |  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 |  | 3rd & 5th IMDs into n78 4th IMD into B28 |  |  | - |  |  | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A_n5A-n78A The 4th IMD issue will be covered in DC_3A-28A_n5A |
|  |  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 4th IMD into B3 |  |  |  |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 4th IMD will be covered in DC_3A-28A_n5A |
|  |  | DC_3A_n78A, DC_3C_n78A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_28A_n78A | - |  | 3rd IMD into B3 5th IMD into n5 |  |  | - |  |  | - The 3rd IMD will be covered in DC_3A-28A_n78A - The 5th IMD will be covered in DC_28A_n5A-n78A |
| DC_7-28_n5-n78 |  | DC_7A_n5A, DC_7C_n5A | - |  | 2nd & 4th IMDs into n78 3rd & 5th IMDs into B28 |  |  | - |  |  | - These IMDs problem will be covered in DC_7A_n5A-n78A – These IMDs problems will be covered in DC_7A-28A_n5A |
|  |  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 |  | 5th IMD into B7 |  |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD issue will be covered in 7A-28A_n5A |
|  |  | DC_7A_n78A, DC_7C_n78A | - |  | 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 |  |  | Yes |  |  | 2nd & 5th IMDs will be covered in DC_7A_n5A-n78A Other 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  |  | DC_28A_n78A | - |  | 5th IMD into n5 2nd IMD into B7 |  |  | Yes |  |  | - The 5th IMD will be covered in DC_28A_n5A-n78A - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_1-7_n3-n78 |  | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  |  | DC_1A_n78A | - |  | 4th IMD into B7 2nd IMD into n3 |  |  | Yes |  |  | The 4th IMD will be covered in DC_1A-7A_n78A The 2nd IMD will be covered in DC_1A_n3A-n78A |
|  |  | DC_7A_n3A | 2nd harmonic from n3 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - The IMD problem already covered in DC_7A_n3A-n78A |
|  |  | DC_7A_n78A |  |  | 3rd & 4th IMDs into n3 4th IMD into B1 |  |  | Yes |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A - The 4th IMD will be covered in DC_1A-7A_n78A |
| DC_2-7_n38-n66 |  | DC_2A_n66A | - |  | - |  |  | - |  |  | No issue |
| DC_2-7_n38-n78 |  | DC_2A_n78A | - |  | - |  |  | - |  |  | No issue |
| DC_2-66_n41-n260 |  | DC_2A_n41A | High order harmonic into n260 |  | - |  |  | - |  |  | - No harmonic problem |
|  |  | DC_66A_n41A | High order harmonic into n260 |  | 4th IMD into B2 |  |  | - |  |  | - No harmonic problem - The 4th IMD issue will be covered in DC_2A-66A_n41A |
| DC_2-66_n41-n261 |  | DC_2A_n41A | High order harmonic into n261 |  | - |  |  | - |  |  | - No harmonic problem |
|  |  | DC_66A_n41A | High order harmonic into n261 |  | 4th IMD into B2 |  |  | - |  |  | - No harmonic problem - The 4th IMD issue will be covered in DC_2A-66A_n41A |
| DC_7-66_n38-n78 |  | DC_66A_n78A | - |  | - |  |  | - |  |  | No issue |
| DC_1-3_n77-n257 |  | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 2nd, 4th & 5th IMDs into B3 |  |  | Yes |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs issue will be covered in DC_1A-3A_n77A |
|  |  | DC_3A_n77A | 7th & 8th harmonics from n77 into n257 |  | 2nd & 5th IMD into B1 |  |  | - |  |  | - No harmonic problems - These IMD problem will be covered in DC_1A-3A_n77A_ |
|  |  | DC_1A_n257A/D/G/H/I | 2nd harmonic from B1 into n77 |  | - |  |  | Yes |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  |  | DC_3A_n257A//D/G/H/I | 2nd harmonic from B3 into B77 |  | - |  |  | - |  |  | - Harmonic problem already covered in DC_3A_n77A |
| DC_3-20_n1-n7 |  | DC_3A_n1A, DC_3C_n1A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_3A_n7A, DC_3C_n7A | - |  | 2nd IMD into B20 |  |  | - |  |  | - 2nd IMD issue will be covered in DC_3A-20A_n7A |
|  |  | DC_20A_n1A | - |  | - |  |  | Yes |  |  | No issue |
|  |  | DC_20A_n7A | - |  | - |  |  | - |  |  | No issue |
| DC_3-20_n1-n28 |  | DC_3A_n1A, DC_3C_n1A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_3A_n28A, DC_3C_n28A | 3rd harmonic from n28 into n1 |  | 4th IMD into n1 |  |  |  |  |  | - Harmonic problems already solved in DC_1A_n28A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A |
|  |  | DC_20A_n1A | - |  | - |  |  | Yes |  |  | No issue |
|  |  | DC_20A_n28A | 3rd harmonic from n28 into n1 |  | 5th IMD into n1 |  |  | - |  |  | - Harmonic problem already covered in DC_1A_n28A - 5th IMD issue also covered in DC_20A_n1A-n28A |
| DC_3-20_n7-n28 |  | DC_3A_n7A, DC_3C_n7A | - |  | 2nd IMD into B20 2nd IMD into n28 |  |  | - |  |  | - 2nd IMD issue will be covered in DC_3A-20A_n7A - 2nd IMD issue will be covered in DC_3A_n7A-n28A |
|  |  | DC_3A_n28A, DC_3C_n28A | - |  | 3rd IMD into n7 |  |  | - |  |  | - 3rd IMD problem was solved in 3DL_DC_3A_n7A-n28A |
|  |  | DC_20A_n7A | - |  | 3rd & 5th IMDs into n28 |  |  | - |  |  | - These IMDs problem were solved in DC_20A_n7A-n28A |
|  |  | DC_20A_n28A | - |  | 5th IMD into n7 |  |  | - |  |  | - 5th IMD problem same as DC_7A-20A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3-20_n7-n78 |  | DC_3A_n7A | 2nd harmonic from 3 into n78 |  | 2nd IMD into B20 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD issue will be covered in DC_3A-20A_n7A - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  |  | DC_3A_n78A | - |  | - |  |  | Yes |  |  | No issue |
|  |  | DC_20A_n7A | 4th harmonic from B20 into n78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_20A_n7A-n78A |
|  |  | DC_20A_n78A |  |  | 2nd IMD into n7 3rd IMD into B3 |  |  | Yes |  |  | - 2nd IMD problem same as DC_7A-20A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - 3rd IMD issue will be covered in DC_3A-20A_n78A |
| DC_3-20_n1-n78 |  | DC_3A_n1A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - 2nd harmonic will be covered in DC_3A_n78A - The MSD issue will be covered in 3DL_3A_n1A-n78A |
|  |  | DC_3A_n78A | - |  | 5th IMD into n1 |  |  | - |  |  | - The 5th IMD issue will be covered in DC_3A_n1A-n78A |
|  |  | DC_20A_n1A | 4th harmonic from B20 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem already solved in DC_20A_n78A - 3rd IMD will be covered in DC_20A_n1A-n78A |
|  |  | DC_20A_n78A | - |  | 3rd IMD into B3 3rd IMD into n1 |  |  | Yes |  |  | - 3rd IMD issue will be covered in DC_3A-20A_n78A - 3rd IMD issue will be covered in DC_20A_n1A-n78A |
| DC_3-20_n28-n78 |  | DC_3A_n28A | 2nd harmonic into n78 5th harmonic into n78 |  | 5th IMD into n78 |  |  | - |  |  | - Harmonic problems already solved in DC_3A_n78A or CA_n28A-n78A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  |  | DC_3A_n78A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_20A_n28A | 4th harmonic from B20 into n78 5th harmonic from n28 into n78 |  | 4th & 5th IMDs into n78 |  |  |  |  |  | - Harmonic problems already solved in DC_20A_n78A or CA_n28A-n78A - These IMD issue will be covered in DC_20A_n28A-n78A |
|  |  | DC_20A_n78A | - |  | 3rd IMD into B3 4th IMD into n28 |  |  | - |  |  | - 3rd IMD issue will be covered in DC_3A-20A_n78A - 4th IMD issue will be covered in DC_20A_n28A-n78A |
| DC_2-66_n41-n71 DC_2-66_n41(2A)-n71 |  | DC_2A_n41A | - |  | 2nd & 5th IMDs into n71 |  |  | - |  |  | - These IMD issue will be covered in DC_2A_n41A-n71A |
|  |  | DC_2A_n71A | 4th harmonic from n71 into n41 |  | 2nd IMD into n41 |  |  | - |  |  | - The harmonic problem will be treated CA_n41A_n71A - 2nd IMD issue will be covered in DC_2A_n41A-n71A |
|  |  | DC_66A_n41A |  |  | 4th IMD into B2 |  |  | - |  |  | - 4th IMD issue will be covered in DC_2A-66A_n41A |
|  |  | DC_66A_n71A | 4th harmonic from n71 into n41 |  | - |  |  | - |  |  | The harmonic problem will be treated CA_n41A_n71A |
| DC_1-18_n3-n78 |  | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  |  | DC_1A_n78A |  |  | 2nd IMD into n3 5th IMD into B18 |  |  | Yes |  |  | - The 2nd IMD will be covered in DC_1A_n3A-n78A - The 5th IMD will be covered in DC_1A-18A_n78A |
|  |  | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 |  | 3rd & 5th IMDs into n78 |  |  | - |  |  | - These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issues will be covered in DC_18A_n3A-n78A |
|  |  | DC_18A_n78A | - |  | 3rd IMD into n3 3rd IMD into B1 |  |  | - |  |  | - 3rd IMD issue will be covered in DC_18A_n3A-n78A - 3rd IMD issue will be covered in DC_1A-18A_n78A |
| DC_1-28_n3-n78 |  | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  |  | DC_1A_n78A | - |  | 2nd IMD into n3 5th IMD into B28 |  |  | Yes |  |  | - The 2nd IMD will be covered in DC_1A_n3A-n78A - The 5th IMD will be covered in DC_1A-28A_n78A |
|  |  | DC_28A_n3A | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 3rd harmonic from B28 into B1 |  | 4th IMD into B1 5th IMD into n78 |  |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. or CA_1A-28A - 4th IMD problem will be covered in DC_1A-28A_n3A - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  |  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into n3 |  |  | - |  |  | - 3rd harmonic will be covered in CA_1A-28A - 3rd IMD issue will be covered in DC_1A-28A_n78A - 3rd IMD will be covered in DC_28A_n3A-n78A |
| DC_1-8_n3-n28 |  | DC_1A_n3A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_1A_n28A | - |  | 5th IMD into n3 5th IMD into B8 |  |  | Yes |  |  | - 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 - 5th IMD issue will be covered in DC_1A-8A_n28A |
|  |  | DC_8A_n3A | - |  | 2nd & 5th IMDs into n28 |  |  | - |  |  | - These IMD issues will be covered in DC_8A_n3A-n28A |
|  |  | DC_8A_n28A | 2nd harmonic from B8 into n3 3rd harmonic from n28 into B1 |  | 4th IMD into B1 |  |  | - |  |  | These harmonic problem will be covered in DC_8A_n3A or DC_1A_n28A 4th IMD issue will be covered in DC_1A-8A_n28A |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 |  | DC_3A_n1A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - 2nd harmonic will be covered in DC_3A_n78A The MSD issue will be covered in 3DL_3A_n1A-n78A |
|  |  | DC_3A_n78A | - |  | 5th IMD into n1 |  |  | - |  |  | - The 5th IMD issue will be covered in DC_3A_n1A-n78A |
|  |  | DC_8A_n1A | 2nd harmonic from B8 into B3 4th harmonic from B8 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - The harmonic problem already covered in CA_3A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  |  | DC_8A_n78A | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 |  |  | - |  |  | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 |  | DC_7A_n1A | - |  | 4th & 5th IMDs into n78 5th IMD into B8 |  |  | Yes |  |  | - These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_7A-8A_n1A |
|  |  | DC_7A_n78A |  |  | 2nd & 5th IMDs into B8 4th IMD into n1 |  |  | Yes |  |  | - 2nd & 5th IMDs will be covered in DC_7A-8A_n78A - 4th IMD will be covered in DC_7A_n1A-n78A |
|  |  | DC_8A_n1A | 3rd harmonic from B8 into B7 4th harmonic from B8 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - The harmonic problem already covered in CA_7A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  |  | DC_8A_n78A | 3rd harmonic from B8 into B7 |  | 2nd IMD into B7 |  |  | Yes |  |  | - The harmonic problem already covered in CA_7A-8A - 2nd IMD issue will be covered in DC_7A-8A_n78A |
| DC_1-3_n38-n78 |  | DC_3A_n38A | 2nd harmonic from B3 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | - The harmonic problem already covered in DC_3A_n78A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  |  | DC_3A_n78A | - |  | 5th IMD into B1 |  |  | - |  |  | - The 5th IMD issue will be covered in DC_1A-3A_n78A |
| DC_3-20_n38-n78 |  | DC_3A_n38A | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B20 3rd IMD into n78 |  |  | - |  |  | - The harmonic problem already covered in DC_3A_n78A - 2nd & 3rd IMDs issue will be covered in DC_3A-20A_38A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  |  | DC_3A_n78A | - |  | - |  |  | - |  |  | No issue |
| DC_1-20_n3-n78 |  | DC_1A_n3A | 2nd harmonic from 3 into n78 |  | 2nd & 4th IMDs into n78 |  |  | Yes |  |  | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  |  | DC_20A_n3A | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 3rd & 5th IMDs into n78 |  |  | - |  |  | - Harmonic problems was already covered in DC_3A_n78A or DC_20A_n78A. - 3rd &5th IMDs problems will be covered in DC_20A_n3A-n78A. |
| DC_1-20_n3-n38 |  | DC_1A_n3A | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_20A_n3A | 3rd harmonic from B20 into n38 |  | 2nd & 3rd IMDs into n38 |  |  | - |  |  | - Harmonic problems was already covered in DC_20A_n38A. - These IMDs problems will be covered in DC_20A_n3A-n38A. |
| DC_7-20_n3-n78 |  | DC_7A_n3A | 2nd harmonic from n3 into n78 |  | 2nd IMD into B20 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - 2nd IMD problem will be covered in DC_7A-20A_n3A - 3rd IMD problem already covered in DC_7A_n3A-n78A |
|  |  | DC_20A_n3A | - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 |  | 2nd & 3rd IMDs into nB7 3rd & 5th IMDs into n78 |  |  | - |  |  | - Harmonic problems was already covered in DC_3A_n78A or DC_20A_n78A. - 2nd & 3rd IMDs problems will be covered in DC_7A-20A_n3A. - 3rd &5th IMDs problems will be covered in DC_20A_n3A-n78A. |
| DC_2-66_n66-n78 |  | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs into n78 5th IMD into B66 |  |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A - 5th IMD will be covered in DC_2A-66A_n66A |
|  |  | DC_2A_n78A | - |  | 4th IMD into B66/n66 |  |  | - |  |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  |  | DC_66A_n66A2 | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - For the DC_66A_n66A, only allowed single uplink transmission. So the 2nd & 4th IMDs do not impact to n78 |
|  |  | DC_66A_n78A | - |  | 2nd , 4th & 5th IMDs into B2 5th IMD into n66 |  |  |  |  |  | - 2nd , 4th & 5th IMDs issue will be covered in DC_2A-66A_n78A - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_7-66_n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  |  | DC_7A_n78A | - |  | 4th IMD into B66/n66 |  |  | Yes |  |  | - The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  |  | DC_66A_n66A2 | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - For the DC_66A_n66A, only allowed single uplink transmission. So the 2nd & 4th IMDs do not impact to n78 |
|  |  | DC_66A_n78A |  |  | 5th IMD into n66 |  |  |  |  |  | - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 |  | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A |
|  |  | DC_2A_n78A | - |  | 4th IMD into n66 |  |  | - |  |  | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  |  | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 |  |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  |  | DC_7A_n78A | - |  | 4th IMD into n66 4th IMD into B2 |  |  | Yes |  |  | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
| DC_7-7-66-n66-n78 |  | DC_7A_n66A | 2nd harmonic from n66 into n78 |  | 3rd IMD into n78 4th IMD into B7 |  |  | - |  |  | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A 4th IMD issue will be covered in DC_7A-7A_n66A |
|  |  | DC_7A_n78A | - |  | 4th IMD into B66/n66 |  |  | Yes |  |  | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
|  |  | DC_66A_n66A2 | 2nd harmonic from B66 into n78 |  | 2nd & 4th IMDs into n78 |  |  |  |  |  | - Harmonic problem already covered in DC_66A_n78A - For the DC_66A_n66A, only allowed single uplink transmission. So the 2nd & 4th IMDs do not impact to n78 |
|  |  | DC_66A_n78A |  |  | 5th IMD into n66 |  |  |  |  |  | - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_1-18_n3-n257 |  | DC_1A_n3A | High order harmonic into n257 |  | - |  |  | -- |  |  | No harmonic impact to n257 |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_18A_n3A | High order harmonics into n257 |  | - |  |  | - |  |  | No harmonic problem to n257 |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_1-28_n3-n257 |  | DC_1A_n3A | High order harmonic into n257 |  | - |  |  | -- |  |  | No harmonic impact to n257 |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_28A_n3A | 3rd harmonic from B28 into B1 |  | 4th IMD into B1 |  |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. or CA_1A-28A - 4th IMD problem will be covered in DC_1A-28A_n3A |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_1-41_n78-n257 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B41 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 |  | - |  |  | - |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 4th IMD into B1 |  |  | - |  |  | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_1-42_n78-n257 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 4th IMD into B42 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - B42 and n78 will be synchronous operation. |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_3-41_n78-n257 |  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - |  |  | - Up to 7th harmonic order will be not any impact to n257 |
|  |  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd & 4th IMDs into B3 |  |  | - |  |  | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_3-42_n78-n257 |  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 |  | 4th & 5th IMDs into B42 - |  |  | - |  |  | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These MSD issues are already covered in 3DL_DC_3A-42A_n78A with 2UL_DC_3A_n78A |
|  |  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_41-42_n78-n257 |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - |  |  | - No harmonic impact to n257 |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_1-18_n78-n257 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B18 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD issue will be covered in DC_1A-18A_n78A |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_3-18_n78-n257 |  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  |  |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  |  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  |  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B3 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_18-42_n78-n257 |  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  |  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_3-28_n78-n257 |  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  |  |  |  |  |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  |  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 |  | - |  |  | - |  |  | - Harmonic problem will be solved in DC_3A_n78A |
|  |  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into n3 |  |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD will be covered in DC_28A_n3A-n78A |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_28-41_n78-n257 |  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | 2nd IMD into B41 |  |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 - 2nd IMD issue will be covered in DC_28A-41A_n78A |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 |  | 2nd & 5th IMDs into B28 |  |  | - |  |  | - No harmonic impact to n257 - 2nd & 5th IMDs will be covered in DC_28A-41A_n78A |
|  |  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_28-42_n78-n257 |  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 |  | - |  |  | - |  |  | Up to 7th harmonic order will be not any impact to n257 |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_1-28_n78-n257 |  | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 5th IMD into B28 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMD issue will be covered in DC_1A-28A_n78A |
|  |  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | - |  | - |  |  | - |  |  | No issue |
|  |  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 |  | -3rd IMD into B1 |  |  | - |  |  | - 3rd harmonic already covered in CA_1A-28A Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A |
|  |  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - |  | - |  |  | - |  |  | No issue |
| DC_1-11_n77-n257 |  | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 2nd & 5th IMDs into B11 |  |  | Yes |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs issue will be covered in DC_1A-11A_n77A |
|  |  | DC_1A_n257A | 2nd harmonic from B1 into n77 |  | - |  |  | - |  |  | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  |  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 |  | 2nd & 5th IMDs into B1 |  |  | - |  |  | No harmonic problem to n257 - These IMDs issue will be covered in DC_1A-11A_n77A |
|  |  | DC_11A_n257A | - |  | - |  |  | - |  |  | No issue |
| DC_8-11_n77-n257 |  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 |  | 3rd & 4th IMDs into B11 |  |  | - |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 The MSD issue will be covered in 3DL_DC_8A-11A_n77A |
|  |  | DC_8A_n257A | 4th harmonic from B8 into n77 |  | - |  |  | - |  |  | - Harmonic problem will be solved in DC_8A_n77A |
|  |  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 |  | 3rd & 4th IMDs into B8 |  |  | - |  |  | No harmonic problem to n257 - These IMDs issue will be covered in DC_8A-11A_n77A |
|  |  | DC_11A_n257A | - |  | - |  |  | - |  |  | No issue |
| DC_46-66_n25-n41 | DC_66A_n25A |  | 3rd harmonics into B46 |  | 3rd & 5th IMDs into B46 |  |  | - |  |  | - 3rd harmonics were covered in CA_46A-66A and DC_46A_n25A - 3rd & 5th IMDs will be covered in DC_46A-66A_n25A |
|  | DC_66A_n41A |  | 2nd harmonic from n41 into B46 |  | 4th IMD into n25 3rd & 4th IMDs into B46 |  |  | - |  |  | The harmonic was covered in DC_46A_n41A 4th IMD into n25 was covered in DC_66A_n25A-n41A 3rd & 4th IMDs into B46 were covered in DC_46A-66A_n41A |
| DC_46-66_n41-n71 DC_46-66_n41(2A)_n71 | DC_66A_n41A |  | 2nd harmonic from n41 into B46 |  | 3rd & 4th IMDs into B46 |  |  | - |  |  | The harmonic was covered in DC_46A_n41A 3rd & 4th IMDs into B46 were covered in DC_46A-66A_n41A |
|  | DC_66A_n71A |  | 4th harmonic from n71 into n41 3rd harmonic from B66 into B46 |  | 4th & 5th IMDs into B46 |  |  | - |  |  | These harmonic problem will be treated CA_n41A_n71A or CA_46A-66A The 4th & 5th IMDs were covered in DC_46A-66A_n71A |
| DC_2-46_n41-n71 | DC_2A_n41A |  | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 |  | 2nd & 5th IMDs into n71 4th IMD into B46 |  |  | - |  |  | - These harmonic problem were covered in DC_46A_n41A or CA_2A-46A - These IMD issue will be covered in DC_2A_n41A-n71A or DC_2A-46A_n41A |
|  | DC_2A_n71A |  | 4th harmonic from n71 into n41 3rd harmonic from B2 into B46 |  | 2nd IMD into n41 4th & 5th IMDs into B46 |  |  | - |  |  | - The harmonic problem will be treated CA_n41A_n71A or CA_2A-46A - 2nd IMD issue will be covered in DC_2A_n41A-n71A - 4th & 5th IMDs will be covered in DC_2A-46A_n71A |
| DC_2-46_n41-n66 | DC_2A_n41A |  | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 |  | 4th IMD into B46 |  |  | - |  |  | - These harmonic problem were covered in DC_46A_n41A or CA_2A-46A - These IMD issue will be covered in DC_2A-46A_n41A |
|  | DC_2A_n66A |  | 3rd harmonic from B2 into B46 |  | 3rd & 5th IMDs into B46 |  |  | - |  |  | - The harmonic problem was covered in CA_2A-46A - These IMDs issue will be covered in DC_2A-46A_n66A |
| DC_13-66_n5-n48 | DC_13A_n48A |  | - |  | 3rd IMD into B66 |  |  | - |  |  | - The IMD problem will be covered in DC_13A-66A_n48A |
|  | DC_66A_n5A |  | 2nd harmonic from n66 into n48 |  | 5th IMD into n48 4th IMD into B13 |  |  | - |  |  | - 2nd harmonic issue will be solved in CA_n48-n66 - These IMDs problem will be covered in DC_66A_n5A-n48A or DC_13A-66A_n5A |
|  | DC_66A_n48A |  | - |  | - |  |  | - |  |  | No issue |
| DC_7-28_n3-n78 | DC_7A_n3A DC_7C_n3A |  | 2nd harmonic from n3 into n78 |  | 2nd IMD into B28 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem already solved in CA_n3A-n78A - 2nd IMD problem will be covered in DC_7A-28A_n3A - 3rd IMD problem already covered in DC_7A_n3A-n78A |
|  | DC_7A_n78A DC_7C_n78A |  | - |  | 3rd & 4th IMDs into n3 2nd & 5th IMDs into B28 |  |  | - |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A - The 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  | DC_28A_n3A |  | 2nd harmonic from n3 into n78 5th harmonic from B28 into n78 |  | 3rd IMD into B7 5th IMD into n78 |  |  | - |  |  | - These harmonic issue already covered in CA_n3A-n78A or DC_28A_n78A. - 3rd IMD problem was covered in DC_7A-28A_n3A - 5th IMD problem same as DC_3A_n28A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_28A_n78A |  | - |  | 2nd IMD into B7 3rd IMD into n3 |  |  | - |  |  | - 2nd IMD will be covered in DC_7A-28A_n78A - 3rd IMD will be covered in DC_28A_n3A-n78A |
| DC_1-7_n7-n78 | DC_1A_n7A |  | - |  | 4th & 5th IMDs into n78 |  |  | - |  |  | These IMD problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |
|  | DC_1A_n78A |  | - |  | 4th IMD into B7/n7 |  |  | - |  |  | The 4th IMD will be covered in DC_1A-7A_n78A |
|  | DC_7A_n7A2 |  | - |  | - |  |  | - |  |  | No issue |
|  | DC_7A_n78A |  | - |  | 4th IMD into B1 |  |  | - |  |  | - 4th IMD will be covered in DC_7A_n1A-n78A |
| DC_3-7_n7-n78 DC_3-3-7_n7-n78 | DC_3A_n7A DC_3C_n7A |  | 2nd harmonic from 3 into n78 |  | 3rd IMD into B7 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem will be covered in DC_3A-7A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A |  | - |  | - |  |  | - |  |  | No issue |
|  | DC_7A_n7A2 |  | - |  | - |  |  | - |  |  | No issue |
|  | DC_7A_n78A |  | - |  | 3rd & 4th IMDs into B3 |  |  | - |  |  | - The 3rd & 4th IMDs will be covered in DC_3A-7A_n78A |
| DC_7-28_n7-n78 | DC_7A_n7A2 |  | - |  | - |  |  | - |  |  | No issue |
|  | DC_7A_n78A |  | - |  | 2nd & 5th IMDs into B28 |  |  | - |  |  | - The 2nd & 5th IMDs will be covered in DC_7A-28A_n78A |
|  | DC_28A_n7A |  | 5th harmonic from B28 into n78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A |  | - |  | 2nd IMD into B7/n7 |  |  | - |  |  | - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B |  | 2nd harmonic from 3 into n78 |  | 2nd IMD into B28 3rd IMD into n78 |  |  | - |  |  | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD problem will be covered in DC_3A-28A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A |  | - |  | - |  |  | - |  |  | No issue |
|  | DC_28A_n7A DC_28A_n7B |  | 5th harmonic from B28 into n78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A |  | - |  | 3rd IMD into B3 2nd IMD into n7 |  |  | - |  |  | - 3rd IMD will be covered in DC_3A-28A_n78A - 2nd IMD will be covered in DC_7A-28A_n78A |
| DC_1-28_n7-n78 | DC_1A_n7A DC_1A_n7B |  | - |  | 4th & 5th IMDs into n78 5th IMD into B28 |  |  | - |  |  | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A |  | - |  | 4th IMD into n7 5th IMD into B28 |  |  | - |  |  | The 4th IMD will be covered in DC_1A-7A_n78A 5th IMD will be covered in DC_1A-28A_n78A |
|  | DC_28A_n7A DC_28A_n7B |  | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 |  | 2nd & 4th IMDs into n78 |  |  | - |  |  | - 3rd harmonic issue will be covered in CA_1A-28A - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A |  | 3rd harmonic from B28 into B1 |  | - 2nd IMD into n7 - 3rd IMD into B1 |  |  | - |  |  | - 3rd harmonic already covered in CA_1A-28A - 2nd IMD will be covered in DC_7A-28A_n78A - 3rd IMD issue will be covered in DC_1A-28A_n78A |
| DC_1-11_n3-n28 | DC_1A_n3A |  | - |  | 3rd & 5th IMDs into B11 |  |  | - |  |  | - 3rd & 5th IMDs issue will be covered in DC_1A-11A_n3A |
|  | DC_1A_n28A |  | 2nd harmonic from n28 into B11 |  | 5th IMD into n3 |  |  | Yes |  |  | - 2nd harmonic problem will be covered in DC_11A_n28A - 5th IMD problem same as DC_1A-3A_n28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_11A_n3A |  | - |  | 3rd IMD into B1 5th IMD into n28 |  |  |  |  |  | - 3rd IMD issue will be covered in DC_1A-11A_n3A - 5th IMD issue will be covered in DC_11A_n3A-n28A |
|  | DC_11A_n28A |  | 3rd harmonic from n28 into B1 |  | 2nd & 3rd IMDs into B1 |  |  | - |  |  | 3rd harmonic will be covered in DC_1A_n28A 2nd & 3rd IMDs issue will be covered in DC_1A-11A_n28A |
| DC_8-11_n3-n28 | DC_8A_n3A |  | - |  | 2nd & 5th IMDs into n28 |  |  | - |  |  | - These IMD issues will be covered in DC_8A_n3A-n28A |
|  | DC_8A_n28A |  | 2nd harmonic from B8 into n3 2nd harmonic from n28 into B11 |  | - |  |  | - |  |  | The 2nd harmonic problem will be covered in DC_8A_n3A The 2nd harmonic problem will be covered in DC_11A_n28A |
|  | DC_11A_n3A |  | - |  | 5th IMD into n28 |  |  |  |  |  | - 5th IMD issue will be covered in DC_11A_n3A-n28A |
|  | DC_11A_n28A |  | - |  | - |  |  | - |  |  | No issue |
| DC_2-46_n66-n71 | DC_2A_n66A |  | 3rd harmonic from B2 into B46 |  | 3rd & 5th IMDs into B46 |  | - |  |  |  | - The harmonic problem was covered in CA_2A-46A - These IMDs issue will be covered in DC_2A-46A_n66A |
|  | DC_2A_n71A |  | 3rd harmonic from B2 into B46 |  | 4th & 5th IMDs into B46 |  | - |  |  |  | - The harmonic problem will be treated in CA_2A-46A - 4th & 5th IMDs will be covered in DC_2A-46A_n71A |
| DC_46-66_n25-n71 | DC_66A_n25A |  | 3rd harmonics into B46 |  | 3rd & 5th IMDs into B46 |  | - |  |  |  | - 3rd harmonics were covered in CA_46A-66A and DC_46A_n25A - 3rd & 5th IMDs will be covered in DC_46A-66A_n25A |
|  | DC_66A_n71A |  | 3rd harmonic from B66 into B46 3rd harmonic from n71 into n25 |  | 4th & 5th IMDs into B46 |  | - |  |  |  | These harmonic problem will be treated in CA_46A-66A and in CA_n25A-n71A. The 4th & 5th IMDs were covered in DC_46A-66A_n71A |
| DC_2-46_n41-n71 DC_2-46_n41(2A)-n71 | DC_2A_n41A |  | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 |  | 2nd & 5th IMDs into n71 4th IMD into B46 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_2A_n71A |  | 3rd harmonic from B2 into B46 4th harmonic from n71 into n41 |  | 2nd IMD into n41 4th & 5th IMDs into B46 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3_n40-n78 | DC_1A_n40A |  | - |  | 4th IMD & 5th IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | - |  | 2nd IMD into B3 4th IMD into n40 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n40A |  | 2nd harmonic from B3 into n78 |  | 5th IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A |  |  |  | 5th IMD into B1 & n40 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-28_n40-n78 | DC_1A_n40A |  | - |  | 4th IMD & 5th IMD into n78 4th IMD into B28 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | - |  | 4th IMD into n40 5th IMD into B28 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n40A |  | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 |  | 3rd & 4th IMDs into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n78A |  | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into n40 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_3-28_n40-n78 | DC_3A_n40A |  | 2nd harmonic from B3 into n78 |  | 5th IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A |  |  |  | 5th IMD into n40 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n40A |  | 5th harmonic from B28 into n78 |  | 3rd & 4th IMDs into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n78A |  | - |  | 3rd IMD into B3 3rd IMD into n40 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3_n28-n77 DC_1-3_n28(2A)-n77 | DC_1A_n28A |  | 5th harmonic from n28 into n78 |  | 3rd IMD into n78 5th IMD into n3 |  | Yes |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  |  |  | 2nd, 4th & 5th IMDs into B3 5th IMD into n28 |  | Yes |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n28A |  | 3rd harmonic from n28 into n1 2nd harmonic from B3 and 5th harmonic from n28 |  | 4th IMD into n1 3rd, 4th & 5th IMDs into n77 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A |  |  |  | 2nd & 5th IMD into B1 3rd IMD into n28 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-8_n28-n77 DC_1-8_n28(2A)-n77 | DC_1A_n28A |  | 5th harmonic from n28 into n77 |  | 5th IMD into B8 3rd & 4th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  |  |  | 5th IMD into n28 & n8 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n28A |  | 3rd harmonic from n28 into B1 harmonic from B8 and 5th harmonic from n28 into n77 |  | 4th IMD into B1 4th & 5th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n77A |  |  |  | 3rd IMD into B1 4th IMD into n28 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-42_n28-n77 DC_1-42_n28(2A)-n77 | DC_1A_n28A |  | 5th harmonic from n28 into n77 |  | 3rd IMD into B42 3rd & 4th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  |  |  | 4th & 5th IMDs into B42 5th IMD into n28 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_3-8_n28-n77 DC_3-8_n28(2A)-n77 | DC_3A_n28A |  |  |  | 3rd, 4th & 5th IMDs into n77 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A |  |  |  | 3rd IMD into n28 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n28A |  | harmonic from B8 and 5th harmonic from n28 into n77 2nd harmonic from B8 into B3 |  | 4th & 5th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n77A |  | 2nd harmonic from B8 into B3 |  | 3rd IMD into B3 4th IMD into n28 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_3-42_n28-n77 DC_3-42_n28-n77(2A) | DC_3A_n28A |  | 2nd harmonic from B3 into B42 & n77 5th harmonic from n28 into B42 & n77 |  | 3rd, 4th & 5th IMDs into n77 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A |  |  |  | 3rd IMD into n28 5th IMD into B42 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_8-42_n28-n77 DC_8-42_n28-n77(2A) | DC_8A_n28A |  | harmonic from B8 and 5th harmonic from n28 into n77 |  | 4th & 5th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n77A |  | 4th harmonic from B8 into B42 & n77 |  | 4th IMD into n28 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_2-66_n38-n78 | DC_2A_n38A |  | 2nd harmonic from B2 |  | 3rd IMD into n78 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_2A_n78A |  | - |  | 4th IMD into n66 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_66A_n38A |  | 2nd harmonic from B66 into n78 |  | 3rd IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_66A_n78A |  | - |  | 2nd , 4th & 5th IMDs into B2 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-18_n3-n77 | DC_18A_n3A |  | 2nd harmonic from n3 into n77 4th harmonic from B18 into n77 |  | 3rd 4th & 5th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n77A |  | - |  | 3rd IMD into B1 & n3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-28_n3-n77 | DC_28A_n3A |  | 2nd harmonic from n3 and 4th harmonic from B18 into n77 3rd harmonic from B28 into B1 |  | 3rd 4th & 5th IMDs into n77 4th IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n77A |  | 3rd harmonic from B28 into B1 |  | 3rd & 4th IMDs into n3 3rd IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-41_n3-n77 | DC_41A_n3A |  | 2nd harmonic from n3 into n77 |  | 3rd & 5th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A |  | - |  | 3rd & 4th IMDs into n3 4th IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-41_n3-n78 | DC_41A_n3A |  | 2nd harmonic from n3 into n78 |  | 3rd IMDs into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n78A |  | - |  | 3rd & 4th IMDs into n3 4th IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-41_n28-n77 | DC_1A_n28A |  | 5th harmonic from n28 into n77 |  | 4th IMD into B41 3rd & 4th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  |  |  | 4th & 5th IMDs into B41 5th IMD into n28 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n28A |  | 3rd harmonic from n28 into B1 5th harmonic from n28 into n77 |  | 2nd , 3rd & 4th IMDs into n77 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A |  | - |  | 2nd & 3rd IMDs into n28 4th IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-41_n28-n78 | DC_1A_n28A |  | 5th harmonic from n28 into n78 |  | 4th IMD into B41 3rd IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | - |  | 4th IMD into B41 5th IMD into n28 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n28A |  | 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n78A |  | - |  | 2nd & 5th IMDs into n28 4th IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_3-41_n28-n77 | DC_3A_n28A |  | 2nd harmonic from B3 into n77 5th harmonic from n28 into n77 |  | 2nd & 3rd IMDs into B41 3rd, 4th & 5th IMDs into n77 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A |  |  |  | 3rd IMD into n28 5th IMD into B41 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n28A |  | 5th harmonic from n28 into n77 |  | 2nd , 3rd & 4th IMDs into n77 2nd IMD into B3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A |  | - |  | 2nd & 3rd IMDs into n28 3rd & 4th IMDs into B3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_3-41_n28-n78 | DC_3A_n28A |  | 2nd harmonic from B3 into n78 5th harmonic from n28 into n78 |  | 2nd & 3rd IMDs into B41 5th IMD into n78 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A |  |  |  |  |  |  |  |  |  | No issue |
|  | DC_41A_n28A |  | 5th harmonic from n28 into n78 |  | 2nd & 4th IMDs into n78 2nd IMD into B3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n78A |  | - |  | 2nd & 5th IMDs into n28 3rd & 4th IMDs into B3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_18-41_n3-n77 | DC_18A_n3A |  | 2nd harmonic from n3 into n77 4th harmonic from B18 into n77 |  | 3rd 4th & 5th IMDs into n77 2nd & 3rd IMDs into B41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n77A |  | - |  | 3rd IMD into n3 2nd & 3rd IMDs into B41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n3A |  | 2nd harmonic from n3 into n77 |  | 3rd & 5th IMDs into n77 2nd & 3rd IMDs into B18 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A |  | - |  | 2nd & 3rd IMDs into B18 3rd & 4th IMDs into n3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_18-41_n3-n78 | DC_18A_n3A |  | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 |  | 3rd & 5th IMDs into n78 2nd & 3rd IMDs into B41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n78A |  |  |  | 2nd IMD into B41 3rd IMD into n3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n3A |  | 2nd harmonic from n3 into n78 |  | 3rd IMDs into n78 2nd & 3rd IMDs into B18 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n78A |  | - |  | 2nd & 5th IMDs into B18 3rd & 4th IMDs into n3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3_n41-n78 | DC_1A_n41A |  | - |  | 3rd , 4th & 5th IMDs into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | - |  | 2nd IMD into B3 4th IMD into n41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n41A |  | 2nd harmonic from B3 into n78 |  | 3rd IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A |  |  |  | 5th IMD into B1 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-20_n41-n78 | DC_1A_n41A |  | - |  | 3rd , 4th & 5th IMDs into n78 5th IMD into B20 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | - |  | 5th IMD into B20 4th IMD into n41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_20A_n41A |  | 4th harmonic from B20 into n78 |  | 2nd & 4th IMDs into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_20A_n78A |  | 3rd harmonic from B20 into n41 |  | 3rd IMD into B1 2nd IMD into n41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_3-20_n41-n78 | DC_3A_n41A |  | 2nd harmonic from B3 into n78 |  | 2nd & 3rd IMDs into B20 3rd IMD into n78 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A |  | - |  | - |  | - |  |  |  | No issues |
|  | DC_20A_n41A |  | 4th harmonic from B20 into n78 |  | 2nd & 4th IMDs into n78 2nd IMD into B3 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_20A_n78A |  | 3rd harmonic from B20 into n41 |  | 3rd IMD into B3 2nd IMD into n41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3_n28-n257 | DC_1A_n28A |  | High order harmonics from B1 or n28 into n257 |  | 5th IMD into B3 |  | Yes |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257 |  | - |  | - |  | - |  |  |  | No issues |
|  | DC_3A_n28A |  | 3rd harmonic from n28 into B1 High order harmonics from B3 or n28 into n257 |  | 4th IMD into B1 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A |  | - |  | - |  | - |  |  |  | No issues |
| DC_1-41_n3-n257 | DC_41A_n3A DC_41C_n3A |  | High order harmonics from B41 or n3 into n257 |  | - |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257 DC_41C_n257 |  | - |  | - |  | - |  |  |  | No issues |
| DC_1-41_n28-n257 | DC_1A_n28A |  | High order harmonics from B1 or n28 into n257 |  | 4th IMD into B41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A DC_1A_n257I |  | - |  | - |  | - |  |  |  | No issues |
|  | DC_41A_n28A |  | 3rd harmonic from n28 into B1 High order harmonics from B41 or n28 into n257 |  | - |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I |  | - |  | - |  | - |  |  |  | No issues |
| DC_3-41_n28-n257 | DC_3A_n28A |  | High order harmonics from B3 or n28 into n257 |  | 2nd & 3rd IMDs into B41 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A DC_3A_n257I |  |  |  |  |  |  |  |  |  | No issues |
|  | DC_41A_n28A |  | High order harmonics from B41 or n28 into n257 |  | 2nd IMD into B3 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I |  |  |  |  |  |  |  |  |  | No issues |
| DC_1-41_n77-n257 | DC_1A_n77A |  | High order harmonics from B1 or n77 into n257 |  | 4th IMD into B41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A DC_1A_n257I |  | - |  | - |  | - |  |  |  | No issues |
|  | DC_41A_n77A |  | High order harmonics from B41 or n77 into n257 |  | - |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I |  | - |  | - |  | - |  |  |  | No issues |
| DC_3-41_n77-n257 | DC_3A_n77A |  | High order harmonics from B3 or n77 into n257 |  | 5th IMD into B41 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A DC_3A_n257I |  | 2nd harmonic from B3 into n77 |  |  |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A |  | High order harmonics from B41 or n77 into n257 |  | 3rd & 4th IMDs into B3 |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I |  |  |  |  |  |  |  |  |  | No issues |
| DC_18-41_n3-n257 | DC_18A_n3A |  | High order harmonics from B18 or n3 into n257 |  | 2nd & 3rd IMDs into B41 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n257A |  | - |  | - |  | - |  |  |  | No issues |
|  | DC_41A_n3A |  | High order harmonics from B41 or n3 into n257 |  | 2nd & 3rd IMDs into B18 |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A |  | - |  | - |  | - |  |  |  | No issues |
| DC_41-42_n77-n257 | DC_41A_n77A |  | High order harmonics from B41 or n77 into n257 |  | - |  | - |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41A_n257I |  | - |  | - |  | - |  |  |  | No issues |
| DC_3-7_n1-n257 DC_3-3-7_n1-n257 DC_3-7-7_n1-n257 DC_3-3-7-7_n1-n257 | DC_3A_n1A |  | High order harmonics from B3 or n1 into n257 |  |  |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257 |  |  |  |  |  |  |  |  |  | No issues |
|  | DC_7A_n1A |  | High order harmonics from B7 or n1 into n257 |  |  |  |  |  |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_7A_n257A |  |  |  |  |  |  |  |  |  | No issues |
| Note 1: These band combinations should be completed the dual uplink EN-DC firstly Note 2: only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3-3 summarizes the EN-DC band combinations with self-interference problems for 5DL/2UL DC operation.

Table 5.3-3: Summary of Self-interference analysis for LTE 3 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1-3-5_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A and DC_3A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
| DC_1-3-7_n28-n78 | DC_1A_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd IMD into B7 3rd IMD into n78 | Yes | - Harmonic problems already solved in DC_1A_n78A or DC_n28A-n78A - 2nd IMD problem was solved in 3DL_DC_1A-7A_n28A - 3rd IMD problem was solved in 3DL_DC_1A_n28A-n78A |
|  | DC_1A_n78A | - | 2nd IMD into B3 4th IMD into B1 4th IMD into n78 5th IMD into n28 | Yes | - 2nd IMD problem was solved in 3DL_DC_1A-3A_n78A - 4th IMD problem was solved in DC_1A-n78A - 4th IMD problem is no issue for TDD band - 5th IMD problem was solved in 3DL_DC_1A_n28A-n78A |
|  | DC_3A_n28A | 2nd harmonic into n78 3rd harmonic into B1 5th harmonic into n78 | 3rd IMD into B7 4th IMD into B1 5th IMD into n78 |  | - Harmonic problems already solved in DC_3A_n78A or DC_1A_n28A or DC_28A-n78A - 3rd IMD problem was solved in 3DL_DC_3A-7A_n28A - 4th IMD problem was solved in 3DL_DC_1A-3A_n28A - 5th IMD problem was solved in 3DL_DC_3A_n28A-n78A |
|  | DC_3A_n78A | 2nd harmonic into n78 | 2nd,4th & 5th IMDs into B3 4th & 5th IMDs into n78 5th IMD into B1 |  | - 2nd harmonic problem already solved in DC_3A-n78A - 2nd,4th & 5th IMDs problems already solved in DC_3A-n78A - 4th & 5th IMDs problems are no issue for TDD band -5th IMD problem was solved in 3DL_DC_1A-3A_n78A |
|  | DC_7A_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd IMD into B3 2nd,4th IMDs into n78 |  | - Harmonic problem was solved in DC_1A_n28A or DC_n28A-n78A - 2nd IMD problem was solved in 3DL_DC_3A-7A_n28A - 2nd & 4th IMDs problems were solved in 3DL_DC_7A_n28A-n78A |
|  | DC_7A_n78A | - | 2nd & 5th IMDs into n28 3rd & 4th IMDs into B3 4th IMD into n78 4th IMD into B1 |  | - 2nd & 5th IMDs problems were solved in 3DL_DC_7A-28A_n78A - 3rd & 4th IMDs problem were solved in 3DL_DC_3A-7A_n78A - 4th IMD problem is no issue for TDD band - 4th IMD problem was solved in 3DL_DC_1A-7A_n78A |
| DC_1-3-7-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B7 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-7A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 3rd & 4th IMDs into B3 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B7 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-7A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 3rd & 4th IMDs into B3 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-8_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B8 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A and DC_1A-8A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 | 3rd IMD into B3 | - | - Harmonic problem already be solved in CA_3A_8A - 3rd IMD already covered in DC_3A-8A_n78A |
|  | DC_8A_n257A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem will be solved in DC_8A_n78A |
| DC_1-5-7-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B7 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-5-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B7 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-5A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 4th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_1A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_3-5-7-7_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 3rd & 4th IMDs into B3 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_3A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_3-5-7_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 | 2nd & 5th IMDs into B5 3rd & 4th IMDs into B3 | - | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problems will be covered in DC_5A-7A_n78A and DC_3A-7A_n78A |
|  | DC_7A_n257A | - | - | - | No issues |
| DC_1-3-21_n77-n78 | DC_3A_n77A | 2nd harmonic from B3 into n78 | 2nd & 5th IMDs into B1 4th & 5th IMDs into B21 4th & 5th IMDs into n78 | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n77A and 3DL_DC_3A-21A_n77A and 3DL_DC_3A_n77A-n78A with 2UL_DC_3A_n77A |
|  | DC_3A_n78A | 2nd harmonic from B3 into n77 | 5th IMD into B1 4th & 5th IMDs into B21 4th & 5th IMDs into n77 | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A and 3DL_DC_3A_n77A-n78A with 2UL_DC_3A_n78A |
| DC_1-3-21_n77-n79 | DC_3A_n77A | - | 2nd & 5th IMDs into B1 4th & 5th IMDs into B21 3rd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n77A and 3DL_DC_3A-21A_n77A and 3DL_DC_3A_n77A-n79A with 2UL_DC_3A_n77A |
|  | DC_3A_n79A | 2nd harmonic from B3 into n77 | 5th IMD into B1 3rd IMD into B21 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_3A_n77A These MSD issues are already covered in 3DL_DC_1A-3A_n79A and 3DL_DC_3A-21A_n79A and 3DL_DC_3A_n77A-n79A with 2UL_DC_3A_n79A |
|  | DC_1A_n77A | - | 2nd, 4th & 5th IMDs into B3 2nd & 5th IMDs into B21 3rd, 4th & 5th IMD into n79 | Yes | - These IMDs issue will be covered in DC_1A-3A_n77A - 2nd & 5th IMDs will be covered in DC_1A-21A_n77A - 3rd, 4th & 5th IMDs will be covered in DC_1A_n77A-n79A |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 4th IMD into B21 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-21A_n79A The MSD issue are already covered in 3DL_DC_1A_n77A-n79A |
|  | DC_21A_n77A |  | 2nd & 5th IMDs into B1 2nd & 5th IMDs into B3 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A These MSD issues are already covered in 3DL_DC_3A-21A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A |  | 3rd IMD into B3 2nd IMD into n77 |  | These MSD issues are already covered in DC_3A-21A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_1-3-21_n78-n79 | DC_3A_n78A | - | 5th IMD into B1 4th & 5th IMDs into B21 3rd IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A and 3DL_DC_3A_n78A-n79A with 2UL_DC_3A_n78A |
|  | DC_3A_n79A | 2nd harmonic from B3 into n78 | 5th IMD into B1 3rd IMD into B21 5th IMD into n78 | - | 2nd harmonic issue already covered in DC_3A_n78A These MSD issues are already covered in 3DL_DC_1A-3A_n79A and 3DL_DC_3A-21A_n79A and 3DL_DC_3A_n78A-n79A with 2UL_DC_3A_n79A |
|  | DC_1A_n78A | - | 2nd IMD into B3 2nd & 5th IMDs into B21 3rd & 5th IMD into n79 | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n78A. These MSD issues are already covered in 3DL_DC_1A-21A_n78A. These MSD issues are already covered in 3DL_DC_1A_n78A-n79A |
|  | DC_1A_n79A | - | 4th IMD into B21 5th IMD into n78 | - | The MSD issue are already covered in 3DL_DC_1A-21A_n79A The MSD issue are already covered in 3DL_DC_1A_n78A-n79A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd IMDs into B3 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A These MSD issue is already covered in 3DL_DC_3A-21A_n78A. These MSD issue is already covered in 3DL_DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 3rd IMD into B3 2nd IMD into n78 | - | These MSD issues are already covered in DC_3A-21A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_1-3-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 3rd IMD into n78 | Yes | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 | 4th IMD into B1 5th IMD into n78 |  | Harmonic issues are already covered in DC_3A_n78A, DC_1A_n28A and CA_n28A-n78A. These MSD issues are already covered in 3DL_DC_1A-3A_n28A with 2UL_DC_3A_n28A and 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_28A_n28A1 | 3rd harmonic from B28 into B1 | 3rd & 5th IMDs into B1 5th IMD into n78 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n28 | Yes | The MSD issue will be covered in 3DL_1A-3A_n78A with 2UL_DC_1A_n78A and 3DL_1A_n28A-n78A with 2UL_DC_1A_n78A |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A with 2UL_DC_3A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n28 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A and 3DL_DC_3A-28A_n28A-n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_1-7-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 2nd IMD into B7 3rd IMD into n78 | - | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_1A-7A_n28A and 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_7A_n28A DC_7C_n28A | 3rd harmonic into B1 5th harmonic into n78 | 2nd,4th IMDs into n78 | - | Harmonic issue already covered in DC_1A_n28A or CA_n28A-n78A These MSD issues are already covered in 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |
|  | DC_28A_n28A1 | 3rd harmonic from B28 into B1 | 3rd & 5th IMDs into B1 5th IMD into n78 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_1A-28A_n28A with 2UL_DC_28A_n28A and 3DL_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_1A_n78A | - | 4th IMD into B7 5th IMD into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_1A-7A_n78A with 2UL_DC_1A_n78A and 3DL_DC_1A_n28A-n78A with 2UL_DC_1A_n78A |
|  | DC_7A_n78A DC_7C_n78A | - | 4th IMD into B1 2nd & 5th IMDs into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_1A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 2nd IMD into B7 5th IMD into n28 |  | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in 3DL_DC_1A-28A_n78A and 3DL_DC_7A-28A_n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_1-19-42_n77-n79 | DC_19A_n77A | 6th harmonic from B19 into n79 | 3rd IMD into B1 2nd, 3rd , 4th & 5th IMDs into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_1A-19A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n77 | 4th IMD into B1 2nd IMD into B42 2nd & 3rd IMDs into n77 | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_1A-19A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n79A |
|  | DC_1A_n77A |  | 5th IMD into B19 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-19A_n77A. These MSD issues are already covered in 3DL_DC_1A-42A_n77A. These MSD issues are already covered in 3DL_DC_1A_n77A-n79A. |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 3rd & 4th IMDs into B19 5th IMD into B42 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-19A_n79A The MSD issue are already covered in 3DL_DC_1A-42A_n79A The MSD issue are already covered in 3DL_DC_1A_n77A-n79A |
| DC_1-19-42_n78-n79 | DC_19A_n78A | 6th harmonic from B19 into n79 | 3rd IMD into B1 2nd, 3rd, 4th & 5th IMD into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_1A-19A_n78A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n78 | 4th IMD into B1 2nd IMD into B42 2nd & 3rd IMDs into n78 | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_1A-19A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n79A |
|  | DC_1A_n78A | - | 5th IMD into B19 4th IMD into B42 3rd & 5th IMD into n79 | - | The MSD issue will be covered in 3DL_DC_1A-19A_n78A. The MSD issue will be covered in 3DL_DC_1A-42A_n78A. These MSD issue will be covered in 3DL_DC_1A_n78A-n79A. |
|  | DC_1A_n79A | - | 3rd & 4th IMDs into B19 5th IMD into B42 5th IMD into n78 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-19A_n79A The MSD issue are already covered in 3DL_DC_1A-42A_n79A The MSD issue are already covered in 3DL_DC_1A_n78A-n79A |
| DC_1-21-42_n77-n78 | DC_1A_n77A | - | 2nd & 5th IMDs into B21 4th & 5th IMD into B42 4th & 5th IMD into n78 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A and 3DL_DC_1A-42A_n77A and 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n77A |
|  | DC_1A_n78A | 2nd harmonic from B1 into n77 | 2nd & 5th IMDs into B21 4th IMD into B42 4th & 5th IMD into n77 | - | The harmonic already covered in DC_1A_n77A These MSD issues are already covered in 3DL_DC_1A-21A_n78A and 3DL_DC_1A-42A_n78A and 3DL_DC_1A_n77A-n78A with 2UL_DC_1A_n78A. |
| DC_1-21-42_n77-n79 | DC_1A_n77A | - | 2nd & 5th IMDs into B21 4th & 5th IMD into B42 3rd, 4th & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A and 3DL_DC_1A-42A_n77A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n77A |
|  | DC_1A_n79A | 2nd harmonic from B1 into n77 | 4th IMD into B21 5th IMD into B42 5th IMD into n77 | - | 2nd harmonic issue already covered in DC_1A_n77A The MSD issue are already covered in 3DL_DC_1A-21A_n79A and 3DL_DC_1A-42A_n79A and 3DL_DC_1A_n77A-n79A with 2UL_DC_1A_n79A |
|  | DC_21A_n77A | - | 2nd & 5th IMDs into B1 5th IMD into B42 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n77A These MSD issues are already covered in 3DL_DC_21A-42A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A | - | 2nd IMD into B42 2nd IMD into n77 | - | These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_1-21-42_n78-n79 | DC_1A_n78A | - | 2nd & 5th IMDs into B21 4th IMD into B42 3rd & 5th IMD into n79 | - | These MSD issues are already covered in 3DL_DC_1A-21A_n78A and 3DL_DC_1A-42A_n78A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n78A. |
|  | DC_1A_n79A | - | 4th IMD into B21 5th IMD into B42 5th IMD into n78 | - | The MSD issue are already covered in 3DL_DC_1A-21A_n79A and 3DL_DC_1A-42A_n79A and 3DL_DC_1A_n78A-n79A with 2UL_DC_1A_n79A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A. These MSD issue is already covered in 3DL_DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 2nd IMD into B42 2nd IMD into n78 | - | These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_3-7-28_n28-n78 | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 5th harmonic from n28 into n78 | 3rd IMD into B7 5th IMD into n78 |  | Harmonic issues are already covered in DC_3A_n78A or CA_n28A-n78A. These MSD issues are already covered in 3DL_DC_3A-7A_n28A with 2UL_DC_3A_n28A and 3DL_DC_3A_n28A-n78A with 2UL_DC_3A_n28A |
|  | DC_7A_n28A DC_7C_n28A | 5th harmonic from n28 into n78 | 2nd IMD into B3 2nd,4th IMDs into n78 | - | Harmonic issue already covered in CA_n28A-n78A These MSD issues are already covered in 3DL_DC_3A-7A_n28A with 2UL_DC_7A_n28A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n28A |
|  | DC_28A_n28A1 | - | 5th IMD into n78 | - | The MSD issue will be covered in 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n28A |
|  | DC_3A_n78A DC_3C_n78A | - | - | - | No issue |
|  | DC_7A_n78A DC_7C_n78A | - | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 or n28 | - | The MSD issue will be covered in 3DL_DC_3A-7A_n78A with 2UL_DC_7A_n78A and 3DL_DC_7A_n28A-n78A with 2UL_DC_7A_n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into B7 5th IMD into n28 |  | The MSD issue will be covered in 3DL_DC_3A-28A_n78A and 3DL_DC_7A-28A_n78A and 3DL_DC_28A_n28A-n78A with 2UL_DC_28A_n78A |
| DC_19-21-42_n77-n78 | DC_19A_n77A | 4th harmonic from B19 into n78 | 4th IMD into B21 2nd IMD into n78 | - | 4th harmonic problem already covered in DC_19A_n78A These MSD issues are already covered in 3DL_DC_19A-21A_n77A and 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n77A. |
|  | DC_19A_n78A | 4th harmonic from B19 into n77 | 2nd & 5th IMDs into n77 | - | 4th harmonic problem already covered in DC_19A_n77A These MSD issues are already covered in 3DL_DC_19A_n77A-n78A with 2UL_DC_19A_n78A. |
| DC_19-21-42_n77-n79 | DC_19A_n77A | 6th harmonic from B19 into n79 | 4th IMD into B21 2nd, 3rd , 4th & 5th IMDs into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A-21A_n77A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n77A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n77 | 5th IMD into B21 2nd IMD into B42 2nd & 3rd IMDs into n77 | - | 4th harmonic will be covered in DC_19A_n77A The MSD issue will be covered in 3DL_DC_19A-21A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n77A-n79A with 2UL_DC_19A_n79A |
|  | DC_21A_n77A | - | 3rd & 4th IMDs into B19 5th IMD into B42 2nd & 4th IMDs into n79 | - | These MSD issues are already covered in 3DL_DC_19A-21A_n77A These MSD issues are already covered in 3DL_DC_21A-42A_n77A These MSD issues are already covered in 3DL_21A_n77A-n79A |
|  | DC_21A_n79A | - | 5th IMD into B19 2nd IMD into B42 2nd IMD into n77 | - | These MSD issues are already covered in DC_19A-21A_n79A These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n77A-n79A |
| DC_19-21-42_n78-n79 | DC_19A_n78A | 6th harmonic from B19 into n79 | 2nd, 3rd, 4th & 5th IMD into n79 | - | 6th harmonic problem already covered in DC_19A_n79A These MSD issues are already covered in 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n78A. |
|  | DC_19A_n79A | 4th harmonic from B19 into n78 | 5th IMD into B21 2nd IMD into B42 2nd & 3rd IMDs into n78 | - | 4th harmonic will be covered in DC_19A_n78A The MSD issue will be covered in 3DL_DC_19A-21A_n79A and 3DL_DC_19A-42A_n79A and 3DL_DC_19A_n78A-n79A with 2UL_DC_19A_n79A |
|  | DC_21A_n78A | - | 3rd & 4th IMDs into B19 2nd & 4th IMDs into n79 | - | These MSD issue is already covered in DC_19A-21A_n78A. These MSD issue is already covered in DC_21A_n78A-n79A |
|  | DC_21A_n79A | - | 5th IMD into B19 2nd IMD into B42 2nd IMD into n78 | - | These MSD issues are already covered in DC_19A-21A_n79A These MSD issues are already covered in DC_21A-42A_n79A These MSD issues are already covered in DC_21A_n78A-n79A |
| DC_1-3-7_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - These IMD issue will be covered in DC_3A-7A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 4th IMD into B7 | - | The MSD issue will be covered in 3DL_DC_1A-3A_n78A. The MSD issue will be covered in 3DL_DC_1A_n5A-n78A. The MSD issue will be covered in 3DL_DC_1A-7A_n78A. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_7A_n78A, DC_7C_n78A |  | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 | - | The MSD issue will be covered in DC_1A-7A_n78A. The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. |
| DC_1-3-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 4th IMD into B28 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 4th IMD into B3 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 4th IMD will be covered in DC_3A-28A_n5A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 5th IMD into B28 | - | The MSD issue will be covered in 3DL_DC_1A-3A_n78A. The MSD issue will be covered in 3DL_DC_1A_n5A-n78A. The MSD issue will be covered in 3DL_DC_1A-28A_n78A. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n5 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in DC_1A-28A_n78A. The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. |
| DC_1-7-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 5th IMD into B7 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_1A_n78A | - | 5th IMD into n5 4th IMD into B7 5th IMD into B28 | - | The MSD issue will be covered in DC_1A_n5A-n78A. The MSD issue will be covered in DC_1A-7A_n78A The MSD issue will be covered in DC_1A-28A_n78A.. |
|  | DC_7A_n78A, DC_7C_n78A |  | 4th IMD into B1 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_1A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 5th IMD into n5 2nd IMD into B7 | - | Harmonic issue already covered in DC_1A_n28A The MSD issue will be covered in DC_1A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_3-7-28_n5-n78 | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 4th IMD into B28 3rd & 5th IMDs into n78 | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-7A_n5A - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 4th IMD into B3 5th IMD into B7 |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 4th IMD will be covered in DC_3A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_3A_n78A, DC_3C_n78A | - | - | - | No issue |
|  | DC_7A_n78A |  | 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n78A | - | 3rd IMD into B3 5th IMD into n5 2nd IMD into B7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_28A_n5A-n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 5th IMD into B66 2nd & 4th IMDs into n78 | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - 5th IMD will be covered in DC_2A-66A_n66A - 2nd & 4th IMDs will be covered in DC_2A_n66A-n78A |
|  | DC_2A_n78A | - | 4th IMD into B66/n66 | - | 4th IMD problem same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD into n78 | - | The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 3rd IMD issue will be covered in DC_7A_n66A-n78A |
|  | DC_7A_n78A | - | 4th IMD into n66 4th IMD into B2 | Yes | The 4th IMD problem same as DC_7A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 4th IMD issue will be covered in DC_2A-7A_n78A |
|  | DC_66A_n66A2 | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic problem already covered in DC_66A_n78A - For the DC_66A_n66A, only allowed single uplink transmission. So the 2nd & 4th IMDs do not impact to n78 |
|  | DC_66A_n78A | - | 2nd , 4th & 5th IMDs into B2 5th IMD into n66 |  | - 2nd , 4th & 5th IMDs issue will be covered in DC_2A-66A_n78A - 5th IMD issue will be covered in DC_66A_n66A-n78A |
| DC_1-3-20_n38-n78 | DC_3A_n78A | - | 5th IMD into B1 | - | The MSD issue will be covered in 3DL_1A-3A_n78A. |
|  | DC_20A_n78A | 3rd harmonic from B20 into n38 | 3rd IMD into B1 3rd IMD into B3 2nd IMD into n38 | - | The 3rd harmonic issue will be covered in DC_20A_n38A - 3rd IMD issue will be covered in DC_1A-20A_n78A - 3rd IMD issue will be covered in DC_3A-20A_n78A - 2nd IMD issue will be covered in DC_20A_n38A-n78A |
|  | DC_3A_n38A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B20 3rd IMD into n78 | - | - The harmonic problem already covered in DC_3A_n78A - 2nd & 3rd IMDs issue will be covered in DC_3A-20A_38A - 3rd IMD issue will be covered in DC_3A_n38A-n78A |
|  | DC_20A_n38A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs into n78 |  | - The harmonic problem already covered in DC_20A_n78A - 2nd & 4th IMDs issue will be covered in DC_20A_n38A-n78A |
| DC_1-20-38_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
|  | DC_20A_n3A | 3rd harmonic from B20 into n38 - 2nd harmonic from n3 into n78 - 4th harmonic from B20 into n78 | 2nd & 3rd IMDs into n38 3rd & 5th IMDs into n78 | - | - Harmonic problems was already covered in DC_20A_n38A or DC_20A_n78A or CA_n3A-n78A. - These IMDs problems will be covered in DC_20A_n3A-n38A - These IMDs problems will be covered in DC_20A_n3A-n78A. |
|  | DC_1A_n78A | - | 2nd IMD into n3 5th IMD into B20 4th IMD into B38 | Yes | The MSD issue will be covered in DC_1A_n3A-n78A. The MSD issue will be covered in DC_1A-20A_n78A. The MSD issue will be covered in DC_1A-38A_n78A |
|  | DC_20A_n78A | 3rd harmonic from B20 into B38 | 3rd IMD into B1 3rd IMD into n3 2nd IMD into B38 | - | The 3rd harmonic issue will be covered in CA_20A-38A - 3rd IMD issue will be covered in DC_1A-20A_n78A - 3rd IMD issue will be covered in DC_20A_n3A-n78A - 2nd IMD issue will be covered in DC_20A-38A_n78A |
| DC_1-7-20_n3-n78 | DC_1A_n3A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_1A_n3A-n78A. |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 | DC_3A_n1A | 2nd harmonic from B3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems will be covered in DC_3A_n1A-n78A. |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | The MSD issue will be covered in 3DL_3A_n1A-78A. |
|  | DC_7A_n1A | - | 4th & 5th IMDs into n78 5th IMD into B8 | Yes | - These IMD problems were already covered in DC_7A_n1A-n78A with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_7A-8A_n1A |
|  | DC_7A_n78A | - | 4th IMD into n1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B8 | Yes | - Up to 7th harmonic order will be not any impact to n257 - 4th IMD problem will be covered in DC_7A_n1A-n78A 3rd & 4th IMDs issue will be covered in DC_3A-7A_n78A 2nd & 5th IMDs issue will be covered in DC_7A-8A_n78A |
|  | DC_8A_n1A | 2nd harmonic from B8 into B3 3rd harmonic from B8 into B7 4th harmonic from B8 into n78 | 3rd IMD into n78 | - | - The harmonic problem already covered in CA_3A-8A or CA_7A-8A or DC_8A_n78A - 3rd IMD issue will be covered in DC_8A_n1A-n78A |
|  | DC_8A_n78A | 2nd harmonic from B8 into B3 3rd harmonic from B8 into B7 | 3rd IMD into B3 2nd IMD into B7 | -Yes | - Harmonic problem already be solved in CA_3A_8A or CA_7A-8A - 3rd IMD already covered in DC_3A-8A_n78A - 2nd IMD issue will be covered in DC_7A-8A_n78A |
| DC_1-3-41_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B41 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-41A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 3rd & 4th IMDs into B3 | - | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_1-3-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - 5th IMD issue will be covered in DC_1A-3A_n78A - These MSD issues are already covered in 3DL_DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-41-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 4th IMD into B41 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-41A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 | - | - No harmonic impact to n257 - 4th IMD issue will be covered in DC_1A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-41-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These MSD issues are already covered in 3DL_DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 | - | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-3-18_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B18 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-18A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
| DC_1-18-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 5th IMD into B18 4th IMD into B42 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-18A_n78A - The IMD problem will be covered in DC_1A-42A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_1A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-18-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These IMDs issue will be covered in DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD issue will be covered in DC_3A-18A_n78A |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_3-28-41_n78-n257 | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - Up to 7th harmonic order will be not any impact to n257 |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B41 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_3A-28A_n78A - 2nd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 | - | - No harmonic impact to n257 - 3rd & 4th IMDs issue will be covered in DC_3A-41A_n78A - 2nd & 5th IMDs issue will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
| DC_3-28-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 | - | - 2nd harmonic already covered in CA_3A-42A and Up to 7th harmonic order will be not any impact to n257 - These IMDs issue will be covered in DC_3A-42A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_3A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_28-41-42_n78-n257 | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd IMD into B41 | - | Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_28A-41A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 2nd & 5th IMDs into B28 | - | - No harmonic impact to n257 - 2nd & 5th IMDs issue will be covered in DC_28A-41A_n78A |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-3-28_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B28 | Yes | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-3A_n78A - The IMD problem will be covered in DC_1A-28A_n78A |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 | - | - 3rd harmonic already covered in CA_1A-28A and Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A - 3rd IMD issue will be covered in DC_3A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
| DC_1-28-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 5th IMD into B28 4th IMD into B42 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_1A-28A_n78A - The IMD problem was not impact to B42 due to synchronous TDD operation |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 | - | - 3rd harmonic already covered in CA_1A-28A and Up to 7th harmonic order will be not any impact to n257 - 3rd IMD issue will be covered in DC_1A-28A_n78A |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - | - | No issue |
| DC_1-8-11_n77-n257 | DC_1A_n77A | 7th & 8th Harmonics from n77 into n257 | 5th IMD into B8 2nd & 5th IMDs into B11 | Yes | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 5th IMDs issue will be covered in DC_1A-8A_n77A - These IMDs issue will be covered in DC_1A-11A_n77A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 | - | - | - Harmonic problem already discussed and will be solved in DC_1A_n77A |
|  | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | 3rd IMD into B1 3rd & 4th IMDs into B11 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - The MSD issue will be covered in 3DL_DC_1A-8A_n77A - The MSD issue will be covered in 3DL_DC_8A-11A_n77A |
|  | DC_8A_n257A | 4th harmonic from B8 into n77 | - | - | - Harmonic problem will be solved in DC_8A_n77A |
|  | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | 2nd & 5th IMDs into B1 3rd & 4th IMDs into B8 | - | No harmonic problem to n257 - These IMDs issue will be covered in DC_1A-11A_n77A - These IMDs issue will be covered in DC_8A-11A_n77A |
|  | DC_11A_n257A | - | - | - | No issue |
| DC_2-46-66_n41-n71 | DC_2A_n41A | 2nd harmonic from n41 into B46 3rd harmonic from B2 into B46 | 4th IMD into B46 2nd & 5th IMDs into n71 | - | - These harmonic problem were covered in DC_46A_n41A or CA_2A-46A - 4th IMD issue will be covered in DC_2A-46A_n41A - 2nd & 5th IMDs issue will be covered in DC_2A_n41A-n71A |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 3rd harmonic from B2 into B46 | 2nd IMD into n41 4th & 5th IMDs into B46 | - | - The harmonic problem will be treated CA_n41A_n71A or CA_2A-46A - 2nd IMD issue will be covered in DC_2A_n41A-n71A - 4th & 5th IMDs will be covered in DC_2A-46A_n71A |
|  | DC_66A_n41A | 2nd harmonic from n41 into B46 | 3rd & 4th IMDs into B46 4th IMD into B2 | - | - The harmonic was covered in DC_46A_n41A - 3rd & 4th IMDs into B46 were covered in DC_46A-66A_n41A - 4th IMD will be covered in DC_2A-66A_n41A |
|  | DC_66A_n71A | 4th harmonic from n71 into n41 3rd harmonic from B66 into B46 | 4th & 5th IMDs into B46 | - | These harmonic problem will be treated CA_n41A_n71A or CA_46A-66A The 4th & 5th IMDs were covered in DC_46A-66A_n71A |
| DC_3-7-28_n7-n78 | DC_3A_n7A | 2nd harmonic from 3 into n78 | 2nd IMD into B28 3rd IMD into n78 3rd IMD into B7 | - | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD problem will be covered in DC_3A-28A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. - 3rd IMD problem already covered in DC_3A_n7A. |
|  | DC_3A_n78A | - | - | - | No issue |
|  | DC_7A_n7A2 | - | - | - | No issue |
|  | DC_7A_n78A | - | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 | - | The MSD issue will be covered in DC_3A-7A_n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
|  | DC_28A_n7A | 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 | - | - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into B7/n7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_1-3-28_n7-n78 | DC_1A_n7A DC_1A_n7B | - | 4th & 5th IMDs into n78 5th IMD into B28 | - | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A |  | 2nd IMD into B3 5th IMD into B28 4th IMD into n7 | Yes | - 2nd IMD problem will be covered in DC_1A-3A_n78A - 5th IMD problem will be covered in DC_1A-28A_n78A - 4th IMD problem will be covered in DC_1A_n7A-n78A |
|  | DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | 2nd harmonic from 3 into n78 | 2nd IMD into B28 3rd IMD into n78 | - | - Harmonic problem was solved in DC_3A_n78A - 2nd IMD problem will be covered in DC_3A-28A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_28A_n7A DC_28A_n7B | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 | - | - 3rd harmonic issue will be covered in CA_1A-28A - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - | 3rd IMD into B3 2nd IMD into n7 | - | The MSD issue will be covered in DC_3A-28A_n78A. The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_1-3-7_n7-n78 | DC_1A_n7A | - | 4th & 5th IMDs into n78 | - | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A |
|  | DC_1A_n78A |  | 2nd IMD into B3 4th IMD into B7/n7 | Yes | - 2nd IMD problem will be covered in DC_1A-3A_n78A - 4th IMD problem will be covered in DC_1A_n7A-n78A |
|  | DC_3A_n7A DC_3C_n7A | 2nd harmonic from 3 into n78 | 3rd IMD into B7 3rd IMD into n78 | - | - Harmonic problem was solved in DC_3A_n78A - 3rd IMD problem will be covered in DC_3A-7A_n7A. - 3rd IMD problem already covered in DC_3A_n7A-n78A. |
|  | DC_3A_n78A DC_3C_n78A | - | 5th IMD into B1 | - | - The IMD issue will be covered in DC_1A-3A_n78A |
|  | DC_7A_n7A2 | - | - | - | No issue |
|  | DC_7A_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 | Yes | - 4th IMD problem will be covered in DC_7A_n1A-n78A 3rd & 4th IMDs issue will be covered in DC_3A-7A_n78A |
| DC_1-3-21_n78-n257 | DC_1A_n78A | - | 2nd IMD into B3 2nd & 5th IMDs into B21 | Yes | These MSD issues are already covered in 3DL_DC_1A-3A_n78A. These MSD issues are already covered in 3DL_DC_1A-21A_n78A. |
|  | DC_3A_n78A | - | 5th IMD into B1 4th & 5th IMDs into B21 | - | These MSD issues are already covered in 3DL_DC_1A-3A_n78A and 3DL_DC_3A-21A_n78A |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 2nd IMDs into B3 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A These MSD issue is already covered in 3DL_DC_3A-21A_n78A. |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_21A_n257A | - | - | - | No issue |
| DC_1-19-42_n78-n257 | DC_1A_n78A | - | 5th IMD into B19 4th IMD into B42 | - | The MSD issue will be covered in 3DL_DC_1A-19A_n78A. The MSD issue will be covered in 3DL_DC_1A-42A_n78A. |
|  | DC_19A_n78A | - | 3rd IMD into B1 | - | The MSD issue already covered in 3DL_DC_1A-19A_n78A with 2UL_DC_19A_n78A. |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_19A_n257A | 4th harmonic from B19 into n78 | - | - | 4th harmonic problem was solved in DC_19A_n78A |
|  | DC_42A_n257A | - | - | - | No issue |
| DC_1-21-42_n78-n257 | DC_1A_n78A | - | 5th IMD into B19 4th IMD into B42 | - | The MSD issue will be covered in 3DL_DC_1A-19A_n78A. The MSD issue will be covered in 3DL_DC_1A-42A_n78A. |
|  | DC_21A_n78A | - | 2nd & 5th IMDs into B1 | - | These MSD issue is already covered in 3DL_DC_1A-21A_n78A. |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_21A_n257A | - | - | - | No issue |
|  | DC_42A_n257A | - | - | - | No issue |
| DC_1-7-28_n7-n78 | DC_1A_n7A | - | 4th & 5th IMDs into n78 5th IMD into B28 | - | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A |  | 5th IMD into B28 4th IMD into B7/n7 | Yes | - 5th IMD problem will be covered in DC_1A-28A_n78A - 4th IMD problem will be covered in DC_1A_n7A-n78A |
|  | DC_7A_n7A2 | - | - | - | No issue |
|  | DC_7A_n78A | - | 4th IMD into B1 2nd & 5th IMDs into B28 | Yes | - 4th IMD problem will be covered in DC_7A_n1A-n78A - 2nd & 5th IMDs problem will be covered in DC_7A-28A_n78A |
|  | DC_28A_n7A | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 | - | - 3rd harmonic issue will be covered in CA_1A-28A - 5th harmonic issue will be covered in DC_28-n78 - 2nd & 4th IMDs will be covered in DC_28A_n7A-n78A |
|  | DC_28A_n78A | - | 2nd IMD into n7 | - | The MSD issue will be covered in DC_7A-28A_n78A. |
| DC_19-21-42_n78-n257 | DC_19A_n78A | - | - | - | No issue |
|  | DC_19A_n257A | 4th harmonic from B19 into n78 | - | - | 4th harmonic problem was solved in DC_19A_n78A |
|  | DC_21A_n78A | - | 3rd & 4th IMDs into B19 | - | These MSD issue is already covered in DC_19A-21A_n78A. |
|  | DC_21A_n257A | - | - | - | No issues |
|  | DC_42A_n257A | - | - | - | No issue |
| DC_1-3-28_n40-n78 | DC_1A_n40A | - | 4th IMD & 5th IMD into n78 4th IMD into B28 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | 5th IMD into B28 2nd IMD into B3 4th IMD into n40 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n40A | 2nd harmonic from B3 into n78 | 5th IMD into n78 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A | - | 5th IMD into B1 5th IMD into n40 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n40A | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 3rd & 4th IMDs into n78 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_28A_n78A | - | 3rd IMD into B3 3rd IMD into n40 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3-8_n28-n77 DC_1-3-8_n28-n77(2A) | DC_1A_n28A | 5th harmonic from n28 into n77 | 5th IMD into B3 3rd & 4th IMDs into n77 5th IMD into B8 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  | 5th IMD into B8 2nd, 4th & 5th IMDs into B3 5th IMD into n28 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n28A | 2nd harmonic from B3 into n77 5th harmonic from n28 into n77 3rd harmonic from n28 into B1 | 4th IMD into B1 3rd, 4th & 5th IMDs into n77 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A |  | 2nd & 5th IMDs into B1 3rd IMD into n28 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n28A | 4th harmonic from B8 and 5th harmonic from n28 into n77 2nd harmonic from B8 into B3 3rd harmonic from n28 into B1 | 4th & 5th IMDs into n77 4th IMD into B1 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n77A | 2nd harmonic from B8 into B3 | 3rd IMD into B1 3rd IMD into B3 4th IMD into n28 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-8-42_n28-n77 DC_1-8-42_n28-n77(2A) | DC_1A_n28A | 5th harmonic from n28 into n77 | 3rd IMD into B42 3rd & 4th IMDs into n77 5th IMD into B8 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  | 5th IMD into B8 5th IMD into n28 4th & 5th IMD into B42 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n28A | 4th harmonic from B8 and 5th harmonic from n28 into n77 3rd harmonic from n28 into B1 | 4th & 5th IMDs into n77 4th IMD into B1 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_8A_n77A |  | 3rd IMD into B1 4th IMD into n28 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3-41_n28-n77 | DC_1A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 5th IMD into B3 4th IMD into B41 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n77A |  | 2nd, 4th & 5th IMDs into B3 5th IMD into n28 4th IMD into B41 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n28A | 2nd harmonic from B3 into n77 5th harmonic from n28 into n77 3rd harmonic from n28 into B1 | 4th IMD into B1 3rd, 4th & 5th IMDs into n77 2nd & 3rd IMDs into B41 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A |  | 2nd & 5th IMDs into B1 3rd IMD into n28 5th IMD into B41 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n28A | 5th harmonic from n28 into n77 3rd harmonic from n28 into B1 | 2nd , 3rd & 4th IMDs into n77 2nd IMD into B3 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A |  | 3rd & 4th IMDs into B3 2nd & 3rd IMDs into n28 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3-41_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 | 3rd IMD into n78 5th IMD into B3 4th IMD into B41 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n78A |  | 5th IMD into B28 2nd IMD into B3 4th IMD into B41 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n28A | 2nd harmonic from B3 into n78 5th harmonic from n28 into n78 3rd harmonic from n28 into B1 | 4th IMD into B1 5th IMD into n78 2nd & 3rd IMDs into B41 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n78A | - | 5th IMD into B1 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n28A | 5th harmonic from n28 into n78 3rd harmonic from n28 into B1 | 2nd & 4th IMDs into n78 2nd IMD into B3 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n78A |  | 2nd & 5th IMDs into n28 3rd & 4th IMDs into B3 4th IMD into B1 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-18-41_n3-n77 | DC_18A_n3A | 2nd harmonic from n3 into n77 4th harmonic from B18 into n77 | 3rd 4th & 5th IMDs into n77 2nd & 3rd IMDs into B41 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n77A | - | 3rd IMD into B1 3rd IMD into n3 2nd & 3rd IMDs into B41 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n3A DC_41C_n3A | 2nd harmonic from n3 into n77 | 3rd & 5th IMDs into n77 2nd & 3rd IMDs into B18 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A DC_41C_n77A |  | 3rd & 4th IMDs into B3 2nd & 3rd IMDs into B18 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-18-41_n3-n78 | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 | 3rd & 5th IMDs into n78 2nd & 3rd IMDs into B41 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n78A |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B41 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n3A DC_41C_n3A | 2nd harmonic from n3 into n78 | 3rd IMD into n78 2nd & 3rd IMDs into B18 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n78A DC_41C_n78A |  | 4th IMD into B1 3rd & 4th IMDs into n3 2nd & 5th IMDs into B18 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-3-41_n28-n257 | DC_1A_n28A | High order harmonic from B1 or n28 into n257 | 5th IMD into B3 4th IMD into B41 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A | - |  | - | No issues |
|  | DC_3A_n28A | High order harmonic from B3 or n28 into n257 3rd harmonic from n28 into B1 | 4th IMD into B1 2nd & 3rd IMDs into B41 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A | - |  | - | No issues |
|  | DC_41A_n28A | High order harmonic from B41 or n28 into n257 3rd harmonic from n28 into B1 | 2nd IMD into B3 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A | - |  | - | No issues |
| DC_1-3-41_n77-n257 | DC_1A_n77A | High order Harmonics from B1 or n77 into n257 | 2nd, 4th & 5th IMDs into B3 4th IMD into B41 | Yes | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A | High order Harmonics from B3 or n77 into n257 | 2nd & 5th IMDs into B1 5th IMD into B41 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A | 2nd harmonic from B3 into n77 |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A | High order Harmonics from B41 or n77 into n257 | 3rd & 4th IMDs into B3 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A | - |  | - | No issues |
| DC_1-3-42_n77-n257 | DC_1A_n77A | High order Harmonics from B1 or n77 into n257 | 2nd, 4th & 5th IMDs into B3 4th & 5th IMD into B42 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A | High order Harmonics from B3 or n77 into n257 | 2nd & 5th IMDs into B1 5th IMD into B42 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A | 2nd harmonic from B3 into n77 |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
| DC_1-18-41_n3-n257 | DC_18A_n3A | High order harmonic from B18 or n3 into n257 | 2nd & 3rd IMDs into B41 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_18A_n257A DC_18A_n257I | - | - | - | No issues |
|  | DC_41A_n3A DC_41C_n3A | High order harmonic from B41 or n3 into n257 | 2nd & 3rd IMDs into B18 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41C_n257A DC_41A_n257I DC_41C_n257I | - | - | - | No issues |
| DC_1-41-42_n77-n257 | DC_1A_n77A | High order Harmonics from B1 or n77 into n257 | 4th IMD into B41 4th & 5th IMD into B42 | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A | 2nd harmonic from B1 into n77 | - | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A | High order Harmonics from B41 or n77 into n257 | - |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A | - | - | - | No issues |
| DC_3-41-42_n77-n257 | DC_3A_n77A | High order Harmonics from B3 or n77 into n257 | 5th IMD into B41 5th IMD into B42 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A | 2nd harmonic from B3 into n77 |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A | High order Harmonics from B41 or n77 into n257 | 3rd & 4th IMDs into B3 |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A | - | - | - | No issues |
| Note 1: These band combinations should be completed the dual uplink EN-DC firstly Note 2: Only single switched UL is supported. |  |  |  |  |  |

Table 5.3-4 summarizes the EN-DC band combinations with self-interference problems for 6DL/2UL DC operation.

Table 5.3-4: Summary of Self-interference analysis for LTE 4 bands & NR 2 bands DL and 2 bands UL EN-DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues |  | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1-3-5-7_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 |  | 2nd IMD into B3 4th IMD into B7 5th IMD into B5 | Yes | - Up to 7th harmonic order will be not any impact to n257 - These IMDs problem will be covered in DC_1A-3A_n78A, DC_1A-7A_n78A and DC_1A-5A_n78A - Proximity also follow LTE_CA_1A-3A |
|  | DC_1A_n257A | 2nd harmonic from B1 into n78 |  | - | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 |  | 5th IMD into B1 | - | - Up to 7th harmonic order will be not any impact to n257 - 5th IMD problems will be covered in DC_1A-3A_n78A. |
|  | DC_3A_n257A | 2nd harmonic from B3 into n78 |  | - | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_5A_n78A | 7th & 8th harmonics from n78 into n257 |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 - 3rd IMD problems will be covered in DC_1A-5A_n78A, DC_3A-5A_n78A and DC_5A-7A_n78A. |
|  | DC_5A_n257A | 4th harmonic from B5 into n78 |  | - | - | - Harmonic problem will be solved in DC_5A_n78A |
|  | DC_7A_n78A | 7th harmonic from B7 into n257 |  | 3rd & 4th IMDs into B3 4th IMD into B1 2nd & 5th IMDs into B5 | - | - Up to 7th harmonic order will be not any impact to n257 - The IMD problem will be covered in DC_3A-7A_n78A, DC_1A-7A_n78A and DC_5A-7A_n78A |
|  | DC_7A_n257A | - |  | - | - | No issues |
| DC_1-3-7-28_n28-n78 | DC_1A_n28A | 5th harmonic from n28 into n78 |  | 2nd IMD into B7 3rd IMD into n78 | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n28A DC_3C_n28A | 2nd harmonic from B3 into n78 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 4th IMD into B1 3rd IMD into B7 5th IMD into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n28A DC_7C_n28A | 3rd harmonic from n28 into B1 5th harmonic from n28 into n78 |  | 2nd IMD into B3 2nd,4th IMDs into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n28A1 | 3rd harmonic from n28 into B1 |  | 3rd & 5th IMDs into B1 5th IMD into n78 | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n78A | - |  | 2nd IMD into B3 4th IMD into B7 5th IMD into B28 or n28 | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A DC_3C_n78A | - |  | 5th IMD into B1 | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n78A DC_7C_n78A | - |  | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 or n28 | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 |  | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 5th IMD into n28 | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-7-28_n5-n78 | DC_1A_n5A | 4th harmonic from n5 into n78 | 3rd IMD into n78 |  | - | - 4th harmonic already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - The 3rd IMD issue will be covered in DC_1_n5A-n78A |
|  | DC_3A_n5A, DC_3C_n5A | 2nd harmonic from B3 into n78 | 2nd & 3rd IMDs into B7 4th IMD into B28 3rd & 5th IMDs into n78 |  | - | - 2nd harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The IMD issue will be covered in DC_3A-7A_n5A - The IMD issue will be covered in DC_3A-28A_n5A These IMD issue will be covered in DC_3A_n5A-n78A |
|  | DC_7A_n5A, DC_7C_n5A | - | 3rd & 5th IMDs into B28 2nd & 4th IMDs into n78 |  | - | These IMDs problem will be covered in DC_7A-28A_n5A These IMDs problem will be covered in DC_7A_n5A-n78A |
|  | DC_28A_n5A | 3rd harmonic from B28 into B1 4th harmonic from n5 into n78 5th harmonic from B28 into n78 | 5th IMD into B1 4th IMD into B3 5th IMD into B7 |  |  | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - The 5th IMD will be covered in DC_1A-28A_n5A - The 4th IMD will be covered in DC_3A-28A_n5A - The 5th IMD will be covered in DC_7A-28A_n5A |
|  | DC_1A_n78A | - | 2nd IMD into B3 5th IMD into n5 4th IMD into B7 5th IMD into B28 |  | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A, DC_3C_n78A | - | 5th IMD into B1 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n78A, DC_7C_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into n5 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 5th IMD into n5 2nd IMD into B7 |  | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-41-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 4th IMD into B41 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | - Harmonic problem will be solved in DC_3A_n78A |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 4th IMD into B1 3rd & 4th IMDs into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-18-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B18 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_18A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_18A_n257A, DC_18A_n257G, DC_18A_n257H, DC_18A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-28-42_n78-n257 | DC_1A_n78A | 7th & 8th Harmonics from n78 into n257 | 2nd IMD into B3 5th IMD into B28 4th IMD into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n78 | - |  | Yes | - Harmonic problem already discussed and will be solved in DC_1A_n78A |
|  | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 5th IMD into B1 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 7th & 8th harmonics from n78 into n257 | 3rd IMD into B1 3rd IMD into B3 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_3-28-41-42_n78-n257 | DC_3A_n78A | 2nd harmonic from B3 into B42 7th & 8th harmonics from n78 into n257 | 4th & 5th IMDs into B42 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n78 | - |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd IMD into B3 2nd IMD into B41 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n257A, DC_28A_n257G, DC_28A_n257H, DC_28A_n257I | - | - |  | - | No issue |
|  | DC_41A_n78A | 7th & 8th harmonics from n78 into n257 | 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_41A_n257A, DC_41A_n257G, DC_41A_n257H, DC_41A_n257I | - | - |  | - | No issue |
|  | DC_42A_n257A, DC_42A_n257G, DC_42A_n257H, DC_42A_n257I | - | - |  | - | No issue |
| DC_1-3-7-28_n7-n78 | DC_1A_n7A | - | 4th & 5th IMDs into n78 5th IMD into B28 |  | Yes | - 4th & 5th IMDs problems were already covered in DC_7_n1-n78 with 2UL_DC_7A_n1A - 5th IMD issue will be covered in DC_1A-28A_n7A |
|  | DC_1A_n78A | - | 2nd IMD into B3 4th IMD into B7 5th IMD into B28 |  | Yes | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n7A | 2nd harmonic from 3 into n78 | 3rd IMD into B7 2nd IMD into B28 3rd IMD into n78 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_3A_n78A | - | 5th IMD into B1 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_7A_n7A2 | - | - |  | - | No issue |
|  | DC_7A_n78A | - | 4th IMD into B1 3rd & 4th IMDs into B3 2nd & 5th IMDs into B28 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n7A | 3rd harmonic from B28 into B1 5th harmonic from B28 into n78 | 2nd & 4th IMDs into n78 |  | - | All desense problems are covered in lower fallback mode. |
|  | DC_28A_n78A | 3rd harmonic from B28 into B1 | 3rd IMD into B1 3rd IMD into B3 2nd IMD into B7 |  | - | All desense problems are covered in lower fallback mode. |
| DC_1-3-41-42_n77-n257 | DC_1A_n77A | High order Harmonics from B1 or n77 into n257 | 2nd, 4th & 5th IMDs into B3 4th IMD into B41 4th & 5th IMD into B42 |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_1A_n257A, DC_1A_n257G, DC_1A_n257H, DC_1A_n257I | 2nd harmonic from B1 into n77 | - |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n77A | High order Harmonics from B3 or n77 into n257 | 2nd & 5th IMDs into B1 5th IMD into B41 5th IMD into B42 |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_3A_n257A, DC_3A_n257G, DC_3A_n257H, DC_3A_n257I | 2nd harmonic from B3 into n77 | - |  | - | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n77A | High order Harmonics from B41 or n77 into n257 | 3rd & 4th IMDs into B3 |  |  | All harmonic and IMD problem were covered in lower DC or CA band combinations |
|  | DC_41A_n257A DC_41A_n257G,DC_41A_n257H, DC_41A_n257I | - | - |  | - | No issue |
| Note 1: These band combinations should be completed the dual uplink EN-DC firstly Note 2: Only single switched UL is supported. |  |  |  |  |  |  |

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-5: Proposed MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) |  | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n78A | 1 | IMD2 | ¦fB1 +fn3¦ | 1950 | 5 |  | 25 | 2140 | 5 | 2.1 | N/A |  |
|  | n3 |  |  | 1750 |  | 5 | 25 | 1845 | 5 |  |  |  |
|  | n78 |  |  | 3700 |  | 10 | 52 | 3700 | 10 |  | 28.4 |  |
|  | 1 | IMD4 | ¦3*fn3 -fB1¦ | 1950 |  | 5 | 25 | 2140 | 5 | 1.2 | N/A |  |
|  | n3 |  |  | 1770 |  | 5 | 25 | 1865 | 5 |  |  |  |
|  | n78 |  |  | 3360 |  | 10 | 52 | 3360 | 10 |  | 11.2 |  |
|  | 1 | IMD2 | ¦fn78 -fB1¦ | 1950 |  | 5 | 25 | 2140 | 5 | 1.9 | N/A |  |
|  | n78 |  |  | 3780 |  | 10 | 52 | 3780 | 10 |  |  |  |
|  | n3 |  |  | 1735 |  | 5 | 25 | 1830 | 5 |  | 27.9 |  |
| DC_1A-7A_n78A | 1 | IMD4 | ¦fB7 -3*fB1¦ | 1970 |  | 5 | 25 | 2160 | 5 | 1.2 | N/A |  |
|  | 7 |  |  | 2520 |  | 5 | 25 | 2640 | 5 |  |  |  |
|  | n78 |  |  | 3390 |  | 10 | 52 | 3390 | 10 |  | 10.1 |  |
|  | 1 | IMD5 | ¦3*fB7 -2*fB1¦ | 1970 |  | 5 | 25 | 2160 | 5 | 0.3 | N/A |  |
|  | 7 |  |  | 2520 |  | 5 | 25 | 2640 | 5 |  |  |  |
|  | n78 |  |  | 3620 |  | 10 | 52 | 3620 | 10 |  | 3.8 |  |
|  | 1 | IMD4 | ¦fn78 -3*fB1¦ | 1977.5 |  | 5 | 25 | 2167.5 | 5 | 0.9 | N/A |  |
|  | n78 |  |  | 3305 |  | 10 | 52 | 3305 | 10 |  |  |  |
|  | 7 |  |  | 2507.5 |  | 5 | 25 | 2627.5 | 5 |  | 9.1 |  |
| DC_3A_n1A-n78A, DC_3C_n1A-n78A and DC_3A_n1A-n77A | n1 | IMD2 | ¦fn1 +fB3¦ | 1950 |  | 5 | 25 | 2140 | 5 | 2.1 | N/A |  |
|  | 3 |  |  | 1750 |  | 5 | 25 | 1845 | 5 |  |  |  |
|  | n78/n77 |  |  | 3700 |  | 10 | 52 | 3700 | 10 |  | 28.4 |  |
|  | n1 | IMD4 | ¦3*fB3 -fn1¦ | 1950 |  | 5 | 25 | 2140 | 5 | 1.2 | N/A |  |
|  | 3 |  |  | 1770 |  | 5 | 25 | 1865 | 5 |  |  |  |
|  | n78/n77 |  |  | 3360 |  | 10 | 52 | 3360 | 10 |  | 11.2 |  |
|  | 3 | IMD2 | ¦fn77 –fB3¦ | 1775 |  | 5 | 25 | 1870 | 5 | 2.4 | N/A |  |
|  | n77 |  |  | 3915 |  | 10 | 52 | 3915 | 10 |  |  |  |
|  | n1 |  |  | 1950 |  | 5 | 25 | 2140 | 5 |  | 31.0 |  |
|  | 3 | IMD5 | ¦2*fn78 -3*fB3¦ | 1770 |  | 5 | 25 | 1865 | 5 | 0.3 | N/A |  |
|  | n78/n77 |  |  | 3720 |  | 10 | 52 | 3720 | 10 |  |  |  |
|  | n1 |  |  | 1940 |  | 5 | 25 | 2130 | 5 |  | 3.5 |  |
| DC_3A_n7A-n78A | 3 | IMD3 | ¦2*fn7-fB3¦ | 1730 |  | 5 | 25 | 1825 | 5 | 1.5 | N/A |  |
|  | n7 |  |  | 2560 |  | 5 | 25 | 2680 | 5 |  |  |  |
|  | n78 |  |  | 3390 |  | 10 | 52 | 3390 | 10 |  | 16.1 |  |
| DC_3A_n20A-n78A | 3 | IMD3 | ¦fB3+2*fn20¦ | 1730 |  | 5 | 25 | 1825 | 5 | 1.5 | N/A |  |
|  | n20 |  |  | 845 |  | 5 | 25 | 804 | 5 |  |  |  |
|  | n78 |  |  | 3420 |  | 10 | 52 | 3420 | 10 |  | 16.1 |  |
|  | 3 | IMD5 | ¦3*fB3 -2*fn20¦ | 1730 |  | 5 | 25 | 1825 | 5 | 0.4 | N/A |  |
|  | n20 |  |  | 845 |  | 5 | 25 | 804 | 5 |  |  |  |
|  | n78 |  |  | 3500 |  | 10 | 52 | 3500 | 10 |  | 4.5 |  |
| DC_3A_n28A-n78A | 3 | IMD5 | ¦3*fB3 -2*fn28¦ | 1750 |  | 5 | 25 | 1845 | 5 | 0.8 | N/A |  |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  |  |  |
|  | n78 |  |  | 3764 |  | 10 | 52 | 3764 | 10 |  | 4.5 |  |
| DC_7A-n28A-n78A | 7 | IMD2 | ¦fB7 +fB28¦ | 2565 |  | 5 | 25 | 2685 | 5 | 2.1 | N/A |  |
|  | n28 |  |  | 745 |  | 5 | 25 | 800 | 5 |  |  |  |
|  | n78 |  |  | 3310 |  | 10 | 52 | 3310 | 10 |  | 29.7 |  |
|  | 7 | IMD4 | ¦2*fB7 -2*fB28¦ | 2565 |  | 5 | 25 | 2685 | 5 | 1.1 | N/A |  |
|  | n28 |  |  | 745 |  | 5 | 25 | 800 | 5 |  |  |  |
|  | n78 |  |  | 3640 |  | 10 | 52 | 3640 | 10 |  | 10.3 |  |
|  | 7 | IMD2 | ¦fB78 -fB7¦ | 2565 |  | 5 | 25 | 2685 | 5 | 2.0 | N/A |  |
|  | n78 |  |  | 3365 |  | 10 | 52 | 3365 | 10 |  |  |  |
|  | n28 |  |  | 745 |  | 5 | 25 | 800 | 5 |  | 28.8 |  |
|  | 7 | IMD5 | ¦2*fB78 -3*fB7¦ | 2520 |  | 5 | 25 | 2640 | 5 | 0.7 | N/A |  |
|  | n78 |  |  | 3387 |  | 10 | 52 | 3387 | 10 |  |  |  |
|  | n28 |  |  | 731 |  | 5 | 25 | 786 | 5 |  | 4.0 |  |
| DC_28A-n8A-n78A | 28 | IMD4 | ¦3*fB8 +fB28¦ | 728 |  | 5 | 25 | 783 | 5 | 1.1 | N/A |  |
|  | n8 |  |  | 910 |  | 5 | 25 | 955 | 5 |  |  |  |
|  | n78 |  |  | 3458 |  | 10 | 52 | 3458 | 10 |  | 9.1 |  |
|  | 28 | IMD5 | ¦fB8 +4*fB28¦ | 713 |  | 5 | 25 | 768 | 5 | 0.7 | N/A |  |
|  | n8 |  |  | 910 |  | 5 | 25 | 955 | 5 |  |  |  |
|  | n78 |  |  | 3762 |  | 10 | 52 | 3762 | 10 |  | 3.7 |  |
|  | 28 | IMD5 | ¦fB78 -4*fB28¦ | 713 |  | 5 | 25 | 768 | 5 | 0.5 | N/A |  |
|  | n78 |  |  | 3787 |  | 10 | 52 | 3787 | 10 |  |  |  |
|  | n8 |  |  | 890 |  | 5 | 25 | 935 | 5 |  | 4.3 |  |
| DC_7A_n1A-n78A, DC_7C_n1A-n78A | n1 | IMD4 | ¦fB7 -3*fn1¦ | 1970 |  | 5 | 25 | 2160 | 5 | 1.2 | N/A |  |
|  | 7 |  |  | 2520 |  | 5 | 25 | 2640 | 5 |  |  |  |
|  | n78 |  |  | 3390 |  | 10 | 52 | 3390 | 10 |  | 10.1 |  |
|  | n1 | IMD5 | ¦3*fB7 -2*fn1¦ | 1970 |  | 5 | 25 | 2160 | 5 | 0.3 | N/A |  |
|  | 7 |  |  | 2520 |  | 5 | 25 | 2640 | 5 |  |  |  |
|  | n78 |  |  | 3620 |  | 10 | 52 | 3620 | 10 |  | 3.8 |  |
|  | 7 | IMD4 | ¦2*fn78 -2*fB7¦ | 2530 |  | 5 | 25 | 2650 | 5 | 0.9 | N/A |  |
|  | n78 |  |  | 3610 |  | 10 | 52 | 3610 | 10 |  |  |  |
|  | n1 |  |  | 1970 |  | 5 | 25 | 2160 | 5 |  | 9.0 |  |
| DC_7A_n3A-n78A | 7 | IMD3 | ¦2*fB7-fn3¦ | 2560 |  | 5 | 25 | 2680 | 5 | 1.5 | N/A |  |
|  | n3 |  |  | 1730 |  | 5 | 25 | 1825 | 5 |  |  |  |
|  | n78 |  |  | 3390 |  | 10 | 52 | 3390 | 10 |  | 16.1 |  |
|  | 7 | IMD3 | ¦2*fB7 -fn78¦ | 2565 |  | 5 | 25 | 2685 | 5 | 1.4 | N/A |  |
|  | n78 |  |  | 3310 |  | 10 | 52 | 3310 | 10 |  |  |  |
|  | n3 |  |  | 1725 |  | 5 | 25 | 1820 | 5 |  | 15.6 |  |
|  | 7 | IMD4 | ¦2*fn78 -2*fB7¦ | 2565 |  | 5 | 25 | 2685 | 5 | 0.8 | N/A |  |
|  | n78 |  |  | 3480 |  | 10 | 52 | 3480 | 10 |  |  |  |
|  | n3 |  |  | 1735 |  | 5 | 25 | 1830 | 5 |  | 9.2 |  |
| DC_20A_n1A-n78A | 20 | IMD3 | ¦fn1 +2*fB20¦ | 845 |  | 5 | 25 | 804 | 5 | 1.5 | N/A |  |
|  | n1 |  |  | 1940 |  | 5 | 25 | 2130 | 5 |  |  |  |
|  | n78 |  |  | 3630 |  | 10 | 52 | 3630 | 10 |  | 16.0 |  |
|  | 20 | IMD3 | ¦fn78 -2*fB20¦ | 835 |  | 5 | 25 | 794 | 5 | 1.3 | N/A |  |
|  | n78 |  |  | 3790 |  | 10 | 52 | 3790 | 10 |  |  |  |
|  | n1 |  |  | 1930 |  | 5 | 25 | 2120 | 5 |  | 15.3 |  |
| DC_20A_n3A-n78A | 20 | IMD3 | ¦2*fB20+fn3¦ | 845 |  | 5 | 25 | 804 | 5 | 1.5 | N/A |  |
|  | n3 |  |  | 1730 |  | 5 | 25 | 1825 | 5 |  |  |  |
|  | n78 |  |  | 3420 |  | 10 | 52 | 3420 | 10 |  | 16.1 |  |
|  | 20 | IMD5 | ¦2*fB20 -3*fn3¦ | 845 |  | 5 | 25 | 804 | 5 | 0.4 | N/A |  |
|  | n3 |  |  | 1730 |  | 5 | 25 | 1825 | 5 |  |  |  |
|  | n78 |  |  | 3500 |  | 10 | 52 | 3500 | 10 |  | 4.5 |  |
|  | 20 | IMD3 | ¦2*fB20-fn78¦ | 845 |  | 5 | 25 | 804 | 5 | 1.3 | N/A |  |
|  | n78 |  |  | 3550 |  | 10 | 52 | 3550 | 10 |  |  |  |
|  | n3 |  |  | 1765 |  | 5 | 25 | 1860 | 5 |  | 15.7 |  |
| DC_1A_n40A-n78A | 1 | IMD4 | ¦3*fB1-fn40¦ | 1930 |  | 5 | 25 | 2120 | 5 | 1.0 | N/A |
|  | n40 |  |  | 2340 |  | 5 | 25 | 2340 | 5 |  |  |
|  | n78 |  |  | 3450 |  | 10 | 50 | 3450 | 10 |  | 9.8 |
|  | 1 | IMD5 | ¦2*fB1 -3*fn40¦ | 1925 |  | 5 | 25 | 2115 | 5 | 0.4 | N/A |
|  | n40 |  |  | 2390 |  | 5 | 25 | 2390 | 5 |  |  |
|  | n78 |  |  | 3320 |  | 10 | 50 | 3320 | 10 |  | 4.2 |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1960 |  | 5 | 25 | 2150 | 5 | 0.8 | N/A |
|  | n78 |  |  | 3520 |  | 10 | 50 | 3520 | 10 |  |  |
|  | n40 |  |  | 2360 |  | 5 | 25 | 2360 | 5 |  | 10.6 |
| DC_3A_n1A-n79A | 3 | IMD5 | ¦4*fB3-fn1¦ | 1720 |  | 5 | 25 | 1815 | 5 | 0.5 | N/A |
|  | n1 |  |  | 1930 |  | 5 | 25 | 2120 | 5 |  |  |
|  | n79 |  |  | 4950 |  | 40 | 216 | 4950 | 40 |  | 4.7 |
|  | 3 | IMD5 | ¦4*fB3 -fn79¦ | 1750 |  | 5 | 25 | 1845 | 5 | 0.5 | N/A |
|  | n79 |  |  | 4860 |  | 40 | 216 | 4860 | 10 |  |  |
|  | n1 |  |  | 1950 |  | 5 | 25 | 2140 | 5 |  | 3.6 |
| DC_3A_n40A-n78A | 3 | IMD5 | ¦2*fB3-3*fn40¦ | 1730 |  | 5 | 25 | 1825 | 5 | 0.5 | N/A |
|  | n40 |  |  | 2360 |  | 5 | 25 | 2360 | 5 |  |  |
|  | n78 |  |  | 3620 |  | 10 | 50 | 3620 | 10 |  | 4.8 |
|  | 3 | IMD5 | ¦3*fB3 -2*fn78¦ | 1720 |  | 5 | 25 | 1815 | 5 | 0.5 | N/A |
|  | n78 |  |  | 3760 |  | 10 | 50 | 3760 | 10 |  |  |
|  | n40 |  |  | 2360 |  | 5 | 25 | 2360 | 5 |  | 4.4 |
| DC_66A_n25A-n41A | 66 | IMD4 | ¦2*fB66 -2*fn41¦ | 1715 |  | 5 | 25 | 2115 | 5 | 1.1 | N/A |
|  | n41 |  |  | 2685 |  | 10 | 50 | 2685 | 10 |  |  |
|  | n25 |  |  | 1860 |  | 5 | 25 | 1940 | 5 |  | 11.0 |
| DC_40A_n41A-n79A | 40 | IMD2 | ¦fB40+fn41¦ | 2340 |  | 5 | 25 | 2340 | 5 |  | N/A |  |
|  | n41 |  |  | 2600 |  | 10 | 50 | 2600 | 10 |  |  |  |
|  | n79 |  |  | 4940 |  | 40 | 216 | 4940 | 40 |  | 30.5 |  |
| DC_8A_n3A-n28A | 8 | IMD2 | ¦fB8-fn3¦ | 912.5 |  | 5 | 25 | 957.5 | 5 |  | N/A |  |
|  | n3 |  |  | 1712.5 |  | 5 | 25 | 1807.5 | 5 |  |  |  |
|  | n28 |  |  | 745 |  | 5 | 25 | 800 | 5 |  | 30.4 |  |
|  | 8 | IMD5 | ¦3*fB8 -2*fn3¦ | 910 |  | 5 | 25 | 955 | 5 |  | N/A |  |
|  | n3 |  |  | 1750 |  | 5 | 25 | 1845 | 5 |  |  |  |
|  | n28 |  |  | 715 |  | 5 | 25 | 770 | 5 |  | 2.6 |  |
| DC_8A_n1A-n78A | 8 | IMD3 | ¦2*fB8+fn1¦ | 900 |  | 5 | 25 | 945 | 5 |  | N/A |  |
|  | n1 |  |  | 1945 |  | 5 | 25 | 2135 | 5 |  |  |  |
|  | n78 |  |  | 3745 |  | 10 | 50 | 3745 | 10 |  | 14.9 |  |
| DC_2A_n41A-n71A | 2 | IMD2 | ¦fB2-fn41¦ | 1900 |  | 5 | 25 | 1980 | 5 |  | N/A |  |
|  | n41 |  |  | 2530 |  | 10 | 50 | 2530 | 10 |  |  |  |
|  | n71 |  |  | 676 |  | 5 | 50 | 630 | 5 |  | 28.7 |  |
|  | 2 | IMD5 | ¦3*fB2 -2*fn41¦ | 1900 |  | 5 | 25 | 1980 | 5 |  | N/A |  |
|  | n41 |  |  | 2530 |  | 10 | 50 | 2530 | 10 |  |  |  |
|  | n71 |  |  | 686 |  | 5 | 50 | 640 | 5 |  | 3.8 |  |
|  | 2 | IMD2 | ¦fB2+fn71¦ | 1900 |  | 5 | 25 | 1980 | 5 |  | N/A |  |
|  | n41 |  |  | 2586 |  | 10 | 50 | 2586 | 10 |  | 29.2 |  |
|  | n71 |  |  | 686 |  | 5 | 50 | 640 | 5 |  | N/A |  |
| DC_1A-n3A_n28A | 1 | IMD5 | ¦2*fB1 -3*fn28¦ | 1975 |  | 5 | 25 | 2165 | 5 |  | N/A |  |
|  | n28 |  |  | 710.5 |  | 5 | 25 | 765.5 | 5 |  |  |  |
|  | n3 |  |  | 1723.5 |  | 5 | 25 | 1818.5 | 5 |  | 4.0 |  |
| DC_39A_n41A-n79A | 39 | IMD2 | ¦fB39+fn41¦ | 1900 |  | 5 | 25 | 1900 | 5 |  | N/A |  |
|  | n41 |  |  | 2620 |  | 10 | 50 | 2620 | 10 |  |  |  |
|  | n79 |  |  | 4520 |  | 40 | 216 | 4520 | 40 |  | 29.8 |  |
|  | 39 | IMD5 | ¦4*fB39 -fn41¦ | 1885 |  | 5 | 25 | 1885 | 5 |  | N/A |  |
|  | n41 |  |  | 2660 |  | 10 | 50 | 2660 | 10 |  |  |  |
|  | n79 |  |  | 4880 |  | 40 | 216 | 4880 | 40 |  | 4.4 |  |
|  | 39 | IMD2 | ¦fB39 -fn79¦ | 1900 |  | 5 | 25 | 1900 | 5 |  | N/A |  |
|  | n79 |  |  | 4520 |  | 40 | 216 | 4520 | 40 |  |  |  |
|  | n41 |  |  | 2620 |  | 10 | 50 | 2620 | 10 |  | 30.2 |  |
|  | 39 | IMD5 | ¦4*fB39 -fn79¦ | 1885 |  | 5 | 25 | 1885 | 5 |  | N/A |  |
|  | n79 |  |  | 4880 |  | 40 | 216 | 4880 | 40 |  |  |  |
|  | n41 |  |  | 2660 |  | 10 | 50 | 2660 | 10 |  | 4.3 |  |
| DC_12A_n7A-n78A | 12 | IMD4 | ¦2*fB12 -2*fn7¦ | 708 |  | 5 | 25 | 738 | 5 |  | N/A |  |
|  | n7 |  |  | 2520 |  | 5 | 25 | 2640 | 5 |  |  |  |
|  | n78 |  |  | 3624 |  | 10 | 50 | 3624 | 10 |  | 8.4 |  |
|  | 12 | IMD2 | ¦fB12 -fn78¦ | 708 |  | 5 | 25 | 738 | 5 |  | N/A |  |
|  | n78 |  |  | 3360 |  | 10 | 50 | 3360 | 10 |  |  |  |
|  | n7 |  |  | 2542 |  | 5 | 25 | 2662 | 5 |  | 28.7 |  |
| DC_2A_n66A-n78A | 2 | IMD2 | ¦fB2+fn66¦ | 1880 |  | 5 | 25 | 1960 | 5 |  | N/A |  |
|  | n66 |  |  | 1740 |  | 5 | 25 | 2140 | 5 |  |  |  |
|  | n78 |  |  | 3620 |  | 10 | 50 | 3620 | 10 |  | 29.4 |  |
|  | 2 | IMD4 | ¦fB2 -3*fn66¦ | 1880 |  | 5 | 25 | 1960 | 5 |  | N/A |  |
|  | n66 |  |  | 1740 |  | 5 | 25 | 2140 | 5 |  |  |  |
|  | n78 |  |  | 3340 |  | 10 | 50 | 3340 | 10 |  | 8.9 |  |
| DC_12A_n66A-n78A | 12 | IMD5 | ¦2*fB12-3*fn66¦ | 703 |  | 5 | 25 | 733 | 5 |  | N/A |  |
|  | n66 |  |  | 1720 |  | 5 | 25 | 2120 | 5 |  |  |  |
|  | n78 |  |  | 3754 |  | 10 | 50 | 3754 | 10 |  | 4.1 |  |
|  | 12 | IMD3 | ¦2*fB12 -fn78¦ | 703 |  | 5 | 25 | 733 | 5 |  | N/A |  |
|  | n78 |  |  | 3546 |  | 10 | 50 | 3546 | 10 |  |  |  |
|  | n66 |  |  | 1740 |  | 5 | 25 | 2140 | 5 |  | 16.5 |  |
| DC_7A_n66A-n78A | 7 | IMD3 | ¦2*fB7-fn66¦ | 2542 |  | 5 | 25 | 2662 | 5 |  | N/A |  |
|  | n66 |  |  | 1740 |  | 5 | 25 | 2140 | 5 |  |  |  |
|  | n78 |  |  | 3344 |  | 10 | 50 | 3344 | 10 |  | 16.0 |  |
| DC_20A_n7A-n28A | 20 | IMD3 | ¦2*fB20-fn7¦ | 857 |  | 5 | 25 | 816 | 5 |  | N/A |  |
|  | n7 |  |  | 2512 |  | 5 | 25 | 2632 | 5 |  |  |  |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  | 15.7 |  |
|  | 20 | IMD5 | ¦4*fB20-fn7¦ | 837 |  | 5 | 25 | 796 | 5 |  | N/A |  |
|  | n7 |  |  | 2555 |  | 5 | 25 | 2675 | 5 |  |  |  |
|  | n28 |  |  | 738 |  | 5 | 25 | 793 | 5 |  | 2.8 |  |
| DC_20A_n7A-n78A | 20 | IMD2 | ¦fB20+fn7¦ | 837 |  | 5 | 25 | 796 | 5 |  | N/A |  |
|  | n7 |  |  | 2555 |  | 5 | 25 | 2675 | 5 |  |  |  |
|  | n78 |  |  | 3392 |  | 10 | 50 | 3392 | 10 |  | 28.8 |  |
|  | 20 | IMD4 | ¦2*fB20-2*fn7¦ | 837 |  | 5 | 25 | 796 | 5 |  | N/A |  |
|  | n7 |  |  | 2555 |  | 5 | 25 | 2675 | 5 |  |  |  |
|  | n78 |  |  | 3436 |  | 10 | 50 | 3436 | 10 |  | 8.5 |  |
| DC_18A_n3A-n78A | 18 | IMD3 | ¦2*fB18+fn3¦ | 820 |  | 5 | 25 | 865 | 5 |  | N/A |  |
|  | n3 |  |  | 1750 |  | 5 | 25 | 1845 | 5 |  |  |  |
|  | n78 |  |  | 3390 |  | 10 | 50 | 3390 | 10 |  | 15.2 |  |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 |  | 5 | 25 | 865 | 5 |  | N/A |  |
|  | n3 |  |  | 1750 |  | 5 | 25 | 1845 | 5 |  |  |  |
|  | n78 |  |  | 3610 |  | 10 | 50 | 3610 | 10 |  | 4.0 |  |
| DC_3A_n40A-n79A | 3 | IMD5 | ¦4*fB3-fn40¦ | 1720 |  | 5 | 25 | 1815 | 5 |  | N/A |  |
|  | n40 |  |  | 2330 |  | 5 | 25 | 2330 | 5 |  |  |  |
|  | n79 |  |  | 4550 |  | 40 | 216 | 4550 | 40 |  | 4.7 |  |
|  | 3 | IMD5 | ¦4*fB3-fn79¦ | 1720 |  | 5 | 25 | 1815 | 5 |  | N/A |  |
|  | n79 |  |  | 4550 |  | 40 | 216 | 4550 | 40 |  |  |  |
|  | n40 |  |  | 2330 |  | 5 | 25 | 2330 | 5 |  | 3.2 |  |
| DC_3A_n41A-n79A | 3 | IMD2 | ¦fB3+fn41¦ | 1770 |  | 5 | 25 | 1865 | 5 |  | N/A |  |
|  | n41 |  |  | 2670 |  | 10 | 50 | 2670 | 10 |  |  |  |
|  | n79 |  |  | 4440 |  | 40 | 216 | 4440 | 40 |  | 30.8 |  |
|  | 3 | IMD5 | ¦4*fB3-fn41¦ | 1770 |  | 5 | 25 | 1865 | 5 |  | N/A |  |
|  | n41 |  |  | 2550 |  | 10 | 50 | 2550 | 10 |  |  |  |
|  | n79 |  |  | 4530 |  | 40 | 216 | 4530 | 40 |  | 5.2 |  |
| DC_8A_n40A-n79A | 8 | IMD4 | ¦3*fB8+fn40¦ | 885 |  | 5 | 25 | 930 | 5 |  | N/A |  |
|  | n40 |  |  | 2305 |  | 5 | 25 | 2305 | 5 |  |  |  |
|  | n79 |  |  | 4960 |  | 40 | 216 | 4960 | 40 |  | 10.7 |  |
|  | 8 | IMD4 | ¦3*fB8-fn79¦ | 885 |  | 5 | 25 | 930 | 5 |  | N/A |  |
|  | n79 |  |  | 4960 |  | 40 | 216 | 4960 | 40 |  |  |  |
|  | n40 |  |  | 2305 |  | 5 | 25 | 2305 | 5 |  | 9.2 |  |
| DC_8A_n41A-n79A | 8 | IMD3 | ¦2*fB8+fn41¦ | 910 |  | 5 | 25 | 955 | 5 |  | N/A |  |
|  | n41 |  |  | 2650 |  | 10 | 50 | 2650 | 10 |  |  |  |
|  | n79 |  |  | 4470 |  | 40 | 216 | 4470 | 40 |  | 16.3 |  |
|  | 8 | IMD3 | ¦2*fB8-fn79¦ | 910 |  | 5 | 25 | 955 | 5 |  | N/A |  |
|  | n79 |  |  | 4470 |  | 40 | 216 | 4470 | 40 |  |  |  |
|  | n41 |  |  | 2650 |  | 10 | 50 | 2650 | 10 |  | 15.5 |  |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 |  | 5 | 25 | 1917.5 | 5 |  | N/A |  |
|  | n40 |  |  | 2302.5 |  | 5 | 25 | 2302.5 | 5 |  |  |  |
|  | n79 |  |  | 4980 |  | 40 | 216 | 4980 | 40 |  | 5.8 |  |
| DC_13A_n48A-n66A | 13 | IMD3 | ¦2*fB13 -fn48¦ | 782 |  | 5 | 25 | 751 | 5 |  | N/A |  |
|  | n48 |  |  | 3680 |  | 5 | 25 | 3680 | 5 |  |  |  |
|  | n66 |  |  | 1716 |  | 5 | 25 | 2116 | 5 |  | 14.4 |  |
|  | 13 | IMD5 | ¦2*fB13 -3*fn66¦ | 782 |  | 5 | 25 | 751 | 5 |  | N/A |  |
|  | n66 |  |  | 1716 |  | 5 | 25 | 2116 | 5 |  |  |  |
|  | n48 |  |  | 3584 |  | 5 | 25 | 3584 | 5 |  | 2.8 |  |
| DC_66A_n5A-n48A | 66 | IMD5 | ¦3*fB66 -2*fn5¦ | 1750 |  | 5 | 25 | 2150 | 5 |  | N/A |  |
|  | n5 |  |  | 834 |  | 5 | 25 | 879 | 5 |  |  |  |
|  | n48 |  |  | 3582 |  | 5 | 25 | 3582 | 5 |  | 3.3 |  |
| DC_11A_n3A-n28A | 11 | IMD5 | ¦3*fB11 -2*fn3¦ | 1438 |  | 5 | 25 | 1486 | 5 |  | N/A |  |
|  | n3 |  |  | 1758 |  | 5 | 25 | 1853 | 5 |  |  |  |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  | 2.5 |  |
| DC_1A_n8A-n40A | 1 | IMD4 | ¦2*fB1 -2*fn40¦ | 1930 |  | 5 | 25 | 2120 | 5 |  | N/A |  |
|  | n40 |  |  | 2395 |  | 5 | 25 | 2395 | 5 |  |  |  |
|  | n8 |  |  | 885 |  | 5 | 25 | 930 | 5 |  | 9.2 |  |
| DC_1A_n28A-n40A | 1 | IMD4 | ¦2*fB1 -2*fn28¦ | 1930 |  | 5 | 25 | 2120 | 5 |  | N/A |  |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  |  |  |
|  | n40 |  |  | 2374 |  | 5 | 25 | 2374 | 5 |  | 10.1 |  |
|  | 1 | IMD4 | ¦2*fB1 -2*fn40¦ | 1930 |  | 5 | 25 | 2120 | 5 |  | N/A |  |
|  | n40 |  |  | 2314 |  | 5 | 25 | 2314 | 5 |  |  |  |
|  | n28 |  |  | 713 |  | 5 | 25 | 768 | 5 |  | 8.6 |  |
| DC_3A_n8A-n78A | 3 | IMD3 | ¦fB3 +2*fn8¦ | 1740 |  | 5 | 25 | 1835 | 5 |  | N/A |  |
|  | n8 |  |  | 900 |  | 5 | 25 | 945 | 5 |  |  |  |
|  | n78 |  |  | 3540 |  | 10 | 50 | 3540 | 10 |  | 16.3 |  |
| DC_7A_n1A-n40A | 7 | IMD3 | ¦fB7 -2*fn40¦ | 2540 |  | 5 | 25 | 2660 | 5 |  | N/A |  |
|  | n40 |  |  | 2335 |  | 5 | 25 | 2335 | 5 |  |  |  |
|  | n1 |  |  | 1940 |  | 5 | 25 | 2130 | 5 |  | 15.2 |  |
| DC_7A_n8A-n78A | 7 | IMD2 | ¦fB7 +fn8¦ | 2555 |  | 5 | 25 | 2675 | 5 |  | N/A |  |
|  | n8 |  |  | 900 |  | 5 | 25 | 945 | 5 |  |  |  |
|  | n78 |  |  | 3455 |  | 10 | 50 | 3455 | 10 |  | 28.5 |  |
|  | 7 | IMD2 | ¦fB7 -fn78¦ | 2555 |  | 5 | 25 | 2675 | 5 |  | N/A |  |
|  | n78 |  |  | 3500 |  | 10 | 50 | 3500 | 10 |  |  |  |
|  | n8 |  |  | 900 |  | 5 | 25 | 945 | 5 |  | 29.7 |  |
| DC_2A_n7A-n78A | 2 | IMD5 | ¦2*fB2 -3*fn7¦ | 1900 |  | 5 | 25 | 1980 | 5 |  | N/A |  |
|  | n7 |  |  | 2525 |  | 5 | 25 | 2645 | 5 |  |  |  |
|  | n78 |  |  | 3775 |  | 10 | 50 | 3775 | 10 |  | 4.2 |  |
| DC_13A_n7A-n78A | 13 | IMD2 | ¦fB13 +fn7¦ | 782 |  | 5 | 25 | 751 | 5 |  | N/A |  |
|  | n7 |  |  | 2530 |  | 5 | 25 | 2650 | 5 |  |  |  |
|  | n78 |  |  | 3312 |  | 10 | 50 | 3312 | 10 |  | 29.0 |  |
|  | 13 | IMD2 | ¦fB13 -fn78¦ | 782 |  | 5 | 25 | 751 | 5 |  | N/A |  |
|  | n78 |  |  | 3432 |  | 10 | 50 | 3432 | 10 |  |  |  |
|  | n7 |  |  | 2530 |  | 5 | 25 | 2650 | 5 |  | 27.9 |  |
| DC_7A_n8A-n40A | 7 | IMD5 | ¦2*fB7 -3*fn8¦ | 2530 |  | 5 | 25 | 2650 | 5 |  | N/A |
|  | n8 |  |  | 905 |  | 5 | 25 | 950 | 5 |  |  |
|  | n40 |  |  | 2345 |  | 5 | 25 | 2345 | 5 |  | 3.0 |
| DC_3A_n28A-n77A | 3 | IMD3 | ¦2*fB3 +fn28¦ | 1720 |  | 5 | 25 | 1815 | 5 |  | N/A |
|  | n28 |  |  | 733 |  | 5 | 25 | 788 | 5 |  |  |
|  | n77 |  |  | 4173 |  | 10 | 50 | 4173 | 10 |  | 15.9 |
|  | 3 | IMD3 | ¦2*fB3 -fn77¦ | 1712.5 |  | 5 | 25 | 1807.5 | 5 |  | N/A |
|  | n77 |  |  | 4195 |  | 10 | 50 | 4195 | 10 |  |  |
|  | n28 |  |  | 715 |  | 5 | 25 | 770 | 5 |  | 15.3 |
| DC_8A_n28A-n77A | 8 | IMD4 | ¦3*fB8 +fn28¦ | 910 |  | 5 | 25 | 955 | 5 |  | N/A |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  |  |
|  | n77 |  |  | 3473 |  | 10 | 50 | 3473 | 10 |  | 10.3 |
|  | 8 | IMD4 | ¦3*fB8 -fn77¦ | 910 |  | 5 | 25 | 955 | 5 |  | N/A |
|  | n77 |  |  | 3495 |  | 10 | 50 | 3495 | 10 |  |  |
|  | n28 |  |  | 710 |  | 5 | 25 | 765 | 5 |  | 11.6 |
| DC_42A_n28A-n77A | 42 | IMD2 | ¦fB42 +fn28¦ | 3410 |  | 5 | 25 | 3410 | 5 |  | N/A |
|  | n28 |  |  | 733 |  | 5 | 25 | 788 | 5 |  |  |
|  | n77 |  |  | 4143 |  | 10 | 50 | 4143 | 10 |  | 29.2 |
| DC_2A_n38A-n78A | 2 | IMD3 | ¦fB2 -2*fn38¦ | 1870 |  | 5 | 25 | 1950 | 5 |  | N/A |
|  | n38 |  |  | 2610 |  | 5 | 25 | 2610 | 5 |  |  |
|  | n78 |  |  | 3350 |  | 10 | 50 | 3350 | 10 |  | 14.8 |
| DC_66A_n38A-n78A | 66 | IMD3 | ¦fB66 -2*fn38¦ | 1760 |  | 5 | 25 | 2160 | 5 |  | N/A |
|  | n38 |  |  | 2610 |  | 5 | 25 | 2610 | 5 |  |  |
|  | n78 |  |  | 3460 |  | 10 | 50 | 3460 | 10 |  | 15.0 |
| DC_18A_n3A-n77A | 18 | IMD3 | ¦2*fB18 +fn3¦ | 820 |  | 5 | 25 | 865 | 5 |  | N/A |
|  | n3 |  |  | 1770 |  | 5 | 25 | 1865 | 5 |  |  |
|  | n77 |  |  | 3410 |  | 10 | 50 | 3410 | 10 |  | 16.3 |
|  | 18 | IMD3 | ¦2*fB18 -fn77¦ | 820 |  | 5 | 25 | 865 | 5 |  | N/A |
|  | n77 |  |  | 3505 |  | 10 | 50 | 3505 | 10 |  |  |
|  | n3 |  |  | 1770 |  | 5 | 25 | 1865 | 5 |  | 15.7 |
| DC_28A_n3A-n77A | 28 | IMD3 | ¦fB28 +2*fn3¦ | 733 |  | 5 | 25 | 788 | 5 |  | N/A |
|  | n3 |  |  | 1720 |  | 5 | 25 | 1815 | 5 |  |  |
|  | n77 |  |  | 4173 |  | 10 | 50 | 4173 | 10 |  | 15.9 |
|  | 28 | IMD3 | ¦2*fB28 -fn77¦ | 735 |  | 5 | 25 | 790 | 5 |  | N/A |
|  | n77 |  |  | 3320 |  | 10 | 50 | 3320 | 10 |  |  |
|  | n3 |  |  | 1755 |  | 5 | 25 | 1850 | 5 |  | 17.0 |
| DC_41A_n3A-n77A DC_41A_n3A-n78A | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 |  | 5 | 25 | 2580 | 5 |  | N/A |
|  | n3 |  |  | 1720 |  | 5 | 25 | 1815 | 5 |  |  |
|  | n77/n78 |  |  | 3440 |  | 10 | 50 | 3440 | 10 |  | 16.8 |
|  | 41 | IMD3 | ¦2*fB41 -fn77¦ | 2620 |  | 5 | 25 | 2620 | 5 |  | N/A |
|  | n77/n78 |  |  | 3400 |  | 10 | 50 | 3400 | 10 |  |  |
|  | n3 |  |  | 1745 |  | 5 | 25 | 1840 | 5 |  | 16.4 |
| DC_41A_n28A-n77A DC_41A_n28A-n78A | 41 | IMD2 | ¦fB41 +fn28¦ | 2580 |  | 5 | 25 | 2580 | 5 |  | N/A |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  |  |
|  | n77/n78 |  |  | 3323 |  | 10 | 50 | 3323 | 10 |  | 28.2 |
|  | 41 | IMD2 | ¦fB41 -fn77¦ | 2642 |  | 5 | 25 | 2642 | 5 |  | N/A |
|  | n77/n78 |  |  | 3440 |  | 10 | 50 | 3440 | 10 |  |  |
|  | n28 |  |  | 743 |  | 5 | 25 | 798 | 5 |  | 30.8 |
| DC_1A_n41A-n78A | 1 | IMD3 | ¦fB1 -2*fn41¦ | 1930 |  | 5 | 25 | 2120 | 5 |  | N/A |
|  | n41 |  |  | 2650 |  | 5 | 25 | 2650 | 5 |  |  |
|  | n78 |  |  | 3370 |  | 10 | 50 | 3370 | 10 |  | 19.6 |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1970 |  | 5 | 25 | 2160 | 5 |  | N/A |
|  | n78 |  |  | 3400 |  | 10 | 50 | 3400 | 10 |  |  |
|  | n41 |  |  | 2510 |  | 5 | 25 | 2510 | 5 |  | 11.5 |
| DC_3A_n41A-n78A | 3 | IMD3 | ¦2*fn41 –fB3¦ | 1720 |  | 5 | 25 | 1815 | 5 |  | N/A |
|  | n41 |  |  | 2580 |  | 5 | 25 | 2580 | 5 |  |  |
|  | n78 |  |  | 3440 |  | 10 | 50 | 3440 | 10 |  | 16.4 |
| DC_20A_n41A-n78A | 20 | IMD2 | ¦fB20 +fn41¦ | 837 |  | 5 | 25 | 796 | 5 |  | N/A |
|  | n41 |  |  | 2650 |  | 5 | 25 | 2650 | 5 |  |  |
|  | n78 |  |  | 3487 |  | 10 | 50 | 3487 | 10 |  | 28.8 |
|  | 20 | IMD2 | ¦fB20 -fn78¦ | 837 |  | 5 | 25 | 796 | 5 |  | N/A |
|  | n78 |  |  | 3370 |  | 10 | 50 | 3370 | 10 |  |  |
|  | n41 |  |  | 2533 |  | 5 | 25 | 2533 | 5 |  | 29.8 |
| DC_1A_n75A-n78A | 1 | IMD2 | ¦fB1 -fn78¦ | 1930 |  | 5 | 25 | 2120 | 5 |  | N/A |
|  | n78 |  |  | 3400 |  | 10 | 50 | 3400 | 10 |  |  |
|  | n75 |  |  | - |  | - | - | 1470 | 5 |  | 29.0 |
| DC_3A_n75A-n78A | 3 | IMD2 | ¦fB3 -fn78¦ | 1782.5 |  | 5 | 25 | 1877.5 | 5 |  | N/A |
|  | n78 |  |  | 3305 |  | 10 | 50 | 3305 | 10 |  |  |
|  | n75 |  |  | - |  | - | - | 1514.5 | 5 |  | 10.0 |

6	EN-DC band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

6.1	DC_3A_n1A-n77A

6.1.1	Operating bands for DC

Table 6.1.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n1A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.1.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.1.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n77A with dual uplink on DC_3A_n1A and DC_3A_n77A were calculated and listed in Table 6.1.3-1 and 6.1.3-2 respectively.

Table 6.1.3-1: Harmonic and IMD analysis for DC_3_n1-n77 with UL on DC_3_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.1.3-2: Harmonic and IMD analysis for DC_3_n1-n77 with UL on DC_3_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2490 | 5010 | 5985 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 780 | 270 | 4815 | 6690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7770 | 8310 | 10185 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 930 | 2055 | 8115 | 10890 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9555 | 11610 | 14385 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4980 | 3030 | 10020 | 11970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11415 | 3840 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18585 | 10140 | 11340 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6330 | 1245 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16170 | 11730 | 13755 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n77 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.1.3-1 and Table 6.1.3-2:

-	2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n77

-	2nd and 5th order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band n1

6.1.4	∆TIB and ∆RIB values

For DC_3A_n1A-n77, the TIB,c and RIB values are given in the tables below.

Table 6.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n1A-n77A | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n77A | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.1.5	MSD

Table 6.1.5-1 lists the MSD required for the dual connectivity configuration.

Table 6.1.5-1: MSD for the DC configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A |  | N/A |
|  | n77 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A |  | N/A |
|  | n77 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 |  | IMD2 |
|  | n77 | 3915 | 10 | 52 | 3915 | N/A | TDD | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 |  | IMD5 |
|  | n77 | 3720 | 10 | 52 | 3720 | N/A | TDD | N/A |

6.2	DC_1A_n78A-n257

6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A_n78A-n257D | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257E | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257F | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257G | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257H | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257I | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257J | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257K | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257L | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n78A-n257M | DC_1A_n78A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.3	Co-existence studies

Based on co-existence studies of DC_1A_n78A and DC_1A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_1A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_1A_n257A: Harmonic problem already specified in TR 37.863-01-01

6.2.4	∆TIB and ∆RIB values

For DC_1A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n78-n257 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n78-n257 | 1 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.2.5	MSD

As mentioned in 6.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.3	DC_3_n78-n257

6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A_n78A-n257D | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257E | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257F | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257G | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257H | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257I | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257J | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257K | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257L | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n78A-n257M | DC_3A_n78A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.2-2: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3C_n78A-n257A | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_3C_n78A-n257D | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257E | DC_3A_n78A DC_3A_n257A |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 740 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257F | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257G | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257H | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257I | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257J | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 640 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257K | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 740 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257L | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 840 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n78A-n257M | DC_3A_n78A DC_3A_n257A |  | 3 |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.3	Co-existence studies

Based on co-existence studies of DC_3A_n78A and DC_3A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_3A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_3A_n257A: Harmonic problem is already specified in TR 37.863-01-01

6.3.4	∆TIB and ∆RIB values

For DC_3_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n257 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n257 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.3.5	MSD

As mentioned in 6.3.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.4	DC_5A_n78A-n257

6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A_n78-n257 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.4.2	Channel bandwidths per operating band for DC

Table 6.4.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_5A_n78A-n257D | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257E | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257F | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 910 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257G | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257H | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257I | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257J | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 610 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257K | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 710 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257L | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 810 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n78A-n257M | DC_5A_n78A DC_5A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 910 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.4.3	Co-existence studies

Based on co-existence studies of DC_5A_n78A and DC_5A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_5A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_5A_n257A: Harmonic problem is already specified in TR 37.863-01-01

6.4.4	∆TIB and ∆RIB values

For DC_5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A_n78-n257 | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A_n78-n257 | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.4.5	MSD

IMD issues are already analysed in TR 37.863-01-01 and there is no IMD issues to 3rd own Rx bands. Harmonics are expected in DC_5A_n78A-n257 configuration. DC_5A_n78A has 7th and 8th harmonics from n78 into n257 but there is no harmonic problems by 7th and 8th order between FR1 and FR2. Furthermore, DC_5A_n257A has 4th harmonic from Band 5 into n78 but this harmonic problem will be solved in DC_5A_n78A

6.5	DC_7A_n78A-n257

6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A_n78-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.5.2	Channel bandwidths per operating band for DC

Table 6.5.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_7A_n78A-n257D | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257E | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257F | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257G | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257H | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257I | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257J | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 620 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257K | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 720 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257L | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 820 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n78A-n257M | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 920 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.5.3	Co-existence studies

Based on co-existence studies of DC_7A_n78A and DC_7A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

- DC_7A_n78A: No harmonic problems by 7th & 8th order between FR1 and FR2

- DC_7A_n257A: There is no issue on this configuration

6.5.4	∆TIB and ∆RIB values

For DC_7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A_n78-n257 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A_n78-n257 | 7 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.5.5	MSD

No IMD issues are expected but harmonics are expected in DC_7A_n78A-n257 DC configuration. DC_7A_n78A has 7th and 8th harmonics from n78 into n257 but there is no harmonic problems by 7th and 8th order between FR1 and FR2.

6.6	DC_7A-7A_n78A-n257

6.6.1	Operating bands for DC

Table 6.6.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-7A_n78-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.6.2	Channel bandwidths per operating band for DC

Table 6.6.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_7A-7A_n78A-n257D | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257E | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257F | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257G | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257H | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257I | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257J | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257K | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257L | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A_n78A-n257M | DC_7A_n78A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.6.3	Co-existence studies

Based on co-existence studies of DC_7A_n78A and DC_7A_n257A specified in TR 37.863-01-01, 7th harmonic from Band 7 fall into Band n257 when dual uplink is configured with Band 7+n78. There is no additional harmonic and intermodulation impact for the additional band receiver.

6.6.4	∆TIB and ∆RIB values

For DC_7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-7A_n78-n257 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-7A_n78-n257 | 7 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.6.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 repectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.
Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257.

6.7	DC_1A_n3A-n78A

6.7.1	Operating bands for DC

Table 6.7.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.7.2	Channel bandwidths per operating band for DC

Table 6.7.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n3A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.7.3	Co-existence studies

Co-existence studies of Band 1 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.7.4	∆TIB and ∆RIB values

For DC_1A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n78 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

6.7.5	MSD

Based on Table 5.3.5 in TR 37.716-21-21 [2] it is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.7.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |

6.8	DC_3_n1-n78

6.8.1	Operating bands for DC

Table 6.8.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.8.2	Channel bandwidths per operating band for DC

Table 6.8.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n78A | DC_3A_n1A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C_n1A-n78A | DC_3A_n1A DC_3A_n78A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.8.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n78A with dual uplink on DC_3A_n1A and DC_3A_n78A were calculated and listed in Table 6.8.3-1 and 6.8.3-2 respectively.

Table 6.8.3-1: Harmonic and IMD analysis for DC_3_n1-n78 with UL on DC_3_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.8.3-2: Harmonic and IMD analysis for DC_3_n1-n78 with UL on DC_3_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 270 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3030 | 10020 | 11170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11415 | 3840 | 3040 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6330 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.8.3-1 and Table 6.8.3-2:

-	2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n78

-	5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n1

6.8.4	∆TIB and ∆RIB values

For DC_3A_n1A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.8.5	MSD

Based on Table 5.3.5 in TR 37.716-21-21 [2] it is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.8.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n78A or DC_3C_n1A-n78A | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | FDD | IMD5 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3720 | 10 | 52 | 3720 | N/A | TDD | N/A |

6.9	DC_3A_n7A-n78A

6.9.1	Operating bands for DC

Table 6.9.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n7A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.9.2	Channel bandwidths per operating band for DC

Table 6.9.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n78A | DC_3A_n7A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.9.3	Co-existence studies

Co-existence studies of Band 3 + Band n7 + Band n78 with UL Band 3 +Band n7 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	4th order IMD generated by dual uplink of Band 3 + Band n7 may also fall into own Rx of band n7.

-	3rd order IMD generated by dual uplink of Band 3 + Band n7 may also fall into own Rx of band n78.

Co-existence studies of Band 3 + Band n7 + Band n78 with UL Band 3 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 3.

-	4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

6.9.4	∆TIB and ∆RIB values

For DC_3A_n7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n7-n78 | 3 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n7-n78 | 3 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

6.9.5	MSD

2nd order harmonic problem generated by uplink of Band 3 already have been solved in DC_3A_n78A.

2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 already have been solved in DC_3A_n78A.

4th order IMD generated by dual uplink of Band 3 + Band n7 already have been solved in DC_3A_n7A.

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 product produced by Band 3 and n7 that impact the reference sensitivity of Band n78. Following DC_7A-n3A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.9.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.9.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 16.1 | TDD | IMD3 ¦2*fn7-*fB3¦ |

6.10	DC_3C_n7A-n78A

6.10.1	Operating bands for DC

Table 6.10.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C_n7A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.10.2	Channel bandwidths per operating band for DC

Table 6.10.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3C_n7A-n78A | DC_3C_n7A DC_3A_n7A DC_3C_n78A DC_3A_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.10.3	Co-existence studies

Co-existence studies of Band 3 + Band n7 + Band n78 already have been captured in current specification for DC_3A_n7A-n78A.

6.10.4	∆TIB and ∆RIB values

For DC_3C_n7A-n78A, the TIB,c and RIB values can reuse the same requirement for DC_3A_n7A-n78A.

6.10.5	MSD

MSD requirement for DC_3C_n7A-n78A can reuse the same requirement for DC_3A_n7A-n78A.

6.11	DC_13A_n5A-n260A

6.11.1	Operating bands for DC

Table 6.11.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13A_n5A-n260A | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.11.2	Channel bandwidths per operating band for DC

Table 6.11.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_13A_n5A-n260A | DC_13A_n5A DC_13A_n260A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

6.11.3	Co-existence studies

Co-existence studies of Band 13 + Band n5 + Band n260 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.11.4	∆TIB and ∆RIB values

For DC_13A_n5A-n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n5-n260 | 13 | 0.5 |
|  | n5 | 0.5 |
|  | n260 | 0 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n5-n260 | 13 | 0 |
|  | n5 | 0 |
|  | n260 | 0 |

6.11.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_13A_n5A-n260A.

6.12	DC_66A_n5A-n260A

6.12.1	Operating bands for DC

Table 6.12.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n5A-n260A | 66 | 1710 MHz | – | 1710 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.12.2	Channel bandwidths per operating band for DC

Table 6.12.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_66A_n5A-n260A | DC_66A_n5A DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

6.12.3	Co-existence studies

Co-existence studies of Band 66 + Band n5 + band n260 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.12.4	∆TIB and ∆RIB values

For DC_66A_n5A-n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | 0.3 |
|  | n5 | 0.3 |
|  | n260 | 0 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | 0 |
|  | n5 | 0 |
|  | n260 | 0 |

6.12.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_66A_n5A-n260A.

6.13	DC_3A_n20A-n78A

6.13.1	Operating bands for DC

Table 6.13.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit | BS transmit / UE receive |  |
|  |  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| DC_3A_n20A-n78A | 3 | 1710 MHz -– 1785 MHz | 1805 MHz –1880 MHz | FDD |
|  | n20 | 832 MHz -– 862 MHz | 791 MHz – 821 MHz | FDD |
|  | n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |

6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 140 |
|  |  | n20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.13.3	Co-existence studies

Co-existence studies of Band 3 + Band n20 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.13.4	∆TIB and ∆RIB values

For DC_ the TIB,c and RIB,c values are derived from CA_3-20-42 and are given in the tables below.

Table 6.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n20-n78 | 3 | 0.6 |
|  | n20 | 0.3 |
|  | n78 | 0.8 |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n20-n78 | 3 | 0.2 |
|  | n20 | 0 |
|  | n78 | 0.5 |

6.13.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_3_n20-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.13.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | TDD | IMD3 |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 52 | 3500 | 4.5 | TDD | IMD5 |

6.14	DC_28A_n8A-n78A

6.14.1	Operating bands for DC

Table 6.14.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n8A-n78A | 28 | 703 MHz |  | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.14.2	Channel bandwidths per operating band for DC

Table 6.14.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n8A-n78A | DC_28A_n8A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 140 |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.14.3	Co-existence studies

Co-existence studies of Band 28 + Band n8 + band n78 are already captured in Table 5.3-1 of TR 37.864-41-21 [2].

6.14.4	∆TIB and ∆RIB values

For DC_28A_n8A-n78A, the TIB,c and RIB,c values are derived from CA_8-28-41 and are given in the tables below.

Table 6.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n8-n78 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.3 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n8-n78 | 28 | 0.1 |
|  | n8 | 0.2 |
|  | n78 | 0 |

6.14.5	MSD

Based on the co-existence studies in TR 37.864-41-21 [2], DC_28A_n8A-n78A need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.14.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3458 | 10 | 52 | 3458 | 9.1 | TDD | IMD4 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3762 | 10 | 52 | 3762 | 3.7 | TDD | IMD5 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | TDD | IMD5 |
|  | n78 | 3787 | 10 | 52 | 3787 | N/A | FDD | N/A |

6.15	DC_28A_n8A-n258A

6.15.1	Operating bands for DC

Table 6.15.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n8A-n258A | 28 | 703 MHz |  | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n258 | 24250 MHz |  | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A_n8A-n258A | DC_28A_n8A DC_28A_n258A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |

6.15.3	Co-existence studies

Co-existence studies of Band 28 + Band n8 + band n258 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.15.4	∆TIB and ∆RIB values

For DC_28_n8-n258, the TIB,c and RIB,c values are given in the tables below.

Table 6.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n8-n258 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n258 | 0 |

Table 6.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n8-n258 | 28 | 0.1 |
|  | n8 | 0.2 |
|  | n258 | 0 |

6.15.5	MSD

Based on the co-existence findings in TR [2] MSD does not need to be defined for DC_28_n8-n258.

6.16	DC_3C_n28A-n78A

6.16.1	Operating bands for DC

Table 6.16.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3C_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.16.2	Channel bandwidths per operating band for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3C_n28A-n78A | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.16.3	Co-existence studies

Co-existence studies of Band 3 + Band n28 + Band n78 with UL Band 3 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	2nd , 4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band 3.

-	4th , 5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

Co-existence studies of Band 3 + Band n28 + Band n78 with UL Band 3 +Band n28 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd order harmonic generated by uplink of Band 3 may also fall into own Rx of band n78.

-	5th order harmonic generated by uplink of Band n28 may also fall into own Rx of band n78.

-	5th order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n78.

6.16.4	∆TIB and ∆RIB values

For DC_3C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |

Table 6.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n28-n78 | 3 | 0.2 |
|  | n28 | 0 |
|  | n78 | 0.5 |

6.16.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], 5th order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n78. Following DC_3A_n28A-n78A requirement in table 7.3B.2.3.5.2-1 of TS 38.101-3, reference sensitivity exception requirements are defined in Table 6.16.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.16.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A |  | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 ¦2*fn28-3*fB3¦ |

6.17	DC_7C_n28A-n78A

6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7C_n28A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7C_n28A-n78A | DC_7C_n28A DC_7C_n78A | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.17.3	Co-existence studies

Co-existence studies of Band 7 + Band n28 + Band n78 with UL Band 7 +Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	2nd , 5th  order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n28.

-	4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n78. But there is no issue for TDD band.

Co-existence studies of Band 7 + Band n28 + Band n78 with UL Band 7 +Band n28 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	5th order harmonic generated by uplink of Band n28 may also fall into own Rx of band n78.

-	2nd , 4th order IMD generated by dual uplink of Band 7 + Band n28 may also fall into own Rx of band n78.

6.17.4	∆TIB and ∆RIB values

For DC_7C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n28-n78 | 7 | 0.3 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |

Table 6.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n28-n78 | 7 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

6.17.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 and IMD5 product produced by Band 7 and n78 that impact the reference sensitivity of Band n28. Following DC_7A-n28A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.17.5-1.

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 and IMD4 product produced by Band 7 and n28 that impact the reference sensitivity of Band n78. Following DC_7A-n28A_n78A requirement in table 5.2-5 of TR 37.864-41-21 [3], reference sensitivity exception requirements are defined in Table 6.17.5-1.

It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.17.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | FDD | IMD2 ¦f n78-f B7 ¦ |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | TDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n28 | 731 | 5 | 25 | 786 | 4.0 | FDD | IMD5 ¦2*f n78-3*f B7 ¦ |
|  | n78 | 3387 | 10 | 50 | 3387 | N/A | TDD | N/A |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD2 ¦fn28+fB7¦ |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | 10.3 | TDD | IMD4 ¦2*f B7-2*f n28¦ |

6.18	DC_66_n71-n260

6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n71A-n260 | 66 | 1710 MHz | – | 1710 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_66A_n71A-n260A | DC_66A_n71A DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |
| DC_66A_n71A-n260(2A) | DC_66A_n71A DC_66A_n260A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.18.3	Co-existence studies

Co-existence studies of DC_66_n71 and DC_66_n260 are already captured in Rel-15 TR 37.863-01-01 [2].

6.18.4	∆TIB and ∆RIB values

For DC_66_n71-n260, the TIB,c and RIB,c values are given in the tables below.

Table 6.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n71-n260 | 66 | 0.3 |
|  | n71 | 0.3 |
|  | n260 | 0 |

Table 6.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n71-n260 | 66 | 0 |
|  | n71 | 0 |
|  | n260 | 0 |

6.18.5	MSD

No further MSD is needed.

6.19	DC_66A_n71A-n261

6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n71A-n261A DC_66A_n71A-n261(2A) | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 50 MHz |  |  | 100 MHz | 200 MHz |  | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n71A-n261A | DC_66A_n71A DC_66A_n261A | 66 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | n71 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes | Yes |  | Yes | Yes |  |  |
| DC_66A_n71A-n261(2A) | DC_66A_n71A DC_66A_n261A | 66 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 840 |
|  |  | n71 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n261 | See CA_n261(2A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.19.3	Co-existence studies

Co-existence studies of DC_66A-n71A are captured in TR 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_66A_n261A there is no requirements specified.

6.19.4	∆TIB and ∆RIB values

For DC_66A_n71A-n261A or DC_66A_n71A-n261(2A), the TIB,c and RIB,c values are given in the tables below.

Table 6.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n71-n261 | 66 | 0.3 |
|  | n71 | 0.3 |
|  | n261 | 0 |

Table 6.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n71-n261 | 66 | 0 |
|  | n71 | 0 |
|  | n261 | 0 |

6.19.5	MSD

Based on the co-existence study in 37.716-21-21, MSD does not need to be defined.

6.20	Void

6.21	DC_1A_n7A-n78A

6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n7A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n7A-n78A | DC_1A_n7A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.21.3	Co-existence studies

Co-existence studies of Band 1 + Band n7 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	4th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band n7

-	4th and 5th order IMD generated by dual uplink of Band 1 + Band n7 may also fall into own Rx of band n78

6.21.4	∆TIB and ∆RIB values

For DC_1A_n7A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n7-n78 | 1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 6.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n7-n78 | 1 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

6.21.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 product produced by Band 1 and n78 that impact the reference sensitivity of Band n7. Following DC_1A-7A_n78A requirement, reference sensitivity exception requirements are defined in Table 6.21.5-1. It is needed to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

In addition, there is IMD4 and IMD5 product produced by Band 1 and n7 that impact the reference sensitivity of Band n78. The MSD is shown in the following table.

Table 6.21.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 |  | IMD4 ¦fn78-3*fB1¦ |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A |  | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | TDD | IMD4 ¦fn7-3*fB1¦ |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A |  | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 3.8 | TDD | IMD5 ¦3*fn7-2*fB1¦ |

6.22	DC_8_n77-n257

6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n77-n257 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_8A_n77A-n257A | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_8A_n77A-n257D | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n77A-n257G | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n77A-n257H | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n77A-n257I | DC_8A_n77A DC_8A_n257A | 8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.22.3	Co-existence studies

Based on the co-existence studies of DC_8A-n77A and DC_8A-n257A are captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.22.4	∆TIB and ∆RIB values

For DC_8A_n77A-n257A, the TIB,c and RIB,c values are given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n77-n257 | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n77-n257 | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.22.5	MSD

As mentioned in 6.22.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.23	DC_1A_n40A-n78A

6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n40A-n78A | DC_1A_n40A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 300 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n78 | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.23.3	Co-existence studies

The result of co-existence studies for DC_1A-n40A-n78A are provided in TR 37.716-21-21, Table 5.3-2.

6.23.4	∆TIB and ∆RIB values

For DC_1A_n40A-n78A or DC_1A_n40A-n78(2A), the TIB,c and RIB,c values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n40-n78 | 1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 6.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n40-n78 | 1 | 0 |
|  | n40 | 0 |
|  | n78 | 0.5 |

### 6.23.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 & IMD5 products produced by Band 1 and n40 that impact the reference sensitivity of NR n78.

In addition, there is IMD4 product produced by Band 1 and n78 that impact the reference sensitivity of Band n40. The required MSD are shown in the following table.

Table 6.23.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_1A_n40A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | IMD4 | ¦3*fB1-fn40¦ | 1930 | 5 | 25 | 2120 | N/A |
|  | n40 |  |  | 2340 | 5 | 25 | 2340 |  |
|  | n78 |  |  | 3450 | 10 | 50 | 3450 | 9.8 |
|  | 1 | IMD5 | ¦2*fB1 -3*fn40¦ | 1925 | 5 | 25 | 2115 | N/A |
|  | n40 |  |  | 2390 | 5 | 25 | 2390 |  |
|  | n78 |  |  | 3320 | 10 | 50 | 3320 | 4.2 |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1960 | 5 | 25 | 2150 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 10.6 |

6.24	DC_3_n1-n79

6.24.1	Operating bands for DC

Table 6.24.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n1A-n79A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.24.2	Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

6.24.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n79A with dual uplink on DC_3A_n1A and DC_3A_n79A were calculated and listed in Table 6.24.3-1 and 6.24.3-2 respectively.

Table 6.24.3-1: Harmonic and IMD analysis for DC_3_n1-n79 with UL on DC_3_n1

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 540 | 270 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 | 7530 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.24.3-2: Harmonic and IMD analysis for DC_3_n1-n79 with UL on DC_3_n79

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 13200 | 15000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2615 | 3290 | 6110 | 6785 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1580 | 830 | 7015 | 8290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7820 | 8570 | 10510 | 11785 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 130 | 955 | 11415 | 13290 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 6580 | 5230 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9530 | 10355 | 14910 | 16785 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 12220 | 13570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 18290 | 15815 | 2740 | 1840 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11580 | 9630 | 3445 | 4870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19310 | 21785 | 11240 | 12140 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16620 | 18570 | 13930 | 15355 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n79 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.24.3-1 and Table 6.24.3-2:

-	5th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n79

-	5th order IMD generated by dual uplink of Band 3 + Band n79 may also fall into own Rx of band n1

6.24.4	∆TIB and ∆RIB values

For DC_3A_n1A-n79, the TIB,c and RIB values are given in the tables below.

Table 6.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n1A-n79A | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n79A | 3 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

### 6.24.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n1 that impact the reference sensitivity of NR n79.

In addition, there is IMD5 product produced by Band 3 and n79 that impact the reference sensitivity of NR Band n1. The required MSD are shown in the following table.

Table 6.24.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n1A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n79A | 3 | IMD5 | ¦4*fB3-fn1¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n1 |  |  | 1930 | 5 | 25 | 2120 |  |
|  | n79 |  |  | 4950 | 40 | 216 | 4950 | 4.7 |
|  | 3 | IMD5 | ¦4*fB3 -fn79¦ | 1750 | 5 | 25 | 1845 | N/A |
|  | n79 |  |  | 4860 | 40 | 216 | 4860 |  |
|  | n1 |  |  | 1950 | 5 | 25 | 2140 | 3.6 |

6.25	DC_3_n40-n78

6.25.1	Operating bands for DC

Table 6.25.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-n40A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.25.2	Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.25.3	Co-existence studies

The result of co-existence studies for DC_3A-n40A-n78A are provided in TR 37.716-21-21, Table 5.3-2.

6.25.4	∆TIB and ∆RIB values

For DC_3A_n40A-n78A, the TIB,c and RIB,c values are given in the tables below.

Table 6.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n40A-n78A | 3 | 0.6 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 6.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n40A-n78A | 3 | 0.2 |
|  | n40 | 0 |
|  | n78 | 0.5 |

### 6.25.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n40 that impact the reference sensitivity of NR band n78.

In addition, there is IMD5 product produced by Band 3 and n78 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.25.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n40A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n78A | 3 | IMD5 | ¦2*fB3-3*fn40¦ | 1730 | 5 | 25 | 1825 | N/A |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 4.8 |
|  | 3 | IMD5 | ¦3*fB3 -2*fn78¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n78 |  |  | 3760 | 10 | 50 | 3760 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 4.4 |

6.26	DC_66_n25-n41

6.26.1	Operating bands for DC

Table 6.26.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n41 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.26.2	Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_66A_n25A-n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.26.3	Co-existence studies

Co-existence studies of Band 66 + Band n25 + Band n41 are captured in Table 6.26.3-1 and 6.26.3-2 below.

Table 6.26.3-1 lists up to 7th order harmonics and IMD up to 5th order from dual uplink of B66 and n25 falls into one of these receiving bands.

Table 6.26.3-1: Harmonic and IMD analysis 66_n25

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 5550 | 5745 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 7400 | 7660 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 9250 | 9575 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10680 | 11100 | 11490 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12460 | 12950 | 13405 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 205 | 70 | 3560 | 3695 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1505 | 1710 | 1920 | 2120 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5475 | 5410 | 5610 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 410 | 140 | 7120 | 7390 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3215 | 3490 | 3770 | 4035 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7255 | 7260 | 7525 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5950 | 5620 | 5270 | 4925 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9440 | 8690 | 9035 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2325 | 1990 | 1640 | 1300 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9305 | 8830 | 9170 |

Based on Table 6.26.3-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42. 43, 48, 49, n77 and n78.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77 and n78.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 21, 24, 33, 34, 36, 37, 39, 46, 50, 65, 70, 74 and 75, and also own Rx frequencies of band 66 and n25.

- 4th order IMD products may fall into Rx frequencies of bands 42, 43, n77 and n78.

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 11, 21, 23, 24, 32, 34, 36, 40, 45, 46, 47, 50, 65, 70, 74, 75, 76 and n79, and also own Rx frequencies of band 66 and n25.

Therefore, there is no harmonic/IMD problem into the own 3rd band B41 frequency range.

Table 6.26.3-2 lists up to 7th order harmonics and IMD up to 5th order from dual uplink of B66 and n41 falls into one of these receiving bands.

Table 6.26.3-2: Harmonic and IMD analysis 66_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7488 | 8070 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 9984 | 10760 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12480 | 13450 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10680 | 14976 | 16140 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12460 | 17472 | 18830 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 980 | 716 | 4206 | 4470 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1064 | 3212 | 3670 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6250 | 6702 | 7160 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1432 | 8412 | 8940 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2844 | 5708 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8030 | 9198 | 9850 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8204 | 4624 | 4150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12540 | 9336 | 9810 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3928 | 348 | 250 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11630 | 10122 | 10720 |

Based on Table 6.26.3-2, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42. 46, 48, 49, n77, n78 and n79.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 2nd order IMD products may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 44, 67, 68 and n79.

- 3rd order IMD products may fall into Rx frequencies of bands 5, 6, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 42, 43, 44, 46, 47, 48, 49, 67, 68, n77 and n78.

- 4th order IMD products may fall into Rx frequencies of bands 2, 3, 7, 9, 11, 21, 24, 25, 32, 33, 35, 36, 37, 38, 39, 45, 46, 47, 50, 51, 74, 75 and 76.

- 5th order IMD products may fall into Rx frequencies of bands n77 and n79.

The 4th IMD product by dual uplink of B66 and n41 will be impacted the own 3rd Rx band in Band 25 Rx frequency range.

6.26.4	∆TIB and ∆RIB values

For DC_66_n25-n41, the TIB,c and RIB values are derived as below.

Table 6.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.51 |
|  |  | 1.02 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.26.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 66 and n41 that impact the reference sensitivity of NR band n25. The required MSD is shown in the following table.

Table 6.26.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_66A_n25A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25A-n41A | 66 | IMD4 | ¦2*fB66 -2*fn41¦ | 1715 | 5 | 25 | 2115 | N/A |
|  | n41 |  |  | 2685 | 10 | 50 | 2685 |  |
|  | n25 |  |  | 1860 | 5 | 25 | 1940 | 11.0 |

The update needed in 38-101-3 as in following table.

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | DL LCRB | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | 5 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | 10 | N/A | TDD |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 | FDD | IMD4 |

6.27	DC_3_n1-n257, DC_3-3_n1-n257

6.27.1	Operating bands for DC

Table 6.27.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n257 DC_3-3_n1-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.27.2	Channel bandwidths per operating band for DC

Table 6.27.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3_n1-n257 | DC_3A_n1A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_3-3_n1-n257 | DC_3A_n1A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |

6.27.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n1A-n257A with dual uplink on DC_3A_n1A and DC_3A_n257A were calculated and listed in Table 6.27.3-1 and 6.27.3-2 respectively.

Table 6.27.3-1: Harmonic and IMD analysis for DC_3_n1-n257 with UL on DC_3_n1

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 540 | 270 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 7260 | 7530 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.27.3-2: Harmonic and IMD analysis for DC_3_n1-n257 with UL on DC_3_n257

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 26500 | 29500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 53000 | 59000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 79500 | 88500 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24715 | 27790 | 28210 | 31285 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 26080 | 22930 | 51215 | 57290 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 29920 | 33070 | 54710 | 60785 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | -24370 | -21145 | 77715 | 86790 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 55580 | 49430 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 31630 | 34855 | 81210 | 90285 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 56420 | 62570 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 116290 | 104215 | 19360 | 22660 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 85080 | 75930 | 47645 | 53870 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 107710 | 119785 | 33340 | 36640 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 82920 | 92070 | 58130 | 64355 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n257 will be captured in the constituent fallback modes. There are no self-interference issues for the 3rd own receive band of this configuration.

6.27.4	∆TIB and ∆RIB values

For DC_3A_n1A-n257, the TIB,c and RIB values are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n1A-n257A | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n1A-n257A | 3 | 0 |
|  | n1 | 0 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.27.5	MSD

There are no additional requirements for this configuration.

6.28	DC_1_n77-n258

6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n77-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.28.2	Channel bandwidths per operating band for DC

Table 6.28.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n77A-n258A | DC_1A_n77A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.28.3	Co-existence studies

Co-existence studies of DC_1A_n77A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.28.4	∆TIB and ∆RIB values

For DC_1_n77-n258, the TIB,c and RIB values are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n77-n258 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n258 | 0 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n77-n258 | 1 | 0.2 |
|  | n77 | 0.5 |
|  | n258 | 0 |

6.28.5	MSD

There is no additional MSD issue for DC_1_n77-n258.

6.29	DC_1_n78-n258

6.29.1	Operating bands for DC

Table 6.29.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n78-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.29.2	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A-n258A | DC_1A_n78A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.29.3	Co-existence studies

Co-existence studies of DC_1A_n78A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.29.4	∆TIB and ∆RIB values

For DC_1_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n78-n258 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n78-n258 | 1 | 0 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.29.5	MSD

There is no additional MSD issue for DC_1_n78-n258.

6.30	DC_1_n79-n258

6.30.1	Operating bands for DC

Table 6.30.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n79-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.30.2	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n79A-n258A | DC_1A_n79A DC_1A_n258A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.30.3	Co-existence studies

Co-existence studies of DC_1A_n79A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_1A_n258A there is no requirements specified.

6.30.4	∆TIB and ∆RIB values

For DC_1_n79-n258, the TIB,c and RIB values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n79-n258 | 1 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n79-n258 | 1 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

6.30.5	MSD

There is no additional MSD issue for DC_1_n79-n258.

6.31	DC_3_n77-n258

6.31.1	Operating bands for DC

Table 6.31.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n77-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.31.2	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n77A-n258A | DC_3A_n77A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.31.3	Co-existence studies

Co-existence studies of DC_3A_n77A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_3A_n258A there is no requirements specified.

6.31.4	∆TIB and ∆RIB values

For DC_3_n77-n258, the TIB,c and RIB values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n77-n258 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n258 | 0 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n77-n258 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n258 | 0 |

6.31.5	MSD

There is no additional MSD issue for DC_3_n77-n258.

6.32	DC_3_n78-n258

6.32.1	Operating bands for DC

Table 6.32.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.32.2	Channel bandwidths per operating band for DC

Table 6.32.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n78A-n258A | DC_3A_n78A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.32.3	Co-existence studies

Co-existence studies of DC_3A_n78A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_3A_n258A there is no requirements specified.

6.32.4	∆TIB and ∆RIB values

For DC_3_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

6.32.5	MSD

There is no additional MSD issue for DC_3_n78-n258.

6.33	DC_3_n79-n258

6.33.1	Operating bands for DC

Table 6.33.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n79-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.33.2	Channel bandwidths per operating band for DC

Table 6.33.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| EN-DC configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n79A-n258A | DC_3A_n79A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |

6.33.3	Co-existence studies

Co-existence studies of DC_3A_n79A are captured in 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_3A_n258A there is no requirements specified.

6.33.4	∆TIB and ∆RIB values

For DC_3_n79-n258, the TIB,c and RIB values are given in the tables below.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n79-n258 | 3 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

Table 6.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n79-n258 | 3 | 0 |
|  | n79 | 0 |
|  | n258 | 0 |

6.33.5	MSD

There is no additional MSD issue for DC_3_n79-n258.

6.34	DC_3-3_n78_n257

6.34.1	Operating bands for DC

Table 6.34.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.34.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| DC configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 10 |  | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-3A_n78A-n257A | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 | Yes |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-3A_n78A-n257D | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257E | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257F | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257G | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257H | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257I | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257J | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 640 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257K | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 740 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257L | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 840 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A_n78A-n257M | DC_3A_n78A DC_3A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 940 |  |
|  |  | n78 | 15 | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.34.3	Co-existence studies

Based on co-existence studies of DC_3A-n78A and DC_3A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.34.4	∆TIB and ∆RIB values

For DC_3-3_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n78-n257 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3_n78-n257 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.34.5	MSD

No additional MSD requirement is needed.

6.35	DC_20_n3-n78

6.35.1	Operating bands for DC

Table 6.35.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n3-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.35.2	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20_n3-n78 | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.35.3	Co-existence studies

Co-existence studies of Band 20 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.35.4	∆TIB and ∆RIB values

For DC_20_n3-n78, the TIB,c and RIB values are same as for DC_3-20_n78 and are given in the tables below.

Table 6.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n3-n78 | 20 | 0.3 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 6.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |

6.35.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_20_n3-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.35.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | TDD | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3500 | 10 | 52 | 3500 | 4.5 | TDD | IMD5 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 52 | 3550 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | FDD | IMD3 |

6.36	DC_7_n1-n78

6.36.1	Operating bands for DC

Table 6.36.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.36.2	Channel bandwidths per operating band for DC

Table 6.36.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7_n1-n78 |  | DC_7A_n1A DC_7A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7C_n1A-n78A |  | DC_7A_n1A DC_7C_n1A DC_7A_n78A DC_7C_n78A | 7 | 15 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.36.3	Co-existence studies

Co-existence studies of Band 7 + Band n1 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.36.4	∆TIB and ∆RIB values

For DC_7_n1-n78, the TIB,c and RIB values are same as for DC_1-7_n78 and are given in the tables below.

Table 6.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

6.36.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_7_n1-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.36.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n1A-n78A, DC_7C_n1A-n78A | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | TDD | IMD4 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 3.8 | TDD | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | FDD | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |

6.37	DC_20_n1-n78

6.37.1	Operating bands for DC

Table 6.37.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20_n1-n78 | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.37.3	Co-existence studies

Co-existence studies of Band 20 + Band n1 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.37.4	∆TIB and ∆RIB values

For DC_20_n1-n78, the TIB,c and RIB values are same as for DC_1-20_n78 and are given in the tables below.

Table 6.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n1-n78 | 20 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n1-n78 | n78 | 0.5 |

6.37.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_20_n1-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.37.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 52 | 3630 | 16.0 | TDD | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 52 | 3790 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | FDD | IMD3 |

6.38	DC_7_n3-n78

6.38.1	Operating bands for DC

Table 6.38.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7_n3-n78 | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.38.3	Co-existence studies

Co-existence studies of Band 7 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.38.4	∆TIB and ∆RIB values

For DC_7_n3-n78, the TIB,c and RIB values are same as for DC_3-7_n78 and are given in the tables below.

Table 6.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n3-n78 | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n3-n78 | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

6.38.5	MSD

Based on the co-existence studies in TR 37.716-21-21 [2] DC_7_n3-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.38.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3390 | 10 | 52 | 3390 | 16.1 | TDD | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.6 | FDD | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3480 | 10 | 52 | 3480 | N/A | TDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 9.2 | FDD | IMD4 |

6.39				DC_40A_n41A-n79A

6.39.1		Operating bands for DC

Table 6.39.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40-n41-n791,2 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. |  |  |  |  |  |  |  |  |

6.39.2		Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

6.39.3		Co-existence studies

Based on co-existence studies of Band 40 + Band n41, Band 40 + Band n79 captured in TR 37.716-11-11, the own Rx impact of the 3rd band is the followings, which is the same as that for CA_n40-n41-n79 captured in TR 38.716-03-02.

-	2nd  order IMD (4796 MHz – 5090 MHz) generated by 2UL of B40 + n41 fall into part of own band n79.

- 	4th order IMD (4210 MHz – 4704 MHz) generated by dual uplink of Band 40 + Band n41 may fall into part of own band n79. However, since this band combination will only be used in China where the frequency range above 4800 MHz for band n79 is allocated to IMT in China, there are no MSD issue.

-	As only inter-band carrier aggregation without simultaneous Rx/Tx between n40 and n41 is supported, there are no MSD issue for n41 due to dual uplink operation in Band 40 + Band n79

6.39.4		∆TIB and ∆RIB values

The same TIB,c and RIB,c values specified for 1 band UL for CA_n40-n41-n79 in TR 38.716-03-02 are used as below.

Table 6.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n41-n79 | 40 | 0.51 |
|  | n41 | 0.51 |
|  | n79 | 0.8 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and n41. |  |  |

Table 6.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_40_n41-n79 | 40 | 01 |
|  | n41 | 0.51 |
|  | n79 | 0.5 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and n41. |  |  |

6.39.5		MSD

Based on the analysis in the UE-coexistence studies, compared to its fall back mode, there is additional MSD issue due to dual uplink DC_40A_n41A operation falling into n79 DL of the third band for this band combination. The required MSD are shown in the following table.

Table 6.39.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_40A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_40A_n41A-n79A | 40 | IMD2 | ¦fB40+fn41¦ | 2340 | 5 | 25 | 2340 | N/A |
|  | n41 |  |  | 2600 | 10 | 50 | 2600 |  |
|  | n79 |  |  | 4940 | 40 | 216 | 4940 | 30.5 |

6.40	DC_3-3_n1-n78

6.40.1	Operating bands for DC

Table 6.40.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.40.2	Channel bandwidths per operating band for DC

Table 6.40.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-3A_n1A-n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.40.3	Co-existence studies

Based on co-existence studies of the constituent low order combination, DC_3A_n1A-n78A, The summary of self-interference issues for the 3rd own receive band are listed below.

-	2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may also fall into own Rx of band n78

-	5th order IMD generated by dual uplink of Band 3 + Band n78 may also fall into own Rx of band n1

6.40.4	∆TIB and ∆RIB values

For DC_3-3_n1-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n1-n78 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

Table 6.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3_n1-n78 | 3 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: HTF is assumed for Band 3 and Band n1. |  |  |

6.40.5	MSD

The MSD needed can be covered by the constituent low order combination, DC_3_n1-n78. No additional MSD requirement is needed.

6.41	DC_7-7_n1-n78

6.41.1	Operating bands for DC

Table 6.41.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-7_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.41.2	Channel bandwidths per operating band for DC

Table 6.41.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-7A_n1A-n78A | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.41.3	Co-existence studies

Based on co-existence studies of the constituent low order combination, DC_7A_n1A-n78A, The summary of self-interference issues for the 3rd own receive band are listed below.

-	4th and 5th order IMD generated by dual uplink of Band 7 + Band n1 may also fall into own Rx of band n78

-	4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n1

6.41.4	∆TIB and ∆RIB values

For DC_7-7_n1-n78, the TIB,c and RIB values are same as for DC_7_n1-n78 and are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-7_n1-n78 | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-7_n1-n78 | 7 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

6.41.5	MSD

The MSD needed can be covered by the constituent low order combination, DC_7_n1-n78. No additional MSD requirement is needed.

6.42	DC_1_n5-n78

6.42.1	Operating bands for DC

Table 6.42.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.42.2	Channel bandwidths per operating band for DC

Table 6.42.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A_n5A-n78A | DC_1A_n5A DC_1A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.42.3	Co-existence studies

Co-existence analysis for DC_1_n5 shows that there is IMD3 impact from DC_1_n5 UL to Band n78 DL.

Co-existence analysis for DC_1_n78 shows that there is IMD5 impact from DC_1_n78 UL to Band n5 DL.

6.42.4	∆TIB and ∆RIB values

For DC_1_n5-n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n5-n78 | 1 | 0.3 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n5-n78 | 1 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

6.42.5	MSD

MSD values by dual uplink are reused from DC_1A-5A_n78A for DC_1_n5-n78 band combination. So the below table is proposed to capture in TS38.101-3.

Table 6.42.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n5A-n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 ¦fn78-2*fB5¦ |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 ¦2*fn78-3*fB1¦ |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |

6.43	DC_3_n5-n78

6.43.1	Operating bands for DC

Table 6.43.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.43.2	Channel bandwidths per operating band for DC

Table 6.43.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A_n5A-n78A | DC_3A_n5A DC_3A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.43.3	Co-existence studies

Co-existence analysis for DC_3_n5 shows that there are IMD3 and IMD5 impact from DC_3_n5 UL to Band n78 DL.

Co-existence analysis for DC_3_n78 shows that there is no impact from DC_3_n78 UL to Band n5 DL.

6.43.4	∆TIB and ∆RIB values

For DC_3_n5-n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n5-n78 | 3 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n5-n78 | 3 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

6.43.5	MSD

MSD values by dual uplink are reused from DC_3A_n20A-n78A for DC_3_n5-n78 band combination. So the below table is proposed to capture in TS38.101-3.

Table 6.43.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n78 | 3420 | 10 | 52 | 3420 | 16.1 | IMD3 ¦fB3+2*fn5¦ |

6.44	DC_7_n5-n78

6.44.1	Operating bands for DC

Table 6.44.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n5-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.44.2	Channel bandwidths per operating band for DC

Table 6.44.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_7A_n5A-n78A | DC_7A_n5A DC_7A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5ADC_7A_n78A DC_7C_n78A | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.44.3	Co-existence studies

Co-existence analysis for DC_7_n5 shows that there are IMD2 impact from DC_7_n5 UL to Band n78 DL.

Co-existence analysis for DC_7_n78 shows that there IMD2 and IMD5 impact from DC_7_n78 UL to Band n5 DL.

6.44.4	∆TIB and ∆RIB values

For DC_7_n5-n78 the TIB,c and RIB,c values are given in the tables below.

Table 6.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n5-n78 | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n5-n78 | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

6.44.5	MSD

MSD values by dual uplink from UL 7_n78 into DL n5 are reused from DC_5A-7A_n78A for DC_7_n5-n78 band combination.

So the below table is proposed to capture in TS38.101-3.

Table 6.44.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | n5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 ¦fn78-fB7¦ |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 ¦2*fn78-3fB7¦ |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 ¦fB7 +fn5¦ |

6.45	DC_28_n5-n78

6.45.1	Operating bands for DC

Table 6.45.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n5-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.45.2	Channel bandwidths per operating band for DC

Table 6.45.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

6.45.3	Co-existence studies

Co-existence analysis for DC_28_n5 shows that there is IMD5 impact from DC_28_n5 UL to Band n78 DL.

Co-existence analysis for DC_28_n78 shows that there is IMD5 impact from DC_28_n78 UL to Band n5 DL.

6.45.4	∆TIB and ∆RIB values

For DC_28_n5-n78 the TIB,c and RIB,c values are reused from DC_18-28_n78 and are given in the tables below.

Table 6.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n5-n78 | 28 | 0.5 |
|  | n5 | 0.5 |
|  | n78 | 0.8 |

Table 6.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n5-n78 | n78 | 0.5 |

6.45.5	MSD

MSD values for impact from UL 28_n78 into DL n5 are reused from DC_18A-28A_n78A. So the below table is proposed to capture in TS38.101-3.

Table 6.45.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n5A-n78A | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 3.8 | IMD5 ¦fn78 -4*fB28¦ |
|  | n78 | 3721 | 10 | 50 | 3721 | N/A | N/A |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3721 | 10 | 50 | 3721 | 4.7 | IMD5 ¦fn5 +4*fB28¦ |

6.46	DC_2A_n41A-n261A

6.46.1	Operating bands for DC

Table 6.46.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n41A-n261A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz |  |

6.46.2	Channel bandwidths per operating band for DC

Table 6.46.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_2A_n41A-n261A | DC_2A_n41A | This part can be omitted unless there is some particular reason. |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n41A-n261(2A) | DC_2A_n41A | See CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

6.46.3	Co-existence studies

Based on co-existence studies of DC_2A-n41A presented in TR 37.716-01-01, there is no harmonic or intermodulation impact for the own receive band.

6.46.4	∆TIB and ∆RIB values

For DC_2_n41-n261, the TIB,c and RIB,c values are given in the tables below. FR2 does not have relaxation.

Table 6.46.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.46.4-2: ΔRIB

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n41 | 2 | 0 |
|  | n41 | 0 |

6.46.5	MSD

As mentioned in 6.46.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.47	DC_66A_n41A-n261A

6.47.1	Operating bands for DC

Table 6.47.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n41A-n261A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz |  |

6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | [40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_66A_n41A-n261A | DC_66A_n41A | This part can be omitted unless there is some particular reason. |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A-n261(2A) | DC_66A_n41A | See CA_ n261(2A) BCS0 in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

6.47.3	Co-existence studies

Co-existence studies have been done for DC_66_n41 and are captured into TR 37.716-11-11.

6.47.4	∆TIB and ∆RIB values

For DC_66_n41-n261, the TIB,c and RIB,c values are same as for DC_66_n41 which is already specified in TS 38.101-3. n261 TIB,c and RIB,c are 0 dB.

Table 6.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n261 | 66 | 0.5 |
|  | n41 | [0.81] |
|  |  | [1.32] |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.47.4-2: ΔRIB

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-261 | 66 | 0.5 |
|  | n41 | [0.51] |
|  |  | [12] |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.47.5	MSD

Based on the studies made for DC_66_n41 MSD due to close proximity is needed and is already specified in TS 38.101-3.

6.48	DC_2_n41-n260

6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n41-n260 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_2A_n41A-n260A | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A_n41A-n260(2A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n41A-n260(3A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1320 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n41A- n260(4A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1720 |
|  |  | n41 | 15 |  | Yes | § | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.48.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band n260 DL.

Co-existence analysis for DC_2_n260 shows that there is no impact from DC_2_n260 UL to Band n41 DL.

6.48.4	∆TIB and ∆RIB values

For DC_2_n41-n260 the TIB,c and RIB,c values are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41-n260 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n41-n260 | 2 | 0 |
|  | n41 | 0 |
|  | n260 | 0 |

6.48.5	MSD

No further MSD is needed.

6.49	DC_66_n41-n260

6.49.1	Operating bands for DCTable 6.49.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n41-n260 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_66A_n41A-n260A | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_66A_n41A-n260(66A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 920 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A-n260(3A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1320 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n41A- n260(4A) | DC_66A_n41A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1720 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) in Table 5.5A.2-1 of TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.49.3	Co-existence studies

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n260 DL.

Co-existence analysis for DC_66_n260 shows that there is no impact from DC_66_n260 UL to Band n41 DL.

6.49.4	∆TIB and ∆RIB values

For DC_66_n41-n260 the TIB,c and RIB,c values are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n260 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n41-n260 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n260 | 0 |

6.49.5	MSD

No further MSD is needed.

6.50	DC_3_n77-n257

6.50.1	Operating bands for DC

Table 6.50.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n77A-n257A | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_3A_n77A-n257D | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n77A-n257G | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n77A-n257H | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n77A-n257I | DC_3A_n77A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.50.3	Co-existence studies

Based on the co-existence studies of DC_3A-n77A and DC_3A-n257A are captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.50.4	∆TIB and ∆RIB values

For DC_3_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n77-n257 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n77-n257 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.50.5	MSD

As mentioned in 6.50.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.51	DC_8_n3-n28

6.51.1	Operating bands for DC

Table 6.51.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.51.2	Channel bandwidths per operating band for DC

Table 6.51.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8_n3-n28 | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 60 |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.51.3	Co-existence studies

Based on the co-existence studies of DC_8A-n3A and DC_8A-n28A captured in TR 37.716-11-11 and 37.863-01-01, 2nd and 5th order IMD generated by dual uplink of Band 8 + Band n3 may also fall into own Rx of band n28.

6.51.4	∆TIB and ∆RIB values

For DC_8A_n3A-n28A, the TIB,c and RIB,c values are given in the tables below.

Table 6.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n28 | 8 | 0.6 |
|  | n3 | 0.3 |
|  | n28 | 0.5 |

Table 6.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n28 | 8 | 0.2 |
|  | n3 | 0 |
|  | n28 | 0.1 |

### 6.51.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 8 and n3 that impact the reference sensitivity of NR band n28. The required MSD are shown in the following table.

Table 6.51.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n3A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n3A-n28A | 8 | IMD2 | ¦fB8-fn3¦ | 912.5 | 5 | 25 | 957.5 | N/A |
|  | n3 |  |  | 1712.5 | 5 | 25 | 1807.5 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 30.4 |
|  | 8 | IMD5 | ¦3*fB8 -2*fn3¦ | 910 | 5 | 25 | 955 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 2.6 |

6.52	DC_11_n77-n257

6.52.1	Operating bands for DC

Table 6.52.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n77-n257 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.52.2	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_11A_n77A-n257A | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_11A_n77A-n257D | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_11A_n77A-n257G | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 310 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_11A_n77A-n257H | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 410 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_11A_n77A-n257I | DC_11A_n77A DC_11A_n257A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 510 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.52.3	Co-existence studies

Based on the co-existence studies of DC_11A-n77A and DC_11A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.52.4	∆TIB and ∆RIB values

For DC_11_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n77-n257 | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n77-n257 | 11 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.52.5	MSD

As mentioned in 6.52.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.53		DC_3A_n40A-n41A

6.53.1	Operating bands for DC

Table 6.53.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n40A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n41A | DC_3A_n40A DC_3A_n41A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 210 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.53.3	Co-existence studies

The co-existence studies of Band 3 + Band n40 and Band 3 + Band n41 have been captured in TR 37.863-01-01 and TR 37.716-11-11, where there are no additional harmonic and IMD impact for the additional band receiver.

    - 6.53.4	∆TIB and ∆RIB values

For DC_3A_n40A-n41A, the TIB,c and RIB values are given in the tables below.

Table 6.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n40A-n41A | 3 | 0.5 |
|  | n40 | 0.5 |
|  | n41 | 0.51 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. |  |  |

Table 6.53.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n40A-n41A | 3 | 0 |
|  | n40 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. |  |  |

6.53.5	MSD

According the co-existing studies, there is no need to specify additional MSD requirement for this UL DC configuration.

6.54	DC_8A_n1A-n78A

6.54.1	Operating bands for DC

Table 6.54.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n1A-n78A | 8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.54.2	Channel bandwidths per operating band for DC

Table 6.54.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n1A-n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 130 |
|  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.54.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_8A_n1A-n78A with dual uplink on DC_8A_n1A and DC_8A_n78A were calculated and listed in Table 6.54.3-1 and 6.54.3-2 respectively.

Table 6.54.3-1: Harmonic and IMD analysis for DC_8_n1-n78 with UL on DC_8_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

Table 6.54.3-2: Harmonic and IMD analysis for DC_8_n1-n78 with UL on DC_8_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5630 | 7480 | 8515 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5840 | 4770 | 8360 | 9430 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 13230 | 9240 | 10345 |

The coexistence studies of Band 8 + Band n1 and Band 8 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.54.3-1 and Table 6.54.3-2:

-	3rd order IMD generated by dual uplink of Band 8 + Band n1 may fall into own Rx of band n78

6.54.4	∆TIB and ∆RIB values

For DC_8A_n1A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n78 | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n78 | 8 | 0.2 |
|  | n1 | 0 |
|  | n78 | 0.5 |

6.54.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 8 and n1 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.54.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n1A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n78A | 8 | IMD3 | ¦2*fB8+fn1¦ | 900 | 5 | 25 | 945 | N/A |
|  | n1 |  |  | 1945 | 5 | 25 | 2135 |  |
|  | n78 |  |  | 3745 | 10 | 50 | 3745 | 14.9 |

6.55	DC_66_n41-n71

6.55.1	Operating bands for DC

Table 6.55.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n41-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n41A-n71A | DC_66A_n41A DC_66A_n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

6.55.3	Co-existence studies

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n71 DL.

Co-existence analysis for DC_66_n71 shows that there ia no impact from DC_66_n71 UL to Band n41 DL.

6.55.4	∆TIB and ∆RIB values

For DC_66_n41-n71 the TIB,c and RIB,c values are derived from LTE combination CA_4-7-28 and are given in the tables below.

Table 6.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.55.5	MSD

Based on the co-existence studies there is no need to define MSD values.

6.56	DC_8_n78-n257

6.56.1	Operating bands for DC

Table 6.56.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n78-n257 | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_8A_n78A-n257A | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_8A_n78A-n257D | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257E | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 710 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257F | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 910 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257G | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 310 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257H | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 410 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257I | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257J | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 610 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257K | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 710 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257L | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 810 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A_n78A-n257M | DC_8A_n78A DC_8A_n257A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 910 |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | n257 |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.56.3	Co-existence studies

Based on co-existence studies of DC_8A-n78A and DC_8A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the third band receiver.

- DC_8A_n78A: Harmonic problem is already specified in TR 37.863-01-01.

- DC_8A_n257A: No harmonic problems by 7th & 8th order between LTE and FR2

6.56.4	∆TIB and ∆RIB values

For DC_8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n78-n257 | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 8. |  |  |

Table 6.56.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8_n78-n257 | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |
| NOTE 1: HTF is assumed for Band 8. |  |  |

6.56.5	MSD

As mentioned in 6.56.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.57	DC_2_n41-n71

6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n41A-n71A | DC_2A_n41A DC_2A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 140 |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

6.57.3	Co-existence studies

As can be seen in the co-existence analysis for DC_2_n41 in TR [2] shows that there are IMD2 and IMD5 impact from DC_2_n41 UL to Band n71 DL.

As can be seen in the co-existence analysis for DC_2_n71 in TR [3] shows that there IMD2 impact from DC_2_n71 UL to Band n41 DL.

6.57.4	∆TIB and ∆RIB values

For DC_2_n41-n71 the TIB,c and RIB,c values are derived from LTE combinations CA_2-7-12, CA_2-7-28, CA_3-7-28 and DC_3-7_n28 and are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.3 |

Table 6.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n41-n71 | 2 | 0 |
|  | n41 | 0 |
|  | n71 | 0 |

6.57.5	MSD

MSD values for UL 2_n41 into DL n71 and UL 2_n71 into DL n41 are needed. Below table is to be captured in Table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.57.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 686 | 5 | 50 | 640 | 3.8 | IMD5 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |

6.58	DC_1_n3-n28

6.58.1	Operating bands for DC

Table 6.58.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.58.2	Channel bandwidths per operating band for DC

Table 6.58.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.58.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n28A captured in TR 37.716-11-11 and 37.863-01-01, 5th order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band n3.

6.58.4	∆TIB and ∆RIB values

For DC_1_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n28 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |

Table 6.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n28 | 1 | 0 |
|  | n3 | 0 |
|  | n28 | 0.2 |

6.58.5	MSD

As mentioned in 6.58.3, IMD5 of B1 and n28 to Band n3 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 6.58.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-n3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |  |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |  |

6.59 	DC_39_n41-n79

### 6.59.1	Operating bands for DC

Table 6.59.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39A_n41A-n79A | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.59.2	Channel bandwidths per operating band for DC

Table 6.59.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n41A-n79A | DC_39A_n41A DC_39A_n79A | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.59.3	Co-existence studies

The co-existence studies of Band 39 + Band n41 and Band 39 + Band n79 have been captured in TR 37.716-01-01, where:

- IMD2 & IMD5 products produced by Band 39 and n41 that impact the reference sensitivity of NR band n79.

- IMD2 & IMD5 products produced by Band 39 and n79 that impact the reference sensitivity of NR band n41.

    - 6.59.4	∆TIB and ∆RIB values

For DC_39_n41-n79, the TIB,c and RIB values are given in the tables below.

Table 6.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39_n41-n79 | 39 | 0.5 |
|  | n41 | 0.5 |
|  | n79 | 0.8 |

Table 6.59.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39_n41-n79 | 39 | 0.2 |
|  | n41 | 0.2 |
|  | n79 | 0.5 |

### 6.59.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 39 and n41 that impact the reference sensitivity of NR band n79.

In addition, there are IMD2 & IMD5 products produced by Band 39 and n79 that impact the reference sensitivity of NR band n41. The required MSD are shown in the following table.

Table 6.59.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n41A-n79A | 39 | IMD2 | ¦fB39-fn41¦ | 1900 | 5 | 25 | 1900 | N/A |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 |  |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 | 29.8 |
|  | 39 | IMD5 | ¦4*fB39 -fn41¦ | 1885 | 5 | 25 | 1885 | N/A |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 |  |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 | 4.4 |
|  | 39 | IMD2 | ¦fB39 -fn79¦ | 1900 | 5 | 25 | 1900 | N/A |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 |  |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 | 30.2 |
|  | 39 | IMD5 | ¦4*fB39 -fn79¦ | 1885 | 5 | 25 | 1885 | N/A |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 |  |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 | 4.3 |

6.60	DC_12_n7-n78

6.60.1	Operating bands for DC

Table 6.60.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n7-n78 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.60.2	Channel bandwidths per operating band for DC

Table 6.60.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration |  | UL Configuration |  | NR Band |  | SCS [kHz] |  | 5 |  | 10 |  | 15 |  | 20 |  | 40 |  | 50 |  | 60 |  | 80 |  | 90 |  | 100 |  | Max BW [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n7A-n78A |  | DC_12A_n7A DC_12A_n78A |  | 12 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_12A_n7(2A)-n78A |  | DC_12A_n7A DC_12A_n78A |  | 12 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_12A_n7A-n78(2A) |  | DC_12A_n7A DC_12A_n78A |  | 12 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n7(2A)-n78(2A) |  | DC_12A_n7A DC_12A_n78A |  | 12 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 250 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.60.3	Co-existence studies

Table 6.60.3-1 and Table 6.60.3-2 list 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.60.3-1: Band 12 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2584 | 3101 | 3999 | 4516 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2402 | 1868 | 5884 | 6901 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4698 | 5232 | 7299 | 8316 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1703 | 1152 | 9184 | 10701 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5397 | 5948 | 10599 | 12099 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 5168 | 6202 | 7998 | 9032 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1004 | 436 | 12484 | 14501 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 6096 | 6664 | 13899 | 15916 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 5503 | 4452 | 8468 | 10002 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11298 | 12832 | 8697 | 9748 |

Table 6.60.3-2: Band 12 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1784 | 1871 | 3199 | 3286 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 1172 | 1068 | 4284 | 4441 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3898 | 4002 | 5699 | 5856 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 473 | 352 | 6784 | 7011 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4597 | 4718 | 8199 | 8409 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3568 | 3742 | 6398 | 6572 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 226 | 364 | 9284 | 9581 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5296 | 5434 | 10699 | 10996 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3043 | 2852 | 6068 | 6312 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 8898 | 9142 | 7097 | 7288 |

Based on Table 6.60.3-1 and Table 6.60.3-2, there is a MSD issue for the following cases.

-	2nd order IMD of DC_12A_n78A UL will fall into Rx of bands n7

-	4th order IMD of DC_12A_n7 UL will fall into Rx of bands n78

6.60.4	∆TIB and ∆RIB values

For DC_12_n7-n78, the TIB,c and RIB values are given in the tables below.

Table 6.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n7-n78 | 12 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |

Table 6.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n7-n78 | 12 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

### 6.60.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 12 and n7 that impact the reference sensitivity of NR band n78.

In addition, there is IMD2 product produced by Band 12 and n78 that impact the reference sensitivity of NR Band n7. The required MSD are shown in the following table.

Table 6.60.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_12A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n7A-n78A DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | IMD4 | ¦2*fB12 -2*fn7¦ | 708 | 5 | 25 | 738 | N/A |
|  | n7 |  |  | 2520 | 5 | 25 | 2640 |  |
|  | n78 |  |  | 3624 | 10 | 50 | 3624 | 9.0 |
|  | 12 | IMD2 | ¦fB12 -fn78¦ | 708 | 5 | 25 | 738 | N/A |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 |  |
|  | n7 |  |  | 2542 | 5 | 25 | 2662 | 29.6 |

6.61	DC_2_n66-n78

### 6.61.1	Operating bands for DC

Table 6.61.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A_n66A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.61.2	Channel bandwidths per operating band for DC

Table 6.61.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_2A_n66A-n78A | DC_2A_n66A DC_2A_n78A | 2 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 |  | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.61.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_2A_n66A-n78A with dual uplink on DC_2A_n66A and DC_2A_n78A were calculated and listed in Table 6.61.3-1 and 6.61.3-2 respectively.

Table 6.61.3-1: Harmonic and IMD analysis for DC_2_n66-n78 with UL on DC_2_n66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 5130 | 5340 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 70 | 200 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1920 | 2110 | 1510 | 1710 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5600 | 5270 | 5470 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3770 | 4020 | 3220 | 3490 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7510 | 6980 | 7250 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 140 | 400 | 7120 | 7380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4930 | 5270 | 5620 | 5930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 9030 | 9110 | 9420 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1310 | 1640 | 1990 | 2310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9160 | 8970 | 9290 |

Table 6.61.3-2: Harmonic and IMD analysis for DC_2_n66-n78 with UL on DC_2_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13310 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2780 | 3900 | 10300 | 11420 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11290 | 13350 | 3600 | 4340 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6080 | 7700 | 870 | 2050 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

The coexistence studies of Band2 + Band n66 and Band 2 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.61.3-1 and Table 6.61.3-2:

-	2nd and 4th order IMD generated by dual uplink of Band 2 + Band n66 may also fall into own Rx of band n78

-	4th order IMD generated by dual uplink of Band 2 + Band n78 may also fall into own Rx of band n66.

### 6.61.4	∆TIB and ∆RIB values

For DC_2A_n66A-n78A, the TIB,c and RIB,c values can reuse the values of DC_2A-66A_n78A as given in the tables below.

Table 6.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n78 | 2 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.61.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n78 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |

### 6.61.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD4 products produced by Band 2 and n66 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.61.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_2A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A-n78A | 2 | IMD2 | ¦fB2+fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 29.4 |
|  | 2 | IMD4 | ¦fB2 -3*fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3340 | 10 | 50 | 3340 | 8.9 |

Based on Table 6.61.3-2,  there is 4th order IMD generated by dual uplink of Band 2 + Band n78that impact the reference sensitivity of NR band n66, and the required MSD can reuse the MSD value of DC_2A-66A_n78A as defined in Table 7.3B.2.3.5.2-1 in TS38.101-3.

Table 6.61.5-2: MSD exception for Scell due to dual uplink operation for EN-DC_2A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A-n78A | 2 | IMD4 | ¦3*fB2-fn78¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | [10.3] |
|  | n78 |  |  | 3480 | 10 | 50 | 3480 | N/A |

6.62	DC_12_n66-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.62.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 12 and n66 that impact the reference sensitivity of NR band n78.

In addition, there is IMD3 product produced by Band 12 and n78 that impact the reference sensitivity of NR Band n66. The required MSD are shown in the following table.

Table 6.62.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_12A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n66A-n78A | 12 | IMD5 | ¦2*fB12-3*fn66¦ | 703 | 5 | 25 | 733 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 |  |
|  | n78 |  |  | 3754 | 10 | 50 | 3754 | 4.1 |
|  | 12 | IMD3 | ¦2*fB12 -fn78¦ | 703 | 5 | 25 | 733 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 |  |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 16.5 |

6.63	DC_7_n66-n78 and DC_7-7_n66-n78

6.63.1	Operating bands for DC

Table 6.63.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.63.2	Channel bandwidths per operating band for DC

Table 6.63.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band |  | SCS (kHz) |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |
| DC_7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |  | 7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |  | 7 | 15 |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 180 |  |
|  |  |  | n66 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |  | 7 | 15 |  | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 180 |  |
|  |  |  | n66 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |

6.63.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_7A_n66A-n78A, DC_7C_n66A-n78A and DC_7A-7A_n66A-n78A with dual uplink on DC_7A_n66A and DC_7A_n78A were calculated and listed in Table 6.63.3-1 and 6.63.3-2 respectively.

Table 6.63.3-1: Harmonic and IMD analysis for DC_7_n66-n78 and DC_7-7_n66-n78 with UL on DC_7_n66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 5130 | 5340 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 720 | 860 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3430 | 850 | 1060 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6710 | 6920 | 5920 | 6130 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5720 | 6000 | 2560 | 2840 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9210 | 9490 | 7630 | 7910 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4270 | 4620 | 8220 | 8570 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10 | 340 | 3940 | 4290 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10130 | 10480 | 10920 | 11270 |

Table 6.63.3-2: Harmonic and IMD analysis for DC_7_n66-n78 and DC_7-7_n66-n78 with UL on DC_7_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1460 | 2600 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10630 | 12700 | 6200 | 6980 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4760 | 6400 | 100 | 1110 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

The coexistence studies of Band7 + Band n66 and Band 7 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.63.3-1 and Table 6.63.3-2:

-	3rd order IMD generated by dual uplink of Band 7 + Band n66 may also fall into own Rx of band n78

-	4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n66.

6.63.4	∆TIB and ∆RIB values

For DC_7A_n66A-n78A, DC_7C_n66A-n78A and DC_7A-7A_n66A-n78A, the TIB,c and RIB,c values can reuse the values of DC_7A-66A_n78A as given in the tables below.

Table 6.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n66-n78 DC_7-7_n66-n78 | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.63.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7_n66-n78 DC_7-7_n66-n78 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 6.63.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 7 and n66 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.63.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_7A_n66A-n78A, EN-DC_7C_n66A-n78A and EN-DC_7A-7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | IMD3 | ¦2*fB7-fn66¦ | 2542 | 5 | 25 | 2662 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3344 | 10 | 50 | 3344 | 16.0 |

Based on Table 6.63.3-2  there is 4th order IMD generated by dual uplink of Band 7 + Band n78that impact the reference sensitivity of NR band n66, and the required MSD can reuse the MSD value of DC_7A-66A_n78A as defined in Table 7.3B.2.3.5.2-1 in TS38.101-3.

Table 6.63.5-2: MSD exception for Scell due to dual uplink operation for EN-DC_7A_n66A-n78A and EN-DC_7A-7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | IMD4 | ¦2*fn78-2*fB7¦ | 2550 | 5 | 25 | 2685 | N/A |
|  | n66 |  |  | 1750 | 5 | 25 | 2150 | 8.7 |
|  | n78 |  |  | 3625 | 10 | 50 | 3475 | N/A |

6.64	DC_20_n1-n28

6.64.1	Operating bands for DC

Table 6.64.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n28 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| NOTE: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

6.64.2	Channel bandwidths per operating band for DC

Table 6.64.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | uplink configuration | E-UTRA and NR Band | scs [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n1A-n28A | DC_20A_n1A DC_20A_n28A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 60 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.64.3	Co-existence studies

Table 6.64.3-1: Harmonic and IMD analysis for DC_20_n1-n28 with UL on DC_20_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1058 | 1148 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 316 | 196 | 2978 | 3128 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3584 | 3704 | 4672 | 4822 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 516 | 666 | 4898 | 5108 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4416 | 4566 | 6592 | 6802 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2296 | 2116 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7088 | 6818 | 1528 | 1348 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 | 8782 | 5248 | 5428 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4276 | 4036 | 1254 | 1464 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 | 7664 | 6336 | 6546 |

Table 6.64.3-2: Harmonic and IMD analysis for DC_20_n1-n28 with UL on DC_20_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 733 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1406 | 1466 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2109 | 2199 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 159 | 99 | 1535 | 1595 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 931 | 1021 | 544 | 634 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367 | 2457 | 2238 | 2328 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 1763 | 1883 | 1247 | 1367 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3199 | 3319 | 2941 | 3061 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 198 | 318 | 3070 | 3190 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2100 | 1950 | 2745 | 2595 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 | 3794 | 4031 | 4181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 535 | 385 | 1180 | 1030 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 | 3923 | 3902 | 4052 |

The coexistence studies of Band 20 + Band n1 and Band 20 + Band n28 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.64.3-1 and Table 6.64.3-2:

-	3rd harmonic of n28 uplink will interfere band 1 downlink but this is already specified

6.64.4	∆TIB and ∆RIB values

For DC_20A_n1A-n28, the TIB,c and RIB values are given in the tables below and are based on E-UTRA CA_1-20-28.

Table 6.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n1-n28 | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n28 | 0.6 |

Table 6.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n1-n28 | 20 | 0 |
|  | n1 | 0.2 |
|  | n28 | 0.2 |

### 6.64.5	MSD

Based on section 6.64.3, RAN4 do not need to study the additional MSD requirements.

6.65	DC_20_n7-n28

### 6.65.1	 Operating bands for DC

Table 6.65.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n7A-n28A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.65.2 	Configuration for DC

Table 6.65.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n7A-n28A5,6 | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 100 |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.65.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.65.3-1.

Table 6.65.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 733 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1406 | 1466 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2109 | 2199 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 2812 | 2932 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 3515 | 3665 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 159 | 99 | 1535 | 1595 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 931 | 1021 | 544 | 634 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367 | 2457 | 2238 | 2328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1763 | 1883 | 1247 | 1367 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 | 3319 | 2941 | 3061 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 198 | 318 | 3070 | 3190 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2100 | 1950 | 2745 | 2595 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 535 | 385 | 1180 | 1030 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 | 3794 | 4031 | 4181 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 | 3923 | 3902 | 4052 |

For UE coexistence study of Band 20 + Band n7, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.65.3-2.

Table 6.65.3-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1638 | 1738 | 3332 | 3432 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 906 | 776 | 4138 | 4308 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4164 | 4294 | 5832 | 6002 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 86 | 6638 | 6878 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 | 5156 | 8332 | 8572 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3476 | 3276 | 6664 | 6864 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9448 | 9138 | 948 | 758 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6046 | 5776 | 2414 | 2644 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 | 11142 | 5828 | 6018 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 | 9434 | 7496 | 7726 |

IMD5 may fall into Rx of band n7 with UL DC_20A_n28A.

IMD3 and IMD5 may fall into Rx of band n28 with UL DC_20A_n7A.

### 6.65.4	 ∆TIB and ∆RIB values

Table 6.65.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n7-n28 | 20 | 0.5 |
|  | n7 | 0.3 |
|  | n28 | 0.5 |

Table 6.65.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n7-n28 | 20 | 0.2 |
|  | n7 | 0 |
|  | n28 | 0.2 |

### 6.65.5	MSD

Based on Table 6.65-3 , there are IMD3 & IMD5 products produced by Band 20 and n7 that impact the reference sensitivity of NR band n28. Also, there is IMD5 product produced by Band 20 and NR band n28 that impact the reference sensitivity of NR band n7. The required MSD are shown in the following table.

Table 6.65.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n7A-n28A with UL DC_20A_n7A in NR FR1 (three bans)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n7A-n28A | 20 | IMD3 | ¦2*fB20-fn7¦ | 857 | 5 | 25 | 816 | N/A |
|  | n7 |  |  | 2512 | 5 | 25 | 2632 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 15.7 |
|  | 20 | IMD5 | ¦4*fB20-fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 2.8 |

For IMD5 generated by UL DC_20A_n28A, the MSD exception can be referred to DC_7A-20A_n28A with UL DC_20A_n28A.

Table 6.65.5-2: MSD test points for Scell due to dual uplink operation for EN-DC 20A_n7A-n28A with UL_DC_20A_n28A in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n7A-n28A | n7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
|  | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |

6.66	DC_20_n7-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.66.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD4 products produced by Band 20 and n7 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.66.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n7A-n78A | 20 | IMD2 | ¦fB20+fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n78 |  |  | 3392 | 10 | 50 | 3392 | 28.8 |
|  | 20 | IMD4 | ¦2*fB20-2*fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n78 |  |  | 3436 | 10 | 50 | 3436 | 8.5 |

6.67	DC_3_n7-n28

### 6.67.1	Operating bands for DC

Table 6.67.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n7-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |

### 6.67.2	Channel bandwidths per operating band for DC

Table 6.67.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n7A-n28A | DC_3A_n7A DC_3A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 90 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_3C_n28A | 3 | 15 | See CA_3C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.67.3	Co-existence studies

The harmonics and intermodulation products analysis for 2UL bands 3 and 7 with 3DL bands 3, 7 and 28 is presented in clause 6.11.1.2 in TR 36.879-13.

The harmonics and intermodulation products analysis for 2UL bands 3 and 28 with 3DL bands 3, 7 and 28 is presented in clause 6.21.1.2 in TR 36.715-00-02.

Based on the above studies,

-	2nd order IMD generated by dual uplink of Band 3 + Band n7 may fall into own Rx of band n28

-	3rd order IMD generated by dual uplink of Band 3 + Band n28 may fall into own Rx of band n7

### 6.67.4	∆TIB and ∆RIB values

For DC_3_n7-n28, the TIB,c and RIB values can be based on E-UTRA CA_3-7-28 given in the tables below.

Table 6.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n7-n28 | 3 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.3 |

Table 6.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n7-n28 | 3 | 0 |
|  | n7 | 0 |
|  | n28 | 0 |

### 6.67.5	MSD

Table 6.67.5-1 lists the MSD for DC_3A_n7A-n28A which is reused from E-UTRA 3DL/2UL CA_3A_7A-28A in TS 36.101. The MSD values were previously derived in clause 6.11.1.4 in TR 36.879-13 and clause 6.21.1.4 in TR 36.715-00-02.

Table 6.67.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| DC_3A_n7A-n28A | DC_3A_n7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | n7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | n28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | CA_3A-n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | n7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  | IMD3 |
|  |  | n28 | 743 | 5 | 25 | 798 | 5 | N/A |  | N/A |

6.68	DC_18_n3-n78

.

6.68.1	Operating bands for DC

Table 6.68.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n3A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.68.2	Channel bandwidths per operating band for DC

Table 6.68.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78 | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.68.3	Co-existence studies

Co-existence studies of Band 18 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.68.4	∆TIB and ∆RIB values

For DC_18A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n78 | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18_n3-n78 | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 6.68.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 & IMD5 products produced by Band 18 and n3 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.68.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n78A | 18 | IMD3 | ¦2*fB18+fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 15.2 |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n78 |  |  | 3610 | 10 | 50 | 3610 | 4.0 |

6.69	DC_3_n40-n79

.

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n40A-n79A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n79A | DC_3A_n40A DC_3A_n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.69.3	Co-existence studies

The co-existence studies of Band 3 + Band n40 and Band 3 + Band n79 have been captured in TR 37.716-01-01, where:

- IMD5 products produced by Band 3 and n40 that impact the reference sensitivity of NR band n79.

- IMD5 product produced by Band 3 and n79 that impact the reference sensitivity of NR Band n40.

    - 6.69.4	∆TIB and ∆RIB values

For DC_3_n40-n79, the TIB,c and RIB values are given in the tables below.

Table 6.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n40-n79 | 3 | 0.5 |
|  | n40 | 0.5 |
|  | n79 | 0 |

Table 6.69.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n40-n79 | 3 | 0 |
|  | n40 | 0 |
|  | n79 | 0 |

### 6.69.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n40 that impact the reference sensitivity of NR band n79.

In addition, there is IMD5 product produced by Band 3 and n79 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.69.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n79A | 3 | IMD5 | ¦4*fB3-fn40¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 |  |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 | 4.7 |
|  | 3 | IMD5 | ¦4*fB3-fn79¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 |  |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 | 3.2 |

6.70	DC_3_n41-n79

.

### 6.70.1	Operating bands for DC

Table 6.70.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n41A-n79A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.70.2	Channel bandwidths per operating band for DC

Table 6.70.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41A-n79A | DC_3A_n41A DC_3A_n79A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.70.3	Co-existence studies

The co-existence studies of Band 3 + Band n41 and Band 3 + Band n79 have been captured in TR 37.716-01-01, where:

-  Both IMD2 & IMD5 products produced by Band 3 and n41 that impact the reference sensitivity of NR band n79.

    - 6.70.4	∆TIB and ∆RIB values

For DC_3_n41-n79, the TIB,c and RIB values are given in the tables below.

Table 6.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n41-n79 | 3 | 0.5 |
|  | n41 | 0.33 |
|  |  | 0.84 |
|  | n79 | 0 |
| NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz |  |  |

Table 6.70.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n41-n79 | 3 | 0 |
|  | n41 | 03 |
|  |  | 0.54 |
|  | n79 | 0 |
| NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz |  |  |

### 6.70.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 3 and n41 that impact the reference sensitivity of NR band n79. The required MSD are shown in the following table.

Table 6.70.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41A-n79A | 3 | IMD2 | ¦fB3-fn41¦ | 1770 | 5 | 25 | 1865 | N/A |
|  | n41 |  |  | 2670 | 10 | 50 | 2670 |  |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 30.8 |
|  | 3 | IMD5 | ¦4*fB3-fn41¦ | 1770 | 5 | 25 | 1865 | N/A |
|  | n41 |  |  | 2550 | 10 | 50 | 2550 |  |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 5.2 |

6.71	DC_8_n40-n79

.

### 6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n40A-n79A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n79A | DC_8A_n40A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.71.3	Co-existence studies

The co-existence studies of Band 8 + Band n40 and Band 8 + Band n79 have been captured in TR 37.716-01-01, where:

- IMD4 products produced by Band 8 and n40 that impact the reference sensitivity of NR band n79.

- IMD4 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n40.

    - 6.71.4	∆TIB and ∆RIB values

For DC_8_n40-n79, the TIB,c and RIB values are given in the tables below.

Table 6.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n40-n79 | 8 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0 |

Table 6.71.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n40-n79 | 8 | 0 |
|  | n40 | 0 |
|  | n79 | 0 |

### 6.71.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 8 and n40 that impact the reference sensitivity of NR band n79.

In addition, there is IMD4 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.71.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n79A | 8 | IMD4 | ¦3*fB8+fn40¦ | 885 | 5 | 25 | 930 | N/A |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 |  |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 | 10.7 |
|  | 8 | IMD4 | ¦3*fB8-fn79¦ | 885 | 5 | 25 | 930 | N/A |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 |  |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 | 9.2 |

6.72	DC_8_n41-n79

.

### 6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n41A-n79A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n41A-n79A | DC_8A_n41A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.72.3	Co-existence studies

The co-existence studies of Band 8 + Band n41 and Band 8 + Band n79 have been captured in TR 37.716-01-01, where:

- IMD3 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n79.

- IMD3 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n41.

    - 6.72.4	∆TIB and ∆RIB values

For DC_8_n41-n79, the TIB,c and RIB values are given in the tables below.

Table 6.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n41-n79 | 8 | 0.3 |
|  | n41 | 0.3 |
|  | n79 | 0 |

Table 6.72.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n41-n79 | 8 | 0 |
|  | n41 | 0 |
|  | n79 | 0 |

### 6.72.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n79.

In addition, there is IMD3 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n41. The required MSD are shown in the following table.

Table 6.72.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n41A-n79A | 8 | IMD3 | ¦2*fB8+fn41¦ | 910 | 5 | 25 | 955 | N/A |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 |  |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 | 16.3 |
|  | 8 | IMD3 | ¦2*fB8-fn79¦ | 910 | 5 | 25 | 955 | N/A |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 |  |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 | 15.5 |

6.73	DC_39_n40-n79

### 6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39A_n40A-n79A | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.73.2	Channel bandwidths per operating band for DC

Table 6.73.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n40A-n79A | DC_39A_n40A DC_39A_n79A | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.73.3	Co-existence studies

The co-existence studies of Band 39 + Band n40 and Band 39 + Band n79 have been captured in TR 37.716-01-01, where:

- IMD4 products produced by Band 39 and n40 that impact the reference sensitivity of NR band n79.

    - 6.73.4	∆TIB and ∆RIB values

For DC_39_n40-n79, the TIB,c and RIB values are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39_n40-n79 | 39 | 0.3 |
|  | n40 | 0 |
|  | n79 | 0.8 |

Table 6.73.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39_n40-n79 | 39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |

### 6.73.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 39 and n40 that impact the reference sensitivity of NR band n79. The required MSD is shown in the following table.

Table 6.73.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 | 5 | 25 | 1917.5 | N/A |
|  | n40 |  |  | 2302.5 | 5 | 25 | 2302.5 |  |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 5.8 |

6.74	DC_7-7_n66-n78

[Editor Note]: DC operating bands,DL/UL configuration, coexistence and delta Tib/Rib will be updated.

### 6.74.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 7 and n66 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.74.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_7A-7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-7A_n66A-n78A | 7 | IMD3 | ¦2*fB7 -fn66¦ | 2555 | 5 | 25 | 2675 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 15.1 |

6.75	DC_3A_n3A-n77A

6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n3-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | UL Configuration |  | Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n3A-n77A | DC_3A_n3A DC_3A_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.75.3	UE co-existence studies

The harmonic and IMD products caused by DC_3A_n3A-n77A and protected bands follow from those derived for the constituent fallback modes, DC_3A_n3A, DC_3A_n77A, and CA_n3A-n77A. The co-existence studies of DC_3A_n3A and DC_3A_n77A are already captured in TR 37.863-01-01 [6], and the co-existence studies of CA_n3A-n77A are captured in TR 37.865-01-01. In rel-15, only Single Tx Switched UL mode is supported for DC_3A_n3A.

-	The 2nd harmonic interference from Band 3 or Band n3 UL will fall into Band n77 DL frequency range

-	Note that in Rel.15, only single tx switched UL mode is supported for DC_3A_n3A.

For dual uplink of Band 3 and Band n3 in rel-16, RAN4 need further study of self desense problem as below

-	The 2nd IMD and 4th IMD will be impact to Band n78 due to dual uplink transmission by DC_3A_n3A.

6.75.4	ΔTIB and ΔRIB values

For DC_3A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n3A-n77A | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n3A-n77A | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

6.75.5	MSD

IMD problems are already analysed in constituent low order combinations, additional analysis is not needed.

- 	Uplink DC configuration is DC_3A_n3A: Harmonic problem already discussed and solved in DC_3A_n77A, CA_n3A-n7A. Note that only Single Tx Switched UL mode is supported for DC_3A_n3A in Rel.15.

- 	Uplink DC configuration is DC _3A_n77A: Harmonic problem already discussed and solved in DC_3A_n77A, CA_n3A-n77A. IMD problem already discussed and solved in DC_3A_n77A.

However, RAN4 need further studies on the self desense problem as for UE with dual PA capability of DC_3A_n3A in Rel-16. Based on self desens analysis results, these IMD2 and IMD4 MSD for DC_3_n3_n77/n78 under non-contiguous DC_3_n3 is not needed based on MPR equation in TS 38.101-3.

- 	To add a note in the TS38.101-3, Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands):

Note X: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified.

6.76	DC_3A_n3A-n78A

6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.76.2	Channel bandwidths per operating band for DC

Table 6.76.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | UL Configuration |  | Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n3A-n78A | DC_3A_n3A DC_3A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.76.3	UE co-existence studies

The harmonic and IMD products caused by DC_3A_n3A-n78A and protected bands follow from those derived for the constituent fallback modes, DC_3A_n3A, DC_3A_n78A, and CA_n3A-n78A. The co-existence studies of DC_3A_n3A and DC_3A_n78A are already captured in TR 37.863-01-01 [6], and the co-existence studies of CA_n3A-n78A are captured in TR 37.865-01-01. In rel-15, only Single Tx Switched UL mode is supported for DC_3A_n3A.

-	The 2nd harmonic interference from Band 3 or Band n3 UL will fall into Band n78 DL frequency range

-	Note that in Rel.15, only single tx switched UL mode is supported for DC_3A_n3A.

For dual uplink of Band 3 and Band n3 in rel-16, RAN4 need further study of self desense problem as below

-	The 2nd IMD and 4th IMD will be impact to Band n78 due to dual uplink transmission by DC_3A_n3A.

6.76.4	ΔTIB and ΔRIB values

For DC_3A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A_n3A-n78A | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A_n3A-n78A | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

6.76.5	MSD

IMD problems are already analysed in constituent low order combinations, additional analysis is not needed.

- 	Uplink DC configuration is DC_3A_n3A: Harmonic problem already discussed and solved in DC_3A_n78A, CA_n3A-n78A. Note that only Single Tx Switched UL mode is supported for DC_3A_n3A in Rel.15.

- 	Uplink DC configuration is DC _3A_n78A: Harmonic problem already discussed and solved in DC_3A_n78A, CA_n3A-n78A. IMD problem already discussed and solved in DC_3A_n78A.

However, RAN4 need further studies on the self desense problem as for UE with dual PA capability of DC_3A_n3A in Rel-16. Based on self desense analysis results, these IMD2 and IMD4 MSD for DC_3_n3_n77/n78 under non-contiguous DC_3_n3 is not needed based on MPR equation in TS 38.101-3.

- 	To add a note in the TS38.101-3, Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands):

Note X: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified.

6.77	DC_3_n1-n28

6.77.1	Operating bands for DC

Table 6.77.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| NOTE: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | uplink configuration | E-UTRA and NR Band | scs [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 60 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_3C_n28A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.77.3	Co-existence studies

Table 6.77.3-1: Harmonic and IMD analysis for DC_3_n1-n28 with UL on DC_3_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 135 | 270 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1650 | 2055 | 2250 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5340 | 5550 | 5550 | 5745 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 3150 | 3435 | 3975 | 4230 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7050 | 7335 | 7470 | 7725 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 540 | 270 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6210 | 5895 | 5220 | 4860 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9390 | 9705 | 8760 | 9120 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2520 | 2190 | 1515 | 1170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9180 | 9510 | 8970 | 9315 |

Table 6.77.3-2: Harmonic and IMD analysis for DC_3_n1-n28 with UL on DC_3_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 733 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1466 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2199 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 977 | 2413 | 2518 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2687 | 2867 | 379 | 244 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4303 | 3116 | 3251 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 4397 | 4652 | 324 | 489 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6088 | 3819 | 3984 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1954 | 2164 | 4826 | 5036 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1222 | 1027 | 6437 | 6107 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4717 | 7543 | 7873 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1461 | 3949 | 3664 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5769 | 6536 | 6821 |

The coexistence studies of Band 3 + Band n1 and Band 3 + Band n28 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.1.3-1 and Table 6.1.3-2:

-	3rd harmonic of n28 uplink will interfere band 1 downlink but this is already covered in fallback cases

- 	4th order IMD of bands 3 and n28 will interfere n1

6.77.4	∆TIB and ∆RIB values

For DC_3A_n1A-n28, the TIB,c and RIB values are given in the tables below and are based on E-UTRA CA_1-3-28.

Table 6.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n28 | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |

Table 6.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n28 | 3 | 0 |
|  | n1 | 0 |
|  | n28 | 0.2 |

6.77.5	MSD

4th order IMD of bands 3 and n28 will interfere n1 and MSD level in DC_1A-3A_n28A can be reused.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |

6.78	DC_28_n3-n78

6.78.1	Operating bands for DC

Table 6.78.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n3A-n78A | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 2805 MHz | – | 2880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.78.2	Channel bandwidths per operating band for DC

Table 6.78.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n3A-n78A | DC_28A_n3A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.78.3	Co-existence studies

Co-existence studies of Band 28 + Band n3 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

6.78.4	∆TIB and ∆RIB values

For DC_28A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n3-n78 | 28 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.78.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n3-n78 | 28 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 6.78.5	MSD

As mentioned in Table 5.3-1 of TR 37.716-21-21 [2], there is no need to specify additional MSD requirement for this UL DC configuration.

6.79	DC_5_n7-n78

6.79.1	Operating bands for DC

Table 6.79.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n7-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.79.2	Channel bandwidths per operating band for DC

Table 6.79.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration |  | UL Configuration |  | NR Band |  | SCS [kHz] |  | 5 |  | 10 |  | 15 |  | 20 |  | 40 |  | 50 |  | 60 |  | 80 |  | 90 |  | 100 |  | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n7A-n78A |  | DC_5A_n7A DC_5A_n78A |  | 5 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| DC_5A_n7(2A)-n78A |  | DC_5A_n7A DC_5A_n78A |  | 5 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150 |  |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |
| DC_5A_n7A-n78(2A) |  | DC_5A_n7A DC_5A_n78A |  | 5 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |  |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A_n7(2A)-n78(2A) |  | DC_5A_n7A DC_5A_n78A |  | 5 |  | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 250 |  |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.79.3	Co-existence studies

Co-existence studies of DC_7A_n5A-n78A in Clause 6.44.3 can be reused for the case DC_5A_n7A UL to Band n78 DL. It shows that there is IMD2 impact from DC_5A_n7A UL to Band n78 DL.

Co-existence studies of DC_5A-7A_n78A in Clause 6.7.3 of TR 37.863-02-01 can be reused for the case DC_5A_n78A UL to Band n7 DL. It shows that there is IMD2 impact from DC_5A_n78A to Band n7 DL.

6.79.4	∆TIB and ∆RIB values

For DC_5_n7-n78, the TIB,c and RIB values are same as for DC_5-7_n78 and are given in the tables below.

Table 6.79.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n7-n78 | 5 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 6.79.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n7-n78 | 5 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

6.79.5	MSD

The MSD value for the case DC_5A_n7A UL to Band n78 DL is reused from DC_7A_n5A-n78A. And the MSD value for the case DC_5A_n78A UL to Band n7 DL is reused from DC_5A-7A_n78A.

Table 6.79.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n7A-n78A DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 ¦fn78-fb5¦ |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 ¦fB7 +fn5¦ |

6.80	DC_7_n7-n78

6.80.1	Operating bands for DC

Table 6.80.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n7-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.80.2	Channel bandwidths per operating band for DC

Table 6.80.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A_n7A-n78A | DC_7A_n78A DC_7A_n7A1 | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A1 | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | See CA_78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.80.3	Co-existence studies

Co-existence studies of DC_7A_n78A can be reused for the combination. There is no MSD issue for the combination.

6.80.4	∆TIB and ∆RIB values

For DC_7_n7-n78, the TIB,c and RIB values are given in the tables below.

Table 6.80.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n7-n78 | 7 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |

Table 6.80.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n7-n78 | 7 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

6.80.5	MSD

There is no MSD issue for the combination.

6.81	DC_66_n7-n78

6.81.1	Operating bands for DC

Table 6.81.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n7-n78 DC_66-66_n7-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.81.2	Channel bandwidths per operating band for DC

Table 6.81.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration |  | UL Configuration |  | NR Band |  | SCS [kHz] |  | 5 |  | 10 |  | 15 |  | 20 |  | 40 |  | 50 |  | 60 |  | 80 |  | 90 |  | 100 |  | Max BW [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n7A-n78A |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_66A-66A_n7A-n78A |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | See CA_66A-66A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_66A_n7(2A)-n78A |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_66A-66A_n7(2A)-n78A |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | See CA_66A-66A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 180 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_66A_n7A-n78(2A) |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 240 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n7A-n78(2A) |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | See CA_66A-66A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 260 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n7(2A)-n78(2A) |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 260 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A-66A_n7(2A)-n78(2A) |  | DC_66A_n7A DC_66A_n78A |  | 66 |  | See CA_66A-66A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 280 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.81.3	Co-existence studies

Co-existence studies of DC_7A-66A_n78A in Clause 5.1.28.4 of TR 37.716-21-11 can be reused for the case DC_66A_n78A UL to Band n7 DL. It shows that there is no MSD issue for the case.

Table 6.81.3-1 list Band 66 and Band n7 UL 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.81.3-1: Band 66 and Band n7 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12500 | 12850 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 720 | 860 | 4210 | 4350 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -850 | -1060 | 3220 | 3430 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5920 | 6130 | 6710 | 6920 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -2560 | -2840 | 5720 | 6000 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7630 | 7910 | 9210 | 9420 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1440 | 1720 | 8420 | 8700 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4270 | 4620 | 8220 | 8570 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9340 | 9690 | 11710 | 12060 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 10 | -340 | 3940 | 4290 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10920 | 11270 | 10130 | 10480 |

Based on Table 6.81.3-1, there is a MSD issue for the following case.

-	3rd order IMD may fall into Rx of bands n78

6.81.4	∆TIB and ∆RIB values

For DC_66_n7-n78, the ∆TIB,c and ∆RIB values are given in the tables below.

Table 6.81.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n7-n78 DC_66-66_n7-n78 | 66 | 0.6 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |

Table 6.81.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n7-n78 DC_66-66_n7-n78 | 66 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

6.81.5	MSD

Band 66 and Band 3 have similar UL frequency range, the MSD value for DC_3A_n7A-n78A can be reused for the case 3rd order IMD products of DC_66A_n7A UL fall into Band n78 DL

Table 6.81.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n7A-n78A DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |

6.82	DC_3_n1-n7

6.82.1	Operating bands for DC

Table 6.82.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n7 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

6.82.2	Channel bandwidths per operating band for DC

Table 6.82.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 90 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A | 3 | 15 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |

6.82.3	Co-existence studies

Co-existence analysis for DC_3_n1 shows that there is no impact from DC_3_n1 UL to Band n7 DL.

Co-existence analysis for DC_3_n7 shows that there is no impact from DC_3_n7 UL to Band n1 DL.

6.82.4	∆TIB and ∆RIB values

For DC_3_n1-n7 the TIB,c and RIB,c values are same as for DC_1-3_n7_and are given in the tables below.

Table 6.82.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n7 | 3 | 0.6 |
|  | n1 | 0.6 |
|  | n7 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.82.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n7 | 3 | 0 |
|  | n1 | 0 |
|  | n7 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

6.82.5	MSD

There is no need to define MSD.

6.83	DC_18_n78A-n257

6.83.1	Operating bands for DC

Table 6.83.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18_n78-n257 | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.83.2	Channel bandwidths per operating band for DC

Table 6.83.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_18A_n78A-n257A | DC_18A_n78A DC_18A_n257A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 215 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_18A_n78A-n257G | DC_18A_n78A DC_18A_n257A DC_18A_n257G | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 315 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A_n78A-n257H | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 415 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 515 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.83.3	Co-existence studies

Based on co-existence studies of DC_18A_n78A and DC_18A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.83.4	∆TIB and ∆RIB values

For DC_18_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n78-n257 | 18 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.83.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18A_n78-n257 | 18 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.83.5	MSD

As mentioned in 6.83.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.84	DC_1_n3A-n257

6.84.1	Operating bands for DC

Table 6.84.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.84.2	Channel bandwidths per operating band for DC

Table 6.84.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] |  | 5 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A_n3A-n257A | DC_1A_n3A DC_1A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A_n3A-n257G | DC_1A_n3A DC_1A_n257A DC_1A_n257G | 1 | 15 |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 250 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n3A-n257H | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | n3 | 15 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.84.3	Co-existence studies

Based on co-existence studies of DC_1A-n3A and DC_1A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.84.4	∆TIB and ∆RIB values

For DC_1_n3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n257 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 6.84.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n3-n257 | 1 | 0 |
|  | n3 | 0 |
|  | n257 | 0 |

6.84.5	MSD

As mentioned in 6.84.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.85	DC_18_n3A-n257

6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18_n3-n257 | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] |  | 5 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_18A_n3A-n257A | DC_18A_n3A DC_18A_n257A | 18 | 15 |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_18A_n3A-n257G | DC_18A_n3A DC_18A_n257A DC_18A_n257G | 18 | 15 |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 245 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A_n3A-n257H | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H | 18 | 15 |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 345 |  |
|  |  | n3 | 15 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | 18 | 15 |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 445 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.85.3	Co-existence studies

Based on co-existence studies of DC_18A-n3A and DC_18A_n257A specified in TR 37.716-11-11 and TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.85.4	∆TIB and ∆RIB values

For DC_18_n3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n257 | 18 | 0.3 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 6.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18_n3-n257 | 18 | 0 |
|  | n3 | 0 |
|  | n257 | 0 |

6.85.5	MSD

As mentioned in 6.85.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.86	DC_28_n3A-n257

6.86.1	Operating bands for DC

Table 6.86.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n3-n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 2805 MHz | – | 2880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.86.2	Channel bandwidths per operating band for DC

Table 6.86.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] |  | 5 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A_n3A-n257A | DC_28A_n3A DC_28A_n257A | 28 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_28A_n3A-n257G | DC_28A_n3A DC_28A_n257A DC_28A_n257G | 28 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n3A-n257H | DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 28 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | n3 | 15 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n3A-n257I | DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 28 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | n3 | 15 |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.86.3	Co-existence studies

Based on co-existence studies of DC_28A-n3A and DC_28A_n257A specified in TR 37.716-11-11 and TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.86.4	∆TIB and ∆RIB values

For DC_28_n3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n3-n257 | 28 | 0.3 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 6.86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n3-n257 | 28 | 0 |
|  | n3 | 0 |
|  | n257 | 0 |

6.86.5	MSD

As mentioned in 6.86.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.87	DC_41_n78A-n257

6.87.1	Operating bands for DC

Table 6.87.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n78-n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.87.2	Channel bandwidths per operating band for DC

Table 6.87.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_41A_n78A-n257A | DC_41A_n78A DC_41A_n257A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_41A_n78A-n257G | DC_41A_n78A DC_41A_n257A DC_41A_n257G | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n78A-n257H | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n78A-n257A | DC_41A_n78A DC_41C_n78A DC_41A_n257A DC_41C_n257A | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 240 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_41C_n78A-n257G | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41C_n78A DC_41C_n257A DC_41C_n257G | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n78A-n257H | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.87.3	Co-existence studies

Based on co-existence studies of DC_41A_n78A and DC_41A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.87.4	∆TIB and ∆RIB values

For DC_41_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n78-n257 | 41 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A_n78-n257 | 41 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.87.5	MSD

As mentioned in 6.87.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.88	DC_42_n78A-n257

6.88.1	Operating bands for DC

Table 6.88.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n78-n257 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.88.2	Channel bandwidths per operating band for DC

Table 6.88.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_42A_n78A-n257A | DC_42A_n257A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_42A_n78A-n257G | DC_42A_n257A DC_42A_n257G | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n78A-n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n78A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n78A-n257A | DC_42A_n257A DC_42C_n257A | 42 | See CA_n42C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 240 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_42C_n78A-n257G | DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 42 | See CA_n42C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n78A-n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 42 | See CA_n42C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n78A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 42 | See CA_n42C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.88.3	Co-existence studies

Based on co-existence studies of DC_42A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.88.4	∆TIB and ∆RIB values

For DC_42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n78-n257 | 42 | 0 |
|  | n78 | 0 |
|  | n257 | 0 |

Table 6.88.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42A_n78-n257 | 42 | 0 |
|  | n78 | 0 |
|  | n257 | 0 |

6.88.5	MSD

As mentioned in 6.88.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.89	DC_28_n78A-n257

6.89.1	Operating bands for DC

Table 6.89.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n78-n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

6.89.2	Channel bandwidths per operating band for DC

Table 6.89.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A_n78A-n257A | DC_28A_n78A DC_28A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_28A_n78A-n257G | DC_28A_n78A DC_28A_n257A DC_28A_n257G | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n78A-n257H | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.89.3	Co-existence studies

Based on co-existence studies of DC_28A_n78A and DC_28A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.89.4	∆TIB and ∆RIB values

For DC_28_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n78-n257 | 28 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 6.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28A_n78-n257 | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

6.89.5	MSD

As mentioned in 6.89.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.90	DC_66_n66-n78

### 6.90.1	Operating bands for DC

Table 6.90.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A_n66A-n78A | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.90.2	Channel bandwidths per operating band for DC

Table 6.90.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_66A_n66A-n78A |  | DC_66A_n66A1 DC_66A_n78A |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | NOTE1: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.90.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_66A_n66A-n78A with dual uplink DC_66A_n78A were calculated and listed in Table 6.90.3-1. There is no intermodulation product for DC_66A_n66A-n78A with dual uplink DC_66A_n66A, since DC_66A_n66A only supports single switched UL.

Table 6.90.3-1: Harmonic and IMD analysis for DC_66_n66-n78 with UL on DC_66_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 2090 | 5010 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 260 | 380 | 4820 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7360 | 8310 | 9380 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2040 | 8120 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9140 | 11610 | 13180 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3040 | 4180 | 10020 | 11160 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11420 | 13490 | 3040 | 3820 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16980 | 10140 | 10920 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6340 | 7980 | 1260 | 2470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14960 | 11730 | 12940 |

The coexistence studies of Band 66 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.90.3-1

-	5th order IMD generated by dual uplink of Band 66 + Band n78 may also fall into own Rx of band n66.

### 6.90.4	∆TIB and ∆RIB values

For DC_66A_n66A-n78A, the TIB,c and RIB,c values can reuse the values of DC_66A-66A_n78A as given in the tables below.

Table 6.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n78 | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.90.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n78 | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

### 6.90.5	MSD

Based on Table 6.90.3-1,  there is 5th order IMD generated by dual uplink of Band 66 + Band n78 that impact the reference sensitivity of NR band n66, and the required MSD is given as below table:

Table 6.90.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_66A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n66A-n78A | 66 | IMD5 | ¦ 2*fn78-3*fB66¦ | 1775 | 5 | 25 | 2175 | N/A |
|  | n66 |  |  | 1725 | 5 | 25 | 2125 | 2.8 |
|  | n78 |  |  | 3725 | 10 | 50 | 3725 | N/A |

6.91	DC_2_n7-n78

6.91.1	Operating bands for DC

Table 6.91.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n7-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.91.2	Channel bandwidths per operating band for DC

Table 6.91.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration |  | UL Configuration |  | NR Band |  | SCS [kHz] |  | 5 |  | 10 |  | 15 |  | 20 |  | 40 |  | 50 |  | 60 |  | 80 |  | 90 |  | 100 |  | Max BW [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n7A-n78A |  | DC_2A_n7A DC_2A_n78A |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_2A_n7(2A)-n78A |  | DC_2A_n7A DC_2A_n78A |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_2A_n7A-n78(2A) |  | DC_2A_n7A DC_2A_n78A |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 260 |  |
|  |  |  |  | n7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n7(2A)-n78(2A) |  | DC_12A_n7A DC_12A_n78A |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 260 |  |
|  |  |  |  | n7 |  | See CA_n7(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.91.3	Co-existence studies

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is a MSD issue for the following cases.

-	5th order IMD of DC_2A_n7 UL will fall into Rx of bands n78

6.91.4	∆TIB and ∆RIB values

For DC_2_n7-n78, the TIB,c and RIB values are given in the tables below.

Table 6.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n7-n78 | 2 | 0.6 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |

Table 6.91.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n7-n78 | 2 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

### 6.91.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 2 and n7 that impact the reference sensitivity of NR band n78.

The required MSD levels from interested companies are provided in Table 6.91.5-1. Based on the MSD levels, RAN4 will specify the MSD exception requirements as shown in the following Table 6.91.5-2.

Table 6.91.5-1: Proposed MSD levels from interested companies for EN-DC_2A_n7A-n78A

| DC bands | UL DC | IMD |  | HW (R4-200846) | LG (R4-2001130) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n7A-n78A DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | IMD5 | ¦2*fB2 -3*fn7¦ | 4.5 | 3.9 | 4.2 |
|  | n7 |  |  |  |  |  |
|  | n78 |  |  |  |  |  |

Table 6.91.5-2: MSD exception for Scell due to dual uplink operation for EN-DC_2A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n7A-n78A DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | IMD5 | ¦2*fB2 -3*fn7¦ | 1900 | 5 | 25 | 1980 | N/A |
|  | n7 |  |  | 2525 | 5 | 25 | 2645 | N/A |
|  | n78 |  |  | 3775 | 10 | 50 | 3775 | 4.2 |

6.92	DC_1A_n8A-n78A

6.92.1	Operating bands for DC

Table 6.92.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.92.2	Channel bandwidths per operating band for DC

Table 6.92.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n8A-n78A | DC_1A_n8A DC_1A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.92.3	Co-existence studies

Co-existence studies of Band 1 + Band n8 + Band n78 are already captured in Table 5.3-1 of TR 37.716-21-21 [2].

-	4th order harmonic generated by uplink of Band 8 may also fall into own Rx of band n78

-	3rd order IMD generated by dual uplink of Band 1 + Band n8 may also fall into own Rx of band n78

-	5th order IMD generated by dual uplink of Band 1 + Band n78 may also fall into own Rx of band n8

6.92.4	∆TIB and ∆RIB values

For DC_1A_n8A-n78A, the TIB,c and RIB values are given in the tables below.

Table 6.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n8-n78 | 1 | 0.3 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n8-n78 | 1 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.92.5	MSD

MSD values by dual uplink are reused from DC_1A-8A_n78A and DC_1A-8A_n77A for DC_1_n8-n78 band combination. So the below table is proposed to capture in TS38.101-3.

Table 6.92.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A |  | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | TDD | IMD3 ¦2*fn8+fB1¦ |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 3.3 |  | IMD5 ¦2*fn78-3*fB1¦ |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | TDD | N/A |

6.93	DC_13_n48-n66

6.93.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 13 and NR band n48 that impact the reference sensitivity of NR band n66. Also there is IMD5 produced by Band 13 and NR Band n66 that impact the reference sensitivity of NR band n48. These required MSD are shown in the following Table.

Table 6.93.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3680 | 5 | 25 | 3680 | N/A | N/A |
|  | n66 | 1716 | 5 | 25 | 2116 | 14.4 | IMD3 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |

6.94	DC_66_n5-n48

6.94.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 66 and NR band n5 that impact the reference sensitivity of NR band n48. The required MSD is shown in the following Table.

Table 6.94.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n48 | 3582 | 5 | 25 | 3582 | 3.3 | IMD5 |

6.95	DC_11_n3-n28

6.95.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 11 and NR band n3 that impact the reference sensitivity of NR band n28. The required MSD is shown in the following Table.

Table 6.95.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_11A_n3A-n28A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n3 | 1758 | 5 | 25 | 1853 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 2.5 | IMD5 |

6.96	DC_1_n8-n40

6.96.1	Operating bands for DC

Table 6.96.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n8A-n40A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.96.2	Channel bandwidths per operating band for DC

Table 6.96.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

6.96.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_1A_n8A-n40A with dual uplink on DC_1A_n8A and DC_1A_n40A were calculated and listed in Table 6.96.3-1 and 6.96.3-2 respectively.

Table 6.96.3-1: Harmonic and IMD analysis for DC_1_n8-n40 with UL on DC_1_n8

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1005 | 1100 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 220 | 90 | 2925 | 3080 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3680 | 3810 | 4720 | 4875 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 660 | 825 | 4845 | 5060 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4560 | 4725 | 6640 | 6855 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2200 | 2010 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7040 | 6765 | 1740 | 1540 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8560 | 8835 | 5440 | 5640 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4180 | 3930 | 1095 | 1320 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7520 | 7770 | 6480 | 6705 |

Table 6.96.3-2: Harmonic and IMD analysis for DC_1_n8-n40 with UL on DC_1_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 6900 | 7200 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 320 | 480 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1440 | 1660 | 2620 | 2880 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6140 | 6360 | 6520 | 6780 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 3640 | 4920 | 5280 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8060 | 8340 | 8820 | 9180 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 960 | 640 | 8440 | 8760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7680 | 7220 | 5620 | 5280 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11120 | 11580 | 9980 | 10320 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3360 | 2940 | 1340 | 960 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10740 | 11160 | 10360 | 10740 |

Based on Table 6.96.3-1 and Table 6.96.3-2:

-	4th order IMD generated by dual uplink of Band 1 + Band n8 may fall into own Rx of band 1

-	4nd order IMD generated by dual uplink of Band 1 + Band n40 may fall into own Rx of band n8

The first case has already been studied for DC_1A_n8A and the MSD has been defined in TS 38.101-3.

6.96.4	∆TIB and ∆RIB values

For DC_1A_n8A-n40A, the TIB,c and RIB values are given in the tables below.

Table 6.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n8-n40 | 1 | 0.3 |
|  | n8 | 0.3 |
|  | n40 | 0.5 |

Table 6.96.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n8-n40 | 1 | 0 |
|  | n8 | 0.2 |
|  | n40 | 0.5 |

6.96.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 & IMD5 produced by Band 1 and NR band n40 that impact the reference sensitivity of NR band n8. The required MSD is shown in the following Table. The MSD value in the table below is proposed to be captured in TS38.101-3.

Table 6.96.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 ¦2*fB1-2*fn40¦ |

6.97	DC_1_n28-n40

6.97.1	Operating bands for DC

Table 6.97.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n40 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.97.2	Configuration for DC

Table 6.97.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A | 1A | CA_n28A-n40A |

6.97.3	Co-existence studies

Co-existence analysis for DC_1_n28 shows that there is IMD4 impact from DC_1_n28 UL to NR Band n40 DL.

Co-existence analysis for DC_1_n40 shows that there is IMD4 impact from DC_1_n40 UL to NR Band n28 DL.

6.97.4	∆TIB and ∆RIB values

For DC_1_n28-n40 the TIB,c and RIB,c values are same as for CA_1-28-40 and are given in the tables below.

Table 6.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n40 | 1 | 0.6 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |

Table 6.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n28-n40 | 1 | 0 |
|  | n28 | 0.2 |
|  | n40 | 0 |

6.97.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 produced by Band 1 and NR band n28 that impact the reference sensitivity of NR band n40. Also there is IMD4 producted by Band 1 and NR band n40 that impact the reference sensitivity of NR band n28. These required MSD are shown in the following Table.

Table 6.97.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
|  | n28 | 713 | 5 | 25 | 768 | 8.6 | IMD4 |

6.98	DC_3_n1-n40

6.98.1	Operating bands for DC

Table 6.98.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.98.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A | 3A | CA_n1A-n40A |

6.98.3	Co-existence studies

Co-existence analysis for DC_3_n1 shows that there is IMD5 impact from DC_3_n1 UL to NR Band n40 DL.

Co-existence analysis for DC_3_n40 shows that there is no impact from DC_3_n40 UL to NR Band n1 DL.

6.98.4	∆TIB and ∆RIB values

For DC_3_n1-n40 the TIB,c and RIB,c values are same as for DC_3-40_n1 and are given in the tables below.

Table 6.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n40 | 1 | 0.5 |
|  | n28 | 0.5 |
|  | n40 | 0.5 |

Table 6.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n40 | 1 | 0 |
|  | n28 | 0 |
|  | n40 | 0 |

6.98.5	MSD

MSD values for impact from UL 3_n1 into DL n40 are reused from DC_3A-40A_n1A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n40A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |

6.99	DC_3_n8-n78

6.99.1	Operating bands for DC

Table 6.99.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n8-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.99.2	Configuration for DC

Table 6.99.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n8A-n78A | DC_3A_n8A DC_3A_n78A | 3A | CA_n8A-n78A |

6.99.3	Co-existence studies

Co-existence analysis for DC_3_n8 shows that there are IMD3 and IMD5 impact from DC_3_n8 UL to NR Band n78 DL.

Co-existence analysis for DC_3_n78 shows that there is no impact from DC_3_n78 UL to NR Band n8 DL.

6.99.4	∆TIB and ∆RIB values

For DC_3_n8-n78 the TIB,c and RIB,c values are same as for DC_3-8_n78 and are given in the tables below.

Table 6.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n8-n78 | 3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n8-n78 | 3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.99.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3&IMD5 produced by Band 3 and NR band n8 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following Table.

Table 6.99.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |

6.100	DC_7_n1-n40

6.100.1	Operating bands for DC

Table 6.100.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.100.2	Configuration for DC

Table 6.100.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A | 7A | CA_n1A-n40A |

6.100.3	Co-existence studies

Co-existence analysis for DC_7_n1 shows that there is no impact from DC_7_n1 UL to NR Band n40 DL.

Co-existence analysis for DC_7_n40 shows that there are IMD3 and IMD5 impact from DC_7_n40 UL to NR Band n1 DL.

6.100.4	∆TIB and ∆RIB values

For DC_7_n1-n40 the TIB,c and RIB,c values are reused from CA_1-7-40 and are given in the tables below.

Table 6.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n40 | n1 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |

Table 6.100.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n40 | n1 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |

6.100.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3&IMD5 produced by Band 7 and NR band n40 that impact the reference sensitivity of NR band n1. The required MSD is shown in the following Table.

Table 6.100.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 15.2 | IMD3 |

6.101	DC_7_n8-n78

6.101.1	Operating bands for DC

Table 6.101.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n8-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.101.2	Configuration for DC

Table 6.101.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n8A-n78A | DC_7A_n8A DC_7A_n78A | 7A | CA_n8A-n78A |

6.101.3	Co-existence studies

Co-existence analysis shows that there are IMD2 and IMD4 impact from DC_7_n8 UL to NR Band n78 DL.

Co-existence analysis shows that there is IMD2 and IMD5 impact from DC_7_n78 UL to NR Band n8 DL.

6.101.4	∆TIB and ∆RIB values

For DC_7_n8-n78 the TIB,c and RIB,c values are same as for DC_7-8_n78 and are given in the tables below.

Table 6.101.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n8-n78 | 7 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.101.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n8-n78 | 7 | 0 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

6.101.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2&IMD4 produced by Band 7 and NR band n8 that impact the reference sensitivity of NR band n78. Also ther are IMD2&IMD5 produced by Band 7 and NR band n78 that impact the reference sensitivity of NR band n8. These required MSDs are shown in the following Table.

Table 6.101.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 28.5 | IMD2 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 29.7 | IMD2 |

6.102	DC_13_n7-n78

6.102.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2&IMD4 produced by Band 13 and NR band n7 that impact the reference sensitivity of NR band n78. Also ther is IMD2 produced by Band 13 abd NR band n78 that impact the reference sensitivity of NR band n7. These required MSDs are shown in the following Table.

Table 6.102.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n78 | 3312 | 10 | 50 | 3312 | 29.0 | IMD2 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n78 | 3432 | 10 | 50 | 3432 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 27.9 | IMD2 |

6.103	DC_28_n7-n78

6.103.1	Operating bands for DC

Table 6.103.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n7-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.103.2	Channel bandwidths per operating band for DC

Table 6.103.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.103.3	Co-existence studies

Co-existence analysis for DC_28_n78 shows that there is IMD2 impact from DC_28_n78 UL to NR Band n7 DL.

Co-existence analysis for DC_28_n7 shows that there are IMD2 and IMD4 impacts from DC_28_n7 UL to NR Band n78 DL.

6.103.4	∆TIB and ∆RIB values

For DC_28_n7-n78 the TIB,c and RIB,c values are same as for DC_7-28_n78 and are given in the tables below.

Table 6.103.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n7-n78 | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |

Table 6.103.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n7-n78 | 28 | 0 |
|  | n7 | 0 |
|  | n78 | 0.5 |

6.103.5	MSD

MSD is needed to be defined in same way as been done for existing combinations DC_7A-28A_n78A and DC_7A_n28A-n78A, and MSD need to be added like table below in 38.101-3.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |

6.104	DC_2_n41-n66

6.104.1	Operating bands for DC

Table 6.104.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n41-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2210 MHz | – | 2200 MHz | FDD |

6.104.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n41A-n66A | DC_2A_n41A DC_2A_n66A | 2A | CA_n41A-n66A |
| DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A | 2A | CA_n41C-n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A | 2A | CA_n41(2A)-n66A |

6.104.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to NR Band n66 DL.

Co-existence analysis for DC_2_n66 shows that there is no impact from DC_2_n66 UL to NR Band n41 DL.

6.104.4	∆TIB and ∆RIB values

For DC_2_n41-n66 the TIB,c and RIB,c values are same as for CA_2-7-66 and are given in the tables below.

Table 6.104.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n41-n66 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |

Table 6.104.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n41-n66 | 2 | 0.3 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |

6.104.5	MSD

MSD is not needed to be defined.

6.105	DC_66_n25-n71

6.105.1	Operating bands for DC

Table 6.105.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n71 | 66 | 1710 MHz | – | 1780 MHz | 2210 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

6.105.2	Configuration for DC

Table 6.105.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A | 66A | CA_n25A-n71A |

6.105.3	Co-existence studies

Co-existence analysis for DC_66_n25 shows that there is no impact from DC_66_n25 UL to NR Band n71 DL.

Co-existence analysis for DC_66_n71 shows that there is no impact from DC_66_n71 UL to NR Band n25 DL.

6.105.4	∆TIB and ∆RIB values

For DC_66_n25-n71 the TIB,c and RIB,c values are same as for DC_2-66_n71 and are given in the tables below.

Table 6.105.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n71 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n71 | 0.3 |

Table 6.105.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n71 | 66 | 0.3 |
|  | n25 | 0.5 |
|  | n71 | 0 |

6.105.5	MSD

MSD is not needed to be defined.

6.106	DC_66_n12-n258

6.106.1	Operating bands for DC

Table 6.106.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n12-n258 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.106.2	Channel bandwidths per operating band for DC

Table 6.106.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n12A-n258A | DC_66A_n258A DC_66A_n12A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.106.3	Co-existence studies

Co-existence analysis is summarized in Table 5.3-1.

6.106.4	∆TIB and ∆RIB values

For DC_66_n12-n258 the TIB,c and RIB,c values are derived from DC_12_n66 combination.

Table 6.106.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n12-n258 | 66 | 0.5 |
|  | n12 | 0.8 |
|  | n258 | 0 |

Table 6.106.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n12-n258 | 66 | 0 |
|  | n12 | 0 |
|  | n258 | 0 |

6.106.5	MSD

There is no need to define MSD.

6.107	DC_66_n12-n260

6.107.1	Operating bands for DC

Table 6.107.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n12-n260 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.107.2	Channel bandwidths per operating band for DC

Table 6.107.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n12A-n260A | DC_66A_n260A DC_66A_n12A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.107.3	Co-existence studies

Co-existence analysis is summarized in Table 5.3-1.

6.107.4	∆TIB and ∆RIB values

For DC_66_n12-n260 the TIB,c and RIB,c values are derived from DC_12_n66 combination.

Table 6.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n12-n260 | 66 | 0.5 |
|  | n12 | 0.8 |
|  | n260 | 0 |

Table 6.107.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n12-n260 | 66 | 0 |
|  | n12 | 0 |
|  | n260 | 0 |

6.107.5	MSD

There is no need to define MSD.

6.108	DC_66_n12-n261

6.108.1	Operating bands for DC

Table 6.108.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n12-n261 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

6.108.2	Channel bandwidths per operating band for DC

Table 6.108.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n12A-n261A | DC_66A_n261A DC_66A_n12A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.108.3	Co-existence studies

Co-existence analysis is summarized in Table 5.3-1.

6.108.4	∆TIB and ∆RIB values

For DC_66_n12-n261 the TIB,c and RIB,c values are derived from DC_12_n66 combination.

Table 6.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n12-n261 | 66 | 0.5 |
|  | n12 | 0.8 |
|  | n261 | 0 |

Table 6.108.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n12-n261 | 66 | 0 |
|  | n12 | 0 |
|  | n261 | 0 |

6.108.5	MSD

There is no need to define MSD.

6.109	DC_28_n77-n257

6.109.1	Operating bands for DC

Table 6.109.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n77-n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.109.2	Channel bandwidths per operating band for DC

Table 6.109.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_28A_n77A-n257A | DC_28A_n77A DC_28A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_28A_n77A-n257D | DC_28A_n77A DC_28A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n77A-n257G | DC_28A_n77A DC_28A_n257A DC_28A_n257G | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n77A-n257H | DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A_n77A-n257I | DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

6.109.3	Co-existence studies

Based on the co-existence studies of DC_28A-n77A and DC_28A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.109.4	∆TIB and ∆RIB values

For DC_28_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n77-n257 | 28 | 0.5 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.109.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n77-n257 | 28 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.109.5	MSD

As mentioned in 6.109.3, there is no need to specify additional MSD requirement for this UL DC configuration..

6.110	DC_42_n77-n257

6.110.1	Operating bands for DC

Table 6.110.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n77-n257 | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.110.2	Channel bandwidths per operating band for DC

Table 6.110.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_42A_n77A-n257A | DC_42A_n257A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n77A-n257G | DC_42A_n257A DC_42A_n257G | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 320 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n77A-n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 420 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n77A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n77A-n257A | DC_42A_n257A | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n77A-n257G | DC_42A_n257A DC_42A_n257G | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n77A-n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n77A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.110.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

6.110.4	∆TIB and ∆RIB values

For DC_42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.110.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n77-n257 | 42 | 0 |
|  | n77 | 0 |
|  | n257 | 0 |

Table 6.110.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_42_n77-n257 | 42 | 0 |
|  | n77 | 0 |
|  | n257 | 0 |

6.110.5	MSD

As mentioned in 6.110.3, there is no need to specify additional MSD requirement for this DC configuration.

6.111	DC_42_n79-n257

6.111.1	Operating bands for DC

Table 6.111.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n79-n257 | 42 | 3400MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.111.2	Channel bandwidths per operating band for DC

Table 6.111.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_42A_n79A-n257A | DC_42A_n257A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n79A-n257G | DC_42A_n257A DC_42A_n257G | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 320 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n79A-n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 420 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42A_n79A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 520 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n79A-n257A | DC_42A_n257A | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n79A-n257G | DC_42A_n257A DC_42A_n257G | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n79A-n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n79A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.111.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

6.111.4	∆TIB and ∆RIB values

For DC_42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.111.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n79-n257 | 42 | 0 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 6.111.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_42_n79-n257 | 42 | 0 |
|  | n79 | 0 |
|  | n257 | 0 |

6.111.5	MSD

As mentioned in 6.111.3, there is no need to specify additional MSD requirement for this DC configuration.

6.112	DC_2_n12-n258

6.112.1	Operating bands for DC

Table 6.112.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n12-n258 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

6.112.2	Channel bandwidths per operating band for DC

Table 6.112.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n12A-n258A | DC_2A_n258A DC_2A_n12A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.112.3	Co-existence studies

Co-existence analysis for DC_2_n258 shows that there is no impact from DC_2_n258 UL to Band n12 DL.

Co-existence analysis for DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band n258 DL.

6.112.4	∆TIB and ∆RIB values

For DC_2_n12-n258 the TIB,c and RIB,c values are same as for DC_12_n2_and are given in the tables below.

Table 6.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n12-n258 | 2 | 0.3 |
|  | n12 | 0.3 |
|  | n258 | 0 |

Table 6.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n12-n258 | 2 | 0 |
|  | n12 | 0 |
|  | n258 | 0 |

6.112.5	MSD

There is no need to define MSD.

6.113	DC_2_n12-n260

6.113.1	Operating bands for DC

Table 6.113.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n12-n260 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

6.113.2	Channel bandwidths per operating band for DC

Table 6.113.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n12A-n260A | DC_2A_n260A DC_2A_n12A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.113.3	Co-existence studies

Co-existence analysis for DC_2_n260 shows that there is no impact from DC_2_n260 UL to Band n12 DL.

Co-existence analysis for DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band n260 DL.

6.113.4	∆TIB and ∆RIB values

For DC_2_n12-n260 the TIB,c and RIB,c values are same as for DC_12_n2_and are given in the tables below.

Table 6.113.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n12-n260 | 2 | 0.3 |
|  | n12 | 0.3 |
|  | n260 | 0 |

Table 6.113.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n12-n260 | 2 | 0 |
|  | n12 | 0 |
|  | n260 | 0 |

6.113.5	MSD

There is no need to define MSD.

6.114	DC_2_n12-n261

6.114.1	Operating bands for DC

Table 6.114.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n12-n261 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

6.114.2	Channel bandwidths per operating band for DC

Table 6.114.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n12A-n261A | DC_2A_n261A DC_2A_n12A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 435 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n12 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.114.3	Co-existence studies

Co-existence analysis for DC_2_n261 shows that there is no impact from DC_2_n261 UL to Band n12 DL.

Co-existence analysis for DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band n261 DL.

6.114.4	∆TIB and ∆RIB values

For DC_2_n12-n261 the TIB,c and RIB,c values are same as for DC_12_n2_and are given in the tables below.

Table 6.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n12-n261 | 2 | 0.3 |
|  | n12 | 0.3 |
|  | n261 | 0 |

Table 6.114.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n12-n261 | 2 | 0 |
|  | n12 | 0 |
|  | n261 | 0 |

6.114.5	MSD

There is no need to define MSD.

6.115	DC_2_n71-n261

6.115.1	Operating bands for DC

Table 6.115.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n71-n261 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

6.115.2	Channel bandwidths per operating band for DC

Table 6.115.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n71A-n261A | DC_2A_n261A DC_2A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 440 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_2A_n71A-n261(2A) | DC_2A_n261A DC_2A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 840 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | See CA_261(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

6.115.3	Co-existence studies

Co-existence analysis for DC_2_n261 shows that there is no impact from DC_2_n261 UL to Band n71 DL.

Co-existence analysis for DC_2_n71 shows that there is no impact from DC_2_n71 UL to Band n261 DL.

6.115.4	∆TIB and ∆RIB values

For DC_2_n71-n261 the TIB,c and RIB,c values are same as for DC_2_n71_and are given in the tables below.

Table 6.115.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n71-n261 | 2 | 0.3 |
|  | n71 | 0.3 |
|  | n261 | 0 |

Table 6.115.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n71-n261 | 2 | 0 |
|  | n71 | 0 |
|  | n261 | 0 |

6.115.5	MSD

There is no need to define MSD.

## 6.116	DC_18_n3-n77

### 6.116.1	Operating bands for DC

Table 6.116.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n3A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.116.2	Channel bandwidths per operating band for DC

Table 6.116.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.116.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_18A_n3A-n77A with dual uplink on DC_18A_n3A and DC_18A_n77A were calculated and listed in Table 6.116.3-1 and 6.116.3-2 respectively.

Table 6.116.3-1: Harmonic and IMD analysis for DC_18_n3-n77 with UL on DC_18_n3

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 880 | 970 | 2525 | 2615 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 50 | 155 | 2590 | 2755 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3340 | 3445 | 4235 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 660 | 780 | 4300 | 4540 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4155 | 4275 | 5945 | 6185 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1760 | 1940 | 5050 | 5230 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6010 | 6325 | 1475 | 1610 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7655 | 7970 | 4970 | 5105 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3470 | 3725 | 930 | 1125 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6760 | 7015 | 5865 | 6060 |

Table 6.116.3-2: Harmonic and IMD analysis for DC_18_n3-n77 with UL on DC_18_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 815 | 830 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1630 | 1660 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2445 | 2490 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2470 | 3385 | 4115 | 5030 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1640 | 2570 | 5770 | 7585 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4930 | 5860 | 7415 | 9230 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 810 | 1755 | 9070 | 11785 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5745 | 6690 | 10715 | 13430 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 4940 | 6770 | 8230 | 10060 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12370 | 15985 | 20 | 940 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14015 | 17630 | 6560 | 7520 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8240 | 10970 | 4110 | 5955 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11530 | 14260 | 9045 | 10890 |

The coexistence studies of Band18 + Band n3 and Band18 + Band n77 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.116.3-1 and Table 6.116.3-2:

- 3rd, 4th and 5th order IMD generated by dual uplink of Band 18 + Band n3 may also fall into own Rx of band n77

- 3rd order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band n3.

### 6.116.4	∆TIB and ∆RIB values

For DC_18A_n3A-n77A, the TIB,c and RIB,c values can reuse the values of DC_3A-18A_n77A as given in the tables below.

Table 6.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n77 | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.116.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n77 | 18 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 6.116.5	MSD

Table 6.116.5-1 shows the required MSD in the following table, comes from evaluation results.

Table 6.116.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n77A | 18 | IMD3 | ¦2*fB18 +fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 |  |
|  | n77 |  |  | 3410 | 10 | 50 | 3410 | 16.3 |
|  | 18 | IMD3 | ¦2*fB18 -fn77¦ | 820 | 5 | 25 | 865 | N/A |
|  | n77 |  |  | 3505 | 10 | 50 | 3505 |  |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 15.7 |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1725 | 5 | 25 | 1820 |  |
|  | n77 |  |  | 3535 | 10 | 50 | 3535 | 3.7 |

## 6.117	DC_28_n3-n77

### 6.117.1	Operating bands for DC

Table 6.117.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28A_n3A-n77A | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.117.2	Channel bandwidths per operating band for DC

Table 6.117.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_28A_n3A-n77A | DC_28A_n3A DC_28A_n77A | 28 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.117.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_28A_n3A-n77A with dual uplink on DC_28A_n3A and DC_28A_n77A were calculated and listed in Table 6.117.3-1 and 6.117.3-2 respectively.

Table 6.117.3-1: Harmonic and IMD analysis for DC_28_n3-n77 with UL on DC_28_n3

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 214 | 379 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6092 | 6437 | 1027 | 1282 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3634 | 3949 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

Table 6.117.3-2: Harmonic and IMD analysis for DC_28_n3-n77 with UL on DC_28_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3497 | 4003 | 4948 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1804 | 2794 | 5852 | 7697 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5696 | 7303 | 9148 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1056 | 2091 | 9152 | 11897 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6444 | 10603 | 13348 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5104 | 6994 | 8006 | 9896 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12452 | 16097 | 308 | 1388 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 17548 | 6112 | 7192 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 8404 | 11194 | 4356 | 6291 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 14096 | 8709 | 10644 |

The coexistence studies of Band28 + Band n3 and Band28 + Band n77 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.117.3-1 and Table 6.117.3-2:

- 3rd, 4th and 5th order IMD generated by dual uplink of Band 28 + Band n3 may also fall into own Rx of band n77.

- 3rd and 4th order IMD generated by dual uplink of Band 28 + Band n77 may also fall into own Rx of band n3.

### 6.117.4	∆TIB and ∆RIB values

For DC_28A_n3A-n77A, the TIB,c and RIB,c values can reuse the values of DC_3A-28A_n77A as given in the tables below.

Table 6.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n3-n77 | 28 | 0.5 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.117.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28_n3-n77 | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 6.117.5	MSD

Table 6.117.5-1 shows the evaluated MSD:

Table 6.117.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_28A_n3A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n3A-n77A | 28 | IMD3 | ¦2*fB28-fn77¦ | 735 | 5 | 25 | 790 | N/A |
|  | n3 |  |  | 1755 | 5 | 25 | 1850 | 17.01 |
|  | n77 |  |  | 3320 | 10 | 52 | 3320 | N/A |
|  | 28 | IMD3 | ¦fB28 +2*fn3¦ | 733 | 5 | 25 | 788 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 15.92 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified NOTE 2: This band is subject to IMD4 and IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.118	DC_41_n3-n77

### 6.118.1	Operating bands for DC

Table 6.118.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n3A-n77A DC_41C_n3A-n77A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.118.2	Channel bandwidths per operating band for DC

Table 6.118.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A | 41 |  | 15 | See CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.118.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n3A-n77A with dual uplink on DC_41A_n3A and DC_41A_n77A were calculated and listed in Table 6.118.3-1 and 6.118.3-2 respectively.

Table 6.118.3-1: Harmonic and IMD analysis for DC_41_n3-n77 with UL on DC_41_n3

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3207 | 3670 | 730 | 1074 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6702 | 7165 | 5916 | 6260 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5703 | 6360 | 2440 | 2859 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9198 | 9855 | 7626 | 8045 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4150 | 4644 | 8199 | 9050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9336 | 9830 | 11694 | 12545 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 250 | 363 | 3918 | 4650 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10122 | 10735 | 10908 | 11640 |

Table 6.118.3-2: Harmonic and IMD analysis for DC_41_n3-n77 with UL on DC_41_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1704 | 5796 | 6890 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 792 | 2080 | 3910 | 5904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9580 | 9096 | 11090 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3288 | 4770 | 7210 | 10104 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 12270 | 12396 | 15290 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1220 | 3408 | 11592 | 13780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510 | 14304 | 5784 | 7460 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 19490 | 13284 | 14960 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4520 | 7608 | 912 | 1470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 17980 | 14088 | 16470 |

The coexistence studies of Band41 + Band n3 and Band41 + Band n77 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.118.3-1 and Table 6.118.3-2:

- 3rd and 5th order IMD generated by dual uplink of Band 41 + Band n3 may also fall into own Rx of band n77.

- 3rd and 4th order IMD generated by dual uplink of Band 41 + Band n77 may also fall into own Rx of band n3.

### 6.118.4	∆TIB and ∆RIB values

For DC_41_n3-n77, the TIB,c and RIB,c values can reuse the values of DC_3A-41A_n77A as given in the tables below.

Table 6.118.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n77 | 41 | 0.31/082 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.118.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n77 | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.118.5	MSD

Table 6.118.5-1 shows the required MSD:

Table 6.118.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_41_n3-n77

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n3A-n77A DC_41C_n3A-n77A | 41 | IMD3 | ¦2*fB41-fn77¦ | 2620 | 5 | 25 | 2620 | N/A |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 16.4 |
|  | n77 |  |  | 3400 | 10 | 50 | 3400 | N/A |
|  | 41 | IMD4 | ¦2*fB41-2*fn77¦ | 2565 | 5 | 25 | 2565 | N/A |
|  | n3 |  |  | 1725 | 5 | 25 | 1820 | 8.3 |
|  | n77 |  |  | 3475 | 10 | 50 | 3475 | N/A |
|  | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 |  |
|  | n77 |  |  | 3440 | 10 | 50 | 3440 | 16.81 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.119	DC_41_n3-n78

### 6.119.1	Operating bands for DC

Table 6.119.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.119.2	Channel bandwidths per operating band for DC

Table 6.119.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A | 41 |  | 15 | See CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.119.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n3A-n78A with dual uplink on DC_41A_n3A and DC_41A_n78A were calculated and listed in Table 6.119.3-1 and 6.119.3-2 respectively.

Table 6.119.3-1: Harmonic and IMD analysis for DC_41_n3-n78 with UL on DC_41_n3

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 5130 | 5355 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3207 | 3670 | 730 | 1074 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6702 | 7165 | 5916 | 6260 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5703 | 6360 | 2440 | 2859 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9198 | 9855 | 7626 | 8045 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4150 | 4644 | 8199 | 9050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9336 | 9830 | 11694 | 12545 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 250 | 363 | 3918 | 4650 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10122 | 10735 | 10908 | 11640 |

Table 6.119.3-2: Harmonic and IMD analysis for DC_41_n3-n78 with UL on DC_41_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1304 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1220 | 2608 | 11592 | 12980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510 | 12704 | 6184 | 7460 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4520 | 6408 | 112 | 1470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

The coexistence studies of Band41 + Band n3 and Band41 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.119.3-1 and Table 6.119.3-2:

- 3rd order IMD generated by dual uplink of Band 41 + Band n3 may also fall into own Rx of band n78.

- 3rd and 4th order IMD generated by dual uplink of Band 41 + Band n78 may also fall into own Rx of band n3.

### 6.119.4	∆TIB and ∆RIB values

For DC_41_n3-n78, the TIB,c and RIB,c values can reuse the values of DC_3A-41A_n78A as given in the tables below.

Table 6.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n78 | 41 | 0.31/082 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.119.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n78 | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.119.5	MSD

Table 6.119.5-1 shows the required MSD:

Table 6.119.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_41_n3-n78

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | IMD3 | ¦2*fB41-fn78¦ | 2620 | 5 | 25 | 2620 | N/A |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 16.4 |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | N/A |
|  | 41 | IMD4 | ¦2*fB41-2*fn78¦ | 2565 | 5 | 25 | 2565 | N/A |
|  | n3 |  |  | 1725 | 5 | 25 | 1820 | 8.3 |
|  | n78 |  |  | 3475 | 10 | 50 | 3475 | N/A |
|  | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 |  |
|  | n78 |  |  | 3440 | 10 | 50 | 3440 | 16.8 |

## 6.120	DC_41_n28-n77

### 6.120.1	Operating bands for DC

Table 6.120.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n28A-n77A DC_41C_n28A-n77A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.120.2	Channel bandwidths per operating band for DC

Table 6.120.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n28A-n77A | DC_41A_n28A DC_41A_n77A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A | 41 |  | 15 | See CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.120.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n28A-n77A with dual uplink on DC_41A_n28A and DC_41A_n77A were calculated and listed in Table 6.120.3-1 and 6.120.3-2 respectively.

Table 6.120.3-1: Harmonic and IMD analysis for DC_41_n28-n77 with UL on DC_41_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1748 | 1987 | 3199 | 3438 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4244 | 4677 | 1000 | 1284 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5695 | 6128 | 3902 | 4186 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6740 | 7367 | 252 | 581 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8191 | 8818 | 4605 | 4934 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3496 | 3974 | 6398 | 6876 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 122 | 496 | 9236 | 10057 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5308 | 5682 | 10687 | 11508 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2748 | 3271 | 5992 | 6664 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7101 | 7624 | 8894 | 9566 |

Table 6.120.3-2: Harmonic and IMD analysis for DC_41_n28-n77 with UL on DC_41_n77

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 12600 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1704 | 5796 | 6890 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 792 | 2080 | 3910 | 5904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9580 | 9096 | 11090 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3288 | 4770 | 7210 | 10104 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 12270 | 12396 | 15290 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1220 | 3408 | 11592 | 13780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510 | 14304 | 5784 | 7460 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 19490 | 13284 | 14960 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4520 | 7608 | 912 | 1470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 17980 | 14088 | 16470 |

The coexistence studies of Band41 + Band n28 and Band41 + Band n77 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.120.3-1 and Table 6.120.3-2:

- 2nd, 3rd and 4th order IMD generated by dual uplink of Band 41 + Band n28 may also fall into own Rx of band n77.

- 2nd and 3rd order IMD generated by dual uplink of Band 41 + Band n77 may also fall into own Rx of band n28.

### 6.120.4	∆TIB and ∆RIB values

For DC_41_n28-n77, the TIB,c and RIB,c values can reuse the values of DC_28A-41A_n77A as given in the tables below.

Table 6.120.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n77 | 41 | 0.3 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.120.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n77 | 41 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 6.120.5	MSD

Table 6.120.5-1 shows the required MSD:

Table 6.120.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_41_n28-n77

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n28A-n77A DC_41C_n28A-n77A | 41 | IMD2 | ¦fB41+fn28¦ | 2580 | 5 | 25 | 2580 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | N/A |
|  | n77 |  |  | 3323 | 10 | 50 | 3323 | 28.21 |
|  | 41 | IMD2 | ¦fB41 -fn77¦ | 2642 | 5 | 25 | 2642 | N/A |
|  | n77 |  |  | 3440 | 10 | 50 | 3440 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 30.82 |
| NOTE 1: This band is subject to IMD3 and IMD4 also which MSD is not specified NOTE 2: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.121	DC_41_n28-n78

### 6.121.1	Operating bands for DC

Table 6.121.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.121.2	Channel bandwidths per operating band for DC

Table 6.121.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A | 41 |  | 15 | See CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.121.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n28A-n78A with dual uplink on DC_41A_n28A and DC_41A_n78A were calculated and listed in Table 6.121.3-1 and 6.121.3-2 respectively.

Table 6.121.3-1: Harmonic and IMD analysis for DC_41_n28-n78 with UL on DC_41_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1748 | 1987 | 3199 | 3438 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4244 | 4677 | 1000 | 1284 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5695 | 6128 | 3902 | 4186 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6740 | 7367 | 252 | 581 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8191 | 8818 | 4605 | 4934 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3496 | 3974 | 6398 | 6876 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 122 | 496 | 9236 | 10057 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5308 | 5682 | 10687 | 11508 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2748 | 3271 | 5992 | 6664 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7101 | 7624 | 8894 | 9566 |

Table 6.121.3-2: Harmonic and IMD analysis for DC_41_n28-n78 with UL on DC_41_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1304 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1220 | 2608 | 11592 | 12980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510 | 12704 | 6184 | 7460 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4520 | 6408 | 112 | 1470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

The coexistence studies of Band41 + Band n28 and Band41 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.121.3-1 and Table 6.121.3-2:

- 2nd and 4th order IMD generated by dual uplink of Band 41 + Band n28 may also fall into own Rx of band n78.

- 2nd and 5th order IMD generated by dual uplink of Band 41 + Band n78 may also fall into own Rx of band n28.

### 6.121.4	∆TIB and ∆RIB values

For DC_41_n28-n78, the TIB,c and RIB,c values can reuse the values of DC_28A-41A_n78A as given in the tables below.

Table 6.121.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n78 | 41 | 0.3 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.121.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n78 | 41 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 6.121.5	MSD

Table 6.121.5-1 shows the required MSD, come from evaluation results:

Table 6.121.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_41_n28-n78

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | IMD2 | ¦fB41 +fn28¦ | 2580 | 5 | 25 | 2580 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | N/A |
|  | n78 |  |  | 3323 | 10 | 50 | 3323 | 28.21 |
|  | 41 | IMD2 | ¦fB41 -fn78¦ | 2642 | 5 | 25 | 2642 | N/A |
|  | n78 |  |  | 3440 | 10 | 50 | 3440 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 30.82 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified NOTE 2: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

6.122	DC_7_n8-n40

6.122.1	Operating bands for DC

Table 6.122.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n8-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.122.2	Configuration for DC

Table 6.122.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A | 7A | CA_n8A-n40A |

6.122.3	Co-existence studies

Co-existence analysis for DC_7_n8 shows that there is IMD5 impact from DC_7_n8 UL to NR Band n40 DL.

Co-existence analysis for DC_7_n40 shows that there no impact from DC_7_n40 UL to NR Band n8 DL.

6.122.4	∆TIB and ∆RIB values

For DC_7_n8-n40 the TIB,c and RIB,c values are reused from CA_7-8-40 and are given in the tables below.

Table 6.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n8-n40 | 7 | 0.5 |
|  | n8 | 0.6 |
|  | n40 | 0.6 |

Table 6.122.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n8-n40 | 7 | 0 |
|  | n8 | 0.2 |
|  | n40 | 0.5 |

6.122.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 7 and NR band n8 that impact the reference sensitivity of NR band n40. The required MSD is shown in the following Table.

Table 6.122.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n8A-n40A | 7 | IMD5 | ¦2*fB7 -3*fn8¦ | 2530 | 5 | 25 | 2650 | 5 | N/A |
|  | n8 |  |  | 905 | 5 | 25 | 950 | 5 |  |
|  | n40 |  |  | 2345 | 5 | 25 | 2345 | 5 | 3.0 |

6.123	DC_28_n40-n78

6.123.1	Operating bands for DC

Table 6.123.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.123.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00 and TR 37.716-11-11. There are 3rd & 4th IMDs problems from dual uplink DC_28A_n40A which could impact own Rx band n78. Also 3rd IMD problem from dual uplink DC_28A_n78A could impact own Rx band n40. But CA_n40-n78 only allow one uplink transmission, then there are no self-interference problems

6.123.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 6.123.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n40-n78 | 28 | 0.5 |
|  | n40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 6.123.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n40-n78 | 28 | 0.2 |
|  | n40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

6.123.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD requirements since CA_n40-n78 only allow one uplink transmission

6.124	DC_3_n28-n77

6.124.1	Operating bands for DC

Table 6.124.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.124.2	Channel bandwidths per operating band for DC

Table 6.124.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A_n28A-n77A | DC_3A_n28A DC_3A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.124.3	Co-existence studies

Based on the co-existence studies of DC_3A-n28A and DC_3A-n77A captured in TR 37.863-01-01, 3rd, 4th and 5th order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n77 and 3rd order IMD generated by dual uplink of Band 3 + Band n77 may also fall into own Rx of band n28.

6.124.4	∆TIB and ∆RIB values

For DC_3_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.124.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n77 | 3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.124.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n28-n77 | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.124.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3, IMD4 & IMD5 produced by Band 3 and NR band n28 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 3 and NR band n77 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.124.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n28A-n77A | 3 | IMD3 | ¦2*fB3 +fn28¦ | 1720 | 5 | 25 | 1815 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 10 | 15.9 |
|  | 3 | IMD3 | ¦2*fB3 -fn77¦ | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 5 | 15.3 |

6.125	DC_8_n28-n77

6.125.1	Operating bands for DC

Table 6.125.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n28-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.125.2	Channel bandwidths per operating band for DC

Table 6.125.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n28A-n77A | DC_8A_n28A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_8A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.125.3	Co-existence studies

Based on the co-existence studies of DC_8A-n28A and DC_8A-n77A captured in TR 37.716-11-11 and 37.863-01-01, 4th and 5th order IMD generated by dual uplink of Band 8 + Band n28 may also fall into own Rx of band n77 and  4th order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n28.

6.125.4	∆TIB and ∆RIB values

For DC_8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n28-n77 | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.125.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n28-n77 | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.125.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 produced by Band 8 and NR band n28 that impact the reference sensitivity of NR band n77. Also there is IMD4 produced by Band 8 and NR band n77 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.125.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n28A-n77A | 8 | IMD4 | ¦3*fB8 +fn28¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3473 | 10 | 50 | 3473 | 10 | 10.3 |
|  | 8 | IMD4 | ¦3*fB8 -fn77¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n77 |  |  | 3495 | 10 | 50 | 3495 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.6 |

6.126	DC_42_n28-n77

6.126.1	Operating bands for DC

Table 6.126.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n28-n77 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.126.2	Channel bandwidths per operating band for DC

Table 6.126.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n28A-n77A | DC_42A_n28A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42A_n28A-n77(2A) | DC_42A_n28A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n28A-n77A | DC_42A_n28A DC_42C_n28A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_42C_n28A-n77(2A) | DC_42A_n28A DC_42C_n28A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.126.3	Co-existence studies

Based on the co-existence studies of DC_42A-n28A and DC_42A-n77A, 2nd order IMD generated by dual uplink of Band 42 + Band n28 may also fall into own Rx of band n77.

6.126.4	∆TIB and ∆RIB values

For DC_42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n28-n77 | 42 | 0.5 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 6.126.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n28-n77 | 42 | 0.2 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

6.126.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 42 and NR band n28 that impact the reference sensitivity of NR band n77. The required MSD si shown in the following Table.

Table 6.126.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n28A-n77A | 42 | IMD2 | ¦fB42 +fn28¦ | 3410 | 5 | 25 | 3410 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 |  |
|  | n77 |  |  | 4143 | 10 | 50 | 4143 | 10 | 29.2 |

6.127	DC_2_n38-n78

6.127.1	Operating bands for DC

Table 6.127.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n38-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.127.2	Channel bandwidths per operating band for DC

Table 6.127.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.127.3	Co-existence studies

Table 6.127.3-1 list Band 2 and Band n78 UL 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.127.3-1: Band 2 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 3800 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 7600 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 11400 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 15200 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 19000 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | -1950 | -1390 | 5150 | 5710 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 4690 | 5750 | -100 | 520 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8450 | 9510 | 7000 | 7620 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 7990 | 9550 | 1750 | 2430 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 11750 | 13310 | 8850 | 9030 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 3900 | 2780 | 10300 | 11420 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 11290 | 13350 | 3600 | 4340 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 6080 | 7700 | -2050 | -870 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 12150 | 13330 | 13600 | 15220 |

Based on Table 6.127.3-1, there is no MSD issue.

Table 6.127.3-2 list Band 2 and Band n38 UL 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.127.3-2: Band 2 and Band n38 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10280 | 10480 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12850 | 13100 | 9250 | 9550 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 770 | 660 | 4420 | 4530 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 3230 | 3390 | 1080 | 1250 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6990 | 7150 | 6270 | 6440 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 5800 | 6010 | 2930 | 3160 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 9560 | 9770 | 8120 | 8300 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1540 | 1320 | 8840 | 9060 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 8370 | 8630 | 4780 | 5070 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 12130 | 12390 | 9970 | 10260 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 3890 | 4160 | 310 | 590 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 10690 | 10970 | 11410 | 11680 |

Based on Table 6.127.3-2, there is an IMD3 issue produced by Band 2 and NR band n38 that impact the reference sensitivity of NR band n78.

6.127.4	∆TIB and ∆RIB values

For DC_2_n38-n78, the ∆TIB,c and ∆RIB values are given in the tables below.

Table 6.127.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n78 | 2 | 0.6 |
|  | n38 | 0.9 |
|  | n78 | 0.8 |

Table 6.127.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n38-n78 | 2 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

6.127.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 2 and NR band n38 that impact the reference sensitivity of NR band n78. The required MSD si shown in the following Table.

Table 6.127.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n38A-n78A | 2 | IMD3 | ¦fB2 -2*fn38¦ | 1870 | 5 | 25 | 1950 | 5 | N/A |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 |  |
|  | n78 |  |  | 3350 | 10 | 50 | 3350 | 10 | 14.8 |

6.128	DC_66_n38-n78

6.128.1	Operating bands for DC

Table 6.128.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n38-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.128.2	Channel bandwidths per operating band for DC

Table 6.128.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.128.3	Co-existence studies

Table 6.128.3-1 list Band 66 and Band n78 UL 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.128.3-1: Band 66 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 3800 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 7600 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 11400 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 15200 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 19000 | 8550 | 8900 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | -2090 | -1520 | 5010 | 5580 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | -4820 | -5890 | -380 | 260 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 8310 | 9380 | 6720 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | -8120 | -9690 | 1330 | 2040 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 11610 | 13180 | 8430 | 8640 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | -4180 | -3040 | 10020 | 11160 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 11420 | 13490 | 3040 | 3820 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 14910 | 16980 | 10140 | 10920 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | -6340 | -7980 | -2470 | -1260 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11730 | 12940 | 13320 | 14960 |

Table 6.128.3-1shows that there is no MSD issue for the case.

Table 6.128.3-2 list Band 66 and Band n38 UL 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.128.3-2: Band 66 and Band n38 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 12850 | 13100 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 790 | 910 | 4280 | 4400 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 800 | 990 | 3360 | 3530 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5990 | 6180 | 6850 | 7020 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 2510 | 2770 | 5930 | 6150 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 7700 | 7960 | 9420 | 9570 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 1580 | 1820 | 8560 | 8800 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 4220 | 4550 | 8500 | 8770 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 9410 | 9740 | 11990 | 12260 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 110 | 200 | 4150 | 4440 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11130 | 11420 | 10270 | 10580 |

Based on Table 6.128.3-1, there is a MSD issue for the following case.

-	3rd order IMD may fall into Rx of bands n78

6.128.4	∆TIB and ∆RIB values

For DC_66_n38-n78, the ∆TIB,c and ∆RIB values are given in the tables below.

Table 6.128.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n78 | 66 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 6.128.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n38-n78 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n78 | 0.5 |

6.128.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 66 and NR band n38 that impact the reference sensitivity of NR band n78. The required MSD si shown in the following Table.

Table 6.128.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n38A-n78A | 66 | IMD3 | ¦fB66 -2*fn38¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 |  |
|  | n78 |  |  | 3460 | 10 | 50 | 3460 | 10 | 15.0 |

6.129	DC_18_n3-n77

6.129.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 18 and NR band n3 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 18 and NR band n77 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.129.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n77A | 18 | IMD3 | ¦2*fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n77 |  |  | 3410 | 10 | 50 | 3410 | 10 | 16.3 |
|  | 18 | IMD3 | ¦2*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 3505 | 10 | 50 | 3505 | 10 |  |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 | 15.7 |

6.130	DC_28_n3-n77

6.130.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 28 and NR band n3 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 28 and NR band n77 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.130.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n3A-n77A | 28 | IMD3 | ¦fB28 +2*fn3¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 10 | 15.9 |
|  | 28 | IMD3 | ¦2*fB28 -fn77¦ | 735 | 5 | 25 | 790 | 5 | N/A |
|  | n77 |  |  | 3320 | 10 | 50 | 3320 | 10 |  |
|  | n3 |  |  | 1755 | 5 | 25 | 1850 | 5 | 17.0 |

6.131	DC_41_n3-n77

6.131.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 41 and NR band n3 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 41 and NR band n77 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.131.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n3A-n77A | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n77 |  |  | 3440 | 10 | 50 | 3440 | 10 | 16.8 |
|  | 41 | IMD3 | ¦2*fB41 -fn77¦ | 2620 | 5 | 25 | 2620 | 5 | N/A |
|  | n77 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 5 | 16.4 |

6.132	DC_41_n3-n78

6.132.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 41 and NR band n3 that impact the reference sensitivity of NR band n78. Also there is IMD3 produced by Band 41 and NR band n78 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.132.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n3A-n78A | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n78 |  |  | 3440 | 10 | 50 | 3440 | 10 | 16.8 |
|  | 41 | IMD3 | ¦2*fB41 -fn78¦ | 2620 | 5 | 25 | 2620 | 5 | N/A |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 5 | 16.4 |

6.133	DC_41_n28-n78

6.133.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 41 and NR band n28 that impact the reference sensitivity of NR band n78. Also there is IMD2 produced by Band 41 and NR band n78 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.133.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n28A-n78A | 41 | IMD2 ¦fB41 +fn28¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | N/A |
|  | n78 |  | 3323 | 10 | 50 | 3323 | 10 | 28.2 |
|  | 41 | IMD2 ¦fB41 -fn78¦ | 2642 | 5 | 25 | 2642 | 5 | N/A |
|  | n78 |  | 3440 | 10 | 50 | 3440 | 10 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | 30.8 |

6.134	DC_1_n75-n78

### 6.134.1	 Operating bands for DC

Table 6.134.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n75A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.134.2 	Configuration for DC

Table 6.134.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n75A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 170 |
|  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.134.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.134.3-1.

Table 6.134.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 1880 | 5220 | 5780 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

IMD2 and IMD5 may fall into Rx of band n75 with UL DC_1A_n78A.

### 6.134.4	 ∆TIB and ∆RIB values

Table 6.134.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n75-n78 | 1 | 0.5 |
|  | n75 | N/A |
|  | n78 | 0.8 |

Table 6.134.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n75-n78 | 1 | 0 |
|  | n75 | 0 |
|  | n78 | 0.5 |

6.134.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 1 and NR band n78 that impact the reference sensitivity of the S-DL NR band n75. The required MSD is shown in the following Table.

Table 6.134.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | IMD2 | ¦fB1 -fn78¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n75 |  |  | - | - | - | 1470 | 5 | 30.4 |

6.135	DC_3_n75-n78

### 6.135.1	 Operating bands for DC

Table 6.135.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.135.2 	Configuration for DC

Table 6.135.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n75A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 170 |
|  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A_n75A-n78(2A) | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 270 |
|  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.135.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.135.3-1.

Table 6.135.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 270 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3030 | 10020 | 11170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11415 | 3840 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6330 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

IMD2, IMD4 and IMD5 may fall into Rx of band n75 with UL DC_3A_n78A.

### 6.135.4	 ∆TIB and ∆RIB values

Table 6.135.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n75-n78 | 3 | 0.6 |
|  | n75 | N/A |
|  | n78 | 0.8 |

Table 6.135.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n75-n78 | 3 | 0.2 |
|  | n75 | 0 |
|  | n78 | 0.5 |

6.135.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 3 and NR band n78 that impact the reference sensitivity of the S-DL NR band n75 which was not directely impact to n75, but 2nd IMD sidelobe will be impacted. The required MSD is shown in the following Table. For IMD4 and IMD5 generated by UL DC_3A_n78A, the MSD exception can be referred to DC_3A-32A_n78A (approved TP R4-2003935) with UL DC_3A_n78A.

Table 6.135.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n75A-n78A | 3 | IMD2 | ¦fB3 -fn78¦ | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n75 |  |  | - | - | - | 1514.5 | 5 | 10.0 |
|  | 3 | IMD4 | ¦3*fB3 -fn78¦ | 1730 | 5 | 25 | 1825 | 5 | N/A |
|  | n78 |  |  | 3720 | 10 | 50 | 3720 | 10 |  |
|  | n75 |  |  | - | - | - | 1470 | 5 | 4.9 |

6.136	DC_1_n28-n77

6.136.1	Operating bands for DC

Table 6.136.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.136.2	Channel bandwidths per operating band for DC

Table 6.136.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n28A-n77A | DC_1A_n28A DC_1A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| C_1A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.136.3	Co-existence studies

Based on the co-existence studies of DC_1A_n28A and DC_1A_n77A captured in TR 37.863-01-01, 3rd and 4th order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band n77 and 5th order IMD generated by dual uplink of Band 1 + band n77 may also fall into own Rx of band n28.

6.136.4	∆TIB and ∆RIB values

For DC_1_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.136.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n77 | 1 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 6.136.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n28-n77 | 1 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

6.136.5	MSD

As mentioned in 6.136.3, IMD3 of B1 and n28 to Band n77 Rx and IMD5 of B1 and n77 to Band n28 Rx need to be addressed for REFSENS relaxation. Since the MSD requirements for DC_1A_n28A-n78A  has already been captured in TS 38.101-3, they will be reused for this band combination.

6.137	DC_7_n1-n257, DC_7-7_n1-n257

6.137.1	Operating bands for DC

Table 6.137.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n257 DC_7-7_n1-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.137.2	Channel bandwidths per operating band for DC

Table 6.137.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n1A-n257A | DC_7A_n1A DC_7A_n257A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 440 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_7A-7A_n1A-n257A | DC_7A_n1A DC_7A_n257A | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  | 460 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

6.137.3	Co-existence studies

Based on co-existence studies of DC_7A_n1A and DC_7A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.137.4	∆TIB and ∆RIB values

For DC_7_n1-n257, DC_7-7_n1-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.137.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n257 DC_7-7_n1-n257 | 7 | 0.6 |
|  | n1 | 0.5 |
|  | n257 | 0 |

Table 6.137.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n257 DC_7-7_n1-n257 | 7 | 0 |
|  | n1 | 0 |
|  | n257 | 0 |

6.137.5	MSD

No additional MSD requirement is needed.

6.138	DC_20_n78-n92

6.138.1	Operating bands for DC

Table 6.138.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n78-n92 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n92 | 832 MHz | – | 862 MHz | 1432 MHz | – | 1517 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.138.2	Channel bandwidths per operating band for DC

Table 6.138.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink Configuration |  | E-UTRA and NR Band |  | Subcarrier spacing (kHz) |  | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz |  | 100 MHz | Maximum aggregated bandwidth [MHz] |  |
| DC_20A_n78A-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |  | 20 |  | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |  |
|  |  |  | n92 |  | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |
| DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |  | 20 |  | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |  |
|  |  |  | n92 |  | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n78 |  |  | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.138.3	Co-existence studies

Co-existence study is the same as DC_20A-n78A and can refer to TR 37.863-01-01.

6.138.4	∆TIB and ∆RIB values

For DC_20_n78-n92, the TIB,c and RIB values are same as for DC_20_n78 defined in TR 37.863-01-01.

6.138.5	MSD

MSD is the same as DC_20A-n78A and can refer to TR 37.863-01-01. The conclusion is there is no MSD issue for this DC configuration.

## 6.139	DC_8_n40-n41

### 6.139.1	Operating bands for DC

Table 6.139.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n40A-n41A | 8 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.139.2	Channel bandwidths per operating band for DC

Table 6.139.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n41A | DC_8A_n40A DC_8A_n41A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.139.3	Co-existence studies

The co-existence studies of Band 8 + Band n40 and Band 8 + Band n41 have been captured in TR 37.716-01-01, where:

-   IMD5 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n40.

    - 6.139.4	∆TIB and ∆RIB values

For DC_8_n40-n41, the TIB,c and RIB values are given in the tables below.

Table 6.139.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n40-n41 | 8 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |

Table 6.139.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n40-n41 | 8 | 0 |
|  | n40 | 0 |
|  | n41 | 0 |

### 6.139.5	MSD

Based on Table 5.3-1 , considering CA_n40-n41 operation should synchronous operation. So No self desense problem due to IMD5. No self desense problem for this combination needs to be considered.

## 6.140	DC_39_n40-n41

### 6.140.1	Operating bands for DC

Table 6.140.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39A_n40A-n41A | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.140.2	Channel bandwidths per operating band for DC

Table 6.140.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n40A-n41A | DC_39A_n40A DC_39A_n41A | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.140.3	Co-existence studies

The co-existence studies of Band 39 + Band n40 and Band 39 + Band n41 have been captured in TR 37.716-01-01, where:

-   IMD3 products produced by Band 39 and n40 that impact the reference sensitivity of NR band n41.

    - 6.140.4	∆TIB and ∆RIB values

For DC_39_n40-n41, the TIB,c and RIB values are given in the tables below.

Table 6.140.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39_n40-n41 | 39 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |

Table 6.140.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39_n40-n41 | 39 | 0 |
|  | n40 | 0 |
|  | n41 | 0 |

### 6.140.5	MSD

Based on Table 5.3-1, considering CA_n40-n41 operation should synchronous operation, no self desense problem due to IMD3.No self desense problem for this combination needs to be considered.

6.141		DC_1_n41-n78

### 6.141.1	 Operating bands for DC

Table 6.141.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.141.2 	Configuration for DC

Table 6.141.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n41A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.141.3	 Co-existence studies

For UE coexistence study of Band 1 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.141.3-1.

Table 6.141.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 1880 | 5220 | 5780 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

For UE coexistence study of Band 1 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.141.3-1.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 | 1464 | 3012 | 3460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6650 | 6912 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3070 | 3444 | 5508 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8256 | 8630 | 9408 | 10050 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1540 | 1032 | 8832 | 9340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8840 | 8004 | 5424 | 4990 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4230 | 3528 | 948 | 380 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 | 12740 | 10176 | 10610 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 | 12030 | 10752 | 11320 |

IMD4 may fall into Rx of band n41 with UL DC_1A_n78A.

IMD3, IMD4 and IMD5 may fall into Rx of band n78 with UL DC_1A_n41A.

### 6.141.4	 ∆TIB and ∆RIB values

Table 6.141.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n41-n78 | 1 | 0.5 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 6.141.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n41-n78 | 1 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 6.141.5	REFSENS requirements

For IMD4 generated by UL DC_1A_n78A, the MSD exception can be referred to DC_1A-41A_n78A with UL DC_1A_n78A.

For IMD4 and IMD5 generated by UL DC_1A_n41A, the MSD exception can be referred to DC_1A_n7A-n78A with UL DC_1A_n7A.

Table 6.141.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n41A-n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2515 | 10 | 50 | 2515 | 11.5 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 19.6 | IMD3 ¦2*fn41-fB1¦ |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2520 | 10 | 25 | 2520 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 ¦fn41-3*fB1¦ |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 3.8 | IMD5 ¦3*fn41-2*fB1¦ |

6.142		DC_3_n41-n78

### 6.142.1	 Operating bands for DC

Table 6.142.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n41A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.142.2 	Configuration for DC

Table 6.142.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n41A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.142.3	 Co-existence studies

For UE coexistence study of Band 3 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.142.3-1.

Table 6.142.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1515 | 2090 | 5010 | 5585 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 270 | 4815 | 5890 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7370 | 8310 | 9385 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2055 | 8115 | 9690 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9155 | 11610 | 13185 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3030 | 10020 | 11170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11415 | 3840 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6330 | 1245 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16985 | 10140 | 10940 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14970 | 11730 | 12955 |

For UE coexistence study of Band 3 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.142.3-1.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1960 | 1422 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9050 | 8199 | 4644 | 4150 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4650 | 3918 | 363 | 250 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

IMD3 may fall into Rx of band n78 with UL DC_3A_n41A.

### 6.142.4	 ∆TIB and ∆RIB values

Table 6.142.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n41-n78 | 3 | 0.6 |
|  | n41 | 0.33 |
|  |  | 0.84 |
|  | n78 | 0.8 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 6.142.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n41-n78 | 3 | 0.2 |
|  | n41 | 03 |
|  |  | 0.54 |
|  | n78 | 0.5 |
| NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 – 2515 MHz. |  |  |

### 6.142.5	REFSENS requirements

For IMD3 generated by UL DC_3A_n41A, the MSD exception can be referred to DC_3A_n7A-n78A with UL DC_3A_n7A.

Table 6.142.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n41A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | IMD3 ¦2*fn41-fB3¦ |

6.143		DC_20_n41-n78

### 6.143.1	 Operating bands for DC

Table 6.143.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n41A-n78A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.143.2 	Configuration for DC

Table 6.143.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n41A-n78A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.143.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.143.3-1.

Table 6.143.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 2968 | 4132 | 4662 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 | 1576 | 5738 | 6768 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5524 | 7432 | 8462 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 | 714 | 9038 | 10568 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6386 | 10732 | 12262 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5936 | 4876 | 8264 | 9324 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 | 12338 | 148 | 472 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 | 8176 | 4014 | 5104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 16062 | 6628 | 7248 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 13124 | 9096 | 10186 |

For UE coexistence study of Band 20 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.143.3-1.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1634 | 1858 | 3328 | 3552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1026 | 772 | 4130 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4160 | 4414 | 5824 | 6242 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 194 | 90 | 6626 | 7238 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4992 | 5276 | 8320 | 8932 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3716 | 3268 | 6656 | 7104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9928 | 9122 | 952 | 638 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6406 | 5764 | 2406 | 2884 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10816 | 11622 | 5824 | 6138 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9152 | 9794 | 7488 | 7966 |

IMD2 may fall into Rx of band n41 with UL DC_20A_n78A.

IMD2 and IMD4 may fall into Rx of band n78 with UL DC_20A_n41A.

### 6.143.4	 ∆TIB and ∆RIB values

Table 6.143.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n41-n78 | 20 | 0.5 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 6.143.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n41-n78 | 20 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 6.143.5	REFSENS requirements

For IMD2 generated by UL DC_20A_n78A, the MSD exception can be referred to DC_7A-20A_n78A with UL DC_20A_n78A.

For IMD2 generated by UL DC_20A_n41A, the MSD exception can be referred to DC_7A_n28A-n78A with UL DC_7A_n28A.

Table 6.143.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n41A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n41 | 2675 | 10 | 50 | 2675 | 29.8 | IMD2 ¦fn78-fB20¦ |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD2y ¦fB20 +fn41¦ |
| NOTE y: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.144		DC_20_n75-n78

### 6.144.1	 Operating bands for DC

Table 6.144.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n75A-n78A DC_20A_n75A-n78(2A) | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.144.2 	Configuration for DC

Table 6.144.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n75A-n78A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 170 |
|  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes |  |
| DC_20A_n75A-n78(2A) | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 270 |
|  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.144.3	 Co-existence studies

For UE coexistence study of Band 20 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.144.3-1.

Table 6.144.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 2968 | 4132 | 4662 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 | 1576 | 5738 | 6768 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5524 | 7432 | 8462 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 | 714 | 9038 | 10568 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6386 | 10732 | 12262 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5936 | 4876 | 8264 | 9324 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 | 12338 | 148 | 472 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 | 8176 | 4014 | 5104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 16062 | 6628 | 7248 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 13124 | 9096 | 10186 |

Based on the table, there is no harmonic and IMD issue.

### 6.144.4	 ∆TIB and ∆RIB values

Table 6.144.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n75-n78 | 20 | 0.5 |
|  | n75 | N/A |
|  | n78 | 0.8 |

Table 6.144.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n75-n78 | 20 | 0 |
|  | n75 | 0 |
|  | n78 | 0.5 |

### 6.144.5	REFSENS requirements

There is no MSD requirements.

6.145	DC_3_n8-n40

6.145.1	Operating bands for DC

Table 6.145.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n8-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.145.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A | 3A | CA_n8A-n40A |

6.145.3	Co-existence studies

Co-existence analysis for DC_3_n8 shows that there is no impact from DC_3_n8 UL to NR Band n40 DL.

Co-existence analysis for DC_3_n40 shows that there is no impact from DC_3_n40 UL to NR Band n8 DL.

6.145.4	∆TIB and ∆RIB values

For DC_3_n8-n40 the TIB,c and RIB,c values are same as for CA_3-8-40 and are given in the tables below.

Table 6.145.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n8-n40 | 3 | 0.5 |
|  | n8 | 0.3 |
|  | n40 | 0.5 |

Table 6.145.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n8-n40 | 3 | 0 |
|  | n8 | 0 |
|  | n40 | 0 |

6.145.5	MSD

MSD is not needed to be defined.

6.146	DC_3_n28-n40

6.146.1	Operating bands for DC

Table 6.146.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n28-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.146.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A | 3A | CA_n28A-n40A |

6.146.3	Co-existence studies

Co-existence analysis for DC_3_n28 shows that there is no impact from DC_3_n28 UL to NR Band n40 DL.

Co-existence analysis for DC_3_n40 shows that there is no impact from DC_3_n40 UL to NR Band n28 DL.

6.146.4	∆TIB and ∆RIB values

For DC_3_n28-n40 the TIB,c and RIB,c values are same as for CA_3-28-40 and are given in the tables below.

Table 6.146.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n40 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |

Table 6.146.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n28-n40 | 3 | 0 |
|  | n28 | 0 |
|  | n40 | 0 |

6.146.5	MSD

MSD is not needed to be defined.

6.147	DC_7_n28-n40

6.147.1	Operating bands for DC

Table 6.147.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n28-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

6.147.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A | 7A | CA_n28A-n40A |

6.147.3	Co-existence studies

Co-existence analysis for DC_7_n28 shows that there is no impact from DC_7_n28 UL to NR Band n40 DL.

Co-existence analysis for DC_7_n40 shows that there is no impact from DC_7_n40 UL to NR Band n28 DL.

6.147.4	∆TIB and ∆RIB values

For DC_7_n28-n40 the TIB,c and RIB,c values are same as for CA_7-28-40 and are given in the tables below.

Table 6.147.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n28-n40 | 7 | 0.5 |
|  | n28 | 0.3 |
|  | n40 | 0.6 |

Table 6.147.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n28-n40 | 7 | 0 |
|  | n28 | 0 |
|  | n40 | 0.5 |

6.147.5	MSD

MSD is not needed to be defined.

## 6.148	DC_3_n40-n258

### 6.148.1	Operating bands for DC

Table 6.148.1-1: DC band combination of LTE 2DL/1UL + one NR band

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n40A-n258A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.148.2	Channel bandwidths per operating band for DC

Table 6.148.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + one NR band

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | Uplink configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n40A-n258A | DC_3A_n40A DC_3A_n258A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 500 |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |

### 6.148.3	Co-existence studies

Based on co-existence studies of DC_3A_n40A and DC_3A_n258A specified in TR 37.863-01-01 and TR 37.716-11-11, respectively. There is no additional harmonic and intermodulation impact for the additional band receiver.

    - 6.148.4	∆TIB and ∆RIB values

For DC_3_n40-n258, the TIB,c and RIB values are given in the tables below.

Table 6.148.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n40-n258 | 3 | 0.5 |
|  | n40 | 0.5 |
|  | n258 | 0 |

Table 6.148.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n40-n258 | 3 | 0 |
|  | n40 | 0 |
|  | n258 | 0 |

### 6.148.5	MSD

As mentioned in 6.148.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.149	DC_20_n1-n7

### 6.149.1	Operating bands for DC

Table 6.149.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n7 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

### 6.149.2	Channel bandwidths per operating band for DC

Table 6.149.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 90 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |

### 6.149.3	Co-existence studies

The harmonics and intermodulation products analysis for 2UL bands 20 and n1 with 3DL bands 20, n1 and n7 is presented in clause 6.149.3-1.

The harmonics and intermodulation products analysis for 2UL bands 20 and n7 with 3DL bands 20, n1 and n7 is presented in clause 6.149.3-2.

Table 6.149.3-1: Harmonic and IMD analysis for DC_20_n1-n7 with UL on DC_20_n1

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 832 | 862 | 1920 | 1980 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1664 | 1724 | 3840 | 3960 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2496 | 2586 | 5760 | 5940 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1058 | 1148 | 2752 | 2842 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -316 | -196 | 2978 | 3128 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3584 | 3704 | 4672 | 4822 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 516 | 666 | 4898 | 5108 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4416 | 4566 | 6592 | 6802 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -2296 | -2116 | 5504 | 5684 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -7088 | -6818 | -1528 | -1348 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8512 | 8782 | 5248 | 5428 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4276 | -4036 | 1254 | 1464 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7424 | 7664 | 6336 | 6546 |

Table 6.149.3-2: Harmonic and IMD analysis for DC_20_n1-n7 with UL on DC_20_n7

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 832 | 862 | 2500 | 2570 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1664 | 1724 | 5000 | 5140 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2496 | 2586 | 7500 | 7710 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1638 | 1738 | 3332 | 3432 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -906 | -776 | 4138 | 4308 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 4164 | 4294 | 5832 | 6002 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | -74 | 86 | 6638 | 6878 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4996 | 5156 | 8332 | 8572 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -3476 | -3276 | 6664 | 6864 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -9448 | -9138 | -948 | -758 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10832 | 11142 | 5828 | 6018 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -6046 | -5776 | 2414 | 2644 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 9164 | 9434 | 7496 | 7726 |

Based on the above studies, there is no harmonic or intermodulation issues to own receiver bands except for the cases that are covered already in 2UL/2DL EN-DC, i.e.,

- the 3rd order IMD of 20+n7 falls in band n7

- the 5th order IMD of 20+n7 falls in band n7.

### 6.149.4	∆TIB and ∆RIB values

For DC_20_n1-n7, the TIB,c and RIB values can be based on DC_7-20_n1given in the tables below, which is also the same as E-UTRA CA_1-7-20 relaxations.

Table 6.149.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n1-n7 | 20 | 0.3 |
|  | n1 | 0.5 |
|  | n7 | 0.6 |

Table 6.149.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n1-n7 | 20 | 0 |
|  | n1 | 0 |
|  | n7 | 0 |

### 6.149.5	MSD

All MSD issues are covered in 2DL/2UL fallback modes.

6.150		DC_(n)41-n78

### 6.150.1	 Operating bands for DC

Table 6.150.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.150.2 	Configuration for DC

Table 6.150.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)41AA-n78A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_(n)41CA-n78A | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_(n)41DA-n78A | 41 | See CA_41D Bandwidth combination set 0 at Table 5.6A.1-1 TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.150.3	 Co-existence studies

For UE coexistence study of Band 41 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.150.3-1.

Table 6.150.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984 | 10760 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12480 | 13450 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1304 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2608 | 1220 | 11592 | 12980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 | 10510 | 7460 | 6184 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 | 4520 | 1470 | 112 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

There was 4th IMD problem into Scell n41 by dual uplink DC_41A_n78A. However, B41 and n41 will be operated by synchronous operation, then no IMD problem into Scell n41.

### 6.150.4	 ∆TIB and ∆RIB values

Table 6.150.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_(n)41-n78 | 41 | 0.3 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 6.150.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_(n)41-n78 | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 6.150.5	REFSENS requirements

There is no the MSD requirements.

6.151	DC_1_n28-n257

6.151.1	Operating bands for DC

Table 6.151.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.151.2	Channel bandwidths per operating band for DC

Table 6.151.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n28A-n257A | DC_1A_n28A DC_1A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n28A-n257G | DC_1A_n28A DC_1A_n257A DC_1A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n28A-n257H | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.151.3	Co-existence studies

Based on the co-existence studies of DC_1A-n28A and DC_1A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.151.4	∆TIB and ∆RIB values

For DC_1_n28-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.151.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n257 | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n257 | 0 |

Table 6.151.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n257 | 1 | 0 |
|  | n28 | 0.2 |
|  | n257 | 0 |

6.151.5	MSD

As mentioned in 6.151.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.152	DC_3_n28-n257

6.152.1	Operating bands for DC

Table 6.152.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n28-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.152.2	Channel bandwidths per operating band for DC

Table 6.152.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n28A-n257A | DC_3A_n28A DC_3A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n28A-n257G | DC_3A_n28A DC_3A_n257A DC_3A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n28A-n257H | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A_n28A-n257I | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.152.3	Co-existence studies

Based on the co-existence studies of DC_3A-n28A and DC_3A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.152.4	∆TIB and ∆RIB values

For DC_3_n28-n257, the TIB,c and RIB,c values coming from the values for DC_3_n28 are given in the tables below.

Table 6.152.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n257 | 3 | 0.3 |
|  | n28 | 0.3 |
|  | n257 | 0 |

Table 6.152.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n257 | 3 | 0 |
|  | n28 | 0 |
|  | n257 | 0 |

6.152.5	MSD

As mentioned in 6.152.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.153	DC_41_n3-n257

6.153.1	Operating bands for DC

Table 6.153.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n3-n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.153.2	Channel bandwidths per operating band for DC

Table 6.153.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n3A-n257A | DC_41A_n3A DC_41A_n257A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |
|  |  | n3 | Please see BCS of CA_n3A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n3A-n257G | DC_41A_n3A DC_41A_n257A DC_41A_n257G | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 250 |
|  |  | n3 | Please see BCS of CA_n3A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n3A-n257H | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |
|  |  | n3 | Please see BCS of CA_n3A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |
|  |  | n3 | Please see BCS of CA_n3A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n3A-n257A | DC_41A_n3A DC_41A_n257A DC_41C_n3A DC_41C_n257A | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 470 |
|  |  | n3 | Please see BCS of CA_n3A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n3A-n257G | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41C_n3A DC_41C_n257A DC_41C_n257G | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | n3 | Please see BCS of CA_n3A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n3A-n257H | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 370 |
|  |  | n3 | Please see BCS of CA_n3A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 470 |
|  |  | n3 | Please see BCS of CA_n3A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.153.3	Co-existence studies

Based on the co-existence studies of DC_41A-n3A captured in TR 37.716-11-11 and DC_41A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.153.4	∆TIB and ∆RIB values

For DC_41_n3-n257, the TIB,c and RIB,c values coming from the values for DC_41_n3 are given in the tables below.

Table 6.153.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n257 | 41 | 0.3 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 6.153.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n257 | 41 | 0 |
|  | n3 | 0 |
|  | n257 | 0 |

6.153.5	MSD

As mentioned in 6.153.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.154	DC_41_n28-n257

6.154.1	Operating bands for DC

Table 6.154.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n28-n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.154.2	Channel bandwidths per operating band for DC

Table 6.154.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n28A-n257A | DC_41A_n28A DC_41A_n257A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n28A-n257G | DC_41A_n28A DC_41A_n257A DC_41A_n257G | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n28A-n257H | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n28A-n257I | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n28A-n257A | DC_41A_n28A DC_41A_n257A DC_41C_n28A DC_41C_n257A | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n28A-n257G | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41C_n28A DC_41C_n257A DC_41C_n257G | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n28A-n257H | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n28A-n257I | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.154.3	Co-existence studies

Based on the co-existence studies of DC_41A-n28A captured in TR 37.716-11-11 and DC_41A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.154.4	∆TIB and ∆RIB values

For DC_41_n28-n257, the TIB,c and RIB,c values coming from the values for DC_41_n28 are given in the tables below.

Table 6.154.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n257 | 41 | 0.3 |
|  | n28 | 0.3 |
|  | n257 | 0 |

Table 6.154.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n257 | 41 | 0 |
|  | n28 | 0 |
|  | n257 | 0 |

6.154.5	MSD

As mentioned in 6.154.3, there is no need to specify additional MSD requirement for this UL DC configuration.

6.155	DC_41_n77-n257

6.155.1	Operating bands for DC

Table 6.155.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n77-n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.155.2	Channel bandwidths per operating band for DC

Table 6.155.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n77A-n257A | DC_41A_n77A DC_41A_n257A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n77A-n257G | DC_41A_n77A DC_41A_n257A DC_41A_n257G | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 320 |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n77A-n257H | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 420 |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 520 |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n77A-n257A | DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n77A-n257G | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n77A-n257H | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  |  |  | n77 |  | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 41 | Please see BCS of CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  |  | n77 |  | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.155.3	Co-existence studies

Based on the co-existence studies of DC_41A-n77A and DC_41A-n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

6.155.4	∆TIB and ∆RIB values

For DC_41_n77-n257, the TIB,c and RIB,c values coming from the values for DC_41_n77 are given in the tables below.

Table 6.155.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n77-n257 | 41 | 0.3 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 6.155.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n77-n257 | 41 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

6.155.5	MSD

As mentioned in 6.155.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7	EN-DC band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

7.1	DC_1A-3A_n78A-n257

7.1.1	Operating bands for DC

Table 7.1.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.1.2	Channel bandwidths per operating band for DC

Table 7.1.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.1.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 3 + Band n78, Band 1 + n257 and Band 3 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th and 8th harmonics from n78 may fall into n257 and 2nd, 3rd and 5th order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n78

-	7th and 8th harmonics from n78 may fall into n257 and 5th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n78

-	2nd harmonic from Band 1 may fall into n78 and 3rd order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n257

-	2nd harmonic from Band 3 may fall into n78 and 3rd order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n257.

7.1.4	∆TIB and ∆RIB values

For DC_1A-3A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.1.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD issues on 3rd own Rx band will be covered in DC_1A-3A_n78A

-	Uplink DC configuration is DC _3A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD problems will be covered in DC_1A-3A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and solved in DC_1A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

7.2	DC_1A-5A_n78A-n257

7.2.1	Operating bands for DC

Table 7.2.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.2.2	Channel bandwidths per operating band for DC

Table 7.2.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-5A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 630 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 830 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_1A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.2.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 5 + Band n78, Band 1 + n257 and Band 5 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th and 8th harmonics from n78 may fall into n257 and 5th order IMD may fall into own Rx of Band 5 when dual uplink is configured with Band 1+ Band n78

-	7th and 8th harmonics from n78 may fall into n257 and 3rd order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 5 + Band n78

-	2nd harmonic from Band 1 may fall into n78 when dual uplink is configured with Band 1 + Band n257

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + Band 257

7.2.4	∆TIB and ∆RIB values

For DC_1A-5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A_n78-n257 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.2.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in 37.863-01-01 [6] and TR 37.863-02-01 respectively and fallback modes are captured in 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and 5th IMD problems is already covered in DC_1A-5A_n78A

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problems by 7th & 8th order between FR1 and FR2 and 3rd order IMD problems is already covered in DC_1A-5A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem is already discussed and is solved in DC_1A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.3	DC_1A-7A_n78A-n257

7.3.1	Operating bands for DC

Table 7.3.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.3.2	Channel bandwidths per operating band for DC

Table 7.3.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-7A_n78A-n257D | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257E | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257F | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257G | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257H | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257J | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257K | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257L | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A_n78A-n257M | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 7 + Band n78, Band 1 + n257 and Band 7 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th & 8th harmonics from n78 may fall into n257 and 4th order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 1 + n78

-	7th harmonic from Band 7 may fall into n257 and 4th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 7 + n78

-	2nd harmonic from Band 1 may fall into n78 when dual uplink is configured with Band 1 + n257

7.3.4	∆TIB and ∆RIB values

For DC_1A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n78-n257 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A_n78-n257 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.3.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and is solved in DC_1A_n78A

7.4	DC_1A-7A-7A_n78A-n257

7.4.1	Operating bands for DC

Table 7.4.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.4.2	Channel bandwidths per operating band for DC

Table 7.4.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-7A-7A_n78A-n257D | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257E | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257F | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257G | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257H | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257I | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257J | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 660 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257K | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257L | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 860 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-7A-7A_n78A-n257M | DC_1A_n78A DC_7A_n78A DC_1A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.4.3	Co-existence studies

Co-existence studies of Band 1 + Band n78, Band 7 + Band n78, Band 1 + n257 and Band 7 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th & 8th harmonics from n78 may fall into n257 and 4th order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 1 + n78

-	7th harmonic from Band 7 may fall into n257 and 4th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 7 + n78

-	2nd harmonic from Band 1 may fall into n78 when dual uplink is configured with Band 1 + n257

7.4.4	∆TIB and ∆RIB values

For DC_1A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-7A_n78-n257 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A-7A_n78-n257 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.4.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and 4th IMD problem is already covered in DC_1A-7A_n78A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and is solved in DC_1A_n78A

7.5	DC_3A-5A_n78A-n257

7.5.1	Operating bands for DC

Table 7.5.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.5.2	Channel bandwidths per operating band for DC

Table 7.5.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-5A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 630 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 730 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 830 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_3A_n257A DC_5A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 930 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.5.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 5 + Band n78, Band 3 + n257 and Band 5 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th & 8th harmonics from n78 may fall into n257 when dual uplink is configured with Band 3 + n78

-	7th & 8th harmonics from n78 may fall into n257 and 3rd order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 5A + n78

-	2nd harmonic from Band 3 may fall into n78 when dual uplink is configured with Band 3 + n257

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + n257

7.5.4	∆TIB and ∆RIB values

For DC_3A-5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.5.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_3A_n78A: Up to 7th harmonic order will be not any impact to n257

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problems by 7th & 8th order between FR1 and FR2 and 3rd IMD problem is covered in DC_3A-5A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.6	DC_3A-7A_n78A-n257

7.6.1	Operating bands for DC

Table 7.6.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.6.2	Channel bandwidths per operating band for DC

Table 7.6.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-7A_n78A-n257D | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257E | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257F | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257G | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257H | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257J | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 640 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257K | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 740 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257L | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 840 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A_n78A-n257M | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 940 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.6.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 7 + Band n78, Band 3 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 when dual uplink is configured with Band 3 + Band n78

-	7th harmonic from Band 7 may fall into n257 and 3rd & 4th IMDs may fall into Band 3 when dual uplink is configured with Band 7 + Band n78

-	2nd harmonic from Band 3 may fall into n78 when dual uplink is configured with Band 3 + Band n257

7.6.4	∆TIB and ∆RIB values

For DC_3A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.6.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01  and TR 37.863-02-01  respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_3A_n78A: Up to 7th harmonic order will be not any impact to n257

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not impact to n257 and IMD problems are already covered in DC_3A-7A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

7.7	DC_3A-7A-7A_n78A-n257

7.7.1	Operating bands for DC

Table 7.7.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.7.2	Channel bandwidths per operating band for DC

Table 7.7.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-7A-7A_n78A-n257D | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257E | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257F | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257G | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257H | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257I | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257J | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 660 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257K | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257L | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 860 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-7A-7A_n78A-n257M | DC_3A_n78A DC_7A_n78A DC_3A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.7.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 7 + Band n78, Band 3 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 when dual uplink is configured with Band 3 + Band n78

-	7th harmonic from Band 7 may fall into n257 and 3rd & 4th IMDs may fall into Band 3 when dual uplink is configured with Band 7 + Band n78

-	2nd harmonic from Band 3 may fall into n78 when dual uplink is configured with Band 3 + Band n257

7.7.4	∆TIB and ∆RIB values

For DC_3A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-7A_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-7A_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.7.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_3A_n78A: Up to 7th harmonic order will be not any impact to n257

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not impact to n257 and IMD problems are already covered in DC_3A-7A_n78A

-	Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n78A

7.8	DC_5A-7A_n78A-n257

7.8.1	Operating bands for DC

Table 7.8.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A_n78-n257 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.8.2	Channel bandwidths per operating band for DC

Table 7.8.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-7A_n78A-n257D | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257E | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 730 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257F | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 930 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257G | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257H | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257I | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257J | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 630 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257K | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 730 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257L | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 830 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A_n78A-n257M | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 930 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.8.3	Co-existence studies

Co-existence studies of Band 5 + Band n78, Band 7 + Band n78, Band 5 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 and 2nd order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 5 + Band n78

-	7th harmonic from Band 7 may fall into Band n257 and 2nd order and 5th order IMD may fall into own Rx of Band 5 when dual uplink is configured with Band 7 + Band n78

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + Band n257

7.8.4	∆TIB and ∆RIB values

For DC_5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A_n78-n257 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-7A_n78-n257 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.8.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problem by 7th & 8th order between FR1 and FR2 and 2nd IMD problem is already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD problems are already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.9	DC_5A-7A-7A_n78A-n257

7.9.1	Operating bands for DC

Table 7.9.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5A-7A-7A_n78-n257 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.9.2	Channel bandwidths per operating band for DC

Table 7.9.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_5A-7A-7A_n78A-n257D | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257E | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257F | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257G | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257H | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257I | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257J | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 650 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257K | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257L | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 850 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_5A-7A-7A_n78A-n257M | DC_5A_n78A DC_7A_n78A DC_5A_n257A DC_7A_n257A | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.9.3	Co-existence studies

Co-existence studies of Band 5 + Band n78, Band 7 + Band n78, Band 5 + n257 and Band 7 + n257 are captured in TR 37.863-01-01

-	7th & 8th harmonics from n78 may fall into n257 and 2nd order IMD may fall into own Rx of Band 7 when dual uplink is configured with Band 5 + Band n78

-	7th harmonic from Band 7 may fall into Band n257 and 2nd order and 5th order IMD may fall into own Rx of Band 5 when dual uplink is configured with Band 7 + Band n78

-	4th harmonic from Band 5 may fall into n78 when dual uplink is configured with Band 5 + Band n257

7.9.4	∆TIB and ∆RIB values

For DC_5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5A-7A-7A_n78-n257 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5A-7A-7A_n78-n257 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.9.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_5A_n78A: There is no harmonic problem by 7th & 8th order between FR1 and FR2 and 2nd IMD problem is already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_7A_n78A: Up to 7th harmonic order will be not any impact to n257 and IMD problems are already covered in DC_5A-7A_n78A

-	Uplink DC configuration is DC_5A_n257A: Harmonic problem is already solved in DC_5A_n78A

7.10	DC_1A-3C_n28A-n78A

7.10.1	Operating bands for DC

Table 7.10.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3C_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.10.2	Channel bandwidths per operating band for DC

Table 7.10.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.10.3	Co-existence studies

The harmonic and IMD products caused by DC_1A-3C_n28A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.10.4	∆TIB and ∆RIB values

For DC_1A-3C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.10.5	MSD

All MSD requirements for DC_1A-3C_n28A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.11	DC_1A-7C_n28A-n78A

7.11.1	Operating bands for DC

Table 7.11.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7C_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.11.2	Channel bandwidths per operating band for DC

Table 7.11.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.11.3	Co-existence studies

The harmonic and IMD products caused by DC_1A-7C_n28A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.11.4	∆TIB and ∆RIB values

For DC_1A-7C_n28A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.11.5	MSD

All MSD requirements for DC_1A-7C_n28A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.12	DC_1-3_n7-n78

7.12.1	Operating bands for DC

Table 7.12.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.12.2	Channel bandwidths per operating band for DC

Table 7.12.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 260 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 280 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.12.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3_n7-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.12.4	∆TIB and ∆RIB values

For DC_1-3_n7-n78, the TIB,c and RIB values are given in the tables below.

Table 7.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

Table 7.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n7-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |

7.12.5	MSD

All MSD requirements for DC_1-3_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.13	DC_3-7_n28-n78

7.13.1	Operating bands for DC

Table 7.13.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n28-n78 | 3 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.13.2	Channel bandwidths per operating band for DC

Table 7.13.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_3C_n28A DC_3C_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_3C_n28A DC_3C_n78A DC_7C_n28A DC_7C_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.13.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7_n28-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

7.13.4	∆TIB and ∆RIB values

For DC_3-7_n28-n78, the TIB,c and RIB values are given in the tables below.

Table 7.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

7.13.5	MSD

All MSD requirements for DC_3-7_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.14	DC_1-8_n77-n257

7.14.1	Operating bands for DC

Table 7.14.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.14.2	Channel bandwidths per operating band for DC

Table 7.14.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_1A-8A_n77A-n257D | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

7.14.3	Co-existence studies

Based on the co-existence studies of DC_1A_n77A, DC_1A_n257A, DC_8A_n77A and DC_8A_n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.14.4	∆TIB and ∆RIB values

For DC_1A-8A_n77A-n257A, the TIB,c and RIB,c values are given in the tables below.

Table 7.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n257 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.14.5	MSD

As mentioned in 7.14.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.15	DC_1-19_n77-n79

7.15.1	Operating bands for DC

Table 7.15.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.15.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.15.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.15.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.15.4	∆TIB and ∆RIB values

For DC_1-19_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |

Table 7.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.5 |

7.15.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.16	DC_1-19_n78-n79

7.16.1	Operating bands for DC

Table 7.16.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.16.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.16.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.16.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.16.4	∆TIB and ∆RIB values

For DC_1-19_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |

Table 7.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.5 |

7.16.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.17	DC_1-21_n77-n79

7.17.1	Operating bands for DC

Table 7.17.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.17.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.17.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.17.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.17.4	∆TIB and ∆RIB values

For DC_1-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n77-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |

Table 7.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n77-n79 | n77 | 0.5 |

7.17.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.18	DC_1-21_n78-n79

7.18.1	Operating bands for DC

Table 7.18.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.18.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.18.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.18.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.18.4	∆TIB and ∆RIB values

For DC_1-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n78-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n78 | 0.8 |

Table 7.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n78-n79 | n78 | 0.5 |

7.18.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.19	DC_1-3_n77-n79

7.19.1	Operating bands for DC

Table 7.19.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.19.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.19.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n77A-n79A | DC_1A_n77A DC_3A_n77A DC_1A_n79A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.19.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.19.4	∆TIB and ∆RIB values

For DC_1-3_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n77-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |

Table 7.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n77-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |

7.19.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.20	DC_1-3_n78-n79

7.20.1	Operating bands for DC

Table 7.20.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.20.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.20.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n78A-n79A | DC_1A_n78A DC_3A_n78A DC_1A_n79A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.20.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.20.4	∆TIB and ∆RIB values

For DC_1-3_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n78-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |

Table 7.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n78-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |

7.20.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.21	DC_1-42_n77-n79

7.21.1	Operating bands for DC

Table 7.21.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.21.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.21.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-42A_n77A-n79A DC_1A-42C_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.21.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.21.4	∆TIB and ∆RIB values

For DC_1-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n77-n79 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

7.21.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.22	DC_1-42_n78-n79

7.22.1	Operating bands for DC

Table 7.22.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.22.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.22.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-42A_n78A-n79A DC_1A-42C_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.22.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.22.4	∆TIB and ∆RIB values

For DC_1-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n78-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n78-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

7.22.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.23	DC_19-21_n77-n79

7.23.1	Operating bands for DC

Table 7.23.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n77-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.23.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.23.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.23.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.23.4	∆TIB and ∆RIB values

For DC_19-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n77-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |

Table 7.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n77-n79 | n77 | 0.5 |

7.23.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.24	DC_19-21_n78-n79

7.24.1	Operating bands for DC

Table 7.24.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n78-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.24.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.24.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.24.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.24.4	∆TIB and ∆RIB values

For DC_19-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n78-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |

Table 7.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n78-n79 | n78 | 0.5 |

7.24.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.25	DC_19-42_n77-n79

7.25.1	Operating bands for DC

Table 7.25.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n77-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.25.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.25.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-42A_n77A-n79A DC_19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.25.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.25.4	∆TIB and ∆RIB values

For DC_19-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n77-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |

7.25.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.26	DC_19-42_n78-n79

7.26.1	Operating bands for DC

Table 7.26.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n78-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.26.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.26.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-42A_n78A-n79A DC_19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.26.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.26.4	∆TIB and ∆RIB values

For DC_19-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n78-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

7.26.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.27	DC_21-42_n77-n79

7.27.1	Operating bands for DC

Table 7.27.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n77-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.27.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.27.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A-42A_n77A-n79A DC_21A-42C_n77A-n79A | DC_21A_n77A DC_21A_n79A | 21 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.27.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.27.4	∆TIB and ∆RIB values

For DC_21-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n77-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |

7.27.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.28	DC_21-42_n78-n79

7.28.1	Operating bands for DC

Table 7.28.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n78-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.28.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.28.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_21A-42A_n78A-n79A DC_21A-42C_n78A-n79A | DC_21A_n78A DC_21A_n79A | 21 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.28.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.28.4	∆TIB and ∆RIB values

For DC_21-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n78-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

7.28.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.29	DC_3-19_n77-n79

7.29.1	Operating bands for DC

Table 7.29.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.29.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.29.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.29.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.29.4	∆TIB and ∆RIB values

For DC_3-19_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n77-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |

Table 7.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |

7.29.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.30	DC_3-19_n78-n79

7.30.1	Operating bands for DC

Table 7.30.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n78-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.30.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.30.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.30.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.30.4	∆TIB and ∆RIB values

For DC_3-19_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n78-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |

Table 7.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |

7.30.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.31	DC_3-21_n77-n79

7.31.1	Operating bands for DC

Table 7.31.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.31.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.31.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-21A_n77A-n79A | DC_3A_n77A DC_21A_n77A DC_3A_n79A DC_21A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.31.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.31.4	∆TIB and ∆RIB values

For DC_3-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n77-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |

Table 7.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n77-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |

7.31.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.32	DC_3-21_n78-n79

7.32.1	Operating bands for DC

Table 7.32.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n78-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.32.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.32.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-21A_n78A-n79A | DC_3A_n78A DC_21A_n78A DC_3A_n79A DC_21A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 235 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.32.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.32.4	∆TIB and ∆RIB values

For DC_3-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n78-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |

Table 7.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n78-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |

7.32.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.33	DC_3-42_n77-n79

7.33.1	Operating bands for DC

Table 7.33.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.33.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.33.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-42A_n77A-n79A DC_3A-42C_n77A-n79A | DC_3A_n77A DC_3A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.33.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.33.4	∆TIB and ∆RIB values

For DC_3-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n77-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 7.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n77-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

7.33.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.34	DC_3-42_n78-n79

7.34.1	Operating bands for DC

Table 7.34.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n78-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

7.34.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 7.34.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-42A_n78A-n79A DC_3A-42C_n78A-n79A | DC_3A_n78A DC_3A_n79A | 3 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

7.34.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.34.4	∆TIB and ∆RIB values

For DC_3-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n78-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 7.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n78-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

7.34.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.35	DC_3-3-7_n78_n257, DC_3-3-7-7_n78_n257,

7.35.1	Operating bands for DC

Table 7.35.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3-7_n78-n257, DC_3-3-7-7_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.35.2	Channel bandwidths per operating band for DC

Table 7.35.2-1: Supported bandwidths per DC two LTE bands + inter-band NR 2DL/1UL

|  |  |  | DC configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  |  | E-UTRA and NR Band |  | SCS [kHz] |  |  | 5 |  |  |  | 10 |  | 15 |  | 20 |  | 40 |  |  |  | 50 |  |  | 60 |  |  | 80 |  | 90 |  | 100 |  |  | 200 |  | 400 | Maximum aggregated bandwidth [MHz] |  |  |
| DC_3A-3A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 560 |  |  |
|  |  |  |  | 7 |  | 15 |  |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 580 |  |  |
|  |  |  |  | 7 |  |  |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |
| DC_3A-3A-7A_n78A-n257D |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  | 3 |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 560 |  |
|  |  |  |  |  | 7 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  | See CA_n257D in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257E |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 760 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257E in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257F |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 960 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257F in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257G |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257G in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257H |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257H in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257I |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257I in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257J |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 660 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257J in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257K |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 760 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257K in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257L |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 860 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257L in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A_n78A-n257M |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  |  | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 960 |
|  |  |  |  |  | 7 |  |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257M in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257D |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |  | 3 |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 580 |  |  |
|  |  |  |  |  | 7 |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  | See CA_n257D in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257E |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 780 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257E in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257F |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 980 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257F in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257G |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257G in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257H |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257H in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257I |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257I in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257J |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 680 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257J in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257K |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 780 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257K in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257L |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 880 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257L in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-3A-7A-7A_n78A-n257M |  |  |  | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | 3 |  |  | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 980 |
|  |  |  |  |  | 7 |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  |  | 15 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  | See CA_n257M in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.35.3	Co-existence studies

Based on co-existence studies of DC_3A-n78A, DC_7A-n78A,  DC_3A_n257A and DC_7A_n257A specified in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.35.4	∆TIB and ∆RIB values

For DC_3-3-7_n78-n257 and DC_3-3-7-7_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n78-n257, DC_3-3-3-7_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3-7_n78-n257, DC_3-3-3-7_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.35.5	MSD

No additional MSD requirement is needed.

7.36	DC_3-7_n1-n78

7.36.1	Operating bands for DC

Table 7.36.1-1: DC band combination of two LTE bands + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.36.2	Channel bandwidths per operating band for DC

Table 7.36.2-1: Supported bandwidths per DC band combination of two LTE bands + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.36.3	Co-existence studies

Co-existence studies of Band 3 + Band n78 and Band 7 + Band n78 are captured in TR 37.863-01-01.

Co-existence studies of Band 3 + Band n1 and Band 7 + Band n1 are captured in TR 37.716-11-11.

-	2nd harmonics from Band 3 may fall into n78 when dual uplink is configured with either Band 3 + Band n1 or Band 3 + Band n78.

7.36.4	∆TIB and ∆RIB values

For DC_3-7_n1-n78, the TIB,c and RIB values are same as for CA_1-3-7-42 and DC_1-3-7_n78, and are given in the tables below.

Table 7.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n1 | 0.7 |
|  | n78 | 0.8 |

Table 7.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n78 | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |

7.36.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

7.37	DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78

7.37.1	Operating bands for DC

Table 7.37.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.37.2	Channel bandwidths per operating band for DC

Table 7.37.2-1: Supported bandwidths per DC two LTE bands + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-3A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.37.3	Co-existence studies

The Co-existence studies of DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 can be covered by the fallback combinations DC_3-7_n1-n78 in section 7.36.

7.37.4	∆TIB and ∆RIB values

The TIB,c and RIB values are same as for DC_3-7_n1-n78.

Table 7.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n1 | 0.7 |
|  | n78 | 0.8 |

Table 7.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78 | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |

7.37.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

7.38	DC_1-3_n5-n78

7.38.1	Operating bands for DC

Table 7.38.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.38.2	Channel bandwidths per operating band for DC

Table 7.38.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.38.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.38.4	∆TIB and ∆RIB values

For DC_1-3_n5-n78, the TIB,c and RIB values are same as for DC_1-3-5_n78 and are given in the tables below.

Table 7.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n5-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n5 | 0.3 |
|  | n78 | 0.8 |

Table 7.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.38.5	MSD

All MSD requirements for DC_1-3_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.39	DC_1-7_n5-n78

7.39.1	Operating bands for DC

Table 7.39.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.39.2	Channel bandwidths per operating band for DC

Table 7.39.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-7A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.39.3	Co-existence studies

The harmonic and IMD products caused by DC_1-7_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.39.4	∆TIB and ∆RIB values

For DC_1-7_n5-n78, the TIB,c and RIB values are same as for DC_1-5-7_n78 and are given in the tables below.

Table 7.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n5-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n5-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.39.5	MSD

All MSD requirements for DC_1-7_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.40	DC_1-28_n5-n78

7.40.1	Operating bands for DC

Table 7.40.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.40.2	Channel bandwidths per operating band for DC

Table 7.40.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.40.3	Co-existence studies

The harmonic and IMD products caused by DC_1-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.40.4	∆TIB and ∆RIB values

For DC_1-28_n5-n78, the TIB,c and RIB values are derived from DC_1-20_n28-n78 and are given in the tables below.

Table 7.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n5-n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n5-n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.40.5	MSD

All MSD requirements for DC_1-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.41	DC_3-7_n5-n78

7.41.1	Operating bands for DC

Table 7.41.3-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.41.2	Channel bandwidths per operating band for DC

Table 7.41.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A-7A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A DC_7A_n5A DC_7A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3A-7C_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7C_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.41.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.41.4	∆TIB and ∆RIB values

For DC_3-7_n5-n78, the TIB,c and RIB values are same as for DC_3-5-7_n78 and are given in the tables below.

Table 7.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n5-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n5-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.41.5	MSD

All MSD requirements for DC_3-7_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.42	DC_3-28_n5-n78

7.42.1	Operating bands for DC

Table 7.42.3-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.42.2	Channel bandwidths per operating band for DC

Table 7.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.42.3	Co-existence studies

The harmonic and IMD products caused by DC_3-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.42.4	∆TIB and ∆RIB values

For DC_3-28_n5-n78, the TIB,c and RIB values are derived from DC_3-20_n28-n78 and are given in the tables below.

Table 7.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n5-n78 | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n5-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.42.5	MSD

All MSD requirements for DC_3-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.43	DC_7-28_n5-n78

7.43.1	Operating bands for DC

Table 7.43.7-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n5-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.43.2	Channel bandwidths per operating band for DC

Table 7.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_7A-28A_n5A-n78A | DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

7.43.3	Co-existence studies

The harmonic and IMD products caused by DC_7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

7.43.4	∆TIB and ∆RIB values

For DC_7-28_n5-n78, the TIB,c and RIB values are derived from DC_7-20_n28-n78 and are given in the tables below.

Table 7.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n5-n78 | 7 | 0.3 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 7.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n5-n78 | 7 | 0 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

7.43.5	MSD

All MSD requirements for DC_7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.44	DC_1A-3A_n77A-n257

7.44.1	Operating bands for DC

Table 7.44.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.44.2	Channel bandwidths per operating band for DC

Table 7.44.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A_n77A-n257A | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A_n77A-n257D | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257D DC_3A_n257A DC_3A_n257D | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n77A-n257G | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n257A DC_3A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n77A-n257H | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n257A DC_3A_n257G DC_3A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n77A-n257I | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.44.3	Co-existence studies

Co-existence studies of Band 1 + Band n77, Band 3 + Band n77, Band 1 + n257 and Band 3 + n257 are captured in TR 37.863-01-01, own Rx impact of the 3rd band is the followings

-	7th and 8th harmonics from n77 may fall into n257 and 2nd, 3rd and 5th order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n77

-	7th and 8th harmonics from n77 may fall into n257 and 5th order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n77

-	2nd harmonic from Band 1 may fall into n77 and 3rd order IMD may fall into own Rx of Band 3 when dual uplink is configured with Band 1 + Band n257

-	2nd harmonic from Band 3 may fall into n77 and 3rd order IMD may fall into own Rx of Band 1 when dual uplink is configured with Band 3 + Band n257.

7.44.4	∆TIB and ∆RIB values

For DC_1A-3A_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n77-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n77-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.44.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively and fallback modes are captured in TR 37.864-41-21. Therefore, additional analysis is not needed.

-	Uplink DC configuration is DC_1A_n77A: Up to 7th harmonic order will be not any impact to n257 and IMD issues on 3rd own Rx band will be covered in DC_1A-3A_n77A

-	Uplink DC configuration is DC _3A_n77A: Up to 7th harmonic order will be not any impact to n257 and IMD problems will be covered in DC_1A-3A_n77A

-	Uplink DC configuration is DC_1A_n257A: Harmonic problem already discussed and solved in DC_1A_n77A

-    Uplink DC configuration is DC_3A_n257A: Harmonic problem is already solved in DC_3A_n77A

7.45	DC_2-66_n41-n71

7.45.1	Operating bands for DC

Table 7.45.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

7.45.2	Channel bandwidths per operating band for DC

Table 7.45.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

7.45.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there are IMD2 and IMD5 impacts from DC_2_n41 UL to Band n71 DL, but no impact on 66 DL

Co-existence analysis for DC_2_n71 shows that there is IMD2 impact from DC_2_n71 UL to Band n41 DL, but no impact on 66 DL

Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL, but no impact on n71 DL

Co-existence analysis for DC_66_n71 shows that there is no impact from DC_66_n71 UL to either Band 2 DL or Band n41 DL

7.45.4	∆TIB and ∆RIB values

For DC_2-66_n41-n71 the TIB,c and RIB,c values are derived from LTE combinations CA_2-7-12-66 and are given in the tables below.

Table 7.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41-n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41-n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

7.45.5	MSD

UL 2_n41 into DL n71 create IMD2 and UL 2_n71 into DL n41 create IMD2, and they are both captured in the lower order combination DC_2_n41-n71.

UL 66_n41 into DL n2 create IMD4, and this is captured in the lower order combination DC_2-66_n41.

Based on the co-existence studies there are no further MSD needed to be defined.

7.46	DC_2-66_n41-n260

7.46.1	Operating bands for DC

Table 7.46.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n41-n260 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

7.46.2	Channel bandwidths per operating band for DC

Table 7.46.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A-n260A | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n260(2A) | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(2A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n260(3A) | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1340 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(3A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n260(4A) | DC_2A_n41A DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 1740 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n260 | See CA_n260(4A) BCS 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.46.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to either Band 66 DL or Band n260 DL.

Co-existence analysis for DC_66_n41 shows that there is no impact from DC_66_n41 UL to Band n260 DL, but that there are IMD4 from UL_66_n41 into band 2 DL.

7.46.4	∆TIB and ∆RIB values

For DC_2-66_n41-n260 the TIB,c and RIB,c values are based on DC_2-66_n41 and are given in the tables below.

Table 7.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41-n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. |  |  |

Table 7.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41-n260 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n260 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

7.46.5	MSD

UL 66_n41 into band DL creates IMD4 which are captured in lower order combination DC_2-66_n41. Based on the co-existence studies there are no further needs to define additional MSD.

7.47	DC_2A-66A_n41A-n261

7.47.1	Operating bands for DC

Table 7.47.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A-n41A-n261 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 7.47.2	Channel bandwidths per operating band for DC

Table 7.47.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_2A-66A_n41A-n261A | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n261A | DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| DC_2A-66A_n41A-n261(2A) | DC_2A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-66A_n41A-n261(2A) | DC_66A_n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 940 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | § | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.47.3	Co-existence studies

Co-existence studies of DC_2A-n41A are captured in TR 37.716-01-01, for DC_66_n41 are captured in TR 37.716-11-11 and for EN-DC between E-UTRA and FR2 DC_2A_n261A or DC_66A_n261A there will no requirements specified.

7.47.4	∆TIB and ∆RIB values

For DC_2A-66A_n41A-n261A, the TIB,c and RIB,c values are given in the tables below based on DC_66A_n41A-n261A and DC_2A_n41A-n261A in 37.716-21-21.

Table 7.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-66A_n41A-n261A | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n261 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2A-66A_n41A-n261A | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n261 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

7.47.5	MSD

MSD due to close proximity is needed and is already specified in TS 38.101-3 for DC_66_n41.

7.48	DC_3-8_n78-n257

7.48.1	Operating bands for DC

Table 7.48.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n78-n257 | 3 | 1710MHz | – | 1785MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.48.2	Channel bandwidths per operating band for DC

Table 7.48.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-8A_n78A-n257A |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-8A_n78A-n257D |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257E |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257F |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257G |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 330 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257H |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 430 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257I |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257J |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 630 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257K |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257L |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 830 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-8A_n78A-n257M |  | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A | 3 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.48.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR 37.863-02-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

7.48.4	∆TIB and ∆RIB values

For DC_3-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n78-n257 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.48.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n78-n257 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.48.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.49	DC_1-8_n78-n257

7.49.1	Operating bands for DC

Table 7.49.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

7.49.2	Channel bandwidths per operating band for DC

Table 7.49.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-8A_n78A-n257A |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-8A_n78A-n257D |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257E |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257F |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257G |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 330 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257H |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 430 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257I |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257J |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 630 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257K |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257L |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 830 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257M |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.49.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR 37.863-02-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

7.49.4	∆TIB and ∆RIB values

For DC_1-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n78-n257 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.49.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n78-n257 | 1 | 0 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.49.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.50	DC_2-7_n38-n78

7.50.1	Operating bands for DC

Table 7.50.1-1: DC band combination of two LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n38-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.50.2	Channel bandwidths per operating band for DC

Table 7.50.2-1: Supported bandwidths per DC band combination of two LTE band + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_2A-7A-7A_n38A-n78A | DC_2A-n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See the CA_7A-7A configuration in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_2A-7A_n38A-n78A | DC_2A-n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_2A-7C_n38A-n78A | DC_2A-n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | See the CA_7C configuration in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.50.3	Co-existence studies

The co-existence studies for the lower order DC configuration (DC_2-7_n78) is already captured in TR 37.716-21-11. For additional receiver band (band n38), there is no MSD issue for this DC configuration.

7.50.4	∆TIB and ∆RIB values

For DC_2-7_n38-n78, the TIB,c and RIB values are given in the tables below.

Table 7.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n38-n78 | 2 | 0.6 |
|  | n78 | 0.8 |

Table 7.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n38-n78 | 2 | 0.2 |
|  | 7 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

7.50.5	MSD

No MSD is required for this DC configuration.

7.51	DC_7A-7A-66A_n38A-n78A

7.51.1	Operating bands for DC

Table 7.51.1-1: DC band combination of two LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-7A-66A_n38A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.51.2	Channel bandwidths per operating band for DC

Table 7.51.2-1: Supported bandwidths per DC band combination of two LTE band + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-7A-66A_n38A-n78A |  | DC_66A_ n78A | 7 | 15 | See the CA_7A-7A configuration in TS36.101 |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.51.3	Co-existence studies

The co-existence studies for the lower order DC configuration (DC_7-66_n78) is already captured in TR 37.716-21-11. For additional receiver band (band n38), there is no MSD issue for this DC configuration.

7.51.4	∆TIB and ∆RIB values

For DC_7A-7A-66A_n38A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-7A-66A_n38A-n78A | 66 | TBD |
|  | n78 | 0.8 |

Table 7.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-7A-66A_n38A-n78A | 7 | 0 |
|  | 66 | TBD |
|  | n38 | TBD |
|  | n78 | 0.5 |

7.51.5	MSD

No MSD is required for this DC configuration.

7.52		DC_1-3_n38-n78

7.52.1		Operating bands for DC

Table 7.52.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n38-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.52.2	Channel bandwidths per operating band for DC

Table 7.52.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n38A-n78A | DC_3A_n38A DC_3A_n78A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.52.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively, which means the harmonic and IMD products caused by DC_1-3_n38-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

7.52.4	∆TIB and ∆RIB values

For DC_1-3_n38-n78, the TIB,c and RIB values are given in the tables below.

Table 7.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n38-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | n38 | 0.6 |
|  | n78 | 0.8 |

Table 7.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n38-n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | n38 | 0 |
|  | n78 | 0.5 |

7.52.5	MSD

All MSD requirements for DC_1-3_n38-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.53	DC_1-8_n3-n28

7.53.1	Operating bands for DC

Table 7.53.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

7.53.2	Channel bandwidths per operating band for DC

Table 7.53.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 70 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.53.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n3, DC_1-8_n28, DC_1_n3-n28 and DC_8_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.53.4	∆TIB and ∆RIB values

For DC_1-8_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |

Table 7.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | n3 | 0 |
|  | n28 | 0.2 |

7.53.5	MSD

As mentioned in 7.53.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.54	DC_3-8_n1-n78, DC_3-3-8_n1-n78

7.54.1	Operating bands for DC

Table 7.54.1-1: DC band combination of two LTE bands + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.54.2	Channel bandwidths per operating band for DC

Table 7.54.2-1: Supported bandwidths per DC band combination of two LTE bands + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.54.3	Co-existence studies

Co-existence studies of Band 3 + Band n78 and Band 8 + Band n78 are captured in TR 37.863-01-01.

Co-existence studies of Band 3 + Band n1 and Band 8 + Band n1 are captured in TR 37.716-11-11.

Own Rx impact on the 3rd band:

- 2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may fall into own Rx of band n78.

- 5th order IMD generated by dual uplink of Band 3 + Band n78 may fall into own Rx of band n1.

- 3rd order IMD generated by dual uplink of Band 8 + Band n1 may fall into own Rx of band n78.

- 3rd order IMD generated by dual uplink of Band 8 + Band n78 may fall into own Rx of band 3.

7.54.4	∆TIB and ∆RIB values

For DC_3-8_n1-n78 and DC_3-3-8_n1-n78, the TIB,c and RIB values are same as for DC_1-3-8_n78, and are given in the tables below.

Table 7.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.54.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

7.55	DC_7-8_n1-n78, DC_7-7-8_n1-n78

7.55.1	Operating bands for DC

Table 7.55.1-1: DC band combination of two LTE bands + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.55.2	Channel bandwidths per operating band for DC

Table 7.55.2-1: Supported bandwidths per DC band combination of two LTE bands + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-8A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A-7A-8A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.55.3	Co-existence studies

Co-existence studies of Band 7 + Band n78 and Band 8 + Band n78 are captured in TR 37.863-01-01.

Co-existence studies of Band 7 + Band n1 and Band 8 + Band n1 are captured in TR 37.716-11-11.

Own Rx impact on the 3rd band:

- 4th and 5th order IMD generated by dual uplink of Band 7 + Band n1 may fall into own Rx of band n78.

- 5th order IMD generated by dual uplink of Band 7 + Band n1 may fall into own Rx of band 8

- 2nd and 5th order IMD generated by dual uplink of Band 7 + Band n78 may fall into own Rx of band 8.

- 4th order IMD generated by dual uplink of Band 7 + Band n78 may fall into own Rx of band n1.

- 3rd order IMD generated by dual uplink of Band 8 + Band n1 may fall into own Rx of band n78.

- 2nd order IMD generated by dual uplink of Band 8 + Band n78 may fall into own Rx of band 7.

7.55.4	∆TIB and ∆RIB values

For DC_7-8_n1-n78 and DC_7-7-8_n1-n78, the TIB,c and RIB values are given in the tables below.

Table 7.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

7.55.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

7.56	DC_1A-18A_n3-n78

7.56.1	Operating bands for DC

Table 7.56.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.56.2	Channel bandwidths per operating band for DC

Table 7.56.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 165 |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.56.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.56.4	∆TIB and ∆RIB values

For DC_1A-18A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n78 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n3-n78 | 1 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.56.5	MSD

All MSD requirements for this configuration are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.57	DC_1A-28A_n3-n78

7.57.1	Operating bands for DC

Table 7.57.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A_n3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 2805 MHz | – | 2880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.57.2	Channel bandwidths per operating band for DC

Table 7.57.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-28A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 170 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.57.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.57.4	∆TIB and ∆RIB values

For DC_1A-28A_n3A-n78A, the TIB,c and RIB values are given in the tables below.

Table 7.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n3-n78 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n3-n78 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.57.5	MSD

All MSD requirements for this configuration are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.58	DC_3-20_n1-n28

7.58.1	Operating bands for DC

Table 7.58.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20_n1-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |
| NOTE: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 7.58.2	Channel bandwidths per operating band for DC

Table 7.58.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 80 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_3C_n28A DC_20A_n28A | 3 | See CA_3C in TS 38.101-1 or Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes7 |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 7: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.58.3	Co-existence studies

Co-existence studies of DC_3_n1A, DC_3_n28, DC_3_n20, DC_20_n1, DC_20_n28 and DC_1_n28 are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.58.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n28 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n28 | 0.6 |

Table 7.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n1-n28 | 3 | 0 |
|  | 20 | 0 |
|  | n1 | 0.2 |
|  | n28 | 0.2 |

7.58.5	MSD

MSD was studied in lower order band combinations for fallback modes. No need for additional MSD analysis.

7.59	DC_1-11_n77-n257

7.59.1	Operating bands for DC

Table 7.59.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.59.2	Channel bandwidths per operating band for DC

Table 7.59.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-11A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_1A-11A_n77A-n257D | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-11A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 330 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-11A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 430 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-11A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 530 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

7.59.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n77, DC_1-11_n257, DC_1_n77-n257 and DC_11_n77-n257. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.59.4	∆TIB and ∆RIB values

For DC_1-11_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n77-n257 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.59.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n77-n257 | 1 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.59.5	MSD

As mentioned in 7.59.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.60	DC_8-11_n77-n257

7.60.1	Operating bands for DC

Table 7.60.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n77-n257 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.60.2	Channel bandwidths per operating band for DC

Table 7.60.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_8A-11A_n77A-n257A | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 520 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_8A-11A_n77A-n257D | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 520 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-11A_n77A-n257G | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 320 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-11A_n77A-n257H | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 420 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-11A_n77A-n257I | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  | 520 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

7.60.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n77, DC_8-11_n257, DC_8_n77-n257 and DC_11_n77-n257. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.60.4	∆TIB and ∆RIB values

For DC_8-11_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n77-n257 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n77-n257 | 8 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.60.5	MSD

As mentioned in 7.60.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.61	DC_1A-18A_n3A-n257

7.61.1	Operating bands for DC

Table 7.61.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.61.2	Channel bandwidths per operating band for DC

Table 7.61.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-18A_n3A-n257A | DC_1A_n3A DC_1A_n257A DC_18A_n3A DC_18A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-18A_n3A-n257G | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_18A_n3A DC_18A_n257A DC_18A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 265 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A_n3A-n257H | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 365 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 465 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.61.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.61.4	∆TIB and ∆RIB values

For DC_1-18_n3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n257 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 7.61.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n3-n257 | 1 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |
|  | n257 | 0 |

7.61.5	MSD

As mentioned in 7.61.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.62	DC_1A-28A_n3A-n257

7.62.1	Operating bands for DC

Table 7.62.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A_n3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 2805 MHz | – | 2880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.62.2	Channel bandwidths per operating band for DC

Table 7.62.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-28A_n3A-n257A | DC_1A_n3A DC_1A_n257A DC_28A_n3A DC_28A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-28A_n3A-n257G | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_28A_n3A DC_28A_n257A DC_28A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n3A-n257H | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 370 |  |
|  |  | 28 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.62.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.62.4	∆TIB and ∆RIB values

For DC_1-28_n3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.62.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n3-n257 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |
|  | n257 | 0 |

Table 7.62.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n3-n257 | 1 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |
|  | n257 | 0 |

7.62.5	MSD

As mentioned in 7.62.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.63	DC_1A-41_n78A-n257

7.63.1	Operating bands for DC

Table 7.63.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-41_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.63.2	Channel bandwidths per operating band for DC

Table 7.63.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-41A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_41A_n78A DC_41A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-41A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_41A_n78A DC_41C_n78A DC_41A_n257A DC_41C_n257A | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 260 |  |
|  |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-41C_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41C_n78A DC_41C_n257A DC_41C_n257G | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |  |
|  |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |  |
|  |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 41 | See CA_n41C in Table 5.6A1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |  |
|  |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.63.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.63.4	∆TIB and ∆RIB values

For DC_1-41_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n78-n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n78-n257 | 1 | 0 |
|  | 41 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.63.5	MSD

As mentioned in 7.63.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.64	DC_3A-41A_n78A-n257

7.64.1	Operating bands for DC

Table 7.64.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.64.2	Channel bandwidths per operating band for DC

Table 7.64.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-41A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_41A_n78A DC_41A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-41A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.64.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.64.4	∆TIB and ∆RIB values

For DC_3-41_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n78-n257 | 3 | 0.6 |
|  | 41 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n78-n257 | 3 | 0.2 |
|  | 41 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.64.5	MSD

As mentioned in 7.64.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.65	DC_3A-42A_n78A-n257

7.65.1	Operating bands for DC

Table 7.65.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.65.2	Channel bandwidths per operating band for DC

Table 7.65.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-42A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-42A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.65.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.65.4	∆TIB and ∆RIB values

For DC_3-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n78-n257 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n78-n257 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.65.5	MSD

As mentioned in 7.65.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.66	DC_41A-42A_n78A-n257

7.66.1	Operating bands for DC

Table 7.66.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41-42_n78-n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.66.2	Channel bandwidths per operating band for DC

Table 7.66.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_41A-42A_n78A-n257A | DC_41A_n78A DC_41A_n257A DC_42A_n257A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_41A-42A_n78A-n257G | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42A_n78A-n257H | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42A_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.66.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.66.4	∆TIB and ∆RIB values

For DC_41-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41-42_n78-n257 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.66.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41-42_n78-n257 | 41 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.66.5	MSD

As mentioned in 7.66.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.67	DC_1A-42A_n78A-n257

7.67.1	Operating bands for DC

Table 7.67.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.67.2	Channel bandwidths per operating band for DC

Table 7.67.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.67.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.67.4	∆TIB and ∆RIB values

For DC_1-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n78-n257 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n78-n257 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.67.5	MSD

As mentioned in 7.67.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.68	DC_1A-18A_n78A-n257

7.68.1	Operating bands for DC

Table 7.68.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-18_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.68.2	Channel bandwidths per operating band for DC

Table 7.68.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-18A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_18A_n78A DC_18A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 235 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-18A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_18A_n78A DC_18A_n257A DC_18A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 335 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 435 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 535 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.68.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.68.4	∆TIB and ∆RIB values

For DC_1-18_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n78-n257 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.68.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18_n78-n257 | 1 | 0 |
|  | 18 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.68.5	MSD

As mentioned in 7.68.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.69	DC_3A-18A_n78A-n257

7.69.1	Operating bands for DC

Table 7.69.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-18_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.69.2	Channel bandwidths per operating band for DC

Table 7.69.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-18A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_18A_n78A DC_18A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 235 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-18A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_18A_n78A DC_18A_n257A DC_18A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 335 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 18.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-18A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 435 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 18.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-18A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 535 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 18.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.69.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.69.4	∆TIB and ∆RIB values

For DC_3-18_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n78-n257 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.69.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18_n78-n257 | 3 | 0.2 |
|  | 18 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.69.5	MSD

As mentioned in 7.69.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.70	DC_18A-42A_n78A-n257

7.70.1	Operating bands for DC

Table 7.70.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18-42_n78-n257 | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 42 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.70.2	Channel bandwidths per operating band for DC

Table 7.70.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_18A-42A_n78A-n257A | DC_18A_n78A DC_18A_n257A DC_42A_n257A | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 235 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_18A-42A_n78A-n257G | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_42A_n257A DC_42A_n257G | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A-42A_n78A-n257H | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A-42A_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.70.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.70.4	∆TIB and ∆RIB values

For DC_18-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-42_n78-n257 | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_18-42_n78-n257 | 18 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.70.5	MSD

As mentioned in 7.70.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.71	DC_3A-28A_n78A-n257

7.71.1	Operating bands for DC

Table 7.71.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.71.2	Channel bandwidths per operating band for DC

Table 7.71.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-28A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_28A_n78A DC_28A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-28A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.71.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.71.4	∆TIB and ∆RIB values

For DC_3-28_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n78-n257 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n78-n257 | 3 | 0.2 |
|  | 28 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.71.5	MSD

As mentioned in 7.71.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.72	DC_28A-41A_n78A-n257

7.72.1	Operating bands for DC

Table 7.72.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28-41_n78-n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.72.2	Channel bandwidths per operating band for DC

Table 7.72.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A-41A_n78A-n257A | DC_28A_n78A DC_28A_n257A DC_41A_n78A DC_41A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_28A-41A_n78A-n257G | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A-41A_n78A-n257H | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A-41A_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.72.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.72.4	∆TIB and ∆RIB values

For DC_28-41_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-41_n78-n257 | 28 | 0.5 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-41_n78-n257 | 28 | 0.2 |
|  | 41 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.72.5	MSD

As mentioned in 7.72.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.73	DC_28A-42A_n78A-n257

7.73.1	Operating bands for DC

Table 7.73.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28-42_n78-n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.73.2	Channel bandwidths per operating band for DC

Table 7.73.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A-42A_n78A-n257A | DC_28A_n78A DC_28A_n257A DC_42A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_28A-42A_n78A-n257G | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_42A_n257A DC_42A_n257G | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A-42A_n78A-n257H | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A-42A_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.73.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.73.4	∆TIB and ∆RIB values

For DC_28-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-42_n78-n257 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-42_n78-n257 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.73.5	MSD

As mentioned in 7.73.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.74	DC_1A-28A_n78A-n257

7.74.1	Operating bands for DC

Table 7.74.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-28_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.74.2	Channel bandwidths per operating band for DC

Table 7.74.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-28A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_28A_n78A DC_28A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 240 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-28A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.74.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

7.74.4	∆TIB and ∆RIB values

For DC_1-28_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n78-n257 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n78-n257 | 1 | 0 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.74.5	MSD

As mentioned in 7.74.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.75	DC_2-7_n66-n78 and DC_2-7-7_n66-n78

### 7.75.1	Operating bands for DC

Table 7.75.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-7A_n66A-n78A DC_2A-7A-7A_n66A-n78A DC_2A-7C_n66A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.75.2	Channel bandwidths per operating band for DC

Table 7.75.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) |  |  | 5 | 10 | 15 | 20 |  | 25 |  | 30 |  | 40 | 50 |  |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |
| DC_2A-7A_n66A-n78A DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |  | 2 | 15 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  |  | 7 | 15 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 15 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |  | 2 | 15 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  |  | 7 | 15 |  |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 | 15 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |  | 2 |  | 15 | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 200 |  |
|  |  |  | 7 |  | 15 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n66 |  | 15 | Yes |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  |  | Yes | Yes |  | Yes |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  |  | Yes | Yes |  | Yes |  | Yes | Yes | Yes |  |  | Yes | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  | 60 |  |  | Yes | Yes |  | Yes |  | Yes | Yes | Yes |  |  | Yes | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |

### 7.75.3	Co-existence studies

Co-existence studies of DC_2A-7A_n66A-n78A, DC_2A-7C_n66A-n78A and DC_2A-7A-7A_n66A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.75.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_2A-7A_n66A-n78A, DC_2A-7C_n66A-n78A and DC_2A-7A-7A_n66A-n78A are given in the tables below.

Table 7.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 7.75.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 7.75.5	MSD

All MSD requirements for DC_2A-7A_n66A-n78A, DC_2A-7C_n66A-n78A and DC_2A-7A-7A_n66A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.76	DC_2-66_n66-n78

7.76.1	Operating bands for DC

Table 7.76.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-66A_n66A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.76.2	Channel bandwidths per operating band for DC

Table 7.76.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_2A-66A_n66A-n78A |  | DC_2A_n66A DC_2A_n78A DC_66A_n66A1 |  | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  |  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n66 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | NOTE1: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.76.3	Co-existence studies

The harmonic and IMD products caused by DC_2A-66A_n66A-n78A and protected bands follow from those derived for the constituent fallback modes.

7.76.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_2A-66A_n66A-n78A are given in the tables below.

Table 7.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 7.76.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |

### 7.76.5	MSD

All MSD requirements for DC_2A-66A_n66A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.77	DC_7-66_n66-n78 and DC_7-7-66_n66-n78

### 7.77.1	Operating bands for DC

Table 7.77.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-66A_n66A-n78A DC_7A-7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.77.2	Channel bandwidths per operating band for DC

Table 7.77.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | UL Configuration |  | E-UTRA and NR Band |  | SCS (kHz) |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 | Maximum aggregated bandwidth [MHz] |  |
| DC_7A-66A_n66A-n78A |  |  | DC_7A_n66A DC_7A_n78A DC_66A_n66A1 DC_66A_n78A |  | 7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 180 |  |
|  |  |  |  |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| DC_7A-7A-66A_n66A-n78A |  |  | DC_7A_n66A DC_7A_n78A DC_66A_n66A1 DC_66A_n78 |  | 7 |  | 15 |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 200 |  |
|  |  |  |  |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| DC_7C-66A_n66A-n78A |  | DC_7A_n66A DC_7A_n78A DC_66A_n66A1 DC_66A_n78A |  |  | 7 | 15 |  | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 200 |  |
|  |  |  |  |  | 66 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n66 | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |
|  | NOTE1: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.77.3	Co-existence studies

The harmonic and IMD products caused by DC_7A-66A_n66A-n78A, DC_7C-66A_n66A-n78A and DC_7A-7A-66A_n66A-n78A and protected bands follow from those derived for the constituent fallback modes.

### 7.77.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7A-66A_n66A-n78A, DC_7C-66A_n66A-n78A and DC_7A-7A-66A_n66A-n78A are given in the tables below.

Table 7.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 7.77.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 7.77.5	MSD

All MSD requirements for DC_7A-66A_n66A-n78A, DC_7C-66A_n66A-n78A and DC_7A-7A-66A_n66A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.78		DC_1-20_n3-n38

7.78.1		Operating bands for DC

Table 7.78.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-20_n3-n38 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | TDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

7.78.2	Channel bandwidths per operating band for DC

Table 7.78.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 (MHz) | 10 (MHz) | 15 (MHz) | 20 (MHz) | 25 (MHz) | 30 (MHz) | 40 (MHz) | 50 (MHz) | 60 (MHz) | 80 (MHz) | 90 (MHz) | 100 (MHz) | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A | 1 | Please see BCS0 of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |

7.78.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively, which means the harmonic and IMD products caused by DC_1-20_n3-n38 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

7.78.4	∆TIB and ∆RIB values

For DC_1-20_n3-n38, the TIB,c and RIB values are given in the tables below.

Table 7.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n3-n38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |
|  | n38 | 0.5 |

Table 7.78.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n3-n38 | 1 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |
|  | n38 | 0 |

7.78.5	MSD

All MSD requirements for DC_1-20_n3-n38 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.79		DC_1-20_n3-n78

7.79.1		Operating bands for DC

Table 7.79.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-20_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.79.2	Channel bandwidths per operating band for DC

Table 7.79.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A | 1 | Please see BCS0 of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS0 of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.79.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively, which means the harmonic and IMD products caused by DC_1-20_n3-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

7.79.4	∆TIB and ∆RIB values

For DC_1-20_n3-n78, the TIB,c and RIB values are given in the tables below.

Table 7.79.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n3-n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n3 | 0.3 |
|  | n78 | 0.8 |

Table 7.79.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n3-n78 | 1 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |
|  | n78 | 0.5 |

7.79.5	MSD

All MSD requirements for DC_1-20_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.80		DC_7-20_n3-n78

7.80.1		Operating bands for DC

Table 7.80.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-20_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.80.2	Channel bandwidths per operating band for DC

Table 7.80.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A | 7 | Please see BCS1 of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS0 of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.80.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively, which means the harmonic and IMD products caused by DC_7-20_n3-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

7.80.4	∆TIB and ∆RIB values

For DC_7-20_n3-n78, the TIB,c and RIB values are given in the tables below.

Table 7.80.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n3-n78 | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 7.80.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20_n3-n78 | 7 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |
|  | n78 | 0.5 |

7.80.5	MSD

All MSD requirements for DC_7-20_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.81	DC_3-28_n7-n78

7.81.1	Operating bands for DC

Table 7.81.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n7-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.81.2	Channel bandwidths per operating band for DC

Table 7.81.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-28A_n7A-n78A | DC_3A-n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-28A_n7B-n78A | DC_3A-n7A DC_3A-n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-28A_n7A-n78A | DC_3A-n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-28A_n7B-n78A | DC_3A-n7A DC_3A-n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C-28A_n7A-n78A | DC_3A-n7A DC_3C-n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-28A_n7B-n78A | DC_3A-n7A DC_3C-n7A DC_3A-n7B DC_3C-n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.81.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.81.4	∆TIB and ∆RIB values

For DC_3-28_n7-n78 the TIB,c and RIB,c values are same as for DC_3-28-41_n78 and are given in the tables below.

Table 7.81.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 3 | 1 |
|  | 28 | 0.5 |
|  | n7 | 0.8 |
|  | n78 | 0.8 |

Table 7.81.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | n7 | 0.4 |
|  | n78 | 0.5 |

7.81.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.82	DC_1-28_n7-n78

7.82.1	Operating bands for DC

Table 7.82.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-28_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.82.2	Channel bandwidths per operating band for DC

Table 7.82.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-28A_n7A-n78A | DC_1A-n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-28A_n7B-n78A | DC_1A-n7A DC_1A-n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.82.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.82.4	∆TIB and ∆RIB values

For DC_1-28_n7-n78 the TIB,c and RIB,c values are same as for DC_1-7-28_n78 and are given in the tables below.

Table 7.82.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n7-n78 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 7.82.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n7-n78 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

7.82.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.83	DC_7-28_n3-n78

7.83.1	Operating bands for DC

Table 7.83.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.83.2	Channel bandwidths per operating band for DC

Table 7.83.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-28A_n3A-n78A | DC_7A-n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7C-28A_n3A-n78A | DC_7A-n3A DC_7C-n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.83.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.83.4	∆TIB and ∆RIB values

For DC_7-28_n3-n78 the TIB,c and RIB,c values are same as for DC_3-28-41_n78 and are given in the tables below.

Table 7.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n3-n78 | 7 | 0.8 |
|  | 28 | 0.5 |
|  | n3 | 1 |
|  | n78 | 0.8 |

Table 7.83.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n3-n78 | 7 | 0.5 |
|  | 28 | 0.2 |
|  | n3 | 0.5 |
|  | n78 | 0.5 |

7.83.5	MSD

MSD analysis of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.84	DC_2-46_n41-n66

7.84.1	Operating bands for DC

Table 7.84.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-46_n41-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 46 | 5150 MHz | – | 5350 MHz | 5150 MHz | – | 5350 MHz | TDD |
|  |  | 5470 MHz | – | 5925 MHz | 5470 MHz | – | 5925 MHz |  |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

7.84.2	Channel bandwidths per operating band for DC

Table 7.84.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-46A_n41A-n66A | DC_2A_n41A DC_2A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 180 |
|  |  | 46 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
| DC_2A-46C_n41A-n66A | DC_2A_n41A DC_2A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 200 |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
| DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 220 |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |

7.84.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.84.4	∆TIB and ∆RIB values

For DC_2-46_n41-n66 the TIB,c and RIB,c values are same as for CA_2-7-46-66 and are given in the tables below.

Table 7.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n41-n66 | 2 | 0.5 |
|  | 46 | 0 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |

Table 7.84.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n41-n66 | 2 | 0.3 |
|  | 46 | 0 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |

7.84.5	MSD

There is no need to define MSD.

7.85	DC_2-66_n71-n261

7.85.1	Operating bands for DC

Table 7.85.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n71-n261 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

7.85.2	Channel bandwidths per operating band for DC

Table 7.85.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 100 | 200 | 400 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n71A-n261A | DC_2A-n71A DC_66A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 460 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_2A-66A_n71A-n261(2A) | DC_2A-n71A DC_66A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 860 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | See CA_261(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |

7.85.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.85.4	∆TIB and ∆RIB values

For DC_2-66_n71-n261 the TIB,c and RIB,c values are same as for DC_2-66_n71_and are given in the tables below.

Table 7.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n71-n261 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
|  | n261 | 0 |

Table 7.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n71-n261 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n71 | 0 |
|  | n261 | 0 |

7.85.5	MSD

There is no need to define MSD.

7.86	DC_1-19_n77-n257

7.86.1	Operating bands for DC

Table 7.86.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.86.2	Channel bandwidths per operating band for DC

Table 7.86.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-19A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_19A_n77A DC_19A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_19A_n77A DC_19A_n257A DC_19A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.86.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.86.4	∆TIB and ∆RIB values

For DC_1-19_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n77-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.86.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n77-n257 | 1 | 0 |
|  | 19 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.86.5	MSD

As mentioned in 7.86.3, there is no need to specify additional MSD requirement for this DC configuration.

7.87	DC_1-19_n78-n257

7.87.1	Operating bands for DC

Table 7.87.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.87.2	Channel bandwidths per operating band for DC

Table 7.87.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-19A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_19A_n78A DC_19A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_19A_n78A DC_19A_n257A DC_19A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.87.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.87.4	∆TIB and ∆RIB values

For DC_1-19_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n78-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.87.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n78-n257 | 1 | 0 |
|  | 19 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.87.5	MSD

As mentioned in 7.87.3, there is no need to specify additional MSD requirement for this DC configuration.

7.88	DC_1-19_n79-n257

7.88.1	Operating bands for DC

Table 7.88.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.88.2	Channel bandwidths per operating band for DC

Table 7.88.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-19A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_19A_n79A DC_19A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_19A_n79A DC_19A_n257A DC_19A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.88.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.88.4	∆TIB and ∆RIB values

For DC_1-19_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n79-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.88.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-19_n79-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n79 | 0 |
|  | n257 | 0 |

7.88.5	MSD

As mentioned in 7.88.3, there is no need to specify additional MSD requirement for this DC configuration.

7.89	DC_1-21_n77-n257

7.89.1	Operating bands for DC

Table 7.89.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.89.2	Channel bandwidths per operating band for DC

Table 7.89.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-21A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_21A_n77A DC_21A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.89.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.89.4	∆TIB and ∆RIB values

For DC_1-21_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n77-n257 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.89.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n77-n257 | 1 | 0 |
|  | 21 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.89.5	MSD

As mentioned in 7.89.3, there is no need to specify additional MSD requirement for this DC configuration.

7.90	DC_1-21_n78-n257

7.90.1	Operating bands for DC

Table 7.90.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.90.2	Channel bandwidths per operating band for DC

Table 7.90.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-21A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_21A_n78A DC_21A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.90.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.90.4	∆TIB and ∆RIB values

For DC_1-21_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n78-n257 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.90.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n78-n257 | 1 | 0.2 |
|  | 21 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.90.5	MSD

As mentioned in 7.90.3, there is no need to specify additional MSD requirement for this DC configuration.

7.91	DC_1-21_n79-n257

7.91.1	Operating bands for DC

Table 7.91.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.91.2	Channel bandwidths per operating band for DC

Table 7.91.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-21A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_21A_n79A DC_21A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.91.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.91.4	∆TIB and ∆RIB values

For DC_1-21_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n79-n257 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.91.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n79-n257 | 1 | 0 |
|  | 21 | 0 |
|  | n79 | 0 |
|  | n257 | 0 |

7.91.5	MSD

As mentioned in 7.91.3, there is no need to specify additional MSD requirement for this DC configuration.

7.92	DC_1-3_n79-n257

7.92.1	Operating bands for DC

Table 7.92.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.92.2	Channel bandwidths per operating band for DC

Table 7.92.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_3A_n79A DC_3A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_3A_n79A DC_3A_n257A DC_3A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.92.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.92.4	∆TIB and ∆RIB values

For DC_1-3_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n79-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.92.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n79-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n79 | 0.5 |
|  | n257 | 0 |

7.92.5	MSD

As mentioned in 7.92.3, there is no need to specify additional MSD requirement for this DC configuration.

7.93	DC_1-42_n77-n257

7.93.1	Operating bands for DC

Table 7.93.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.93.2	Channel bandwidths per operating band for DC

Table 7.93.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.93.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.93.4	∆TIB and ∆RIB values

For DC_1-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n77-n257 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.93.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n77-n257 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.93.5	MSD

As mentioned in 7.93.3, there is no need to specify additional MSD requirement for this DC configuration.

7.94	DC_1-42_n79-n257

7.94.1	Operating bands for DC

Table 7.94.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.94.2	Channel bandwidths per operating band for DC

Table 7.94.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-42A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.94.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.94.4	∆TIB and ∆RIB values

For DC_1-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.94.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n79-n257 | 1 | 0.3 |
|  | 42 | 0 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.94.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n79-n257 | 1 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

7.94.5	MSD

As mentioned in 7.94.3, there is no need to specify additional MSD requirement for this DC configuration.

7.95	DC_19-21_n77-n257

7.95.1	Operating bands for DC

Table 7.95.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n77-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.95.2	Channel bandwidths per operating band for DC

Table 7.95.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_19A-21A_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_21A_n77A DC_21A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n77A-n257G | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n77A-n257H | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.95.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.95.4	∆TIB and ∆RIB values

For DC_19-21_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.95.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n77-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.95.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n77-n257 | 19 | 0 |
|  | 21 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.95.5	MSD

As mentioned in 7.95.3, there is no need to specify additional MSD requirement for this DC configuration.

7.96	DC_19-21_n78-n257

7.96.1	Operating bands for DC

Table 7.96.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n78-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.96.2	Channel bandwidths per operating band for DC

Table 7.96.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_19A-21A_n78A-n257A | DC_19A_n78A DC_19A_n257A DC_21A_n78A DC_21A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n78A-n257G | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n78A-n257H | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.96.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.96.4	∆TIB and ∆RIB values

For DC_19-21_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n78-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.96.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n78-n257 | 19 | 0 |
|  | 21 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.96.5	MSD

As mentioned in 7.96.3, there is no need to specify additional MSD requirement for this DC configuration.

7.97	DC_19-21_n79-n257

7.97.1	Operating bands for DC

Table 7.97.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n79-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.97.2	Channel bandwidths per operating band for DC

Table 7.97.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_19A-21A_n79A-n257A | DC_19A_n79A DC_19A_n257A DC_21A_n79A DC_21A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n79A-n257G | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n79A-n257H | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 430 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 530 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.97.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.97.4	∆TIB and ∆RIB values

For DC_19-21_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n79-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.97.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n79-n257 | 19 | 0 |
|  | 21 | 0 |
|  | n79 | 0 |
|  | n257 | 0 |

7.97.5	MSD

As mentioned in 7.97.3, there is no need to specify additional MSD requirement for this DC configuration.

7.98	DC_19-42_n77-n257

7.98.1	Operating bands for DC

Table 7.98.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n77-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.98.2	Channel bandwidths per operating band for DC

Table 7.98.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n77A-n257G | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 335 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n77A-n257H | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 435 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n77A-n257G | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n77A-n257H | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.98.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.98.4	∆TIB and ∆RIB values

For DC_19-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n77-n257 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.98.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n77-n257 | 19 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.98.5	MSD

As mentioned in 7.98.3, there is no need to specify additional MSD requirement for this DC configuration.

7.99	DC_19-42_n78-n257

7.99.1	Operating bands for DC

Table 7.99.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n78-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.99.2	Channel bandwidths per operating band for DC

Table 7.99.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n78A-n257A | DC_19A_n78A DC_19A_n257A DC_42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n78A-n257G | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 335 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n78A-n257H | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 435 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n78A-n257A | DC_19A_n78A DC_19A_n257A DC_42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n78A-n257G | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n78A-n257H | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.99.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.99.4	∆TIB and ∆RIB values

For DC_19-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n78-n257 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.99.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n78-n257 | 19 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.99.5	MSD

As mentioned in 7.99.3, there is no need to specify additional MSD requirement for this DC configuration.

7.100	DC_19-42_n79-n257

7.100.1	Operating bands for DC

Table 7.100.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n79-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.100.2	Channel bandwidths per operating band for DC

Table 7.100.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_19A-42A_n79A-n257A | DC_19A_n79A DC_19A_n257A DC_42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n79A-n257G | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 335 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n79A-n257H | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 435 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42A_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n79A-n257A | DC_19A_n79A DC_19A_n257A DC_42A_n257A | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n79A-n257G | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n79A-n257H | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.100.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.100.4	∆TIB and ∆RIB values

For DC_19-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n79-n257 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.100.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n79-n257 | 19 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

7.100.5	MSD

As mentioned in 7.100.3, there is no need to specify additional MSD requirement for this DC configuration.

7.101	DC_21-42_n77-n257

7.101.1	Operating bands for DC

Table 7.101.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n77-n257 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.101.2	Channel bandwidths per operating band for DC

Table 7.101.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n77A-n257A | DC_21A_n77A DC_21A_n257A DC_42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n77A-n257G | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 335 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n77A-n257H | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 435 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n77A-n257I | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n77A-n257A | DC_21A_n77A DC_21A_n257A DC_42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n77A-n257G | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n77A-n257H | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n77A-n257I | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.101.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.101.4	∆TIB and ∆RIB values

For DC_21-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.101.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n77-n257 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.101.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n77-n257 | 21 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.101.5	MSD

As mentioned in 7.101.3, there is no need to specify additional MSD requirement for this DC configuration.

7.102	DC_21-42_n78-n257

7.102.1	Operating bands for DC

Table 7.102.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n78-n257 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.102.2	Channel bandwidths per operating band for DC

Table 7.102.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n78A-n257A | DC_21A_n78A DC_21A_n257A DC_42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n78A-n257G | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 335 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n78A-n257H | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 435 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n78A-n257I | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n78A-n257A | DC_21A_n78A DC_21A_n257A DC_42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n78A-n257G | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n78A-n257H | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n78A-n257I | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.102.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.102.4	∆TIB and ∆RIB values

For DC_21-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.102.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n78-n257 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.102.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n78-n257 | 21 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.102.5	MSD

As mentioned in 7.102.3, there is no need to specify additional MSD requirement for this DC configuration.

7.103	DC_21-42_n79-n257

7.103.1	Operating bands for DC

Table 7.103.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n79-n257 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.103.2	Channel bandwidths per operating band for DC

Table 7.103.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_21A-42A_n79A-n257A | DC_21A_n79A DC_21A_n257A DC_42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n79A-n257G | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 335 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n79A-n257H | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 435 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42A_n79A-n257I | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 535 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n79A-n257A | DC_21A_n79A DC_21A_n257A DC_42A_n257A | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n79A-n257G | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n79A-n257H | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_21A-42C_n79A-n257I | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 555 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.103.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.103.4	∆TIB and ∆RIB values

For DC_21-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.103.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n79-n257 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.103.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n79-n257 | 21 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

7.103.5	MSD

As mentioned in 7.103.3, there is no need to specify additional MSD requirement for this DC configuration.

7.104	DC_3-19_n77-n257

7.104.1	Operating bands for DC

Table 7.104.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.104.2	Channel bandwidths per operating band for DC

Table 7.104.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-19A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_19A_n77A DC_19A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_19A_n77A DC_19A_n257A DC_19A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.104.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.104.4	∆TIB and ∆RIB values

For DC_3-19_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.104.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n77-n257 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.104.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n77-n257 | 3 | 0.2 |
|  | 19 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.104.5	MSD

As mentioned in 7.104.3, there is no need to specify additional MSD requirement for this DC configuration.

7.105	DC_3-19_n78-n257

7.105.1	Operating bands for DC

Table 7.105.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.105.2	Channel bandwidths per operating band for DC

Table 7.105.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-19A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_19A_n78A DC_19A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_19A_n78A DC_19A_n257A DC_19A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.105.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.105.4	∆TIB and ∆RIB values

For DC_3-19_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.105.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n78-n257 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.105.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n78-n257 | 3 | 0.2 |
|  | 19 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.105.5	MSD

As mentioned in 7.105.3, there is no need to specify additional MSD requirement for this DC configuration.

7.106	DC_3-19_n79-n257

7.106.1	Operating bands for DC

Table 7.106.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n79-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.106.2	Channel bandwidths per operating band for DC

Table 7.106.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-19A_n79A-n257A | DC_3A_n79A DC_3A_n257A DC_19A_n79A DC_19A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n79A-n257G | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_19A_n79A DC_19A_n257A DC_19A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n79A-n257H | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-19A_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 19 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.106.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.106.4	∆TIB and ∆RIB values

For DC_3-19_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.106.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n79-n257 | 3 | 0.3 |
|  | 19 | 0.3 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.106.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n79-n257 | 3 | 0 |
|  | 19 | 0 |
|  | n79 | 0 |
|  | n257 | 0 |

7.106.5	MSD

As mentioned in 7.106.3, there is no need to specify additional MSD requirement for this DC configuration.

7.107	DC_3-21_n77-n257

7.107.1	Operating bands for DC

Table 7.107.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.107.2	Channel bandwidths per operating band for DC

Table 7.107.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-21A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_21A_n77A DC_21A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.107.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.107.4	∆TIB and ∆RIB values

For DC_3-21_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n77-n257 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.107.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n77-n257 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.107.5	MSD

As mentioned in 7.107.3, there is no need to specify additional MSD requirement for this DC configuration.

7.108	DC_3-21_n78-n257

7.108.1	Operating bands for DC

Table 7.108.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.108.2	Channel bandwidths per operating band for DC

Table 7.108.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-21A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_21A_n78A DC_21A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.108.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.108.4	∆TIB and ∆RIB values

For DC_3-21_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n78-n257 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.108.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n78-n257 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.108.5	MSD

As mentioned in 7.108.3, there is no need to specify additional MSD requirement for this DC configuration.

7.109	DC_3-21_n79-n257

7.109.1	Operating bands for DC

Table 7.109.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n79-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.109.2	Channel bandwidths per operating band for DC

Table 7.109.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-21A_n79A-n257A | DC_3A_n79A DC_3A_n257A DC_21A_n79A DC_21A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n79A-n257G | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 335 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n79A-n257H | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 435 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-21A_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 535 |
|  |  | 21 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.109.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.109.4	∆TIB and ∆RIB values

For DC_3-21_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n79-n257 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.109.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n79-n257 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

7.109.5	MSD

As mentioned in 7.109.3, there is no need to specify additional MSD requirement for this DC configuration.

7.110	DC_3-42_n77-n257

7.110.1	Operating bands for DC

Table 7.110.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.110.2	Channel bandwidths per operating band for DC

Table 7.110.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-42A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.110.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.110.4	∆TIB and ∆RIB values

For DC_3-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.110.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n77-n257 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.110.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n77-n257 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.110.5	MSD

As mentioned in 7.110.3, there is no need to specify additional MSD requirement for this DC configuration.

7.111	DC_3-42_n78-n257

7.111.1	Operating bands for DC

Table 7.111.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.111.2	Channel bandwidths per operating band for DC

Table 7.111.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-42A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.111.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.111.4	∆TIB and ∆RIB values

For DC_3-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.111.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n78-n257 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.111.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n78-n257 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

7.111.5	MSD

As mentioned in 7.111.3, there is no need to specify additional MSD requirement for this DC configuration.

7.112	DC_3-42_n79-n257

7.112.1	Operating bands for DC

Table 7.112.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n79-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.112.2	Channel bandwidths per operating band for DC

Table 7.112.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-42A_n79A-n257A | DC_3A_n79A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n79A-n257G | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 340 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n79A-n257H | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 440 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42A_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n79A-n257A | DC_3A_n79A DC_3A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n79A-n257G | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n79A-n257H | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 | See CA_42C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See CA_n79_n257 in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.112.3	Co-existence studies

Based on co-existence studies of the fallback band combinations EN-DC of LTE + NR FR1 and EN-DC of LTE + NR FR2 there is no additional harmonic and intermodulation impact for the proposed band combinations.

7.112.4	∆TIB and ∆RIB values

For DC_3-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n79-n257 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 7.112.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n79-n257 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

7.112.5	MSD

As mentioned in 7.112.3, there is no need to specify additional MSD requirement for this DC configuration.

7.113	DC_2-46_n41-n71

7.113.1	Operating bands for DC

Table 7.113.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-46_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

7.113.2	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.113.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.113.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n41-n71 | 2 | 0.5 |
|  | 46 | 0 |
|  | n41 | 0.5 |
|  | n71 | 0.6 |

Table 7.113.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n41-n71 | 2 | 0 |
|  | 46 | 0 |
|  | n41 | 0 |
|  | n71 | 0.2 |

7.113.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.114	DC_46-66_n25-n41

7.114.1	Operating bands for DC

Table 7.114.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_46-66_n25-n41 | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |

7.114.2	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.114.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.114.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n25-n41 | 46 | 0 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 7.114.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n25-n41 | 46 | 0 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

7.114.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.115	DC_46-66_n41-n71

7.115.1	Operating bands for DC

Table 7.115.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_46-66_n41-n71 | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

7.115.2	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.115.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.115.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n41-n71 | 46 | 0 |
|  | 66 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n71 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 7.115.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n41-n71 | 46 | 0 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.2 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

7.115.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.116	DC_1-18_n3-n77

### 7.116.1	Operating bands for DC

Table 7.116.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n3A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.116.2	Channel bandwidths per operating band for DC

Table 7.116.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.116.3	Co-existence studies

Co-existence studies of DC_1A-18A_n3A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.116.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1A-18A_n3A-n77A are given in the tables below.

Table 7.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.116.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n77 | 1 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.116.5	MSD

All MSD requirements for DC_1A-18A_n3A-n77A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.117	DC_1-28_n3-n77

### 7.117.1	Operating bands for DC

Table 7.117.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-28A_n3A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.117.2	Channel bandwidths per operating band for DC

Table 7.117.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-28A_n3A-n77A | DC_28A_n3A DC_28A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.117.3	Co-existence studies

Co-existence studies of DC_1A-28A_n3A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.117.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1A-28A_n3A-n77A are given in the tables below.

Table 7.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n3-n77 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.117.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n3-n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.117.5	MSD

All MSD requirements for DC_1A-28A_n3A-n77A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.118	DC_1-41_n3-n77

### 7.118.1	Operating bands for DC

Table 7.118.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n3A-n77A DC_1A-41C_n3A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.118.2	Channel bandwidths per operating band for DC

Table 7.118.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.118.3	Co-existence studies

Co-existence studies of DC_1A-41A_n3A-n77A and DC_1A-41C_n3A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.118.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1-41_n3-n77 are given in the tables below.

Table 7.118.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n77 | 1 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.118.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n77 | 1 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.118.5	MSD

All MSD requirements for DC_1-41_n3-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.119	DC_1-41_n3-n78

### 7.119.1	Operating bands for DC

Table 7.119.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n3A-n78A DC_1A-41C_n3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.119.2	Channel bandwidths per operating band for DC

Table 7.119.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.119.3	Co-existence studies

Co-existence studies of DC_1A-41A_n3A-n78A and DC_1A-41C_n3A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.119.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1-41_n3-n78 are given in the tables below.

Table 7.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n78 | 1 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.119.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n78 | 1 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.119.5	MSD

All MSD requirements for DC_1-41_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.120	DC_1-41_n28-n77

### 7.120.1	Operating bands for DC

Table 7.120.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n28A-n77A DC_1A-41C_n28A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.120.2	Channel bandwidths per operating band for DC

Table 7.120.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.120.3	Co-existence studies

Co-existence studies of DC_1A-41A_n28A-n77A and DC_1A-41C_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.120.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1-41_n28-n77 are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 7.120.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n77 | 1 | 0.6 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.120.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n77 | 1 | 0.2 |
|  | 41 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.120.5	MSD

All MSD requirements for DC_1-41_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.121	DC_1-41_n28-n78

### 7.121.1	Operating bands for DC

Table 7.121.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n28A-n78A DC_1A-41C_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.121.2	Channel bandwidths per operating band for DC

Table 7.121.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.121.3	Co-existence studies

Co-existence studies of DC_1A-41A_n28A-n78A and DC_1A-41C_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.121.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1-41_n28-n78 are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 7.121.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.121.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n78 | 1 | 0 |
|  | 41 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.121.5	MSD

All MSD requirements for DC_1-41_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.122	DC_3-41_n28-n77

### 7.122.1	Operating bands for DC

Table 7.122.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-41A_n28A-n77A DC_3A-41C_n28A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.122.2	Channel bandwidths per operating band for DC

Table 7.122.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.122.3	Co-existence studies

Co-existence studies of DC_3A-41A_n28A-n77A and DC_3A-41C_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.122.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_3-41_n28-n77 are given in the tables below.

Table 7.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n77 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 2: Applicable for the frequency range of 2496 – 2515 MHz. |  |  |

Table 7.122.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n77 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 2: Applicable for the frequency range of 2496 – 2515 MHz. |  |  |

### 7.122.5	MSD

All MSD requirements for DC_3-41_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.123	DC_3-41_n28-n78

### 7.123.1	Operating bands for DC

Table 7.123.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-41A_n28A-n78A DC_3A-41C_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.123.2	Channel bandwidths per operating band for DC

Table 7.123.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.123.3	Co-existence studies

Co-existence studies of DC_3A-41A_n28A-n78A and DC_3A-41C_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.123.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_3-41_n28-n78 are given in the tables below that reused the values for DC_3-28-41-n78.

Table 7.123.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n78 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 2: Applicable for the frequency range of 2496 – 2515 MHz. |  |  |

Table 7.123.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n78 | 3 | 0.5 |
|  | 41 | 0.41/0.52 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 2: Applicable for the frequency range of 2496 – 2515 MHz. |  |  |

### 7.123.5	MSD

All MSD requirements for DC_3-41_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.124	DC_18-41_n3-n77

### 7.124.1	Operating bands for DC

Table 7.124.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-41A_n3A-n77A DC_18A-41C_n3A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.124.2	Channel bandwidths per operating band for DC

Table 7.124.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.124.3	Co-existence studies

Co-existence studies of DC_18A-41A_n3A-n77A and DC_18A-41C_n3A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.124.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_18-41_n3-n77 are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 7.124.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3-n77 | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.124.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3-n77 | 18 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.124.5	MSD

All MSD requirements for DC_18-41_n3-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.125	DC_18-41_n3-n78

### 7.125.1	Operating bands for DC

Table 7.125.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A-41A_n3A-n78A DC_18A-41C_n3A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.125.2	Channel bandwidths per operating band for DC

Table 7.125.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  |  | 41 | 15 | See CA-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.125.3	Co-existence studies

Co-existence studies of DC_18A-41A_n3A-n78A and DC_18A-41C_n3A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.125.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_18-41_n3-n78 are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 7.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3-n78 | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.125.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3-n78 | 18 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.125.5	MSD

All MSD requirements for DC_18-41_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.126	DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7-7_n1-n257

7.126.1	Operating bands for DC

Table 7.126.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n257 DC_3-3-7_n1-n257 DC_3-7-7_n1-n257 DC_3-3-7-7_n1-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.126.2	Channel bandwidths per operating band for DC

Table 7.126.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n1A-n257A | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 460 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A_n1A-n257A | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  | 480 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-7A_n1A-n257A | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 480 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A-7A_n1A-n257A | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  | 500 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes | Yes | Yes | Yes |  |

7.126.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.126.4	∆TIB and ∆RIB values

For DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7-7_n1-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n257 DC_3-3-7_n1-n257 DC_3-7-7_n1-n257 DC_3-3-7-7_n1-n257 | 3 | 0.3 |
|  | 7 | 0.6 |
|  | n1 | 0.5 |
|  | n257 | 0 |

Table 7.126.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n257 DC_3-3-7_n1-n257 DC_3-7-7_n1-n257 DC_3-3-7-7_n1-n257 | 3 | 0 |
|  | 7 | 0 |
|  | n1 | 0 |
|  | n257 | 0 |

7.126.5	MSD

No additional MSD requirement is needed.

7.127	DC_1-3_n40-n78

7.127.1	Operating bands for DC

Table 7.127.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.127.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00, TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.127.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.127.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n40-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | n40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.127.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n40-n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

7.127.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.128	DC_1-28_n40-n78

7.128.1	Operating bands for DC

Table 7.128.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.128.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00, TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.128.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.128.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28_n40-n78 | 1 | 0.5 |
|  | 28 | 0.5 |
|  | n40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 7.128.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28_n40-n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

7.128.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.129	DC_3-28_n40-n78

7.129.1	Operating bands for DC

Table 7.129.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.129.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00, TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

7.129.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 7.129.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n40-n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | n40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 7.129.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n40-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

7.129.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

7.130	DC_1-7_n7-n78

7.130.1	Operating bands for DC

Table 7.130.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.130.2	Channel bandwidths per operating band for DC

Table 7.130.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.130.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.130.4	∆TIB and ∆RIB values

For DC_1-7_n7-n78 the TIB,c and RIB,c values are same as for CA_n1-n7-n78 and are given in the tables below.

Table 7.130.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n7-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 7.130.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n7-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

7.130.5	MSD

Since DC_1_n7 UL gives IMD4 towards n78 DL, DC_1_n78 UL gives IMD4 towards n7 DL and DC_7_n78 UL gives IMD4 towards 1 DL, MSD needs to be defined as it is for DC_1A-7A_n78A and DC_1A-7A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n7A-n78A | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7, n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7, n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7, n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |

7.131	DC_3-7_n7-n78

7.131.1	Operating bands for DC

Table 7.131.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n7-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.131.2	Channel bandwidths per operating band for DC

Table 7.131.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A | 3 | See CA_3A-3A in Table 5.6A.1-3 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A | 3 | See CA_3C in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.131.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.131.4	∆TIB and ∆RIB values

For DC_3-7_n7-n78 the TIB,c and RIB,c values are same as for DC_3_n7-n78 and are given in the tables below.

Table 7.131.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n7-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 7.131.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n7-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

7.131.5	MSD

Since DC_3_n7 UL gives IMD3 towards n78 DL and DC_7_n78 UL gives IMD3 and IMD4 towards 3 DL, MSD needs to be defined as it is for DC_3A-7A_n78A and DC_3A-7A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-7A_n7A-n78A DC_3A-3A-7A_n7A-n78A DC_3C-7A_n7A-n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7, n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |

7.132	DC_7-28_n7-n78

7.132.1	Operating bands for DC

Table 7.132.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n7-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.132.2	Channel bandwidths per operating band for DC

Table 7.132.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.132.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

7.132.4	∆TIB and ∆RIB values

For DC_7-28_n7-n78 the TIB,c and RIB,c values are same as for DC_7-28_n78 and are given in the tables below.

Table 7.132.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n7-n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |

Table 7.132.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n7-n78 | 7 | 0 |
|  | 28 | 0 |
|  | n7 | 0 |
|  | n78 | 0.5 |

7.132.5	MSD

Since DC_28_n7 UL gives IMD2 towards n78 DL, DC_7_n78 UL gives IMD2 and IMD5 towards 28 DL and DC_28_n78 UL gives IMD2 towards n7 DL, MSD needs to be defined as it is for DC_7A-28A_n78A and DC_7A_n28A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-28A_n7A-n78A | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | 7, n7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |
|  | 7, n7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |
|  | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |

## 7.133	DC_1-41_n3-n257

### 7.133.1	Operating bands for DC

Table 7.133.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n3A-n257A DC_1A-41C_n3A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 7.133.2	Channel bandwidths per operating band for DC

Table 7.133.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  |  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  |  | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n3A-n257A | DC_41A_n3A DC_41A_n257A |  |  | 1 |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 470 |
|  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n3A-n257A | DC_41A_n3A DC_41A_n257A DC_41C_n3A DC_41C_n257A |  |  | 1 |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 490 |
|  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I |  |  | 1 |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 470 |
|  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257I |  |  | 1 |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 490 |
|  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n3 |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.133.3	Co-existence studies

Co-existence studies of DC_1-41_n3-n257 and protected bands are already covered in the constituent fall-back modes.

### 7.133.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1-41_n3-n257 are given in the tables below, numbers come from  DC_1-41_n3.

Table 7.133.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n257 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.133.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n257 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.133.5	MSD

All MSD requirements for DC_1-41_n3-n257 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.134	DC_3-28_n77-n257

7.134.1	Operating bands for DC

Table 7.134.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.134.2	Channel bandwidths per operating band for DC

Table 7.134.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | UL Configurations |  | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-28A_n77A-n257A |  |  | DC_3A_n77A DC_3A_n257A DC_28A_n77A DC_28A_n257A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77A_n257A in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77A-n257D |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_28A_n77A DC_28A_n257A DC_28A_n257D |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77A_n257D in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77A-n257G |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_28A_n77A DC_28A_n257A DC_28A_n257G |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77A_n257G in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77A-n257H |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77A_n257H in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77A-n257I |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77A_n257I in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77(2A)-n257A |  |  | DC_3A_n77A DC_3A_n257A DC_28A_n77A DC_28A_n257A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 640 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77(2A)_n257A in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77(2A)-n257D |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_28A_n77A DC_28A_n257A DC_28A_n257D |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 640 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77(2A)_n257D in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77(2A)-n257G |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_28A_n77A DC_28A_n257A DC_28A_n257G |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77(2A)_n257G in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77(2A)-n257H |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77(2A)_n257H in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A_n77(2A)-n257I |  |  | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 640 |
|  |  |  |  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n77 | See CA_n77(2A)_n257I in Table 5.5A1-1 in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.134.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-28_n77, DC_3-28_n257, DC_3_n77-n257 and DC_28_n77-n257. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.134.4	∆TIB and ∆RIB values

For DC_3-28_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.134.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n77-n257 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.134.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n77-n257 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.134.5	MSD

As mentioned in 7.134.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.135	DC_3-20_n1-n7

### 7.135.1	Operating bands for DC

Table 7.135.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_30-20_n1-n7 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

### 7.135.2	Channel bandwidths per operating band for DC

Table 7.135.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A | 3 | 15 | See CA_3A-20A BCS in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 20 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A | 3 | 15 | See CA_3C-20A BCS in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 20 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |

### 7.135.3	Co-existence studies

The harmonics and intermodulation studies have been already made in the fallback EN-DC, DC_20A_n1A-n7A, DC_3A_n1A-n7A, DC_3A-20A_n7A and DC_3A-20A_n1A.

### 7.135.4	∆TIB and ∆RIB values

For DC_3-20_n1-n7, the TIB,c and RIB values can be based on E-UTRA CA_1-3-7-20 given in the tables below.

Table 7.135.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n7 | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n7 | 0.6 |

Table 7.135.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n1-n7 | 3 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |
|  | n7 | 0 |

### 7.135.5	MSD

## All MSD issues are covered in 3DL/2UL and 2DL/2UL fallback modes.

7.136	DC_1-3_n28-n77

7.136.1	Operating bands for DC

Table 7.136.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.136.2	Channel bandwidths per operating band for DC

Table 7.136.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 260 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.136.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3_n28, DC_1-3_n77, DC_1_n28-n77 and DC_3_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.136.4	∆TIB and ∆RIB values

For DC_1-3_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.136.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.136.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.136.5	MSD

As mentioned in 7.136.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.137	DC_1-8_n28-n77

7.137.1	Operating bands for DC

Table 7.137.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.137.2	Channel bandwidths per operating band for DC

Table 7.137.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-8A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 250 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.137.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n28, DC_1-8_n77, DC_1_n28-n77 and DC_8_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.137.4	∆TIB and ∆RIB values

For DC_1-8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.137.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.137.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.137.5	MSD

As mentioned in 7.137.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.138	DC_3-8_n28-n77

7.138.1	Operating bands for DC

Table 7.138.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n28-n77 | 3 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

7.138.2	Channel bandwidths per operating band for DC

Table 7.138.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-8A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 150 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-8A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 250 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.138.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-8_n28, DC_3-8_n77, DC_3_n28-n77 and DC_8_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.138.4	∆TIB and ∆RIB values

For DC_3-8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.138.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n28-n77 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.138.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n28-n77 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

7.138.5	MSD

As mentioned in 7.138.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.139		DC_3-20_n38-n78

7.139.1		Operating bands for DC

Table 7.139.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20_n38-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.139.2	Channel bandwidths per operating band for DC

Table 7.139.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A | 3 | Please see BCS0 of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.139.3	Co-existence studies

Co-existence studies of this DL of LTE 2 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively, which means the harmonic and IMD products caused by DC_3-20_n38-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

7.139.4	∆TIB and ∆RIB values

For DC_3-20_n38-n78, the TIB,c and RIB values are given in the tables below.

Table 7.139.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n38-n78 | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 7.139.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n38-n78 | 3 | 0.2 |
|  | 20 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

7.139.5	MSD

All MSD requirements for DC_3-20_n38-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.140	DC_2-66_n38-n78

7.140.1	Operating bands for DC

Table 7.140.1-1: DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n38-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

7.140.2	Channel bandwidths per operating band for DC

Table 7.140.2-1: Supported bandwidths per DC band combination of one LTE band + NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 180 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.140.3	Co-existence studies

The harmonic and IMD products caused by DC_2A-66A_n38A-n78A and protected bands follow from those derived for the constituent fallback modes.

7.140.4	∆TIB and ∆RIB values

For DC_2-66_n38-n78, the ∆TIB,c and ∆RIB values are given in the tables below.

Table 7.140.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n38-n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n38 | 0.9 |
|  | n78 | 0.8 |

Table 7.140.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n38-n78 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n78 | 0.5 |

7.140.5	MSD

No further MSD is needed.

7.141		DC_1-3_n41-n78

### 7.141.1	 Operating bands for DC

Table 7.141.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz |  | 1785 MHz | 1805 MHz |  | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.141.2 	Configuration for DC

Table 7.141.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n41A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.141.3	 Co-existence studies

The harmonic and IMD products caused by DC_1A-3A_n41A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

### 7.141.4	 ∆TIB and ∆RIB values

Table 7.141.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n41-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 7.141.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n41-n78 | 1 | 0 |
|  | 3 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 7.141.5	MSD

All MSD requirements for DC_1A-3A_n41A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.142		DC_1-20_n41-n78

### 7.142.1	 Operating bands for DC

Table 7.142.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-20A_n41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.142.2 	Configuration for DC

Table 7.142.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n41A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.142.3	 Co-existence studies

The harmonic and IMD products caused by DC_1A-20A_n41A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

### 7.142.4	 ∆TIB and ∆RIB values

Table 7.142.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n41-n78 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 7.142.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n41-n78 | 1 | 0 |
|  | 20 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 7.142.5	MSD

All MSD requirements for DC_1A-20A_n41A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.143		DC_3-20_n41-n78

### 7.143.1	 Operating bands for DC

Table 7.143.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n41A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.143.2 	Configuration for DC

Table 7.143.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n41A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.143.3	 Co-existence studies

The harmonic and IMD products caused by DC_3A-20A_n41A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

### 7.143.4	 ∆TIB and ∆RIB values

Table 7.143.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n41-n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 7.143.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n41-n78 | 3 | 0 |
|  | 20 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 7.143.5	MSD

All MSD requirements for DC_3A-20A_n41A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.144		DC_1-7_n3-n78

### 7.144.1	 Operating bands for DC

Table 7.144.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n3A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.144.2 	Configuration for DC

Table 7.144.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n3A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 170 |
|  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.144.3	 Co-existence studies

The harmonic and IMD products caused by DC_1A-7A_n3A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

### 7.144.4	 ∆TIB and ∆RIB values

Table 7.144.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n3-n78 | 1 | 0.5 |
|  | 7 | 0.2 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.144.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n3-n78 | 1 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |
|  | n78 | 0.5 |

### 7.144.5	MSD

All MSD requirements for DC_1A-7A_n3A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.145		DC_3-20_n7-n28

### 7.145.1	 Operating bands for DC

Table 7.145.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n7A-n28A6 DC_3C-20A_n7A-n28A6 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. |  |  |  |  |  |  |  |  |

### 7.145.2 	Configuration for DC

Table 7.145.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n7A-n28A |  | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 120 |
|  |  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  | Yes7 |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes7 |  | Yes7 |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C-20A_n7A-n28A |  | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_3C_n28A DC_20A_n7A DC_20A_n28A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes7 |  | Yes7 |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes7 |  | Yes7 |  |  |  |  |  |  |  |

### 7.145.3	 Co-existence studies

The harmonic and IMD products caused by DC_3A-20A_n7A-n28A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

### 7.145.4	 ∆TIB and ∆RIB values

Table 7.145.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n7-n28 | 3 | 0.5 |
|  | 20 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.5 |

Table 7.145.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n7-n28 | 3 | 0 |
|  | 20 | 0.1 |
|  | n7 | 0 |
|  | n28 | 0.1 |

### 7.145.5	MSD

All MSD requirements for DC_3A-20A_n7A-n28A are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.146	DC_1-3_n28-n257

7.146.1	Operating bands for DC

Table 7.146.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n28-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.146.2	Channel bandwidths per operating band for DC

Table 7.146.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1-3A_n28A-n257A | DC_1_n28A DC_1-n257A DC_3_n28A DC_3-n257A | 1 | Please see BCS of CA_1A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-3A_n28A-n257G | DC_1_n28A DC_1-n257A DC_1-n257G DC_3_n28A DC_3-n257A DC_3-n257G | 1 | Please see BCS of CA_1A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-3A_n28A-n257H | DC_1_n28A DC_1-n257A DC_1-n257G DC_1-n257H DC_3_n28A DC_3-n257A DC_3-n257G DC_3-n257H | 1 | Please see BCS of CA_1A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-3A_n28A-n257I | DC_1_n28A DC_1-n257A DC_1-n257G DC_1-n257H DC_1-n257I DC_3_n28A DC_3-n257A DC_3-n257G DC_3-n257H DC_3-n257I | 1 | Please see BCS of CA_1A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.146.3	Co-existence studies

Based on the co-existence studies of DC_1_n28A,  DC_1_n257A, DC_3_n28A and  DC_3_n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.146.4	∆TIB and ∆RIB values

For DC_1-3_n28-n257, the TIB,c and RIB,c values coming from the values for DC_1-3_n28 are given in the tables below.

Table 7.146.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
|  | n257 | 0 |

Table 7.146.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n257 | 1 | 0 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | n257 | 0 |

7.146.5	MSD

As mentioned in 7.146.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.147	DC_1-41_n28-n257

7.147.1	Operating bands for DC

Table 7.147.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-41_n28-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.147.2	Channel bandwidths per operating band for DC

Table 7.147.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n28A-n257A | DC_1A_n28A DC_1A_n257A DC_41A_n28A DC_41A_n257A | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n28A-n257G | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_41A_n28A DC_41A_n257A DC_41A_n257G | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n28A-n257H | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n28A-n257A | DC_1A_n28A DC_1A_n257A DC_41A_n28A DC_41A_n257A DC_41C_n28A DC_41C_n257A | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n28A-n257G | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41C_n28A DC_41C_n257A DC_41C_n257G | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n28A-n257H | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.147.3	Co-existence studies

Based on the co-existence studies of DC_1_n28A, DC_1_n257A and DC_41_n257A captured in TR 37.863-01-01 and DC_41_n28A captured in TR 37.716-11-11,  there is no additional harmonic and intermodulation impact for the additional band receiver.

7.147.4	∆TIB and ∆RIB values

For DC_1-41_n28-n257, the TIB,c and RIB,c values coming from the values for DC_1-41_n28 are given in the tables below.

Table 7.147.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
|  | n257 | 0 |

Table 7.147.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41_n28-n257 | 1 | 0 |
|  | 41 | 0 |
|  | n28 | 0.2 |
|  | n257 | 0 |

7.147.5	MSD

As mentioned in 7.147.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.148	DC_1-41_n77-n257

7.148.1	Operating bands for DC

Table 7.148.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-41_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.148.2	Channel bandwidths per operating band for DC

Table 7.148.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 1 | Please see BCS of CA_1A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 1 | Please see BCS of CA_1A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.148.3	Co-existence studies

Based on the co-existence studies of DC_1_n77A, DC_41_n77A, DC_1_n257A and DC_41_n257A captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.148.4	∆TIB and ∆RIB values

For DC_1-41_n77-n257, the TIB,c and RIB,c values coming from the values for DC_1-41_n77 are given in the tables below.

Table 7.148.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n77-n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.148.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n77-n257 | 1 | 0 |
|  | 41 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.148.5	MSD

As mentioned in 7.148.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.149	DC_3-41_n28-n257

7.149.1	Operating bands for DC

Table 7.149.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41_n28-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.149.2	Channel bandwidths per operating band for DC

Table 7.149.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n28A-n257A | DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A_n28A-n257G | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_41A_n28A DC_41A_n257A DC_41A_n257G | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A_n28A-n257H | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41A_n28A-n257I | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n28A-n257A | DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41C_n28A DC_41C_n257A | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n28A-n257G | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41C_n28A DC_41C_n257A DC_41C_n257G | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n28A-n257H | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n28A-n257I | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of CA_n28A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.149.3	Co-existence studies

Based on the co-existence studies of DC_3_n28, DC_3_n257 and DC_41_n257 captured in TR 37.863-01-01 and DC_41_n28 captured in TR 37.716-11-11, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.149.4	∆TIB and ∆RIB values

For DC_3-41_n28-n257, the TIB,c and RIB,c values coming from the values for DC_3-41_n28 are given in the tables below.

Table 7.149.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n257 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.3 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.149.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n28-n257 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.149.5	MSD

As mentioned in 7.149.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.150	DC_3-41_n77-n257

7.150.1	Operating bands for DC

Table 7.150.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.150.2	Channel bandwidths per operating band for DC

Table 7.150.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 3 | Please see BCS of CA_3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3-41C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 3 | Please see BCS of CA_3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.150.3	Co-existence studies

Based on the co-existence studies of DC_3_n77, DC_3_n257, DC_41_n77 and DC_41_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.150.4	∆TIB and ∆RIB values

For DC_3-41_n77-n257, the TIB,c and RIB,c values coming from the values for DC_3-41_n77 are given in the tables below.

Table 7.150.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n77-n257 | 3 | 0..6 |
|  | 41 | 0.31/0.82 |
|  | n77 | 0.8 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.150.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41_n77-n257 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n77 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

7.150.5	MSD

As mentioned in 7.150.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.151	DC_18-41_n3-n257

7.151.1	Operating bands for DC

Table 7.151.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18-41_n3-n257 | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.151.2	Channel bandwidths per operating band for DC

Table 7.151.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_18A-41A_n3A-n257A | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41A_n257A | 18 | Please see BCS of CA_18A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A-41A_n3A-n257G | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_41A_n3A DC_41A_n257A DC_41A_n257G | 18 | Please see BCS of CA_18A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18A-41A_n3A-n257H | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 18 | Please see BCS of CA_18A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18-41A_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 18 | Please see BCS of CA_18A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18-41C_n3A-n257A | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41A_n257A DC_41C_n3A DC_41C_n257A | 18 | Please see BCS of CA_18A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18-41C_n3A-n257G | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41C_n3A DC_41C_n257A DC_41C_n257G | 18 | Please see BCS of CA_18A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18-41C_n3A-n257H | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 18 | Please see BCS of CA_18A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_18-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 18 | Please see BCS of CA_18A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of CA_n3A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.151.3	Co-existence studies

Based on the co-existence studies of DC_18_n257A and DC_41_n257A captured in TR 37.863-01-01 and DC_18_n3A and DC_41_n3A captured in TR 37.716-11-11, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.151.4	∆TIB and ∆RIB values

For DC_18-41_n3-n257, the TIB,c and RIB,c values coming from the values for DC_18-41_n3 are given in the tables below.

Table 7.151.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3-n257 | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n257 | 0 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 7.151.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n3-n257 | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n257 | 0 |
| NOTE 1: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 2: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

7.151.5	MSD

As mentioned in 7.151.3, there is no need to specify additional MSD requirement for this UL DC configuration.

7.152	 DC_41-42_n77-n257

7.152.1	Operating bands for DC

Table 7.152.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41-42_n77-n257 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.152.2	Channel bandwidths per operating band for DC

Table 7.152.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_41A-42A_n77A-n257A | DC_41A_n77A DC_41A_n257A DC_42A_n257A | 41 | Please see BCS of CA_41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42A_n77A-n257G | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 41 | Please see BCS of CA_41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 340 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42A_n77A-n257H | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 41 | Please see BCS of CA_41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 440 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42A_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 41 | Please see BCS of CA_41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 540 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42C_n77A-n257A | DC_41A_n77A DC_41A_n257A DC_42A_n257A DC_42C_n257A | 41 | Please see BCS of CA_41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42C_n77A-n257G | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 41 | Please see BCS of CA_41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42C_n77A-n257H | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 41 | Please see BCS of CA_41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A-42C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 41 | Please see BCS of CA_41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42A_n77A-n257A | DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A | 41 | Please see BCS of CA_41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42A_n77A-n257G | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G | 41 | Please see BCS of CA_41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42A_n77A-n257H | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 41 | Please see BCS of CA_41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42A_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 41 | Please see BCS of CA_41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42C_n77A-n257A | DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A DC_42C_n257A | 41 | Please see BCS of CA_41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42C_n77A-n257G | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 41 | Please see BCS of CA_41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42C_n77A-n257H | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 41 | Please see BCS of CA_41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41C-42C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 41 | Please see BCS of CA_41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.152.3	Co-existence studies

Based on the co-existence studies of DC_41_n77, DC_41_n257 and DC_42_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.152.4	∆TIB and ∆RIB values

For DC_41-42_n77-n257, the TIB,c and RIB,c values coming from the values for DC_41-42_n77 are given in the tables below.

Table 7.152.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41-42_n77-n257 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 7.152.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41-42_n77-n257 | 41 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

7.152.5	MSD

As mentioned in 7.152.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8	EN-DC band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

8.1	DC_1A-3A-5A_n78A-n257

8.1.1	Operating bands for DC

Table 8.1.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-5A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.1.2	Channel bandwidths per operating band for DC

Table 8.1.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-5A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 650 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 850 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.1.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.1.4	∆TIB and ∆RIB values

For DC_1A-3A-5A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-5A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-5A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.1.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.2	DC_1A-3A-7A_n78A-n257

8.2.1	Operating bands for DC

Table 8.2.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.2.2	Channel bandwidths per operating band for DC

Table 8.2.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 660 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 760 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 860 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 960 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.2.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.2.4	∆TIB and ∆RIB values

For DC_1A-3A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n78-n257 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n78-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.2.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.3	DC_1A-3A-7A-7A_n78A-n257

8.3.1	Operating bands for DC

Table 8.3.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.3.2	Channel bandwidths per operating band for DC

Table 8.3.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-7A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 780 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 980 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 380 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 480 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 680 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 780 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 880 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 980 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.3.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.3.4	∆TIB and ∆RIB values

For DC_1A-3A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-7A_n78-n257 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-7A_n78-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.3.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.4	DC_1A-5A-7A_n78A-n257

8.4.1	Operating bands for DC

Table 8.4.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.4.2	Channel bandwidths per operating band for DC

Table 8.4.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-5A-7A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 650 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 850 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.4.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.4.4	∆TIB and ∆RIB values

For DC_1A-5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A-7A_n78-n257 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A-7A_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.4.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.5	DC_1A-5A-7A-7A_n78A-n257

8.5.1	Operating bands for DC

Table 8.5.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-5A-7A7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.5.2	Channel bandwidths per operating band for DC

Table 8.5.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-5A-7A-7A_n78A-n257D | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257E | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257F | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257G | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 370 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257H | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 470 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257I | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257J | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 670 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257K | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257L | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 870 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.5.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.5.4	∆TIB and ∆RIB values

For DC_1A-5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-5A-7A-7A_n78-n257 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-5A-7A-7A_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.5.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.6	DC_3A-5A-7A_n78A-n257

8.6.1	Operating bands for DC

Table 8.6.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.6.2	Channel bandwidths per operating band for DC

Table 8.6.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-5A-7A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 350 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 450 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 550 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 650 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 750 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 850 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 950 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.6.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.6.4	∆TIB and ∆RIB values

For DC_3A-5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A-7A_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A-7A_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.6.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.7	DC_3A-5A-7A-7A_n78A-n257

8.7.1	Operating bands for DC

Table 8.7.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-5A-7A-7A_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.7.2	Channel bandwidths per operating band for DC

Table 8.7.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-5A-7A-7A_n78A-n257D | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257E | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257F | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257G | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 370 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257H | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 470 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257I | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257J | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 670 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257K | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257L | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 870 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-5A-7A-7A_n78A-n257M | DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.7.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

8.7.4	∆TIB and ∆RIB values

For DC_3A-5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-5A-7A-7A_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-5A-7A-7A_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.7.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

8.8	DC_1-3-7_n28-n78

8.8.1	Operating bands for DC

Table 8.8.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.8.2	Channel bandwidths per operating band for DC

Table 8.8.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7C_n28A DC_7C_n78A DC_7A_n28A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.8.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7_n28-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 37.716-21-21, Table 5.3-2.

8.8.4	∆TIB and ∆RIB values

For DC_1-3-7_n28-n78, the TIB,c and RIB values are given in the tables below.

Table 8.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

8.8.5	MSD

All MSD requirements for DC_1-3-7_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.9	DC_1-19-42_n77-n79

8.9.1	Operating bands for DC

Table 8.9.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.9.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.9.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A-42A_n77A-n79A DC_1A-19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.9.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.9.4	∆TIB and ∆RIB values

For DC_1-19-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n77-n79 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 8.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

8.9.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed

8.10	DC_1-19-42_n78-n79

8.10.1	Operating bands for DC

Table 8.10.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.10.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.10.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-19A-42A_n78A-n79A DC_1A-19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.10.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.10.4	∆TIB and ∆RIB values

For DC_1-19-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 8.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

8.10.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.11	DC_1-21-42_n77-n79

8.11.1	Operating bands for DC

Table 8.11.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.11.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.11.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A-42A_n77A-n79A DC_1A-21A-42C_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.11.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.11.4	∆TIB and ∆RIB values

For DC_1-21-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n77-n79 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 8.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n77-n79 | 1 | 0.2 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |

8.11.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.12	DC_1-21-42_n78-n79

8.12.1	Operating bands for DC

Table 8.12.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.12.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.12.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-21A-42A_n78A-n79A DC_1A-21A-42C_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 275 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.12.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.12.4	∆TIB and ∆RIB values

For DC_1-21-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n78-n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 8.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n78-n79 | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |

8.12.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.13	DC_1-3-21_n77-n79

8.13.1	Operating bands for DC

Table 8.13.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.13.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.13.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.13.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.13.4	∆TIB and ∆RIB values

For DC_1-3-21_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n77-n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |

Table 8.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n77-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |

8.13.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.14	DC_1-3-21_n78-n79

8.14.1	Operating bands for DC

Table 8.14.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n78-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.14.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.14.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.14.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.14.4	∆TIB and ∆RIB values

For DC_1-3-21_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n78-n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |

Table 8.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n78-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |

8.14.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.15	DC_19-21-42_n77-n79

8.15.1	Operating bands for DC

Table 8.15.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n77-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.15.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.15.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A-42A_n77A-n79A DC_19A-21A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.15.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.15.4	∆TIB and ∆RIB values

For DC_19-21-42_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n77-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |

Table 8.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |

8.15.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.16	DC_19-21-42_n78-n79

8.16.1	Operating bands for DC

Table 8.16.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n78-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

8.16.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.16.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_19A-21A-42A_n78A-n79A DC_19A-21A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  |  |  |  |  |  |  |

8.16.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.16.4	∆TIB and ∆RIB values

For DC_19-21-42_n78-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n78-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |

Table 8.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |

8.16.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. Therefore, additional analysis is not needed.

8.17	DC_1-7-28_n5-n78

8.17.1	Operating bands for DC

Table 8.17.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.17.2	Channel bandwidths per operating band for DC

Table 8.17.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-7A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.17.3	Co-existence studies

The harmonic and IMD products caused by DC_1-7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.17.4	∆TIB and ∆RIB values

For DC_1-7-28_n5-n78, the TIB,c and RIB values are derived from DC_1-7-20_n28-n78 and are given in the tables below.

Table 8.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n5-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n5-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.17.5	MSD

All MSD requirements for DC_1-7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.18	DC_3-7-28_n5-n78

8.18.1	Operating bands for DC

Table 8.18.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n5-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.18.2	Channel bandwidths per operating band for DC

Table 8.18.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_3A-7A-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7A-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3A-7C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.18.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.18.4	∆TIB and ∆RIB values

For DC_3-7-28_n5-n78, the TIB,c and RIB values are derived from DC_3-7-20_n28-n78 and are given in the tables below.

Table 8.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n5-n78 | 3 | 0.6 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n5-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.18.5	MSD

All MSD requirements for DC_3-7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.19	DC_1-3-7_n5-n78

8.19.1	Operating bands for DC

Table 8.19.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.19.2	Channel bandwidths per operating band for DC

Table 8.19.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A-7A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3C_n5A DC_3C_n78A DC_7A_n5A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7C_n5A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3C_n5A DC_3C_n78A DC_7C_n5A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.19.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.19.4	∆TIB and ∆RIB values

For DC_1-3-7_n5-n78, the TIB,c and RIB values are same as for DC_1-3-5-7_n78 and are given in the tables below.

Table 8.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n5-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.19.5	MSD

All MSD requirements for DC_1-3-7_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.20	DC_1-3-28_n5-n78

8.20.1	Operating bands for DC

Table 8.20.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.20.2	Channel bandwidths per operating band for DC

Table 8.20.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3C_n5A DC_3C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 120 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

8.20.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

8.20.4	∆TIB and ∆RIB values

For DC_1-3-28_n5-n78, the TIB,c and RIB values are derived from DC_1-3-20_n28-n78 and are given in the tables below.

Table 8.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n5-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 8.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

8.20.5	MSD

All MSD requirements for DC_1-3-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.21	DC_1-3-8-n78-n257

8.21.1	Operating bands for DC

Table 8.21.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

8.21.2	Channel bandwidths per operating band for DC

Table 8.21.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 |  | 15 |  | 20 |  | 40 |  | 50 | 60 |  | 80 |  | 90 | 100 |  | 200 |  | 400 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-8A_n78A-n257A |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_1A-3A-8A_n78A-n257D |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257E |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257F |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257G |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257H |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257I |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257J |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 650 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257K |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 750 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257L |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 850 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-8A_n78A-n257M |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 950 |  |
|  |  |  | 3 | 15 | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  | Yes |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257A |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |
| DC_1A-3C-8A_n78A-n257D |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257E |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 770 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257F |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 970 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257G |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 370 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257H |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257I |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257J |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 670 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257K |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 770 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257L |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 870 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3C-8A_n78A-n257M |  | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_1A_n257A DC_3A_n257A DC_8A_n257A | 1 | 15 |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 970 |  |
|  |  |  | 3 |  |  | See CA_3C in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 8 | 15 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.21.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL, 3DL/2UL and 4UL/2UL captured in TR 37.863-01-01, TR 37.863-02-01, TR 37.863-03-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

8.21.4	∆TIB and ∆RIB values

For DC_1-3-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.21.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.21.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.22		DC_1-7-20_n3-n78

8.22.1		Operating bands for DC

Table 8.22.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-20_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz |  | 862 MHz | 791 MHz |  | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.22.2	Channel bandwidths per operating band for DC

Table 8.22.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.22.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. which means the harmonic and IMD products caused by DC_1-7-20_n3-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

8.22.4	∆TIB and ∆RIB values

For DC_1-7-20_n3-n78, the TIB,c and RIB values are given in the tables below.

Table 8.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20_n3-n78 | 1 | 0.3 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 8.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-20_n3-n78 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |
|  | n78 | 0.5 |

8.22.5	MSD

All MSD requirements for DC_1-7-20_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.23	DC_3-7-8_n1-n78, DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78

8.23.1	Operating bands for DC

Table 8.23.1-1: DC band combination of three LTE bands + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.23.2	Channel bandwidths per operating band for DC

Table 8.23.2-1: Supported bandwidths per DC band combination of three LTE bands + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 170 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-3A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A | 3 | 15 | See CA_3A-3A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.23.3	Co-existence studies

Co-existence studies of Band 3 + Band n78, Band 7 + Band n78 and Band 8 + Band n78 are captured in TR 37.863-01-01.

Co-existence studies of Band 3 + Band n1, Band 7 + Band n1 and Band 8 + Band n1 are captured in TR 37.716-11-11.

Own Rx impact on the 3rd band:

- 2nd and 4th order IMD generated by dual uplink of Band 3 + Band n1 may fall into own Rx of band n78.

- 5th order IMD generated by dual uplink of Band 3 + Band n78 may fall into own Rx of band n1.

- 4th and 5th order IMD generated by dual uplink of Band 7 + Band n1 may fall into own Rx of band n78.

- 5th order IMD generated by dual uplink of Band 7 + Band n1 may fall into own Rx of band 8.

- 2nd and 5th order IMD generated by dual uplink of Band 7 + Band n78 may fall into own Rx of band 8.

- 3rd and 4th order IMD generated by dual uplink of Band 7 + Band n78 may fall into own Rx of band 3.

- 4th order IMD generated by dual uplink of Band 7 + Band n78 may also fall into own Rx of band n1

- 3rd order IMD generated by dual uplink of Band 8 + Band n1 may fall into own Rx of band n78.

- 3rd order IMD generated by dual uplink of Band 8 + Band n78 may fall into own Rx of band 3.

- 2nd order IMD generated by dual uplink of Band 8 + Band n78 may fall into own Rx of band 7.

8.23.4	∆TIB and ∆RIB values

For DC_3-7-8_n1-n78, DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78, the TIB,c and RIB values are same as DC_1-3-7-8_n78 and are given in the tables below.

Table 8.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 8.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

8.23.5	MSD

IMD problems are already analysed in constituent low order combinations between LTE bands and NR bands. The MSD needed are covered in the constituent low order combinations.

8.24	DC_1-8-11_n77-n257

8.24.1	Operating bands for DC

Table 8.24.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.24.2	Channel bandwidths per operating band for DC

Table 8.24.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A-11A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |
| DC_1A-8A -11A_n77A-n257D | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A -11A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 340 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A -11A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 440 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A -11A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 540 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |

8.24.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n77, DC_1-8-11_n257, DC_1-8_n77-n257, DC_1-11_n77-n257 and DC_8-11_n77-n257. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.24.4	∆TIB and ∆RIB values

For DC_1-8-11_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n77-n257 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.24.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n77-n257 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.24.5	MSD

As mentioned in 8.24.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.25	DC_1A-3A-41A_n78A-n257

8.25.1	Operating bands for DC

Table 8.25.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.25.2	Channel bandwidths per operating band for DC

Table 8.25.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-41A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_41A_n78A DC_41A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-41A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.25.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.25.4	∆TIB and ∆RIB values

For DC_1-3-41_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.25.5	MSD

As mentioned in 8.25.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.26	DC_1A-3A-42A_n78A-n257

8.26.1	Operating bands for DC

Table 8.26.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.26.2	Channel bandwidths per operating band for DC

Table 8.26.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.26.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.26.4	∆TIB and ∆RIB values

For DC_1-3-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-42_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.26.5	MSD

As mentioned in 8.26.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.27	DC_1A-41A-42A_n78A-n257

8.27.1	Operating bands for DC

Table 8.27.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-41-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.27.2	Channel bandwidths per operating band for DC

Table 8.27.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-41A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_41A_n78A DC_41A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-41A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.27.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.27.4	∆TIB and ∆RIB values

For DC_1-41-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41-42_n78-n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-41-42_n78-n257 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.27.5	MSD

As mentioned in 8.27.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.28	DC_3A-41A-42A_n78A-n257

8.28.1	Operating bands for DC

Table 8.28.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41-42_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.28.2	Channel bandwidths per operating band for DC

Table 8.28.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-41A-42A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_41A_n78A DC_41A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-41A-42A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.28.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.28.4	∆TIB and ∆RIB values

For DC_3-41-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42_n78-n257 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 8.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-41-42_n78-n257 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

8.28.5	MSD

As mentioned in 8.28.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.29	DC_1A-3A-18A_n78A-n257

8.29.1	Operating bands for DC

Table 8.29.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.29.2	Channel bandwidths per operating band for DC

Table 8.29.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-18A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_18A_n78A DC_18A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 255 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-18A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_18A_n78A DC_18A_n257A DC_18A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 355 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-18A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 455 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.29.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.29.4	∆TIB and ∆RIB values

For DC_1-3-18_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.29.5	MSD

As mentioned in 8.29.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.30	DC_1A-18A-42A_n78A-n257

8.30.1	Operating bands for DC

Table 8.30.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-18-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.30.2	Channel bandwidths per operating band for DC

Table 8.30.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-18A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_18A_n78A DC_18A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 255 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-18A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 355 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 455 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 555 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.30.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.30.4	∆TIB and ∆RIB values

For DC_1-18-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18-42_n78-n257 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-18-42_n78-n257 | 1 | 0 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.30.5	MSD

As mentioned in 8.30.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.31	DC_3A-18A-42A_n78A-n257

8.31.1	Operating bands for DC

Table 8.31.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-18-42_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.31.2	Channel bandwidths per operating band for DC

Table 8.31.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-18A-42A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_18A_n78A DC_18A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 255 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-18A-42A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 355 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-18A-42A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 455 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-18A-42A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 555 |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.31.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.31.4	∆TIB and ∆RIB values

For DC_3-18-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18-42_n78-n257 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-18-42_n78-n257 | 3 | 0 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.31.5	MSD

As mentioned in 8.31.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.32	DC_3A-28A-41A_n78A-n257

8.32.1	Operating bands for DC

Table 8.32.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28-41_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.32.2	Channel bandwidths per operating band for DC

Table 8.32.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-28A-41A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_28A_n78A DC_28A_n257A DC_41A_n78A DC_41A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-28A-41A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A-41A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A-41A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.32.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.32.4	∆TIB and ∆RIB values

For DC_3-28-41_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28-41_n78-n257 | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 8.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28-41_n78-n257 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 0 1/0.52 |
|  | n78 | 0.5 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2:The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

8.32.5	MSD

As mentioned in 8.32.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.33	DC_3A-28A-42A_n78A-n257

8.33.1	Operating bands for DC

Table 8.33.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28-42_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.33.2	Channel bandwidths per operating band for DC

Table 8.33.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-28A-42A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_28A_n78A DC_28A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-28A-42A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A-42A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A-42A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.33.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.33.4	∆TIB and ∆RIB values

For DC_3-28-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28-42_n78-n257 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28-42_n78-n257 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.33.5	MSD

As mentioned in 8.33.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.34	DC_28A-41A-42A_n78A-n257

8.34.1	Operating bands for DC

Table 8.34.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28-41-42_n78-n257 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.34.2	Channel bandwidths per operating band for DC

Table 8.34.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_28A-41A-42A_n78A-n257A | DC_28A_n78A DC_28A_n257A DC_41A_n78A DC_41A_n257A DC_42A_n257A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_28A-41A-42A_n78A-n257G | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A-41A-42A_n78A-n257H | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_28A-41A-42A_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.34.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.34.4	∆TIB and ∆RIB values

For DC_28-41-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-41-42_n78-n257 | 28 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 8.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-41-42_n78-n257 | 28 | 0.2 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

8.34.5	MSD

As mentioned in 8.34.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.35	DC_1A-3A-28A_n78A-n257

8.35.1	Operating bands for DC

Table 8.35.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.35.2	Channel bandwidths per operating band for DC

Table 8.35.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-28A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_28A_n78A DC_28A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-28A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-28A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.35.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.35.4	∆TIB and ∆RIB values

For DC_1-3-28_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.35.5	MSD

As mentioned in 8.35.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.36	DC_1A-28A-42A_n78A-n257

8.36.1	Operating bands for DC

Table 8.36.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-28-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.36.2	Channel bandwidths per operating band for DC

Table 8.36.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-28A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_28A_n78A DC_28A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 260 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-28A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 360 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 460 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-28A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 560 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.36.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

8.36.4	∆TIB and ∆RIB values

For DC_1-28-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28-42_n78-n257 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-28-42_n78-n257 | 1 | 0 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.36.5	MSD

As mentioned in 8.36.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.37	DC_2-7-66_n66-n78 and DC_2-7-7-66_n66-n78

### 8.37.1	Operating bands for DC

Table 8.37.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2A-7A-66A_n66A-n78A DC_2A-7A-7A-66A_n66A-n78A | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.37.2	Channel bandwidths per operating band for DC

Table 8.37.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | UL Configuration |  |  | E-UTRA and NR Band |  | SCS (kHz) |  | 5 |  | 10 |  | 15 |  | 20 |  | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_2A-7A-66A_n66A-n78A DC_2A-7A-7A-66A_n66A-n78A |  |  | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A1 DC_66A_n78A |  |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 200 |  |
|  |  |  |  |  |  | 7 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_2A-7A-7A-66A_n66A-n78A |  |  | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A1 DC_66A_n78A |  |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 220 |  |
|  |  |  |  |  |  | 7 |  | 15 |  |  |  | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_2A-7C-66A_n66A-n78A |  | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A1 DC_66A_n78A |  |  | 2 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 220 |  |
|  |  |  |  |  | 7 |  | 15 |  | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n66 |  | 15 |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |
|  | NOTE1: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.37.3	Co-existence studies

The harmonic and IMD products caused by DC_2A-7A-66A_n66A-n78A, DC_2A-7C-66A_n66A-n78A and DC_2A-7A-7A-66A_n66A-n78A and protected bands follow from those derived for the constituent fallback modes.

### 8.37.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_2A-7A-66A_n66A-n78A, DC_2A-7C-66A_n66A-n78A and DC_2A-7A-7A-66A_n66A-n78A are given in the tables below.

Table 8.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 8.37.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 8.37.5	MSD

All MSD requirements for DC_2A-7A-66A_n66A-n78A, DC_2A-7C-66A_n66A-n78A and DC_2A-7A-7A-66A_n66A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.38		DC_1-20-38_n3-n78

8.38.1		Operating bands for DC

Table 8.38.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-20-38_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.38.2	Channel bandwidths per operating band for DC

Table 8.38.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 (MHz) | 10 (MHz) | 15 (MHz) | 20 (MHz) | 25 (MHz) | 30 (MHz) | 40 (MHz) | 50 (MHz) | 60 (MHz) | 80 (MHz) | 90 (MHz) | 100 (MHz) | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_38A_n3A DC_1A_n78A DC_20A_n78A DC_38A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 190 |
|  |  | 20 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS0 of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.38.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. which means the harmonic and IMD products caused by DC_1-20-38_n3-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

8.38.4	∆TIB and ∆RIB values

For DC_1-20-38_n3-n78, the TIB,c and RIB values are given in the tables below.

Table 8.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-38_n3-n78 | 1 | 0.5 |
|  | 20 | 0.6 |
|  | 38 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.38.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20-38_n3-n78 | 1 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

8.38.5	MSD

All MSD requirements for DC_1-20-38_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.39	DC_1-3-28_n7-n78

8.39.1	Operating bands for DC

Table 8.39.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.39.2	Channel bandwidths per operating band for DC

Table 8.39.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-28A_n7B-n78A | DC_1A-n7A DC_3A-n7A DC_28A_n7A DC_1A-n7B DC_3A-n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-28A_n7B-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_28A_n7A DC_1A-n7B DC_3A-n7B DC_3C-n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.39.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-28_n7-n78 and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues is provided in TR 37.716-21-21, Table 5.3-2.

8.39.4	∆TIB and ∆RIB values

For DC_1-3-28_n7-n78 the TIB,c and RIB,c values are same as for DC_1-3-7_n28-n78 and are given in the tables below.

Table 8.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 28 | 0.6 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

Table 8.39.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n7-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

8.39.5	MSD

All MSD requirements for DC_1-3-28_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.40	DC_1-3-21_n78-n257

8.40.1	Operating bands for DC

Table 8.40.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.40.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.40.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-21A_n78A-n257A | DC_1A_n78A DC_3A_n78A DC_21A_n78A DC_1A_n257A DC_3A_n257A DC_21A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_21A_n78A DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_1A_n257G DC_3A_n257G DC_21A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_21A_n78A DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_1A_n257G DC_3A_n257G DC_21A_n257G DC_1A_n257H DC_3A_n257H DC_21A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_21A_n78A DC_1A_n257A DC_3A_n257A DC_21A_n257A DC_1A_n257G DC_3A_n257G DC_21A_n257G DC_1A_n257H DC_3A_n257H DC_21A_n257H DC_1A_n257I DC_3A_n257I DC_21A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.40.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.40.4	∆TIB and ∆RIB values

For DC_1-3-21_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n78-n257 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n78-n257 | 1 | 0 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.40.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.41	DC_1-19-42_n78-n257

8.41.1	Operating bands for DC

Table 8.41.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.41.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.41.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-19A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_19A_n78A DC_19A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_19A_n78A DC_19A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.41.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.41.4	∆TIB and ∆RIB values

For DC_1-19-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n78-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n78-n257 | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.41.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.42	DC_1-21-42_n78-n257

8.42.1	Operating bands for DC

Table 8.42.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.42.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.42.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-21A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_21A_n78A DC_21A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_21A_n78A DC_21A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.42.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.42.4	∆TIB and ∆RIB values

For DC_1-21-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n78-n257 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n78-n257 | 1 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.42.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.43	DC_19-21-42_n78-n257

8.43.1	Operating bands for DC

Table 8.43.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n78-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.43.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.43.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_19A-21A-42A_n78A-n257A | DC_19A_n78A DC_19A_n257A DC_21A_n78A DC_21A_n257A DC_42A_n257A | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n78A-n257G | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n78A-n257H | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n78A-n257A | DC_19A_n78A DC_19A_n257A DC_21A_n78A DC_21A_n257A DC_42A_n257A | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n78A-n257G | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 370 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n78A-n257H | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.43.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.43.4	∆TIB and ∆RIB values

For DC_19-21-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n78-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 8.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n78-n257 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

8.43.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.44	DC_1-3-21_n79-n257

8.44.1	Operating bands for DC

Table 8.44.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.44.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.44.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-21A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_3A_n79A DC_3A_n257A DC_21A_n79A DC_21A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.44.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.44.4	∆TIB and ∆RIB values

For DC_1-3-21_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n79-n257 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 8.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n79-n257 | 1 | 0 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

8.44.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.45	DC_1-19-42_n79-n257

8.45.1	Operating bands for DC

Table 8.45.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.45.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.45.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-19A-42A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_19A_n79A DC_19A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_19A_n79A DC_19A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.45.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.45.4	∆TIB and ∆RIB values

For DC_1-19-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n79-n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 8.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n79-n257 | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

8.45.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.46	DC_1-21-42_n79-n257

8.46.1	Operating bands for DC

Table 8.46.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n79-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.46.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.46.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-21A-42A_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_21A_n79A DC_21A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n79A-n257A | DC_1A_n79A DC_1A_n257A DC_21A_n79A DC_21A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n79A-n257G | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n79A-n257H | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.46.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.46.4	∆TIB and ∆RIB values

For DC_1-21-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n79-n257 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 8.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n79-n257 | 1 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

8.46.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.47	DC_19-21-42_n79-n257

8.47.1	Operating bands for DC

Table 8.47.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n79-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.47.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.47.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_19A-21A-42A_n79A-n257A | DC_19A_n79A DC_19A_n257A DC_21A_n79A DC_21A_n257A DC_42A_n257A | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n79A-n257G | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n79A-n257H | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n79A-n257A | DC_19A_n79A DC_19A_n257A DC_21A_n79A DC_21A_n257A DC_42A_n257A | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n79A-n257G | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 370 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n79A-n257H | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.47.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.47.4	∆TIB and ∆RIB values

For DC_19-21-42_n79-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n79-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n79 | 0 |
|  | n257 | 0 |

Table 8.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n79-n257 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n79 | 0 |
|  | n257 | 0 |

8.47.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.48	DC_1-3-21_n77-n257

8.48.1	Operating bands for DC

Table 8.48.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-21_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.48.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.48.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-21A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_21A_n77A DC_21A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-21A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.48.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.48.4	∆TIB and ∆RIB values

For DC_1-3-21_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-21_n77-n257 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-21_n77-n257 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.48.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.49	DC_1-19-42_n77-n257

8.49.1	Operating bands for DC

Table 8.49.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-19-42_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.49.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.49.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-19A-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_19A_n77A DC_19A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_19A_n77A DC_19A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-19A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.49.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.49.4	∆TIB and ∆RIB values

For DC_1-19-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-19-42_n77-n257 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-19-42_n77-n257 | 1 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.49.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.50	DC_1-21-42_n77-n257

8.50.1	Operating bands for DC

Table 8.50.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-21-42_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.50.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.50.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-21A-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_21A_n77A DC_21A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 355 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 455 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 555 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_21A_n77A DC_21A_n257A DC_42A_n257A | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 375 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 475 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-21A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 575 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.50.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.50.4	∆TIB and ∆RIB values

For DC_1-21-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21-42_n77-n257 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21-42_n77-n257 | 1 | 0.2 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.50.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.51	DC_19-21-42_n77-n257

8.51.1	Operating bands for DC

Table 8.51.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n77-n257 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.51.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.51.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_19A-21A-42A_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_21A_n77A DC_21A_n257A DC_42A_n257A | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n77A-n257G | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 350 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n77A-n257H | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 450 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42A_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 550 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_21A_n77A DC_21A_n257A DC_42A_n257A | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n77A-n257G | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_42A_n257A DC_42A_n257G | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 370 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n77A-n257H | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_19A-21A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 19 | See BCS of LTE CA in 5.6A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 570 |  |
|  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See BCS of NR CA in 5.5A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.51.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.51.4	∆TIB and ∆RIB values

For DC_19-21-42_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 8.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n77-n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n77-n257 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.51.5	MSD

MSD analysis of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. In addition, interference between FR1 bands and FR2 band is assumed to be negligible. Therefore, additional analysis is not needed.

8.52	DC_2-46-66_n41-n71

8.52.1	Operating bands for DC

Table 8.52.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-46-66_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

8.52.2	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

8.52.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 8.52.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-66_n41-n71 | 2 | 0.5 |
|  | 46 | 0 |
|  | 66 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n71 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 8.52.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46-66_n41-n71 | 2 | 0.3 |
|  | 46 | 0 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

8.52.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.53	DC_1-3-41_n28-n77

### 8.53.1	Operating bands for DC

Table 8.53.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.53.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1.

Table 8.53.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.53.3	Co-existence studies

Co-existence studies of DC_1-3-41_n28-n77 and protected bands are already covered in the constituent fall-back modes.

### 8.53.4	∆TIB and ∆RIB values

For DC_1-3-41_n28-n77, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 8.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.53.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.53.5	MSD

All MSD requirements for DC_1-3-41_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.54	DC_1-3-41_n28-n78

### 8.54.1	Operating bands for DC

Table 8.54.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.54.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1.

Table 8.54.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A | 1 |  |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 |  |  | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A | 1 |  |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 |  |  | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.54.3	Co-existence studies

Co-existence studies of DC_1-3-41_n28-n78 and protected bands are already covered in the constituent fall-back modes.

### 8.54.4	∆TIB and ∆RIB values

For DC_1-3-41_n28-n78, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 8.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.54.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.54.5	MSD

All MSD requirements for DC_1-3-41_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.55	DC_1-18-41_n3-n77

### 8.55.1	Operating bands for DC

Table 8.55.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-18-41_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.55.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.55.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 205 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.55.3	Co-existence studies

Co-existence studies of DC_1-18-41_n3-n77 and protected bands are already covered in the constituent fall-back modes.

### 8.55.4	∆TIB and ∆RIB values

For DC_1-18-41_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.55.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n77 | 1 | 0.2 |
|  | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.55.5	MSD

All MSD requirements for DC_1-18-41_n3-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.56	DC_1-18-41_n3-n78

### 8.56.1	Operating bands for DC

Table 8.56.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-18-41_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.56.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-1 according to TS 38.101-3.

Table 8.56.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41C_n3A DC_41A_n78A DC_41C_n78A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 205 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.56.3	Co-existence studies

Co-existence studies of DC_1-18-41_n3-n78 and protected bands are already covered in the constituent fall-back modes.

### 8.56.4	∆TIB and ∆RIB values

For DC_1-18-41_n3-n78, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 8.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n78 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.56.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n78 | 1 | 0.2 |
|  | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.56.5	MSD

All MSD requirements for DC_1-18-41_n3-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.57	DC_1-3-28_n40-n78

8.57.1	Operating bands for DC

Table 8.57.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.57.2	Co-existence studies

Co-existence studies are captured in TR 38.716-02-00, TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

8.57.3	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below based on lower order combinations including the same bands.

Table 8.57.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n40-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | 28 | 0.5 |
|  | n40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 8.57.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n40-n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

8.57.4	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

8.58	DC_1-3-7_n7-n78

8.58.1	Operating bands for DC

Table 8.58.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.58.2	Channel bandwidths per operating band for DC

Table 8.58.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.58.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7_n7-n78 and protected bands follow from those derived for the constituent fallback modes.

8.58.4	∆TIB and ∆RIB values

For DC_1-3-7_n7-n78 the TIB,c and RIB,c values are same as for DC_1-3-7_n78 and are given in the tables below.

Table 8.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

Table 8.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n7-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |

8.58.5	MSD

All MSD requirements for DC_1-3-7_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.59	DC_1-7-28_n7-n78

8.59.1	Operating bands for DC

Table 8.59.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.59.2	Channel bandwidths per operating band for DC

Table 8.59.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-7A-28A_n7A-n78A | DC_1A-n7A DC_7A-n7A4 DC_28A_n7A DC_1A_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.59.3	Co-existence studies

The harmonic and IMD products caused by DC_1-7-28_n7-n78 and protected bands follow from those derived for the constituent fallback modes.

8.59.4	∆TIB and ∆RIB values

For DC_1-7-28_n7-n78 the TIB,c and RIB,c values are same as for DC_1-7-28_n78 and are given in the tables below.

Table 8.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n7-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 8.59.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n7-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

8.59.5	MSD

All MSD requirements for DC_1-7-28_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.60	DC_3-7-28_n7-n78

8.60.1	Operating bands for DC

Table 8.60.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n7-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.60.2	Channel bandwidths per operating band for DC

Table 8.60.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A-28A_n7A-n78A | DC_3A-n7A DC_7A-n7A4 DC_28A_n7A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A-28A_n7A-n78A | DC_3A-n7A DC_3C-n7A DC_7A-n7A4 DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A | 3 | See CA_3C in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.60.3	Co-existence studies

The harmonic and IMD products caused by DC_3-7-28_n7-n78 and protected bands follow from those derived for the constituent fallback modes.

8.60.4	∆TIB and ∆RIB values

For DC_3-7-28_n7-n78 the TIB,c and RIB,c values are same as for DC_3-7-28_n78 and are given in the tables below.

Table 8.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n7-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 8.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n7-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

8.60.5	MSD

All MSD requirements for DC_3-7-28_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.61	DC_1-3-41_n28-n257

### 8.61.1	Operating bands for DC

Table 8.61.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n28-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 8.61.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA specified in TS 38.101-3 according to TS 38.101-1.

Table 8.61.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-41A_n28A-n257A | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 490 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n28A-n257A | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41C_n28A DC_41C_n257A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 490 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257I DC_41C_n28A DC_41C_n257I | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 510 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.61.3	Co-existence studies

Co-existence studies of DC_1-3-41_n28-n257 and protected bands are already covered in the constituent fall-back modes.

### 8.61.4	∆TIB and ∆RIB values

For DC_1-3-41_n28-n257, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 8.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n257 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.61.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n257 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.61.5	MSD

All MSD requirements for DC_1-3-41_n28-n257 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.62	DC_1-18-41_n3-n257

### 8.62.1	Operating bands for DC

Table 8.62.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-18-41_n3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 8.62.2	Channel bandwidths per operating band for DC

Supported bandwidths table is omitted since supported bandwidths of inter-band EN-DC is consisting with BCS of LTE CA specified in TS 36.101 and BCS of NR CA including FR2 specified in TS 38.101-3.

Table 8.62.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-18A-41A_n3A-n257A | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41A_n257A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 485 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41C_n3A-n257A | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41C_n3A DC_41A_n257A DC_41C_n257A | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 505 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41A_n3A-n257I | DC_18A_n3A DC_41A_n3A DC_18A_n257A DC_41A_n257A DC_18A_n257I DC_41A_n257I | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 485 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-18A-41C_n3A-n257I | DC_18A_n3A DC_41A_n3A DC_41C_n3A DC_18A_n257A DC_41A_n257A DC_41C_n257A DC_18A_n257I DC_41A_n257I DC_41C_n257I | 1 | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 505 |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.62.3	Co-existence studies

Co-existence studies of DC_1-18-41_n3-n257 and protected bands are already covered in the constituent fall-back modes.

### 8.62.4	∆TIB and ∆RIB values

For DC_1-18-41_n3-n257, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 8.62.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n257 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.62.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18-41_n3-n257 | 1 | 0 |
|  | 18 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.62.5	MSD

All MSD requirements for DC_1-18-41_n3-n257 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.63		DC_1-3-20_n38-n78

8.63.1		Operating bands for DC

Table 8.63.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-20_n38-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz |  | 862 MHz | 791 MHz |  | 821 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

8.63.2	Channel bandwidths per operating band for DC

Table 8.63.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_1A_n38A DC_3A_n38A DC_20A_n38A | 1 | Please see BCS0 of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.63.3	Co-existence studies

Co-existence studies of this DL of LTE 3 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration are already covered by those of the DL of LTE 1 band + NR 2 band and UL of 1 LTE band + 1 NR band DC configuration, and those of the DL of LTE 2 band + NR 1 band and UL of 1 LTE band + 1 NR band DC configuration, respectively. which means the harmonic and IMD products caused by DC_1-3-20_n38-n78 follow from those derived for the constituent fallback modes. Therefore, additional analysis is not needed.

8.63.4	∆TIB and ∆RIB values

For DC_1-3-20_n38-n78, the TIB,c and RIB values are given in the tables below.

Table 8.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n38-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 8.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-20_n38-n78 | 1 | 0 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

8.63.5	MSD

All MSD requirements for DC_1-3-20_n38-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.64		DC_1-3-20_n41-n78

### 8.64.1	 Operating bands for DC

Table 8.64.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-20A_n41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.64.2 	Configuration for DC

Table 8.64.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-20A_n41A-n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 260 |
|  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.64.3	 Co-existence studies

The harmonic and IMD products caused by DC_1A-3A-20A_n41A-n78A and protected bands follow from those derived for the constituent fallback modes. The summary of self-interference issues are provided in TR 38.716-21-21, Table 5.3-2.

### 8.64.4	 ∆TIB and ∆RIB values

Table 8.64.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n41-n78 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 8.64.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-20_n41-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 8.64.5	MSD

All MSD requirements for DC_1A-3A-20A_n41A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

8.65	DC_1-3-41_n77-n257

8.65.1	Operating bands for DC

Table 8.65.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.65.2	Channel bandwidths per operating band for DC

Table 8.65.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-41A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A | 1 | Please see BCS of CA_1A-3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G | 1 | Please see BCS of CA_1A-3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H | 1 | Please see BCS of CA_1A-3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I | 1 | Please see BCS of CA_1A-3A-41A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A | 1 | Please see BCS of CA_1A-3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G | 1 | Please see BCS of CA_1A-3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H | 1 | Please see BCS of CA_1A-3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I | 1 | Please see BCS of CA_1A-3A-41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.65.3	Co-existence studies

Based on the co-existence studies of DC_1_n77, DC_1_n257, DC_3_n77, DC_3_n257, DC_41_n77 and DC_41_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.65.4	∆TIB and ∆RIB values

For DC_1-3-41_n77-n257, the TIB,c and RIB,c values coming from the values for DC_1-3-41_n77 are given in the tables below.

Table 8.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n77-n257 | 1 | 0.6 |
|  | 3 | 0. 6 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.65.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n77-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.65.5	MSD

As mentioned in 8.65.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.66	DC_1-3-42_n77-n257

8.66.1	Operating bands for DC

Table 8.66.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-42_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.66.2	Channel bandwidths per operating band for DC

Table 8.66.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_42A_n257A | 1 | Please see BCS of CA_1A-3A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G | 1 | Please see BCS of CA_1A-3A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | Please see BCS of CA_1A-3A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | Please see BCS of CA_1A-3A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_42A_n257A DC_42C_n257A | 1 | Please see BCS of CA_1A-3A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 1 | Please see BCS of CA_1A-3A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 1 | Please see BCS of CA_1A-3A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 1 | Please see BCS of CA_1A-3A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.66.3	Co-existence studies

Based on the co-existence studies of DC_1_n77, DC_1_n257, DC_3_n77, DC_3_n257 and DC_42_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.66.4	∆TIB and ∆RIB values

For DC_1-3-42_n77-n257, the TIB,c and RIB,c values coming from the values for DC_1-3-42_n77 are given in the tables below.

Table 8.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n77-n257 | 1 | 0.6 |
|  | 3 | 0. 6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.66.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n77-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.66.5	MSD

As mentioned in 8.66.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.67	DC_1-41-42_n77-n257

8.67.1	Operating bands for DC

Table 8.67.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-41-42_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.67.2	Channel bandwidths per operating band for DC

Table 8.67.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A DC_42A_n257A | 1 | Please see BCS of CA_1A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 1 | Please see BCS of CA_1A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | Please see BCS of CA_1A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | Please see BCS of CA_1A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A DC_42A_n257A DC_42C_n257A | 1 | Please see BCS of CA_1A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 1 | Please see BCS of CA_1A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 1 | Please see BCS of CA_1A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 1 | Please see BCS of CA_1A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A | 1 | Please see BCS of CA_1A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G | 1 | Please see BCS of CA_1A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | Please see BCS of CA_1A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | Please see BCS of CA_1A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A DC_42C_n257A | 1 | Please see BCS of CA_1A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257GDC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 1 | Please see BCS of CA_1A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 400 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 1 | Please see BCS of CA_1A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 500 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 1 | Please see BCS of CA_1A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.67.3	Co-existence studies

Based on the co-existence studies of DC_1_n77, DC_1_n257, DC_41_n77, DC_41_n257 and DC_42_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.67.4	∆TIB and ∆RIB values

For DC_1-41-42_n77-n257, the TIB,c and RIB,c values coming from the values for DC_1-41-42_n77 are given in the tables below.

Table 8.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41-42_n77-n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 8.67.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41-42_n77-n257 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

8.67.5	MSD

As mentioned in 8.67.3, there is no need to specify additional MSD requirement for this UL DC configuration.

8.68	DC_3-41-42_n77-n257

8.68.1	Operating bands for DC

Table 8.68.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41-42_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.68.2	Channel bandwidths per operating band for DC

Table 8.68.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A-42A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_42A_n257A | 3 | Please see BCS of CA_3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42A_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 3 | Please see BCS of CA_3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 360 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42A_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | Please see BCS of CA_3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 460 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | Please see BCS of CA_3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 560 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42C_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_42A_n257A DC_42C_n257A | 3 | Please see BCS of CA_3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42C_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 3 | Please see BCS of CA_3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42C_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 3 | Please see BCS of CA_3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41A-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 3 | Please see BCS of CA_3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A | 3 | Please see BCS of CA_3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42A_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G | 3 | Please see BCS of CA_3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42A_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | Please see BCS of CA_3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | Please see BCS of CA_3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42C_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A DC_42C_n257A | 3 | Please see BCS of CA_3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42C_n77A-n257G | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257GDC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 3 | Please see BCS of CA_3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 400 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42C_n77A-n257H | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 3 | Please see BCS of CA_3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 500 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 3 | Please see BCS of CA_3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.68.3	Co-existence studies

Based on the co-existence studies of DC_3_n77, DC_3_n257, DC_41_n77, DC_41_n257 and DC_42_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.68.4	∆TIB and ∆RIB values

For DC_3-41-42_n77-n257, the TIB,c and RIB,c values coming from the values for DC_3-41-42_n77 are given in the tables below.

Table 8.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42_n77-n257 | 3 | 1 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.68.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41-42_n77-n257 | 3 | 0.5 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

8.68.5	MSD

As mentioned in 8.68.3, there is no need to specify additional MSD requirement for this UL DC configuration.

9	EN-DC band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

9.1	DC_1A-3A-5A-7A_n78A-n257

9.1.1	Operating bands for DC

Table 9.1.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-5A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

9.1.2	Channel bandwidths per operating band for DC

Table 9.1.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-5A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 370 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 470 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 570 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 670 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 770 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 870 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 970 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.1.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

9.1.4	∆TIB and ∆RIB values

For DC_1A-3A-5A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.1.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

9.2	DC_1A-3A-5A-7A-7A_n78A-n257

9.2.1	Operating bands for DC

Table 9.2.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-5A-7A-7A_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

9.2.2	Channel bandwidths per operating band for DC

Table 9.2.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-5A-7A-7A_n78A-n257D | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 590 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257E | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 790 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257F | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 990 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257G | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 390 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257H | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 490 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257I | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 590 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257J | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 690 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257K | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 790 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257L | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 890 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 990 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 5 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. Furthermore, harmonics and IMD issues are covered by constituent fallback modes in TR 37.864-41-21. Therefore, additional analysis is not needed.

9.2.4	∆TIB and ∆RIB values

For DC_1A-3A-5A-7A-7A_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A-7A_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-5A-7A-7A_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.2.5	MSD

MSD of this 5DL/2UL DC configuration are already covered by those of 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01 and TR 37.863-02-01 respectively. And constituent fallback modes will be covered with TR 37.864-41-21. Therefore, additional analysis is not needed.

9.3	DC_1-3-7-28_n5-n78

9.3.1	Operating bands for DC

Table 9.3.1-1: DC band combination of LTE 4DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n5-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.3.2	Channel bandwidths per operating band for DC

Table 9.3.2-1: Supported bandwidths per DC band combination of LTE 4DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Max BW [MHz] |
| DC_1A-3A-7A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7A_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
| DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |

9.3.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7-28_n5-n78 and protected bands follow from those derived for the constituent fallback modes.

9.3.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n5-n78, the TIB,c and RIB values are derived from DC_1-3-7-20_n28-n78 and are given in the tables below.

Table 9.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n5-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 9.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n5-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

9.3.5	MSD

All MSD requirements for DC_1-3-7-28_n5-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

9.4	DC_1A-3A-41A-42A_n78A-n257

9.4.1	Operating bands for DC

Table 9.4.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

9.4.2	Channel bandwidths per operating band for DC

Table 9.4.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-41A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_41A_n78A DC_41A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 280 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-41A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 380 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 480 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.4.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

9.4.4	∆TIB and ∆RIB values

For DC_1-3-41-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.4.5	MSD

As mentioned in 9.4.3, there is no need to specify additional MSD requirement for this UL DC configuration.

9.5	DC_1A-3A-18A-42A_n78A-n257

9.5.1	Operating bands for DC

Table 9.5.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

9.5.2	Channel bandwidths per operating band for DC

Table 9.5.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-18A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_18A_n78A DC_18A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 280 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-18A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 380 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-18A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 480 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-18A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.5.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

9.5.4	∆TIB and ∆RIB values

For DC_1-3-18-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.5.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18-42_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.5.5	MSD

As mentioned in 9.5.3, there is no need to specify additional MSD requirement for this UL DC configuration.

9.6	DC_3A-28A-41A-42A_n78A-n257

9.6.1	Operating bands for DC

Table 9.6.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28-41-42_n78-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

9.6.2	Channel bandwidths per operating band for DC

Table 9.6.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-28A-41A-42A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_28A_n78A DC_28A_n257A DC_41A_n78A DC_41A_n257A DC_42A_n257A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 280 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_3A-28A-41A-42A_n78A-n257G | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 380 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A-41A-42A_n78A-n257H | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 480 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-28A-41A-42A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.6.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

9.6.4	∆TIB and ∆RIB values

For DC_3-28-41-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28-41-42_n78-n257 | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 9.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28-41-42_n78-n257 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 01/0.52 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

9.6.5	MSD

As mentioned in 9.6.3, there is no need to specify additional MSD requirement for this UL DC configuration.

9.7	DC_1A-3A-28A-42A_n78A-n257

9.7.1	Operating bands for DC

Table 9.7.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28-42_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

9.7.2	Channel bandwidths per operating band for DC

Table 9.7.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-3A-28A-42A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_28A_n78A DC_28A_n257A DC_42A_n257A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 280 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-3A-28A-42A_n78A-n257G | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_42A_n257A DC_42A_n257G | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 380 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-28A-42A_n78A-n257H | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 480 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-28A-42A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 580 |  |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A1-2 in TS-38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.7.3	Co-existence studies

The harmonic and IMD products caused by this configuration follow from those derived for the constituent fallback modes.

9.7.4	∆TIB and ∆RIB values

For DC_1-3-28-42_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 9.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28-42_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 9.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28-42_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
|  | n257 | 0 |

9.7.5	MSD

As mentioned in 9.7.3, there is no need to specify additional MSD requirement for this UL DC configuration.

9.8	DC_1-3-7-28_n7-n78

9.8.1	Operating bands for DC

Table 9.8.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

9.8.2	Channel bandwidths per operating band for DC

Table 9.8.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Conf | UL Conf | Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_7A-n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-7A-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_7A-n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 3 | See CA_3C in Table 5.6A.1-1 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.8.3	Co-existence studies

The harmonic and IMD products caused by DC_1-3-7-28_n7-n78 and protected bands follow from those derived for the constituent fallback modes.

9.8.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n7-n78 the TIB,c and RIB,c values are same as for DC_1-3-7-28_n78 and are given in the tables below.

Table 9.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

Table 9.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n7-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

9.8.5	MSD

All MSD requirements for DC_1-3-7-28_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

9.9	DC_1-3-41-42_n77-n257

9.9.1	Operating bands for DC

Table 9.9.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41-42_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

9.9.2	Channel bandwidths per operating band for DC

Table 9.9.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-41A-42A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_42A_n257A | 1 | Please see BCS of CA_1A-3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G | 1 | Please see BCS of CA_1A-3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 380 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | Please see BCS of CA_1A-3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 480 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | Please see BCS of CA_1A-3A-41A-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 580 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_42A_n257A DC_42C_n257A | 1 | Please see BCS of CA_1A-3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 1 | Please see BCS of CA_1A-3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 400 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 1 | Please see BCS of CA_1A-3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 500 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 1 | Please see BCS of CA_1A-3A-41A-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42A_n77A-n257A | D DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A | 1 | Please see BCS of CA_1A-3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42A_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G | 1 | Please see BCS of CA_1A-3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 400 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42A_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H | 1 | Please see BCS of CA_1A-3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 500 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I | 1 | Please see BCS of CA_1A-3A-41C-42A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 600 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42C_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_3A_n77A DC_3A_n257A DC_41A_n77A DC_41A_n257A DC_41C_n77A DC_41C_n257A DC_42A_n257A DC_42C_n257A | 1 | Please see BCS of CA_1A-3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 620 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257A in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42C_n77A-n257G | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_41A_n77A DC_41A_n257A DC_41A_n257GDC_41C_n77A DC_41C_n257A DC_41C_n257G DC_42A_n257A DC_42A_n257G DC_42C_n257A DC_42C_n257G | 1 | Please see BCS of CA_1A-3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 420 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257G in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42C_n77A-n257H | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42C_n257A DC_42C_n257G DC_42C_n257H | 1 | Please see BCS of CA_1A-3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 520 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257H in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I | 1 | Please see BCS of CA_1A-3A-41C-42C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  | 620 |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | Please see BCS of CA_n77A-n257I in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.9.3	Co-existence studies

Based on the co-existence studies of DC_1_n77, DC_1_n257, DC_3_n77, DC_3_n257, DC_41_n77, DC_41_n257 and DC_42_n257 captured in TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

9.9.4	∆TIB and ∆RIB values

For DC_1-3-41-42_n77-n257, the TIB,c and RIB,c values coming from the values for DC_1-3-41-42_n77 are given in the tables below.

Table 9.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n77-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 9.9.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41-42_n77-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
|  | n257 | 0 |

9.9.5	MSD

As mentioned in 9.9.3, there is no need to specify additional MSD requirement for this UL DC configuration.

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old | New |
| 2018-08 | RAN4 #88 | R4-1810526 |  |  | TR37.716-21-21 Skeleton for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | N/A | 0.0.1 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #88 - R4-1810307: TP on general part for EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 - R4-1810319: TP to TR 37.864-xx-21: channel bandwidth and co-existence studies for DC_3_n1-n77 - R4-1811502: Revised WID for LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |
| 2018-10 | RAN4 #88BIS | R4-1813794 |  |  | Updated TR37.716-21-21 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.0.1 | 0.1.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #88BIS - R4-1812242: TP on summary of self-interference analysis for EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 - R4-1813779: TP for TR 37.716-21-21 DC_LTE 1band_NR 2bands - R4-1813780: TP for TR 37.716-21-21 DC_LTE 2band_NR 2bands - R4-1813781: TP for TR 37.716-21-21 DC_LTE 3band_NR 2bands - R4-1813782: TP for TR 37.716-21-21 DC_LTE 4band_NR 2bands R4-1812240: Revised WID for LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |
| 2018-11 | RAN4 #89 | R4-1814769 |  |  | Updated TR37.716-21-21 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.1.0 | 0.2.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #89 - R4-1814940: TP to TR 37.716-21-21: finalize UE requirements for DC_3_n1-n77 - R4-1816175: TP for TR 37.716-21-21 for DC_1A_n3A-n78A - R4-1816176: TP for TR 37.716-21-21 for DC_3A_n1A-n78A - RP-182302: Revised WID for LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 |  |  |
| 2019-02 | RAN4 #90 | R4-1900246 |  |  | Updated TR37.716-21-21 v0.3.0 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.2.0 | 0.3.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #90 - R4-1900247, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," - R4-1900249, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," - R4-1900781, "TP for 37.716-21-21 for DC_3A_n7A-n78A," - R4-1900782, "TP for 37.716-21-21 for DC_3C_n7A-n78A," - R4-1901436, "TP for TR 37.716-21-21 for DC_13A_n5A-n260A," - R4-1901438, "TP for TR 37.716-21-21 for DC_66A_n5A-n260A," - R4-1901439, "TP for TR 37.716-21-21 for DC_3A_n20A-n78A," - R4-1901455, "TP for TR 37.716-21-21 for DC_28A_n8A-n78A," - R4-1901456, "TP for TR 37.716-21-21 for DC_28A_n8A-n258A," - R4-1902112, "TP for 37.716-21-21 for DC_3C_n28A-n78A," - R4-1902113, "TP for 37.716-21-21 for DC_7C_n28A-n78A," - R4-1902115, "TP for TR 37.716-21-21 to include DC_66A_n71A-n260A, DC_66A_n71A-n260(2A)," - R4-1902120, "TP to TR 37.716-21-21: DC_66A_n71A-n261A," - R4-1902121, "TP to TR 37.716-21-21 DC_66A_n71A-n261(2A)," - R4-1902466, "TP for 37.716-21-21 for DC_1A_n7A-n78A," |  |  |
| 2019-04 | RAN4 #90BIS | R4-1903007 |  |  | Updated TR37.716-21-21 v0.4.0 for EN-DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | 0.3.0 | 0.4.0 |
|  |  |  |  |  | Approved TPs and revised WID in RAN4 #90BIS - R4-1904438, "TP for TR 37.716-21-21 to include DC_66_n25-n41," Ericsson, T-Mobile US - R4-1903993, "TP for TR 37.716-21-21: DC_3A_n1A-n257A," KDDI - R4-1903992, "TP for TR 37.716-21-21: DC_3A_n1A-n79A," KDDI - R4-1904885, "TP for TR 37.716-21-21 for DC_1-3-7_n28-n78," Huawei, HiSilicon - R4-1904884, "TP for TR 37.716-21-21 for DC_3-7_n28-n78," Huawei, HiSilicon - R4-1903182, "TP for TR 37.716-21-21 for DC_1-7_n28-n78," Huawei, HiSilicon - R4-1903181, "TP for TR 37.716-21-21 for DC_1-3_n28-n78," Huawei, HiSilicon - R4-1904883, "TP for TR 37.716-21-21 for DC_1-3_n7-n78," Huawei, HiSilicon - R4-1904900, "TP for TR 37.716-21-21 DC_3A_n40A-n78A," Nokia, Nokia Shanghai Bell - R4-1904899, "TP for TR 37.716-21-21 DC_1A_n40A-n78A," Nokia, Nokia Shanghai Bell - R4-1903011, "MSD results for new LTE xbands DL_1band UL (x=1,2,3,4) + NR 2bands DL_1band UL DC with self-interference problems," LG Electronics France - R4-1903010, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1903008, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," LG Electronics France - R4-1902993, "Draft CR for TS 38.101-3: Support of DL_n257G-I in DC_1_n77-n257," SoftBank Corp.- R4-1902973, "TP for TR 37.716-21-21: EN-DC_8_n77-n257," SoftBank Corp. |  |  |
| 2019-05 | RAN4 #91 | R4-1906054 |  |  | Updated TR 37.716-21-21 v0.5.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.4.0 | 0.5.0 |
|  |  |  |  |  | Approved TPs and revised WID - R4-1905513, "TP for TR 37.716-21-21 for DC_1_n77-n258," Huawei, HiSilicon - R4-1905514, "TP for TR 37.716-21-21 for DC_1_n78-n258," Huawei, HiSilicon - R4-1905515, "TP for TR 37.716-21-21 for DC_1_n79-n258," Huawei, HiSilicon - R4-1905516, "TP for TR 37.716-21-21 for DC_3_n77-n258," Huawei, HiSilicon - R4-1905517, "TP for TR 37.716-21-21 for DC_3_n78-n258," Huawei, HiSilicon - R4-1905518, "TP for TR 37.716-21-21 for DC_3_n79-n258," Huawei, HiSilicon - R4-1905787, "TP for TR 37.716-21-21: EN-DC_1-8_n77-n257," SoftBank Corp. - R4-1906055, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," LG Electronics France - R4-1906057, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1906191, "TP for DC_1-19-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906192, "TP for DC_1-19-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906193, "TP for DC_1-19_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906194, "TP for DC_1-19_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906195, "TP for DC_1-21-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906196, "TP for DC_1-21-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906197, "TP for DC_1-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906198, "TP for DC_1-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906199, "TP for DC_1-3-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906200, "TP for DC_1-3-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906202, "TP for DC_1-3_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906203, "TP for DC_1-3_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906204, "TP for DC_1-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906205, "TP for DC_1-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906206, "TP for DC_19-21-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906207, "TP for DC_19-21-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906208, "TP for DC_19-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906209, "TP for DC_19-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906210, "TP for DC_19-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906211, "TP for DC_19-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906212, "TP for DC_21-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906213, "TP for DC_21-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906214, "TP for DC_3-19_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906215, "TP for DC_3-19_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906216, "TP for DC_3-21_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906217, "TP for DC_3-21_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906218, "TP for DC_3-42_n77-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906219, "TP for DC_3-42_n78-n79 for TR 37.716-21-21.doc," NTT DOCOMO, INC. - R4-1906329, "TP for TR 37.716-21-21: UE requirements for DC_3-3_n78-n257, DC_3-3-7_n78-n257, DC_3-3-7-7_n78-n257," CHTTL - R4-1906750, “TP for TR 37.716-21-11 to include DC_20_n3-n78," Ericsson, Swisscom - R4-1906751, “TP for TR 37.716-21-11 to include DC_7_n1-n78," Ericsson, CHTTL, Swisscom - R4-1906752, “TP for TR 37.716-21-11 to include DC_20_n1-n78," Ericsson, Swisscom - R4-1906753, "TP for TR 37.716-21-11 to include DC_3-7_n1-n78," Ericsson, CHTTL, Swisscom - R4-1907393, "TP for TR 37.716-21-11 to include DC_7_n3-n78," Ericsson, Swisscom |  |  |
| 2019-08 | RAN4 #92 | R4-1908693 |  |  | Updated TR 37.716-21-21 v0.6.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.5.0 | 0.6.0 |
|  |  |  |  |  | Approved TPs and revised WID - R4-1908278, "TP for TR 37.716-21-21: DC_1A-3A_n77-n257," KDDI - R4-1908289, "TP to TR 37.716-21-21: DC_2_n41-n261," Nokia - R4-1908291, "TP to TR 37.716-21-21: DC_66_n41-n261," Nokia - R4-1908566, "TP for TR37.716-21-21 for DC_40_n41-n79," ZTE Corporation, - R4-1908581, "TP for TR 37.716-21-21: UE requirements for DC_3-3_n1-n78, DC_7-7_n1-n78," CHTTL - R4-1908582, "TP for TR 37.716-21-21: UE requirements for DC_3-3-7_n1-n78, DC_3-7-7_n1-n78, DC_3-3-7-7_n1-n78," CHTTL - R4-1908693, "TR 37.716-21-21 v0.6.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16," LG Electronics France - R4-1909802, "TP for TR 37.716-21-21 to conclude DC_66A_n25A-n41A," Ericsson, T-Mobile US - R4-1909804, "TP for TR 37.716-21-21 to include DC_2_n41-n260," Ericsson, T-Mobile US - R4-1909805, "TP for TR 37.716-21-21 to include DC_66_n41-n260," Ericsson, T-Mobile US - R4-1909826, "TP for TR 37.716-21-21 to include DC_1_n5-n78," Ericsson, Telstra - R4-1909828, "TP for TR 37.716-21-21 to include DC_7_n5-n78," Ericsson, Telstra - R4-1909827, "TP for TR 37.716-21-21 to include DC_3_n5-n78," Ericsson, Telstra - R4-1909829, "TP for TR 37.716-21-21 to include DC_28_n5-n78," Ericsson, Telstra - R4-1909830, "TP for TR 37.716-21-21 to include DC_1-3_n5-n78," Ericsson, Telstra - R4-1909831, "TP for TR 37.716-21-21 to include DC_1-7_n5-n78," Ericsson, Telstra - R4-1909832, "TP for TR 37.716-21-21 to include DC_1-28_n5-n78," Ericsson, Telstra - R4-1909833, "TP for TR 37.716-21-21 to include DC_3-7_n5-n78," Ericsson, Telstra - R4-1909834, "TP for TR 37.716-21-21 to include DC_3-28_n5-n78," Ericsson, Telstra - R4-1909835, "TP for TR 37.716-21-21 to include DC_7-28_n5-n78," Ericsson, Telstra - R4-1909836, "TP for TR 37.716-21-21 to include DC_1-3-7_n5-n78," Ericsson, Telstra - R4-1910191, "TP for TR 37.716-21-21 to include DC_3-7-28_n5-n78," Ericsson, Telstra - R4-1910192, "TP for TR 37.716-21-21 to include DC_1-3-7-28_n5-n78," Ericsson, Telstra - R4-1910193, "TP for TR 37.716-21-21 to include DC_1-7-28_n5-n78," Ericsson, Telstra - R4-1910307, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16," LG Electronics France - R4-1910363, "CR on introduction of completed EN-DC of x bands LTE and 2 band NR from RAN4#92 into TS 38.101-3," LG Electronics France |  |  |
| 2019-10 | RAN4 #92BIS | R4-1911443 |  |  | Updated TR 37.716-21-21 v0.7.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.6.0 | 0.7.0 |
|  |  |  |  |  | Approved TPs and revised WID at #92BIS - R4-1911153, "TP for TR 37.716-21-21: EN-DC_3_n77-n257," SoftBank Corp. - R4-1911155, "TP for TR 37.716-21-21: EN-DC_8_n3-n28," SoftBank Corp. - R4-1911157, "TP for TR 37.716-21-21: EN-DC_11_n77-n257," SoftBank Corp. - R4-1911246, "TP for TR37.716-21-21_ DC_3A_n40A-n41A," ZTE Corporation - R4-1911266, "TP for TR 37.716-21-21: adding support of DC_3-3_n78_n257, DC_3-3-7_n78_n257, DC_3-3-7-7_n78_n257 with n257D to n257M bandwidth classes," CHTTL - R4-1911271, "TP for TR 37.716-21-21: UE requirements for DC_8_n1-n78," CHTTL - R4-1911443, "TR 37.716-21-21 v0.7.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1911447, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16," LG Electronics France - R4-1911448, "MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1911444, "vised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16," LG Electronics France - R4-1911535, "TP to TR 37.716-21-21: Addition of DL_7C_n1A-n78A," Nokia, BT - R4-1911606, "TP for 37.716-21-21 to introduce DC_2A-66A_n41A-n261," Nokia, Nokia Shanghai Bell, T-Mobile USA - R4-1912272, "TP for TR 37.716-21-21 to include DC_66_n41-n71," Ericsson, T-Mobile - R4-1912273, "TP for TR 37.716-21-21 to include DC_2-66_n41-n71," Ericsson, T-Mobile - R4-1912274, "TP for TR 37.716-21-21 to include DC_2-66_n41-n260," Ericsson, T-Mobile - R4-1912568, "updated TP for TR 37.716-21-21: DC_1-3_n7-n78 to add configurations," Huawei, HiSilicon - R4-1912569, "updated TP for TR 37.716-21-21: DC_3-7_n1-n78 to add configurations," Huawei, HiSilicon - R4-1912574, "TP to TR 37.716-21-21: Addition of DL_3A-7C_n1A-n78A, DL_3C-7A_n1A-n78A and DL_3C-7C_n1A-n78A," Nokia, BT - R4-1912576, "TP to TR 37.716-21-21: Addition of DL_3C_n1A-n78A," Nokia, BT - R4-1912594, "TP for TR 37.716-21-21: DC_8A_n78A-n257," Samsung, KT - R4-1912595, "TP for TR 37.716-21-21: DC_3C_n78A-n257," Samsung, KT - R4-1912596, "TP for TR 37.716-21-21: DC_3A-8A_n78A-n257," Samsung, KT - R4-1912597, "TP for TR 37.716-21-21: DC_1A-8A_n78A-n257A," Samsung, KT - R4-1912598, "TP for TR 37.716-21-21: DC_1A-3-8A_n78A-n257A," Samsung, KT - R4-1912599, "TP to TR 37.716-21-21: DC_2A-7A-7A_n38A-n78A," Intel Corporation - R4-1912600, "TP for TR 37.716-21-21 to include DC_2_n41-n71," Ericsson, T-Mobile - R4-1912603, "TP for TR 37.716-21-21: EN-DC_1_n3-n28," SoftBank Corp. - R4-1912613, "TP to TR 37.716-21-21: DC_7A-7A-66A_n38A-n78A," Intel Corporation |  |  |
| 2019-11 | RAN4 #93 | R4-193964 |  |  | Updated TR 37.716-21-21 v0.8.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.7.0 | 0.8.0 |
|  |  |  |  |  | Approved TPs and revised WID at #93 - R4-1913574, "TP for TR 37.716-21-21 to include DC_2A-7A_n38A-n78A," Intel Corporation - R4-1913611, "TP for TR37.716-21-21_ DC_1A-3A_n38A-n78A," ZTE Corporation - R4-1913612, "TP for TR37.716-21-21_ DC_1A-7A-20A_n3A-n78A," ZTE Corporation - R4-1913638, "TP for TR 37.716-21-21: EN-DC_1-8_n3-n28," SoftBank Corp. - R4-1913664, "TP for TR 37.716-21-21: EN-DC_1-11_n77-n257," SoftBank Corp. - R4-1913668, "TP for TR 37.716-21-21: EN-DC_8-11_n77-n257," SoftBank Corp. - R4-1913670, "TP for TR 37.716-21-21: EN-DC_1-8-11_n77-n257," SoftBank Corp. - R4-1913872, "TP for TR 37.716-21-21: UE requirements for DC_3-8_n1-n78, DC_7-8_n1-n78," CHTTL - R4-1913874, "TP for TR 37.716-21-21: UE requirements for DC_3-7-8_n1-n78," CHTTL - R4-1913944, "TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16," LG Electronics - R4-1913964, "TR 37.716-21-21 v0.8.0 update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics - R4-1913965, "Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16," LG Electronics - R4-1913966, "Introducing CR on new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," LG Electronics France - R4-1914024, "TP for TR 37.716-21-21 DC_18A_n78A-n257," KDDI - R4-1914026, "TP for TR 37.716-21-21 DC_1_n3A-n257," KDDI - R4-1914027, "TP for TR 37.716-21-21 DC_18A_n3A-n257," KDDI - R4-1914028, "TP for TR 37.716-21-21 DC_28A_n3A-n257," KDDI - R4-1914034, "TP for TR 37.716-21-21 DC_41_n78A-n257," KDDI - R4-1914035, "TP for TR 37.716-21-21 DC_42_n78A-n257," KDDI - R4-1914601, "TP for TR 37.716-21-21:DC_5A_n7A-n78A," Huawei, HiSilicon - R4-1914036, "TP for TR 37.716-21-21 DC_28_n78A-n257," KDDI - R4-1914602, "TP for TR 37.716-21-21:DC_7A_n7A-n78A," Huawei, HiSilicon - R4-1914603, "TP for TR 37.716-21-21:DC_66A_n7A-n78A," Huawei, HiSilicon - R4-1914698, "TP for TR 37.716-21-21 to include DC_3_n1-n7," Ericsson, BT plc - R4-1914704, "draft Rel-16 CR to 38.101-3 to add missing UL configurations for 1-3-7_n5-n78 and 1-3-28_n5-n78," Ericsson, Telstra - R4-1915646, "TP to TR 37.716-21-21: Addition of 3_n1-n28," Nokia - R4-1915647, "TP to TR 37.716-21-21: Addition of DC_ 20A_n1A-n28A," Nokia - R4-1915648, "TP for TR 37.716-21-21 DC_18A_n3A-n78A," KDDI - R4-1915649, "TP for TR 37.716-21-21 DC_28A_n3A-n78A," KDDI - R4-1915650, "TP for TR 37.716-21-21 DC_1A-18A_n3A-n78A," KDDI - R4-1915651, "TP for TR 37.716-21-21 DC_1A-28A_n3A-n78A," KDDI - R4-1915652, "TP for 37.716-21-21 to introduce DC_3-20A_n1A-n28A," Nokial, BT - R4-1915653, "TP for TR 37.716-21-21:DC_12A_n7A-n78A," Huawei, HiSilicon - R4-1915654, "TP for TR 37.716-21-21 DC_1A-18A_n3A-n257," KDDI - R4-1915655, "TP for TR 37.716-21-21 DC_1A-28A_n3A-n257," KDDI - R4-1915656, "TP for TR 37.716-21-21 DC_1A-41_n78A-n257," KDDI - R4-1915657, "TP for TR 37.716-21-21 DC_3A-41A_n78A-n257," KDDI - R4-1915658, "TP for TR 37.716-21-21 DC_3A-42A_n78A-n257," KDDI - R4-1915660, "TP for TR 37.716-21-21 DC_1A-42A_n78A-n257," KDDI - R4-1915659, "TP for TR 37.716-21-21 DC_41A-42A_n78A-n257," KDDI - R4-1915661, "TP for TR 37.716-21-21 DC_1A-18A_n78A-n257," KDDI - R4-1915662, "TP for TR 37.716-21-21 DC_3A-18A_n78A-n257," KDDI - R4-1915663, "TP for TR 37.716-21-21 DC_18A-42A_n78A-n257," KDDI - R4-1915664, "TP for TR 37.716-21-21 DC_3A-28A_n78A-n257," KDDI - R4-1915665, "TP for TR 37.716-21-21 DC_28A-41A_n78A-n257," KDDI - R4-1915666, "TP for TR 37.716-21-21 DC_28A-42A_n78A-n257," KDDI - R4-1915667, "TP for TR 37.716-21-21 DC_1A-28A_n78A-n257," KDDI - R4-1915668, "TP for TR 37.716-21-21 DC_1A-3A-41A_n78A-n257," KDDI - R4-1915669, "TP for TR 37.716-21-21 DC_1A-3A-42A_n78A-n257," KDDI - R4-1915670, "TP for TR 37.716-21-21 DC_1A-41A-42A_n78A-n257," KDDI - R4-1915671, "TP for TR 37.716-21-21 DC_3A-41A-42A_n78A-n257," KDDI - R4-1915672, "TP for TR 37.716-21-21 DC_1A-3A-18A_n78A-n257," KDDI - R4-1915673, "TP for TR 37.716-21-21 DC_1A-18A-42A_n78A-n257," KDDI - R4-1915674, "TP for TR 37.716-21-21 DC_3A-18A-42A_n78A-n257," KDDI - R4-1915675, "TP for TR 37.716-21-21 DC_3A-28A-41A_n78A-n257," KDDI - R4-1915677, "TP for TR 37.716-21-21 DC_28A-41A-42A_n78A-n257," KDDI - R4-1915676, "TP for TR 37.716-21-21 DC_3A-28A-42A_n78A-n257," KDDI - R4-1915678, "TP for TR 37.716-21-21 DC_1A-3A-28A_n78A-n257," KDDI - R4-1915679, "TP for TR 37.716-21-21 DC_1A-28A-42A_n78A-n257," KDDI - R4-1915680, "TP for TR 37.716-21-21 DC_1A-3A-41A-42A_n78A-n257," KDDI - R4-1915681, "TP for TR 37.716-21-21 DC_1A-3A-18A-42A_n78A-n257," KDDI - R4-1915682, "TP for TR 37.716-21-21 DC_3A-28A-41A-42A_n78A-n257," KDDI - R4-1915683, "TP for TR 37.716-21-21 DC_1A-3A-28A-42A_n78A-n257," KDDI - R4-1915695, "TP for TR37.716-21-21: IMD2 MSD for DC_3_n3-n77," MediaTek Inc., CHTTL - R4-1915692, "TP for TR 37.716-21-21 to include DC_2A-7C_n38A-n78A," Intel Corporation - R4-1915696, "TP for TR37.716-21-21: IMD2 MSD for DC_3_n3-n78," MediaTek Inc., CHTTL |  |  |
| 2020-02 | RAN4 #94 | R4-2001042 |  |  | Updated TR 37.716-21-21 v0.9.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.8.0 | 0.9.0 |
|  |  |  |  |  | - R4-2001064, “Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in rel-16,” LG Electronics - R4-2001066, “Introducing CR on new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16,” LG Electronics - R4-2000182, “Draft CR: Adding EN-DC configurations to DC_3-20_n28-n78,” Nokia - R4-2000263, “TP for TR 37.716-21-21: DC_2A_n66A-n78A,”Samsung - R4-2000264, “TP for TR 37.716-21-21: DC_2A-7A_n66A-n78A and DC_2A-7A-7A_n66A-n78A,” Samsung - R4-2000265, “TP for TR 37.716-21-21: DC_2A-7A-66A_n66A-n78A and DC_2A-7A-7A-66A_n66A-n78A,” Samsung - R4-2002649 TP for TR 37.716-21-21: DC_2A-66A_n66A-n78A,” Samsung - R4-2002650 TP for TR 37.716-21-21: DC_7A_n66A-n78A,” Samsung - R4-2000268, “TP for TR 37.716-21-21: DC_7A-7A_n66A-n78A,” Samsung - R4-2000269, “TP for TR 37.716-21-21: DC_7A-66A_n66A-n78A and DC_7A-7A-66A_n66A-n78A,” Samsung - R4-2000270, “TP for TR 37.716-21-21: DC_66A_n66A-n78A,” Samsung - R4-2002651, “TP for TR37.716-21-21_ DC_1A-20A_n3A-n38A,” ZTE Corporation - R4-2000482, “TP for TR37.716-21-21_ DC_1A-20A_n3A-n78A,” ZTE Corporation - R4-2002652 TP for TR37.716-21-21_ DC_1A-20A-38A_n3A-n78A ZTE Corporation - R4-2000485, “TP for TR37.716-21-21: DC_7A-20A_n3A-n78A,” ZTE Corporation - R4-2002653 TP for TR 37.716-21-21:DC_2_n7-n78 ZTE Corporation - R4-2000851, “TP for TR 37.716-21-21: UE requirements for DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78,” CHTTL - R4-2000856 TP for TR 37.716-21-21: UE requirements for DC_3-3-8_n1-n78, DC_7-7-8_n1-n78 CHTTL - R4-2001092 TP to TR 37.716.21-21: Addition of CA configuration for DC_1A_n8A-n78A Huawei - R4-2001128 TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 LG Electronics - R4-2002654 MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 LG Electronics - R4-2001998 TP for TR 37.716-21-21 to include DC_28_n7-n78 Ericsson - R4-2001999 TP for TR 37.716-21-21 to include DC_3-28_n7-n78 Ericsson - R4-2002000 TP for TR 37.716-21-21 to include DC_1-28_n7-n78 Ericsson - R4-2002001 TP for TR 37.716-21-21 to include DC_1-3-28_n7-n78 Ericsson - R4-2002002 draft CR adding configurations for 1_n7-n78, 3_n7-n78, 1-3_n7-n78, 3_n7-n78 Ericsson - R4-2002008 TP for TR 37.716-21-21 to include DC_7A-28A_n3A-n78A, DC_7C-28A_n3A-n78A Ericsson - R4-2002011 draft Rel-16 CR to 38.101-3 to add new configurations for 2_n41-n71, 66_n25-n41, 66_n41-n71, 2-66_n41-n71 to existing combinations Ericsson - R4-2002013 TP for TR 37.716-21-21 to include DC_2_n41-n66 Ericsson - R4-2002014 TP for TR 37.716-21-21 to include DC_2_n66-n71 Ericsson - R4-2002016 TP for TR 37.716-21-21 to include 66_n25-n71 Ericsson - R4-2002655 TP for TR 37.716-21-21 to include DC_2-46_n41-n66 Ericsson - R4-2002656 TP for TR 37.716-21-21 to include DC_2-66_n71-n261 Ericsson - R4-2002657 TP for TR 37.716-21-21: DC_66A_n12A-n258A Nokia - R4-2002658 TP for TR 37.716-21-21: DC_66A_n12A-n260A Nokia - R4-2002659 TP for TR 37.716-21-21: DC_66A_n12A-n261A Nokia - R4-2000762 Draft CR for TS 38.101-3: Support of n77(2A) in DC_1-8_n77-n257 SoftBank - R4-2000765 Draft CR for TS 38.101-3: Support of n77(2A) in DC_1_n77-n257, 3_n77-n257 and 8_n77-n257 SoftBank - R4-2000776 TP for TR 37.716-21-21: EN-DC_11_n77-n257 SoftBank - R4-2000777 TP for TR 37.716-21-21: EN-DC_28_n77-n257 SoftBank - R4-2000860 TP for EN-DC of 1-3-21_n78-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000861 TP for EN-DC of 1-19-42_n78-n257 for TR37.716-21-21, NTT DoCoMo - R4-2000862 TP for EN-DC of 1-21-42_n78-n257 for TR37.716-21-21, NTT DoCoMo - R4-2000863 TP for EN-DC of 19-21-42_n78-n257 for TR37.716-21-21, NTT DoCoMo - R4-2000864 TP for EN-DC of 1-3-21_n79-n257 for TR37.716-21-21, NTT DoCoMo - R4-2000865 TP for EN-DC of 1-19-42_n79-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000866 TP for EN-DC of 1-21-42_n79-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000867 TP for EN-DC of 19-21-42_n79-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000868 TP for EN-DC of 1-3-21_n77-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000869 TP for EN-DC of 1-19-42_n77-n257 for TR37.716-21-21 NTT DoCoMo - R4-2002660 TP for EN-DC of 1-21-42_n77-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000871 TP for EN-DC of 19-21-42_n77-n257 for TR37.716-21-21 NTT DoCoMo - R4-2000879 draft CR for introduce DC of LTE 2bands + NR 2band for TS 38.101-3 KDDI - R4-2000880 draft CR for introduce DC of LTE 3bands + NR 2band for TS 38.101-3 KDDI - R4-2000882 draft CR for introduce DC of LTE 4bands + NR 2band for TS 38.101-3 KDDI - R4-2001093 draft CR for EN-DC inc NR CA FR1+FR2 w xDL_2ULfor TS 38.101-3 NTT DoCoMo - R4-2001094 TP for DC_1-19_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001095 TP for DC_1-19_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001096 TP for DC_1-19_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001097 TP for DC_1-21_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001098 TP for DC_1-21_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001099 TP for DC_1-21_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001100 TP for DC_1-3_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001101 TP for DC_1-42_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001102 TP for DC_1-42_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001103 TP for DC_19-21_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001104 TP for DC_19-21_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001105 TP for DC_19-21_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001106 TP for DC_19-42_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001107 TP for DC_19-42_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001108 TP for DC_19-42_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001109 TP for DC_21-42_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001110 TP for DC_21-42_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001111 TP for DC_21-42_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001112 TP for DC_3-19_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001113 TP for DC_3-19_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001114 TP for DC_3-19_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001115 TP for DC_3-21_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001116 TP for DC_3-21_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001117 TP for DC_3-21_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001118 TP for DC_3-42_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001119 TP for DC_3-42_n78-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001120 TP for DC_3-42_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001121 TP for DC_42_n77-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2001122 TP for DC_42_n79-n257 for TR 37.716-21-21 NTT DoCoMo - R4-2002661 TP for TR 37.716-21-21 to include DC_2A_n12A-n258A Ericsson - R4-2002662 TP for TR 37.716-21-21 to include DC_2A_n12A-n260A Ericsson - R4-2002663 TP for TR 37.716-21-21 to include DC_2A_n12A-n261A Ericsson - R4-2002664 TP for TR 37.716-21-21 to include DC_2_n71-n261 Ericsson - R4-2001302 TP for 37.716-21-21 to introduce DC_2-46_n41-n71 Nokia - R4-2001303 TP for 37.716-21-21 to introduce DC_2-46-66_n41-n71 Nokia - R4-2001304 TP for 37.716-21-21 to introduce DC_46-66_n25-n41 Nokia - R4-2001305 TP for 37.716-21-21 to introduce DC_46-66_n41-n71 Nokia |  |  |
| 2020-04 | RAN4 #94-e-BIS | R4-2003224 |  |  | Updated TR 37.716-21-21 v0.10.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.9.0 | 0.10.0 |
|  |  |  |  |  | - R4-2003225 Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16 LG Electronics - R4-2003134 TP for TR 37.716-21-21: DC_1A-18A_n3A-n77A kDDI - R4-2003135 TP for TR 37.716-21-21: DC_1A-28A_n3A-n77A KDDI - R4-2003136 TP for TR 37.716-21-21: DC_1A-41A_n3A-n77A and DC_1A-41C_n3A-n77A KDDI - R4-2003137 TP for TR 37.716-21-21: DC_1A-41A_n3A-n78A and DC_1A-41C_n3A-n78A KDDI - R4-2003138 TP for TR 37.716-21-21: DC_1A-41A_n28A-n77A and DC_1A-41C_n28A-n77A Samsung - R4-2003139 TP for TR 37.716-21-21: DC_1A-41A_n28A-n78A and DC_1A-41C_n28A-n78A Samsung - R4-2003140 TP for TR 37.716-21-21: DC_3A-41A_n28A-n77A and DC_3A-41C_n28A-n77A Samsung - R4-2003141 TP for TR 37.716-21-21: DC_3A-41A_n28A-n78A and DC_3A-41C_n28A-n78A Samsung - R4-2003142 TP for TR 37.716-21-21: DC_18A_n3A-n77A” Samsung - R4-2003143 TP for TR 37.716-21-21: DC_18A-41A_n3A-n77A and DC_18A-41C_n3A-n77A Samsung - R4-2003144 TP for TR 37.716-21-21: DC_18A-41A_n3A-n78A and DC_18A-41C_n3A-n78A - R4-2005055 TP for TR 37.716-21-21: DC_28A_n3A-n77A - R4-2005056 TP for TR 37.716-21-21: DC_41_n3-n77 - R4-2005057 TP for TR 37.716-21-21: DC_41_n3-n78 - R4-2005058 TP for TR 37.716-21-21: DC_41_n28-n77 - R4-2005059 TP for TR 37.716-21-21: DC_41_n28-n78 - R4-2003150 TP for TR 37.716-21-21: DC_1-3-41_n28-n77 - R4-2003151 TP for TR 37.716-21-21: DC_1-3-41_n28-n78 - R4-2003153 TP for TR 37.716-21-21: DC_1-18-41_n3-n77 - R4-2003154 TP for TR 37.716-21-21: DC_1-18-41_n3-n78 - R4-2003163 TP for TR 37.716-21-21: DC_2A-7C_n66A-n78A - R4-2003164 TP for TR 37.716-21-21: DC_2A-7C-66A_n66A-n78A - R4-2003165 TP for TR 37.716-21-21: DC_7C_n66A-n78A - R4-2003166 TP for TR 37.716-21-21: DC_7C-66A_n66A-n78A - R4-2003228 TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16 - R4-2005060 MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16 - R4-2005061 TP for TR 37.716-21-21: EN-DC_1_n28-n77 - R4-2005062 TP for TR 37.716-21-21: EN-DC_3_n28-n77 - R4-2005063 TP for TR 37.716-21-21: EN-DC_8_n28-n77 - R4-2003507 TP for TR 37.716-21-21: support of DC_3_n1-n257, DC_3-3_n1-n257, DC_7_n1-n257, DC_7-7_n1-n257 - R4-2003513 TP for TR 37.716-21-21: support of DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-3-7-7_n1-n257 - R4-2005064 TP to TR 37.716-21-21 DC_20A_n78A-n92A - R4-2005065 TP for TR37.716-21-21_ DC_8A_n40A-n41A - R4-2003794 TP for TR37.716-21-21_ DC_39A_n40A-n41A - R4-2003795 TP for TR37.716-21-21_ Updated DC_3A_n40A-n79A and DC_3A_n41A-n79A - R4-2003796 TP for TR37.716-21-21_ Updated DC_8A_n40A-n79A and DC_8A_n41A-n79A - R4-2003797 TP for TR37.716-21-21_ Updated DC_39A_n40A-n79A and DC_39A_n41A-n79A - R4-2005066 TP for TR 37.716-21-21: DC_1A_n41A-n78A - R4-2005067 TP for TR 37.716-21-21: DC_3A_n41A-n78A - R4-2005068 TP for TR 37.716-21-21: DC_20A_n41A-n78A - R4-2003943 TP for TR 37.716-21-21: DC_20A_n75A-n78A\DC_20A_n75A-n78(2A) - R4-2005070 TP for 37.716-21-21 for DC_1A_n8A-n40A - R4-2004066 TP for 37.716-21-21 for DC_1A_n40A-n78(2A) - R4-2005071 TP for TR 37.716-21-21 DC_20A_n7A-n28A - R4-2005072 TP for 37.716-21-21 to introduce DC_28_n40-n78 - R4-2004228 TP for 37.716-21-21 to introduce DC-1-3_n40-n78 - R4-2004229 TP for 37.716-21-21 to introduce DC-1-28_n40-n78 - R4-2004230 TP for 37.716-21-21 to introduce DC-3-28_n40-n78 - R4-2004231 TP for 37.716-21-21 to introduce DC-1-3-28_n40-n78 - R4-2004508 TP to TR 37.716-21-11: CA_3_n7-n28 - R4-2004599 TP for TR 37.716-21-21 to include DC_1-7_n7-n78 - R4-2004600 TP for TR 37.716-21-21 to include DC_3-7_n7-n78 - R4-2004601 TP for TR 37.716-21-21 to include DC_7-28_n7-n78 - R4-2004602 TP for TR 37.716-21-21 to include DC_1-3-7_n7-n78 - eTP for TR 37.716-21-21 to include DC_1-7-28_n7-n78 - R4-2004604 TP for TR 37.716-21-21 to include DC_3-7-28_n7-n78 - R4-2004605 TP for TR 37.716-21-21 to include DC_1-3-7-28_n7-n78 - R4-2005074 TP for TR 37.716-21-21 to include DC_1A_n28A-n40A - R4-2004617 TP for TR 37.716-21-21 to include DC_3A_n1A-n40A - R4-2004618 TP for TR 37.716-21-21 to include DC_3A_n8A-n40A - R4-2005075 TP for TR 37.716-21-21 to include DC_3A_n8A-n78A - R4-2004620 TP for TR 37.716-21-21 to include DC_3A_n28A-n40A - R4-2005076 TP for TR 37.716-21-21 to include DC_7A_n1A-n40A - R4-2005077 TP for TR 37.716-21-21 to include DC_7A_n8A-n40A - R4-2005078 TP for TR 37.716-21-21 to include DC_7A_n8A-n78A - R4-2004624 TP for TR 37.716-21-21 to include DC_7A_n28A-n40A - R4-2003152 TP for TR 37.716-21-21: DC_1-3-41_n28-n257 - R4-2003155 TP for TR 37.716-21-21: DC_1-18-41_n3-n257 - R4-2003156 TP for TR 37.716-21-21: DC_1-41_n3-n257 - R4-2003452 TP for TR 37.716-21-21: EN-DC_1-11_n77-n257 - R4-2003455 TP for TR 37.716-21-21: EN-DC_3-28_n77-n257 - R4-2003458 TP for TR 37.716-21-21: EN-DC_8-11_n77-n257 - R4-2003792 TP for TR37.716-21-21_ DC_3A_n40A-n258A |  |  |
| 2020-04 | RAN4 #94-e- BIS | R4-2005866 |  |  | Updated TR 37.716-21-21 v0.11.0: LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16," | 0.10.0 | 0.11.0 |
| 2020-05 | RAN4 #95-e | R4-2006726 |  |  | TR37.716-21-21 v1.0.0 | 0.11.0 | 1.0.0 |
|  |  |  |  |  | Approved TP and endorsed CRs and Revised WID - R4-206727 Revised WID on LTE (xDL/UL x=1.2,3,4) with NR 2 bands (2DL/1UL) EN DC in Rel-16 LG Electronics - R4-2006410 TP for TR 37.716-21-21 DC_18_n3-n77 update - R4-2006412 TP for TR 37.716-21-21 DC_41_n3-n77 update - R4-2006413 TP for TR 37.716-21-21 DC_41_n3-n78 update - R4-2006589 TP to TR 37.716-21-11: DC_20_n1-n7 - R4-2006590 TP to TR 37.716-21-11: DC_3-20_n1-n7 - R4-2006675 TP for TR 37.716-21-21: EN-DC_1-3_n28-n77 - R4-2006677 TP for TR 37.716-21-21: EN-DC_1-8_n3-n28 - R4-2006678 TP for TR 37.716-21-21: EN-DC_1-8_n28-n77 - R4-2006680 TP for TR 37.716-21-21: EN-DC_3-8_n28-n77 - R4-2008364 TP for TR 37.716-21-21: EN-DC_42_n28-n77 - R4-2006729 TP on summary of self-interference analysis for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16 - R4-2006936 TP to TR 37.716-21-21 DC_20A_n78(2A)-n92A - R4-2007013 TP for TR37.716-21-21: DC_1A-3A-20A_n38A-n78A - R4-2007014 TP for TR37.716-21-21_ DC_3A-20A_n38A-n78A - R4-2008365 TP for TR 37.716-21-21: DC_2A_n38A-n78A - R4-2008366 TP for TR 37.716-21-21: DC_66A_n38A-n78A - R4-2007329 TP for TR 37.716-21-21: DC_2A-66A_n38A-n78A - R4-2007330 TP for TR 37.716-21-21: DC_5_n7-n78 - R4-2007331 TP for TR 37.716-21-21: DC_12_n7-n78 - R4-2007332 TP for TR 37.716-21-21: DC_66_n7-n78 - R4-2007333 TP for TR 37.716-21-21: DC_2_n7-n78 - R4-2008367 TP for TR 37.716-21-21: DC_1A_n75A-n78A\DC_1A_n75A-n78(2A) - R4-2008368 TP for TR 37.716-21-21: DC_3A_n75A-n78A\DC_3A_n75A-n78(2A) - R4-2008029 TP for TR 37.716-21-21: DC_1A-3A_n41A-n78A - R4-2008030 TP for TR 37.716-21-21: DC_1A-20A_n41A-n78A - R4-2008031 TP for TR 37.716-21-21: DC_3A-20A_n41A-n78A - R4-2008032 TP for TR 37.716-21-21: DC_1A-7A_n3A-n78A - R4-2008033 TP for TR 37.716-21-21: DC_3A-20A_n7A-n28A/ DC_3C-20A_n7A-n28A - R4-2008034 TP for TR 37.716-21-21: DC_1A-3A-20A_n41A-n78A - R4-2008369 TP for TR 37.716-21-21: DC_(n)41AA-n78A\ DC_(n)41CA-n78A\ DC_(n)41DA-n78A - R4-2006398 TP for TR 37.716-21-21 DC_1_n28-n257 - R4-2006399 TP for TR 37.716-21-21 DC_1-3_n28-n257 - R4-2006400 TP for TR 37.716-21-21 DC_1-3-41_n77-n257 - R4-2006401 TP for TR 37.716-21-21 DC_1-3-41-42_n77-n257 - R4-2006402 TP for TR 37.716-21-21 DC_1-3-42_n77-n257 - R4-2006403 TP for TR 37.716-21-21 DC_1-41_n28-n257 - R4-2006404 TP for TR 37.716-21-21 DC_1-41_n77-n257 - R4-2006405 TP for TR 37.716-21-21 DC_1-41-42_n77-n257 - R4-2006406 TP for TR 37.716-21-21 DC_3_n28-n257 - R4-2006407 TP for TR 37.716-21-21 DC_3-41_n28-n257 - R4-2006408 TP for TR 37.716-21-21 DC_3-41_n77-n257 - R4-2006409 TP for TR 37.716-21-21 DC_3-41-42_n77-n257 - R4-2006411 TP for TR 37.716-21-21 DC_18-41_n3-n257 - R4-2006414 TP for TR 37.716-21-21 DC_41_n3-n257 - R4-2005415 TP for TR 37.716-21-21 DC_41_n28-n257 - R4-2006416 TP for TR 37.716-21-21 DC_41_n77-n257 - R4-2006417 TP for TR 37.716-21-21 DC_41-42_n77-n257 - R4-2006669 Draft CR for TS 38.101-3: Support of n77(2A) in DC_1-8-11_n77-n257 |  |  |