3GPP TSG-RAN WG4 #95-e	R4-2006937

Online, 25th May – 5th June, 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 38.716-02-00 CA_n78(2A)-n92A

Agenda item:			8.2.2

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 38.716-02-00 to introduce CA_n78(2A)-n92A.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 6.61	CA_n78-n92

### 6.61.1		Common for 1 band UL and 2 bands UL

#### 6.61.1.1		Operating bands for CA

Table 6.61.1.1-1: CA band combination of band n78+n92

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n92 | 832 MHz | – | 862 MHz | 1432 MHz | – | 1517 MHz | FDD |

#### 6.61.1.2		Channel bandwidths per operating band for CA

Table 6.61.1.2-1: Supported bandwidths per CA band combination of band n78+n92

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n78A-n92A | CA_n78A-n92A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n92 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78(2A)-n92A | CA_n78A-n92A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n92 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.61.1.3		UE co-existence studies

Table 6.61.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n78-n92.

Table 6.61.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n92 | 832 | 862 | 1432 | 1517 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |

Based on above table, the 4th harmonics of band n92 UL will fall into the band n78 Rx.

Table 6.61.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |
| n92 | 832 | 862 | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n92 and n78.

#### 6.61.1.4		∆TIB and ∆RIB values

For CA_n78-n92, the (TIB,c and (RIB,c values are given in the tables below.

Table 6.61.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n78-n92 | n78 | 0.8 |
|  | n92 | 0.6 |

Table 6.61.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n78-n92 | n78 | 0.5 |
|  | n92 | 0 |

#### 6.61.1.5		REFSENS requirements

The specific MSD requirement can follow the requirements of CA_n20-n78 as defined in 6.49.1.5.

Table 6.61.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n92 | n784,5 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 | 2.0 | 1.5 | 1.0 |
| fULLB=fDLHB0.50.1 NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL NR-ARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.61.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n92 | n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |

For CA configurations of both CA_n78A-n92A and CA_n78(2A)-n92A, the same requirements defined in this sub-clause apply.

#### 6.61.1.6		Out-of-band blocking requirements

### The combination is subject to be allowed for out-of-band blocking exceptions when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 6.61.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n78-n92

Table 6.61.1.6-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when , where and are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. and are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

### 6.61.2		Specific for 2 bands UL CA

#### 6.61.2.1		UE co-existence studies

Band n78 +Band n92 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis can refer to table 6.49.2.1-1 for CA_n20-n78.

Based on abovementioned table, no IMD issue is observed.

The protected bands required for the 2UL bands CA configuration can refer to table 6.49.2.1-2 by replacing n20 with n92 since n20 and n92 have the exact same UL frequency.

#### 6.61.2.2		REFSENS requirements

There is no specific MSD requirement.

----- Unchanged sections omitted -----

# Reference