3GPP TSG-RAN WG4 Meeting # 95-e                                  R4-200xxxx

Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.716-03-02: CA_n28A-n41A-n78A with two UL bands with two UL bands

Agenda Item:	8.11.2

Document for:	Approval

# Introduction

The WID for NR DC was updated in RAN #87 meeting. This contribution provides a TP for TR 38.716-03-02 to finish the UE RF requirements for the band combination.

# References

[1]	RP-200171, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL”, ZTE

# Text Proposal

<TP for TR 38.716-03-02>

## 5.1.x	CA_n28A-n41A-n78A

- 5.1.x.1	 Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n41-n78 | n28 | 703MHz | – | 748MHz | 758MHz | – | 803MHz | FDD |
|  | n41 | 2496MHz | – | 2690MHz | 2496MHz | – | 2690MHz | TDD |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

- 5.1.x.2 	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

- 5.1.x.3	 Co-existence studies

For UE coexistence study of Band n28 + Band n41, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.x.3-1

Table 5.1.x.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1748 | 1987 | 3199 | 3438 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1284 | 1000 | 4244 | 4677 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3902 | 4186 | 5695 | 6128 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 581 | 252 | 6740 | 7367 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4605 | 4934 | 8191 | 8818 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3974 | 3496 | 6398 | 6876 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10057 | 9236 | 496 | 122 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6664 | 5992 | 2748 | 3271 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10687 | 11508 | 5308 | 5682 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8894 | 9566 | 7101 | 7624 |

For UE coexistence study of Band n28 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.x.3-2.

Table 5.1.x.3-2 Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

For UE coexistence study of Band n41 + Band n78, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.x.3-3.

Table 5.1.x.3-3 Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9984 | 10760 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12480 | 13450 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1304 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2608 | 1220 | 11592 | 12980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 | 10510 | 7460 | 6184 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 | 4520 | 1470 | 112 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

2nd, 4th order IMD generated by dual uplink of Band n28 + Band n41 may fall into own Rx of Band n78.

2nd order IMD generated by dual uplink of Band n28 + Band n78 may fall into own Rx of Band n41.

2nd, 5th order IMD generated by dual uplink of Band n41 + Band n78 may fall into own Rx of Band n28.

- 5.1.x.4 	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and two ULs of Band n28, n41 and n78, the TIB,c and RIB,c values are shown in table 5.1.x.4-1 and table 5.1.x.4-2, respectively.

Table 5.1.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n41-n78 | n28 | 0.5 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28-n41-n78 | n28 | 0.2 |
|  | n41 | 0 |
|  | n78 | 0.5 |

5.1.x.5	 REFSENS requirements

MSD requirements for CA_n28-n41-n78 are shown below. The ENDC requirements of DC_28A-41A_n78A and DC_7A_n28A-n78A can be reused.

Table 5.1.x.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n41A-n78A | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2y |
|  | n41 | 2565 | 5 | 25 | 2565 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | TDD | IMD2z |
| NOTE y: This band is subject to IMD5 also which MSD is not specified. NOTE z: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

<End of TP >