3GPP TSG-RAN WG4 Meeting # 95-e                                                              R4-2007558

Electronic Meeting, 25 May – 5 June, 202

- Source:	ZTE

- Title:	Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL

- Document for:	Approval

Agenda Item:	8.11.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Information about Work Items can be found at http://www.3gpp.org/Work-Items

# Title:  Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL

## Acronym:  NR_CADC_R16_3BDL_2BUL

## Unique identifier: 	 830096

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

{For a Feature: list here the children Building Blocks (optional) and Work Tasks (optional)}

{For a Building Block: list here the parent Feature (mandatory) and children Work Tasks (optional)}

{For a Work Task: list here the parent Building Block (mandatory)}

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830096 | Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | Parent WID |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830196 | Core part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | Child WID |
| 830296 | Perf. part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | Child WID |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

For NR CA, all new NR CA configurations including 3 different bands DL with 2 different bands UL will be defined under this WI.

The NR CA configurations will be introduced in a release independent manner based on TS38.307, which will be updated depending on newly introduced NR CA configurations.

The preconditions to propose NR CA configurations including Intra band CA for 3 different bands DL 2 different bands UL in rel-16 are as follows.

- Requirements for all concerning bands shall be completed and specified in advanced.

Constituent NR band and NR Intra band CA shall be completed and specified in advance. Example 1: If the following configuration is proposed,

| CA configuration | Uplink CA configuration |
| --- | --- |
| CA_n1A-n2A-n3A | CA_n1A-n2A, CA_n1A-n3A_CA_n2A-n3A |

Example 2: If the following configuration is proposed,

| CA configuration | Uplink CA configuration |
| --- | --- |
| CA_n1C-n2A-n3A | CA_n1C-n2A, CA_n1C-n3A, CA_n2A-n3A |

For NR DC, all new NR DC configurations including Intra band CA for 3 different bands DL with 2 different bands UL will be defined under this WI. NR DC configurations, however, will not be accommodated until the outcome of Rel15 late drop handling becomes clear.

For NR DC, all DC configurations consisting of 3 different bands DL with 2 different bands UL will be defined under this WI.

Example 3: If the following configuration is proposed,

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n2A-n3A | DC_n1A-n3A, DC_n2A-n3A |

- NR CA of 2 different bands DL with 1 band UL of CA_n1A-n2A shall be specified in advance.

- NR DC of 2 different bands DL with 2 different bands UL of DC_n1A-n3A, DC_n2A-n3A shall be completed and specified in advance.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

For NR CA, all new NR CA configurations including 3 different bands DL with 2 different bands UL will be defined under this WI.

- Specify the band-combination specific RF requirements for all listed NR CA configurations including 3 different bands DL with 2 different bands UL.

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Analyse combinations that have self-desensitization due to following reasons:

  - TX Harmonic and/or intermodulation overlap of receive band

  - TX signal overlap of receiver harmonic frequency

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons

- For the combination where self-desensitization exists, specify at least needed

  - ∆TIB, c and ∆RIB, c

  - Reference sensitivity exceptions including MSD test cases

- Add conformance testing in RAN5 specifications (to follow at a later stage)

An overview table of these NR CA configurations is provided here:

- CA configurations for 3 different bands DL with 2 band UL 
*Unless otherwise stated, the number of UL CC is one for all band in the configurations.

- Table 1-1: Individual combination names, proponents and supporting companies for NR CA configurations for 3 different bands DL with 2 band ULs within FR1, or within FR2 or including FR2

| NR CA configuration | Uplink Configuration | BCS | contactname, company | contact email | other supporting companies(min. 3) | status(new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (ongoing) DL_n1A-n8A_UL_n1A-n8A_BCS0 (ongoing) DL_n3A-n8A_UL_n3A-n8A_BCS0 (ongoing) DL_n1A-n3A_UL_n1A-n3A_BCS0 |
| CA_n1A-n3A-n28A | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, BT plc, Telstra, LG, Oppo | Stopped | (ongoing) DL_n1A-n28A_UL_n1A-n28A_BCS0 (ongoing) DL_n3A-n28A_UL_n3A-n28A_BCS0 (ongoing) DL_n1A-n3A_UL_n1A-n3A_BCS0 |
| CA_n1A-n3A-n41A | CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A | 0 | Zhang Peng, Huawei | zhangpeng169@huawei.com | HiSilicon, Neul, CATT | completed | (completed) DL_n1A-n41A_ UL_n1A-n41A_BCS0 (completed) DL_n3A-n41A_ UL_n3A-n41A_BCS0 (completed) DL_n1-n3A_UL_n1A-n3A_BCS0 (completed) DL_n1A-n3-n41A _BCS0 |
| CA_n1A-n3A-n50A | CA_n1A-n3A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (ongoing) DL_n1A-n3A_UL_n1A_BCS0 (ongoing) DL_n1A-n3A_UL_n3A_BCS0 (ongoing) DL_n1A-n50A_UL_n1A_BCS0 (ongoing) DL_n3A-n50A_UL_n3A_BCS0 |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | 0 | Liu Bo China Telelom | Liubo1.bri@chinatelecom.cn | China Unicom, China Telecom, Samsung, ZTE, Tejet | Completed | (ongoing) DL_n1A-n3A-n78A_BCS0 (ongoing) DL_n1A-n3A_UL_n1A-n3A_BCS0 (completed) DL_n1A-n78A_UL_n1A-n78A_BCS0 (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | 0 | Bo Liu, China Telecom | liubo1.bri@chinatelecom.cn | CATT, ZTE, Huawei, HiSilicon | Stopped | (ongoing) DL_n1A-n5A _UL_n1A-n5A_BCS0 (completed)DL_n1A-n78A_UL_n1A-n78A _BCS0 (completed)DL_n5A-n78A_UL_n5A-n78A _BCS0 (new) DL_n1A-n5A-n78A _BCS0 |
| CA_n1A-n7A-n28A | CA_n1A-n28A CA_n1A-n7A CA_n7A-n28A | 0 | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | completed | (completed)DL_n1A-n7A-n28A_BCS0, (completed)DL_n1A-n28A_UL_n1A-n28A_BCS0, (completed)DL_n1A-n7A_UL_n1A-n7A_BCS0, (completed)DL_n7A-n28A_UL_n7A-n28A_BCS0, |
| CA_n1A-n7A-n78A | CA_n1A-n78A CA_n1A-n7A CA_n7A-n78A | 0 | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | completed | (completed)DL_n1A-n7A-n78A_BCS0, (completed)DL_n1A-n78A_UL_n1A-n78A_BCS0, (completed)DL_n1A-n7A_UL_n1A-n7A_BCS0,(completed)DL_n7A-n78A_UL_n7A-n78A_BCS0, |
| CA_n1A-n7A-n78(2A) | CA_n1A-n7A CA_n1A-n78A CA_n7A-n78A | 0 | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | completed | (ongoing)DL_n1A-n7A-n78A_UL_n1A-n7A_BCS0 (completed)DL_n1A-n7A_UL_n1A-n7A_BCS0 (Completed)DL_n1A-n7A-n78(2A)_UL_n1A-n78A_BCS0 (Completed)DL_n1A-n7A-n78A_UL_n1A-n78A_BCS0, (Completed) DL_n1A-n7A-n78A_UL_n1A-n78A_BCS0, (Completed)DL_n1A-n78(2A)_UL_n1A-n78A_BCS0 (Completed)DL_n1A-n7A-n78(2A)_UL_n7A-n78A_BCS0, (Completed)DL_n1A-n7A-n78A_UL_n7A-n78A_BCS0, (Completed)DL_n1A-n7A-n78A_UL_n7A-n78A_BCS0, (Completed)DL_n7A-n78(2A)_UL_n7A-n78A_BCS0 |
| CA_n1A-n8A-n50A | CA_n1A-n8A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (new) DL_n1A-n8A_UL_n1A-n8A_BCS0 (new) DL_n1A-n8A_UL_n1A_BCS0 (new) DL_n1A-n8A_UL_n8A_BCS0 (new) DL_n1A-n50A_UL_n1A_BCS0 (new) DL_n8A-n50A_UL_n8A_BCS0 |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | China Unicom, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n1A-n8A-n78A_BCS0 (new) DL_n1A-n8A_UL_n1A-n8A_BCS0 (completed) DL_n1A-n78A_UL_n1A-n78A_BCS0 (completed) DL_n8A-n78A_UL_n8A-n78A_BCS0 |
| CA_n1A-n28A-n50A | CA_n1A-n28A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (new) DL_n1A-n28A_UL_n1A-n28A_BCS0 (new) DL_n1A-n28A_UL_n1A_BCS0 (new) DL_n1A-n28A_UL_n28A_BCS0 (new) DL_n1A-n50A_UL_n1A_BCS0 (ongoing) DL_n28A-n50A_UL_n28A_BCS0 |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, BT plc, Telstra, LG, Oppo | Stopped | (completed) DL_n1A-n78A_UL_n1A-n78A_BCS0 (ongoing) DL_n28A-n78A_UL_n28A-n78A_BCS0 (ongoing) DL_n1A-n28A_UL_n1A-n28A_BCS0 |
| CA_n1A-n28A-n78(2A) | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | 0 | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | Stopped | (ongoing)DL_n1A-n28A-n78A_UL_n1A-n28A_BCS0 (completed)DL_n1A-n28A_UL_n1A-n28A_BCS0 (ongoing)DL_n1A-n28A-n78(2A)_UL_n1A-n78A_BCS0 (ongoing)DL_n1A-n28A-n78A_UL_n1A-n78A_BCS0, (ongoing) DL_n1A-n28A-n78A_UL_n1A-n78A_BCS0, (ongoing)DL_n1A-n78(2A)_UL_n1A-n78A_BCS0 (ongoing)DL_n1A-n28A-n78(2A)_UL_n28A-n78A_BCS0, (ongoing)DL_n1A-n28A-n78A_UL_n28A-n78A_BCS0, (ongoing)DL_n1A-n28A-n78A_UL_n28A-n78A_BCS0, (ongoing)DL_n28A-n78(2A)_UL_n28A-n78A_BCS0 |
| CA_n1A-n50A-n78A | CA_n1A-n78A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (completed) DL_n1A-n78A_UL_n1A-n78A_BCS0 (completed) DL_n1A-n78A_UL_n1A_BCS0 (completed) DL_n1A-n78A_UL_n78A_BCS0 (ongoing) DL_n50A-n78A_UL_n78A_BCS0 (ongoing) DL_n1A-n50A_UL_n1A_BCS0 |
| CA_n2A-n5A-n30A | CA_n2A-n5A CA_n5A-n30A CA_n2A-n30A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n5A_UL_n2A_UL_n2A-n5A_BCS0 DL_n2A-n30A_UL_n2A-n30A_BCS0 DL_n5A-n30A_UL_n5A-n30A_BCS0 |
| CA_n2(2A)-n5A-n30A | CA_n2A-n2A CA_n5A-n30A CA_n2A-n30A CA_n2A-n5A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n5A_UL_n2A-n5A_BCS0 DL_n2A-n2A-n5A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n30A_UL_n2A-n30A_BCS0 DL_n2A-n2A-n30A_UL_n2A-n2A_BCS0 DL_n2A-n5A-n30A_UL_n2A-n5A_BCS0 DL_n2A-n5A-n30A_UL_n5A-n30A_BCS0 DL_n2A-n5A-n30A_UL_n2A-n30A_BCS0 |
| CA_n2A-n5A-n66A | CA_n2A-n66A CA_n5A-n66A CA_n2A-n5A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson, Nokia, Qualcomm, Samsung | Stopped | DL_n2A-n66A_UL_n66A DL_n2A-n66A_UL_n2A DL_n5A-n66A_UL_n66A DL_n5A-n66A_UL_n5A DL_n2A-n5A_UL_n2A DL_n2A-n5A_UL_n5A |
| CA_n2A-n5A-n66(2A) | CA_n2A-n66A CA_n5A-n66A CA_n2A-n5A CA_n66A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson, Nokia, Qualcomm, Samsung | Stopped | DL_n2A-n66A_UL_n66A DL_n2A-n66A_UL_n2A DL_n5A-n66A_UL_n66A DL_n5A-n66A_UL_n5A DL_n2A-n5A_UL_n2A DL_n2A-n5A_UL_n5A DL_n66A-n66A_UL-n66A |
| CA_n2(2A)-n5A-n66A | CA_n2A-n66A CA_n5A-n66A CA_n2A-n5A CA_n2(2A) | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson, Nokia, Qualcomm, Samsung | Stopped | DL_n2A-n66A_UL_n66A DL_n2A-n66A_UL_n2A DL_n5A-n66A_UL_n66A DL_n5A-n66A_UL_n5A DL_n2A-n5A_UL_n2A DL_n2A-n5A_UL_n5A DL_n2A-n2A_UL-n2A |
| CA_n2(2A)-n5A-n66(2A) | CA_n2A-n2A CA_n66A-n66A CA_n2A-n5A CA_n2A-n66A CA_n5A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n5A-n66A_UL_n2A_BCS0 DL_n2A-n2A-n5A-n66A_UL_n5A_BCS0 DL_n2A-n2A-n5A-n66A_UL_n66A_BCS0 DL_n2A-n2A-n66A-n66A_UL_n2A_BCS0 DL_n2A-n2A-n66A-n66A_UL_n66A_BCS0 DL_n2A-n5A-n66A-n66A_UL_n2A_BCS0 DL_n2A-n5A-n66A-n66A_UL_n5A_BCS0 DL_n2A-n5A-n66A-n66A_UL_n66A_BCS0 |
| CA_n2A-n5A-n66(3A) | CA_n2A-n66A CA_n5A-n66A CA_n66A-n66A CA_n2A-n5A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n5A-n66A-n66A_UL_n2A-n5A_BCS0 DL_n2A-n5A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n2A-n5A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n2A-n5A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n5A-n66A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n66A-n66A-n66A_UL_n66A-66A_BCS0 |
| CA_n2A-n5A-n260A | CA_n2A-n5A CA_n2A-n260A CA_n5A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n5A_UL_n2A-n5A_BCS0 DL_n5A-n260A_UL_n5A_UL_n5A-n260A_BCS0 DL_n2A-n260A_UL_n2A_UL_n2A-n260A_BCS0 |
| CA_n2(2A)-n5A-n260A | CA_n5A-n260A CA_n2A-n2A CA_n2A-n5A CA_n2A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n5A_UL_n2A-n5A_BCS0 DL_n2A-n2A-n5A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n260A_UL_n2A-n260A_BCS0 DL_n2A-n2A-n260A_UL_n2A-n2A_BCS0 DL_n2A-n5A-n260A_UL_n2A-n5A_BCS0 DL_n2A-n5A-n260A_UL_n5A-n260A_BCS0 DL_n2A-n5A-n260A_UL_n2A-n260A_BCS0 |
| CA_n2A-n5A-n260M | CA_n2A-n5A CA_n2A-n260M CA_n5A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n5A_UL_n2A-n5A_BCS0 DL_n5A-n260M_UL_n5A_UL_n5A-n260M_BCS0 DL_n2A-n260M_UL_n2A_UL_n2A-n260M_BCS0 |
| CA_n2(2A)-n5A-n260M | CA_n5A-n260M CA_n2A-n2A CA_n2A-n5A CA_n2A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n5A_UL_n2A-n5A_BCS0 DL_n2A-n2A-n5A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n260M_UL_n2A-n260M_BCS0 DL_n2A-n2A-n260M_UL_n2A-n2A_BCS0 DL_n2A-n5A-n260M_UL_n2A-n5A_BCS0 DL_n2A-n5A-n260M_UL_n5A-n260M_BCS0 DL_n2A-n5A-n260M_UL_n2A-n260M_BCS0 |
| CA_n2A-n30A-n66A | CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n30A_UL_n2A-n30A_BCS0 DL_n2A-n66A_UL_n2A-n66A_BCS0 DL_n30A-n66A_UL_n30A-n66A_BCS0 |
| CA_n2A-n30A-n66(2A) | CA_n2A-n30A CA_n30A-n66A CA_n66A-n66A CA_n2A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n30A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n30A-n66A_UL_n30A-n66A_BCS0 DL_n2A-n30A-n66A UL_n2A-n30A_BCS0 DL_n2A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A UL_n66A-66A_BCS0 DL_n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n66A-n66A_UL_n66A-n66A_BCS0 |
| CA_n2(2A)-n30A-n66 | CA_n2A-n2 CA_n2A-n30A CA_n30A-n66A CA_n2A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n30A_UL_n2A_BCS0 DL_n2A-n2A-n30A_UL_n30A_BCS0 DL_n2A-n2A-n66_UL_n66A_BCS0 DL_n2A-n2A-n66_UL_n2A_BCS0 DL_n2A-n30A-n66_UL_n2A_BCS0 DL_n2A-n30A-n66_UL_n30A_BCS0 DL_n2A-n30A-n66_UL_n66A_BCS0 |
| CA_n2A-n30A-n66(3A) | CA_n2A-n66A CA_n30A-n66A CA_n66A-n66A CA_n2A-n30A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n30A-n66A-n66A_UL_n2A-n30A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n30A-n66A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n66A-n66A-n66A_UL_n66A-66A_BCS0 |
| CA_n2A-n30A-n260A | CA_n2A-n30A CA_n30A-n260A CA_n2A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n30A_UL_n2A-n30A_BCS0 DL_n2A-n260A_UL_n2A-n260A_BCS0 DL_n30A-n260A_UL_n30A-n260A_BCS0 |
| CA_n2(2A)-n30A-n260A | CA_n2A-n2A CA_n30A-n260A CA_n2A-n260A CA-n2A-n30A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n30A_UL_n2A-n30A_BCS0 DL_n2A-n2A-n30A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n260A_UL_n2A-n260A_BCS0 DL_n2A-n2A-n260A_UL_n2A-n2A_BCS0 DL_n2A-n30A-n260A_UL_n2A-n30A_BCS0 DL_n2A-n30A-n260A_UL_n30A-n260A_BCS0 DL_n2A-n30A-n260A_UL_n2A-n260A_BCS0 |
| CA_n2(2A)-n30A-n66(2A) | CA_n2A-n2A CA_n66A-n66A CA_n2A-n30A CA_n2A-n66A CA_n30A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n30A-n66A_UL_n2A-n30A_BCS0 DL_n2A-n2A-n30A-n66A_UL_n30A-n66A_BCS0 DL_n2A-n2A-n30A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n2A-n30A-n66A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n2A-n66A-n66A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n2A-n30A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n2A-n30A-n66A-n66A_UL_n2A-n66A_BCS0 |
| CA_n2A-n30A-n260M | CA_n2A-n30A CA_n30A-n260M CA_n2A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n30A_UL_n2A-n30A_BCS0 DL_n2A-n260M_UL_n2A-n260M_BCS0 DL_n30A-n260M_UL_n30A-n260M_BCS0 |
| CA_n2(2A)-n30A-n260M | CA_n2A-n2A CA_n30A-n260M CA_n2A-n260M CA-n2A-n30M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n30A_UL_n2A-n30A_BCS0 DL_n2A-n2A-n30A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n260M_UL_n2A-n260M_BCS0 DL_n2A-n2A-n260M_UL_n2A-n2A_BCS0 DL_n2A-n30A-n260M_UL_n2A-n30A_BCS0 DL_n2A-n30A-n260M_UL_n30A-n260M_BCS0 DL_n2A-n30A-n260M_UL_n2A-n260M_BCS0 |
| CA_n2A-n66A-n260A | CA_n2A-n66A CA_n66A-n260A CA_n2A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n260A_UL_n2A-n260A_BCS0 DL_n66A-n260A_UL_n66A-n260A_BCS0 |
| CA_n2(2A)-n66A-n260A | CA_n2A-n2A CA_n66A-n260A CA_n2A-n260A CA-n2A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n2A-n66A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n260A_UL_n2A-n260A_BCS0 DL_n2A-n2A-n260A_UL_n2A-n2A_BCS0 DL_n2A-n66A-n260A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n260A_UL_n66A-n260A_BCS0 DL_n2A-n66A-n260A_UL_n2A-n260A_BCS0 |
| CA_n2A-n66A-n260M | CA_n2A-n66A CA_n66A-n260M CA_n2A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n260M_UL_n5A-n260M_BCS0 DL_n66A-n260M_UL_n66A-n260M_BCS0 |
| CA_n2(2A)-n66A-n260M | CA_n2A-n2A CA_n66A-n260M CA_n2A-n260M CA-n2A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n2A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n2A-n66A_UL_n2A-n2A_BCS0 DL_n2A-n2A-n260M_UL_n2A-n260M_BCS0 DL_n2A-n2A-n260M_UL_n2A-n2A_BCS0 DL_n2A-n66A-n260M_UL_n2A-n66A_BCS0 DL_n2A-n66A-n260M_UL_n66A-n260M_BCS0 DL_n2A-n66A-n260M_UL_n2A-n260M_BCS0 |
| CA_n2A-n66(2A)-n260A | CA_n2A-n66A CA_n66A-n66A CA_n2A-n260A CA_n66A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n2A-n66A-n260A_UL_n2A-n260A_BCS0 DL_n2A-n66A-n260A_UL_n66A-n66A_BCS0 DL_n2A-n66A-n260A_UL_n2A-n260A_BCS0 DL_n66A-n66A-n260A_UL_n66A-n66A_BCS0 DL_n66A-n66A-n260A_UL_66A-n260A_BCS0 |
| CA_n2A-n66(2A)-n260M | CA_n2A-n66A CA_n66A-n66A CA_n2A-n260M CA_n66A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n2A-n66A-n66A_UL_n2A-n66A_BCS0 DL_n2A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n2A-n66A-n260M_UL_n2A-n260M_BCS0 DL_n2A-n66A-n260M_UL_n66A-n66A_BCS0 DL_n2A-n66A-n260M_UL_n2A-n260M_BCS0 DL_n66A-n66A-n260M_UL_n66A-n66A_BCS0 DL_n66A-n66A-n260M_UL_66A-n260M_BCS0 |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | 0 | Bo Liu, China Telecom | liubo1.bri@chinatelecom.cn | CATT, ZTE, Huawei, HiSilicon | Stopped | (ongoing) DL_n3A-n5A _UL_n3A-n5A_BCS0 (completed)DL_n3A-n78A_UL_n3A-n78A _BCS0 (completed)DL_n5A-n78A_UL_n5A-n78A _BCS0 (ongoing) DL_n3A-n5A-n78A _BCS0 |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n3A-n8A-n41A_BCS0 (completed) DL_n3A-n8A_UL_n3A-n8A_BCS0 (completed) DL_n3A-n41A_UL_n3A-n41A_BCS0 (completed) DL_n8A-n41A_UL_n8A-n41A_BCS0 |
| CA_n3A-n8A-n50A | CA_n3A-n8A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (completed) DL_n3A-n8A_UL_n3A-n8A_BCS0 (completed) DL_n3A-n8A_UL_n3A_BCS0 (completed) DL_n3A-n8A_UL_n8A_BCS0 (ongoing) DL_n3A-n50A_UL_n3A_BCS0 (ongoing) DL_n8A-n50A_UL_n8A_BCS0 |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | China Unicom, Samsung, ZTE, Tejet | Completed | (completed) DL_n3A-n8A-n78A_BCS0 (completed) DL_n3A-n8A_UL_n3A-n8A_BCS0 (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (completed) DL_n8A-n78A_UL_n8A-n78A_BCS0 |
| CA_n3A-n8A-n79A | CA_n3A-n8A CA_n3A-n79A CA_n8A-n79A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | China Unicom, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n3A-n8A-n79A_BCS0 (completed) DL_n3A-n8A_UL_n3A-n8A_BCS0 (completed) DL_n3A-n79A_UL_n3A-n79A_BCS0 (completed) DL_n8A-n79A_UL_n8A-n79A_BCS0 |
| CA_n3A-n28A-n50A | CA_n3A-n28A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (ongoing) DL_n3A-n28A_UL_n3A-n28A_BCS0 (ongoing) DL_n3A-n28A_UL_n3A_BCS0 (ongoing) DL_n3A-n28A_UL_n28A_BCS0 (ongoing) DL_n3A-n50A_UL_n3A_BCS0 (completed) DL_n28A-n50A_UL_n28A_BCS0 |
| CA_n3A-n28A-n77A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | completed | (completed) DL_n3A-n28A_UL_n3A-n28A_BCS0 (completed) DL_n3A-n77A_UL_n3A-n77A_BCS0 (completed) DL_n28A-n77A_UL_n28A-n77A_BCS0 |
| CA_n3A-n28A-n77(2A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A-n77A_UL_n3A-n28A_BCS0 (new) DL_n3A-n28A-n77A_UL_n3A-n77A_BCS0 (new) DL_n3A-n28A-n77A_UL_n28A-n77A_BCS0 (new) DL_n3A-n77(2A)_UL_n3A-n77A_BCS0 (new) DL_n28A-n77(2A)_UL_n28A-n77A_BCS0 |
| CA_n3A-n28A-n77(3A) | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A-n77(2A)_UL_n3A-n28A_BCS0 (new) DL_n3A-n28A-n77(2A)_UL_n3A-n77A_BCS0 (new) DL_n3A-n28A-n77(2A)_UL_n28A-n77A_BCS0 (new) DL_n3A-n77(3A)_UL_n3A-n77A_BCS0 (new) DL_n28A-n77(3A)_UL_n28A-n77A_BCS0 |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, Telstra, LG, Oppo | Completed | (Completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (Completed) DL_n28A-n78A_UL_n28A-n78A_BCS0 (Completed) DL_n3A-n28A_UL_n3A-n28A_BCS0 |
| CA_n3A-n28A-n78(2A) | CA_n3A-n78A CA_n3A-n28A CA_n28A-n78A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Nokia, Ericsson, Samsung | Stopped | (new) DL_n3A-n28A-n78(2A)_BCS0 (completed) DL_n3A-n28A_UL_n3A-n28A (new) DL_n3A-n78(2A)_UL_n3A-n78A (new) DL_n28A-n78(2A)_UL_n28A-n78A |
| CA_n3A-n28A-n257A | CA_n3A-n28A CA_n3A-n257A CA_n28A-n257A | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A_UL_n3A-n28A_BCS0 (completed) DL_n3A-n257A_UL_n3A-n257A_BCS0 (completed) DL_n28A-n257A_UL_n28A-n257A_BCS0 |
| CA_n3A-n28A-n257D | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257D CA_n28A-n257A CA_n28A-n257D | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A-n257A_UL_n3A-n28A_BCS0 (new) DL_n3A-n28A-n257A_UL_n3A-n257A_BCS0 (new) DL_n3A-n28A-n257A_UL_n28A-n257A_BCS0 (completed) DL_n3A-n257D_UL_n3A-n257D_BCS0 (completed) DL_n28A-n257D_UL_n28A-n257D_BCS0 |
| CA_n3A-n28A-n257G | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257G CA_n28A-n257A CA_n28A-n257G | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A-n257A_UL_n3A-n28A_BCS0 (new) DL_n3A-n28A-n257A_UL_n3A-n257A_BCS0 (new) DL_n3A-n28A-n257A_UL_n28A-n257A_BCS0 (completed) DL_n3A-n257G_UL_n3A-n257G_BCS0 (completed) DL_n28A-n257G_UL_n28A-n257G_BCS0 |
| CA_n3A-n28A-n257H | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A-n257G_UL_n3A-n28A_BCS0 (new) DL_n3A-n28A-n257G_UL_n3A-n257G_BCS0 (new) DL_n3A-n28A-n257G_UL_n28A-n257G_BCS0 (completed) DL_n3A-n257H_UL_n3A-n257H_BCS0 (completed) DL_n28A-n257H_UL_n28A-n257H_BCS0 |
| CA_n3A-n28A-n257I | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n3A-n28A-n257H_UL_n3A-n28A_BCS0 (new) DL_n3A-n28A-n257H_UL_n3A-n257H_BCS0 (new) DL_n3A-n28A-n257H_UL_n28A-n257H_BCS0 (completed) DL_n3A-n257I_UL_n3A-n257I_BCS0 (completed) DL_n28A-n257I_UL_n28A-n257I_BCS0 |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | 0 | Wubin, Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, CATT, sanechips | completed | (completed) DL_n3A-n40A-n41A_BCS0 (completed) DL_n3A-n40A_UL_n3A-n40A_BCS0 (completed) DL_n40A_n41A _UL_n40A-n40A_BCS0 (completed) DL_n3A_n41A _UL_n3A-n41A_BCS0 |
| CA_n3A-n41A-n79A | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n3A-n41A-n79A_BCS0 (completed) DL_n3A-n79A_UL_n3A-n79A_BCS0 (completed) DL_n3A-n41A_UL_n3A-n41A_BCS0 (completed) DL_n41A-n79A_UL_n41A-n79A_BCS0 |
| CA_n3A-n50A-n78A | CA_n3A-n78A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (completed) DL_n3A-n78A_UL_n3A_BCS0 (completed) DL_n3A-n78A_UL_n78A_BCS0 (new) DL_n50A-n78A_UL_n78A_BCS0 (new) DL_n3A-n50A_UL_n3A_BCS0 |
| CA_n3A-n77A-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77A-n257A_BCS0 |
| CA_n3A-n77A-n257D | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77A-n257D_BCS0 (completed) DL_n3A-n77A_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257D_UL_n3A-n257D_BCS0 (completed) DL_n77A-n257D_UL-n77A-n257D_BCS0 |
| CA_n3A-n77A-n257G | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n77A-n257A CA_n77A-n257G | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77A-n257G_BCS0 (completed) DL_n3A-n77A_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257G_UL_n3A-n257G_BCS0 (completed) DL_n77A-n257G_UL-n77A-n257G_BCS0 |
| CA_n3A-n77A-n257H | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77A-n257H_BCS0 (completed) DL_n3A-n77A_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257H_UL_n3A-n257H_BCS0 (completed) DL_n77A-n257H_UL-n77A-n257H_BCS0 |
| CA_n3A-n77A-n257I | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77A-n257I_BCS0 (completed) DL_n3A-n77A_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257I_UL_n3A-n257I_BCS0 (completed) DL_n77A-n257I_UL-n77A-n257I_BCS0 |
| CA_n3A-n77(2A)-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77(2A)-n257A_BCS0 |
| CA_n3A-n77(2A)-n257D | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77(2A)-n257D_BCS0 (completed) DL_n3A-n77(2A)_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257D_UL_n3A-n257D_BCS0 (completed) DL_n77(2A)-n257D_UL-n77A-n257D_BCS0 |
| CA_n3A-n77(2A)-n257G | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n77A-n257A CA_n77A-n257G | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77(2A)-n257G_BCS0 (completed) DL_n3A-n77(2A)_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257G_UL_n3A-n257G_BCS0 (completed) DL_n77(2A)-n257G_UL-n77A-n257G_BCS0 |
| CA_n3A-n77(2A)-n257H | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (Completed) DL_n3A-n77(2A)-n257H_BCS0 (Completed) DL_n3A-n77(2A)_UL_n3A-n77A_BCS0 (Completed) DL_n3A-n257H_UL_n3A-n257H_BCS0 (Completed) DL_n77(2A)-n257H_UL-n77A-n257H_BCS0 |
| CA_n3A-n77(2A)-n257I | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n77(2A)-n257I_BCS0 (completed) DL_n3A-n77(2A)_UL_n3A-n77A_BCS0 (completed) DL_n3A-n257I_UL_n3A-n257I_BCS0 (completed) DL_n77(2A)-n257I_UL-n77A-n257I_BCS0 |
| CA_n3A-n77(3A)-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (ongoing) DL_n3A_n77(3A)_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(2A)-n257A_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(2A)-n257A_UL_n3A-n257A_BCS0 (ongoing) DL_n3A_n77(2A)-n257A_UL_n77A-n257A_BCS0 (ongoing) DL n77(3A)-n257A_UL_n77A-n257A_BCS0 |
| CA_n3A-n77(3A)-n257D | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (ongoing) DL_n3A_n77(3A)-n257A_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(3A)-n257A_UL_n3A-n257A_BCS0 (ongoing) DL_n3A_n77(3A)-n257A_UL_n77A-n257A_BCS0 (ongoing) DL_n3A_n77(2A)-n257D_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(2A)-n257D_UL_n3A-n257D_BCS0 (ongoing) DL_n3A_n77(2A)-n257D_UL_n77A-n257D_BCS0 (ongoing) DL_n77(3A)-n257D_UL_n77A-n257D_BCS0 |
| CA_n3A-n77(3A)-n257G | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n77A-n257A CA_n77A-n257G | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (ongoing) DL_n3A_n77(3A)-n257A_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(3A)-n257A_UL_n3A-n257A_BCS0 (ongoing) DL_n3A_n77(3A)-n257A_UL_n77A-n257A_BCS0 (ongoing) DL_n3A_n77(2A)-n257G_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(2A)-n257G_UL_n3A-n257G_BCS0 (ongoing) DL_n3A_n77(2A)-n257G_UL_n77A-n257G_BCS0 (ongoing) DL_n77(3A)-n257G_UL_n77A-n257G_BCS0 |
| CA_n3A-n77(3A)-n257H | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (ongoing) DL_n3A_n77(3A)-n257G_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(3A)-n257G_UL_n3A-n257G_BCS0 (ongoing) DL_n3A_n77(3A)-n257G_UL_n77A-n257G_BCS0 (ongoing) DL_n3A_n77(2A)-n257H_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(2A)-n257H_UL_n3A-n257H_BCS0 (ongoing) DL_n3A_n77(2A)-n257H_UL_n77A-n257H_BCS0 (ongoing) DL_n77(3A)-n257H_UL_n77A-n257H_BCS0 |
| CA_n3A-n77(3A)-n257I | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (ongoing) DL_n3A_n77(3A)-n257H_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(3A)-n257H_UL_n3A-n257H_BCS0 (ongoing) DL_n3A_n77(3A)-n257H_UL_n77A-n257H_BCS0 (ongoing) DL_n3A_n77(2A)-n257I_UL_n3A-n77A_BCS0 (ongoing) DL_n3A_n77(2A)-n257I_UL_n3A-n257I_BCS0 (ongoing) DL_n3A_n77(2A)-n257I_UL_n77A-n257I_BCS0 (ongoing) DL_n77(3A)-n257I_UL_n77A-n257I_BCS0 |
| CA_n3A-n78A-n257A | CA_n3A-n78A CA_n3A-n257A CA_n78A-n257A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n78A-n257A_BCS0 |
| CA_n3A-n78A-n257D | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257D CA_n78A-n257A CA_n78A-n257D | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n78A-n257D_BCS0 (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (completed) DL_n3A-n257D_UL_n3A-n257D_BCS0 (completed) DL_n78A-n257D_UL-n78A-n257D_BCS0 |
| CA_n3A-n78A-n257G | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257G CA_n78A-n257A CA_n78A-n257G | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n78A-n257G_BCS0 (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (completed) DL_n3A-n257G_UL_n3A-n257G_BCS0 (completed) DL_n78A-n257G_UL-n78A-n257G_BCS0 |
| CA_n3A-n78A-n257H | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n78A-n257H_BCS0 (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (completed) DL_n3A-n257H_UL_n3A-n257H_BCS0 (completed) DL_n78A-n257H_UL-n78A-n257H_BCS0 |
| CA_n3A-n78A-n257I | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | completed | (completed) DL_n3A-n78A-n257I_BCS0 (completed) DL_n3A-n78A_UL_n3A-n78A_BCS0 (completed) DL_n3A-n257I_UL_n3A-n257I_BCS0 (completed) DL_n78A-n257I_UL-n78A-n257I_BCS0 |
| CA_n5A-n30A-n66A | CA_n5A-n30A CA_n30A-n66A CA_n5A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n30A_UL_n5A-n30A_BCS0 DL_n5A-n66A_UL_n5A-n66A_BCS0 DL_n30A-n66A_UL_n30A-n66A_BCS0 |
| CA_n5A-n30A-n66((2A) | CA_n5A-n30A CA_n30A-n66A CA_n66A-n66A CA_n5A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n30A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n30A-n66A_UL_n30A-n66A_BCS0 DL_n5A-n30A-n66A UL_n5A-n30A_BCS0 DL_n5A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n66A-n66A UL_n66A-66A_BCS0 DL_n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n66A-n66A_UL_n66A-n66A_BCS0 |
| CA_n5A-n30A-n66(3A) | CA_n5A-n66A CA_n30A-n66A CA_n66A-n66A CA_n5A-n30A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n30A-n66A-n66A_UL_n5A-n30A_BCS0 DL_n5A-n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n5A-n30A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n5A-n30A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n66A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n2A-n66A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n30A-n66A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n66A-n66A-n66A_UL_n66A-66A_BCS0 |
| CA_n5A-n30A-n260M | CA_n5A-n30A CA_n30A-n260M CA_n5A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n30A_UL_n5A-n30A_BCS0 DL_n5A-n260M_UL_n5A-n260M_BCS0 DL_n30A-n260M_UL_n30A-n260M_BCS0 |
| CA_n5A-n30A-n260A | CA_n5A-n260A CA_n30A-n260A CA_n5A-n30A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n30A_UL_n5A-n30A_BCS0 DL_n5A-n260A_UL_n5A-n260A_BCS0 DL_n30A-n260A_UL_n30A-n260A_BCS0 |
| CA_n5A-n66A-n78A | CA_n5A-n66A CA_n66A-n78A CA_n5A-n78A | 0 | Liu Liehai, Huawei | liuliehai@huawei.com | Bell Mobility, TELUS, Hisilicon | Completed | (ongoing) DL_n5A-n66A_UL_n5A-n66A_BCS0 (ongoing) DL_n5A-n78A_UL_n5A-n78A_BCS0 (ongoing) DL_n66A-n78A_UL_n66A-n78A_BCS0 |
| CA_n5A-n66A-n260A | CA_n5A-n66A CA_n66A-n260A CA_n5A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n260A_UL_n5A-n260A_BCS0 DL_n66A-n260A_UL_n66A-n260A_BCS0 |
| CA_n5A-n66(2A)-n260A | CA_n5A-n66A CA_n66A-n66A CA_n5A-n260A CA_n66A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n5A-n66A-n260A_UL_n5A-n260A_BCS0 DL_n5A-n66A-n260A_UL_n66A-n66A_BCS0 DL_n5A-n66A-n260A_UL_n5A-n260A_BCS0 DL_n66A-n66A-n260A_UL_n66A-n66A_BCS0 DL_n66A-n66A-n260A_UL_66A-n260A_BCS0 |
| CA_n5A-n66A-n260M | CA_n5A-n66A CA_n66A-n260M CA_n5A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n66A_UL_n5A_BCS0 DL_n5A-n66A_UL_n66A_BCS0 DL_n5A-n260M_UL_n5A_BCS0 DL_n5A-n260M_UL_260M_BCS0 DL_n66A-n260M_UL_n66A_BCS0 DL_n66A-n260M_UL_n260M_BCS0 |
| CA_n5A-n66(2A)-n260M | CA_n5A-n66A CA_n66A-n66A CA_n5A-n260M CA_n66A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n5A-n66A-n66A_UL_n5A-n66A_BCS0 DL_n5A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n5A-n66A-n260M_UL_n5A-n260M_BCS0 DL_n5A-n66A-n260M_UL_n66A-n66A_BCS0 DL_n5A-n66A-n260M_UL_n5A-n260M_BCS0 DL_n66A-n66A-n260M_UL_n66A-n66A_BCS0 DL_n66A-n66A-n260M_UL_66A-n260M_BCS0 |
| CA_n7A-n25A-n66A | CA_n7A-n25A CA_n25A-n66A CA_n7A-n66A | 0 | Liu Liehai, Huawei | liuliehai@huawei.com | Bell Mobility, TELUS, Hisilicon | Completed | (ongoing) DL_n7A-n25A_UL_n7A-n25A_BCS0 (ongoing) DL_n25A-n66A_UL_n25A-n66A_BCS0 (ongoing) DL_n7A-n66A_UL_n7A-n66A_BCS0 |
| CA_n7A-n28A-n78A | CA_n7A-n78A CA_n7A-n28A CA_n28A-n78A | 0 | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | Stopped | (ongoing)DL_n7A-n28A-n78A_BCS0, (completed)DL_n7A-n78A_UL_n7A-n78A_BCS0, (ongoing)DL_n7A-n28A_UL_n7A-n28A_BCS0, (completed)DL_n28A-n78A_UL_n28A-n78A_BCS0, |
| CA_n7A-n28A-n78(2A) | CA_n7A-n28A CA_n7A-n78A CA_n28A-n78A | 0 | S. Truelove, BT plc. | stephen.truelove@bt.com | Ericsson, Huawei, Rogers Comms. Canada | Stopped | (ongoing)DL_n7A-n28A-n78A_UL_n7A-n28A_BCS0 (completed)DL_n7A-n28A_UL_n7A-n28A_BCS0 (ongoing)DL_n7A-n28A-n78(2A)_UL_n7A-n78A_BCS0 (ongoing)DL_n7A-n28A-n78A_UL_n7A-n78A_BCS0, (ongoing) DL_n7A-n28A-n78A_UL_n7A-n78A_BCS0, (ongoing)DL_n7A-n78(2A)_UL_n7A-n78A_BCS0 (ongoing)DL_n7A-n28A-n78(2A)_UL_n28A-n78A_BCS0, (ongoing)DL_n7A-n28A-n78A_UL_n28A-n78A_BCS0, (ongoing)DL_n7A-n28A-n78A_UL_n28A-n78A_BCS0, (ongoing)DL_n28A-n78(2A)_UL_n28A-n78A_BCS0 |
| CA_n7A-n66A-n78A | CA_n7A-n78A CA_n66A-n78A CA_n7A-n66A | 0 | Liu Liehai, Huawei | liuliehai@huawei.com | Bell Mobility, TELUS, Hisilicon | Completed | (new) DL_n7A-n66A_UL_n7A-n66A_BCS0 (new) DL_n7A-n78A_UL_n7A-n78A_BCS0 (new) DL_n66A-n78A_UL_n66A-n78A_BCS0 |
| CA_n7A-n66A-n78(2A) | CA_n7A-n78A CA_n66A-n78A CA_n7A-n66A | 0 | Liu Liehai, Huawei | liuliehai@huawei.com | Bell Mobility, TELUS, Hisilicon | Stopped | (ongoing) CA_n7A-n66A-n78A (completed) CA_n66A-n78(2A) (completed) CA_n7A-n78(2A) |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n8A-n39A-n41A_BCS0 (completed) DL_n8A-n39A_UL_n8A-n39A_BCS0 (completed) DL_n8A-n41A_UL_n8A-n41A_BCS0 (completed) DL_n39A-n41A_UL_n39A-n41A_BCS0 |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, CATT, Tejet | Stopped | (ongoing) DL_n8A-n40A-n41A_BCS0 (ongoing) DL_n8A-n40A_UL_n8A-n40A_BCS0 (completed) DL_n8A-n41A_UL_n8A-n41A_BCS0 (completed) DL_n40A-n41A_UL_n40A-n41A_BCS0 |
| CA_n8A-n41A-n79A | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n8A-n41A-n79A_BCS0 (completed) DL_n8A-n79A_UL_n8A-n79A_BCS0 (completed) DL_n8A-n41A_UL_n8A-n41A_BCS0 (completed) DL_n41A-n79A_UL_n41A-n79A_BCS0 |
| CA_n8A-n50A-n78A | CA_n8A-n78A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (ongoing) DL_n8A-n78A_UL_n8A-n78A_BCS0 (ongoing) DL_n8A-n78A_UL_n8A_BCS0 (ongoing) DL_n8A-n78A_UL_n78A_BCS0 (ongoing) DL_n50A-n78A_UL_n78A_BCS0 (ongoing) DL_n8A-n50A_UL_n8A_BCS0 |
| CA_n25A-n66A-n78A | CA_n25A-n78A CA_n66A-n78A CA_n25A-n66A | 0 | Liu Liehai, Huawei | liuliehai@huawei.com | Bell Mobility, TELUS, Hisilicon | Completed | (new) DL_n25A-n66A_UL_n25A-n66A_BCS0 (new) DL_n25A-n78A_UL_n25A-n78A_BCS0 (new) DL_n66A-n78A_UL_n66A-n78A_BCS0 |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | 0 | Zhang Peng, Huawei Abdulhadi Mahmoud AbouAlmal, Etisalat | zhangpeng169@huawei.com, aalmal@etisalat.ae | HiSilicon, Etisalat, Ericsson | Completed | DL_n28A-n41A_UL_n28A-n41A (completed) DL_n41A-n78A_UL_n41A-n78A (New) DL_n28A-n78A_UL_n41A-n78A (completed) |
| CA_n28A-n50A-n78A | CA_n28A-n78A | 0 | Christoph Mäder, Swisscom | Christoph.maeder@swisscom.com | Ericsson, LG, Oppo | Stopped | (ongoing) DL_n28A-n78A_UL_n28A-n78A_BCS0 (ongoing) DL_n28A-n78A_UL_n28A_BCS0 (ongoing) DL_n28A-n78A_UL_n78A_BCS0 (ongoing) DL_n50A-n78A_UL_n78A_BCS0 (ongoing) DL_n28A-n50A_UL_n28A_BCS0 |
| CA_n28A-n77A-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77A-n257A_BCS0 |
| CA_n28A-n77A-n257D | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77A-n257D_BCS0 (Completed) DL_n28A-n77A_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257D_UL_n28A-n257D_BCS0 (Completed) DL_n77A-n257D_UL-n77A-n257D_BCS0 |
| CA_n28A-n77A-n257G | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n77A-n257A CA_n77A-n257G | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77A-n257G_BCS0 (Completed) DL_n28A-n77A_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257G_UL_n28A-n257G_BCS0 (Completed) DL_n77A-n257G_UL-n77A-n257G_BCS0 |
| CA_n28A-n77A-n257H | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77A-n257H_BCS0 (Completed) DL_n28A-n77A_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257H_UL_n28A-n257H_BCS0 (Completed) DL_n77A-n257H_UL-n77A-n257H_BCS0 |
| CA_n28A-n77A-n257I | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77A-n257I_BCS0 (Completed) DL_n28A-n77A_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257I_UL_n28A-n257I_BCS0 (Completed) DL_n77A-n257I_UL-n77A-n257I_BCS0 |
| CA_n28A-n77(2A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77(2A)-n257A_BCS0 |
| CA_n28A-n77(2A)-n257D | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77(2A)-n257D_BCS0 (Completed) DL_n28A-n77(2A)_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257D_UL_n28A-n257D_BCS0 (Completed) DL_n77(2A)-n257D_UL-n77A-n257D_BCS0 |
| CA_n28A-n77(2A)-n257G | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n77A-n257A CA_n77A-n257G | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77(2A)-n257G_BCS0 (Completed) DL_n28A-n77(2A)_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257G_UL_n28A-n257G_BCS0 (Completed) DL_n77(2A)-n257G_UL-n77A-n257G_BCS0 |
| CA_n28A-n77(2A)-n257H | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77(2A)-n257H_BCS0 (Completed) DL_n28A-n77(2A)_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257H_UL_n28A-n257H_BCS0 (Completed) DL_n77(2A)-n257H_UL-n77A-n257H_BCS0 |
| CA_n28A-n77(2A)-n257I | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n77(2A)-n257I_BCS0 (Completed) DL_n28A-n77(2A)_UL_n28A-n77A_BCS0 (Completed) DL_n28A-n257I_UL_n28A-n257I_BCS0 (Completed) DL_n77(2A)-n257I_UL-n77A-n257I_BCS0 |
| CA_n28A-n77(3A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n28A_n77(3A)_UL_n28A-n77A_BCS0 (ongoing) DL_n28A_n77(2A)-n257A_UL_n28A-n77A_BCS0 (ongoing) DL_n28A_n77(2A)-n257A_UL_n28A-n257A_BCS0 (ongoing) DL_n28A_n77(2A)-n257A_UL_n77A-n257A_BCS0 (new) DL n77(3A)-n257A_UL_n77A-n257A_BCS0 |
| CA_n28A-n77(3A)-n257D | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n28A_n77(3A)-n257A_UL_n28A-n77A_BCS0 (new) DL_n28A_n77(3A)-n257A_UL_n28A-n257A_BCS0 (new) DL_n28A_n77(3A)-n257A_UL_n77A-n257A_BCS0 (ongoing) DL_n28A_n77(2A)-n257D_UL_n28A-n77A_BCS0 (ongoing) DL_n28A_n77(2A)-n257D_UL_n28A-n257D_BCS0 (ongoing) DL_n28A_n77(2A)-n257D_UL_n77A-n257D_BCS0 (new) DL_n77(3A)-n257D_UL_n77A-n257D_BCS0 |
| CA_n28A-n77(3A)-n257G | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n77A-n257A CA_n77A-n257G | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n28A_n77(3A)-n257A_UL_n28A-n77A_BCS0 (new) DL_n28A_n77(3A)-n257A_UL_n28A-n257A_BCS0 (new) DL_n28A_n77(3A)-n257A_UL_n77A-n257A_BCS0 (ongoing) DL_n28A_n77(2A)-n257G_UL_n28A-n77A_BCS0 (ongoing) DL_n28A_n77(2A)-n257G_UL_n28A-n257G_BCS0 (ongoing) DL_n28A_n77(2A)-n257G_UL_n77A-n257G_BCS0 (new) DL_n77(3A)-n257G_UL_n77A-n257G_BCS0 |
| CA_n28A-n77(3A)-n257H | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n28A_n77(3A)-n257G_UL_n28A-n77A_BCS0 (new) DL_n28A_n77(3A)-n257G_UL_n28A-n257G_BCS0 (new) DL_n28A_n77(3A)-n257G_UL_n77A-n257G_BCS0 (ongoing) DL_n28A_n77(2A)-n257H_UL_n28A-n77A_BCS0 (ongoing) DL_n28A_n77(2A)-n257H_UL_n28A-n257H_BCS0 (ongoing) DL_n28A_n77(2A)-n257H_UL_n77A-n257H_BCS0 (new) DL_n77(3A)-n257H_UL_n77A-n257H_BCS0 |
| CA_n28A-n77(3A)-n257I | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | 0 | Masashi Fushiki, SoftBank | masashi.fushiki@g.softbank.co.jp | Ericsson, Huawei, HiSilicon, Nokia, ZTE | Stopped | (new) DL_n28A_n77(3A)-n257H_UL_n28A-n77A_BCS0 (new) DL_n28A_n77(3A)-n257H_UL_n28A-n257H_BCS0 (new) DL_n28A_n77(3A)-n257H_UL_n77A-n257H_BCS0 (ongoing) DL_n28A_n77(2A)-n257I_UL_n28A-n77A_BCS0 (ongoing) DL_n28A_n77(2A)-n257I_UL_n28A-n257I_BCS0 (ongoing) DL_n28A_n77(2A)-n257I_UL_n77A-n257I_BCS0 (new) DL_n77(3A)-n257I_UL_n77A-n257I_BCS0 |
| CA_n28A-n78A-n257A | CA_n28A-n78A CA_n28A-n257A CA_n78A-n257A | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n78A-n257A_BCS0 |
| CA_n28A-n78A-n257D | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257D CA_n78A-n257A CA_n78A-n257D | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n78A-n257D_BCS0 (Completed) DL_n28A-n78A_UL_n28A-n78A_BCS0 (Completed) DL_n28A-n257D_UL_n28A-n257D_BCS0 (Completed) DL_n78A-n257D_UL-n78A-n257D_BCS0 |
| CA_n28A-n78A-n257G | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257G CA_n78A-n257A CA_n78A-n257G | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n78A-n257G_BCS0 (Completed) DL_n28A-n78A_UL_n28A-n78A_BCS0 (Completed) DL_n28A-n257G_UL_n28A-n257G_BCS0 (Completed) DL_n78A-n257G_UL-n78A-n257G_BCS0 |
| CA_n28A-n78A-n257H | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n78A-n257H_BCS0 (Completed) DL_n28A-n78A_UL_n28A-n78A_BCS0 (Completed) DL_n28A-n257H_UL_n28A-n257H_BCS0 (Completed) DL_n78A-n257H_UL-n78A-n257H_BCS0 |
| CA_n28A-n78A-n257I | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I | 0 | Xiao Shao, KDDI | ko-shou@kddi.com | Huawei, Hisilicon, Sumitomo Electric | Completed | (Completed) DL_n28A-n78A-n257I_BCS0 (Completed) DL_n28A-n78A_UL_n28A-n78A_BCS0 (Completed) DL_n28A-n257I_UL_n28A-n257I_BCS0 (Completed) DL_n78A-n257I_UL-n78A-n257I_BCS0 |
| CA_n30A-n66A-n260A | CA_n30A-n260A CA_n66A-n260A CA_n30A-n66A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n30A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n260A_UL_n30A-n260A_BCS0 DL_n66A-n260A_UL_n66A-n260A_BCS0 |
| CA_n30A-n66(2A)-n260A | CA_n30A-n66A CA_n66A-n66A CA_n30A-n260A CA_n66A-n260A | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n30A-n66A-n260A_UL_n30A-n260A_BCS0 DL_n30A-n66A-n260A_UL_n66A-n66A_BCS0 DL_n30A-n66A-n260A_UL_n30A-n260A_BCS0 DL_n66A-n66A-n260A_UL_n66A-n66A_BCS0 DL_n66A-n66A-n260A_UL_66A-n260A_BCS0 |
| CA_n30A-n66A-n260M | CA_n30A-n260M CA_n66A-n260M CA_n30A-n66M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n30A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n260M_UL_n30A-n260M_BCS0 DL_n66A-n260M_UL_n66A-n260M_BCS0 |
| CA_n30A-n66(2A)-n260M | CA_n30A-n66A CA_n66A-n66A CA_n30A-n260M CA_n66A-n260M | 0 | Marc Grant, AT&T | marc.grant@att.com | Ericsson Qualcomm Nokia | Stopped | DL_n30A-n66A-n66A_UL_n30A-n66A_BCS0 DL_n30A-n66A-n66A_UL_n66A-n66A_BCS0 DL_n30A-n66A-n260M_UL_n30A-n260M_BCS0 DL_n30A-n66A-n260M_UL_n66A-n66A_BCS0 DL_n30A-n66A-n260M_UL_n30A-n260M_BCS0 DL_n66A-n66A-n260M_UL_n66A-n66A_BCS0 DL_n66A-n66A-n260M_UL_66A-n260M_BCS0 |
| CA_n34A-n39A-n41A | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, ZTE, Sanechips | Stopped | (ongoing)DL_n34A-n39A-n41A_BCS0 (ongoing)DL_n34A-n39A_UL_n34A-n39A_BCS0 (completed)DL_n39A_n41A_UL_n39A-n41A_BCS0 (completed)DL_n34A_n41A_UL_n34A-n41A_BCS0 |
| CA_n34A-n39A-n79A | CA_n34A-n39A CA_n34A-n79A CA_n39A-n79A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, ZTE, Sanechips | Stopped | (ongoing)DL_n34A-n39A-n79A_BCS0 (ongoing)DL_n34A-n39A_UL_n34A-n39A_BCS0 (completed)DL_n39A_n79A_UL_n39A-n79A_BCS0 (new)DL_n34A_n79A_UL_n34A-n79A_BCS0 |
| CA_n34A-n40A-n41A | CA_n34A-n40A CA_n34A-n41A CA_n40A-n41A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, ZTE, Sanechips | Stopped | (ongoing)DL_n34A-n40A-n41A_BCS0 (ongoing)DL_n34A-n40A_UL_n34A-n40A_BCS0 (completed)DL_n40A_n41A_UL_n40A-n41A_BCS0 (ongoing)DL_n34A_n41A_UL_n34A-n41A_BCS0 |
| CA_n34A-n40A-n79A | CA_n34A-n40A CA_n34A-n79A CA_n40A-n79A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, ZTE, Sanechips | Stopped | (ongoing)DL_n34A-n40A-n79A_BCS0 (ongoing)DL_n34A-n40A_UL_n34A-n40A_BCS0 (completed)DL_n40A_n79A_UL_n40A-n79A_BCS0 (ongoing)DL_n34A_n79A_UL_n34A-n79A_BCS0 |
| CA_n34A-n41A-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, ZTE, Sanechips | Stopped | (ongoing)DL_n34A-n41A-n79A_BCS0 (ongoing)DL_n34A-n41A_UL_n34A-n41A_BCS0 (completed)DL_n41A_n79A_UL_n41A-n79A_BCS0 (ongoing)DL_n34A_n79A_UL_n34A-n79A_BCS0 |
| CA_n34A-n39A-n40A | CA_n34A-n39A CA_n34A-n40A CA_n39A-n40A | 0 | Wubin, Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, CATT, sanechips | Stopped | (ongoing) DL_n34A-n39A-n40A _BCS0 (ongoing) DL_n34A_n40A _UL_n34A_n40A_BCS0 (ongoing) DL_n34A_n39A _UL_n34A_n39A_BCS0 (ongoing) DL_n39A_n40A _UL_n39A_n40A _BCS0 |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | 0 | Wubin, Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, CATT, sanechips | Stopped | (ongoing) DL_n39A-n40A-n41A_BCS0 (new) DL_n39A-n40A_UL_n39A-n40A_BCS0 (ongoing)DL_n40A_n41A_UL_n40A-n41A_BCS0 (completed)DL_n39A_n41A_UL_n39A-n41A_BCS0 |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n39A-n79A CA_n40A-n79A | 0 | Wubin, Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, CATT, sanechips | Stopped | (ongoing) DL_n39A-n40A-n79A_BCS0 (ongoing) DL_n39A-n40A_UL_n39A-n40A_BCS0 (ongoing)DL_n40A_n79A_UL_n40A-n79A_BCS0 (completed)DL_n39A_n79A_UL_n39A-n79A_BCS0 |
| CA_n39A-n41A-n79A | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | 0 | Yuexia Song CATT | songyuexia@catt.cn | CMCC, Samsung, ZTE, Tejet | Stopped | (ongoing) DL_n39A-n41A-n79A_BCS0 (completed) DL_n39A-n79A_UL_n39A-n79A_BCS0 (completed) DL_n39A-n41A_UL_n39A-n41A_BCS0 (completed) DL_n41A-n79A_UL_n41A-n79A_BCS0 |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | 0 | Wubin,Zhou ZTE Corporation | Zhou.wubin@zte.com.cn | CMCC, CATT, Tejet | completed | (completed) DL_n40A-n41A-n79A_BCS0 (completed) DL_n40A-n79A_UL_n40A-n79A_BCS0 (completed) DL_n40A-n41A_UL_n40A-n41A_BCS0 (completed) DL_n41A-n79A_UL_n41A-n79A_BCS0 |
| CA_n66A-n70A-n71A | CA_n66A-n71A, CA_n70A-n71A | 0 | Antti Immonen, Dish network | antti.immonen@dish.com | Nokia, Ericsson, Samsung | completed | Only UL CA configuration is added; DL CA_n66A-n70A-n71A is completed |
| CA_n66B-n70A-n71A | CA_n66A-n71A, CA_n70A-n71A | 0 | Antti Immonen, Dish network | antti.immonen@dish.com | Nokia, Ericsson, Samsung | completed | Only UL CA configuration is added; DL CA_n66B-n70A-n71A is completed |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A, CA_n70A-n71A | 0 | Antti Immonen, Dish network | antti.immonen@dish.com | Nokia, Ericsson, Samsung | completed | Only UL CA configuration is added; DL CA_n66(2A) -n70A-n71A is completed |
| CA_n66A-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | 0 | Liu Liehai, Huawei | liuliehai@huawei.com | TELUS, Bell Mobility, Rogers, Hisilicon | Stopped | (completed) CA_n66A-n71A (ongoing) CA_n71A-n78A (completed) CA_n66A-n78A |
| CA_n77A-n79A-n257A CA_n77A-n79A-n257G CA_n77A-n79A-n257H CA_n77A-n79A-n257I | CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | 0 | Yuta Oguma, NTT DOCOMO | yuuta.oguma.yt@nttdocomo.com | Fujitsu, NEC, Nokia | completed | (completed)DL_n77A-n257I_UL_n77A-n257I_BCS0 (completed)DL_n79A-n257I_UL_n79A-n257I_BCS0 |
| CA_n78A-n79A-n257A CA_n78A-n79A-n257G CA_n78A-n79A-n257H CA_n78A-n79A-n257I | CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | 0 | Yuta Oguma, NTT DOCOMO | yuuta.oguma.yt@nttdocomo.com | Fujitsu, NEC, Nokia | completed | (completed)DL_n78A-n257I_UL_n78A-n257I_BCS0 (completed)DL_n79A-n257I_UL_n79A-n257I_BCS0 |

- DC configurations for 3 different bands DL with 2 band UL

- Table 2-2: Individual configuration names, proponents and supporting companies for NR DC configurations for 2 different bands DL with 2 band UL within FR1, or within FR2 or including FR2

| NR DC configuration | Uplink Configuration | BCS | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n3A-n28A-n257A | DC_n3A-n257A DC_n28A-n257A | 0 | Li yankun Samsung | Yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | (Completed)DL_n3A-n257A_UL_n3A-n257A_BCS0 (Completed)DL_n28A-n257A_UL_n28A-n257A_BCS0 |
| DC_n3A-n28A-n257I | DC_n3A-n257I DC_n28A-n257I | 0 | Li yankun Samsung | Yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | (Completed)DL_n3A-n257I_UL_n3A-n257I_BCS0 (Completed)DL_n28A-n257I_UL_n28A-n257I_BCS0 |
| DC_n3A-n77A-n257A | DC_n3A-n257A DC_n77A-n257A | 0 | Li yankun Samsung | Yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | (Completed)DL_n3A-n257A_UL_n3A-n257A_BCS0 (Completed)DL_n77A-n257A_UL_n77A-n257A_BCS0 |
| DC_n3A-n77A-n257I | DC_n3A-n257I DC_n77A-n257I | 0 | Li yankun Samsung | Yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | Completed)DL_n3A-n257I_UL_n3A-n257I_BCS0 (completed)DL_n77A-n257I_UL_n77A-n257I_BCS0 |
| DC_n3A-n77(2A)_n257A | DC_n3A_n257A DC_n77A_n257A | 0 | Li Yankun, Samsung | yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | DL_n3A-n77A_n257A UL_ n3A-n257A _BCS0-Completed DL_ n77(2A)_n257A UL_ n77A-n257A _BCS0-Completed |
| DC_n3A-n77(2A)-n257I | DC_n3A-n257I DC_n77A-n257I | 0 | Li Yankun, Samsung | yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | DL_n3A-n77A-n257I UL_ n3A-n257I _BCS0-Completed DL_ n77(2A)-n257I UL_ n77A-n257I _BCS0-Completed |
| DC_n3A-n78A-n257A | DC_n3A-n257A DC_n78A-n257A | 0 | Li Yankun, Samsung | yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | DL_n3A-n257A_UL_n3A-n257A_BCS0-Completed DL_n78A-n257A_UL_n78A-n257A_BCS0-Completed |
| DC_n3A-n78A-n257I | DC_n3A-n257I DC_n78A-n257I | 0 | Li Yankun, Samsung | yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | DL_n3A-n257I_UL_n3A-n257I_BCS0-Completed DL_n78A-n257I_UL_n78A-n257I_BCS0-Completed |
| DC_n28A-n77A-n257A | DC_n28A-n257A DC_n77A-n257A | 0 | Li yankun Samsung | Yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | (Completed)DL_n28A-n257A_UL_n28A-n257A_BCS0 (completed)DL_n77A-n257A_UL_n77A-n257A_BCS0 |
| DC_n28A-n77A-n257I | DC_n28A-n257I DC_n77A-n257I | 0 | Li yankun Samsung | Yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | (Completed)DL_n28A-n257I_UL_n28A-n257I_BCS0 (Completed)DL_n77A-n257I_UL_n77A-n257I_BCS0 |
| DC_n28A-n78A-n257A | DC_n28A-n257A DC_n78A-n257A | 0 | Li Yankun, Samsung | yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | DL_n28A-n257A_UL_ n28A-n257A _BCS0-Completed DL_n78A-n257A_UL_n78A-n257A_BCS0-Completed |
| DC_n28A-n78A-n257I | DC_n28A-n257I DC_n78A-n257I | 0 | Li Yankun, Samsung | yankun.li@samsung.com | KDDI, Ericsson, Nokia | Completed | DL_n28A-n257I_UL_ n28A-n257I _BCS0-Completed DL_n78A-n257I_UL_n78A-n257I_BCS0-Completed |

Editor Note:  For NR DC bands specified in table 1-2, the corresponding NR CA configurations defined in the following tables, i.e., dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity.

- Bandwidth combination set for CA configuration

- Table 2-1 Bandwidth combinations for 3 bands DL/2 bands UL CA band combination within FR1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | Uplink CA configurations | NR Bands | SCS (kHz) | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz |  | 25 MHz | 30 MHz |  | 40 MHz | 50 MHz |  | 60 MHz | 70 MHz |  | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_n1A-n3A-n8A | CA_n1A-n3A CA_n1A-n8A CA_n3A-n8A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n28A | CA_n1A-n3A CA_n1A-n28A CA_n3A-n28A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n50A | CA_n1A-n3A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_n1A-n3A-n41A | CA_n1A-n3A CA_n1A-n41A CA_n3A-n41A | n1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes | Yes |  |  |
| CA_n1A-n8A-n50A | CA_n1A-n8A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 130 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_n1A-n28A-n50A | CA_n1A-n28A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 130 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_n1A-n28A-n78A | CA_n1A-n28A CA_n1A-n78A CA_n28A-n78A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 160 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  | Yes |  |  |
| CA_n1A-n50A-n78A | CA_n1A-n78A | n1 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  | Yes |  |  |
| CA_n3A-n8A-n50A | CA_n3A-n8A | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 130 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_n3A-n28A-n50A | CA_n3A-n28A | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 130 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  |  |  |  |
| CA_n3A-n28A-n78A | CA_n3A-n28A CA_n3A-n78A CA_n28A-n78A | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 160 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes |  |  | Yes |  | Yes |  |  |
| CA_n3A-n28A-n78(2A) | CA_n3A-n78A CA_n3A-n28A CA_n28A-n78A | n3 | 15 | Yes |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 250 | 0 |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n3A-n50A-n78A | CA_n3A-n78A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 210 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n1A-n5A-n78A | CA_n1A-n5A CA_n1A-n78A CA_n5A-n78A | n1 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes1 | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes1 | Yes |  |  |
| CA_n3A-n5A-n78A | CA_n3A-n5A CA_n3A-n78A CA_n5A-n78A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes1 | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes1 | Yes |  |  |
| CA_n3A-n8A-n79A | CA_n3A-n8A CA_n3A-n79A CA_n8A-n79A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n3A-n41A-n79A | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 230 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 190 | 1 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n3A-n8A-n78A | CA_n3A-n8A CA_n3A-n78A CA_n8A-n78A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n3A-n40A-n41A | CA_n3A-n40A CA_n3A-n41A CA_n40A-n41A | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  | 210 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
| CA_n8A-n41A-n79A | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 220 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 180 | 1 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n8A-n50A-n78A | CA_n8A-n78A | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 200 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n28A-n78A CA_n41A-n78A | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 220 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
| CA_n28A-n50A-n78A | CA_n28A-n78A | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 200 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n50 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n34A-n39A-n41A | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_n34A-n39A-n79A | CA_n34A-n39A CA_n34A-n79A CA_n39A-n79A | n34 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 155 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_n34A-n40A-n41A | CA_n34A-n40A CA_n34A-n41A CA_n40A-n41A | n34 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 195 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
| CA_n34A-n40A-n79A | CA_n34A-n40A CA_n34A-n79A CA_n40A-n79A | n34 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 195 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
| CA_n34A-n41A-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 215 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n34A-n39A-n40A | CA_n34A-n39A CA_n34A-n40A CA_n39A-n40A | n34 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 135 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 220 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n39A-n79A CA_n40A-n79A | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 220 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n39A-n41A-n79A | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 240 | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 200 | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n1A-n3A-n78A | CA_n1A-n3A CA_n1A-n78A CA_n3A-n78A | n1 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 160 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n39 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 200 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  | 280 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes |  |  |  | Yes |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 200 | 1 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |
| CA_n66A-n70A-n71A | CA_n66A-n71A, CA_n70A-n71A | n66 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n70 | 15 |  | Yes | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66B-n70A-n71A | CA_n66A-n71A, CA_n70A-n71A | n66 | See CA_n66B Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | n70 | 15 |  | Yes | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A, CA_n70A-n71A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 105 | 0 |
|  |  | n70 | 15 |  | Yes | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes1 | Yes1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n71A-n78A | CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n66 | 15 |  | Yes | Yes | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 150 | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes | Yes | Yes |  |  |  | Yes | Yes | Yes |  |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Table 2-2 Bandwidth combinations for 3 bands DL/2 bands UL CA band combination within FR2

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum Aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_nXA-nYA-nZA | CA_nXA-nYA CA_nXA-nZA CA_nYA-nZA | nX | 60 |  |  |  |  |  | 0 |
|  |  |  | 120 |  |  |  |  |  |  |
|  |  | nY | 60 |  |  |  |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |
|  |  | nZ | 60 |  |  |  |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |

- Table 2-3 Bandwidth combinations for 3 bands DL/2 bands UL CA band combination including FR2

| NR CA configuration |  | NR Uplink CA configuration |  | NR Band |  | SCS (kHz) |  |  | 5 MHz |  | 10 MHz |  | 15 MHz |  |  | 20 MHz |  |  | 25 MHz |  |  | 30 MHz |  |  | 40 MHz |  |  | 50 MHz |  |  | 60 MHz |  |  | 80 MHz |  |  | 90 MHz |  |  | 100 MHz |  | 200 MHz |  | 400 MHz |  | Maximum Aggregated bandwidth [MHz] |  | Bandwidth combination set |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n77A-n257A |  | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n3A-n77A-n257D |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257G |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 330 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257H |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 430 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257I |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257A |  | CA_n3A-n78A CA_n3A-n257A CA_n78A-n257A |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n3A-n78A-n257D |  | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257D CA_n78A-n257A CA_n78A-n257D |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257G |  | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n78A-n257A CA_n78A-n257D CA_n78A-n257G |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 330 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257H |  | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n3A-n257H CA_n78A-n257A CA_n78A-n257D CA_n78A-n257G CA_n78A-n257H |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 430 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257I |  | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n78A-n257A CA_n78A-n257D CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257A |  | CA_n3A-n77(2A)A CA_n3A-n257A CA_n77A-n257A |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 630 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257D |  | CA_n3A-n77(2A)A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 630 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257G |  | CA_n3A-n77(2A)A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 430 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257H |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257I |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n3 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 630 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257A |  | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n28A-n77A-n257D |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257G |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257H |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257I |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257A |  | CA_n28A-n78A CA_n28A-n257A CA_n78A-n257A |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n28A-n78A-n257D |  | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257D CA_n78A-n257A CA_n78A-n257D |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257G |  | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n78A-n257A CA_n78A-n257D CA_n78A-n257G |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 320 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257H |  | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n28A-n257H CA_n78A-n257A CA_n78A-n257D CA_n78A-n257G CA_n78A-n257H |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257I |  | CA_n28A-n78A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n78A-n257A CA_n78A-n257D CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n78 |  | 15 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257A |  | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257D |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257G |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 420 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257H |  | CA_n28A-n77(2A)A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257I |  | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257D CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77-n79A-n257A |  | CA_n77A-n257A CA_n79A-n257A |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n77-n79A-n257G |  | CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77-n79A-n257H |  | CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G and n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77-n79A-n257I |  | CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G, n257H, and n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78-n79A-n257A |  | CA_n78A-n257A CA_n79A-n257A |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n78-n79A-n257G |  | CA_n78A-n257A CA_n78A-n257G CA_n79A-n257A CA_n79A-n257G |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78-n79A-n257H |  | CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G and n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78-n79A-n257I |  | CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I |  | n78 |  | 15 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n79 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G, n257H, and n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR CA configuration |  | NR Uplink CA configuration |  | NR Band |  | SCS (kHz) | 5 MHz |  |  | 10 MHz |  | 15 MHz |  |  | 20 MHz |  |  | 25 MHz |  |  | 30 MHz |  |  | 40 MHz |  |  | 50 MHz |  |  | 60 MHz |  |  | 80 MHz |  |  | 90 MHz |  |  | 100 MHz |  |  | 200 MHz |  | 400 MHz |  | Maximum Aggregated bandwidth [MHz] |  | Bandwidth combination set |  |  |
| CA_n3A-n28A-n77(3A) |  | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A |  | n3 |  | 15 | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 350 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n28 |  | 15 | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(3A)-n257A |  | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 |  | 15 | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 730 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  |  |  |  |
| CA_n3A-n77(3A)-n257D |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D |  | n3 |  | 15 | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 730 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257D Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(3A)-n257G |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n77A-n257A CA_n77A-n257G |  | n3 |  | 15 | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 530 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257G Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(3A)-n257H |  | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H |  | n3 |  | 15 | Yes |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 630 |  | 0 |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 60 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | n257 |  | See CA_n257H Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(3A)-n257I | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n3 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 730 |  | 0 |  |
|  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n257 |  | See CA_n257I Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(3A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n28 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  | 0 |  |
|  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n257 |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |
|  |  |  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |
| CA_n28A-n77(3A)-n257D | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D |  | n28 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  | 0 |  |
|  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n257 |  | See CA_n257D Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(3A)-n257G | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n77A-n257A CA_n77A-n257G |  | n28 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 520 |  | 0 |  |
|  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n257 |  | See CA_n257G Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(3A)-n257H | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H |  | n28 |  | 15 |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 620 |  | 0 |  |
|  |  |  |  |  | 30 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n257 |  | See CA_n257H Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(3A)-n257I | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I |  | n28 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 720 |  | 0 |  |
|  |  |  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 |  | See CA_n77(3A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n257 |  | See CA_n257I Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes to be added to release independence TS 38.307.

of all REL-16 NR CA and NR DC combinations that fall into this category is defined by the WI title. See overview table in 4.1 above.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.716-03-02 | NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL |  | TSG#88 | Aijun CAO Cao.aijun@zte.com.cn |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | TSG#88 | Core part |
| 38.101-2 | User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone | TSG#88 | Core part |
| 38.101-3 | User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios | TSG#88 | Core part |
| 38.307 | EN-DC configurations | TSG#88 | Perf. part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Aijun CAO

Company:	ZTE

Email:	cao.aijun@zte.com.cn

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

None

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

CMCC

China Telecom

China Unicom

CATT

Huawei

HiSilicon

Samsung

Tejet