3GPP TR 36.716-04-01 V0.5.0 (2020-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

LTE inter-band CA for 4 bands DL with 1 band UL

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

4.1	TR Maintenance	6

5	4 Band Carrier Aggregation with Single UL: Specific Band Combination Part	7

5.1	CA_n1A-n3A-n8A-n78A	7

5.1.1	Channel bandwidths per operating bands for CA	7

5.1.2	∆TIB,c and ∆RIB,c values	7

5.1.3	REFSENS requirements	7

5.2	CA_n1A-n3A-n28A-n78A	8

5.2.1	Channel bandwidths per operating bands for CA	8

5.2.2	∆TIB,c and ∆RIB,c values	8

5.2.3	REFSENS requirements	8

5.3	CA_n3-n28-n77-n257	9

5.3.1	Channel bandwidths per operating bands for CA	9

5.3.2	∆TIB,c and ∆RIB,c values	9

5.3.3	REFSENS requirements	10

5.4	CA_n3-n28-n78-n257	10

5.4.1	Channel bandwidths per operating bands for CA	10

5.4.2	∆TIB,c and ∆RIB,c values	11

5.4.3	REFSENS requirements	11

5.5	CA_n7-n25-n66-n78	12

5.5.1	Channel bandwidths per operating band for CA	12

5.5.2	∆TIB and ∆RIB values	12

5.5.3	REFSENS requirements	12

5.6	CA_n1A-n3A-n7A-n28A, CA_n1A-n3A-n7B-n28A	13

5.6.1	Channel bandwidths per operating bands for CA	13

5.6.2	∆TIB,c and ∆RIB,c values	13

5.6.3	REFSENS requirements	13

5.7	CA_n1A-n3A-n7A-n78A, CA_n1A-n3A-n7B-n78A	14

5.7.1	Channel bandwidths per operating bands for CA	14

5.7.2	∆TIB,c and ∆RIB,c values	14

5.7.3	REFSENS requirements	14

5.8	CA_n3A-n7A-n28A-n78A, CA_n3A-n7B-n28A-n78A	15

5.8.1	Channel bandwidths per operating bands for CA	15

5.8.2	∆TIB,c and ∆RIB,c values	15

5.8.3	REFSENS requirements	16

Annex A: Change history	16

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report on inter-band CA for 4 bands DL with 1 band UL under Rel-16 time frame. The purpose is to gather the relevant background information and studies in order to address 4 bands DL/1 band UL Inter-band Carrier Aggregation requirements for the Rel-16 band combinations in Table 1-1.

Table 1-1: Release 16 4 bands DL/1 band UL inter-band carrier aggregation combinations

CA combination

CA_n1A-n3A-n8A-n78A

CA_n1A-n3A-n28A-n78A

CA_n3A-n28A-n77A-n257A

CA_n3A-n28A-n77A-n257D

CA_n3A-n28A-n77A-n257G

CA_n3A-n28A-n77A-n257H

CA_n3A-n28A-n77A-n257I

CA_n3A-n28A-n78A-n257A

CA_n3A-n28A-n78A-n257D

CA_n3A-n28A-n78A-n257G

CA_n3A-n28A-n78A-n257H

CA_n3A-n28A-n78A-n257I

CA_n3A-n28A-n77(2A)-n257A

CA_n3A-n28A-n77(2A)-n257D

CA_n3A-n28A-n77(2A)-n257G

CA_n3A-n28A-n77(2A)-n257H

CA_n3A-n28A-n77(2A)-n257I

CA_n3A-n28A-n77(3A)-n257A

CA_n3A-n28A-n77(3A)-n257D

CA_n3A-n28A-n77(3A)-n257G

CA_n3A-n28A-n77(3A)-n257H

CA_n3A-n28A-n77(3A)-n257I

CA_n7A-n25A-n66A-n78A

CA_n1A-n3A-n7A-n28A

CA_n1A-n3A-n7B-n28A

CA_n1A-n3A-n7A-n78A

CA_n1A-n3A-n7B-n78A

CA_n3A-n7A-n28A-n78A

CA_n3A-n7B-n28A-n78A

CA_n2A-n5A-n66A-n260A

CA_n2A-n5A-n30A-n260A

CA_n5A-n30A-n66A-n260A

CA_n2A-n30A-n66A-n260A

CA_n2A-n5A-n66A-n260M

CA_n2A-n5A-n30A-n260M

CA_n5A-n30A-n66A-n260M

CA_n2A-n30A-n66A-n260M

CA_n2(2A)-n5A-n30A-n66A

CA_n2A-n5A-n30A-n66(2A)

CA_n2A-n5A-n30A-n66A

This TR contains a band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		RP-191196, “New WID Rel-16 NR inter-band CA for 4 bands DL with 1 band UL”, RAN#84, Ericsson

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for 4 bands DL/1 band UL Inter-band Carrier Aggregation under Rel-16 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	4 Band Carrier Aggregation with Single UL: Specific Band Combination Part

## 5.1	CA_n1A-n3A-n8A-n78A

### 5.1.1	Channel bandwidths per operating bands for CA

Table 5.1.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A-n78A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes1 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes1 | Yes |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.1.2	∆TIB,c and ∆RIB,c values

- For three simultaneous DLs and one UL of Band n1, n3, n8 and n78, the TIB,c and RIB,c  values are shown in table 5.1.4-1 and  table 5.1.4-2, respectively. Values are derived from DC_1-3-8_n78.

Table 5.1.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n8-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n8-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 5.1.3	REFSENS requirements

- MSD requirements are captured in lower order combinations.

## 5.2	CA_n1A-n3A-n28A-n78A

### 5.2.1	Channel bandwidths per operating bands for CA

Table 5.2.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n28A-n78A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes2 |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes2 |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes1 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes1 | Yes |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.2.2	∆TIB,c and ∆RIB,c values

- For three simultaneous DLs and one UL of Band n1, n3, n28 and n78, the TIB,c and RIB,c  values are shown in table 5.2.4-1 and  table 5.2.4-2, respectively. Values are derived from DC_1-3-28_n78.

Table 5.2.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n28-n78 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 5.2.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n28-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 5.2.3	REFSENS requirements

- MSD requirements are captured in lower order combinations.

## 5.3	CA_n3-n28-n77-n257

### 5.3.1	Channel bandwidths per operating bands for CA

- Table 5.3.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n77A-n257A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n77A-n257D | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77A-n257G | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77A-n257H | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77A-n257I | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.3.2	∆TIB,c and ∆RIB,c values

- For three simultaneous DLs and one UL of Band combinations n3-n28-n77, n3-n28-n257, n3-n77-n257 and n28-n77-n257, the TIB,c and RIB,c  values are shown in table 5.3.4-1 and  table 5.3.4-2, respectively.

Table 5.3.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n77-n257 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n257 | 0 |

Table 5.3.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n77-n257 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n257 | 0 |

### 5.3.3	REFSENS requirements

- MSD requirements are captured in the lower order combinations.

## 5.4	CA_n3-n28-n78-n257

### 5.4.1	Channel bandwidths per operating bands for CA

Table 5.4.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n78A-n257A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n78A-n257D | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257G | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257H | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257I | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.4.2	∆TIB,c and ∆RIB,c values

- For three simultaneous DLs and one UL of Band combinations n3-n28-n78, n3-n28-n257, n3-n78-n257 and n28-n78-n257, the TIB,c and RIB,c  values are shown in table 5.4.4-1 and  table 5.4.4-2, respectively.

Table 5.4.4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n78-n257 | n3 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 5.4.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n28-n78-n257 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

### 5.4.3	REFSENS requirements

- MSD requirements are captured in the lower order combinations.

### 5.5	CA_n7-n25-n66-n78

#### 5.5.1	Channel bandwidths per operating band for CA

Table 5.5.1-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n7A-n25A-n66A-n78A | - | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

#### 5.5.2	∆TIB and ∆RIB values

For three simultaneous DLs and one UL of Band combinations n7-n25-n66, n7-n66-n78 and n25-n66-n78, the TIB,c and RIB,c  values are shown in table 5.5.2-1 and  table 5.5.2-2, respectively.

Table 5.5.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n25-n66-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 5.5.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n25-n66-n78 | n7 | 0.5 |
|  | n25 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

#### 5.5.3	REFSENS requirements

- MSD requirements are captured in the lower order combinations.

## 5.6	CA_n1A-n3A-n7A-n28A, CA_n1A-n3A-n7B-n28A

### 5.6.1	Channel bandwidths per operating bands for CA

Table 5.6.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n7A-n28A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n3A-n7B-n28A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.6.2	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n7-n28 the TIB,c and RIB,c  values are shown in table 5.6.2-1 and  table 5.6.2-2, respectively. Values are derived from DC_1-3-7_n28.

Table 5.6.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n7-n28 | n1 | 0.6 |
|  | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |

Table 5.6.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n7-n28 | n1 | 0 |
|  | n3 | 0 |
|  | n7 | 0 |
|  | n28 | 0.2 |

### 5.6.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.7	CA_n1A-n3A-n7A-n78A, CA_n1A-n3A-n7B-n78A

### 5.7.1	Channel bandwidths per operating bands for CA

Table 5.7.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n7A-n78A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n1A-n3A-n7B-n78A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 5.7.2	∆TIB,c and ∆RIB,c values

For CA_n1-n3-n7-n78 the TIB,c and RIB,c  values are shown in table 5.7.2-1 and  table 5.7.2-2, respectively. Values are derived from DC_1-3-7_n78.

Table 5.7.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n7-n78 | n1 | 0.7 |
|  | n3 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

Table 5.7.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n3-n7-n78 | n1 | 0.3 |
|  | n3 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |

### 5.7.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

## 5.8	CA_n3A-n7A-n28A-n78A, CA_n3A-n7B-n28A-n78A

### 5.8.1	Channel bandwidths per operating bands for CA

Table 5.8.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n7A-n28A-n78A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n3A-n7B-n28A-n78A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n7 | See CA_n7B Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 5.8.2	∆TIB,c and ∆RIB,c values

For CA_n3-n7-n28-n78 the TIB,c and RIB,c  values are shown in table 5.8.2-1 and  table 5.8.2-2, respectively. Values are derived from DC_3-7-28_n78.

Table 5.8.2-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n7-n28-n78 | n3 | 0.6 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.6 |

Table 5.8.2-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n7-n28-n78 | n3 | 0.2 |
|  | n7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 5.8.3	REFSENS requirements

MSD requirements are captured in lower order combinations.

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2019-08 | 3GPP RAN4#92 | R4-1909786 |  |  | Initial TR skeleton |  | 0.0.1 |
| 2019-11 | 3GPP RAN4#93 | R4-1914684 |  |  | Implemented TP´s from RAN4 #92bis: R4-1912260, “TP for TR 38.716-04-01 to include CA_n1-n3-n8-n78”, Ericsson, Swisscom R4-1912261, “TP for TR 38.716-04-01 to include CA_n1-n3-n28-n78”, Ericsson, Swisscom | 0.0.1 | 0.1.0 |
| 2020-02 | 3GPP RAN4#94 | R4-2001504 |  |  | Implemented TP´s from RAN4 #92bis: R4-1912238, “TP for TR 38.716-04-01: updated scope of the NR 4-band CA REL-16 WI”, Ericsson Implemented TP´s from RAN4 #93: R4-1913672, “TP for TR 38.716-04-01: NR CA_n3-n28-n77-n257”, SoftBank Corp R4-1913673, “TP for TR 38.716-04-01: NR CA_n3-n28-n78-n257”, SoftBank Corp | 0.1.0 | 0.2.0 |
| 2020-04 | 3GPP RAN4#94 bis | R4-2004578 |  |  | Correction of implementation of TP from RAN4 #93: R4-1913673, “TP for TR 38.716-04-01: NR CA_n3-n28-n78-n257”, SoftBank Corp Implemented TP from RAN4 #94: R4-2001508, “TP for TR 38.716-04-01 for updated scope from RAN #86”, Ericsson | 0.2.0 | 0.3.0 |
| 2020-05 | 3GPP RAN4#94 bis | R4-2005869 |  |  | Implemented TP from RAN4 #94bis: R4-2004581, “TP for TR 38.716-04-01 for updated scope from RAN #87”, Ericsson R4-2004072, “TP to TR 38.716-04-01 for CA_n7-n25-n66-n78”, Huawei, HiSilicon, Bell Mobility, Telus | 0.3.0 | 0.4.0 |
| 2020-06 | 3GPP RAN4#95 | R4-2006047 |  |  | Implemented TP’s from RAN4 #95: R4-2006611, “TP to TR 38.716-04-01 for CA_n7-n25-n66-n78”, Huawei, HiSilicon, Bell Mobility, Telus R4-2007631, “TP for TR 38.716-04-01 to include CA_n1-n3-n7-n28”, Ericsson, Telstra R4-2007632, “TP for TR 38.716-04-01 to include CA_n1-n3-n7-n78”, Ericsson, Telstra R4-2007633, “TP for TR 38.716-04-01 to include CA_n3-n7-n28-n78”, Ericsson, Telstra | 0.4.0 | 0.5.0 |