3GPP TSG RAN-WG4 Meeting #95-e	R4-2009119

Electronic Meeting, May 25-June 05, 2020

Title:	[DRAFT] LS on impact of NTA offset on UE Rx-Tx time difference measurement

Response to:

Release:	Rel-16

Work Item:	NR_pos-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name: Muhammad Kazmi

Tel. Number: +46730371481

E-mail Address: Muhammad.kazmi@ericsson.com

Attachments: None

1. Overall Description:

RAN4 would like to inform that gNB can configure the UE with any one of the three possible values of NTA,offset, in FR1. However, LMF is not aware of the NTA,offset used in the UE’s serving cell where the UE transmits SRS for performing the UE Rx-Tx time difference measurement. The values of NTA,offset are defined in Table 7.1.2-2 of TS 38.133.

Therefore, UE needs to signal the NTA offset used in the UE’s serving cell along with the Rx-Tx time difference measurement results to the LMF only once during the positioning session. The use of NTA,offset is up to the LMF implementation.

2. Actions:

To TSG-RAN WG2group.

ACTION: 	RAN4 requests RAN2 to kindly specify necessary signalling in their specification.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-WG4 Meeting #96-e      	August 17-28, 2020, Electronic Meeting

TSG-WG4 Meeting #96-ebis     	October 26-Nov 13, 2020, Electronic Meeting