3GPP TSG-RAN WG4 Meeting #95-e	R4-2007842

Electronic Meeting, 25 May – 5 June, 2020

Title:	[draft] reply LS on agreements related to NR Positioning

Response to:	R1-1913522

Release:	Release 16

Work Item:	NR_pos-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

#### Name:	Li Zhang

E-mail Address:	zhangli164@huawei.com

Attachments: -

1. Overall Description:

RAN4 discussed the scheduling restriction and need for measurement gaps due to PRS measurement, which is related to the issue pointed out in LS R1-1913522.

Agreement: In case DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE, at least in FR2, the UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE. Behaviour in FR1 is up to RAN4 to decide. To RAN4: ACTION: RAN1 also respectfully asks RAN4 to decide the UE behaviour in FR1 for DL PRS processing when other DL signals and channels are transmitted to the UE in the active BWP without a measurement gap.

Based on RAN4 discussion, the UE behaviour in FR1 (for DL PRS processing when other DL signals and channels are transmitted to the UE in the active BWP without a measurement gap) depends on whether UE requests measurement gaps for PRS measurement

- If UE requested measurement gaps for PRS measurement but the gaps are not configured, UE behaviour is same as for FR2

- If UE did not request measurement gaps for PRS measurement, UE is expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE

2. Actions:

To RAN1:

RAN4 kindly asks RAN1 to take above information into account in the future specification work for positioning support.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #96			24 – 28 August 2020, Toulouse, France

TSG-RAN4 Meeting #96bis		12 – 16 October 2020, Kochi, India