3GPP TSG-RAN WG4 Meeting#95e	R4-2006292

e-meeting, 25th May – 5th  June, 2020

Agenda Item:	14.2.4

Source:	CATT, CMCC

Title:	Study on support of unsynchronized operation between TDD bands

Document for:	Decision

- Introduction

This contribution will discuss the demand on unsynchronized operation for some TDD bands.

- Discussion

The RF requirements for some TDD band combinations have been specified only based on synchronized operation. E.g.

EN-DC/CA between Band 39 and Band 41

EN-DC/CA between Band 34 and Band 41

EN-DC/CA between Band 40 and Band 41

Considering the need of network deployment flexibility, it is necessary to introduce unsynchronized operation for these combinations as well. However, the Band 41 filter may have some problem to mitigate both WLAN interference at 2.4GHz and interference around 2GHz. There are might be the following two options to carry out such work

Option 1: Set up a study item to carry out corresponding study.

Once there is a conclusion for the combinations in the SI, the specification work for each combination can be moved to each basket WI.

Option 2: Carry out study under each combination discussion.

The benefit of the first option is that the SI discussion are more focus and more traceable. And we just update the basket WID for those combinations that is feasible.

The potential WIs to be impacted include,

EN-DC basket WI

NR CA basket WI

Proposal 1: It is proposed to set up a study item in RAN#88e to carry out the corresponding studies for unsynchronized operation between TDD bands.

- Conclusion

This paper discussed the needs to study unsynchronized operation between TDD bands. And we came up with the following proposal for approval.

Proposal: It is proposed to set up a study item in RAN#88e to carry out the corresponding studies.