3GPP TSG RAN WG 4 Meeting #95-e		TDoc R4-2006127

Electronic Meeting, 25 May - 5 June, 2020

3GPP TSG RAN WG 1 Meeting #100bis-e		TDoc R1-2003069

Electronic Meeting, 20 - 30 April, 2020

Title:	LS on criterion of pathloss measurement failure for power control of SRS for positioning

Response to:

Release:	Rel-16

Work Item:	NR_POS

Source:	RAN1

To:	RAN4

Cc:

Contact person:	Xiaotao Ren

renxiaotao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN1 had discussed the issue of criterion for inaccurate pathloss measurement for power control of SRS for positioning. RAN1 agreed that the pathloss reference for SRS for positioning may be a SSB from the serving cell, a SSB from a non-serving cell, a CSI-RS resource from the serving cell, a DL-PRS resource from a TRP of the serving cell, or a DL-PRS resource from a TRP of a non-serving cell.

According to the descriptions in current specification 38.213 as shown below, if the UE determines that the UE is not able to accurately measure, the UE calculates  using a RS resource obtained from the SSB of the serving cell that the UE uses to obtain MIB.

--------------------------------------------Start of related Descriptions in 38.213 g10------------------------------------- 7.3.1 UE behaviour ----------------------------------------------------- Uncorrelated part omitted ----------------------------------------------------- If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config on active UL BWP of carrier of serving cell , the UE determines the SRS transmission power in SRS transmission occasion as [dBm] where, - and are provided by p0 and alpha respectively, for active UL BWP of carrier of serving cell , and SRS resource set is indicated by SRS-ResourceSetId from SRS-ResourceSet, and - is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell , using RS resource indexed in a serving or non-serving cell for SRS resource set [6, TS 38.214]. A configuration for RS resource index associated with SRS resource set is provided by pathlossReferenceRS - if a ssb-Index is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower If the UE determines that the UE is not able to accurately measure , the UE calculates using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain. ----------------------------------------------------- Uncorrelated part omitted ----------------------------------------------------- -------------------------------------------End of related Descriptions in 38.213 g10---------------------------------------

RAN1 had identified possible ambiguity in the above descriptions, specifically the meaning of “the UE is not able to accurately measure”. It is RAN1 understanding that the criterion of inaccurate pathloss measurement is within the scope of RAN4’s work.

# 2	Actions

To RAN4:

ACTION: 	RAN1 kindly requests RAN4 to take the above information into consideration. RAN1 kindly requests RAN4 to define the criterion for determining whether or not a UE can measure the pathloss accurately. Any feedback on the defined criterion for inaccurate pathloss measurement would help RAN1 to clarify the meaning of “the UE is not able to accurately measure” in TS 38.213.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-RAN WG1 Meeting #101-e		25 May – 5 June 2020, 		e-Meeting

TSG-RAN WG1 Meeting #102		24 – 28 August 2020, 		Toulouse, France