3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010494

Electronic Meeting, 17-28 Aug., 2020

Agenda item:	7.4.1.3

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on IAB-MT feature list

Document for:	Discussion

- Introduction

In the previous RAN4 meetings, the feature lists of IAB-MT was discussed, and the agreements were captured in WF [1]. We further present our views about the remaining issues in this paper

- Discussion

In general aspect, in RAN#88-e it was approved that RAN1&4 take into account the signaling framework used by RAN2 to finalize the IAB-MT feature list [2].

T1-P1: RAN1&4 take into account the signaling framework used by RAN2 and finalize the IAB-MT feature list.

And in RAN1 reply LS [3], RAN1 agreed the feedback that in general the mentioned features can be made optional for IAB-MTs with no impact from a RAN1 perspective. Hence we focus to discuss the remaining issues identified in [1].

- The following features need more study, companies can provide technical analysis in next meeting.

  - 2-8, 2-11, 2-12, 3-1, 3-2 and 3-3.

For 3-1, 3-2 and 3-3, it was agreed in RAN#88-e [2] that these RAN4 features related to topology adaptation should remain optional.

T2-P3: RF/RRM Rel-15 UE Features related to topology adaptation (i.e. FG 3-1/3-2/3-3) should remain optional for IAB-MTs in Rel-16.

For 2-8, power class is not defined for IAB-MT, hence it could be not needed. We agreed to define two IAB-MT classes for potentially different RF requirement sets, Wide Area IAB-MT and Local Area IAB-MT. It was agreed that no upper limit on the output power for FR2 IAB-MT and wide area IAB-MT of FR1. It is FFS on Local Area IAB-MT. The output power for IAB-MT is a declared parameter.

For 2-11, the output power is declared by vendor and MPR might be not applicable to IAB-MT. Modified MPR behaviour should be not applicable for IAB-MT.

For 2-12, the feature is applicable for the roaming UE. For IAB-MT it is not necessary since it is deployed by the operator and the requirements should be known beforehand.

- Conclusion

The paper discusses the remaining issues for IAB-MT feature list and provides our views for related features.

References

[1] R4-2009067, WF on IAB-MT features, CATT

[2] RP-201292, Summary of email discussion [IAB_UE_features], AT&T

[3] R1-2004954, Reply LS on RAN4 IAB-MT feature list agreement, RAN WG1