3GPP TSG-RAN WG4 Meeting #96-e 	R4-2011294

Online, 17 – 28 Aug 2020

Source: 	Huawei

Title: 	IAB-MT Sensitivity parameters

Agenda Item:	7.4.2.2.1

Document for:	Approval

# Introduction

In the last meeting almost all the IAB-MT Rx sensitivu6y parameters were agreed however there was a remaining open item on the FRC’s used for the sensitivity requirement and their effect on the final sensitivity requirement.

The WF R4-2008785 states:

FRC to be used by IAB-MT : Selected UE FRC can be for IAB-MT based the same criteria as BS

Companies are encouraged to bring FRC proposals in next RAN4 meeting.

These agreements have been implemented in the text proposals in this document.

# Discussion

The IAB-MT will be based on the same criteria as the BS but will be UL based so hence the FRC’s will be similar to the UE.

The BS and the UE sensitivity figures are calculated as follows:

## 2.1 BS Reference sensitivity derivation

The derivation of the BS sensitivity requirement is clearly documented in TR 38.817-2

Reference sensitivity shall be specified according following formula:

Where:

-	BW is the maximum transmission bandwidth

-	NF is BS noise figure, equal toN 5 dB for Wide Area BS, 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for which we reach 95% throughput. Each company provided simulation results, and average will be done for each BW.

The SNR for each FRC is described in the annex

Table B.3 Simulation results summary for RF Fixed Reference Channels

|  |  |  | Ericsson R4-1802310 R4-1802311 R4-1802312 R4-1802313 R4-1802314 | Huawei R4-1801675 R4-1801676 R4-1801677 R4-1801678 R4-1601679 | ZTE R4-1802412 R4-1802414 R4-1802415 R4-1802416 R4-1802417 | CATT R4-1801447 R4-1801448 R4-1801450 | Nokia R4-1802737 R4-1802738 R4-1802742 | Average | Standard Deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS | FR1 | G-FR1-A1-1 | -1.1 | -1.5 | -1.3 | -1.1 | -1.1 | -1.2 | 0.2 |
|  |  | G-FR1-A1-2 | -0.6 | -1 | -1 | -0.8 | -0.8 | -0.8 | 0.2 |
|  |  | G-FR1-A1-3 | -0.7 | -1 | -1 | -0.8 | -0.8 | -0.9 | 0.1 |
|  |  | G-FR1-A1-4 | -1.1 | -1.2 | -1.2 | -1 | -1.1 | -1.1 | 0.1 |
|  |  | G-FR1-A1-5 | -1.2 | -1.2 | -1.3 | -1.2 | -1.2 | -1.2 | 0 |
|  |  | G-FR1-A1-6 | -1 | -1.3 | -1.2 | -1.1 | -0.9 | -1.1 | 0.2 |

The FRC’s are defined in the annex with the following appropriate characteristics.

Table 2.1-1: Wide area Refsens derivation for FR1 conducted

|  | BS Channel BW (MHz) | Subcarrier spacing (kHz) | Number of PRBS | Symbols per block | SNR (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| G-FR1-A1-1 | 5 | 15 | 25 | 12 | -1.2 | -101.7 |
| G-FR1-A1-2 | 5 | 30 | 11 | 12 | -0.8 | -101.8 |
| G-FR1-A1-3 | 10 | 60 | 11 | 12 | -0.9 | -98.9 |
| G-FR1-A1-4 | 20 | 15 | 106 | 12 | -1.1 | -95.3 |
| G-FR1-A1-5 | 20 | 30 | 51 | 12 | -1.2 | -95.6 |
| G-FR1-A1-6 | 20 | 60 | 24 | 12 | -1.1 | -95.7 |

The IAB-MT will however use the UL FRCs in the same way as the UE.

## 2.2 UE Reference sensitivity derivation

The derivation of the UE sensitivity values is not quite as clear as the BS.

In TR 38.817-1 the receiver sensitivity is derived as follows:

For below 6GHz, the REFSENS level can be calculated by the equation below:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was tentatively agreed as -1dB, the tentative value is the same as that for LTE. However, SNR will be further evaluated by link level simulation. Actually, the SNR under AWGN channel has no difference for different SCS and the demodulation performance for DFT-s-OFDM and CP-OFDM is the same as well. Therefore, the same SNR will be used to calculate REFSENS for different SCS.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for LTE refarming bands for NR.

In 38.101-1 the FRC data is given in Annex A, for example annex A.3.2

For example:

Table 2.2-1: UE FRC derivation FR1, 15kHz FRC

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 1672 | 3368 | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 1.338 | 2.694 | 4.096 | 5.530 | 6.970 | 8.403 | 11.270 | 13.9392 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

## 2.3 Comparing BS and UE FRC’s

The BS uses a smaller number of FRC’s to specify sensitivity, with the same FRC (and associated noise BW) for different channel BW’s.

The UE has a different FRCs for each channel BW and hence has a larger number of test cases.

The BS has the following cases:

Table 2.3-1: FR1 BS RFC signal BWs

|  |  | SCS (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 15 |  | 30 | 60 |
| CBW (MHz) | 5 | 4.5 | 4.32 | x | x |
|  | 10 | 4.5 | 4.32 | 3.96 | 7.92 |
|  | 15 | 4.5 | 4.32 | 3.96 | 7.92 |
|  | 20 | 19.08 | 18.9 | 18.36 | 17.28 |
|  | 25 | 19.08 | 18.9 | 18.36 | 17.28 |
|  | 30 | 19.08 | 18.9 | 18.36 | 17.28 |
|  | 40 | 19.08 | 18.9 | 18.36 | 17.28 |
|  | 50 | 19.08 | 18.9 | 18.36 | 17.28 |
|  | 60 | x | x | 18.36 | 17.28 |
|  | 70 | x | x | 18.36 | 17.28 |
|  | 80 | x | x | 18.36 | 17.28 |
|  | 90 | x | x | 18.36 | 17.28 |
|  | 100 | x | x | 18.36 | 17.28 |

There are only FRC’s and hence sensitivity requirements per SCS.

The UE has the following cases:

Table 2.3-2: FR1 UE RFC signal BWs

|  |  | SCS (kHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 | 30 | 60 |
| CBW (MHz) | 5 | 4.5 | 3.96 | x |
|  | 10 | 9.36 | 8.64 | 7.92 |
|  | 15 | 14.22 | 13.68 | 12.96 |
|  | 20 | 19.08 | 18.36 | 17.28 |
|  | 25 | 23.94 | 12.4 | 22.32 |
|  | 30 | 28.8 | 28.08 | 27.36 |
|  | 40 | 38.88 | 38.16 | 36.72 |
|  | 50 | 48.6 | 47.88 | 46.8 |
|  | 60 | x | 58.32 | 56.88 |
|  | 70 | x | x | x |
|  | 80 | x | 78.12 | 77.04 |
|  | 90 | x | 88.2 | 87.12 |
|  | 100 | x | 98.28 | 97.2 |

The highlighted cells represent the same BW’ as those use for the BS, with the same number of RB’s and sub-carriers.

It is a simple task therefore to replace the FRC’s used in the BS sensitivity requirement with UE FRC’s of the same noise BW.

Table 2.3-2: FRC’s for the FR1 IAB-MT

| BS Reference channel | equivalent UE reference channel (TS 38.101-1, Annex A3.3) |
| --- | --- |
| G-FR1-A1-1 | Table A.3.3.2-1, 5MHz CBW |
| G-FR1-A1-2 | Table A.3.3.2-2, 5MHz CBW |
| G-FR1-A1-3 | Table A.3.3.2-3, 10MHz CBW |
| G-FR1-A1-4 | Table A.3.3.2-1, 20MHz CBW |
| G-FR1-A1-5 | Table A.3.3.2-2, 20MHz CBW |
| G-FR1-A1-6 | Table A.3.3.2-3, 20MHz CBW |

## 2.4	Other sensitivity parameters

The Other parameters used to derive the sensitivity requirements are:

- Noise figure

- SNR

- IM

The Noise figure is a HW parameters based on the receiver loss and LNA noise figure. The front end of the IAB-MT receiver its antenna etc are more like a BS than a UE. It has also been agreed that the IAB_MT requirements are based on the same criteria as the BS, hence the BS noise figure assumptions can be used (for wide area)

Below 6GHz : NF= 5dB7

24.25 – 33.4 GHz: NF=10dB

37 – 52.6 GHz; NF = 12dB

Implementation margin is based on the HW implementation, for the BS 2dB is used, for the UE 2.5dB is used. Once again as the assumption is that the HW is like a BS and we will use the same criteria as the BS the IM used is 2dB.

The SNR parameter is based on simulation and is derived from the results from multiple vendors. For the BS this is captured in TR 38.817-2 but for the UE in TR 38.817-1 it states the SNR will be -1dB for all SCS.

In this case the SNR is based on the modulation and hence using the UE value is sensible, the BS value varies between -0.8 to -1.2dB so there will be a small difference ion some of the final sensitivity values for The IAB-MT compared to the same BS values.

## 2.5	FR1 sensitivity requirements.

Using all the values derived in the previous sections we can calculate the final IAB-MT sensitivity requirements:

Note the shaded rows refer to NB-IoT cases which are not relevant for IAB.

Table 2.5-1: NR Wide Area IAB-MT reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | BS Reference sensitivity power level, PREFSENS | signal BW | IM | SNR | NF | IAB-MT reference sensitivity power level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (dBm) | MHz | dB | dB | dB | dBm |
| 5, 10, 15 | 15 | Table A.3.3.2-1, 5MHz CBW (Note 1) | -101.7 | 4.5 | 2 | -1 | 5 | -101.5 |
|  |  | G-FR1-A1-10 (Note 3) | -101.7 (Note 2) |  |  |  |  |  |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW (Note 1) | -101.8 | 3.96 | 2 | -1 | 5 | -102.0 |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW (Note 1) | -98.9 | 9.92 | 2 | -1 | 5 | -98.0 |
| 20, 25, 30, 40, 50 | 15 | Table A.3.3.2-1, 20MHz CBW (Note 1) | -95.3 | 19.08 | 2 | -1 | 5 | -95.2 |
|  |  | G-FR1-A1-11 (Note 4) | -95.3 (Note 2) |  |  |  |  |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW (Note 1) | -95.6 | 18.36 | 2 | -1 | 5 | -95.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW (Note 1) | -95.7 | 17.28 | 2 | -1 | 5 | -95.6 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |  |
| NOTE 2: Void |  |  |  |  |  |  |  |  |
| NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |  |  |  |  |
| NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |  |  |  |  |  |

Table 2.5-2: Local Area IAB-MT reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS | signal BW | IM | SNR | NF | IAB-MT reference sensitivity power level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (dBm) | MHz | dB | dB | dB | dBm |
| 5, 10, 15 | 15 | Table A.3.3.2-1, 5MHz CBW (Note 1) | -93.7 | 4.5 | 2 | -1 | 13 | -93.5 |
|  |  | G-FR1-A1-10 (Note 3) | -93.7 (Note 2) |  |  |  |  |  |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW (Note 1) | -93.8 | 3.96 | 2 | -1 | 13 | -94.0 |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW (Note 1) | -90.9 | 9.92 | 2 | -1 | 13 | -90.0 |
| 20, 25, 30, 40, 50 | 15 | Table A.3.3.2-1, 20MHz CBW (Note 1) | -87.3 | 19.08 | 2 | -1 | 13 | -87.2 |
|  |  | G-FR1-A1-11 (Note 4) | -87.3 (Note 2) |  |  |  |  |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW (Note 1) | -87.6 | 18.36 | 2 | -1 | 13 | -87.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW (Note 1) | -87.7 | 17.28 | 2 | -1 | 13 | -87.6 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |  |
| NOTE 2: Void |  |  |  |  |  |  |  |  |
| NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. |  |  |  |  |  |  |  |  |
| NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |  |  |  |  |  |

## 2.5	FR2 sensitivity requirements.

As with FR1 for the FR2 BS FRC’s there are equivalent UE with the same signal BW.

Figure 2.5-1 FR2 REFSENS BS FRC’s

|  |  | SCS (kHz) |  |
| --- | --- | --- | --- |
|  |  | 60 | 120 |
| CBW (MHz) | 50 | 47.52 | 46.08 |
|  | 100 | 47.52 | 95.04 |
|  | 200 | 47.52 | 95.04 |
|  | 400 | x | 95.04 |

Figure 2.5-2 FR2 REFSENS UE FRC’s

|  |  | SCS (kHz) |  |
| --- | --- | --- | --- |
|  |  | 60 | 120 |
| CBW (MHz) | 50 | 47.52 | 46.08 |
|  | 100 | 95.04 | 95.04 |
|  | 200 | 190.08 | 95.04 |
|  | 400 | x | 95.04 |

As with FR1 the equivalent signal BW for the UE FRC’s are the same as the BS so the same parameters can be used.

Figure 2.5-3 FR2 equivalent IAB-MT FRC’s

| BS Reference channel | equivalent UE reference channel (TS 38.101-2, Annex A3.3) |
| --- | --- |
| G-FR2-A1-1 | Table A.3.3.2-1, 50MHz CBW |
| G-FR2-A1-2 | Table A.3.3.2-2, 50MHz CBW |
| G-FR2-A1-3 | Table A.3.3.2-3, 100MHz CBW |

The other parameter likely to change is the SNR value, for BS the following were used:

Table 10.3.3.4-1: SNR values for FR2 OTA RF sensitivity

| FRC | SNR (dB) |
| --- | --- |
| G-FR2-A1-1 | -1.1 |
| G-FR2-A1-2 | -1.1 |
| G-FR2-A1-3 | -1.2 |

There are no values for the UE FR2 listed in 37.817-1, however these are in the same range as the FR1 SNR values, as such using -1dB as with FR1 is a good assumption.

As for FR2 the OTA range is a declared value inside a range and the range has been rounded to 1dB, the additional possible 0.1dB difference due to the SNR value would be lost in the rounding process. As such the same declaration range as the BS can be used for the UE. Note it is agreed already that the LA IAB-MT will use the range covering both LA and MR BS.

# 3	Summary

In this paper we have looked at the FRCs and the other parameters which are used to derive the reference sensitivity requirements.

The UE requirement has many more FRCs as each is modified to optimize the usage of the channel BW, however for the BS the same FRC is used for many channel BW (for example 4.5MHz signal BW is used for 5, 10 and 20MHz channel BW cases).

It has previously been agreed that the selected UE FRC can be for IAB-MT based the same criteria as BS.

As such the appropriate signal BW FRC is selected from the UE definitions and used in the same way as the BS FRC’s to generate the IAB-MT specification.

In addition the SNR value specified or the UE is different, as this is dependent on the modulated signal the UE value is used which changes the final reference sensitivity values slightly from the BS values.

The sensitivity values for FR1 are given in tables 2.3-1 and 2.3-2 in this document, the equivalent FRC;s for the IAB-MT are given in table 2.3-3

For FR2 the as the value is in a range which is rounded to 1dB and the difference in the SNR is small the range is the same for both IAB-MT and the BS. The equivalent FRC’s re given in table 2.5-3