3GPP TSG-RAN WG4 Meeting #97-e                                                             R4-2014398

# Electronic Meeting, 2 - 13 Nov., 2020

Source: 	CATT

Title: 	Simulation results for NR HST PUSCH demodulation requirement

Agenda item:	7.15.3.2.1

Document for:	Discussion

- Introduction

In the RAN4#96-e meeting, introducing NR HST PUSCH requirements for multi-path fading channel with high Doppler values in WF [1] was approved as follows:

- Multi-path fading scenarios under consideration - The following models are under consideration: - TDLC300-600 FO=0Hz TDLC300-1200 FO=0Hz - Specification of multi-path fading channel under high Doppler - Introduce PUSCH requirements for HST multi-path fading channel under high Doppler, with configuration - TDLC300-600 FO=0Hz (15kHz), TDLC300-1200 FO=0Hz (30kHz) - MCS: 2 - Waveform: CP-OFDM - DM-RS: 1+1+1 - Antenna: 1T2R - Bandwidth: 5MHz for 15kHz SCS, 10MHz for 30kHz SCS

This paper will provide our evaluation results for NR HST PUSCH requirement for multi-path fading channel with high Doppler values for simulation alignment.

- Evaluation results

The simulation assumptions [1] are shown in Table A-1 in Annex.

The evaluation results for  NR HST PUSCH requirement for multi-path fading channel with high Doppler values are shown in Table 2-1.

Table 2-1: Evaluation results for NR HST PUSCH requirement for multi-path fading channel with high Doppler values

| Channel model | Propagation condition | Carrier frequency (GHz) | SCS | CBW | Frequency Offset(Hz) | Tx/Rx | MCS | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal | Impairment |
| Multi-path fading | TDLC300-600 | 2.1 | 15KHz | 5MHz | 0 | 1T2R | 2 | -4.51 | -2.01 |
|  |  | 3.6 | 30KHz | 10MHz | 0 | 1T2R | 2 | -4.94 | -2.44 |
|  | TDLC300-1200 | 2.1 | 15KHz | 5MHz | 0 | 1T2R | 2 | -1.46 | 1.04 |
|  |  | 3.6 | 30KHz | 10MHz | 0 | 1T2R | 2 | -4.62 | -2.12 |

- Conclusion

This paper provides our evaluation results for NR HST PUSCH requriement for multi-path fading channel with high Doppler values for simulation alignment.

- References

- R4-2012676, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#96-e.

- Annex

Table A-1: Simulation assumptions for NR HST PUSCH with CP-OFDM

| Parameter | value |  |  |
| --- | --- | --- | --- |
|  | v=350km/h | V=500km/h | multi-path fading |
| Transform precoding | Disabled | Disabled | Disabled |
| Number of Tx | 1 | 1 | 1 |
| Number of Rx | Tunnel: 2, 1 Open space: 2, 8 | Tunnel: 2, 1 Open space: 2, 8 | 2 |
| Number of layers | 1 | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| symbols length | 14 | 14 | 14 |
| start symbol index | 0 | 0 | 0 |
| Time domain resource allocation type | Type A | Type A | Type A |
| MCS index | Tunnel: MCS 2 and MCS 16(excluding for 1T1R) Open space: MCS 2 and MCS 16 | Tunnel: MCS 2, MCS 16 (excluding for 1T1R) Open space: MCS 2 and MCS 16 | MCS 2 |
| PTRS | Not configured | Not configured | Not configured |
| Timing offset | 0 | 0 | 0 |
| Frequency offset | 0 | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 | 4 |
| RV | {0,2,3,1} | {0,2,3,1} | {0,2,3,1} |
| DMRS type | type 1 | Type1 | Type1 |
| Number of DMRS symbols | 1+1+1 | 1+1+1 | 1+1+1 |
| Front loaded DMRS start symbol | l0 = 2 or 3 | l0 = 2 or 3 | l0 = 2 or 3 |
| SCS and BW | 15kHz: 10MHz/5MHz; 30kHz: 40MHz/10MHz |  | 15kHz: 5MHz; 30kHz: 10MHz |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth | Full applicable test bandwidth |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz | 15kHz SCS: 1.8GHz 30kHz SCS: 3.6GHz | 15kHz SCS: 1.8GHz 30kHz SCS: 3.6GHz |
| fd | 15kHz SCS: 1340Hz 30kHz SCS: 2334Hz | 15kHz SCS: 1740Hz 30kHz SCS: 3334Hz | 15kHz SCS: TDLC300-600 FO=0Hz 30kHz SCS: TDLC300-1200 FO=0Hz |
| Propagation condition | Tunnel: Ds=300m, Dmin=2m; Open space: Ds=700m, Dmin=150m |  | N/A |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput | SNR @70% of maximum throughput |