3GPP TSG-RAN WG4 Meeting # 97-e 	                              R4-2015511

Electronic Meeting, 2-13 Nov., 2020

Agenda item:	7.4.5.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on test cases for IAB

Document for:	Discussion

- Introduction

The core requirements for IAB were completed in the RAN4#96e meeting [1], and the performance works are scheduled for the coming RAN4#97e meeting. We have a separate paper to discuss the principles and general considerations to move the performance work forward [2]. In this paper, we further provide our views in details about each requirement case by case based on our observations in [2].

- Discussion

As analyzed in [2], compared with the original UE, the IAB-MT has some different characteristics in terms of scenarios, mobility, IAB classes, etc. It could be observed that the channel conditions for IAB-MT is much stable than the original UE as the IAB is considered without mobility and it is deployed with pre-planning by operators especially for wide area IAB. Thus, it is reasonable to consider simplifying the performance requirements for IAB-MT compared with the performance requirements for the original UE which has multiple test cases with different configurations for each requirements. Also there are some issues should be taken in to account when designing the test cases for IAB. Taking TS 38.133 as the baseline, we provide our views on the test case shall be defined corresponding to the core requirements in the following parts.

RRC re-establishment and RRC release with redirection

For RRC re-establishment testing for original UE, we have multiple test cases in different scenarios for intra/inter frequency, known/unknown target cell, FR1/FR2, with/without serving cell timing. As for IAB-MT, there is no need to consider all these combinations as these maybe not typical scenarios for IAB-MT. Firstly, as the requirements for cell re-selection and RRM measurement are not defined for IAB-MT, it is not easy to make sure that the IAB-MT under testing has the information of the target cell (either intra frequency or inter frequency), thus it is proposed to only considered the unknown target cell in the testing. Also for the inter-frequency RRC re-establishment, as defined in the core requirements, the total delay is related to the number of inter frequency layers. In the test requirements for the original UE, UE shall establish the RRC connection to the inter-frequency target cell immediately after searching the intra-frequency layers and the information of the inter frequency is given to the UE prior to the test. However, for the IAB-MT, for the same reason, there is no cell re-selection and RRM measurement procedures defined for IAB-MT, thus it is not easy for IAB-MT to directly turn to the target frequency configured in the test. The worst case could be that the IAB-MT finds the target frequency after sweeping all possible frequencies, so it is hard to define number of frequency layers. What more, for IAB-MT, there could prior information about the candidate frequencies for re-establishment as the IAB is deployed by planning, thus how the IAB-MT search for the inter-frequency for re-establishment may depends on IAB-MT’s implementations.

Proposal 1: As there is no cell reselection and RRM measurement requirements, only unknown target cell should be considered in the testing.

Observation 1: It hard to define the number of frequency layers for inter frequency RRC re-establishment.

Another issue is related to different types of IAB-MT. According to RRM discussion, the RLM and link recovery requirements are only defined for local area IAB-MT. As the typical manner to trigger the RRC re-establishment is through RLF, to our best knowledge, there is no reliable manner to trigger the RRC re-establishment for wide area IAB-MT in the testing.

Observation 2: How to trigger the RRC re-establishment for wide area IAB-MT should be further discussed.

Based on the analysis above, it is proposed to define the test cases for RRC re-establishment as follows:

| Test cases | IAB type | IAB-MT class | Conducted/OTA |
| --- | --- | --- | --- |
| Intra-frequency RRC Re-establishment in FR1 with unknown target cell | IAB 1-H | Wide area / Local area | conducted |
| Intra-frequency RRC Re-establishment in FR1 with unknown target cell | IAB 1-O | Wide area / Local area | OTA |
| Intra-frequency RRC Re-establishment in FR2 with unknown target cell | IAB 2-O | Wide area / Local area | OTA |

And for the RRC release with redirection, it is suggested to have the following test cases:

| Test cases | IAB type | IAB-MT class | Conducted/OTA | Parameters |
| --- | --- | --- | --- | --- |
| Redirection from NR in FR1 to NR in FR1 | IAB 1-H | Wide area / Local area | conducted | Non-DRX |
| Redirection from NR in FR1 to NR in FR1 | IAB 1-O | Wide area / Local area | OTA | Non-DRX |
| Redirection from NR in FR2 to NR in FR2 | IAB 2-O | Wide area / Local area | OTA | Non-DRX |

Random Access

It could be observed from the performance requirements for original UE, there are multiple test cases to test the UE behavior in the random access process especially when the random access is considered unsuccessful. For original UE, the random access process could usually happen caused by various reasons, and the unsuccessful random access is also quite common as the UE could be in dense scenarios and the conflicts could usually happen. However, for IAB-MT, the density is much lower than original UE and there is IAB-MT specific RA resource as defined in RAN1’s spec [3] by prach-ConfigurationPeriodScaling, prach-ConfigurationFrameOffset and prach-ConfigurationSOffset.

Observation 3: There is specific RA resource defined for IAB-MT.

Observation 4: The RA process for IAB-MT is much simple and happens infrequently compared with original UE.

Also for the original UE, the corresponding test cases are defined according to the absolute/relative power tolerance defined in TS 38.101. However, for IAB-MT, there is no absolute power tolerance and the relative power tolerance is only defined for local area IAB-MT, which means it is hard to conduct the corresponding testing.

Based on the observation above, we suggest there is no need to test the RA process of IAB-MT as what has been defined for original UE. It could be observed that the test cases for RRC re-establishment and RRC release with re-direction are ended with the successful transmission of PRACH on the target cell. From our understanding, there is no need to define separate test cases for random access for IAB-MT.

Proposal 2: It is suggested not to define separate test cases of RA for IAB-MT.

Transmit timing and Timing advance

For the test cases for transmit timing and timing advance of IAB-MT, it is suggested to have following test cases and take TS 38.133 as the baseline.

| Test cases | IAB type | IAB-MT class | Conducted/OTA | Parameters |
| --- | --- | --- | --- | --- |
| Transmit Timing Test for FR1 | IAB 1-H | Wide area / Local area | conducted | Non-DRX |
| Transmit Timing Test for FR1 | IAB 1-O | Wide area / Local area | OTA | Non-DRX |
| Transmit Timing Test for FR2 | IAB 2-O | Wide area / Local area | OTA | Non-DRX |
| FR1 timing advance adjustment accuracy | IAB 1-H | Wide area / Local area | conducted | Non-DRX |
| FR1 timing advance adjustment accuracy | IAB 1-O | Wide area / Local area | OTA | Non-DRX |
| FR2 timing advance adjustment accuracy | IAB 2-O | Wide area / Local area | OTA | Non-DRX |

RLM and link recovery

The RLM and link recovery requirements are only defined for local area IAB-MT, obviously the corresponding test cases are only for local area IAB-MT. There will be test cases for SSB-based and CSI-RS based RLM for FR1 and FR2 for OOS and IS evaluations, which means for certain type of IAB, there will be 4 test cases for RLM and 2 test cases for BFD and link recovery as only non-DRX mode are considered.

The test case for RLM are suggested as follows:

| Test cases | IAB type | IAB-MT class | Conducted/OTA | Parameters |
| --- | --- | --- | --- | --- |
| RLM OOS Test for FR1 PCell configured with SSB-based RLM RS | IAB 1-H | Local area | conducted | Non-DRX |
| RLM OOS Test for FR1 PCell configured with SSB-based RLM RS | IAB 1-O | Local area | OTA | Non-DRX |
| RLM OOS Test for FR2 PCell configured with SSB-based RLM RS | IAB 2-O | Local area | OTA | Non-DRX |
| RLM IS Test for FR1 PCell configured with SSB-based RLM RS | IAB 1-H | Local area | conducted | Non-DRX |
| RLM IS Test for FR1 PCell configured with SSB-based RLM RS | IAB 1-O | Local area | OTA | Non-DRX |
| RLM IS Test for FR2 PCell configured with SSB-based RLM RS | IAB 2-O | Local area | OTA | Non-DRX |
| RLM OOS Test for FR1 PCell configured with CSI-RS-based RLM RS | IAB 1-H | Local area | conducted | Non-DRX |
| RLM OOS Test for FR1 PCell configured with CSI-RS -based RLM RS | IAB 1-O | Local area | OTA | Non-DRX |
| RLM OOS Test for FR2 PCell configured with CSI-RS -based RLM RS | IAB 2-O | Local area | OTA | Non-DRX |
| RLM IS Test for FR1 PCell configured with CSI-RS -based RLM RS | IAB 1-H | Local area | conducted | Non-DRX |
| RLM IS Test for FR1 PCell configured with CSI-RS -based RLM RS | IAB 1-O | Local area | OTA | Non-DRX |
| RLM IS Test for FR2 PCell configured with CSI-RS -based RLM RS | IAB 2-O | Local area | OTA | Non-DRX |

The test cases for BFD and link recovery are suggested as follows:

| Test cases | IAB type | IAB-MT class | Conducted/OTA | Parameters |
| --- | --- | --- | --- | --- |
| BFD and LR Test for FR1 PCell configured with SSB-based BFD and LR | IAB 1-H | Local area | conducted | Non-DRX |
| BFD and LR Test for FR1 PCell configured with SSB-based BFD and LR | IAB 1-O | Local area | OTA | Non-DRX |
| BFD and LR Test for FR2 PCell configured with SSB-based BFD and LR | IAB 2-O | Local area | OTA | Non-DRX |
| BFD and LR Test for FR1 PCell configured with CSI-RS-based BFD and LR | IAB 1-H | Local area | conducted | Non-DRX |
| BFD and LR Test for FR1 PCell configured with CSI-RS -based BFD and LR | IAB 1-O | Local area | OTA | Non-DRX |
| BFD and LR Test for FR2 PCell configured with CSI-RS -based BFD and LR | IAB 2-O | Local area | OTA | Non-DRX |

Proposal 3: Define the test cases in the above tables for RRM requirements for IAB.

- Conclusions

Proposal 1: As there is no cell reselection and RRM measurement requirements, only unknown target cell should be considered in the testing.

Observation 1: It hard to define the number of frequency layers for inter frequency RRC re-establishment.

Observation 2: How to trigger the RRC re-establishment for wide area IAB-MT should be further discussed.

Observation 3: There is specific RA resource defined for IAB-MT.

Observation 4: The RA process for IAB-MT is much simple and happens infrequently compared with original UE.

Proposal 2: It is suggested not to define separate test cases of RA for IAB-MT.

Proposal 3: Define the test cases in the above tables for RRM requirements for IAB.

References

[1] R4-2012234 on RLM requirements for IAB-MT, ZTE Corporation

[2] R4-2015510

[3] TS 38.211