3GPP TSG-RAN4 Meeting #98-e	R4-2103553

# Online, 25 Jan. - 5 Feb., 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1461 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on activation time in direct SCell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For known target cell in FR1, in legacy SCell activation delay Tactivation_time is distinct for whether measCycleSCell is larger than 160ms or not. The background is that if the latest measurement sample is more than 160ms ago, then UE may need additional time for AGC setting. On one hand, the measurement sampling depends on not only measCycleSCell, but also the fact that whether DRX is configured and how many carriers are configured to be measured. If UE is configured with long DRX and/or large number of carriers to mesure, then the actual measurement sampling rate would be much longer than 160ms. On the other hand, in direct SCell activation scenario above condition doesn’t apply anymore. measCycleSCell is used to control the measurement on deactivated SCell. In direct SCell actiation procedure, the SCell “skips” the deactivated state and directly enters activated state. Therefore, measCycleSCell is not suitable as condition for direct SCell activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Tactivation_time for known target cell in FR1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tactivation_time in direct SCell activation delay requirement for known target cell in FR1 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.3.4, 8.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 8.3.4	Direct SCell Activation at SCell addition

The requirements in this clause apply for UE being configured in the RRC reconfiguration message, TS 38.331 [2], with one SCell for which the parameter sCellState is set to activated.

The UE shall configure the SCell in activated state upon successful completion of the RRC reconfiguration procedure within the specified delay. Upon receiving the RRC reconfiguration message in slot n, the UE shall be capable to transmit valid CSI report and apply actions for the directly activated SCell no later than in slot n+NdirectNR slot length ,

where:

Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3ms

TRRC_Process: RRC procedure delay defined in clause 12 of TS 38.331 [2],

T1: Delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCConnectionReconfigurationComplete message,

Note:	T1 is UE implementation dependent.

If the SCell is known and belongs to FR1, TCSI_Reporting is specified in clause 8.3.2 and Tactivation_time is defined as:

-	TFirstSSB+ 5ms, if the SCell has been measured within measurement gap before activation and TSSB_measurement_period_inter, as specified in Table 9.3.5-1, is equal to or smaller than 1280ms; or if the SCell has been measured without measurement gap before activation and T SSB_measurement_period_intra, as specified in Table 9.3.9-1, is equal to or smaller than 800ms.

-	TFirstSSB_MAX + Trs + 5ms, if the SCell has been measured within measurement gap before activation and TSSB_measurement_period_inter, as specified in Table 9.3.5-1, is larger than 1280ms; or if the SCell has been measured without measurement gap before activation and T SSB_measurement_period_intra, as specified in Table 9.3.9-1, is larger than 800ms.

Otherwise, Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX shall override the existing ones:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n +  TRRC_Process+T1NR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n +  TRRCProcess+T1NR slot length

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS38.321 [7] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The UE may be allowed to cause interruptions to serving cells on other component carriers during an interruption window, as specified in clause 8.2. The starting point of an interruption window on spCell or any activated SCell shall not occur before slot n+1, and shall not occur after slot n+1+TRRC_Process+T1+TXNR slot length, where NR slot length is with respect to the numerology of the SCell being activated, and TX is:

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes TFineTiming.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

Starting from the slot n+TRRC_Process + T1NR slot length until the UE has completed the direct SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

### 8.3.5	Direct SCell Activation at Handover

The requirements in this clause apply for UE being configured in the RRC reconfiguration message, TS 38.331 [2], for handover with one SCell for which the parameter sCellState is set to activated.

The UE shall configure the SCell in activated state upon successful completion of the RRC reconfiguration procedure within the specified delay. Upon receiving the RRC reconfiguration message in slot n, the UE shall be capable to transmit valid CSI report and apply actions for the directly activated SCell no later than in slot n+NdirectNR slot length ,

Where:

Ndirect = TRRC_process + Tinterrupt + T2 + T3 + Tactivation_time + TCSI_Reporting - 3ms

TRRC_Process: RRC procedure delay defined in clause 12 of TS 38.331 [2],

Tinterrupt: Interruption time during handover as specified in clause 6.1.1,

T2: Delay from slot n+TRRC_Process+TinterruptNR slot length until UE has obtained a valid TA command for the target PCell,

T3: Delay for applying the received TA for uplink transmission in the target PCell, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213,

If the SCell is known and belongs to FR1, TCSI_Reporting is specified in clause 8.3.2 and Tactivation_time is defined as:

-	TFirstSSB+ 5ms, if the SCell has been measured within measurement gap before activation and TSSB_measurement_period_inter, as specified in Table 9.3.5-1, is equal to or smaller than 1280ms; or if the SCell has been measured without measurement gap before activation and T SSB_measurement_period_intra, as specified in Table 9.3.9-1, is equal to or smaller than 800ms.

-	TFirstSSB_MAX + Trs + 5ms, if the SCell has been measured within measurement gap before activation and TSSB_measurement_period_inter, as specified in Table 9.3.5-1, is larger than 1280ms; or if the SCell has been measured without measurement gap before activation and T SSB_measurement_period_intra, as specified in Table 9.3.9-1, is larger than 800ms.

Otherwise, Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX shall override the existing ones:

-	TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n +  TRRC_Process+Tinterrupt+T2+T3NR slot length

-	TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n +  TRRCProcess+Tinterrupt+T2+T3NR slot length

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.321 [7] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The UE may be allowed to cause interruptions to PCell during an interruption window, as specified in clause 8.2. The starting point of an interruption window on PCell shall not occur before slot n+1+ TRRC Processing+Tinterrupt+T2+T3NR slot length, and not occur after slot n+1+TRRC Processing+Tinterrupt+T2+T3+TXNR slot length, where NR slot length is with respect to the numerology of the SCell being activated, and TX is:

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes TFineTiming.

The length of the interruption window depends on the frequency band relation between the aggressor SCell and the victim PCell.

Starting from the slot n+TRRC_Process + Tinterrupt+T2+ T3NR slot length and until the UE has completed the direct SCell activation, the UE shall report CQI index = 0 (out of range) if the UE has available uplink resources to report CQI for the SCell.

### 8.3.6	Direct SCell Activation at RRCResume

The requirements in this clause apply for UE being configured in the RRC reconfiguration message in TS38.331 [2] for RRC Resume with one SCell for which the parameter sCellState is set to activated.

The requirements in clause 8.3.4 shall apply, except that the definition of T1 shall be deemed to be replaced with

T1: Delay from slot n+TRRC_ProcessNR slot length until the transmission of RRCResumeComplete message,

End of Change