3GPP TSG-RAN WG4 Meeting #98-e                     	  R4-2102697

# Electronic Meeting, Jan. 25 - Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7055 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to requirements for NCSG patterns |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  | Date: |  |  | 15/01/2021 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing references in Table 8.1.2.1.2-1 to interruption contol sections for NCSG usage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 3.3, abbreviation for term NCSG is added, since not contained in TS 21.905. In subclause 7.10.2.4, a reference is editorally corrected. In subclause 8.1.2.1.2, two notes and one figure are moved to appropriate locations, missing references to interruption contol sections for NCSG pattern usage are added in Table 8.1.2.1.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined references to interruption contol sections for NCSG usage remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.10.2.4, 8.1.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [26] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [26].

1x RTT	CDMA2000 1x Radio Transmission Technology

ABS	Almost Blank Subframe

ARQ	Automatic Repeat Request

AP	Access Point

AWGN	Additive White Gaussian Noise

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

CA	Carrier Aggregation

CC	Component Carrier

CCCH SDU	Common Control Channel SDU

CE	Coverage Enhancement

CGI	Cell Global Identifier

CPICH	Common Pilot Channel

CPICH Ec/No	CPICH Received energy per chip divided by the power density in the band

CRS	Cell-specific Reference Signals

C-RNTI	Cell RNTI

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DL	Downlink

DMTC	Discovery signal Measurement Timing Configuration

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DUT	Device Under Test

E-CID	Enhanced Cell-ID (positioning method)

ECGI	Evolved CGI

eDRX_IDLE	Extended IDLE-mode DRX

eDRX_CONN	Extended CONNECTED-mode DRX

eNB	E-UTRAN NodeB

EN-DC	E-UTRA-NR Dual Connectivity

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FDD	Frequency Division Duplex

FS3	Frame Structure type 3

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigational Satellite System

GSM	Global System for Mobile communication

HARQ	Hybrid Automatic Repeat Request

HD-FDD	Half-Duplex FDD

HO	Handover

HRPD	High Rate Packet Data

IDC	In-Device Coexistence

IEEE	Institute of Electrical and Electronics Engineers

LBT	Listen before talk

LPP	LTE Positioning Protocol

LWA	LTE-WLAN Aggregation

MAC 	Medium Access Control

MCG	Master Cell Group

MeNB	Master eNB

MBMS	Multimedia Broadcast Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MBSFN ABS	MBSFN Almost Blank Subframe

MDT	Minimization of Drive Tests

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MPDCCH	MTC Physical Downlink Control Channel

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NG-RAN	NG  Radio Access Network

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NSA	Non-standalone

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPSS	Narrowband Primary Synchronization Signal

NRS	Narrowband Refernce Signal

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NSCH	Narrowband Synchronization Channel

NSSS 	Narrowband Secondary Synchronization Signal

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference of Arrival

PBCH	Physical Broadcast Channel

P-CCPCH	Primary Common Control Physical Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PHICH	Physical Hybrid-ARQ Indicator CHannel

PLMN	Public Land Mobile Network

PMCH	Physical Multicast Channel

PRACH	Physical Random Access CHannel

ProSe	Proximity-based Services

PRS	Positioning Reference Signal

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control Channel

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared CHannel

psTAG	Primary Secondary Timing Advance Group

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared Channel

RS-SINR	Reference Signal Signal to Noise and Interference Ratio

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

QAM	Quadrature Amplitude Modulation

RACH	Random Access Channel

RAT	Radio Access Technology

RNC	Radio Network Controller

RNTI	Radio Network Temporary Identifier

RRC	Radio Resource Control

RRM	Radio Resource Management

SCE	Small Cell Enhancement

SCH	Synchronization Channel

SCell	Secondary Cell

SCG	Secondary Cell GroupSDU	Service Data Unit

SCS	Subcarrier spacing

SeNB	Secondary eNB

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SLSS	SideLink Synchronization Sequence

SON	Self Optimized Network

SPDCCH	Short Physical Downlink Control channel

SPUCCH	Short Physical Uplink Control channel

SRS	Sounding Reference Signal

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSS	Secondary Synchronization Signal

SSTD	SFN and subframe time difference

sTAG	Secondary Timing Advance Group

TAG	Timing Advance Group

TDD	Time Division Duplex

TP	Transmission Point

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunication System

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

WLAN	Wireless Local Area Network

WB-RSRQ	Wide Bandwith RSRQ

End of Change 1

Start of Change 2

#### 7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements

If the UE is configured for RSTD measurements on cells belonging to a SCC with deactivated SCell(s) and also with a measCycleSCell for performing E-UTRA carrier aggregation measurements as defined in Section 8.3 on the same SCC as configured for the RSTD measurements, then the total allowed interruption on the active serving cell(s) is the maximum of the interruption due to E-UTRA carrier aggregation measurements specified in Section 7.8 and the interruption due to the RSTD measurements on SCC specified in this Section.

End of Change 2

Start of Change 3

##### 8.1.2.1.2	Network controlled small gap

A UE may reconfigure the receiver bandwidth, carrier frequency or turn on/off one of the RF chains when performing measurements on PCell, activated SCell/PSCell, deactivated SCell and/or unused RF chain. This may cause interruptions on PCell or activated SCell(s) or both that are specified in Section 7.8.

If the UE requires network controlled small gap (NCSG) to prevent the interruption and UE is not configured with asynchrouns DC,

-	When UE is not configured with measurement gap, the E-UTRAN can explicitly provide a single NCSG pattern with constant repetition period per UE.

-	When UE is configured with Gap Pattern ID #0 on some of, but not all, serving carriers including PCC and SCC(s), a single NCSG pattern with NCSG Pattern ID #0 in Table 8.1.2.1.2-1 can be implicitly configured on the serving carrier(s), where measurement gap is not configured.

Note:	As shown in Figure 8.1.2.1.2-1, subframes of serving carrier 1 from i+1 to i+6 are used as measurement gap. The NCSG can be implicitly configured on other serving carrier subframes from j+1 to j+6, where no measurement gap is configured.

Figure 8.1.2.1.2-1: Measurement GAP and NCSG

-	When UE is configured with Gap Pattern ID #1 on some of, but not all, serving carriers including PCC and SCC(s), a single NCSG pattern with NCSG Pattern ID #1 in Table 8.1.2.1.2-1 can be implicitly configured on the serving carrier(s), where measurement gap is not configured.

-	When UE measurement gap is configured on all serving carriers including PCC and SCC(s), NCSG should not be configured.

Note:	As shown in Figure 8.1.2.1.2-1, subframes of serving carrier 1 from i+1 to i+6 are used as measurement gap. The NCSG can be implicitly configured on other serving carrier subframes from j+1 to j+6, where no measurement gap is configured.

Figure 8.1.2.1.2-1: Measurement GAP and NCSG

If the UE requires NCSG to prevent the interruption and the UE supporting asynchronous DC is configured with PSCell which is asynchnous with PCell,

-	When there is no measurement gap configured among MCG and SCG cell subframes, the E-UTRAN can explicitly provide a single NCSG pattern with constant repetition period per serving carrier.

-	When Gap Pattern ID #0 is configured for UE on MCG (or SCG) and no measurement gap is configured on SCG (or MCG), a single NCSG pattern with NCSG Pattern ID #2 can be implicitly configured on SCG (or MCG).

Note:	As shown in Figure 8.1.2.1.2-2, one serving carrier subframes from i+1 to i+6 are used as measurement gap. The NCSG can be implicitly configured on other serving carrier subframes from j+1 to j+7, where no measurement gap is configured.

-	When Gap Pattern ID #1 is configured for UE on MCG (or SCG) and no measurement gap is configured on SCG (or MCG), a single NCSG pattern with NCSG Pattern ID #3 can be implicitly configured on SCG (or MCG).

Note:	As shown in Figure 8.1.2.1.2-2, one serving carrier subframes from i+1 to i+6 are used as measurement gap. The NCSG can be implicitly configured on other serving carrier subframes from j+1 to j+7, where no measurement gap is configured.

Figure 8.1.2.1.2-2: Measurement GAP and NCSG for dual connectivity

UEs shall only support those NCSG patterns listed in Table 8.1.2.1.2-1 that are relevant to its measurement capabilities.

Table 8.1.2.1.2-1: NCSG Configurations supported by the UE

| NCSG Pattern Id | Visible interruption length before measurement (VIL1, ms) | Measurement Length during which there is no gap (ML, ms) | Visible interruption length after measurement (VIL2, ms) | Visible interruption Repetition Period (VIRP, ms) | Purpose |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 4 | DL: 1 UL: 2 | 40 | Interruption control according to requirements in sections 7.8.2.5, 7.8.2.6, 7.8.2.9, 7.8.2.12, 7.10.2.1, 7.10.2.2, 7.10.2.3 |
| 1 | 1 | 4 | DL: 1 UL: 2 | 80 | Interruption control according to requirements in sections 7.8.2.5, 7.8.2.6, 7.8.2.9, 7.8.2.12, 7.10.2.1, 7.10.2.2, 7.10.2.3 |
| 2 | 2 | 3 | 2 | 40 | Interruption control according to requirements in sections 7.8.2.5, 7.8.2.6, 7.8.2.9, 7.8.2.12, 7.10.2.1, 7.10.2.2, 7.10.2.3 |
| 3 | 2 | 3 | 2 | 80 | Interruption control according to requirements in sections 7.8.2.5, 7.8.2.6, 7.8.2.9, 7.8.2.12, 7.10.2.1, 7.10.2.2, 7.10.2.3 |

During the VIL1 and VIL2, the UE is not expected to transmit and receive any data. During ML, the UE is expected to transmit and receive data on the corresponding serving carrier(s).

A UE that is capable of identifying and measuring inter-frequency and/or inter-RAT cells without gaps, but needs interruption, and is configured with the network controlled small gap for such measurement (NCSG Pattern Id #0-3) shall follow requirements as if Gap Pattern Id #0 or Gap Pattern Id #1 had been used and shall not make any autonomous interruption outside the visual interruption of the configured network controlled small gap for the measurement, and the minimum available time Tinter1 of 60 ms and 30 ms shall be assumed for the corresponding requirement for visible interruption repetition period (VIRP) of 40 ms and 80 ms, respectively.

End of Change 3

For Information Only

#### 7.8.2.5	Interruptions during measurements on SCC for intra-band CA

If the UE supports ncsg-r14 and has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

PCell interruptions due to measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

If indicated by the network using IE allowInterruptions [2], PCell interruptions due to measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2].

Each interruption shall not exceed 5 subframes.

#### 7.8.2.6	Interruptions during measurements on SCC for inter-band CA

If the UE supports ncsg-r14 and has been configured with NCSG pattern with ID 0,1,2,3, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the UE has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.If the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, PCell interruptions due to measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

If the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, and if indicated by the network using IE allowInterruptions [2], PCell interruptions due to measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2].

Each interruption shall not exceed 1 subframe.

…

#### 7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells

If the PCell is not in the same band as any of the SCells being activated or deactivated and if the UE supports ncsg-r14 and has been configured with NCSG pattern with ID 0,1,2,3, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the PCell is not in the same band as any of the SCells being activated or deactivated and if the UE has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the PCell is in the same band as any of the SCells being activated or deactivated or if the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, when one SCell is deactivated, the UE is allowed due to measurements on the SCC with deactivated SCell:

-	an interruption on PCell with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] for the deactivated SCell is 640 ms or longer.

-	an interruption on PCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2] for the deactivated SCell if indicated by the network using IE allowInterruptions [2],

Each interruption shall not exceed:

-	1 subframes if the PCell is not in the same band as the deactivated SCell

-	5 subframes if the PCell is in the same band as the deactivated SCell

-	an interruption on any activated SCell with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] for the deactivated SCell is 640 ms or longer .

-	an interruption on any activated SCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2] for the deactivated SCell if indicated by the network using IE allowInterruptions [2].

Each interruption shall not exceed:

-	1 subframes if the activated SCell is not in the same band as the deactivated SCell

-	5 subframes if the activated SCell is in the same band as the deactivated SCell

If the PCell is in the same band as any of the SCells being activated or deactivated or if the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, when two, three, or four SCells are deactivated, the UE is allowed due to measurements on the SCCs with deactivated SCells:

-	an interruption on PCell with up to 0.5% probability of missed ACK/NACK when any of the configured measCycleSCell [2] for the deactivated SCells is 640 ms or longer.

-	an interruption on PCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2] for the deactivated SCells if indicated by the network using IE allowInterruptions [2].

-	an interruption on an activated SCell with up to 0.5% probability of missed ACK/NACK when any of the configured measCycleSCell [2] for the deactivated SCells is 640 ms or longer.

-	an interruption on an activated SCell with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell [2] for the deactivated SCells if indicated by the network using IE allowInterruptions [2].

Each interruption on the PCell shall not exceed:

-	1 subframes if the PCell is not in the same band as any of the deactivated SCells

-	5 subframes if the PCell is in the same band as any of the deactivated SCells

Each interruption on the activated Cell shall not exceed:

-	1 subframe if the activated SCell is not in the same band as any of the deactivated SCells

-	5 subframes if the the activated SCell is in the same band as any of the deactivated SCells

…

#### 7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3

If the UE supports ncsg-r14 and has been configured with NCSG pattern with ID 0,1,2,3, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the UE has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, and if one SCell is deactivated,

- the UE is allowed due to RSSI measurements on the SCC with deactivated SCell:

-	an interruption on PCell with up to 0.5% probability of missed ACK/NACK when any of the configured rmtc-Period [2] and the configured measCycleSCell [2] for the deactivated SCell is 640 ms or longer,

-	an interruption on any activated SCell with up to 0.5% probability of missed ACK/NACK when any of the configured rmtc-Period [2] and the configured measCycleSCell [2] for the deactivated SCell is 640 ms or longer.

- no interruption is allowed if both of the configured rmtc-Period [2] and the configured measCycleSCell [2] for the deactivated SCell are below 640 ms.

If the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers and if two, three, or four SCells are deactivated,

- the UE is allowed due to RSSI measurements on the SCCs with deactivated SCells:

-	an interruption on PCell with up to 0.5% probability of missed ACK/NACK when:

any of the configured rmtc-Period [2] and the configured measCycleSCell [2] for the deactivated SCells is 640 ms or longer, or

RSSI windows with the length of measDuration [2] for at least some of the SCCs with the deactivated SCells within their respective rmtc-Period [2] are not within [20] ms;

-	an interruption on an activated SCell with up to 0.5% probability of missed ACK/NACK when:

any of the configured rmtc-Period [2] and the configured and the configured measCycleSCell [2] for the deactivated SCells is 640 ms or longer, or

RSSI windows with the length of measDuration [2] for at least some of the SCCs with the deactivated SCells within their respective rmtc-Period [2] are not within [20] ms.

- no interruption is allowed if both of the configured rmtc-Period [2] and the configured measCycleSCell [2] for the deactivated SCell are below 640 ms and RSSI windows with the length of measDuration [2] for all the SCCs with the deactivated SCells within their respective rmtc-Period [2] are within [20] ms.

Each allowed interruption shall not exceed:

-	1 subframe on the PCell, and

-	5 subframes on the activated SCell.

…

#### 7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell

If the UE supports ncsg-r14 and has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

PCell interruptions due to RSTD measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the the PRS periodicityis 640 ms or longer. Each interruption shall not exceed 5 subframes.

#### 7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell

If the UE supports ncsg-r14 and has been configured with NCSG pattern with ID 0,1,2,3, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the UE has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, PCell interruptions due to RSTD measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the PRS periodicity  is 640 ms or longer. Each interruption shall not exceed 1 subframe.

#### 7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells

If the PCell is not in the same band as any of the SCells being activated or deactivated and if the UE supports ncsg-r14 and has been configured with NCSG pattern with ID 0,1,2,3, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the PCell is not in the same band as any of the SCells being activated or deactivated and if the UE has been configured with gap pattern with ID 0,1 and there is no inter-frequency and inter-RAT frequency layer to be monitored, the UE shall not make any autonomous interruptions outside of the configured gap patterns.

If the PCell is in the same band as any of the SCells being activated or deactivated or if the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, when one SCell is activated and the other SCell is deactivated, then due to RSTD measurements on the SCC with deactivated SCell the UE is allowed:

●	an interruption on PCell with up to 0.5% probability of missed ACK/NACK when when the PRS periodicity  is 640 ms or longer. Each interruption shall not exceed:

○	1 subframe if the PCell is not in the same band as the deactivated SCell

○	5 subframes if the PCell is in the same band as the deactivated SCell

●	an interruption on the activated SCell with up to 0.5% probability of missed ACK/NACK when the PRS periodicityis 640 ms or longer. Each interruption shall not exceed:

○	1 subframe if the activated SCell is not in the same band as the deactivated SCell

○	5 subframes if the activated SCell is in the same band as the deactivated SCell

If the PCell is in the same band as any of the SCells being activated or deactivated or if the UE does not support ncsg-r14 or has not been configured with gap pattern with ID 0,1 or NCSG pattern with ID 0,1,2,3 or the UE has been configured with gap pattern with ID 0,1 to monitor inter-frequency/inter-RAT frequency layers, when both SCells are deactivated, then due to RSTD measurements on one or both SCCs with deactivated SCells the UE is allowed:

●	an interruption on PCell with up to 1.0% probability of missed ACK/NACK when the configure PRS periodicityis 640 ms or longer in any of the SCCs. Each interruption shall not exceed:

○	1 subframe if the PCell is not in the same band as any of the deactivated SCells

○	5 subframes if the PCell is in the same band as any of the deactivated SCells